SOURCES SOUGHT (MARKET SURVEY)

Replace Septic System for Namekagon River Visitor Center

Saint Croix National Scenic Riverway, Trego, WI

(Reference 140P6026B0008)

The National Park Service (NPS) is conducting a market survey to determine the interest and capability of contractors for a possible upcoming project.  All costs associated with providing information in response to this market survey shall be the responsibility of the firm.  Any information submitted is voluntary.  The Contracting Officer will make an acquisition strategy determination based on the results of this market survey.

The work shall take place at Saint Croix National Scenic Riverway, Trego, WI. A summary of the work follows:

The required work includes replacing the sanitary sewer and septic system from the Namekagon Visitor Center southwest to the new drain field. Demolition of the existing system will include the abandonment of the 5 existing septic tanks/cisterns and demo of any other existing components necessary for installation of the new system. The new septic tanks, lift station, and drain field shall be constructed outside of the existing system to allow for use of the existing system with minimal interruptions for cross over connections. The new sanitary sewer should be a 4-6” diameter PVC or HDPE for the gravity portion and 1.5”-2” diameter HDPE for the force main section. New septic tank will be installed and sized to meet State and County design requirements. New lift station will be required to distribute the wastewater to the new drain field. Lift station shall be a dual pump lift station with stainless steel components, integral wet well, and safety hatch. Drain field will be designed to meet State and County requirements pending the results of the septic test pits by a certified septic installer.

The work of this contract includes furnishing all labor, equipment, and materials necessary to complete all contract requirements.

The NAICS code for this project is 238910 - Site Preparation Contractors, with a small business size standard of $19M.

Contractors must have active registration as a small business and any socio-economic groups represented in SAM (System for Award Management) under NAICS code 238910 to be considered/confirmed.

All scope of work documents/specifications will be provided if/when the government proceeds with a formal solicitation.

Interested firms must submit the following information:

| Name of Firm as shown in SAM.gov |  |
| --- | --- |
| Address |  |
| Phone Number |  |
| Email Address |  |
| SAM UEI Number |  |
| SAM CAGE Code |  |

Size status/SBA certifications.  Mark all that apply.  See FAR Part 19, Small Business Programs.

☐ Small Business (meets size standard for NAICS)

☐ Women Owned Small Business (WOSB)

☐ Economically Disadvantaged Women Owned Small Business (EDWOSB)

☐ HUBZone

☐ Service Disabled Veteran Owned Small Business (SDVOSB)

☐ Indian Small Business Economic Enterprise (ISBEE)

☐ Other (specify) _________________

Capability statement - A brief summary of capabilities:  Contractor may reply below and/or provide attachments.

This section or the attachments MUST INCLUDE: - Brief explanation of your company’s ability/capabilities to perform the work required for this project. - All information being requested in the table above regarding Company Name, POC info, UEI Number, CAGE Code and all of the size status/SBA certifications.

This market research is being conducted to acquire additional information about potential contractors that may be interested in performing this work. This is not request for quote (RFQ) and does not constitute any commitment by the Government to issue any solicitation. All interested firms must respond to future solicitation announcements separately from responses to this market survey. Any solicitation that may be issued will be posted on SAM.gov.

All responses must be submitted via email not later than 1930 CDT, May 8, 2026, to email address james_bissaillon@ios.doi.gov and include the reference number (140P6026B0008) in the email subject line.