SALIENT CHARACTERISTICS

Brand Name or Equivalent

M5555 – Perimeter, Automated – Zeiss Humphrey Field Analyzer 3

- Must be capable in performing perimetry- visual field testing

- Liquid trial lens technology is included

- Must have automated eye alignment

- Fixation monitoring system (RelEYE) is a must

- Must have the capability to do guided progression analysis (GPA)

- Shall have fixation control: head tracking, gaze tracking, video eye monitor, Heijl- Krakau blind spot monitor, and vertex monitoring

- Must have visual field index

- Must include power supply and adapters for USA conventional 120V outlet

- Electrical Requirement must have: 100-120v- 50/60 Hz, 4.0A

- Must have smart touch interface

- Must have operating system: windows 10

- Shall have internal storage: 500 Gb

- Must include FORUM Connectivity

- Must include FORUM glaucoma Workplace

- Must have DICOM Connectivity

- Must include Integrated Diagnostic Imaging Platform for Glaucoma, (Glaucoma Workplace) connects Cirrus HD-OCT

- Must have SITA database

- Must include table

- Dimension: 18”D x 20”W x 23”H

- Quantity: 1