## B. 2 – DRAFT - Statement of Work

## ___________________________________________________

A. BACKGROUND

The Iowa City VA Health Care System is requesting a one-time purchase for a Carto 3 Mapping System. All equipment provided must Brand Name Only and be new; and which includes all the software, hardware, and any accessories required to be fully operational. This requirement is also requesting professional installation and clinical education and on-site training.

B. SALIENT CHARACTERISTICS

Specifications – Brand Name Only

Carto Mapping System:

- Must be capable of 3D electroanatomical mapping of cardiac chambers to support diagnosis of cardiac arrhythmias.

- Must have real-time visualization of catheter position, cardiac anatomy, and intracardiac electrical activity without continuous fluoroscopy.

- Must support catheter navigation, activation mapping, voltage mapping, and substrate mapping during EP procedures.

- Must have rapid generation of high-resolution anatomical maps of atria and ventricles.

- Must have voltage mapping identifying scar tissue and low-voltage regions.

- Must be able to visualize wavefront movement across cardiac tissue.

- Must have high-fidelity acquisition of bipolar and unipolar signals.

- Must have adjustable bandpass and notch filters for noise reduction.

- Must have simultaneous real-time display of surface ECG and intracardiac signals.

- Must have temporal alignment of electrical signals with spatial location.

- Must be able to integrate CT, MRI, and fluoroscopic images.

- Must be capable of anatomical image registration with real-time catheter position data.

- Must be capable of overlaying pre-acquired images onto live 3D maps.

- Must be compatible with RF and cryo-related mapping catheters.

- Must display ablation points with configurable parameters (time, power, contact force).

- Must have real-time visualization of catheter-tissue contact force and stability metrics.

- Must display ablation parameters (temperature and power) during energy delivery.

- Must have high-resolution multi-monitor workstation.

- Must connect to hospital network and EP recording systems.

nGEN System:

- RF Generator – cardiac radio-frequency (RF) ablation generator delivering RF energy to create ablative lesions in cardiac tissue as part of electrophysiology (EP) procedures.

  - Must be capable of temperature controlled and power-controlled RF ablation.

  - Must be capable of high-power ablations up to approximately 90W power delivery.

  - Must have multiple independent power delivery channels for segmental ablation procedures.

  - Must fully integrate with Carto3 system, allowing automatic recognition of catheter type and automatic loading of ablation settings.

  - Must have real-time temperature feedback from the catheter sensors to adjust irrigation and RF power to maintain safe ablation temperatures.

  - Must have touchscreen control.

  - Touchscreen must display procedural parameters: power, temperature, irrigation.

- Pump – peristaltic irrigation pump that supplies irrigation fluid to irrigated ablation catheters during RF delivery to control tissue temperature and lesion formation.

  - Must be automatically controlled by the nGen generator when connected.

  - Must adjust irrigation flow based on catheter type and RF delivery.

TRUPULSE Generator – Pulsed Field Ablation (PFA) energy generator to deliver controlled pulsed electrical energy to compatible cardiac ablation catheters to treat arrhythmias during electrophysiology procedures.

- Must deliver PFA energy to cardiac tissue via catheter.

- Must work with both PFA and RF energy delivery.

- Must have controlled electrical pulses for non-thermal ablation (irreversible electroporation) to cardiac tissue.

- Touchscreen must allow for selection of PFA or RF and delivery parameters.

- Must be capable of starting energy delivery via pedal or on-screen activation.

- Must have a safety cutoff to stop energy delivery immediately.

Warranty Requirements

Manufacturer must offer a (1) one-year minimum warranty that product is free from defects in materials and workmanship.  All defects will be replaced or repaired by the manufacturer at no cost to the buyer.

Installation

Installation is required by the Contractor.  Contractor shall contract the Point-of-Contact within ten (10) business days from issuance of award to schedule installation.

Training

Clinical Education and On-Site Training is required.  Contractor shall contact the Point-of-Contact within ten (10) business days from issuance of award to schedule the Clinical Education and On-Site Training.

C. PERIOD OF PERFORMANCE:

TBD

D.  TYPE OF CONTRACT

This is a Firm-Fixed-Priced, one-time supply contract with no option year(s).

E. POINT-OF-CONTACT (POC)

This contract is assigned a POC and will provide general instructions for administrating the contract.  The POC is not authorized to grant permission(s) to change the scope of the project. The POC that is assigned for this contract will ensure that the supplies/services provided by this company are received based on the contractual terms and conditions.

| Location | Iowa City Health Care System |
| --- | --- |
| POC: | TBD |

G. DELIVERABLES OR DELIVERY SCHEDULE

The vendor shall deliver the equipment via standard delivery with F.O.B. destination as scheduled. See Section C for delivery schedule period-of-performance.

The equipment shall be delivered to the following address:

Iowa City VA Health Care System

601 Highway 6 West

Iowa City, IA 52246

H. CHANGES TO CONTRACT

Before performing any action in a non-contractual nature or is considered out-of-scope, the Contractor shall contact the Point of Contact (POC) and the Contract Officer identifying the reason(s) for change. All changes shall have prior authorization from a VA Contract Officer in the form of a written modification.

I. POST-AWARD PERFORMANCE CONFERENCE

The POC, Contract Specialist, or Contract Officer may schedule a post-award performance conference with Contractor, if deemed necessary, for contract orientation purposes.

J. SECURITY

B1. GENERAL. This entire section applies to all acquisitions requiring any Information Security and Privacy language. Contractors, contractor personnel, subcontractors and subcontractor personnel will be subject to the same federal laws, regulations, standards, VA directives and handbooks, as VA personnel regarding information and information system security and privacy.

B2. VA INFORMATION CUSTODIAL LANGUAGE.

a. The Government shall receive unlimited rights to data/intellectual property first produced and delivered in the performance of this contract or order (hereinafter “contract”) unless expressly stated otherwise in this contract. This includes all rights to source code and all documentation created in support thereof. The primary clause used to define Government and Contractor data rights is FAR 52.227-14 Rights in Data – General. The primary clause used to define computer software license (not data/intellectual property first produced under this contractor or order) is FAR 52.227-19, Commercial Computer Software License.

b. Information made available to the contractor by VA for the performance or administration of this contract will be used only for the purposes specified in the service agreement, SOW, PWS, PD, and/or contract. The contractor shall not use VA information in any other manner without prior written approval from a VA Contracting Officer (CO). The primary clause used to define Government and Contractor data rights is FAR 52.227-14 Rights in Data – General.

c. VA information will not be co-mingled with any other data on the contractor’s information systems or media storage systems. The contractor shall ensure compliance with Federal and VA requirements related to data protection, data encryption, physical data segregation, logical data segregation, classification requirements and media sanitization.

d. VA reserves the right to conduct scheduled or unscheduled audits, assessments, or investigations of contractor Information Technology (IT) resources to ensure information security is compliant with Federal and V VA requirements. The contractor shall provide all necessary access to records (including electronic and documentary materials related to the contracts and subcontracts) and support (including access to contractor and subcontractor staff associated with the contract) to VA, VA's Office Inspector General (OIG), and/or Government Accountability Office (GAO) staff during periodic control assessments, audits, or investigations.

The contractor may only use VA information within the terms of the contract and applicable Federal law, regulations, and VA policies. If new Federal information security laws, regulations or VA policies become applicable after execution of the contract, the parties agree to negotiate contract modification and adjustment necessary to implement the new laws, regulations, and/or policies.

f. The contractor shall not make copies of VA information except as specifically authorized and necessary to perform the terms of the contract. If copies are made for restoration purposes, after the restoration is complete, the copies shall be destroyed in accordance with VA Directive 6500, VA Cybersecurity Program and VA Information Security Knowledge Service.

g. If a Veterans Health Administration (VHA) contract is terminated for default or cause with a business associate, the related local Business Associate Agreement (BAA) shall also be terminated and actions taken in accordance with VHA Directive 1605.05, Business Associate Agreements. If there is an executed national BAA associated with the contract, VA will determine what actions are appropriate and notify the contactor.

h. The contractor shall store and transmit VA sensitive information in an encrypted form, using VA-approved encryption tools which are, at a minimum, Federal Information Processing Standards (FIPS) 140-2, Security Requirements for Cryptographic Modules (or its successor) validated and in conformance with VA Information Security Knowledge Service requirements. The contractor shall transmit VA sensitive information using VA approved Transport Layer Security (TLS) configured with FIPS based cipher suites in conformance with National Institute of Standards and Technology (NIST) 800-52, Guidelines for the Selection, Configuration and Use of Transport Layer Security (TLS) Implementations.

i. The contractor’s firewall and web services security controls, as applicable, shall meet or exceed VA’s minimum requirements.

j. Except for uses and disclosures of VA information authorized by this contract for performance of the contract, the contractor may use and disclose VA information only in two situations: (i) in response to a qualifying order of a court of competent jurisdiction after notification to VA CO (ii) with written approval from the VA CO. The contractor shall refer all requests for, demands for production of or inquiries about, VA information and information systems to the VA CO for response.

k. Notwithstanding the provision above, the contractor shall not release VA records protected by Title 38 U.S.C. § 5705, Confidentiality of medical quality-assurance records and/or Title 38 U.S.C. § 7332, Confidentiality of certain medical records pertaining to drug addiction, sickle cell anemia, alcoholism or alcohol abuse or infection with Human Immunodeficiency Virus (HIV). If the contractor is in receipt of a court order or other requests for the above-mentioned information, the contractor shall immediately refer to such court order or other requests to the VA CO for response.

l. Information made available to the contractor by VA for the performance or administration of this contract or information developed by the contractor in performance or administration of the contract will be protected and secured in accordance with VA Directive 6500 and Identity and Access Management (IAM) Security processes specified in the VA Information Security Knowledge Service.

m. Any data destruction done on behalf of VA by a contractor shall be done in accordance with National Archives and Records Administration (NARA) requirements as outlined in VA Directive 6300, Records and Information Management, VA Handbook 6300.1, Records Management Procedures, and applicable VA Records Control Schedules.

n. The contractor shall provide its plan for destruction of all VA data in its possession according to VA Directive 6500 and NIST 800-88, Guidelines for Media Sanitization prior to termination or completion of this contract. If directed by the COR/CO, the contractor shall return all Federal Records to VA for disposition.

o. Any media, such as paper, magnetic tape, magnetic disks, solid state devices or optical discs that is used to store, process, or access VA information that cannot be destroyed shall be returned to VA. The contractor shall hold the appropriate material until otherwise directed by the Contracting Officer’s Representative (COR) or CO. Items shall be returned securely via VA-approved methods. VA sensitive information must be transmitted utilizing VA-approved encryption tools which are validated under FIPS 140-2 (or its successor) and NIST 800-52. If mailed, the contractor shall send via a trackable method (USPS, UPS, FedEx, etc.) and immediately provide the COR/CO with the tracking information. Self-certification by the contractor that the data destruction requirements above have been met shall be sent to the COR/CO within 30 business days of termination of the contract.

p. All electronic storage media (hard drives, optical disks, CDs, back-up tapes, etc.) used to store, process or access VA information will not be returned to the contractor at the end of lease, loan, or trade-in. Exceptions to this paragraph will only be granted with the written approval of the VA CO.

B3. ACCESS TO VA INFORMATION AND VA INFORMATION SYSTEMS. – This section is not applicable.

B4. TRAINING. – This section is not applicable.

B5. SECURITY INCIDENT INVESTIGATION.

a. The contractor, subcontractor, their employees, or business associates shall immediately (within one hour) report suspected security / privacy incidents to the VA OIT’s Enterprise Service Desk (ESD) by calling (855) 673-4357 (TTY: 711). The ESD is OIT’s 24/7/365 single point of contact for IT-related issues. After reporting to the ESD, the contractor, subcontractor, their employees, or business associates shall, within one hour, provide the COR/CO the incident number received from the ESD.

b. To the extent known by the contractor/subcontractor, the contractor/ subcontractor's notice to VA shall identify the information involved and the circumstances surrounding the incident, including the following: (1) The date and time (or approximation of) the Security Incident occurred.

(1) The date and time (or approximation of) the Security Incident occurred.

(2) The names of individuals involved (when applicable).

(3) The physical and logical (if applicable) location of the incident.

(4) Why the Security Incident took place (i.e., catalyst for the failure).

(5) The amount of data belonging to VA believed to have been compromised.

(6) The remediation measures the contractor is taking to ensure no future incidents of a similar nature.

c. After the contractor has provided the initial detailed incident summary to VA, they will continue to provide written updates on any new and relevant circumstances or facts they discover. The contractor, subcontractor, and their employes shall fully cooperate with VA or third-party entity performing an independent risk analysis on behalf of VA. Failure to cooperate may be deemed a material breach and grounds for contract termination.

d. VA IT contractors shall follow VA Handbook 6500, Risk Management Framework for VA Information Systems VA Information Security Program, and VA Information Security Knowledge Service guidance for implementing an Incident Response Plan or integrating with an existing VA implementation.

e. In instances of theft or break-in or other criminal activity, the contractor/subcontractor must concurrently report the incident to the appropriate law enforcement entity (or entities) of jurisdiction, including the VA OIG, and the VA Office of Security and Law Enforcement. The contractor, its employees, and its subcontractors and their employees shall cooperate with VA and any law enforcement authority responsible for the investigation and prosecution of any possible criminal law violation(s) associated with any incident. The contractor/subcontractor shall cooperate with VA in any civil litigation to recover VA information, obtain monetary or other compensation from a third party for damages arising from any incident, or obtain injunctive relief against any third party arising from, or related to, the incident.

f. The contractor shall comply with VA Handbook 6500.2, Management of Breaches Involving Sensitive Personal Information, which establishes the breach management policies and assigns responsibilities for the oversight, management and reporting procedures associated with managing of breaches.

g. With respect to unsecured Protected Health Information (PHI), the contractor is deemed to have discovered a data breach when the contractor knew or should have known of breach of such information. When a business associate is part of VHA contract, notification to the covered entity (VHA) shall be made in accordance with the executed BAA.

h. If the contractor or any of its agents fails to protect VA sensitive personal information or otherwise engages in conduct which results in a data breach involving any VA sensitive personal information the contractor/subcontractor processes or maintains under the contract; the contractor shall pay liquidated damages to the VA as set forth in clause 852.211-76, Liquidated Damages—Reimbursement for Data Breach Costs.

B6. INFORMATION SYSTEM DESIGN AND DEVELOPMENT.

a. Information systems designed or developed on behalf of VA at non-VA facilities shall comply with all applicable Federal law, regulations, and VA policies. This includes standards for the protection of electronic Protected Health Information (PHI), outlined in 45 C.F.R. Part 164, Subpart C and information and system security categorization level designations in accordance with FIPS 199, Standards for Security Categorization of Federal Information and Information Systems and FIPS 200, Minimum Security Requirements for Federal Information Systems. Baseline security controls shall be implemented commensurate with the FIPS 199 system security categorization (reference VA Handbook 6500 and VA Trusted Internet Connections (TIC) Architecture).

b. Contracted new developments require creation, testing, evaluation, and authorization in compliance with VA Assessment and Authorization (A&A) processes in VA Handbook 6500 and VA Information Security Knowledge Service to obtain an Authority to Operate (ATO). VA Directive 6517, Risk Management Framework for Cloud Computing Services, provides security and privacy requirements for cloud environments.

c. VA IT contractors, subcontractors and third-party service providers shall address and/or integrate applicable VA Handbook 6500, VA Handbook 6517, Risk Management Framework for Cloud Computing Services and Information Security Knowledge Service specifications in delivered IT systems/solutions, products and/or services. If systems/solutions, products and/or services do not directly match VA security requirements, the contractor shall work though the COR/CO to identify the VA organization responsible for governance or resolution. Contractors shall comply with FAR 39.1, specifically the prohibitions referenced.

d. The contractor (including producers and resellers) shall comply with Office of Management and Budget (OMB) M-22-18 and M-23-16 when using third-party software on VA information systems or otherwise affecting the VA information. This includes new software purchases and software renewals for software developed or modified by major version change after the issuance date of M-22-18 (September 14, 2022). The term “software” includes firmware, operating systems, applications, and application services (e.g., cloud-based software), as well as products containing software. The contractor shall provide a self-attestation that secure software development practices are utilized as outlined by Executive Order (EO)14028 and NIST Guidance. A third-party assessment provided by either a certified Federal Risk and Authorization Management Program (FedRAMP) Third Party Assessor Organization (3PAO) or one approved by the agency will be acceptable in lieu of a software producer's self-attestation.

e. The contractor shall ensure all delivered applications, systems and information systems are compliant with Homeland Security Presidential Directive (HSPD) 12 and VA Identity and Access management (IAM) enterprise identity management requirements as set forth in OMB M-19-17, M-05-24, FIPS 201-3, Personal Identity Verification (PIV) of Federal Employees and Contractors (or its successor), M-21-31 and supporting NIST guidance. This applies to Commercial Off-The-Shelf (COTS) product(s) that the contractor did not develop, all software configurations and all customizations.

f. The contractor shall ensure all contractor delivered applications and systems provide user authentication services compliant with VA Handbook 6500, VA Information Security Knowledge Service, IAM enterprise requirements and NIST 800-63, Digital Identity Guidelines, for direct, assertion-based authentication and/or trust-based authentication, as determined by the design and integration patterns. Direct authentication at a minimum must include Public Key Infrastructure (PKI) based authentication supportive of PIV and/or Common Access Card (CAC), as determined by the business need and compliance with VA Information Security Knowledge Service specifications.

g. The contractor shall use VA authorized technical security baseline configurations and certify to the COR that applications are fully functional and operate correctly as intended on systems in compliance with VA baselines prior to acceptance or connection into an authorized VA computing environment. If the Defense Information Systems Agency (DISA) has created a Security Technical Implementation Guide (STIG) for the technology, the contractor may configure to comply with that STIG. If VA determines a new or updated VA configuration baseline needs to be created, the contractor shall provide required technical support to develop the configuration settings. FAR 39.1 requires the population of operating systems and applications includes all listed on the NIST National Checklist Program Checklist Repository.

h. The standard installation, operation, maintenance, updating and patching of software shall not alter the configuration settings from VA approved baseline configuration. Software developed for VA must be compatible with VA enterprise installer services and install to the default “program files” directory with silently install and uninstall. The contractor shall perform testing of all updates and patching prior to implementation on VA systems.

i. Applications designed for normal end users will run in the standard user context without elevated system administration privileges.

j. The contractor-delivered solutions shall reside on VA approved operating systems. Exceptions to this will only be granted with the written approval of the COR/CO.

k. The contractor shall design, develop, and implement security and privacy controls in accordance with the provisions of VA security system development life cycle outlined in NIST 800-37, Risk Management Framework for Information Systems and Organizations: A System Life Cycle Approach for Security and Privacy, VA Directive and Handbook 6500, and VA Handbook 6517.

l. The Contractor shall comply with the Privacy Act of1974 (the Act), FAR 52.224-2 Privacy Act, and VA rules and regulations issued under the Act in the design, development, or operation of any system of records on individuals to accomplish a VA function.

m. The contractor shall ensure the security of all procured or developed information systems, systems, major applications, minor applications, enclaves, and platform information technologies, including their subcomponents (hereinafter referred to as “Information Systems”) throughout the life of this contract and any extension, warranty, or maintenance periods. This includes security configurations, workarounds, patches, hotfixes, upgrades, replacements, and any physical components which may be necessary to remediate all security vulnerabilities published or known to the contractor anywhere in the information systems (including systems, operating systems, products, hardware, software, applications, and firmware). The contractor shall ensure security fixes do not negatively impact the Information Systems.

n. When the contractor is responsible for operations or maintenance of the systems, the contractor shall apply the security fixes within the timeframe specified by the associated controls on the VA Information Security Knowledge Service. When security fixes involve installing third party patches (such as Microsoft OS patches or Adobe Acrobat), the contractor shall provide written notice to the VA COR/CO that the patch has been validated as to not affecting the Systems within 10 business days.

B7. INFORMATION SYSTEM HOSTING, OPERATION, MAINTENANCE OR USE.

a. The contractor shall comply with all Federal laws, regulations, and VA policies for Information systems (cloud and non-cloud) that are hosted, operated, maintained, or used on behalf of VA at non-VA facilities. Security controls for collecting, processing, transmitting, and storing of VA sensitive information, must be in place. The controls will be tested by VA or a VA sanctioned 3PAO and approved by VA prior to hosting, operation, maintenance or use of the information system or systems by or on behalf of VA. This includes conducting compliance risk assessments, security architecture analysis, routine vulnerability scanning, system patching, change management procedures and the completion of an acceptable contingency plan for each system. The contractor’s security control procedures shall be the same as procedures used to secure VA-operated information systems.

b. Outsourcing (contractor facility, equipment, or staff) of systems or network operations, telecommunications services or other managed services require Assessment and Authorization (A&A) of the contractor’s systems in accordance with VA Handbook 6500 as specified in VA Information Security Knowledge Service. Major changes to the A&A package may require reviewing and updating all the documentation associated with the change. The contractor’s cloud computing systems shall comply with FedRAMP and VA Directive 6517 requirements.

c. The contractor shall return all electronic storage media (hard drives, optical disks, CDs, back-up tapes, etc.) on non-VA leased or non-VA owned IT equipment used to store, process or access VA information to VA in accordance with A&A package requirements. This applies when the contract is terminated or completed and prior to disposal of media. The contractor shall provide its plan for destruction of all VA data in its possession according to VA Information Security Knowledge Service requirements and NIST 800-88. The contractor shall send a self-certification that the data destruction requirements above have been met to the COR/CO within 30 business days of termination of the contract.

d. All external internet connections to VA network involving VA information must be in accordance with VA Trusted Internet Connection (TIC) Reference Architecture and VA Directive and Handbook 6513, Secure External Connections and reviewed and approved by VA prior to implementation. Government-owned contractor-operated systems, third party or business partner networks require a Memorandum of Understanding (MOU) and Interconnection Security Agreements (ISA).

e. Contractor procedures shall be subject to periodic, announced, or unannounced assessments by VA officials, the OIG or a 3PAO. The physical security aspects associated with contractor activities are also subject to such assessments. The contractor shall report, in writing, any deficiencies noted during the above assessment to the VA COR/CO. The contractor shall use VA’s defined processes to document planned remedial actions that address identified deficiencies in information security policies, procedures, and practices. The contractor shall correct security deficiencies within the timeframes specified in the VA Information Security Knowledge Service.

f. All major information system changes which occur in the production environment shall be reviewed by the VA to determine the impact on privacy and security of the system. Based on the review results, updates to the Authority to Operate (ATO) documentation and parameters may be required to remain in compliance with VA Handbook 6500 and VA Information Security Knowledge Service requirements.

g. The contractor shall conduct an annual privacy and security self-assessment on all information systems and outsourced services as required. Copies of the assessment shall be provided to the COR/CO. The VA/Government reserves the right to conduct assessment using government personnel or a third-party if deemed necessary. The contractor shall correct or mitigate any weaknesses discovered during the assessment.

h. VA prohibits the installation and use of personally owned or contractor-owned equipment or software on VA information systems. If non-VA owned equipment must be used to fulfill the requirements of a contract, it must be stated in the service agreement, SOW, PWS, PD or contract. All security controls required for government furnished equipment must be utilized in VA approved Other Equipment (OE). Configuration changes to the contractor OE, must be funded by the owner of the equipment. All remote systems must use a VA-approved antivirus software and a personal (host-based or enclave based) firewall with a VA-approved configuration. The contractor shall ensure software on OE is kept current with all critical updates and patches. Owners of approved OE are responsible for providing and maintaining the anti-virus software and the firewall on the non-VA owned OE. Approved contractor OE will be subject to technical inspection at any time.

i. The contractor shall notify the COR/CO within one hour of disclosure or successful exploits of any vulnerability which can compromise the confidentiality, integrity, or availability of the information systems. The system or effected component(s) need(s) to be isolated from the network. A forensic analysis needs to be conducted jointly with VA. Such issues will be remediated as quickly as practicable, but in no event longer than the timeframe specified by VA Information Security Knowledge Service. If sensitive personal information is compromised reference VA Handbook 6500.2 and Section 5, Security Incident Investigation.

j. For cases wherein the contractor discovers material defects or vulnerabilities impacting products and services they provide to VA, the contractor shall develop and implement policies and procedures for disclosure to VA, as well as remediation. The contractor shall, within 30 business days of discovery, document a summary of these vulnerabilities or defects. The documentation will include a description of the potential impact of each vulnerability and material defect, compensating security controls, mitigations, recommended corrective actions, root cause analysis and/or workarounds (i.e., monitoring). Should there exist any backdoors in the products or services they provide to VA (referring to methods for bypassing computer authentication), the contractor shall provide the VA CO/CO written assurance they have permanently remediated these backdoors.

k. All other vulnerabilities, including those discovered through routine scans or other assessments, will be remediated based on risk, in accordance with the remediation timelines specified by the VA Information Security Knowledge Service and/or the applicable timeframe mandated by Cybersecurity & Infrastructure Security Agency (CISA) Binding Operational Directive (BOD) 22-01 and BOD 19-02 for Internet-accessible systems Exceptions to this paragraph will only be granted with the approval of the COR/CO.

B8. SECURITY AND PRIVACY CONTROLS COMPLIANCE TESTING, ASSESSMENT AND AUDITING.

a. Should VA request it, the contractor shall provide a copy of their (corporation’s, sole proprietorship’s, partnership’s, limited liability company (LLC), or other business structure entity’s) policies, procedures, evidence, and independent report summaries related to specified cybersecurity frameworks (International Organization for Standardization (ISO), NIST Cybersecurity Framework (CSF), etc.). VA or its third-party/partner designee (if applicable) are further entitled to perform their own audits and security/penetration tests of the contractor’s IT or systems and controls, to ascertain whether the contractor is complying with the information security, network or system requirements mandated in the agreement between VA and the contractor.

b. Any audits or tests of the contractor or third-party designees/partner VA elects to carry out will commence within 30 business days of VA notification. Such audits, tests and assessments may include the following: (a): security/penetration tests which both sides agree will not unduly impact contractor operations; (b): interviews with pertinent stakeholders and practitioners; (c): document review; and (d): technical inspections of networks and systems the contractor uses to destroy, maintain, receive, retain, or use VA information.

c. As part of these audits, tests and assessments, the contractor shall provide all information requested by VA. This information includes, but is not limited to, the following: equipment lists, network or infrastructure diagrams, relevant policy documents, system logs or details on information systems accessing, transporting, or processing VA data.

d. The contractor and at its own expense, shall comply with any recommendations resulting from VA audits, inspections, and tests. VA further retains the right to view any related security reports the contractor has generated as part of its own security assessment. The contractor shall also notify VA of the existence of any such security reports or other related assessments, upon completion and validation.

e. VA appointed auditors or other government agency partners may be granted access to such documentation on a need-to-know basis and coordinated through the COR/CO. The contractor shall comply with recommendations which result from these regulatory assessments on the part of VA regulators and associated government agency partners. transfer, installation, dealing in or use of any information and communications technology or service, including ongoing activities, such as managed services, data transmission, software updates, repairs or the platforming or data hosting of applications for consumer download.

b. When contracting terms require the contractor to procure equipment, the contractor shall purchase or acquire the equipment from an Original Equipment Manufacturer (OEM) or an authorized reseller of the OEM. The contractor shall attest that equipment procured from an OEM or authorized reseller or distributor are authentic. If procurement is unavailable from an OEM or authorized reseller, the contractor shall submit in writing details of the circumstances prohibiting this from happening and procure a product waiver from the VA COR/CO.

c. All contractors shall establish, implement, and provide documentation for risk management practices for supply chain delivery of hardware, software (to include patches) and firmware provided under this agreement. Documentation will include chain of custody practices, inventory management program, information protection practices, integrity management program for sub-supplier provided components, and replacement parts requests. The contractor shall make spare parts available. All contractor(s) shall specify how digital delivery for procured products, including patches, will be validated, and monitored to ensure consistent delivery. The contractor shall apply encryption technology to protect procured products throughout the delivery process.

d. If a contractor provides software or patches to VA, the contractor shall publish or provide a hash conforming to the FIPS Security Requirements for Cryptographic Modules (FIPS 140-2 or successor).

e. The contractor shall provide a software bill of materials (SBOM) for procured (to include licensed products) and consist of a list of components and associated metadata which make up the product. SBOMs must be generated in one of the data formats defined in the National Telecommunications and Information Administration (NTIA) report “The Minimum Elements for a Software Bill of Materials (SBOM).”

f. Contractors shall use or arrange for the use of trusted channels to ship procured products, such as U.S. registered mail and/or tamper-evident packaging for physical deliveries.

g. Throughout the delivery process, the contractor shall demonstrate a capability for detecting unauthorized access (tampering).

h. The contractor shall demonstrate chain-of-custody documentation for procured products and require tamper-evident packaging for the delivery of this hardware.

B9 - PRODUCT INTEGRITY, AUTHENTICITY, PROVENANCE, ANTI-COUNTERFEIT AND ANTI-TAMPERING.

a. The contractor shall comply with Code of Federal Regulations (CFR) Title 15 Part 7, “Securing the Information and Communications Technology and Services (ICTS) Supply Chain”, which prohibits ICTS Transactions from foreign adversaries. ICTS Transactions are defined as any acquisition, importation, transfer, installation, dealing in or use of any information and communications technology or service, including ongoing activities, such as managed services, data transmission, software updates, repairs or the platforming or data hosting of applications for consumer download.

b. When contracting terms require the contractor to procure equipment, the contractor shall purchase or acquire the equipment from an Original Equipment Manufacturer (OEM) or an authorized reseller of the OEM. The contractor shall attest that equipment procured from an OEM or authorized reseller or distributor are authentic. If procurement is unavailable from an OEM or authorized reseller, the contractor shall submit in writing details of the circumstances prohibiting this from happening and procure a product waiver from the VA COR/CO.

c. All contractors shall establish, implement, and provide documentation for risk management practices for supply chain delivery of hardware, software (to include patches) and firmware provided under this agreement. Documentation will include chain of custody practices, inventory management program, information protection practices, integrity management program for sub-supplier provided components, and replacement parts requests. The contractor shall make spare parts available. All contractors shall specify how digital delivery for procured products, including patches, will be validated and monitored to ensure consistent delivery. The contractor shall apply encryption technology to protect procured products throughout the delivery process.

d. If a contractor provides software or patches to VA, the contractor shall publish or provide a hash conforming to the FIPS Security Requirements for Cryptographic Modules (FIPS 140-2 or successor).

e. The contractor shall provide a software bill of materials (SBOM) for procured (to include licensed products) and consist of a list of components and associated metadata which make up the product. SBOMs must be generated in one of the data formats defined in the National Telecommunications and Information Administration (NTIA) report “The Minimum Elements for a Software Bill of Materials (SBOM).”

f. Contractors shall use or arrange for the use of trusted channels to ship procured products, such as U.S. registered mail and/or tamper-evident packaging for physical deliveries.

g. Throughout the delivery process, the contractor shall demonstrate a capability for detecting unauthorized access (tampering).

h. The contractor shall demonstrate chain-of-custody documentation for procured products and require tamper-evident packaging for the delivery of this hardware.

B10. VIRUSES, FIRMWARE AND MALWARE

a. The contractor shall execute due diligence to ensure all provided software and patches, including third-party patches, are free of viruses and/or malware before releasing them to or installing them on VA information systems.

b. The contractor warrants it has no knowledge of and did not insert, any malicious virus and/or malware code into any software or patches provided to VA which could potentially harm or disrupt VA information systems. The contractor shall use due diligence, if supplying third-party software or patches, to ensure the third-party has not inserted any malicious code and/or virus which could damage or disrupt VA information systems.

c. The contractor shall provide or arrange for the provision of technical justification as to why any “false positive” hit has taken place to ensure their code’s supply chain has not been compromised. Justification may be required, but is not limited to, when install files, scripts, firmware, or other contractor-delivered software solutions (including third-party install files, scripts, firmware, or other software) are flagged as malicious, infected, or suspicious by an anti-virus vendor.

d. The contractor shall not upload (intentionally or negligently) any virus, worm, malware or any harmful or malicious content, component and/or corrupted data/source code (hereinafter “virus or other malware”) onto VA computer and information systems and/or networks. If introduced (and this clause is violated), upon written request from the VA CO, the contractor shall:

(1) Take all necessary action to correct the incident, to include any and all assistance to VA to eliminate the virus or other malware throughout VA’s information networks, computer systems and information systems; and

(2) Use commercially reasonable efforts to restore operational efficiency and remediate damages due to data loss or data integrity damage, if the virus or other malware causes a loss of operational efficiency, data loss, or damage to data integrity.

B11. CRYPTOGRAPHIC REQUIREMENT.

The contractor shall document how the cryptographic system supporting the contractor’s products and/or services protect the confidentiality, data integrity, authentication and non-repudiation of devices and data flows in the underlying system.

- The contractor shall use only approved cryptographic methods as defined in FIPS 140-2 (or its successor) and NIST 800-52 standards when enabling encryption on its products.

- c. The contractor shall provide or arrange for the provision of an automated remote key-establishment method which protects the confidentiality and integrity of the cryptographic keys.

- d. The contractor shall ensure emergency re-keying of all devices can be remotely performed within 30 business days.

- e. The contractor shall provide or arrange for the provision of a method for updating cryptographic primitives or algorithms.

B12. PATCHING GOVERNANCE.

a. The contractor shall provide documentation detailing the patch management, vulnerability management, mitigation, and update processes (to include third-party) prior to the connection of electronic devices, assets, or equipment to VA’s assets. This documentation will include information regarding the follow:

(1) The resources and technical capabilities to sustain the program or process (e.g., how the integrity of a patch is validated by VA); and

(2) The approach and capability to remediate newly reported zero-day vulnerabilities for contractor products.

b. The contractor shall verify and provide documentation all procured products (including third-party applications, hardware, software, operating systems, and firmware) have appropriate updates and patches installed prior to delivery to VA.

c. The contractor shall provide or arrange the provision of appropriate software and firmware updates to remediate newly discovered vulnerabilities or weaknesses for their products and services within 30 days of discovery. Updates to remediate critical or emergent vulnerabilities will be provided within seven business days of discovery. If updates cannot be made available by contractor within these time periods, the contractor shall submit mitigations, methods of exploit detection and/or workarounds to the COR/CO prior to the above deadlines.

d. The contractor shall provide or arrange for the provision of appropriate hardware, software and/or firmware updates, when those products, including open-source software, are provided to the VA, to remediate newly discovered vulnerabilities or weaknesses. Remediations of products or services provided to the VA’s system environment must be provided within 30 business days of availability from the original supplier and/or patching source. Updates to remediate critical vulnerabilities applicable to the Contractor’s use of the third-party product in its system environment will be provided within seven business days of availability from the original supplier and/or patching source. If applicable third-party updates cannot be integrated, tested, and made available by Contractor within these time periods, mitigations and/or workarounds will be provided to the COR/CO before the above deadlines.

B13. SPECIALIZED DEVICES/SYSTEMS (MEDICAL DEVICES, SPECIAL PURPOSE SYSTEMS, RESEARCH SCIENTIFIC COMPUTING).

a. Contractor supplies/delivered Medical Devices, Special Purpose Systems-Operational Technology (SPS-OT) and Research Scientific Computing Devices shall comply with all applicable Federal law, regulations, and VA policies. New developments require creation, testing, evaluation, and authorization in compliance with processes specified on the Specialized Device Cybersecurity Department Enterprise Risk Management (SDCD-ERM) Portal, VA Directive 6550, Pre-Procurement Assessment and Implementation of Medical Devices/Systems, VA Handbook 6500, and the VA Information Security Knowledge Service. Deviations from Federal law, regulations, and VA Policy are identified and documented as part of VA Directive 6550 and/or the VA Enterprise Risk Analysis (ERA) processes for Specialized Devices/Systems processes.

b. All contractors and third-party service providers shall address and/or integrate applicable VA Handbook 6500 and Information Security Knowledge Service specifications in delivered IT systems/solutions, products and/or services. If systems/solutions, products and/or services do not directly match VA security requirements, the contractor shall work though the COR/CO for governance or resolution.

c. The contractor shall certify to the COR/CO that devices/systems that have completed the VA Enterprise Risk Analysis (ERA) process for Specialized Devices/Systems are fully functional and operate correctly as intended. Devices/systems must follow the VA ERA authorized configuration prior to acquisition and connection to the VA computing environment. If VA determines a new VA ERA needs to be created, the contractor shall provide required technical support to develop the configuration settings. Major changes to a previously approved device/system will require a new ERA.

d. The contractor shall comply with all practices documented by the Food Drug and Administration (FDA) Premarket Submission for Management of Cybersecurity in Medical Devices and Postmarket Management of Cybersecurity in Medical Devices.

The contractor shall design devices capable of accepting all applicable security patches with or without the support of the contractor personnel. If patching can only be completed by the contractor, the contractor shall commit the resources needed to patch all applicable devices at all VA locations. If unique patching instructions or packaging is needed, the contractor shall provide the necessary information in conjunction with the validation/testing of the patch. The contractor shall apply security patches within 30 business days of the patch release and have a formal tracking process for any security patches not implemented to include explanation when a device cannot be patched.

f. The contractor shall provide devices able to install and maintain VA-approved antivirus capabilities with the capability to quarantine files and be updated as needed in response to incidents. Alternatively, a VA-approved whitelisting application may be used when the contractor cannot install an anti-virus / anti-malware application.

g. The contractor shall verify and document all software embedded within the device which does not contain any known viruses or malware before delivery to or installation at a VA location.

h. Devices and other equipment or systems containing media (hard drives, optical disks, solid state, and storage via chips/firmware) with VA sensitive information will be returned to the contractor with media removed. When the contract requires return of equipment, the options available to the contractor are the following:

(1) The contractor shall accept the system without the drive, firmware, and solid state.

(2) VA’s initial device purchase includes a spare drive or other replacement media which must be installed in place of the original drive at time of turn-in; or

(3) Due to the highly specialized and sometimes proprietary hardware and software associated with the device, if it is not possible for VA to retain the hard drive, firmware, and solid state, then:

(a) The equipment contractor shall have an existing BAA if the device being traded in has sensitive information stored on it and hard drive(s) from the system are being returned physically intact.

(b) Any fixed hard drive, Complementary Metal-Oxide-Semiconductor (CMOS), Programmable Read-Only Memory (PROM), solid state and firmware on the device must be non-destructively sanitized to the greatest extent possible without negatively impacting system operation. Selective clearing down to patient data folder level is recommended using VA approved and validated overwriting technologies/methods/tools. Applicable media sanitization specifications need to be pre-approved and described in the solicitation, contract, or order.

B14 DATA CENTER PROVISIONS: – This section is not applicable.

K. PRIVACY

NARA Records Management Language for Contracts (July 10, 2019)

https://www.archives.gov/records-mgmt/policy/records-mgmt-language

RECORDS MANAGEMENT OBLIGATIONS

A.  Applicability

This clause applies to all Contractors whose employees create, work with, or otherwise handle Federal records, as defined in Section B, regardless of the medium in which the record exists.

B.  Definitions

“Federal record” as defined in 44 U.S.C. § 3301, includes all recorded information, regardless of form or characteristics, made or received by a Federal agency under Federal law or in connection with the transaction of public business and preserved or appropriate for preservation by that agency or its legitimate successor as evidence of the organization, functions, policies, decisions, procedures, operations, or other activities of the United States Government or because of the informational value of data in them.

The term Federal record:

- Includes [Agency] records.

- Does not include personal materials.

- Applies to records created, received, or maintained by Contractors pursuant to their [Agency] contract.

- May include deliverables and documentation associated with deliverables.

C.  Requirements

- Contractor shall comply with all applicable records management laws and regulations, as well as National Archives and Records Administration (NARA) records policies, including but not limited to the Federal Records Act (44 U.S.C. chs. 21, 29, 31, 33), NARA regulations at 36 CFR Chapter XII Subchapter B, and those policies associated with the safeguarding of records covered by the Privacy Act of 1974 (5 U.S.C. 552a). These policies include the preservation of all records, regardless of form or characteristics, mode of transmission, or state of completion.

- In accordance with 36 CFR 1222.32, all data created for Government use and delivered to, or falling under the legal control of, the Government are Federal records subject to the provisions of 44 U.S.C. chapters 21, 29, 31, and 33, the Freedom of Information Act (FOIA) (5 U.S.C. 552), as amended, and the Privacy Act of 1974 (5 U.S.C. 552a), as amended and must be managed and scheduled for disposition only as permitted by statute or regulation.

- In accordance with 36 CFR 1222.32, Contractor shall maintain all records created for Government use or created in the course of performing the contract and/or delivered to, or under the legal control of the Government and must be managed in accordance with Federal law. Electronic records and associated metadata must be accompanied by sufficient technical documentation to permit understanding and use of the records and data.

- VA Fargo Health Care System and its Contractors are responsible for preventing the alienation or unauthorized destruction of records, including all forms of mutilation. Records may not be removed from the legal custody of VA Fargo Health Care System or destroyed except for in accordance with the provisions of the agency records schedules and with the written concurrence of the Head of the Contracting Activity. Willful and unlawful destruction, damage or alienation of Federal records is subject to the fines and penalties imposed by 18 U.S.C. 2701. In the event of any unlawful or accidental removal, defacing, alteration, or destruction of records, Contractor must report to VA Fargo Health Care System. The agency must report promptly to NARA in accordance with 36 CFR 1230.

- The Contractor shall immediately notify the appropriate Contracting Officer upon discovery of any inadvertent or unauthorized disclosures of information, data, documentary materials, records or equipment. Disclosure of non-public information is limited to authorized personnel with a need-to-know as described in this contract. The Contractor shall ensure that the appropriate personnel, administrative, technical, and physical safeguards are established to ensure the security and confidentiality of this information, data, documentary material, records and/or equipment is properly protected. The Contractor shall not remove material from Government facilities or systems, or facilities or systems operated or maintained on the Government’s behalf, without the express written permission of the Head of the Contracting Activity. When information, data, documentary material, records and/or equipment is no longer required, it shall be returned to VA Fargo Health Care System control or the Contractor must hold it until otherwise directed. Items returned to the Government shall be hand carried, mailed, emailed, or securely electronically transmitted to the Contracting Officer or address prescribed in this contract. Destruction of records is EXPRESSLY PROHIBITED unless in accordance with Paragraph (4).

- The Contractor is required to obtain the Contracting Officer's approval prior to engaging in any contractual relationship (sub-contractor) in support of this contract requiring the disclosure of information, documentary material and/or records generated under, or relating to, contracts. The Contractor (and any sub-contractor) is required to abide by Government and VA Fargo Health Care System guidance for protecting sensitive, proprietary information, classified, and controlled unclassified information.

- The Contractor shall only use Government IT equipment for purposes specifically tied to or authorized by the contract and in accordance with VA Fargo Health Care System policy.

- The Contractor shall not create or maintain any records containing any non-public VA Fargo Health Care System information that are not specifically tied to or authorized by the contract.

- The Contractor shall not retain, use, sell, or disseminate copies of any deliverable that contains information covered by the Privacy Act of 1974 or that which is generally protected from public disclosure by an exemption to the Freedom of Information Act.

- The VA Fargo Health Care System owns the rights to all data and records produced as part of this contract. All deliverables under the contract are the property of the U.S. Government for which [FACILITY] shall have unlimited rights to use, dispose of, or disclose such data contained therein as it determines to be in the public interest. Any Contractor rights in the data or deliverables must be identified as required by FAR 52.227-11 through FAR 52.227-20.

- Training.  All Contractor employees assigned to this contract who create, work with, or otherwise handle records are required to take VA Fargo Health Care System -provided records management training. The Contractor is responsible for confirming training has been completed according to agency policies, including initial training and any annual or refresher training.

D.  Flowdown of Requirements to Subcontractors

- The Contractor shall incorporate the substance of this clause, its terms and requirements including this paragraph, in all subcontracts under this [contract vehicle], and require written subcontractor acknowledgment of same.

- Violation by a subcontractor of any provision set forth in this clause will be attributed to the Contractor.

End of Statement of Work