Schriever Space Force Base, Colorado

Wing Information & Communications Support

# (WICS-II)

Performance Work Statement FA255021C0005

25 April 2024

Note: Requirements not planned to be included in the consolidated effort have been removed

Table of Contents

  - Background	5

  - Scope	5

  - Overview of Principle Systems	5

  - Program Management	8

  - Government Management Team (GMT)	8

  - Coverage Hours	8

  - Mission-Essential Contractor Services	8

  - Exercise/Inspection Support	8

  - Contractor Manpower Reporting	9

  - Personnel Management	9

  - Contractor Reimbursable Travel	10

  - Local Purchase	10

  - Quality Management	11

  - Training	11

  - Program Management Support	12

  - Phase-In/Phase-Out	14

  - Operations	16

  - Global Broadcast Service Operations Center (GBSOC)	16

  - Broadcast Management	16

  - End User Support	17

  - GBSOC Service Desk	17

  - System Administration	19

  - Configuration Management (CM)	19

  - Transfer of Operations (TOOP)	20

  - GBSOC Contingency Operations	20

  - Mission Systems Maintenance	21

  - Maintenance	21

  - Standard Maintenance Practices	22

  - Corrective Maintenance	22

  - Preventive Maintenance Inspection (PMI) Performance	22

  - Corrosion Control	22

  - Test, Measurement, and Diagnostic Equipment (TMDE)	22

  - TO Accounts and Technical Publication Sets	22

  - Job Data Documentation	23

  - Deficiency Reports	23

  - Cannibalization	23

  - Depot Assistance	23

  - Electro-Magnetic Interference (EMI)	23

  - Crypto Equipment	23

  - Self-Sufficiency Program	24

  - MOC Mission Systems Reporting	24

  - Global Broadcast Service (GBS)	25

  - Wideband Global Satcom (WGS)	25

    - Equipment Housekeeping	26

    - IAT Level 2 Certification	26

    - Monthly Outage Listing	26

    - Configuration Documentation	26

  - Certification of Special Equipment	27

4.20 Information Technology Equipment Custodian (ITEC)	27

  - Systems and Software Support	27

  - Technical Support	29

  - Mission Impacting Reporting (MIR) Support	30

  - Supply and Logistics Support	33

  - Supply Support	33

  - Logistics Support	35

  - [Removed – will not be included in consolidated requirement]	36

  - Team SAFB Cybersecurity Office (TSCO)and Information/Communication Systems Support	37

  - Team SAFB Cybersecurity Office (TSCO)	37

  - TEMPEST	38

  - Risk Management Framework (RMF) Authorization Service	38

  - Cyber Readiness (CR) Support:	38

  - Travel	39

  - Special Security Representative/ Sensitive Compartmented Information Facility (SCIF)/ MOD Facilitator – Systems ISSO	40

  - [Removed – will not be included in consolidated requirement]	41

- Reserved	42

- Space Digital Integrated Network (SDIN) and Global Command and Control System (GCCS)	43

  - General	43

  - SDIN and GCCS COOP	44

  - Work Status Reports	44

  - Reserved	44

  - Reserved	44

  - Support Services/Maintenance Contracts	44

  - Integration	44

  - Asset Management	45

  - SDIN and GCCS Network Services	45

  - GCCS	46

  - SDIN	48

  - Information Assurance (IA) Operations	53

- Operational Systems Management	54

  - External Agency Interaction	55

  - Installation Package for Coordination/Planning	56

  - Deficiency Resolution Monitoring and Change Tracking	56

Report and track deficiencies/changes	56

  - Meeting Support	56

- Technical Control Facility (TCF) Operations	57

  - General Requirements	57

  - Technical Control Functional Requirements	58

  - Space Control Network Functions	59

  - Technical Control Work Center Functions	59

  - COMSEC Requirements	60

- Other Support Requirements	61

  - Security Support	61

  - Reporting of Lost Badges	61

  - Retrieving Identification Media	61

  - For Controlled Unclassified Information (CUI) Data Handling	61

  - Non-Disclosure Agreements (NDAs)	61

  - Industrial Security	61

  - Safes	62

  - Information Security Program	62

  - Personnel Clearance Requirements	62

  - Operations Security	62

  - Physical Security	62

  - Resource Protection	63

  - General Safety	63

13.14. Electro-Magnetic Frequency (EMF) Radiation	64

  - Facility Infrastructure Support	64

  - General Housekeeping	65

- Government-Furnished Equipment (GFE), Property (GFP) and Information (GFI) . 66

  - GFE and GFP	66

  - GFI	66

- WICS-II Service Summary Items	67

  - Performance Objectives/Thresholds	67

Appendix A

WICS-II CDRL Requirements	71

Appendix B

Acronyms	72

Appendix C

Definitions	78

Appendix D

Publications and Forms	92

Appendix E

Mission Essential Contractor Services	93

# Chapter 1 Background and Contract Scope

# Background

The mission of Space Base Delta 1 (SBD 1), is to enable and empower combat-ready space and homeland defense operations through mission critical global support to protect and defend our Nation. SBD 1 currently operates and supports satellite operation centers at Schriever Space Force Base (SSFB), Colorado, remote tracking stations and other command and control facilities around the world. SBD 1 personnel monitor satellites during launch, put satellites in their proper orbits following launch, and operate the satellites while they are in orbit. They ensure effective and efficient satellite operations and proper disposal of the satellites at the end of life.

SBD 1 operates and supports satellite programs including the Global Positioning System (GPS), Defense Satellite Communication Systems (DSCS), Wideband Global SATCOM (WGS), Military Strategic and Tactical Relay (MILSTAR), Defense Meteorological Satellite Program, Space Based Space Surveillance (SBSS), Operationally Responsive Space, Advanced Extremely High Frequency (AEHF), Mission Impact Reporting (MIR), Space Digital Integrated Network (SDIN) and Global Command and Control System (GCCS) and the worldwide Satellite Control Network (SCN) supporting 185 satellites.

# Scope

The scope of this contract encompasses all services required to support the operation, maintenance, and logistics of the SSFB Installation information and communications systems as these exist at the time of contract award and as they evolve over the course of contract performance. This effort includes the ability to provide support to various SSFB Installation information and communications systems and associated tenant organizations. During the performance of this contract, the contractor shall provide non-personal services in support of operations, mission systems maintenance and maintenance management, logistics management, communication support, security and emergency notification systems, testing and all other requirements within the WICS II Performance Work Statement (PWS). Support also includes potential future requirements, emerging technologies, system growth, and new/upgraded equipment as needed.

# Overview of Principle Systems

    - Global Broadcast Service (GBS): GBS is a satellite communication based, worldwide, high-capacity broadcast system that provides a one-way, high-speed flow of high-volume national security information to forces garrisoned, deployed, or on the move with tactical GBS satellite receive suites (RS). GBS provides a content priority-based delivery of classified and unclassified video and data products. GBS is a Defense Information Systems Agency (DISA) Core Data Center (CDC)-centric architecture providing centralized Satellite Broadcast Manager (SBM) system for continuous operations, management, and control of DISA dedicated assets assigned to GBS and GBS Mission Unique Equipment (MUE). The GBS Operations Center (GBSOC) is the centralized service center that provides broadcast and event management, troubleshooting, coordination, reporting, and overall status and situational awareness of the GBS system and services. Additionally, the GBSOC provides GBS RS end-user service desk support. Under the WICS-II contract, GBS resources include the following:

      - GBSOC at SSFB

      - GBS Continuity Operations (COOP) at Peterson Space Force Base (PSFB), Colorado - GBS COOP at Hill Air Force Base (HAFB), Utah

      - GBS SBM at Tinker Air Force Base, Oklahoma (OKC): The DISA CDC, formally known as DECC, houses the GBS “Primary” SBM equipment and provides GBS Department of Defense Information Network (DoDIN) network connectivity.

      - GBS SBM at Mechanicsburg, Pennsylvania (MECH): The DISA CDC houses the GBS “Backup” COOP SBM equipment and provides DoDIN network connectivity.

    - Wideband Global SATCOM (WGS): WGS is used for high priority command and control communication such as the exchange of wartime information between defense officials and battlefield commanders. The military also uses WGS to transmit space operations and early warning data to various systems and users. Under the WICS-II contract, WGS resources at SSFB include the following:

      - Dual AN/GSC-52(V)5 WGS Earth Terminals

      - Four AN/USC-28 Spread Spectrum Multiple Access (SSMA) Anti-jam (A/J) modular- demodulator (MODEMs)

      - AN/MSC-66 Digital Communications Satellite Subsystem (DCSS)

      - OM-88 MODEMs

      - Interconnect Facility

    - Family of Advanced Beyond Line-of-Sight Terminal (FAB-T): FAB-T (formerly referred to as MILSTAR) is used for high priority, secure and jam resistant worldwide communications to meet the requirements of the Armed Forces of the United States. Under the WICS-II contract, FAB-T resources at SSFB include, but are not limited to, the following:

      - AN/FRC-191(V)1 FAB-T Command Post Terminal

      - 10FT Antenna System Tower

      - Data Set Manager

    - Global Command and Control System (GCCS): The GCCS mission system is comprised of Uniplexed Information and Computing System (UNIX) and Internet Protocol (IP) based operating systems. GCCS supports the following sites:

      - SSFB, Colorado

      - PSFB, Colorado

      - Buckley Space Force Base (BSFB) Colorado

      - United States Air Force Academy (USAFA), Colorado

      - F.E. Warren AFB, Wyoming

      - Malmstrom AFB (MAFB), Montana

      - Los Angeles AFB (LAAFB), California

      - Vandenberg SFB (VSFB), California

    - Space Digital Integrated Network (SDIN): The SDIN mission system consists of a

combination of Circuit-to-Packet (CTP) technology, Routed Networks and Synchronous Optical Network (SONET), High-Altitude Electromagnetic Pulse Protected networks. SDIN supports the following sites:

      - SSFB

      - PSFB

      - BSFB

      - VSFB

      - Eglin AFB, Florida

      - Cheyenne Mountain Space Force Station (CMSFS), Colorado

      - Cape Cod Space Force Station (SFS), Massachusetts

      - Cavalier AFS, North Dakota

      - Clear AFS, Alaska

      - Dahlgren Naval Air Station (NAS), Virginia

      - Diego Garcia Air Station (AS)

      - Site R, Socorro AS, New Mexico

      - Kwajalein AS, Republic of the Marshall Islands

      - Maui AS, Hawaii

      - Royal Air Force Base, Fylingdales, United Kingdom (UK)

      - Millstone Lincoln Labs, Offutt Air Force Base (OAFB), Nebraska

      - Thule Air Base (AB), Greenland

    - Mission Impact Reporting (MIR): MIRs are the only and singularly important feedback mechanism to determine how the SCN is performing day-to-day, satellite contact by satellite contact and is the primary tool used to debrief SCN operations. MIRs feed multiple SCN performance metrics as mandated in the Headquarters (HQ) United States Air Force (USAF) “2002 Satellite Control System Operations Requirements Document” when calculating contact success rate and enables the SBD 1, USAF, and USSF SpOC at tactical and strategic levels in executing resource management decisions for space access.

# Chapter 2 Program Management

# Program Management

Provide all personnel, tools, materials, transportation, supervision, and other items necessary to perform WICS-II PWS requirements. The Contractor’s furnished materials and equipment shall include vehicles, vehicle fuel, tools, custodial supplies, office administrative supplies and equipment, and any other supplies and materials not furnished by the Government, which the Contractor believes necessary to perform these requirements.

# Government Management Team (GMT)

Notify the GMT of any problems affecting the contractor’s performance impacting mission accomplishment or any issues the contractor considers not within the scope of this contract. The GMT consists of the Contracting Officer (CO), the Government Program Manager (GPM), the Contracting Specialist and the Contracting Officer’s Representative (COR).

    - Provide a Program Management Plan, (Contract Data Requirements List (CDRL) A101 – Program Management Plan). Provide an overview of the general approach and methodology to implement and oversee the performance of the WICS-II PWS.

# CDRL A101: Program Management Plan

# Coverage Hours

Contractor coverage must mirror the respective Government work center hours, to include Federal holidays, unless otherwise identified within the PWS.

# Mission-Essential Contractor Services

Ensure continued support of all locations identified in this PWS during crises, as required by DoDI 1100.22, policy and procedures for determining workforce mix, and, unless otherwise directed by the CO. Provide CDRL A102 - Mission-Essential Contractor Services Plan IAW Defense Federal Acquisition Regulation Supplement (DFARS) 252.237-7023, Continuation of Essential Contractor Services, for providing assurance of continuation of mission essential services during crises conditions. See Appendix E, which identifies mission essential areas on this contract.

# CDRL A102: Mission-Essential Contractor Services Plan

# Exercise/Inspection Support

    - Fully participate in all SBD 1 exercises/inspections to the level dictated by the scenarios in progress. This support will be required on a non-mission interference basis during normal contractor duty hours.

    - Not be required to increase manpower or man-hours during exercise participation unless specifically stated in this PWS or authorized by the CO.

    - The contractor may experience delays entering and/or exiting the base due to exercises, increased security, base closure or late reporting due to weather conditions. Anticipate exercises on a quarterly basis. In all cases, ensure mission essential work center coverage while minimizing potential harm to contractor employees.

# Contractor Manpower Reporting

    - Report ALL contractor labor hours at all sites (including subcontractor labor hours) required for performance of services provided under this contract. Reporting shall be accomplished for the Space Acquisition Management-Directive (SAM-D Schriever) formerly known as 50th Contracting Squadron (50 CONS), DoD activity address code (DoDAAC) number FA2550, via a secure data collection site. The contractor is required to fill in all required data fields at http://www.SAM.gov.

    - Reporting inputs will be for the labor executed during the period of performance (PoP) for each government fiscal year (FY), which runs 1 October through 30 September. While inputs may be reported any time during the FY, all data shall be reported no later than 31 October of each calendar year.

# Personnel Management

    - Contractor Program Manager

      - Identify in writing, email is sufficient, to the CO and GPM, any individual designated to act as the Contractor Program Manager (CPM).

      - The CPM or designee shall have full authority to act for the contractor on all matters relating to daily operations of the WICS-II contract.

      - Advise the GPM immediately or within 15 minutes, of any problems that affect the contractor’s performance.

      - The CPM or designee shall be available at all times for discussions via teleconference.

# Contracting Manager

      - Identify in writing, an email is sufficient, to the CO and GPM, any individual designated to act as the Contracting Manager.

      - The Contracting Manager or designee shall have the authority to act on behalf of the company in all matters relating to contract performance and to review and sign all contract actions.

      - The Contracting Manager shall be available Monday through Friday, between the hours of 0800-1500 Mountain Standard Time (MST), for discussions via teleconference.

# Contractor Employees

      - All contract employees who interface with the government, either electronically or in person shall be able to read, write, speak, and understand English.

      - Select personnel who are fully certified and/or qualified/trained and have the necessary security clearances to provide the government the services required by the WICS-II PWS.

      - Identify/introduce themselves as contractor personnel and display distinguishing badges or other visible identification for meetings with government personnel, appropriately identify themselves as contractor employees in telephone conversations and in formal and informal written correspondence.

# Contractor Conduct

      - The contractor employees shall be subject to regulations and directives governing the activities of personnel on government installations.

      - The installation commander may direct the contractor, in writing, to remove any employee from the installation when retention of such employee endangers life, property, security, or where such an employee violates reasonable and necessary base regulations or displays improper behavior.

    - When a mishap occurs or an employee displays improper behavior, the Government reserves the right to immediately remove an individual from the installation.

# Contractor Reimbursable Travel

    - Travel may be required to perform the services required by this PWS. This may include Government directed travel and COOP. All travel requirements shall be pre-approved by the GPM or CO and is on a strictly cost reimbursable basis. Costs associated with contractor travel and billing shall be in accordance with Federal Acquisition Regulation (FAR) 31.205-46, Travel Costs. The contractor shall be responsible for all employee travel arrangements by commercial means. Contractor employee travel shall be IAW current Joint Travel Regulations. Transportation may be provided by the military depending on access to bases and availability of commercial transportation. The contractor shall travel using the lowest cost mode transportation commensurate with the mission requirements. Travel will be reimbursed on a cost reimbursable basis; no profit or fee will be paid. Travel within the El Paso County area will not be reimbursed.

    - Submit a Travel Authorization Request (TAR) to the GPM or CO and receive approval prior to booking travel arrangements. The TAR shall include trip number, traveler name, travel dates, trip destination(s), description of trip, and estimated trip costs, e.g. airfare, lodging, per diem, rental car, etc. The TAR amount will be the approved trip reimbursable cost. The TAR requires recommendation from the COR and CPM or designee and approval by the GPM or CO. The TAR may be in contractor format.

    - Submit a cost report within 30 days of conclusion of trip. The trip report will contain trip number, traveler name, trip location, actual travel dates, and detail actual costs compared to estimated costs. Costs over the approved TAR amount must be justified and receive approval prior to submission of cost voucher. The cost report may be in contractor format.

# Local Purchase

Purchases may be needed in the performance of the PWS requirements. All purchase actions require a recommendation from the COR. Purchases will be coordinated and approved through either the GPM or the CO. Purchases shall be handled IAW contractor policy and procedures and formally documented IAW DFARS 252.244-7001. Purchases will be reimbursed on a cost reimbursable basis; no profit or fee will be paid.

# Special Purpose Vehicles and Equipment

When the need arises to utilize special purpose vehicles, one-time use vehicles or equipment, notify the government. When directed by the Government, directly lease the required special purpose vehicles, subject to the cost limitation directed by the Government, with the cost of the lease allocable to the appropriate FY. All lease actions require recommendation from the COR, coordination and approval through either the GPM or the CO.

# SDIN Technical Refresh Local Purchase

Purchase, install, configure, integrate and test technical refresh equipment as part of a future requirement for SDIN technical refresh. Using the contractor’s purchasing system IAW DFARS 252.244-7001, any equipment or supplies purchase will be coordinated through the SDIN Program Manager, and the GPM, and approved by the CO prior to commitment of funds. To obtain approval for purchasing equipment, the contractor must provide adequate supporting documentation to the CO. The contractor will be provided authority to use Government sources of supply, such as General Services Administration (GSA) schedules, to obtain the best value for the Government. Documentation must be provided with cost vouchers when invoicing for reimbursement.

# Quality Management

Have an independent quality program, encompassing all aspects of the contract; ensuring service delivery meets or exceeds performance requirements in the PWS. Implement the quality program through a Quality Control Plan (QCP) (CDRL A103). The QCP shall be submitted and accepted by the GPM, Quality Assurance Manager and CO.

CDRL A103: Quality Control Plan

# Training

    - Contractor Specialized Training

      - Conduct a qualification or certification training program for contractor personnel. Create, update and conduct a position certification/qualification training program to ensure contractor personnel are certified/qualified to perform operations and maintenance requirements identified as requiring certification/qualification.

      - Ensure personnel are properly trained and have appropriate DoD approved cybersecurity baseline certification(s) before performing any network operations tasks. Provide the GMT copies of the appropriate certification(s) prior to being given privileged access to the network and/or assuming cybersecurity responsibilities and ensure they remain in good standing. All contractor personnel shall complete and sign a formal statement of assigned cybersecurity responsibilities. Immediately remove personnel not in good standing from cybersecurity tasks. Maintain the currency of employees by providing initial and refresher training as required to meet the PWS requirements.

        - Be certified IAW Methods and Procedures Technical Order (MPTO) 00-33A-1001, Department of Defense Directive (DoDD) 8140.01, Department of Defense (DoD) 8570.01-M, Air Force Manual (AFM) 17-1303 and DoD Approved 8570 Baseline Certifications.

        - Contractor personnel working on SDIN and GCCS networks shall be certified at the IAT Level II.

        - Have personnel certified at the Information Assurance Management (IAM) Level II to maintain the Risk Management Framework (RMF) package.

        - The certification/qualification program is to be contractor self-contained, requiring no technical or materiel support from the government and is subject to government review.

# Ancillary Training

      - Participate in ancillary training as required by the work being performed and IAW applicable documentation (instructions, standards, etc.).

      - At a minimum, the following courses via the Air Force myLearning application, Computer Based Training (CBT), or classroom attendance shall be completed within thirty (30) days after contract award (Table 2-1 – Ancillary Training). Other training may be required at the request of the COR. The training records shall contain, at a minimum, the completed/signed formal statement of assigned cybersecurity responsibilities and copies of the appropriate certifications.

# Table 2-1 – Ancillary Training

| Ancillary Training Course | Frequency | Method |
| --- | --- | --- |
| Office of Special Investigation /Counter Intel Brief | Annual | Brief |
| Cyber Awareness Challenge | Annual | myLearning |
| Force Protection | Annual | myLearning |
| Restricted Area Escort Training | Annual | CBT |
| Secure Voice Training | Semi-Annual | Briefing |

    - Training Records

      - Maintain training records for personnel. All training records shall be made available for COR review upon request. Establish and maintain a training document or database, or use the existing maintenance information systems database, to track contractor training requirements. The document or database will, at a minimum, reflect training due dates, training completion dates and projected due dates.

# Training and Education Costs

The Government will provide training and materials as new or modified systems come online. Specialized government provided training (e.g., not commercially available) may be required as directed.

# Program Management Support

    - Self Inspection/Management Program

Utilize the current government self-inspection/management program, i.e., Management Internal Control Toolset (MICT) or its follow-on system. The contractor shall establish MICT accounts and maintain up-to-date compliance checklists for use during assessments. Applicable check lists/sheets shall be utilized, and pertinent data shall be entered into the MICT system.

# Management Information System

Provide government access to a single management information system, whether government- owned or contractor-owned, for management, organization, and consolidation of all CDRLs, program documents, metrics, data, and knowledge. It shall be searchable and allow generated user interfaces and include workflow tools to assist in continuous improvement of WICS-II artifacts.

# Meeting Support

Support the government during reviews and meetings, to include providing impact assessments for operations, maintenance, and logistics to SBD 1 personnel. Meetings include, but are not

limited to, teleconferences, Utility Outage Board, Modification Control Boards (MCB), Operations Review Boards (ORB), Cybersecurity, Cyber Readiness (CR) and Operations Review Panels (ORP) as defined in unit operating instructions (OIs) and/or policies.

Participate in and/or, support conferences, technical and operational meetings, working groups and reviews with coordination with the COR. Some meetings are scheduled weekly or monthly, others are ad hoc meetings to discuss immediate emergencies, which may be on or off SSFB. Provide the appropriate personnel as required by the agenda to clarify technical details, determine solutions or recommendations, or for other technical expertise. Examples of these meetings are: Command and Control (C2) Working Group for GCCS, and MCB meetings to review, discuss and receive approval for new requirements.

# Program Management Review (PMR)

Provide PMRs to the government IAW CDRL A105 – Program Management Review. The PMR will be held in the Colorado Springs, CO area (location will be determined by the GMT). For all PMRs, the contractor shall provide the briefing content to the CO and GPM at least three duty days prior to the PMR. The contractor shall present the first PMR within 30 days of contract start. PMR’s shall be presented monthly for the first 3 months, and quarterly thereafter.

# CDRL A105: Program Management Review

      - Multi-functional Team (MFT)

A bi-weekly MFT meeting will be held to review current contract and operational events. Inputs shall be provided to the GPM for the MFT. Contractor attendance at these meetings is required (location to be determined by the GMT).

# Administrative Support

      - Documentation

        - Responsible for creating, maintaining and delivering Government-required documents, records, and reports used for accomplishment of WICS-II requirements.

        - Provide the original record or a reproducible copy of record within five working days of receipt of request

        - All documentation, products, and data developed in support of the WICS-II contract are the property of the US Government.

# Publications Support

      - Contract/Work Center Operating Instructions (OIs)/Standard Operating Procedures (SOPs)

Upon contract award, OIs/SOPs used by the incumbent contractor will be provided to the successor contractor. The successor contractor shall use, change, or develop their own OIs/ SOPs (CDRL A106: Contract/Work Center OIs/SOPs). The OIs/SOPs shall be implemented, maintained, and complied with. All OIs/SOPs required for compliance will be accepted by and accessible to the GMT. Contract OIs will address contract wide direction/policy. Work center OIs/SOPs will address work center specific direction/policy/ procedures. All OIs/SOPs, once accepted by the GMT, will become directive in nature. Annually review all OIs/SOPs to ensure they are accurate and update when necessary. All OIs/SOPs become the property of the

government, will be maintained by the contractor and will be incorporated into the WICS-II Master Document List (MDL)/Bidders’ Library (BL).

# CDRL A106: Contract/Work Center OIs/SOPs

      - New Directive Publications

Review new/updated publications (draft and final) to assess technical content and impact to the WICS-II contract. Notify the CO within 30 days of receipt if there are potential risks to the WICS-II contract.

# Master Document List (MDL)/Bidder’s Library (BL)

Provide a complete MDL/BL IAW CDRL A108: MDL/BL. The MDL/BL encompasses all publications/documents required to perform the requirements of the PWS. Maintain the MDL for the duration of the contract performance. The MDL/BL (CDRL A108) shall be delivered, within 30 days of CO request.

# CDRL A108: Master Document List (MDL)/Bidder’s Library (BL)

    - Records Management Program

Manage all data created for government use or legally controlled by the government, in support of the functional activity or required by DoDD and AF publications, IAW the DoDD and AF records management procedures in DoDD 5015.2, DoD 5015.2-STD, Air Force Policy Directive (AFPD) AFPD 33-3, Air Force Instruction (AFI) AFI 33-322, and the Air Force Records Disposition Schedule located at https://www.my.af.mil/afrims/afrims/afrims/rims.cfm or their subsequent replacements. All contractor records management personnel shall possess a valid National Agency Check with Local Agency and Credit Check (NACLC).

# Records Custodian (RC)

At SSFB GBSOC, Technical Control Facility (TCF) and Military Satellite Communications (MILSATCOM), perform RC duties in order to comply with the DoD, AF governing directives, to include the Air Force Records Information Management System (AFRIMS).

# Phase-In/Phase-Out

    - Phase-In Plan

Upon award, the government will provide the successful contractor a phase-out plan from the incumbent contractor. The successful contractor shall deliver a phase-in plan (CDRL A110 - Phase-In Plan) which complements the incumbent’s phase-out plan. The GMT will meet with the contractor to review implementation of the phase-in plan. The plan shall identify that a complete certified, qualified, trained, and cleared workforce will be available no later than contract start, ensuring no degradation in service.

# CDRL A110: Phase-In Plan

    - Phase-Out Plan

Develop and implement a transition plan for phase-out (CDRL A111 - Phase-Out Plan). Establish procedures with the successors to ensure a smooth transition of all requirements. Maintain overall contract responsibility until the completion of the contract PoP. Ensure all responsibilities are transferred ensuring no degradation in service.

CDRL A111: Phase-Out Plan

Chapter 3

Global Broadcast Service (GBS) Operations

# Operations

Perform 24/7 on-site operations activities, unless identified in the following paragraphs. Plan, schedule, direct, organize, coordinate, and execute all actions necessary to conduct continuous operations and sustain the operability of the GBSOC at SSFB. Conduct remote administration, from the GBSOC, of the GBS MUE at OKC and MECH. Maintain the equipment IAW applicable TOs, manuals, and OIs/SOPs.

# Global Broadcast Service Operations Center (GBSOC)

Maintain a GBSOC controllable SBM operational availability (Ao) threshold of 99.00%.

# Broadcast Management

    - Monitor and maintain real-time situational awareness of the GBS broadcast.

    - Monitor the status of uplink sites for outages that affect GBS broadcasts.

    - Maintain situational awareness of satellite beam pointing.

    - Monitor GBS broadcast bandwidth allocation and assist with resolution of bandwidth allocation issues.

    - Assist the Regional SATCOM Support Centers (RSSC), Wideband SATCOM Operations Centers (WSOC), and Combatant Command (CCMD) Theater Information Managers (TIM) with satellite coverage coordination, broadcast issues, and resolving resource conflicts.

    - Monitor the broadcast rate of product delivery from the SBM to Receive Broadcast Manager (RBM), and status of products queued, transmitted and sent.

    - Manage the GBS broadcast content to ensure the specified products are available via the appropriate uplink facility according to schedule. This includes assigning video streams to satellite transponders and determining appropriate data rates.

    - Support TIMs in maintaining the accuracy of the Electronic Program Guide, Tuning Guide, and Product Catalog.

    - Perform web crawl reception troubleshooting to resolve anomalies; perform verification checks on each web crawl every 180 days at a minimum to ensure accurate data.

    - Process requests for broadcast of the GBS banner and troubleshoot anomalies.

    - Prepare and post Mission Data Sheets to support end-user RS configuration.

    - Coordinate with TIMs to disable and re-enable GBS RSs.

    - Execute GBS broadcast remote deny actions.

    - Set and monitor government prioritized flow control of dynamically allocated bandwidth.

    - Manage over-the-air broadcasting of Information Assurance Vulnerability Alert releases, and software and system updates to fielded RBMs.

# End User Support

    - Provide consultation and problem resolution to fielded GBS RS end users from the GBSOC. Respond to GBS end-user service requests for configuring RSs, troubleshooting broadcast reception issues, and understanding the GBS broadcast and broadcast products.

    - Respond within 15 minutes to service request calls from GBS fielded RS end users. For GBS non-DoD fielded RS users, support is limited to the status of the satellite, beam coverage, and requesting GBS traffic content routed to the SATCOM downlink broadcast string.

    - Issue CypherCast Windows Client (CWC) and Kencast licenses for fielded GBS RS.

# GBSOC Service Desk

    - Operate the GBSOC service desk, as the operational focal point for GBS daily broadcast services.

    - Provide event and incident management troubleshooting, notification and reporting of service interruptions, impairments, or anomalies for systems, circuits, equipment, approved changes, and scheduled maintenance within the GBSOC area of responsibility.

    - Log, prioritize, investigate, resolve and report problems with GBS services. Respond to and initiate corrective action, within 5 minutes or notification, for incidents, problems, and outages affecting GBS services.

    - Use and maintain data in a government service management system to document events, incidents, fault isolation actions, status tracking, status reporting, and related GBSOC activities.

    - Monitor the performance of SBM functions for system and software problems, and operating condition of GBS components.

    - Fulfill requests from the GBS user community using consistent and repeatable methods for which pre-defined approval exists, such as establishing SBM user accounts and issuing Smart Cards.

    - Manage the GBS RS Smart Card program to include procurement, storage and distribution, and provide end-user authorization verification, entitlement, and removal.

    - Report GBS health and status information within the GBSOC area of responsibility through the proper chain of command as determined by GBSOC C2 relationships and reporting procedures established IAW GBSOC OIs and associated reporting guidance. This includes but is not limited to the GBS Event Message (GEM), GBS Advisory Message (GAM), Operations Capability (OPSCAP) reporting, and Commander’s Critical Information Requirements (CCIR) reporting.

    - Maintain daily operations logs that shall include support problems, mission-impacting occurrences, and GBSOC significant events, at a minimum.

    - Support information assurance (IA) activities and Information Assurance Vulnerability Management (IAVM) processes to include alerts for hot fixes and system updates.

    - Assist in resolution of source product provider issues, i.e. news and sports networks.

    - Report GBS equipment deficiencies to the COR utilizing established reporting procedures and guidance IAW GBSOC OIs.

    - The GBSOC provides Tier-1 analytical service desk support up to their expertise level and if the anomalies cannot be resolved, then the GBSOC engineers provide Tier-2 support up to their expertise level. Once the GBSOC engineer’s expertise level has been exceeded, the anomalies are elevated to Tier-3 located at Ogden/OKC. Tier-2 work closely with Tier-3 until the anomalies have been resolved and all GBS services are restored.

    - Manage source integration of new File Transfer Services (FTS) sources into the SBM by adding, editing, deleting and reviewing individual accounts and profiles.

    - Establish and maintain a Service Desk knowledge management repository for knowledge identification and control.

    - Post, review, and update information in a configuration-controlled manner on the SBM Non-Classified Internet Protocol Router Network (NIPRNet) and Secret Internet Protocol Router Network (SIPRNet) web portals requiring wide dissemination to the GBS operational community. Managed content shall include but is not limited to announcements, downloadable documents and other resources, forms, frequently asked questions, useful web links, and point of contact (POC) information.

    - Reserved

    - Maintain a baseline of GBS Broadcast Segment circuit connectivity between sources, SBMs, and uplink sites.

    - Schedule, track, report, administer, and execute Authorized Service Interruption (ASI) requests under the GBSOC area of responsibility.

    - Support GBS sustaining engineering periodic testing of GBS software to include antivirus updates, security updates, and patch updates.

    - Provide GBSOC test points of contact, participate in test-based meetings, review test plans, assist test participants under routine service desk calls, rebroadcast and over-the-air software update files, and transmit log files for active test days.

    - Support a transfer of SBM production (on-line) operations between GBS SBMs no less than once annually. The transfer schedule may be disrupted by a SBM COOP event, other government directed activities, such as real-world missions, test activities, or security patches, or software installs. The SBM transfer will consists of a complete operations transfer from the production SBM actively supporting the GBS broadcast to the non-production (off-line) SBM.

    - Conduct and document GBS SBM system checks remotely from the GBSOC at a minimum of every 24-hour period.

    - Provide GBS System Information Systems Security Officer (ISSO) support to the GBS Program Office ISSO, the GBS Program Office Information System Security Manager (ISSM), IAW AFI 17-130.

    - Collect and report operational metrics for the GBSOC area of responsibility (AOR), to include the GBS SBM, SBM Infrastructure, SBM MUE, GBS transport system, and GBSOC, in the monthly status report (CDRL A201 - GBS Monthly Status Report).

# CDRL A201 – GBS Monthly Status Report

# System Administration

    - Provide system administration GBS servers, databases, and applications under the GBSOC area of responsibility.

    - Perform equipment and application administration on SBM Virtual Operating Environment (VOE) servers and SBM MUE.

    - Perform privileged and authorized user account management (create, modify, delete, and set access privileges) for:

    - User access to SBM Application web-based interface.

    - User non-login SBM accounts for content provider file ingest.

    - SBM VOE administrator privileged user service accounts.

    - Administrator accounts for SBM MUE managed by the GBSOC.

    - Maintain user accounts and access for Information Technology Service Management (ITSM) system (currently remedy), and other GBSOC support systems.

    - Maintain and perform reviews of audit logs for security vulnerabilities and system maintenance.

    - Perform GBS SBM and MUE database management and backups.

    - Troubleshoot and report software and database related problems.

    - Perform real world and exercise SBM transfer system administration functions under the GBSOC AOR.

    - Perform limited functional system administration for the GBSOC NIPRNet and SIPRNet workstations. Personnel with a privileged user account are required to possess a minimum Information Assurance Technical (IAT) Level 2 certification IAW Air Force Manual (AFMAN) 17-1301, COMPUTER SECURITY (COMPUSEC).

# Configuration Management (CM)

    - Provide GBSOC related Operations & Maintenance (O&M) support for system engineering, configuration management, technical data management, and software sustainment activities GBSOC AOR.

    - Provide O&M support to engineering and configuration management change processes, to include monitoring Engineering Review Board and Deficiency Review Board actions.

    - Coordinate GBS and GBSOC system configuration changes with the GBS sustainment and operational organizations.

    - Perform source data provider integration actions as required by the GBS source integration processes IAW the current Global Broadcast Service (GBS) TIM Standard Operating Procedures (SOP), GBS CONOPS and SBM Operator’s Guide GBS-SBM-001.

    - Support configuration management of IAVM processes.

    - Coordinate SBM software updates and distribution.

# Transfer of Operations (TOOP)

    - CDC SBM Recurring

Support a transfer of SBM production (on-line) operations between the primary CDC, which is located at OKC, and the alternate CDC located at MECH GBS SBMs. The government will be responsible for the decisions to TOOP at least once a FY. The transfer may be disrupted by a SBM COOP event or other government directed activities, such as real-world missions, test activities, security patches or software installs. The SBM transfer will consist of a complete operations transfer from the production SBM actively supporting the GBS broadcast to the non- production (off-line) SBM.

# CDC SBM Tabletop Exercise

Conduct a government planned tabletop exercise with personnel who would be involved in an actual TOOP to include DISA once per year IAW SLA 943-USAF-GBS. Typically, tabletop exercises are conducted within one hour.

# GBSOC Contingency Operations

    - Relocate and provide limited GBSOC operations within one hour of the government directive to relocate to the contingency location at Schriever SFB. Perform limited GBSOC operations using contingency SIPRNet and/or NIPRNet terminal(s) and phones provided by the government. The duration of the contingency relocation will normally not exceed 72 hours. The loss of access to the GBSOC at SSFB or loss of SIPRNet and/or NIPRNet connectivity preventing or limiting GBSOC operations will drive the government decision to direct contractor support of GBSOC operations to relocate to the contingency location. Contractor operations from the contingency location may be a precursor to a GBSOC COOP. Document contingency relocation and operations procedures.

# GBSOC COOP (Real World Emergency)

Relocate the GBSOC mission essential functions (MEFs) with government provided GBSOC COOP capabilities within 12 hours of the government directive to COOP Mission Essential Services, and in collaboration with and under the guidance of the 21st Communications Squadron (21CS). The COOP location is at Buckley SFB, CO.  Alternate COOP locations may be indicated and directed by the government based on mission needs.  Additional time allowances for COOP capabilities will be considered for alternate COOP locations. The contractor will provide pre-relocation, relocation site operations, and return to normal operations support. The contractor will perform GBSOC MEFs once the GBSOC COOP is operational in response to real-world emergencies.

# GBSOC COOP (Exercise)

A COOP exercise shall be performed at least once a FY for a duration of no more than seven days; the government will determine the duration of the exercise.

3.8.4 GBSOC COOP (Maintenance)
A COOP maintenance run will be performed at a minimum every quarter at the primary COOP site to ensure SIPR accounts are still active and to perform functions checks on equipment.

Chapter 4

Mission Systems Maintenance and Reporting

# Mission Systems Maintenance

Perform corrective and preventive maintenance of mission systems identified in Table 4-1 – Mission System Listing IAW technical publications. The contractor is responsible to respond to equipment outages 24/7 unless specified otherwise.

Table 4-1 - Mission Systems Listing

| Mission System Listing |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | GBSOC | OKC | MECH | SSFB |
| Global Broadcast Service |  |  |  |  |
| AN/PSR-11 GBS Suitcase Portable Receive Suite AN/PSR-12 GBS Rucksack Portable Receive Suite | X |  |  |  |
| AN/TSR-11 GBS Transportable Ground Receive Suite | X |  |  |  |
| Crypto Equipment | X |  |  |  |
| GBS Satellite Broadcast Manager (SBM) Mission Unique Equipment (MUE) |  | X | X |  |
| Facility Uninterruptible Power System (UPS)/ System Bypass Module (SBM) Building 702 (during normal duty hours only) |  |  |  | X |
| MILSATCOM |  |  |  |  |
| AN/FRC-191(V)1 Family of Advanced Beyond Line-of-Sight Terminal (FAB-T) Command Post Terminal |  |  |  | X |
| FAB-T Data Set Manager |  |  |  | X |
| FAB-T 10FT Antenna System Tower |  |  |  | X |
| AN/GSC-52B(V)5 WGS Earth Terminal |  |  |  | X |
| AN/MSC-66 Digital Communications Satellite Subsystem |  |  |  | X |
| AN/USC-28 Spread Spectrum Multiple Access Anti-Jam (SSMA-AJ) Modem |  |  |  | X |
| Crypto Equipment |  |  |  | X |
| Interconnect Facility (ICF) Equipment |  |  |  | X |
| MD-1366/U Communication Modem |  |  |  | X |
| OM-88 (V)/G AS Modem Group |  |  |  | X |
| OM-88 A(V)1/G Protected Anti-Scintillation (AS) Anti-Jam (AJ) Wideband Net-Centric System (PAAWNS) |  |  |  | X |
| PAAWNS Management Component Site-Node (PMC-S) |  |  |  | X |
| UPS/SBM Building 700 |  |  |  | X |

# Maintenance

    - Mission System Maintenance

    - Repair maintain and align all equipment IAW technical order (TO)/technical manual (TM) or

commercial manuals. In the event no AF TOs/TMs exists, conform to the item TO, TM, or vendor manual, in that order.

  - Standard Maintenance Practices

    - Identify, investigate, analyze and correct all equipment malfunctions or out of tolerance conditions. Direct and monitor maintenance actions and report equipment or system scheduled or unscheduled status changes to the Maintenance Operations Center (MOC). Report any equipment or system outages to the MOC within 15 minutes, update the MOC every two hours during extended unscheduled outages, and notify the MOC once the action is completed. Submit requests for downtime to the government in order to accomplish corrective and preventive maintenance as required.

# Corrective Maintenance

    - Initiate organizational level corrective maintenance and respond to outages impacting mission within 5 minutes. Organizational level maintenance is limited by the source, maintainability, and recoverability (SMR) code as defined in the applicable system TOs. Not all equipment requiring organizational level maintenance has assigned SMR codes. The contractor will reference technical publications for specific guidance. The SBD 1 Chief of Maintenance (SBD 1/CoM) must approve all SMR change request.

# Preventive Maintenance Inspection (PMI) Performance

    - Perform all PMIs IAW applicable technical publications.

# Corrosion Control

    - Perform organizational level corrosion control on equipment to minimize the effects of corrosion IAW TO 1-1-689-1, TO 1-1-689-3, TO 31Z-10-37, TO 1-1-700 and MPTO 00-33A- 1001.

# Test, Measurement, and Diagnostic Equipment (TMDE)

    - Comply with the Air Force Metrology and Calibration Program IAW AFI 21-113, TO 00- 20-2, TO 00-20-14, TO 33K-1-100 series, TO 33-1-27, and Precision Measurement Equipment Laboratory (PMEL) directives.

    - Operate, maintain and establish controls, for the master inventory listing of owning activity, to ensure all TMDE are repaired, calibrated and certified. Perform organizational level maintenance on TMDE. Transport TMDE between the work-center and the PMEL facility. Follow-up on all TMDE in PMEL over 30 days.

# TO Accounts and Technical Publication Sets

    - Establish and maintain TO accounts and technical publication sets as required, for all assigned equipment IAW TO 00-5-1. Establish all sub-accounts through the SBD 1 Technical Order Distribution Office. Have a technical publication set in each work center under contractor control. The technical publication set shall include TOs, commercial manuals, DISA circulars, and other DISA guidelines and directives required for work center use. Verify technical documentation and initiate Air Force Technical Order (AFTO) Form 22, Technical Manual Change Recommendation, when documentation discrepancies are found IAW TO 00-5-1. These technical publication sets shall contain all work center documentation required for the

performance of the requirements. Provide documentation support services for the following types of material (but not limited to):

- Drawings.

- Vendor submittals.

- Commercial manuals.

- Military and DoD standards (limited collection).

# Job Data Documentation

    - Enter all maintenance actions into a maintenance information system. If the maintenance information system is not accessible, document maintenance actions on an AFTO Form 349 and ensure input into maintenance information systems when it is accessible.

# Deficiency Reports

    - Perform deficiency reporting originator duties IAW TO 00-35D-54 using the Joint Deficiency Reporting System (www.jdrs.mil) or other automated information system.

# Cannibalization

    - Identify and obtain cannibalization authorization from the SBD 1/CoM.

# Depot Assistance

    - For equipment identified in Table 4-1 – Mission System Listing, initiate requests, coordinate, support and provide status of all depot maintenance activities. Verify depot maintenance actions are complete. Review and forward the AFTO Form 217 (or equivalent) to the appropriate unit maintenance officer or SBD 1/CoM IAW TO 00-25-108 and AFSPCI21- 108_50SWSUP.

# Electro-Magnetic Interference (EMI)

    - Eliminate EMI as covered under the Spectrum Interference Resolution Program. Assist in the investigation and isolation of EMI. Investigate, identify, and report to the COR all electromagnetic interference that impacts mission support IAW applicable documents.

# Crypto Equipment

    - Perform organizational maintenance on all cryptographic equipment at the locations identified in Table 4-1 – Mission System Listing and IAW with applicable Air Force Communications Security (COMSEC) General Publications (AFKAG) and COMSEC directives.

    - Perform trained primary and alternate COMSEC Responsible Officer (CRO), Secure Voice Responsible Officer and Crypto Access Program administrator duties for the MILSATCOM mission area, SDIN mission network and the GBSOC mission area. The CRO shall be appointed in writing to the designated authority. The CRO shall accomplish the required training IAW AFMAN 17-1302-O, Attachment 5. Perform duties and tasks as assigned and identified in AFMAN 17-1302-O (paragraphs 5.4 and 5.5) to ensure compliance with COMSEC account programs IAW SSFB policies and procedures. The CROs must possess a security clearance commensurate with the highest classification of material as required.

# Self-Sufficiency Program

    - Coordinate for approval on all self-sufficiency efforts with the SBD 1/CoM. Information regarding the self-sufficiency program and SMR codes is contained in 00-20-3 and 00-25-195.

# MOC Mission Systems Reporting

    - The MOC is the 24/7 central POC for SBD 1 mission systems operations and maintenance. Provide on-site support from 0600-1800 (local) Monday - Friday. For all other times, provide on-call support. Operate the SBD 1 MOC IAW Air Force Space Command Instruction (AFSPCI) 21-108 SBD 1 SUP, Chapter 4.

    - Enter equipment status changes in Equipment Status Reporting upon receipt, not to exceed 15 minutes of notification from the user, site, associated work center, or the government according to AFI 21-103, Chapter 6.

    - Update the government approved status sheet(s) within 15 minutes of notification from the user, site, associated work center or the government.

    - Assign and track all job control numbers (JCNs), equipment and maintenance status in the government designated maintenance information system.

    - Request assistance from supporting agencies to verify and validate equipment outage status.

    - Coordinate maintenance and maintenance assistance (MA) requests via email using the distribution list provided by the SBD 1/CoM.

    - IAW AFSPCI 21-108 SBD 1 Supplement, the MOC shall email SBD 1 mission system part requests and mission capable (MICAP) requests to the distribution list provided.

    - Disseminate all information via email regarding equipment status changes when requested by the SBD 1/CoM IAW AFSPCI 21-108 SBD 1 Supplement.

    - Up-channel incident and status reporting by using the government approved spreadsheets to track, document, and provide status of all Outage Control Number (OCNs) / JCNs. Send email notification to the COR and SBD 1/CoM for mission systems conditions that affect a system’s operational capability.

    - The Maintenance Status Report shall include: “as of” date/time; site/side; mission system; OCN; JCN; discrepancy; equipment status; Estimated Time to Restore Operation; and last contact date. Additionally, a remarks/comments column shall contain details including troubleshooting, estimated delivery date, awaiting parts, awaiting maintenance, and any additional comments or status updates.

    - Provide the GPS status report NLT 0700L and all other SBD 1 Mission Systems 0730L Monday through Friday. Upon COR or SBD 1/CoM request, updated status reports may be required.

    - Monitor SSFB utility outages and freeze schedules impacting mission operations and equipment and provide related info to users/customers as requested.

    - The MOC shall process emergency depot level maintenance requests within 5 minutes of receipt and urgent depot level maintenance (UDLM) requests within 30 minutes of receipt IAW TO 00-25-108, applicable supplements and AFSPCI 21-108 SBD 1 supplement.

    - The MOC shall review and update open outage data in Electronic Scheduling Dissemination (ESD) and maintenance information system daily for all assigned SBD 1 mission systems.

    - The MOC will provide all SBD 1 work centers with maintenance information systems equipment inventory list (EIL) to the system and subsystem level semi-annually. The work centers shall verify the maintenance information systems EIL data semi-annually to ensure consistency with SBD 1 assets. This shall be accomplished for all SBD 1 equipment and may require interaction with other contractors. Work centers shall notify the MOC if changes are required. The MOC will update the EIL with applicable information. Maintenance information system data is identified as equipment designator, equipment identification (ID), system reference designator, and work unit codes associated with the equipment. Accomplish historical data documentation stating semi-annual EIL was accomplished in the maintenance information systems.

# Global Broadcast Service

Provide on-call GBS SBM MUE organizational-level touch maintenance at the OKC and MECH facilities under the direction of the GBSOC. On-call maintenance personnel shall provide telephonic response to a request for support within five (5) minutes of initial notification. For problems affecting GBS OPSCAP, the on-call maintenance personnel shall respond on-site to initiate corrective maintenance actions within two (2) hours after a confirmed notification.

# Wideband Global Satcom

    - Large Fixed Earth Terminal Operations and Maintenance

      - Provide 24/7 continuous on-site operations and maintenance of two Large Earth Terminals and associated sub systems, components, baseband and interconnect facility equipment. Comply with Presidential and DoD Directives and Instructions that pertain to the operations and maintenance of the equipment in both peacetime and wartime environments.

      - Comply with all instructions received via the satellite controllers, local TCF/ Circuit Control Office (CCO), Global Terrestrial Critical Control Circuit System (GTC3S) and ECCM Critical Control Circuit (CCC), and provide all requisite reports and information requested.

# FAB-T Operations and Maintenance

      - Provide 24/7 on-site operations and maintenance of the FAB-T, 10FT Antenna System Tower, Data Set Manager, and associated sub systems, components and interconnect facility equipment. Comply with Presidential and DoD Directives and Instructions that pertain to the operations and maintenance of the equipment in both peacetime and wartime environments.

      - Comply with all instructions received via the satellite controllers, local TCF/ CCO, and provide all requisite reports and information requested. Maintain threshold and performance objectives IAW the Joint Operational Requirements Document-FAB-T Communications System.

# Radome and Base Ring Maintenance

      - Provide organizational level maintenance of the Large Fixed Earth Terminal’s radomes, base rings and its associated equipment and components IAW technical documentation.

# Multiple Terminal Outages

      - Respond to outages impacting mission, initiate organizational level corrective maintenance, and perform maintenance to each terminal outage independently and simultaneously, when they occur.

# Status Reporting

      - Perform all status reporting, Gateway reports, Frequency Division Multiple Access (FDMA) reports, Electronic Counter-Countermeasures (ECCM) reports, System Equipment Reporting System (SERS) reports, Hazardous Condition reports, After Action Reports (AAR), Defense Readiness Reporting System (DRRS), and reporting as identified in the Colorado Springs Regional Command Post Commanders Operational Reports Handbook.

# Authorized Service Interruption (ASI)

      - Process, submit, and track ASI messages for assigned equipment.

# Master Station Log (MSL)

      - Maintain a Master Station Log (MSL) that shall contain all events relating to the operations and maintenance of the terminals, equipment configuration changes, and facility and utility events that impact the terminals. The log shall be made available for government review throughout the life of the contract. Contractor shall utilize COR-approved format (MS Office, PDF).

# High-Altitude Electromagnetic Pulse (HEMP)

      - SSFB MILSATCOM Bldg 700 is a HEMP certified facility. The contractor will not make any modifications to the facility or HEMP protection features without obtaining written authorization from the SSFB HEMP Manager. The SSFB HEMP Manager will provide the HEMP training material.

# Equipment Housekeeping

      - Perform general equipment housekeeping duties for all assigned work areas. Floors and workspaces shall be clean and clear of hazards. The contractor will perform quarterly cleaning of equipment racks, under raised floors, cable troughs connecting the facility to the terminal antennas, and cable trays, to include removal and disposal of abandoned equipment, cables and waveguides.

# IAT Level 2 Certification

Contractors assigned to the MILSATCOM work center must have a minimum IAT Level 2 certification IAW AFMAN 17-1303 and maintain this certification throughout the contract.

# Monthly Outage Listing

Prepare and maintain a listing of all satellite terminal link and trunk outages caused by facility utility, equipment failures, or operator/maintainer actions on the MILSATCOM equipment listed in Table 4-1 – Mission System Listing. The listing should include at a minimum the ID / Start Time / End Time / Total Time / Reason for Outage (RFO).

# Configuration Documentation

Prepare and maintain a folder for of each satellite terminal links, trunks and circuits associated with the MILSATCOM equipment listed in Table 4-1 – Mission System Listing. The folder shall contain at a minimum any associated Satellite Access Authorizations (SAA) messages,

Technical Service Orders/Request (TSO/TSR) messages, a circuit layout and routing diagram, cut sheets documenting equipment configuration settings, and a POC listing for the TCF/CCO and Distant End SATCOM. The documentation shall be maintained in both electronic and hard copy formats. Contractor shall utilize government-approved format (MS Office, Visio, PDF).

# Certification of Special Equipment

    - Be responsible for certification of special equipment (e.g. cranes, lifts, hoists and climbing gear) IAW equipment specifications, Occupational Safety and Health Administration (OSHA), Air Force Occupational Safety and Health (AFOSH), federal, and/or state standards, whichever is most stringent. Approval and reimbursement shall be IAW PWS paragraph 2.8 - Local Purchase.

  - Reserved

# Information Technology Equipment Custodian (ITEC)

Appoint a primary and alternate ITEC custodian and maintain IT assets and Air Force Equipment Management System (AFEMS) - Asset Inventory Management (AIM) accounts for all government-owned hardware and software under the MILSATCOM, CM, SDIN/GCCS, MOC and GBSOC AOR IAW AFMAN 17-1203.

  - Reserved

  - Reserved

  - Reserved

# Systems and Software Support

Provide systems management and operations verification support to information and communications systems identified in Table 4-1 – Mission System Listing and IAW publications or commercial manuals. Upon request by SBD 1, take over support and maintenance for any active and mission-necessary NIPR/SIPR database/web application that is determined as “orphaned”, i.e. – no longer has contract support or competent non-contract support from within the using SBD 1 unit.

# Systems Management

      - Troubleshoot and analyze problems by executing all government-approved procedures/ processes necessary to restore operations.

# Software and Database Installation and Checkout (I&C)

Install, checkout, and de-install modifications as approved by applicable MCB government representative, as needed all software changes/updates to information and communications systems. Coordinate and work with the 50th Communications Squadron (50 SC)/SCOO and/or application users, to implement these changes into operational systems.

        - Submit an ASI request to the COR or applicable MCB government representative identifying resources and schedule requirements for required software updates. Provide a realistic installation timeline to the COR or applicable MCB government representative and shall adhere to the government-approved installation schedule.

        - Notify the COR or applicable MCB government representative immediately of any problems affecting the system as a result of I&C. Operationally support the system

owner/manager in order to return the system to the operational configuration as directed by the applicable government configuration control authority.

        - Reserved

        - Reserved

        - Ensure required documentation for I&C of all software modifications is entered into maintenance information systems and SBD 1 equipment/systems when required by the government.

# Mission Support Database Maintenance

For information and communications systems databases identified in the SBD 1 database listing (include current listing in PWS Table 4-1 – Mission System Listing and Table 4-2 –Database Support), receive all user requests for database application and software tool modifications and updates. The contractor develops the MCB agenda, containing the requests, and distributes it to the MCB members. Upon approval by the 50 CS Database MCB and the database (DB) COR, modify and update existing database applications or software tools.

# Regression Testing

Create the regression test plan (CDRL A301 - Regression Test Plan) for each database resource modification/ enhancement, see Table 4-2 – Database Support.

# CDRL A301 – Regression Test Plan

      - Provide written notification to the COR within 1 business day of any problem discovered during testing. Provide written status update of testing results within 1 business day of each test milestone completion. Deliver a final regression test report for each database installation package tested IAW CDRL A302 - Regression Test Report.

# CDRL A302 – Regression Test Report

      - Prepare and forward an installation/de-installation request to the COR or applicable MCB government representative.

# Table 4-2 - Database Support

Databases

4th Space Operations Squadron (4 SOPS) Web Application

SBD 1 Anomaly Tracker (SWAT)

Consolidated Analysis Support Environment

Cybersecurity Vault

Event Tracker

FAB-T Automated Planning System

FAB-T Automated Support Scheduler

MILSATCOM Integrated LOG Entry System (MILES)

Mission Impact Report Trending and Analysis (MIRTA)

FAB-T Incident Summary

Mission Problem Report (MPR)

Phoenix Project Support

Databases

System for Network Analysis and Planning (SNAP)

Temporary Access Records System

Thruster Efficiency

Visitor Access Request

# Technical Support

Provide technical support for SBD 1 mission systems, as identified in this PWS and in Table 4-3 – Technical Support. Assist various organizations or agencies performing technical analysis, modifications, testing, integration, meeting support and technical documentation support to the SBD 1.

# Table 4-3 - Technical Support

| Support | Para | MILSATCOM | OKC/ MECH | GBSOC | Database |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 4.24.1.1 |  | X | X |  |
| Procedure Change Recommendations | 4.25.1 |  | X | X |  |
| Modification Support – Requirements processing | 4.25.2.1 |  |  |  | X |
| Modification Support – Minimize downtime | 4.25.2.2 | X |  |  |  |
| Modification Support – SBD 1 Compliance | 4.25.2.3 | X |  |  |  |
| Installation and Checkout - Perform | 4.24.1.2 4.24.1.2.1 | X |  |  |  |
| Installation and Checkout - Maintenance Information Systems | 4.24.1.2.5 | X* |  |  |  |
| Testing Support | 4.25.3 |  | X | X | X |
| Meeting Support | 2.11.3 |  |  | X | X |
| Contractor Generated Documentation | 4.25.4.1 | X |  | X |  |
| Contractor Submitted Documentation | 4.25.4.1.1 |  |  | X |  |
| Government Delivered Documentation | 4.25.4.2 |  |  | X |  |

*Only systems reported in maintenance information systems

# Procedure Change Recommendations

Recommend improvements, assist the government in developing operational procedures, and associated training documentation.

# Modification Support

      - Provide technical support for submitted requirements processing. Identify, obtain, analyze, and document system change requirements using government provided change request processes and databases.

      - Work with government representatives and associate contractors to minimize equipment downtime and/or impact during project activities. This coordination shall include projects in transition from development and interim support to operations and maintenance support by the SBD 1. Prior to implementation of any system change, obtain approval from the MCB.

      - Comply with P-S GARI 17-201 when installing, modifying, or removing any information or communications system.

# Testing Support

Assist agencies responsible for system testing of mission equipment and support applications (hardware, software, firmware, database, and supporting structures). Operationally assist agencies conducting regression testing.

# Technical Documentation

Develop and perform reviews of government provided technical documentation for accuracy and completeness.

# Contractor Generated Documentation

The final version of documentation and products shall be 100% technically accurate and complete. Documentation includes corrective action forms, technical analysis documentation, and relocation to a back-up facility documentation, operational procedures/checklists, implementation information, and test information.

# Contractor Submitted Documentation

Submit paperwork to initiate resolution of hardware and software issues IAW unit OIs and/or policies and GBS sustainment organization configuration management procedures. These forms shall include but are not limited to: Database Change Requests (DBCR), Operations Procedure Change Requests, Change Request Forms (CRF), Procedure Change Requests, Problem Reports, Deficiency Reports and Software Trouble Reports.

# Government Delivered Documentation

Perform reviews of government provided documentation. Assess whether the mission needs have been met and assess operational, maintenance, and logistic impacts if the applicable mission capability is lost. Review and assess briefing/meeting information, corrective action forms, relocation to a back-up facility documentation, training material, and operational procedures.

# Mission Impacting Reporting (MIR) Support

Provide on-site support, co-located in 22nd Space Operations Squadron (22 SOPS) or any backup location on SSFB designated by the SCN Crew Commander, MIR support 24/7.

# MIR Reporting

Obtain relevant MIR data, database entry, MIR tracking and management, review of mission impacting events, investigation and analysis of problems, determination of root cause, development and documentation of lessons learned, documentation of proposed process

improvement, briefing and presentation of analysis, and direct face-to-face interactions with the SCN Crew Commander in 22 SOPS throughout MIR process.

      - Receive, verify accuracy and ensure MIRs are processed into the Mission Problem Report (MPR) database upon receipt or as close to receipt of the MIR event as possible to provide the most accurate information for the thorough investigation of each MIR IAW 50th Space Wing Instruction 10-217.

      - Verify ROADS and ESD or replacement scheduling system entries of applicable affected satellite supports to determine initial MIR resolution routing, notify maintenance personnel as required, and document equipment status, malfunctions, and corrective actions into the MIRTA database.

      - Directly interrogate the Distributed Communications Controller (DCC) to acquire necessary details for investigating any MIR related issue, as required. DCC event logs and status details may assist root-cause analysis and/or closure determination(s).

      - Coordinate with RTS personnel when applicable ROADS/ESD entry has insufficient or missing data until ROADS entry is resolved.

      - Coordinate with originating satellite operations center if submitted MIRs are unclear or incomplete until MIRs are resolved. Continue to track the MIR resolution status until closure and ensure all MIR statuses are entered, updated and tracked in MPR and MIRTA until closed.

      - Advise of any mission impacting issues to the SCN Crew Commander within 5 minutes.

      - Provide feedback to the originator of the MIR through closure.

      - Document contributing factors, document lessons learned within the MIR process, trend analysis and root cause analysis in the MPR and MIRTA database.

      - Attend and brief MIR related meetings in support of the 22 SOPS mission to include the 22 SOPS Stand-up, SCN User Conference and Deep Dive briefings and the 50th Network Operations Group (NOG) Operational Update. Brief MIR analysis findings at the SCN User Conference and Deep Dive briefings. Additional ad hoc MIR requests shall be supported with no more than 24 per performance period.

      - Lessons learned shall be documented and upon review and approval of the recommended change(s), the COR shall approve a contractor-determined format to document any lesson learned/process change. The closure investigation efforts shall provide the necessary insight to effectively assess overall SCN reliability or possible future mission impacts.

      - MIR analysis shall be accomplished within 4 hours after MIR notification and shall be considered resolved after correctly determining root-cause contributing factors and trending associations to provide any appropriate recommendation to the SCN Crew Commander to minimize operational impacts. Root cause analysis and closure shall be at a sub-system level, when possible. Targeting root cause to an end item or even down to a line replaceable unit/component is ideal but closing at the high level (e.g. range segment) does not meet the intent of root cause.

      - Transfer ESD outage information upon receipt into the MPR database. Post-transfer of data, continue to monitor ESD outages, and update corresponding MPR outage data daily until final closure in each system.

      - Enhance, revise and maintain a classified, interactive MPR sub-set database, MIRTA, for the analysis collection, query and trending of all MIRs and MIR data. This database shall provide the capability for a wide variety of sorting and queries to conduct analysis and trending of the MIRs to minimize operational requirements. The database will be available as a classified database and will provide query capability and trending for SCN operational requirements.

Chapter 5

Supply and Logistics Support

# Supply and Logistics Support

# Supply Support

Manage and provide supply support to work centers identified in this PWS IAW AFI 23-101. Implement positive control and accountability of assets to ensure technical and administrative compliance with contract requirements. Designate points of contact, establish and maintain contact with the 21 SW base supply and Air Force Global Logistics Support Center (AFGLSC), Scott AFB, IL as required.

# After-Hours Support

Establish procedures for after-hours support for mission-impacting conditions IAW work center specific OIs.

# Supplies and Equipment

      - Provide materials, supplies and equipment to include non-stock listed and commercial- off-the-shelf (COTS) items. Manage, maintain, control, procure, issue, inventory, and safeguard government property and supplies. For direct mission support, utilize the SBSS/ Enterprise Solution Supply (ES-S) System as applicable. Perform all necessary functions to ensure accountability. Use authorized controlled transaction identification codes for SBSS/ES-S processing. When not available through SBSS/ES-S or not available in sufficient time through SBSS/ES-S to meet mission and administrative requirements or when processing through normal supply channels are not available or practical using procedures at PWS paragraph 2.8 - Local Purchase.

      - Establish procedures to purchase, receive, store, track and distribute all assets purchased. Advise equipment accountability element of assets that require R-14, Custodian Authorization Custody Receipt Listing (CA/CRL), or R-15, Organizational Visibility List accountability. The contractor shall support and manage the DoD directed item unique identification (IUID) program for all existing and new items. The contractor shall support annual equipment accountability inspections.

      - The 635th Supply Chain Operations Group (635 SCOG) will accomplish MICAP and all back-shop supply actions, e.g. stock control, some equipment and records maintenance actions. For indirect mission support, the contractor is authorized to use the local purchase procedures identified in the PWS paragraph 2.8 - Local Purchase.

      - Prepare and submit adjusted stock levels (ASL) review and document ASLs as requested by the 21st Logistics Readiness Squadron or MAJCOM IAW AFI 23-101. Use AF Form 1996, if required.

      - Account for items via government property records or through SBSS/ES-S and IUID procedures IAW AFI 23-101. This includes all assets sent to Defense Reutilization Management Office.

      - Track items requiring accountability (that are not on the CA/CRL on the R-15, Organizational Visibility List IAW AFI 23-101. The contractor shall support and manage the

DoD directed IUID program for all existing and new items. The contractor shall support annual equipment accountability inspections.

# Equipment List

        - Develop and maintain an automated list of all mission, mission support, and non- mission equipment, software, and tools. This shall be provided using an approved format (MS Excel®, MS Access®, etc.). Site, mission system, location, building, room, rack, etc. shall identify the equipment. The software list shall record each software name, version, license number, location, expiration date and disposition of the replaced software. Upon contract award, the incumbent contractor will provide equipment lists to the successor contractor. Submit any changes to this listing IAW CDRL A401 – Equipment List and Inventory. All equipment lists will become the property of the government.

# CDRL A401 – Equipment List and Inventory

        - Verify and accept all Government furnished equipment required for the operation and maintenance (i.e. spare parts) of the SDIN networks and GCCS. Inventories shall be accomplished IAW CDRL A401 – Equipment List and Inventory.

        - Any defective equipment shall be documented and submitted to the COR for concurrence within thirty (30) days of discovery of the defect. Advise and work with the COR to plan a cost-effective resolution for defects. Identify deficiencies and make recommendations regarding equipment. When any of the Government furnished items are determined to be beyond economical repair, at no fault of the contractor, coordinate with the COR for the return/disposal of the item.

# Tools

        - For MILSATCOM, manage and update a baseline tool inventory and maintain throughout the life of the contract. No less than quarterly, inventory all tools and document any discrepancies between the baseline inventory and actual inventory. (CDRL A401 – Equipment List and Inventory)

          - Replace missing tools with tools of the same quality and vendor when possible, at contractor expense.

          - Replace broken tools utilizing existing warranties, where applicable. If no warranty exists, the contractor is authorized to use the local purchase procedures identified in the PWS paragraph 2.8 - Local Purchase.

        - For GBS, the tools utilized for maintenance shall be contractor provided and managed, no government funded tools provided.

# PMEL Support

When the host PMEL is unable to provide repair parts, use local purchase procedures in order to satisfy mission requirements IAW PWS paragraph 2.8 - Local Purchase.

# Vehicles

Provide all vehicles necessary to perform the requirements listed in this PWS.

# Logistics Support

    - Provide analysis of supply reports and interface with supply personnel for problem resolution.

      - Provide updated status of all critical spares to SBD 1/CoM upon request.

# Logistics Support for Depot Level Maintenance (DLM)

      - Coordinate on-site DLM logistics requirements.

      - Receive, stage, and then ship equipment in support of programmed depot level maintenance visits.

      - Provide parts and consumable materials required and identified by the depot to perform on-site depot maintenance.

# Recording Media

Manage recording media to support mission requirements. Additionally, inventory and label all recording media.

# Warranty Services and Repairs

Track and exercise all available warranties. Assume maintenance responsibility upon warranty expiration.

# Hazardous Materials

Receive, store, issue, inspect, distribute, and dispose of hazardous material IAWAFMAN 32- 7002, local, state, and federal laws. Maintain a file of all material safety data sheets for hazardous materials.

# Traffic Management Support

Accomplish all government property shipments via the following priority: government bill of lading, commercial bill of lading, and/or commercial mail. All MICAP shipments will be accomplished using the fastest means of transportation available.

Chapter 6

Power Management Support

[Removed – will not be included in consolidated requirement]

Chapter 7

Space Base Delta 1 Cybersecurity

TSCO and Information/Communication Systems Support

  - Team Schriever Cybersecurity Office (TSCO) and Information/Communication Systems Support

Provide cybersecurity and information communication services IAW AFPD 17-1, Information Dominance Governance and Management.

# TSCO Cybersecurity Office

    - Provide cybersecurity and technical advisory services to the TSCO Cybersecurity Officer. TSCO contractor personnel must have a minimum IAT Level 2 certification IAW AFMAN 17- 1303 and maintain this certification throughout the contract.

      - Conduct virtual on-site COMPUSEC Staff Assistance Visits (SAV) in support of the all SSFB and Delta 6 Geographically Separated Unit (GSU) Unit Commander’s COMPUSEC Programs.

      - Provide inspection support to the Cybersecurity Officer during IG cybersecurity inspections at SSFB and at the Delta 6 GSU locations.

      - Develop and maintain training materials. Provide initial and refresher cybersecurity training to the Unit Commander’s Support Staff (CSS) or the Unit IAO Express Representatives. Track and document training sessions.

      - Accept, review and provide a response to all Cyberspace Infrastructure Planning System (CIPS) tickets assigned to the TSCO within five business days of receipt.

      - Access the DoD Cyber Scope Website on SIPR and complete the monthly report for submission to Space Operations Command (SpOC).

      - Obtain access to the Information Technology Investment Portfolio Suite (ITIPS) in order to maintain SBD 1 and Delta 6 owned system entries.

      - For all SSFB units and GSUs, maintain an electronic unit records folder for each organization. The contractor will be required to access, add, or modify the TSCO records management files IAW AFI 33-322.

# Computer Security

Provide guidance to SSFB units and Delta 6 GSU to ensure the SBD 1 COMPUSEC program complies with AFI 17-130, AFMAN 17-1301, associated MPTOs and supplements.

      - Reserved.

      - Brief the TSCO Cybersecurity Officer on the status of the COMPUSEC Program.

      - Review software management inventories and perform software management reviews of SSFB and the Delta 6 GSU NIPR/SIPR LANs IAW AFMAN 17-1203, Chapter 3.

      - Research software for approval to use on the SSFB and Delta 6 NIPR/SIPR LAN systems.

      - Monitor and maintain the Ports, Protocols and Services Management Program (PPSM)

for NIPRNet and SIPRNet IAW DoD 8551.01 and AFSSI 8551.

      - Monitor and track the Data Loss Prevention Program (USB external storage device process). Provide procedure instructions to the user of the USB device in the form of instructions and process flow for the approval process.

      - Ensure that a Removable Media User Agreement is on file for each user on an approved waiver spreadsheet.

      - Track the request form from request received in the office through device implementation by the 561 Network Operations Support Center (NOSC). Provide a “whitelist” of approved devices to the SSFB and Delta 6 GSUs by Organizational Unit (OU).

# Cybersecurity Awareness

      - Provide support, guidance, and information to educate network users that are supported by the TSCO.

      - Develop and distribute a bi-monthly cybersecurity newsletter for all SSFB units and Delta 6 GSUs. The TSCO Cybersecurity Officer approves this newsletter prior to distribution. Place newsletter, information, and education materials on the Team SSFB Cybersecurity Collaborative site.

# TEMPEST

Perform tasks identified in Air Force System Security Instruction (AFSSI) 7700 and AFSSI 7702 (to become AFMAN 17-1305) to ensure the SBD 1 TEMPEST program complies with the requirements as defined in DoD 8500.01, AFI 17-130 and AFMAN 17-1301.

    - Conduct TEMPEST assessments at SSFB and the Delta 6 Host Base Cybersecurity Office

    - Maintain Space Force TEMPEST certification through Space Force approved training.

    - Report to the TSCO Cybersecurity Officer for the TEMPEST Program and brief the TSCO on the status of the TEMPEST Program.

# Risk Management Framework (RMF) Authorization Service

Support the SBD 1 ISSM in the review of information system authorization packages for SSFB and Delta 6 unclassified/secret information systems and network enclaves in order to meet the requirements of System Authorization and Federal Information Security Management Act.

    - Obtain access to the Enterprise Mission Assurance Support Service (eMASS-NIPR) and (eMASS-SIPR) in order to create, review, update, and maintain the SSFB and Delta 6 RMF NIPR and SIPR authorization packages.

    - Obtain access to the DISA System Network Approval Process (SNAP), and Global Information Grid Interconnection Approval Process (GIAP), to update entries for SBD 1 owned unclassified/secret network circuits (CCSD).

# Cyber Readiness (CR) Support:

Provide dedicated, embedded, on-site support to the 50 CS and SBD 1 Cybersecurity Officer for CR support and compliance. For this requirement, maintain a minimum of DoD 8140 IAT Level II certification. Focus on compliance of the Vulnerability Management Program using DISA policy guidance. Coordinate all CR efforts with the WCO prior to execution.

# Vulnerability Management

Implement vulnerability remediations IAW the DISA Security Technical Implementation Guides (STIG) and Assured Compliance Assessment Solution (ACAS) scans. Provide patch management IAW TO 00-33A-1109 (Air Force Information Network (AFIN) Vulnerability Management), acknowledge receipt of cyber orders, update the Air Force Network Operations (AFNETOPS) Compliance Tracker (ACT), create Plan of Actions & Milestone (POA&M) as necessary and close cyber orders when all actions are complete. Coordinate with CFP, Program Management Office (PMO) and other offices as required for reporting purposes and to ensure compliance. Maintain Standard Operating Procedures (SOP) to ensure procedure is efficient and properly documented for continuity.

      - Manage the POA&M approval process, perform security inspections and ensure Video Teleconference compliance.

      - Recommend corrective actions for all network vulnerabilities and STIG non- compliances.

      - For un-remediated vulnerabilities, provide POA&M inputs via the AFIN Mission Assurance Center (AMAC) or the approved replacement.

      - For SSFB and the Delta 6 GSU NIPR/SIPR LANs at Kaena Point, Guam, Oakhanger, and New Boston, review network enclave vulnerability reports generated by ACAS and/or the DISA Security Content Automated Protocol Tool report all vulnerability and integrity issues to the government. Assist the government with remediation actions, see Table 7-1 – Vulnerability Reports.

# Table 7-1 - Vulnerability Reports

| Location | NIPR | SIPR |
| --- | --- | --- |
| New Boston AFS, NH | X | X |
| TCS Oakhanger, UK | X | X |
| Kaena Point, HI | X | X |
| Guam Tracking Station, GU | X |  |

      - Monitor the ACT to ensure that the Delta 6 GSUs are compliant. Notify the Delta 6 GSU responsible personnel of any ACT alerts that are overdue for compliance.

# Program Documentation

Develop and maintain a set of CR continuity support program folders. These folders shall contain all CR guidance, regulations, checklist, AFMAN’s, AFI and related material that supports CR. All documentation is the property of the Government.

# Travel

Travel to the contiguous United States (CONUS) / outside contiguous United States (OCONUS) locations as necessary in support of the SBD 1 Cyber Readiness Program. Any travel arrangements require recommendation from the COR and written pre-approval by the PM or CO IAW PWS paragraph 2.7.

# Special Security Representative/Sensitive Compartmented Information Facility (SCIF)/ MOD Facilitator – Systems ISSO

    - Provide technical support labor to perform physical security and cybersecurity oversight functions in support of the Cyber Defense Operations Center (CDOC) within 61 CYS.

    - Provide technical guidance by drafting, coordinating, revising, tracking, and preparing security related documents prescribed by DCO mission partners and/or higher headquarters.

    - Prepare papers in Space Force (SF), SBD 1, AF Intelligence Community (IC), or National Security Agency (NSA) approved formats, including products such as, but not limited to, general security correspondence, accreditation packages and security operating procedures. To accomplish assigned tasks, participate in technical and programmatic meetings to include possible attendance at locations other than SSFB.

    - Demonstrate ability to exercise security oversight and have exceptional oral and written communication skills.

    - Apply knowledge and experience with standard information system security concepts, practices, and procedures. Possess the following qualifications:

    - Thorough understanding and application of network security principles, practices and implementations and a working knowledge of cross-functional integration of information systems into a physical security environment.

    - Knowledge of approved product lists (APL) for hardware and software for the systems utilized in addition to knowledge of AF IC & SSO policies and requirements to bring in the equipment into a SCIF.

    - Knowledge of system functions, security policies, technical security safeguards, and operations security measures.

    - Familiarity with detecting, preventing, and containing computer security compromises in a networked environment.

    - Knowledge of configuration management; system maintenance; and integration testing.

    - Understanding of government off the shelf (GOTS)/COTS tools that scan at the physical layer of all removable and fixed media types including but not limited to: (CDs, hard drives, thumb drives, Zip/Jazz, etc.).

    - Ability to decipher and explain in clear language ICD 503, RMF related NIST SP 800s series, AFMANs, etc.

    - Ability to support forensics and evidentiary preservation.

    - Ability to apply a risk management philosophy when faced with security challenges and the ability to articulate the pros and cons in a clear concise manner.

    - Analytical abilities to decipher complex technical configuration management documents.

    - Working knowledge of all applicable National Reconnaissance Office, IC, DoD policies, procedures and operating instructions related to Information Technology, Information Assurance, Information Management (IT/IA/IM).

    - At a minimum, the contractor must have and maintain a Security+ certification.

Chapter 8 Systems Safety

[Removed – will not be included in consolidated requirement]

Chapter 9 Reserved

Chapter 10 SDIN/GCCS

# Space Digital Integrated Network (SDIN) and Global Command and Control System (GCCS)

# General

Operate and maintain SDIN and GCCS systems in order to reduce the risk of outages and minimize mission impacts as outlined in this section. All contractors working on the SDIN system shall have a minimum Secret security clearance. All Contractors working on the GCCS system shall have a minimum Top Secret (TS) security clearance. The Contractor shall have sufficient personnel with a TS clearance with Sensitive Compartmented Information (SCI) eligibility to perform work requiring that clearance level, such as attending SCI level meetings and entering SCIFs where SDIN or GCCS remote systems are located while performing installation and/or maintenance functions.

    - Normal duty hours for GCCS operations support, 0700 -1700, Monday through Friday, respond with fifteen (15) minutes. After duty hours provide GCCS on-call support telephonically 24/7 within fifteen (15) minutes of first notification to the Communication Focal Point. If required, respond on-site within two (2)-hours to PSFB, SSFB, and USAFA, and four (4)-hours to BSFB.

    - Normal duty hours for SDIN operations support is 24/7. Consoles shall be manned 24/7, unless directed to evacuate or to relocate to the COOP site. Have two hours to have the COOP site up, operational and responding to all calls within fifteen (15) minutes of first notification.

    - These network services support SpOC, United States Space Command (USSPACECOM), United States Strategic Command (USSTRATCOM) North American Aerospace Defense Command (NORAD), United States Northern Command (US NORTHCOM), and Missile Defense Agency (MDA) mission data applications and transport and other missions as appropriate.

    - The primary location to provide these services is SSFB CO, with back-up/contingency locations for SDIN at the TCF at CMAFS, CO and for GCCS at VAFB, CA.

    - Update and maintain architectural network infrastructure diagrams as each change occurs.

    - Identify system process improvements to the COR for review prior to implementation.

    - Respond to short notice taskings COOP, Notice to Airmen (NOTAMs), etc. to meet Government deadlines.

    - Notify and provide written corrective recommendations to the COR/GPM of any equipment or service that is failing or degrading the network performance and may require corrective action within five business days of discovery.

    - Follow all applicable Memorandums of Agreement and, Memorandums of Understanding (MOUs) pertaining to SDIN and GCCS.

    - Perform SDIN and GCCS-SpOC enterprise-wide network administrative activities. These activities may include remote administration/access, system setup, Information

Technology (IT) device configuration and installation, troubleshooting, monitoring, security, infrastructure installation, network protocols, and network encryption systems.

    - Perform configuration changes, including configuration, installation and management of IT devices, as required. Comply with 50 CS established change/configuration procedures.

    - Protect SDIN and GCCS systems from malicious code. Install approved current anti- virus software, updates, patches, and definitions on every computer, information system, network, and stand-alone computing devices under contractor administration within NOTAM specifications.

    - Upon discovery of any security system problems/incidents on SDIN and GCCS, report them to the Mission Defense Teams (719-567-5509) and notify the COR within four (4) hours (next duty day if after hours).

# SDIN and GCCS COOP

    - When a COOP is required, deploy to the backup locations after notification by the GPM or CO. Provide the GMT with 24/7 POC information for contingency response matters.

    - The GPM, or CO, will make the final determination to COOP and the SDIN relocation site is required up and operational within two (2) hours.

    - For GCCS, the contractor will deploy to the primary COOP site after being notified by the GPM, or CO, within twenty-four (24) hours after the initiation of the event.

# Work Status Reports

    - Provide status updates of ongoing taskings upon COR request.

    - Provide a Monthly Activity Report (MAR) IAW CDRL A503 - Monthly Activity Report.

A503 – Monthly Activity Report

# Reserved

# Reserved

# Support Services/Maintenance Contracts

    - Review and track SDIN Hardware (HW)/Software (SW) and GCCS HW/SW warranty maintenance coverage in the MAR and notify the COR on the status of the warranties. (A503 – Monthly Activity Report)

    - Assess HW & SW of the SDIN systems and deliver a plan for technical refresh on an annual basis IAW CDRL A504 - Engineering Study. The plan shall include End of Life/End of Service (EOL/EOS) dates and recommended equipment replacements that integrates with existing equipment.

# CDRL A504 - Engineering Study

# Integration

    - Review project documentation and identify any integration issues that could impact SDIN and GCCS systems as requested by COR.

    - Review, coordinate and support the implementation of technical solutions.

    - Prior to implementation of any system change/upgrade, the Contactor shall obtain approval from the MCB.

    - Perform System Recoverability tests to verify system recovery from hardware and software failures.

    - Perform System Regression testing to verify system-wide stability during software and hardware upgrades.

# Asset Management

    - Perform Government asset management activities (i.e., IT Control Officer, Government provided spares, etc.) in the management of HW/SW lifecycles, including all activities related to asset management associated with HW/SW purchases IAW PWS paragraph 2.8 - Local Purchase.

    - Provide a list of equipment purchased each month with respective costs in the MAR. (CDRL A503 - Monthly Activity Report)

    - Track and maintain software license documentation IAW AFMAN 17-1203 and inform the COR when licenses require renewal. Report software status in the MAR those licenses within four months of expiration. (CDRL A503 – Monthly Activity Report)

    - Maintain the spare inventory at sufficient levels to maintain system operations. Notify the COR of low spare supplies within ten (10) business days of identification to coordinate purchases.

    - Any software, equipment, parts, and materials purchased shall be added to the appropriate inventories (CDRL A401 – Equipment List and Inventory). The property remains with the Government at the conclusion of the contract.

    - Maintain and manage SDIN GFE and critical/non-critical spare equipment. Utilize Bldg 702 located on SSFB as a secure environmentally controlled storage facility capable of storing approximately 15-20 pallets of spares.

    - Maintain all local hand receipts for equipment issued to other sections and units. Ensure that all local obsolete equipment is properly sanitized and prepared for disposal. Notify the GMT within one week of discovery of any missing equipment. The contractor will have 24/7 access to the spare equipment.

# SDIN and GCCS Network Services

    - These network services support SpOC, USSPACECOM, 21USSTRATCOM, NORAD, US NORTHCOM, and MDA mission data applications and transport and other missions as appropriate.

    - Operate and maintain infrastructure devices on SDIN and GCCS managed networks in their AOR. Oversee administration, operate, maintain and document systems security, including security protocols, authentication, authorization, auditing, anti-virus and encryption. SDIN has approximately 50 to 75 Users in Active Directory (AD) along with approximately 40 workstations on the combined networks. GCCS does not manage Users in AD. SDIN has approximately 50 servers encompassing Windows, CentOS, and RedHat. GCCS has approximately 8 servers encompassing Windows and RedHat. There are an estimated 40 workstations on SIPR and NIPR combined, and approximately 50 users in AD.

    - Centrally manage GCCS server hardware, operating systems and applications. Provide subject matter expertise in GCCS system administration and provide technical assistance to Client Support Technicians.

    - Provide routine application management for common GCCS applications. Maintain the application operating requirements and maintain configuration control information such as version and application POCs.

    - Operate and maintain SDIN and GCCS managed services IAW network IT policies and procedures, DoDI 8500.01. Support the management of SDIN and GCCS network equipment locally and at remote locations.

    - Load software updates/patches IAW MPTO 00-33A-1109.

    - Operate and maintain the replication and restoration capabilities for the SDIN and GCCS primary and backup locations.

    - Perform incremental backups on a daily basis and full back-ups on a weekly basis on all servers. The last weekly full backup will serve as the monthly and be kept for at least 6 months. Maintain backup information on-site IAW MPTO 00-33A-1001.

    - Maintain the RMF package for DoD-IT for the SDIN enclave (CDRL A505 - SDIN Risk Management Framework Plan). Provide RMF related inputs for the GCCS enclave to the PMO as required. RMF is a continuous process, package submittal will comply with accreditation authority guidelines to maintain authority to operate (ATO). Ensure the ATO remains current at all times. For the SDIN enclave submit an annual plan to answer the controls within Enterprise Mission Assurance Support Service (EMASS). Provide updates to the RMF process in the MAR (CDRL A503 – Monthly Activity Report) identifying the progress as outlined in the annual plan.

# CDRL A505 – SDIN Risk Management Framework Plan

    - Register and maintain SDIN circuits and trunks that require entry into the DISA Systems/Network Approval Process database or replacement/equivalent system that may be implemented.

# GCCS

    - Active Directory (AD) Management

Operate and maintain all GCCS managed AD servers and software IAW MPTO 00-33A-1001.

      - Perform GCCS AD administration activities as it pertains to their AOR.

# System Administration

      - Install, configure, operate and maintain the GCCS Enterprise for SpOC.

      - Perform Trusts and Domain Management, including establishing trusts with Headquarters SpOC GCCS for users on another domain.

      - Maintain SpOC GCCS network diagrams and as-built drawings. Inspect, update, correct discrepancies, and notify the Government as changes occur within one week of completion.

# User Training

Provide familiarization training to SpOC GCCS customers as required on any updates/changes to the current version/baseline of the GCCS workstation image. Identify the number of personnel trained in the MAR. (CDRL A503 – Monthly Activity Report)

# Network Security

      - Provide COMPUSEC support IAW MPTO 00-33B-5006 for all HW/SW associated with the GCCS Enterprise Network.

      - Assist government with identifying, correcting, and responding to vulnerabilities found through scans of GCCS devices on the network.

      - Manage all required password/identity management changes on the GCCS Enterprise Network.

      - Review all audit and system logs associated with GCCS on both UNIX and Windows platforms and interface with system administrators on vulnerability findings.

      - Interface with DISA and GCCS-AF Program Security Offices to maintain proper GCCS security level.

      - Manage the anti-virus protection for GCCS systems. Perform changes/upgrades/ patches for all managed AD services and software under their control. Perform two (2)-person control for all changes and document.

      - Perform two (2)-person compliance support for changes/upgrades/patches to core servers or infrastructure components and document.

      - Interface with the Designated Accrediting Authority (DAA) and AF GCCS PMO office documenting necessary changes and additions to GCCS RMF package.

# Configuration Management

      - Provide direct configuration management support for GCCS resources.

      - Store and maintain all Government GCCS software versions/patches for both local and remote SpOC GCCS sites.

      - Create copies of all GCCS client/server software required for all SpOC GCCS users.

      - Electronically track all GCCS client/server software distribution to all SpOC GCCS users, and the client server software inventory shall be available upon request by the COR.

      - Interface with DISA GCCS PMO and GCCS-AF Program Offices to coordinate delivery of all GCCS software upgrades as required.

      - Catalog and maintain electronic records of all Government provided, approved and tested GCCS application software, which will be available upon request by the COR.

      - Manage the SpOC Enterprise GCCS software test equipment per PMO direction.

      - Perform and record software testing processes for GCCS software (Windows/UNIX) applications and GCCS-AF PMO approved software additions and upgrades for both servers and clients. The testing processes on GCCS software applications shall be available upon request by the COR.

      - Electronically catalog all SpOC GCCS Enterprise servers and client configurations at time of install and update catalog monthly MAR. (CDRL A503 – Monthly Activity Report)

      - Ensure that all SpOC Enterprise GCCS servers and clients are in compliance with the appropriate GCCS software release.

# Software and Information Technology Equipment (ITE) Management

      - Maintain data on all SpOC Enterprise GCCS server and client hardware, and the HW/SW data shall be available upon request by the COR.

      - Monitor hardware warranty information for all GCCS resources and maintain software licenses (Windows/UNIX).

      - Maintain all local hand receipts for equipment issued to other sections and units. As required, transfer equipment from local ITE account to the gaining unit/organization account. Ensure that all local obsolete equipment is properly sanitized and prepared for disposal. Notify the GMT within one week of discovery of any missing equipment. (CDRL A401 – Equipment List and Inventory)

# Implementation Support

Provide GCCS implementation, coordination, upgrade and installation support within the Enterprise. Develop SpOC Enterprise GCCS technical solutions and implementation plans.

# SDIN

    - The SDIN Network Management Cell (NMC) supports functions for all SDIN networks (Black SDIN, Red SDIN, SONET). Centrally manage SDIN support at the NMC, to include; monitoring all circuits, networks and equipment; managing the help desk; provisioning circuits; starting and tracking trouble tickets; maintaining a station log; initiating troubleshooting to isolate and resolve problems; coordinating all troubleshooting and maintenance functions. Coordinate/perform required maintenance activities through appropriate POCs at the various sites where SDIN network equipment is located.

      - Coordinate issues/problems with the GMT when they can no longer proceed and/or government intervention is needed to resolve them.

      - Implement patch management IAW MPTO 00-33A-1109.

      - Participate in the planning, testing and execution of software, equipment, network management devices, application and database integration prior to deployment on the operational networks. This includes firmware and/or software updates to routers, switches, CTPs, servers and workstations as required due to End of Life equipment or DISA, STIGs/NOTAMs.

      - Review Project Support Agreement, implementation plans, circuit line records, and as- builts annually; and post engineering drawings to the central repository. Support installation, integration and testing of equipment upgrades as required. Document reviews and updates within one (1) week of upgrade completion.

      - Support all SDIN modifications and decommissioning of legacy equipment and services (e.g. annotate record changes, update, provision network, change equipment settings). Upon COR approval, resolve all customer channel requests within five (5) business days affecting hardware resources, system connectivity, and configuration issues associated with new

installations. Be the single focal point for network changes, modifications, downtime, and outages.

      - Provide troubleshooting capabilities at the NMC and coordinate maintenance activity support for SDIN, SONET, and STEEL PIPE.

      - Coordinate with appropriate providers and agencies during maintenance support for all SDIN circuits.

      - Support/review architecture studies related to SDIN. Reviews shall be performed within one (1) week of request by COR.

      - Produce, review and provide input to program documentation including AFTO Form 747- Cyberspace Infrastructure System Acceptance, DD Form 250 commissioning certificates, and accreditation packages in current DoD format IAW DoDI 8510.01 as required.

      - Submit ASI IAW Defense Information Systems Agency Circular (DISAC) 310-70-1. Investigate effects of ASIs submitted and report findings in writing to the SDIN GPM upon request. Execute all ASI notifications to the Government within four (4) hours.

      - Report SDIN availability (i.e. network availability, node isolations/outages that occurred during the month with dates and duration of isolation/outages and high-level comments such as ASI card replacement, unknown, etc.) as requested by the COR and in the MAR. (CDRL A503 – Monthly Activity Report)

      - Review, track, and coordinate all SDIN channel requests and obtain Government approval before provisioning new circuits and trunks.

      - Track all circuit actions and Telecommunication Service Request/ Telecommunication Service Orders electronically from DISA affecting the SDIN and SONET networks. Maintain an electronic circuit folder for each action.

      - Document all changes to the SDIN network systems configurations within ten (10) days after they occur to ensure network architecture documentation/drawings are up to date. The drawings shall be made available upon COR request.

      - Poll all nodes on a daily basis to confirm system status and analyze any trends that indicate degradation of equipment or data links to meet performance thresholds.

# SDIN Sustainment and Equipment Technical Refresh

        - Perform sustainment and coordinate technical refresh installations and equipment replacements of Red SDIN, Black SDIN and SDIN Optical Network equipment. The equipment includes, but is not limited to, network routers, switches, CTP devices, SONET and other optical network equipment. Perform site surveys, purchase of equipment IAW PWS paragraph 2.8, configuration of equipment, accreditation, shipment of equipment to sites, travel to sites, installation of equipment, testing of equipment, and cutovers & integration of the equipment into the SDIN networks.

        - This work will be incrementally funded under a separate Cost Reimbursable Technical Refresh CLIN as funds are available. This Cost Reimbursable CLIN will be used for equipment/material/software/warranty purchases, labor hours (other than the WICS-II direct labor), travel, installation and shipment to achieve an integrated turnkey SDIN operational system. All purchase recommendations must include appropriate certification under the Trade

Agreements Act (TAA) IAW DFARS 252.225-7020. Recommend to the SDIN Manager and the Chief COR (C-COR), the priority of equipment replacement also taking into account EOL/EOS of equipment, site outage/problem history, equipment failure rates and best use of available funds. The priority recommendation(s) will be given to the SDIN Manager, GPM, and C-COR, for validation, and the CO for approval. Once validated, by the SDIN Program Manager, GPM, and C-COR, and approved by the CO, implement the sustainment and technical refresh installs at those sites using the procedures in PWS paragraph 2.8. Table 10-1 – Sites and Networks show the sites and networks at each site and Table 10-2 - Current & Replacement Equipment Components show the old & replacement equipment components.

# Table 10-1 - Sites and Networks

| BASE | Red and Black OOB Connectivity | Red SDIN | Black SDIN | SONET |
| --- | --- | --- | --- | --- |
| ARC SBIRS BSFB, CO |  |  | X |  |
| AZUSA, Los Angeles CA |  | X | X | X |
| BSFB | X | X | X | X |
| Beale AFB, CA |  | X |  |  |
| Boulder |  | X | X |  |
| Cape Cod AFS, MA |  | X |  |  |
| Cavalier AFS, XX |  | X |  | X |
| Clear AFS, AK |  | X |  |  |
| CMSFS | X | X | X | X |
| Dahlgren NAS | X | X | X | X |
| Diego Garcia AS |  | X |  |  |
| Eglin AFB, FL |  | X |  |  |
| Greeley, CO |  |  | X | X |
| Kwajalein |  | X |  |  |
| Maui AS, HI |  | X |  |  |
| Millstone Lincoln Labs |  | X |  |  |
| Offutt AFB, NE | X | X | X | X |
| PSFB | X | X | X | X |
| RAF Flyingdales |  | X |  |  |
| SSFB | X | X | X | X |
| Site R, Blue Ridge Summit, PA |  | X |  |  |
| Socorro AS, NM |  | X |  |  |
| Sunnyvale SASC, CA |  | X | X | X |
| Telemetry and Orbital Test Station (TOTS), El Segundo, CA |  | X | X |  |
| Thule AB |  | X | X | X |
| Vandenberg SFB, CA | X | X | X | X |
| Wright Patterson AFB (WPAFB) , OH |  | X | X |  |

The replacement equipment components shown in Table 10-2 - Current & Replacement Equipment Components, are subject to change.

# Table 10-2 - Current & Replacement Equipment Components

| Current Equipment | Replacement Equipment |
| --- | --- |
| CTP 2024 Chassis | CTP 2024 Chassis |
| CTP 2056 Chassis | CTP 2056 Chassis |
| JUNIPER M7i Router | CISCO ISR 4351 |
| JUNIPER M10i Router | CISCO ISR 4451 |
| JUNIPER J6350 Router | CISCO ISR 4331 |
| JUNIPER EX2200 Switch | CISCO C3560CX-8TC |
| JUNIPER EX3200 Switch | CISCO 3560CX-12TC |
| JUNIPER EX3200 Switch | CISCO 3650-24TS |
| JUNIPER EX3200 Switch | 48PORT W/10G SSFB/CMSFS |
| CISCO 15310-CL Chassis | CISCO NCS4206-SA |
| CISCO 15310-MA Chassis | CISCO NCS4206-SA |
| CISCO 15454-SA-ANSI Chassis | CISCO NCS4216-SA Chassis |
| CISCO 15454-SA-HD Chassis | CISCO NCS4216-SA Chassis |

    - SDIN Preventive Maintenance and Inspection support

Provide preventive maintenance and inspection support to SDIN. This support shall include annual preventive maintenance inspections at all sites. Perform SDIN equipment inventory and verify equipment location while on site.

      - Troubleshoot RED and BLACK SDIN CTP, Router and Switch Networks, and SONET and other optical Network SDIN circuits, trunks and associated equipment. Provide 24/7 emergency troubleshooting/maintenance coverage for Front Range SDIN, Router/Switch Networks and SONET/Optical systems with two-hour response time from the time of notification to arrival at the site, if required. For emergency troubleshooting/maintenance coverage at BSFB, the response time is four (4)-hours.

      - Maintain RED and BLACK SDIN Networks, and SONET and other optical network mock-ups (lab) for integration testing, trouble shooting, etc.

        - The lab shall be located on SSFB to include crypto capabilities in the mockups. An area estimated to be able to house up to five (5) racks at a minimum of two (2) pieces of each type of equipment in racks. The area will be secure and climate controlled. The contractor will have 24/7 access to the lab.

      - Support the SDIN NMC and communications connectivity to every site 24/7.

      - Provide engineering support, integration and testing of SDIN interfaces with SpOC or DoD systems such as MILSTAR, Defense Information System Network, Integrated Tactical Warning/Attack Assessment, Space Control, Space Based Infrared Radar System, Ground-based Midcourse Defense, Jam-Resistant Secure Communications, etc. This includes testing and troubleshooting of circuits.

      - Perform architecture studies related to SDIN annually that includes engineering studies for the replacement, enhancement or upgrade of SDIN equipment, components or network

configurations. Provide program documentation for the aforementioned including trip reports. (CDRL A504 – Engineering Study)

      - Provide documentation and support to perform a change, modification, addition, removal or upgrade that directly affects SDIN equipment as required.

        - The following documentation may be required pending the type of support; engineering studies, technical solutions, meeting minutes, program support agreements/ configuration management packages, as-built and drawing packages, cost estimates, equipment lists, test plans, transfer of materials, trip reports IAW CDRL A506 - SDIN Network Modification

# CDRL A506 – SDIN Network Modification

    - Network Administration

      - Operate, maintain and administer all SDIN servers, virtual servers, management platforms and software.

      - Manage the authentication and setting of user privilege levels for all SDIN users through AD Services.

      - Manage and patch the management workstations and Windows based servers on the SDIN Network on an as required basis.

      - Scan the SDIN network for vulnerabilities monthly and report the top ten (10) vulnerabilities in the MAR (CDRL A503 – Monthly Activity Report)

      - Distribute and update anti-virus software on all SDIN workstations and servers using the host base security system.

      - Monitor network status, health, and maintain an up-to-date visual representation of the network.

      - Manage and monitor all encryption devices on the network from a central view. Upgrade these devices to be compliant with the National Security Agency and STIG requirements. Assist in Fire Fly key loading of these devices with local COMSEC personnel.

      - Configure, operate and manage the CTP’s and routed network devices on the network from either a central Graphical User Interface or Command Line Interface.

      - Configure, provision, operate and manage the SONET devices on the network from a central management platform.

      - Provide emergency troubleshooting/maintenance coverage for Network Administration with a two-hour response time in the Front Range and a four-hour response time to BSFB from notification to arrival on site to begin troubleshooting and remediation efforts.

      - Provide familiarization on networks and equipment interfaces to Government personnel to aid Government technicians in their duties as needed. This is not in-depth or qualification training but a familiarization for the serial CTP circuits and the SONET boxes as they have several types of connections to build a circuit.

# Information Assurance (IA) Operations

    - Provide services and solutions to implement and conduct IA operations IAW DoD Cyber Security RMF and DoDI 8500.01, AFI 17-201, Intelligence Community Directive 503 or the applicable current standards and guidance. This includes certification and accreditation activities. Provide applications services that are in compliance with and support DOD and USAF policies as applicable. Ensure that all infrastructure deliverables comply with the DISA STIG and Computer Network Defense.

      - Ensure un-patched systems do not gain access to the GCCS and SDIN Networks.

      - Provide the ISSO and the SDIN and GCCS-SpOC Information Assurance Officer necessary documentation to ensure system accreditation is maintained.

# IA Support

Provide IA support to ensure that the SDIN and GCCS systems meet all RMF requirements. Obtain required RMF package approval as required and ensure necessary remediation/controls are in place to retain the approval.

      - Prepare all IA information required to secure authority to operate for both SDIN and GCCS. Responsible for the entry of the IA information into the appropriate system (i.e. eMass, etc.) for approval by the SDIN Manager on a continual basis to keep the RMF package current.

# Security/Patch Configuration Administration

Ensure all patches are accomplished IAW DoDI 8500.01, and MPTO 00-33A-1001. Create, update and maintain login/logout scripts to check for patch compliancy daily.

# Compliance Tracking and Reporting

Comply with all applicable NOTAMs, Time Compliance Network Orders, Network Task Orders released by the Government within the specified timeframe. Compliance status and reporting shall be accomplished IAW MPTO 00-33A-1109.

# Chapter 11 Operational Systems Management

# Operational Systems Management

Provide systems management support for 4th Space Operations Squadron (4 SOPS) Mission Systems and associated subsystems, databases, software versions, interfaces, and interdependencies to ensure 4 SOPS maintains system awareness and strict accountability in support of government 24/7 user communications. These mission systems include, but are not limited to, MILSTAR/AEHF, DSCS/WGS, Enhanced Polar System (EPS) satellite systems, Mission Control Segment, and Command and Control System – Consolidated (CCS-C). Core hours of 0800 – 1500 hours Monday through Friday, excluding federal holidays. Maintain continuity binder with operating instructions and applicable procedures for operational systems management.

# Table 11-1 - Mission Systems Overview

| Mission System | Current Equipment/ Software | Projected New/Upgraded Equipment/Soft |
| --- | --- | --- |
| Space Segment | X |  |
| AEHF Satellite | X |  |
| MILSTAR Satellite | X |  |
| Evolved Strategic Satellite (ESS) |  | X |
| DSCS Satellite | X |  |
| WGS Satellite | X |  |
| EPS | X |  |
| EPS – R |  | X |
| Mission Control Segment | X |  |
| Advanced Satellite Mission Control Subsystem | X |  |
| Mission Operations | X |  |
| Mission Operations Data Capture | X |  |
| Mission Planning Element | X |  |
| Mission Planning Subsystem | X |  |
| Operations Support and Sustainment Element | X |  |
| 4 SOPS Test and Training Simulation Element | X |  |
| OSS Test and Training Simulation Element | X |  |

| Mission System | Current Equipment/ Software | Projected New/Upgraded Equipment/Soft |
| --- | --- | --- |
| Space Ground Link Subsystem | X |  |
| Command and Control Subsystem – Consolidated (CCS-C) | X |  |
| CCS-C Assurance and Capacity Enhancement |  | X |
| Enterprise Ground Services (EGS) |  |  |
| Terminal Software | X |  |
| Antenna Calibration Facility – Interim Command and Control | X |  |
| Family of Beyond-Line-of-Site Terminals (FAB-T) | X |  |
| Low Cost Terminal (LCT) |  | X |
| Key Translation Element (KTE) | X |  |
| KTE Mission Support Software | X |  |
| Central AEHF Key Management Station (CAKMS) | X |  |
| Protected SATCOM Key Management Architecture | X |  |
| Key Management Infrastructure | X |  |
| Boundary Protection Device | X |  |
| Virtual Private Network | X |  |
| Local Area Network Personal Computer (LAN PC) | X |  |
| Local Area Network Switch (LAN Switch) | X |  |

# External Agency Interaction

    - Systems management within 4 SOPS depends on interactions with outside agencies. Ensure all required configuration control and project management data, documents, and procedures are identified by interfacing both within 4 SOPS and with external entities. The list provided below is for reference and is subject to change:

      - SpOC

      - SBD 1

      - RSSCs

      - Air Force Materiel Command (AFMC)

      - MILSATCOM Joint Program Office

      - Space and Missile Systems Center/MILSTAR System Sustainment Office

      - Joint Terminal Program Office

      - -USSPACECOM

      - The contractors associated with the organizations listed above, and other government and non-government organizations, as needed.

# Installation Package for Coordination/Planning

    - Develop installation packages supporting updates to mission systems IAW government operating instructions and IAW CDRL A202 - Installation Package Brief for Coordination/Planning and submit packages for government approval prior to implementation.

# CDRL A202 – Installation Package Brief for Coordination/Planning

    - Schedule out-of-cycle releases, as needed, support configuration freezes in conjunction with launch and/or test activities.

# Deficiency Resolution Monitoring and Change Tracking

Monitor the progress of each system deficiency resolution from issue identification through closure by 4 SOPS maintenance officer or designated representative and track system changes.

- Compile and enter change request data into appropriate databases

- Provide assistance with prioritization of changes

- Provide complete coordination of the change requests

- Participate in the Change Approval Process

- Assist 4 SOPS in resolving conflicting system issues with development contractors

- Accomplish content audits prior to software change/release testing and installation

- Audit Version Description Documents and test report for all deliveries

- Capture changes in system baseline

# Meeting Support Report and track deficiencies/changes

Establish meetings, brief and develop meeting minutes in support of installation package coordination, planning, deficiency resolution monitoring and change tracking (CDRL A205 - Meeting Agenda and Meeting Minutes).

CDRL A205 – Meeting Agenda and Minutes

Chapter 12

Technical Control Facility (TCF) Operations

# Technical Control Facility (TCF) Operations

  - General Requirements: Operate and maintain Schriever SFB technical control systems in order to reduce the risk of outages and minimize mission impacts as outlined in this section. All contractors working on the TCF team shall have a minimum Top Secret (TS) security clearance. The contractor shall have sufficient personnel with a TS clearance with Sensitive Compartmented Information (SCI) eligibility to perform work requiring that clearance level, such as attending SCI level meetings and entering SCIFs where technical control remote systems are located while performing installation and/or maintenance functions.

    - Provide an efficient and integrated management approach for the operation of the Schriever SFB TCF. The contractor shall provide and update a Program Management Plan IAW Para 2.1.1. (CDRL A101 – Program Management Plan). The contractor shall support TCF operations 24/7 with at least two personnel on duty. The contractor shall maintain a master station log and brief incoming shift personnel and will develop and/or comply with all local SOPs.

# CDRL A101: Program Management Plan

      - Operate and maintain the TCF IAW DISA Circular 310-70-01, 21 Apr 2012 and associated DISA circulars, notices, and issuances; and AFMAN 17-2101, 22 May 2018 in order to avoid outages when possible and shorten the duration and impact of outages that do occur.

    - Interface with and support other contractors and government agencies in support of O&M pertinent to the contractor’s AOR as identified in the PWS scope. This includes providing administrator or equivalent level access to equipment or to personnel authorized by the government.

    - Provide inputs to government process improvement efforts, including providing analysis of operations within the contractor’s AOR, and answering data calls pertaining to the contractor’s AOR.

    - Respond IAW written and oral government directives to short notice taskings, prioritize work accordingly, and notify the government when complete.

    - All plans and documents required by this PWS shall be submitted for government review and acceptance prior to implementation.

    - The contractor shall attend meetings to perform the requirements of the PWS. Meetings include, but are not limited to SDIN Configuration Working Group, Modification Control Boards (MCBs), DISA Working Groups, etc. The contractor shall provide escorts as required for personnel performing work on their (organization’s) equipment suites in Bldg 400 Mods 3/4, 4B and 4B1A as well as Bldg 700, Rooms 10 and 15 (e.g. DISA, 4 SOPS, 22 SOPS, etc.).

    - Provide a MAR IAW CDRL A503 – Monthly Activity Report, and maintain the records IAW records management procedures in the work centers.

# CDRL A503 – Monthly Activity Report

    - Prepare, review, update, and deliver OIs/SOPs for the government work center where other manuals and directives are not available or do not provide sufficient instructions or guidance IAW CDRL A106 – Contract/Work Center OIs/SOPs.

# CDRL A106: Contract/Work Center OIs/SOPs

    - Support and review all Memorandums of Agreement (MOAs), Memorandums of Understanding (MOUs), Letters of Agreement (LOAs) and Service Level Agreements (SLAs), pertaining to Tech Control. The contractor shall provide input for coordinated changes and updates for MOAs, MOUs, LOAs and SLAs. All documentation shall be maintained by the contractor IAW records management procedures in the work centers and DoDD 5015.2, DoD 5015.2-STD, AFPD 33-3, AFI 33-322, and the Air Force Records Disposition Schedule located at https://www.my.af.mil/afrims/afrims/afrims/rims.cfm or their subsequent replacements. The contractor shall also maintain current copies of MOAs, MOUs, LOAs and SLAs pertaining to TCF support.

    - Maintain a POC list of customers and shall provide this list upon government request.

    - Report security problems within one hour and provide recommended courses of corrective action to address security problems in Tech Control to the government via email or phone within 24 hours of notification.

    - Operate NIPRNET/Unclassified computers, SIPRNET/Classified computers, unclassified phones, Defense Red Switch Network (DRSN) phones, universal secure phones e.g. vIPer ™ and Admin/Unclass phones to perform TCF functions. Utilize CIPS, Technical Control Operating Support System (TCOSS), Facility Circuit Information Tracking (FaCIT), SCN ESD Automated Resource Scheduler/Managed Intelligent Deconfliction Automation (MARS) SolarWinds, and DCC or their replacements as required to perform required functions.

    - The contractor shall conduct daily and/or impromptu briefings with the COR or other designated leadership regarding current status as required.

# Technical Control Functional Requirements

    - Outages and trouble calls:

    - The contractor shall receive trouble/outage calls from DISA, commercial vendors, other stations/facilities, and customers.

    - The contractor shall document station events, outages, troubleshooting efforts and coordination in a Master Station Log and in Outage tickets.

    - The contractor shall coordinate actions with DISA, commercial vendors, other stations/facilities, and customers as needed to coordinate/resolve issues.

    - The contractor shall respond to outages within 10 minutes, troubleshoot outages, and restore services IAW priorities established by circuit Telecommunications Service Priority or local guidance based on mission requirements.

    - The contractor shall operate a variety of test equipment including, but not limited to fiber optic test sets, oscilloscopes, bit error rate test equipment, multi-meters, cable testers, Ethernet testers, and Time-Division Multiplexing (TDM) test sets.

    - The contractor shall dispatch to other facilities on base including customer locations, commercial points of presence, and alternate facilities, which may require entering SCIFs to troubleshoot and resolve outages/issues within 30 minutes of notification/request.

    - The contractor shall log in to or access circuit equipment to verify settings and conduct testing.

    - The contractor shall utilize and update the provided electronic circuit history folders to reference Circuit Layout Records (CLR), TSOs, etc.

    - The contractor shall, within 3 days of identification, communicate CLR errors to Circuit Actions for correction.

    - The contractor shall follow local Emergency Action Plans.

    - The contractor shall follow local load shedding procedures and restoral plans.

    - The contractor shall up-channel priority and critical circuit and facility status changes within 10 minutes of identification.

# Space Control Network (SCN) Functions

    - The contractor shall act as Alternate SCN Network Communications Control (ANCC). The contractor shall utilize the SCN monitoring system (Solar Winds) to monitor the SCN for outages and investigate MIRs. The contractor shall utilize the DCC system if acting as the alternate ANCC. The contractor shall utilize the ESD/MARS SCN scheduling system or replacement to request maintenance windows on SCN resources.

# Technical Control Work Center Functions

    - The contractor shall conduct scheduled PMIs and document them in the Integrated Maintenance Data System (IMDS).

    - The contractor shall inventory tools and test equipment and document the inventories at the beginning of each shift.

    - The contractor shall conduct and document alarm checks at least once per shift.

    - The contractor shall comply with ESD practices.

    - The contractor shall check for ASIs from DISA twice per shift; process and coordinate ASIs within 1 hour of receipt.

    - The contractor shall remotely monitor DRSN alarms, respond and investigate within 10 minutes, and make notifications IAW local instructions within 10 minutes.

    - The contractor shall access and inventory Two Person Integrity (TPI) safe, adhering to TPI requirements.

    - The contractor shall review daily Raday packages to include MSLs and outage tickets.

    - The contractor will be responsible for a base supply CA/CRL equipment account.

# COMSEC Requirements:

    - The contractor shall appoint a primary and alternate CRO and comply with DoD, Air Force/Space Force, and local COMSEC requirements and training including AFMAN 17-1302-O, Incorporating Change 1, 21 Nov 22.

    - The contractor shall operate encryption devices and perform firmware updates on encryption devices as required.

    - The contractor shall coordinate periodic and as needed COMSEC loads, order necessary COMSEC and complete COMSEC records including inventories, destruction reports, and local coordination sheets.

Chapter 13

Other Support Requirements

# Other Support Requirements

# Security Support

    - Appoint a security representative to serve as the liaison between government and contractor personnel. Provide industrial security, information security, operations security (OPSEC), physical security, and resource protection at the locations identified in the PWS.

    - Immediately report any information or circumstances that may pose a threat to the security of DoD personnel, contractor personnel, resources, and classified or unclassified defense information. Report all incidents and violations to the unit security manager upon discovery.

    - Interface with host base programs IAW DD Form 254. Enter into a Visitor Group Security Agreement (VGSA) IAW AFI DODM 5220.22 V2_AFMAN 16-1406V2Obtain appropriate personnel security clearances for employees IAW DoD 5220.22-M.

# Reporting of Lost Badges

Upon discovery or notification, report the occurrence of lost badge(s) to the government Security Manager. If the security manager is not available, then report this action to 50th SFS.

# Retrieving Identification Media

Retrieve and return, to the unit government Security Manager, all identification media, including common access cards and restricted area badges from contract employees that depart for any reason before the contract expires or upon termination of the contract. Notify the COR, GPM and Security Manager within twenty-four (24)-hours when unable to retrieve identification media from contract employees.

# For Controlled Unclassified Information (CUI) Data Handling

CUI Data Handling information provided or generated under this contract will be safeguarded as specified in DoD 5400.11-R DoD Privacy Act Program; DoDI 5200.1 Volume 3 DoD Information Security Program: Protection of Classified Information SCI; DoD 5400.07 DoD Freedom of Information Act Program. Air Force publication that provides clarifying guidance concerning Department of Defense Instruction (DoDI) 5200.48, Controlled Unclassified Information

# Non-Disclosure Agreements (NDAs)

Execute NDAs with all applicable corporate, supplier and sub-tier vendors with proprietary, restricted, competition sensitive, or any other restricted (e.g., non-foreign disclosure due to public law) data that will be used or accessed during the duration of this contract. Provide copies of any NDAs to the GPM upon request.

# Industrial Security

Protect government information, property, equipment, controlled forms and resources which are generated, issued, or supplied under the terms of this PWS and IAW DODM 5220.22 V2_AFMAN 16-1406V2, and DoD 5220.22-M National Industrial Security Program Operating Manual (NISPOM), and all local directives. As stated in, when specified in an executed VGSA,

AFI 16-1404 Air Force Information Security Program security training requirements must satisfy the NISPOM training provision for on base integrated visitor groups IAW PWS paragraph 13.8.

# Safes

Provide safe maintenance, which involves administering the classified safes, inspecting their contents periodically, conducting combination changes as required, and conducting annual inspections with the exception of COMSEC safes. All safes that store COMSEC material will adhere to the rules of AFMAN 17-1302-O, paragraph 7.9.4. All combination changes shall be provided to the government.

# Information Security Program

Contractor employees shall undergo initial, continuing security education/refresher training, annual training and site-specific security training as provided by the OPR security manager as specified in the VGSA and AFI 16-1404. Record the training.

Security training will include industrial security, information security, OPSEC, physical security, and resource protection as specified in AFI 16-1404 and AFI 31-101 to include all applicable supplements. Training specific to the contractor work center should be conducted annually or on a more frequent basis if needed. Training methods may include group briefings, interactive videos, dissemination of instructional materials, CBT, or other media and methods.

# Information Protection

Administer the classified information program and ensure that all personnel authorized to view classified material receive the appropriate training and comply with AFI 16-1404. Ensure personnel complete the Standard Form (SF) 312.

# Personnel Clearance Requirements

The contractor is responsible for obtaining the appropriate level of security clearances for personnel requiring access to classified information. Requests for contractor security clearances shall be kept to an absolute minimum necessary to perform contract requirements.

# Operations Security

Follow the government established OPSEC program IAW AFI 10-701 and all applicable SpOC and base supplements, to include host base directives and guidance. Assist the squadron/site OPSEC coordinators to ensure compliance with host base program instructions.

# Physical Security

Protect and maintain a state of security awareness and safeguard against espionage, sabotage, damage, and theft IAW AFI 31-101 as supplemented and all local directives. Follow established escorted and unescorted entry procedures as identified in AFI 31-101, applicable supplements or host base directives.

    - Unescorted Entry to Restricted Areas: Limit requests for restricted area unescorted entry authority to employees whose duties require entry into such area(s) a minimum of 3 times a week. Coordinate with their (company) security manager to obtain a NACLC for employees who do not require a security clearance but need access to the restricted areas. Contractor

employees whose duties require access to classified information shall possess a minimum of an interim secret security clearance unless otherwise noted in this PWS.

# Escorted Entry into Restricted Areas

      - Provide personnel authorized to serve as restricted area escorts for their employees. Failure of the government to grant a clearance, revocation of a clearance, or revocation of base access privileges for any individual employed by not serve as justification for failure to perform or serve as the basis of any claim or request for equitable adjustment.

      - Required to provide escort duties for escort-required personnel that are on-site to provide maintenance, inspections, or surveys of equipment for which the contractor has responsibility. Provide escort duties for escort-required personnel responding to emergency utility outages in the facility that has a direct impact on equipment operations. For escort purposes only, the contractor has equipment responsibility for the Uninterruptible Power Supplies (UPS) in Bldgs 700 and 702.

# Entry Procedures

Secure work areas and restrict entry into facilities for the purpose of safeguarding government equipment and information. Pass information to security forces to apprehend or detain personnel with inadequate or suspicious identification. Contractor personnel shall not engage in the apprehension or detainment of unauthorized personnel.

# Resource Protection

Establish and manage a program to protect, conserve, and reduce loss, theft, and damage to government resources.

# Controlled and Restricted Areas

Comply with and enforce restricted area circulation control procedures and controlled area procedures IAW AF publications and local directives. Comply with issue and control of USAF restricted area badges, temporary badges, and visitor badges policies. At the end of each work period, secure all government facilities, equipment, and materials.

# Cipher/Lock Combinations

Control access to all areas of contractor responsibility. Change and control combinations IAW DoD 5200.1-R, paragraph C6.4.6.2. For COMSEC lock combinations reference AFMAN 17- 1302-O, paragraph 7.9.4. Provide all combinations to the government within 1 business day of the change.

# General Safety

The contractor and its subcontractors (if applicable) shall comply with Public Law 91-596 OSHA and the Environmental, Safety, and Occupational Health (DoDD 4715.1E). Cooperate with the SBD 1 Safety Program for accident/injury reporting and Government mishap investigations and comply with AFI 91-207 - The United States Air Force Traffic Safety Program and local Occupational and Environmental Safety, Fire Protection, and Health policies.

# Mishap Reporting

Immediately report mishaps involving government/contractor personnel injury or illness and government property/equipment damage resulting from contractor operations to the appropriate

government safety representative and the SBD 1/SE. Notify the GPM and CO of any mishaps no later than the next business day.

# Mishap Investigations

Contractor personnel shall participate in and cooperate with government mishap investigations.

# Safety Hazards

Notify the appropriate government safety representative and the SBD 1/SE of any safety hazards or mishaps associated with the system and/or facility.

# 13.14. Electro-Magnetic Frequency (EMF) Radiation

Perform all management responsibilities for contractor-operated EMF emitters IAW AFOSH Standard 48-9. This includes but is not limited to; maintaining data on all EMF emitters for which the contractor has operational control; conducting an EMF hazard-training program for personnel who are potentially exposed to EMF from contractor-operated sources and all management actions during a suspected overexposure. Report all electro-magnetic frequency radiation accidents or suspected over-exposures by phone to the Radiation Safety Officer (RSO) within 24 hours. The RSO is located at 21 AMDS/SGGB, Bldg 1246, 625 West Ent Avenue, PSFB CO, 80914-2840, Commercial 719-556-7721, DSN 834-7721, Fax ext. 8370. The after-

work hours POC is the Colorado Springs Regional Command Post, DSN 560-2181, 560-2182, 560-2183 or commercial 719-567-2181, 719-567-2182, or 719-567-2183. Keep a written report of the incident on file and available for government review.

# Facility Infrastructure Support

Provide facility, environmental and emergency management support. Contractor personnel assigned to this work activity must have a good working knowledge of facility infrastructure, emergency management, and environmental compliance requirements to support civil engineering efforts,

# Facility Requirements

Support all work areas by identifying repair, modification or new construction work requirements for all real property (RP) and real property installed equipment (RPIE) assigned to this contract. Use the Air Force definition of RP and RPIE in making a determination. Utilize the appropriate forms and processes required by the host installation to define repair or new work requirements. The contractor is responsible for defining work requirements and shall be determined by continuous observation of their work area and input from the government facility manager. The government will submit all identified work requirements for approval; the government will submit the requirement to the appropriate organization.

    - Environmental Management

Ensure all work centers are compliant with all environmental programs IAW applicable directives, executive orders, treaties, agreements, and host base requirements.

      - If the acquisition and storage of hazardous materials, hazardous and universal waste is required, identify personnel and complete required training. If a spill or incident occurs, perform the steps identified by the specific host base SSMA-AJ (SPCC) plan or other host base specifically identified procedures.

      - Manage and turn in any generated hazardous waste to the servicing host base IAW procedures defined by the host base or servicing agency.

      - Comply with host base environmental programs and participate in host base meetings as defined by the government, e.g., hazardous material management, hazardous waste management, affirmative procurement, energy conservation, solid waste management, recycling, environmental compliance programs, and the environmental protection committee (no more than once per month per work area).

      - Ensure host base environmental office is aware of work center and mission specific environmental compliance requirements above those defined by the host-base.

    - Energy Management Ensure all work areas are compliant with all energy management initiatives and host-based programs IAW applicable directives, executive orders, treaties, agreements, and host base requirements.

# General Housekeeping

Perform general housekeeping duties for all assigned work areas. Floors and workspaces shall be clean and clear of hazards.

# Chapter 14 GFE, GFP, GFI

  - Government-Furnished Equipment (GFE), Property (GFP) and Information (GFI)

The Government shall provide adequate facilities; utilities; official telephone service; systems and associated software and desktop computers to perform the services described in the PWS.

# GFE and GFP

Existing individual toolboxes, tools, and ground handling equipment shall be provided at contract start. Listings of inventories shall be provided as Government-furnished information (GFI) during the proposal process and shall be turned over to the Contractor after contract award to update and maintain as changes occur. When required items become unserviceable, they shall be replaced at Contractor’s expense. At that point, the Contractor purchased items become the property of the Contractor. Properly dispose of unserviceable items. Supplies and repairs shall be at the Contractor’s expense and shall not be reimbursed by the Government.

    - The Government shall retain ownership of specialized and mission-unique tools and equipment identified in inventories. The Contractor is not responsible for replacing such tools and equipment; however, maintain them using the CR CLIN. When any of the Government furnished items are determined to be beyond economical repair, at no fault of the contractor, coordinate with the COR for the return/disposal of the item.

    - Items such as appliances, furnishings, or other equipment shall be made available at the discretion of the Government and are not guaranteed to be provided through the life of the contract.

    - Verify and accept all Government furnished equipment required for the operation and maintenance (i.e. spare parts) of the SDIN and GCCS networks.

    - Any defective equipment shall be documented and submitted to the COR for concurrence within thirty (30) days of discovery of the defect. Advise and work with the COR to plan a cost- effective resolution of defective equipment. Identify deficiencies and may make recommendations regarding equipment, as appropriate.

    - Information Technology Assets (ITA). ITA is primarily computer related pieces of equipment and peripherals, to include network pieces of equipment (e.g. printers, scanners). Do not use Government furnished ITA or services for non-contractual related purposes. Provide a primary and alternate equipment custodian to manage an ITA account for equipment in their area of responsibility. Comply with all computer system and ITA accountability and security procedures required by the government. Government furnished ITA shall be disposed of through proper Space Force channels.

Upon completion of the contract, all remaining Government furnished items shall be returned to the Government.

# GFI

Maintain and update government and contractor information.

Chapter 15

WICS-II Service Summary Items

# WICS-II Service Summary Items

The Services Summary (SS) is a list of performance objectives and performance thresholds that shall be verified by Government personnel IAW the Quality Assurance Surveillance Plan and will result in a major finding if the performance threshold is not met. Each performance objective represents a significant task as identified in the PWS by the Government at the time of contract award. The performance threshold represents the minimum acceptable level of performance. The performance objectives and performance thresholds represent the most significant tasks of this contract.

# Performance Objectives/Thresholds

Performance objectives may be added or deleted, and performance thresholds raised or lowered during the course of this contract. For example, repeated contractor performance deficiencies in non-critical areas may force the related requirement to become a performance objective in the

SS. Conversely, changes in technology or mission requirements may eliminate SS objectives.

| Item # | Objective | PWS Ref. | Threshold |
| --- | --- | --- | --- |
| SS-1 | Provide 100% task coverage for all positions, as defined in the PWS. | 2.2, 2.3, 2.4, 2.6 | Within required time frames, during normal duty hours and all contingency scenarios provide 100% task coverage for all positions. |
| SS-2 | Maintain threshold and performance objectives IAW the Joint Operational Requirements Document-MILSTAR Communications System. | 4.17.2.2 | Maintain a monthly .9957 Operational Dependability (DO) rate. |
| SS-3 | Respond to service request calls from GBS fielded RS end users within 15 minutes. | 3.3.2 | Zero responses exceed 15 minutes. |
| SS-4 | Report GBS health and status information within the GBSOC area of responsibility through the proper chain of command as determined by GBSOC C2 relationships and reporting procedures established in IAW GBSOC OIs and associated reporting guidance. This includes but is not limited to the GEM, GAM, OPSCAP reporting, and CCIR reporting. | 3.4.8 | Report IAW established procedures 98% of the time. |
| SS-5 | Respond to and initiate corrective action to incidents, problems, and outages affecting GBS services within 5 minutes. | 3.4.3 | Responses shall be within 5 minutes 100% of the time. |
| SS-6 | Maintain a GBSOC controllable SBM operational availability (Ao) threshold of 99.00%. | 3.1 | Maintain a GBSOC controllable SBM operational availability (Ao) threshold of 99.00%. |

| Item # | Objective | PWS Ref. | Threshold |
| --- | --- | --- | --- |
| SS-7 | Deliver CDRL A202 – Installation Package Brief for Coordination/Planning | 11.2.1 | Install briefing presentation packages must be delivered at least six (6) business days before scheduled software installation. Exceptions will be considered on a case-by-case basis. |
| SS-8 | Reserved |  |  |
| SS-9 | Submit install data within 2 calendar days before each 4 SOPS meeting | 11.2.1 | Data of software installation must be submitted two (2) days before the appropriate 4 SOPS acceptance meeting. |
| SS-10 | Report any equipment or system outages to the MOC within 15 minutes, update the MOC every two hours during extended unscheduled outages, and notify the MOC once the action is completed. | 4.2.1 | Report any maintenance outages to the MOC within 15 minutes, update the MOC every 2 hours during extended unscheduled outages, and notify the MOC once action is complete 100% of the time. |
| SS-11 | Initiate organizational level corrective maintenance and respond to outages impacting mission within 5 minutes. (Except GBS SBM MUE on-call support) | 4.3.1 | Responses shall be within 5 minutes 100% of the time. |
| SS-12 | For problems affecting GBS SBM, the on-call personnel shall initiate corrective maintenance within two (2) hours after a confirmed notification. | 4.16 | The on-call personnel shall initiate corrective maintenance within two (2) hours after a confirmed notification 100% of the time. |
| SS-13 | COMSEC, Crypto and Safes | 4.13.1 4.13.2 10.11.3.7 12.5.1 13.7 | Zero Reportable incidents that result in an investigation or inquiry IAW AFMAN 1302-O para 9.3.1.3 |
| SS-14 | Document contributing factors, document lessons learned within the MIR process, trend analysis and root cause analysis in the MPR and MIRTA database | 4.26.1.8 | 85% of all MIRS submitted need to have the listed information documented in the MIRTA database within one duty day of MIR submission. |
| SS-15 | Industrial and Physical Security | 13.6 13.7 13.8 all 13.10 13.11 all | Zero security incidents. No compromise/loss of classified information. |
| SS-16 | Resource Protection | 13.12 all | No more than 1 incident per month not to exceed 5 incidents per year as identified by the investigative process. |

| Item # | Objective | PWS Ref. | Threshold |
| --- | --- | --- | --- |
| SS-17 | Environmental Compliance | 13.15.2 all | Zero notice of violations or fines from the EPA or other regulatory agencies. |
| SS-18 | Respond to all SDIN and GCCS mission systems network outages within fifteen (15) minutes | 10.1.1 10.1.2 | 95% |
| SS-19 | Resolve customer channel requests affecting hardware resources, system connectivity, and configuration issues associated with new installations within five (5) business days | 10.11.1.5 | 95 % |
| SS-20 | All ASI notifications to the Government must be executed within four (4) hours | 10.11.1.10 | 100% |
| SS-21 | All document reviews and updates shall be performed within one (1) week of upgrade completion | 10.11.1.4 | 90% |
| SS-22 | Provide 24/7 support and on-call support | 3.0 4.0 4.15.1, 4.17.1.1 4.17.2.1 4.26 9.2 9.3 10.1.1 10.1.2 10.2.1 10.8.7, 10.11.2.1 10.11.2.2.1 10.11.2.3 11.0 12.1.1 | 100% |
| SS-23 | Personnel are DoD 8570.01-M certified to the appropriate level | 2.10.1.2.1 2.10.1.2.2 2.10.1.2.4 | 100% |
| SS-24 | Respond to outages within 10 minutes, troubleshoot outages, and restore services IAW priorities established by circuit Telecommunications Service Priority of local guidance based on mission requirements. | 12.2.5 | 95% |
| SS-25 | Dispatch to other facilities on base including customer locations, commercial points of presence, and alternate facilities, which may require entering SCIFs to troubleshoot and resolve outages/issues within 30 minutes of notification/request. | 12.2.7 | 95% |

| Item # | Objective | PWS Ref. | Threshold |
| --- | --- | --- | --- |
| SS-26 | Conduct all scheduled TCF PMIs and document within IMDS within 30 days of overdue | 12.4.1 | 95% |
| SS-27 | Remotely monitor DRSN alarms, respond and investigate within 10 minutes, and make notifications IAW local instructions within 10 minutes. | 12.4.6 | 95% |

# Appendix A

# WICS-II CDRL Requirements

| CDRL | Title | PWS Paragraph |
| --- | --- | --- |
| A101 | Program Management Plan | 2.1.1, 12.1.1 |
| A102 | Mission Essential Contractor Services Plan | 2.3 |
| A103 | Quality Control Plan (QCP) | 2.9 |
| A105 | Program Management Review | 2.11.3.1 |
| A106 | Contract/Work Center Operating Instructions (OIs) & Standard Operating Procedures (SOPs) | 2.11.5.,12.1.8 |
| A108 | Master Document List (MDL) / Bidder’s Library (BL) | 2.11.5.3 |
| A110 | Phase-In Plan | 2.12.1 |
| A111 | Phase-Out Plan | 2.12.2 |
| A201 | GBS Monthly Status Report | 3.4.25 |
| A202 | Installation Package Brief for Coordination/Planning | 11.2.1 |
| Reserved | (previously CDRL A203 System Version Tracker) |  |
| Reserved | (previously CDRL A204 Meeting Agenda) |  |
| A205 | Meeting Agenda and Minutes | 11.4 |
| Reserved | (previously CDRL A206 Update to Mission Systems) |  |
| A301 | Regression Test Plan | 4.24.2.1 |
| A302 | Regression Test Report | 4.24.2.2 |
| A401 | Equipment List and Inventory | 5.1.2.7.1, 5.1.2.7.2, 5.1.2.8.1, 10.8.5, 10.10.6.3 |
| A503 | Monthly Activity Report (MAR) | 10.3.2, 10.6.1, 10.8.2, 10.8.3, 10.9.9, 10.10.3, 10.10.5.9, 10.11.1.11, 10.11.3.4, 12.1.7 |
| A504 | Engineering Study | 10.6.2, 10.11.2.5 |
| A505 | SDIN Risk Management Framework Plan | 10.9.9 |
| A506 | SDIN Network Modification | 10.11.2.6.1 |
| Reserved | (previously CDRL A601 MIR Status Report) |  |
| A701 | Systems Safety Assessment Report (SSAR) | 8.2.1.4 |

| Appendix B Acronyms |  |
| --- | --- |
| Acronym | Definition |
| 22 SOPS | 22nd Space Operations Squadron |
| 24/7 | 24 hours a day, 7 days a week |
| 4 SOPS | 4th Space Operations Squadron |
| 50 CES | 50 Civil Engineering Squadron |
| Reserved |  |
| 50 CS | 50th Communications Squadron |
| SBD 1 | Peterson-Schriever Garrison |
| SBD 1/PMD | SBD 1/ Program Management Directorate |
| SBD 1/SE | SBD 1/ Safety Office |
| SBD 1I | SBD 1 Instruction |
| ACAS | Assured Compliance Assessment Solution |
| ACF-IC2 | AEHF Calibration Facility Interim Command and Control |
| AD | Active Directory |
| AEHF | Advanced Extremely High Frequency |
| AF | Air Force |
| AFEMS | Air Force Equipment Management System |
| AFGLSC | Air Force Global Logistics Support Center |
| AFH | Air Force Handbook |
| AFI | Air Force Instruction |
| AFIN | Air Force Information Network |
| AFKAG | Air Force COMSEC General Publications |
| AFMAN | Air Force Manual |
| AFMC | Air Force Material Command |
| AFMETCAL | Air Force Metrology and Calibration |
| AFNET | Air Force Network |
| Reserved |  |
| AFOSH | Air Force Occupational Safety and Health |
| AFPD | Air Force Policy Directive |
| AFRIMS | Air Force Records Information Management System |
| Reserved |  |
| Reserved |  |
| AFSPCI | Air Force Space Command Instruction |
| AFSSI | Air Force System Security Instruction |
| AFTO | Air Force Technical Order |
| AIM | Asset Inventory Management |
| AJ | Anti-Jam |
| AMAC | Air Force Information Network Mission Assurance Center |
| ANSI | American National Safety Institute |
| ANCC | Alternate SCN Network Communications Control |
| AOR | Area of Responsibility |

| AS | Anti-Scintillation / Air Station |
| --- | --- |
| ASI | Authorized Service Interruption |
| ASL | Adjusted Stock Levels |
| ATO | Authority to Operate |
| BAFB | Buckley Air Force Base |
| Reserved |  |
| BL | Bidders’ Library |
| C2 | Command and Control |
| C4 | Command, Control, Communication, and Computers |
| CA | Custodian Authorization / California |
| CAKMS | Central AEHF Key Management Station |
| CBT | Computer Based Training |
| CC | Commander |
| CCIR | Commander’s Critical Information Requirements |
| CCMD | Combatant Command |
| CCS-C | Command and Control System - Consolidated |
| CDC | Core Data Center |
| CDRL | Contract Data Requirements List |
| CIPS | Cyberspace Infrastructure Planning System |
| CLIN | Contract Line Item Number |
| CM | Configuration Management |
| CMAFS | Cheyenne Mountain Air Force Station |
| CO | Contracting Officer / Colorado |
| CoM | Chief of Maintenance |
| COMPUSEC | Computer Security |
| COMSEC | Communications Security |
| COOP | Continuity of Operations Plan |
| COR | Contracting Officer Representative |
| CPM | Contractor Program Manager |
| CRF | Change Request Form |
| CRL | Custodian Receipt Listing |
| CRO | COMSEC Responsible Officer |
| Reserved |  |
| CTP | Circuit-to-Packet |
| CUI | Controlled Unclassified Information |
| DAA | Designated Approving Authority / Designated Accrediting Authority |
| DBCR | Database Change Request |
| DFAR | Defense Federal Acquisition Regulation |
| DISA | Defense Information System Agency |
| DISAC | DISA Circular |
| DLM | Depot Level Maintenance |
| DoD | Department of Defense |
| DoDD | Department of Defense Directive |
| DoDAAC | Department of Defense Activity Address Code |
| DoDI | Department of Defense Instruction |

| DRSN | Defense Red Switch Network |
| --- | --- |
| DSCS | Defense Satellite Communications System |
| DSN | Defense Switch Network |
| Reserved |  |
| ECO | Engineering Change Order / Equipment Control Office |
| EIL | Equipment Inventory List |
| EITDR | Enterprise Information Technology Data Repository |
| EMASS | Enterprise Mission Assurance Support Service |
| EMF | Electromagnetic Frequency |
| EMI | Electromagnetic Interference |
| EMSEC | Emissions Security |
| EOL/EOS | End of Life / End of Service |
| EPS | Enhanced Polar System |
| ESD | Electronic Scheduling Dissemination system |
| ES-S | Enterprise Solution Supply |
| Reserved |  |
| FAB-T | Family of Beyond Line of Sight Terminals |
| FaCIT | Facility Circuit Information Tracking |
| FAR | Federal Acquisition Regulation |
| FY | Fiscal Year |
| Reserved |  |
| GBS | Global Broadcast Service |
| GBSOC | GBS Operations Center |
| GBSOC AOR | GBS Operations Center Area of Responsibility |
| GCCS | Global Command and Control System |
| GEM | GBS Event Message |
| GFE | Government Furnished Equipment |
| GFP | Government Furnished Property |
| GMT | Government Management Team |
| GPM | Government Program Manager |
| GSA | General Services Administration |
| GSU | Geographically Separated Unit |
| Reserved |  |
| HQ | Headquarters |
| HEMP | High-Altitude Electromagnetic Pulse |
| HW / SW | Hardware / Software |
| I&C | Installation and Checkout |
| IA | Information Assurance |
| Reserved |  |
| IAVM | Information Assurance Vulnerability Management |
| IAW | In Accordance With |
| ID | Identification |
| INMS | Integrated Network Management System |
| ISSM | Information System Security Manager |
| ISSO | Information Systems Security Officer |

| IT | Information Technology |
| --- | --- |
| ITA | Information Technology Assets |
| ITE | Information Technology Equipment |
| ITEC | Information Technology Equipment Custodian |
| IUID | Item Unique Identification |
| JCN | Job Control Number |
| Reserved |  |
| KTE | Key Translation Element |
| LAN | Local Area Network |
| LRU | Line-Replacement Unit |
| Reserved |  |
| MA | Maintenance Assistance / Massachusetts |
| MAJCOM | Major Command |
| MAR | Monthly Activity Report |
| MASS | MILSTAR Automated Support Scheduler |
| MCB | Modification Control Board |
| MDA | Missile Defense Agency |
| MDL | Master Document Listing |
| MECH | Mechanicsburg, PA |
| MICAP | Mission Capable |
| MICT | Management Internal Control Toolset |
| MILSATCOM | Military Satellite Communications |
| MIR | Mission Impact Report |
| MIRTA | Mission Impact Report Tracking and Analysis |
| MOA | Memorandum of Agreement |
| MOC | Maintenance Operations Center |
| MOU | Memorandum of Understanding |
| MPR | Mission Problem Report |
| MPTO | Methods and Procedures Technical Order |
| Reserved |  |
| MSL | Master Station Log |
| MUE | Mission Unique Equipment |
| NACLC | National Agency Check with Local Agency and Credit Check |
| NIPRNet | Non-Classified Internet Protocol Router Network |
| NISPOM | National Industrial Security Program Operating Manual |
| NMC | Network Management Cell |
| NORAD | North American Aerospace Defense Command |
| Reserved |  |
| OA | Operational Availability |
| OCN | Outage Control Number |
| OI | Operating Instruction |
| OKC | Oklahoma City, OK |
| OPSCAP | Operations Capability |
| OPSEC | Operations Security |
| ORB | Operations Review Board |

| ORP | Operations Review Panel |
| --- | --- |
| OSHA | Occupational Safety and Health Administration |
| Reserved |  |
| PAAWNS | Protected Anti-Scintillation Anti-Jam Wideband Net-Centric System |
| PAC | Participating Associate Contractor |
| PAFB | Peterson Air Force Base |
| PoP | Period of Performance |
| PM | Program Manager |
| PMD | Program Management Directorate |
| PMEL | Precision Measurement Equipment Laboratory |
| PMI | Preventive Maintenance Inspection |
| PMO | Program Management Office |
| PMR | Program Management Review |
| POC | Point of Contact |
| PWS | Performance Work Statement |
| Reserved |  |
| QCP | Quality Control Plan |
| RBM | Receive Broadcast Manager |
| RC | Records Custodian |
| RMF | Risk Management Framework |
| ROADS | Remote Tracking Station Online Activity Database System |
| RP | Real Property |
| RPIE | Real Property Installed Equipment |
| RS | Receive Suite |
| RSO | Radiation Safety Officer |
| RSSCs | Regional SATCOM Support Centers |
| RTS | Remote Tracking Station |
| Reserved |  |
| SAFB | Schriever Air Force Base |
| SATCOM | Satellite Communications |
| SBM | Satellite Broadcast Manager |
| SBSS | Space Based Space Surveillance / Standard Base Supply System |
| SCI | Sensitive Compartmented Information |
| SCN | Satellite Control Network |
| SF | Standard Form |
| SIPR | Secret Internet Protocol Router |
| SIPRNet | Secret Internet Protocol Router Network |
| SLA | Service Level Agreement |
| SMR | Source, Maintenance, and Recoverability |
| SNAP | System for Network Analysis and Planning |
| SONET | Synchronous Optical Network |
| SOP | Standard Operating Procedure |
| SOPS | Space Operations Squadron |
| SpOC | Space Operations Command |
| SS | Services Summary |

| SSAR | System Safety Assessment Report |
| --- | --- |
| SVT | System Version Tracker |
| SW | Space Wing / Software |
| SWI | Space Wing Instruction |
| TAR | Travel Authorization Request |
| Reserved |  |
| TCF | Technical Control Facility |
| TCOSS | Technical Control Operating Support System |
| TIM | Theater Information Manager |
| TM | Technical Manual |
| TMDE | Test, Measurement, and Diagnostic Equipment |
| TO | Technical Order |
| TOOP | Transfer of Operations |
| TPI | Two Person Integrity |
| TS | Top Secret |
| TSCO | Team SAFB Cybersecurity Office |
| Reserved |  |
| UDLM | Urgent Depot Level Maintenance |
| UNIX | Uniplexed Information and Computing Information |
| UPS | Uninterruptible Power Supply |
| USAF | United States Air Force |
| USAFA | United States Air Force Academy |
| USB | Universal Serial Bus |
| USSF | United States Space Force |
| US NORTHCOM | United States Northern Command |
| USSPACECOM | United States Space Command |
| USSTRATCOM | United States Strategic Command |
| VAFB | Vandenberg Air Force Base |
| Reserved |  |
| VGSA | Visitors Group Security Agreement |
| WGS | Wideband Global SATCOM |
| WICS II | Wing Information and Communications Support II |

Appendix C Definitions

Acceptance: Indicates a facility, system, or service meets technical and performance standards as required by the government.

Accreditation: DAA authorization to place an automated system into operation.

AFKAG: A short title used on general operational AF COMSEC publications. Publications are controlled in COMSEC channels.

Ancillary Training: Supplementary training necessary to accomplish contract requirements (e.g. fire extinguisher training, first aid buddy care, Cardiopulmonary Resuscitation, etc.)

Annual: A period covering 12 continuous months.

Antenna: A conductor or system of conductors for radiating or receiving radio frequency signals.

Approval to Operate: Concurrence by the DAA that minimum-security requirements are met and there is an acceptable level of risk to the network.

# Back-up:

- A system, component, file, procedure, or person available to replace or help restore a primary item in the event of a failure or externally caused disaster.

- To create or designate a system, component, file, procedure, or person as in “a” above.

Base Equipment Control Officer: Individual responsible for safekeeping and inventory accountability of all assigned Information Technology Equipment.

# Baseline:

- A specification or product that has been formally reviewed and agreed upon, that thereafter serves as the basis for further development, and that can be changed only through formal change control procedures.

- A document or series of documents formally designated and fixed at a specific time during the life cycle of a configuration item. Baselines constitute the current configuration identification.

Bidder’s Library: A library which contains all documents listed in this PWS and required for performance of the contract.

Certification: Affirmation based upon documentation and evaluation, that an individual has satisfactorily fulfilled the requirements for performance in a given position/system. Certification is required to perform operational tasks.

Change Request Form (CRF): A CRF provides a systematic way to report known or suspected problems or deficiencies or new requirement with weapons systems and equipment, to investigate these problems, to determine what to do about them, and to ensure completion of agreed upon corrective actions. A CRF applies to weapon systems, equipment, information systems, hardware and software in advanced development, engineering development, testing and evaluation, or operational use. In addition, a CRF applies to government furnished equipment or material provided to contractors and used as an integral portion of the system.

Circuit: A means of two-way communication between two points comprised of associated “send and receive” channels.

Command, Control, Communications, Computer (C4) System: The C4 system is an integrated system of doctrine, procedures, organizational structures, personnel, equipment, facilities, and communications designed to support a commander through all operational phases. It includes base visual information support systems.

Communications-Electronics Equipment: For purposes of this acquisition, communications- electronics consists of systems control, wide-band, secure communications (cryptographic), power production, and associated equipment.

# Communications Security (COMSEC):

- Measures and controls taken to deny unauthorized persons information derived from telecommunications and to ensure the authenticity of such telecommunications. Communications security includes crypto security, transmission security, emission security, and physical security of communications security material.

- The protection resulting from all measures designed to deny unauthorized persons information of value that might be derived from the possession and study of telecommunications, or to mislead unauthorized persons in their interpretation of the results of such possession and study.

Computer Security (COMPUSEC): The protection resulting from all measures designed to prevent deliberate or inadvertent unauthorized disclosure, acquisition, manipulation, modification, or loss of information contained in a computer system, as well as measures designed to prevent denial of authorized use of the system.

COMSEC Responsible Officer (CRO): The individual within an office or area responsible for COMSEC material received from the CMCS.

Configuration: The arrangement of a computer system or network as defined by the nature, number, and chief characteristics of its functional units. More specifically, the term configuration may refer to a hardware or software configuration; the devices and programs that make up a system, subsystem, or network.

Configuration Control: Process of controlling modifications to hardware, firmware, software, and documentation to ensure the communications and information system is protected against improper modifications.

Configuration Control Board: A board held to review, discuss and approve communications requirements.

Configuration Management (CM): A discipline applying technical and administrative direction and surveillance to: identify and document the functional and physical characteristics of a configuration item, control changes to those characteristics, record and report change processing and implementation status, and verify compliance with specified requirements.

Continuity of Operations Plan (COOP): A contingency plan implemented when the primary GBSOC can no longer function as an operational center and must transfer operations to Hill Air Force Base, UT or the designated COOP site. Once the contractor arrives at the COOP site, establishes SIPR, NIPR connectivity on the GBSOC computers on site and cuts over the

communications from SSFB to the new temporary GBS operations center, then the COOP process is halfway complete. Once the GBSOC redeploys back to the primary site once communications have been restored then the COOP process is complete.

Contracting Officer (CO): The government agent awarding or administering contracts. The Contracting Officer is the only person authorized to contractually obligate the government.

Contracting Officer Representative (COR): Individual who monitors a contractor on a daily basis and who is involved in every aspect of a contract to ensure the contractor is in compliance with that contract. (See entry for Quality Assurance)

Contractor: The individual or company responsible for performing the duties and responsibilities of the PWS.

Controlled Area: Any building, area, or structure containing USAF/USSF resources which are lucrative targets for thefts, compromise, or destruction and to which entry must be limited in order to provide more protection.

Corrective Maintenance: A maintenance term meaning the running of diagnostics, system restoral procedures, and other troubleshooting to restore a system back to an operational condition. Corrective maintenance may include problem resolution if discovered during the restore to operations procedures.

Critical: The degree of impact that a requirement, module, error, fault, failure, or other item has on the development or operation of a system.

Critical Information: Specific facts about friendly intentions, capabilities, or activities vitally needed by adversaries for them to plan and act effectively to guarantee failure or unacceptable consequences for friendly mission accomplishment.

Database Change Request (DBCR): A DBCR is written to document any change to a database.

Database Maintenance (Organizational/Level 1): Organizational Level Database Application Maintenance includes: requirements gathering and analysis; database requirements and change tracking; design, development, and test of changes to database parameters; regression testing and installation testing; assistance for anomaly/problem analysis and resolution; trending of anomalies/problems and their resolution; system performance analysis; file compares to ensure continued conformance to the release configuration for all operational and testing environments; archiving and file conversions; meetings requiring technical assistance for database and its interaction with software, hardware, and operations procedures.

Defense Switched Network (DSN): The worldwide inter-base telecommunications system that provides end-to-end, common-user, and dedicated voice service for the Department of Defense with the capability of incorporating data and other traffic. It replaced the Automatic Voice Network as the principal long haul, non-secure voice communications network within the Defense Communications System.

Designated Approving Authority (DAA): Official with the authority to formally assume responsibility for operating a system at an acceptable level of risk. This term is synonymous with Designated Accrediting Authority, Delegated Accrediting Authority, and Communications and Information Systems Security Manager.

Directive Publication: A directive publication is one, which requires mandatory compliance by the contractor.

Documentation: Refers to, but not be limited to, the following: service notes, subscription services, schematics, engineering sketches, engineering drawings, as built drawings, maps, layouts, computer program software listings, magnetic media cards, Communications-Computer Systems Installation Records (CSIR), memos, correspondence, listings, letters, Security Plan, Security Test and Evaluations, Risk Analysis, technical reports, inspection reports, preventive maintenance instructions, logs and all other file material required in performance of the contract.

Documents: Government regulations, manuals, pamphlets, and technical orders, commercial manuals, preventive maintenance instructions , preventive maintenance worksheets, OI, correspondence files, service notes, subscription services, schematics, engineering sketches and drawings, as-built drawings, maps, layouts, technical RPIE manuals, RPIE system manuals, computer program documentation, software programs and maintenance diagnostics, users manuals, positional handbooks, Communications-Computer Systems

# Downtime:

- The period of time during which a system or component is not operational or has been taken out of service.

- The amount of time necessary to perform a database or software installation. Downtime includes buffer for troubleshooting and/or de-installation.

Emissions Security (EMSEC): Refers to the investigation, study, and control of compromising signals radiating from telecommunication and automated information systems equipment.

EMSEC has changed to TEMPEST.

Enterprise Information Technology Data Repository (EITDR): The Air Force database of record for registering all systems and applications as required by public law and DoD directives. Registration in the EITDR is mandatory for all systems and applications developed by the Air Force, or for which the Air Force is the lead agency, or that requires connection to the AF- Global Information Grid. The EITDR is also the database of record for IT statutory and regulatory compliance. The repository contains compliance data for IA, Internet Protocol version 6 (IPv6), Public Key Enabling (PKE), Clinger-Cohen Act, etc. It is the primary data source for Federal Information Security Management Act reporting and the principal vehicle for gathering and storing system and application data to support planned and ad hoc data calls. The EITDR contains information about program management; system and application interfaces; net worthiness; funding; Capital Investment Reports (CIR); and other supporting data to facilitate IT portfolio management.

Equipment: An all-inclusive term which refers to the hardware device(s) individually and/or collectively of communication and computer systems.

Estimated Time Restored Operations: Estimated time to return failed equipment to the previous operational condition.

Exercise: Activities practiced increasing readiness skills or preparing for an upcoming event. These may include evacuation of all personnel, simulated anomalies, simulated satellite reconfiguration, increased security, simulated natural or man-made disasters, up channel reporting, simulated injuries, and simulated implementation of operational considerations.

Fielded: Pertaining to GBS, if RS equipment have been fielded then that means that it has been sourced and approved by the GBS program office. However, in retrospect, if a customer acquired off the shelf RS equipment that was not GBS program office sourced or approved then it is not fielded.”

Fiscal Year: a period covering 01 Oct through 30 Sep

Forward Supply Point: Provides individual work centers with storage location for priority replacement parts; used to preclude extended delays in restoring critical mission support systems to operations.

Freeze Schedule: A schedule produced depicting required mission asset availability. These assets require approval prior to performing any work that may affect or degrade the mission system prior to or during the launch window. This schedule is used at the squadron level to prevent mission impact, provide situational awareness and to schedule maintenance.

Functional System Administration for the GBSOC NIPRNet and SIPRNet workstations is limited changing workstation hardware and software configuration to restore workstation mission essential function services, troubleshooting workstation mission impacting anomalies, and assisting with installation of GBSOC workstations software that is not part of the standard desktop software load.

Geographically Separated Unit (GSU): 50th Space Wing locations/assets not located on SSFB.

Global Broadcast Service (GBS): GBS is a satellite communication based, world-wide, high- capacity broadcast system that provides a one-way, high-speed flow of high-volume national security information to forces garrisoned, deployed, or on the move with tactical GBS satellite RS. GBS provides a content priority-based delivery of classified and unclassified video and data products. GBS is a DISA CDC-centric architecture providing centralized SBM system for continuous operations, management, and control of DISA dedicated assets assigned to GBS and GBS MUE. The GBSOC is the centralized service center that provides 24/7 broadcast and event management, troubleshooting, coordination, reporting, and overall status situational awareness of GBS system and services. Additionally, the GBSOC provides GBS RS end-user service desk support.

Global Broadcast Service Operations Center Area of Responsibility (GBSOC AOR): The GBSOC AOR is the GBSOC work center. The primary location is SSFB; contingencies could direct the GBSOC AOR during GBSOC Contingency Operations to relocate to Peterson AFB or Hill AFB dependent on situation or circumstances.

GBS Equipment: Refers to any MUE that is accounted for by the GBSOC contractor.”

Government-Furnished Equipment (GFE) / Government Furnished Property (GFP): Equipment or property in the possession of the contractor, acquired directly by the government, or purchased by the contractor for the government.

Government Personnel: Defined as a military member, Department of the Air Force Civilian, or General Schedule (GS) employee, or any member of another branch of military services.

Hardware: The physical components of a system. Hardware includes the parts of a computer that can be seen and touched and may be controlled and put to work by software.

Hardware Levels of Maintenance: Two levels of maintenance, organizational and depot are required to support mission, mission support and communications systems. Under organizational maintenance there will be two categories, on-equipment and off-equipment maintenance. This requires that maintenance be planned and accomplished to ensure optimum effectiveness of equipment with minimum maintenance costs. Cost must be considered in determining repair capabilities but should not override mission requirements. Equipment maintenance is performed at the organizational level by the contractor. The contractor’s Repair Level Analysis, done for acquisitions and/or modifications, identifies items, equipment, modules, etc. for repair and provides recommended repair locations.

- Organizational (Level 1) Maintenance: Flexibility is the key for maintenance to ensure operational equipment availability for mission accomplishment of one-of-a-kind systems. This level of maintenance consists of those equipment tasks normally performed using the resources of an operating command at an operational location. These maintenance actions will be conducted on-site, on-equipment and off-equipment, to include scheduled and unscheduled actions. Downtime will be minimized by removing, replacing, and repairing equipment to an operational condition. Equipment will be repaired by removal and replacement of a line replaceable unit LRU, normally the lowest unit that can be removed from the equipment without cutting or unsoldering any connections. For example, a Printed Circuit Board (PCB), a power supply, a final amplifier, etc., shall be the LRU. Maintenance significant item (MSI) repairs, such as external wire, cable, and connector repairs, and replacement of fuses, lamps, batteries, and other expendable items performed on-equipment or off-equipment are considered organizational.

- On-Equipment Maintenance Those maintenance tasks that are or can be effectively performed on or at the system level. This maintenance includes the following:

  - Fault isolation and troubleshooting through the use of built in test (BIT) equipment and diagnostic procedures.

  - Restoral of prime mission equipment, generally is limited to removal and replacement of faulty LRU or major components, such as assemblies or subassemblies.

  - Servicing, alignment, and lubrication.

  - Preventive (scheduled) maintenance and testing.

  - Corrosion control.

  - Accomplishment of organizational level Time Compliance Technical Orders (TCTO).

  - Installation of software releases and system restoration through defined procedures.

- Off-Equipment Maintenance Those maintenance tasks that are not or cannot be effectively accomplished on or at the system end-item of equipment but require removal to a shop or facility for repair. When driven by equipment design, off-equipment maintenance will consist of calibration, alignment, replacement of LRUs. Assemblies and LRUs requiring repair beyond the capabilities of the organization will be disposed of IAW the SMR Code.

- Depot (Level 2) Maintenance The level of support consisting of those on-equipment and off-equipment support tasks performed using the highly specialized skills, sophisticated shop or special facilities of a supporting command, commercial activity, or inter-service agency at a technology repair center, centralized repair facility, or in some cases at an operating

location. Maintenance performed by depot may also include organizational on-equipment or off-equipment maintenance as determined between operating and supporting commands.

Depot maintenance will be accomplished IAW Air Force equipment maintenance and support policies, instructions, 00-20 series Methods and Procedures Technical Orders.

Hazard: Condition, procedure, or practice that creates a potential for producing death, injury, occupational illness, or equipment or property damage.

Information Assurance (IA): Measures that protect and defend information and information systems by ensuring their availability, integrity, authentication, confidentiality, and nonrepudiation. These measures include providing for restoration of information systems by incorporating protection, detection, and reaction capabilities. IA is now known as Cybersecurity.

Information Systems Security Officer (ISSO): Person who manages the COMSEC program for an information system assigned to him or her by the DAA; including monitoring information system activities and ensuring that the information system is operated, maintained and disposed of according to security policies and practices.

Information Technology Equipment: A machine or group of interconnected machines consisting of input, storage, computing, control and output devices which use electronic circuitry in the main computing element to perform arithmetic/logical operations by means of programmed instructions.

Information Technology Equipment Custodian: An individual who acts as a subordinate to the Equipment Control Officer (ECO) and performs inventory, utilization, and maintenance recording and reporting and other custodial duties as required by the ECO.

Installation/De-installation Plan: A plan that shows step by step procedures for the installation/de-installation of a database or software delivery. The plan includes recovery procedures, and change pages to the original documentation, any operational procedure changes not covered in the User Brief, any special installation/de-installation instructions, any possible operational impacts, and the suggested downtime.

Integration: The merging, when feasible, of the functional and technical characteristics to existing and planned C4 systems to ensure the resulting overall system is consistent with the Air Force C4 Systems Architecture (AFCSA) in the sense it is inter-operable, void of conflicts in purpose, schedule, and technology, and can effectively and efficiently support the Air Force.

Letter of Agreement: A document designed to provide a clear statement of expectations, establish responsibilities, and standardize procedures between two entities desirous of doing business with each other.

Line Replaceable Unit (LRU): A module, subassembly, or printed circuit card within or attached to a computer, which can be replaced or repaired without soldering.

Local Area Network (LAN): A telecommunications system that allows small computers to communicate with each other. Generally, Local Area Networks (LAN) are restricted to relatively small geographical areas within a base (between buildings or offices). Depending on the implementation, these communications networks can provide internal interchange of voice, data, graphics, video, or other forms of electronic messaging.

Maintainability: A characteristic of design and installation expressed as a probability that an item will be restored to a specific condition within a given period of time when the maintenance is performed using prescribed procedures and resources.

Maintenance: All action taken to retain material or equipment in a serviceable condition or to restore it to serviceability, including inspection, adjustment, repair, rebuilding, and reclamation. All supply and repair action taken to keep a force in condition to carry out its mission. The routine recurring work required to keep a facility (plant, building, structure, ground facility, utility system, or other real property) or item of equipment in such condition that it may be continuously utilized at its original or designed capacity and efficiency for its intended purpose. All work shall be accomplished IAW government publications, work specifications, and/or USAF-approved commercial publications.

Maintenance Operations Center (MOC): Assigned to the 50 CS. Coordinates, tracks and reports on all SBD 1 maintenance assets and systems.

Major Contingency / Event / Delay: A catastrophic weather event exceeding normal conditions for that location for that time of the year, or a catastrophic real-world event (e.g. 9-11).

Master Document Listing (MDL): A master listing of all applicable documents to this contract.

Material Safety Data Sheet : A document that lists the general components of a product and includes specific direction for handling if it is considered hazardous, toxic, poisonous, or poses any other health risk.

Memorandum of Understanding (MOU): A non-legally binding document containing cooperative working arrangements between two entities desirous of doing business with each other.

Military Satellite Communications (MILSATCOM): The worldwide military satellite network managed by the Defense Information Systems Agency, comprising orbiting space segments and ground terminals and control segments that operate in the super-high frequency band to provide long-haul multi-channel communications connectivity.

MILSTAR: A multi-service command and control satellite communications system. It provides worldwide, two-way, anti-jam, survivable, secure voice, teletype, and data communications.

Mishap: An unplanned or unsought event, or series of events, resulting in death, injury, occupational illness, or damage to, loss of equipment or property.

Mission Essential: Personnel or equipment, without which the mission could not be accomplished.

Mission Problem Report (MPR): A report of any abnormal events concerning any of the support systems (computer resources, communication equipment, software failures, etc.) which requires investigation by various support personnel for resolution of the reported anomaly.

Modification Control Board (MCB): A group of people responsible for evaluating and approving or disapproving proposed changes to configuration items, and for ensuring implementation of approved changes.

National Agency Check and Law and Credit Check (NACLC): To obtain background information for security investigative and evaluative purposes to make reliability and security

determinations, to allow access to classified information, sensitive areas, or equipment; to ensure that retention is clearly consistent with national security.

# Network:

- An organization of stations capable of intercommunication but not necessarily on the same channel.

- Two or more interrelated communications circuits.

- A system of software and hardware connected to support the exchange of data.

- A combination of circuits and terminals serviced by a single switching or processing center.

- Two or more systems connected by a communications medium.

On-Call: Maintenance performed by personnel who are not on duty but can be readily reached by telephone or other means to respond to equipment outages and malfunctions within a specified time period.

On-Line: A state in which an equipment or subsystem is connected and powered so as to deliver its proper input and output to the system.

On-Site: The term on-site refers to physical presence in the workcenter where operations and/or maintenance takes place. This does not include support facilities such as mess halls or billeting facilities unless they are colocated with the mission equipment.

On-Site Maintenance: That maintenance, either organic or contractor performed, on materiel while it remains under the custody of the using base, location, or site.

Operating Instruction (OI): A documented procedure used to standardize the accomplishment of a task.

# Operational Availability (Ao):

Ao =	Total Time - Downtime (within contractor’s control) Total Time

Ao is defined as being mission capable for the specified percentage of time averaged during a 30-day period. Downtime within the contractor’s control will be adjudicated at the end of each period by the government.

Operations Review Board (ORB): A meeting usually chaired by the unit operations officer, convened in order to determine the facts of an event, such as a failed/marginal support or a system failure/malfunction. The purpose is to determine what happened and what needs to be done to fix the problem and prevent it from happening again.

Operations Review Panel (ORP): A monthly meeting, chaired by the unit operations officer, to review and approve operational procedures.

Operations Security (OPSEC): Process of denying adversaries information about friendly capabilities and intentions by identifying, controlling and protecting indicators associated with planning and conducting military operations and other activities.

Participating Associate Contractor (PAC): A contractor responsible for providing equipment, data, or services to the Government.

Preventive Maintenance (PM): A procedure for inspecting, testing, and reconditioning equipment at regular intervals according to specific instructions; intended to prevent failures in services or to retard deterioration.

Program Manager (PM): The individual in the implementing command with the full authority and responsibility for managing a program. There is only one Program Manager for a given program; however, a Program Manager may manage more than one program.

Program Management Directorate (PMD): The highest government management echelon, along with associated staff elements, which sets policy and controls the discharge of duties associated with this PWS.

Qualification: Affirmation based upon documentation and evaluation, that an individual has satisfactorily fulfilled the requirements for performance in a given position/system.

Qualification is required to perform maintenance tasks.

Quality: The composite of material attributes (including performance features, end results, and characteristics) of an expected item of supply or service to satisfy a given need.

# Quality Assurance:

- A planned and systematic pattern of all actions necessary to provide confidence that an item of supply or services conforms to established requirements.

- A set of activities designed to evaluate the process by which quality is engineered in products and services are performed to customer expectations.

Real Property (RP): A separate building (facility), structure, system, or improvement to which a specific six-digit real property category code has been assigned, where the assignment is based upon the functional character of the facility.

Real Property Installed Equipment (RPIE): Items of equipment normally permanently built-in or attached to the structure, including items with fixed utility connections.

Receive Suite (RS), Fielded: Global Broadcast Service RS that are Combatant Command authorized RS in the Global Broadcast Service Satellite Broadcast Manager database. This includes both Global Broadcast Service Program of Record RS and non-Service Program of Record fielded RS.

Receive Suite (RS), Non-Service Program of Record: RS capable of receiving the Global Broadcast Service broadcast and are Combatant Command authorized in the Global Broadcast Service Satellite Broadcast Manager database (a.k.a, “Abnormal” or “rogue”RSs). It is the responsibility of the procuring office/agency or command to acquire appropriate technical support for any Non-Service Program of Record RS.

Receive Suite (RS), Program of Record: Global Broadcast Service RS procured through the GBS program of record. It includes the legacy AN/TSR-8 (Army), AN/TSR-9 (Navy/Marine Corps), AN/TSR-10 (Air Force) RS, and AN/TSR-11 (Transportable Ground Receive Suite), AN/PRS-11 (Suitcase Portable Receive Suite), and AN/PRS-12 (Rucksack Portable Receive Suite) RS.

Records Manager: The chief of the records function at units where there is a Director of Information Management or other administrative element.

Regression Testing: Selective retesting of a system or component to verify that modifications have not caused unintended effects and the system or component still complies with its specified requirements.

Requirement: A need for a new or improved (information processing) capability which when satisfied will result in an increase in the probability of operational mission success or a decrease in the cost of mission support.

Restricted Area: An area under military jurisdiction in which special security measures are employed to prevent unauthorized entry.

Satellite Control Network (SCN): The primary network system used to perform command and control of DoD Space Vehicle (SV) platforms. The SCN provides tracking, telemetry, and commanding support for virtually all DoD spacecraft as well as selected National Aeronautics and Space Administration (NASA), National Oceanographic and Atmospheric Administration (NOAA) and foreign government space programs.

Scenario: The presentation of a script to the participants of an exercise or rehearsal.

Scheduled Downtime: A prearranged period of unavailability of range resources approved by the appropriate authority.

Scheduled Maintenance: Known or predictable maintenance requirements that can be planned or programmed on either a short- or long-range schedule. This includes accomplishment of scheduled maintenance inspections and servicing, compliance with TCTOs other than the immediate action category, accomplishment of scheduled time change item replacements, and correcting deferred, or delayed discrepancies. It also includes programmed depot level maintenance and follow-on maintenance.

Scheduled Maintenance Outage: A break in channel/system availability or capability specifically scheduled to permit the accomplishment of preventive, organizational, intermediate, and/or depot maintenance.

Self-Sufficiency Program: The objective of the Self-Sufficiency Program is to maximize readiness during peacetime and sustainability during wartime in support of the wing, base, and MAJCOM mission. There are many items that could be repaired at the local level that are currently being disposed of at base level (this includes XB items) or being returned to the depot for repair. Program objectives are to improve mission capability by repairing equipment at the lowest practical level of maintenance, reduce system downtime caused by delays in obtaining repair parts or replacement assets, and minimize transportation and handling costs. The goal is to improve self-sufficiency through maximum use of available resources, not to develop a new, and perhaps costly, repair capability. Even though it makes sense for one site to accomplish the work locally, it may not be practical to expect all sites to do it. In this case, obtain special repair authority from the system/item manager based on a site unique capability and need.

Sensitive Information: Information, the loss, misuse, or unauthorized access to or modification of which could adversely affect the national interest or the conduct of federal programs, or the privacy to which individuals are entitled under 5 USC Section 552a (the Privacy Act), but that has not been specifically authorized under criteria established by an Executive Order or the Act of Congress to be kept secret in the interest of national defense or foreign policy (NSTISSI 1997).

Service Call: A maintenance or repair task, normally accomplished without detailed planning and using materials on hand. Service calls are of two types: emergency and routine (job order).

Services Summary (SS) A listing of the service outputs under the contract that are to be evaluated by the Contracting Officers Representatives on a regular basis, the surveillance methods to be used for these outputs, and the performance requirement of the listed outputs.

SIPR Enterprise Mission Assurance Support: The Enterprise Mission Assurance Support Service (e-MASS), on SIPRNet, is the DoD certification and accreditation tool used to execute the RMF in an automated fashion within a workflow construct. EMASS produces required RMF artifacts, automates IA control inheritance, meets DoD reciprocity requirements, and provides the foundation for the implementation of automated continuous monitoring.

# Software Maintenance/Support:

  - Organizational (Level 1) Software Maintenance/Support Tasks that include: maintain database contents and modify operational elements as necessary to maintain real-time operational readiness and meet changing requirements. Participate in system life-cycle activities to validate operational capability, identify problems, control configuration and certify new software releases, requirements gathering and analysis; requirements and change tracking; regression testing and operations verification/installation testing; assistance for anomaly/problem analysis and resolution; trending of anomalies/problems and their resolution; system performance analysis; file compares to ensure continued conformance to the release configuration for all operational and testing environments; archiving and file conversions; operations training; and meetings requiring technical assistance for software and its interaction with database, hardware, and operations procedures.

  - Depot (Level 2) Software Maintenance/Support Tasks include: provide emergency problem analysis and resolution; develop technical solutions; design, develop and modify software; certify software releases and modifications; maintain integrity of the system; distribute software releases; perform configuration management, identify and perform technology upgrades or modifications; perform special studies and maintain software support resources.

Standard Operating Procedure (SOP): A documented procedure used to standardize the accomplishment of a task.

Standard Base Supply System (SBSS) The SBSS is an accounting system consisting of standardized computer equipment, programs, procedures, and supply policy. This system provides base activities with their supply needs and employs a standardized base-level supply computer (SBLC) to account for supplies; equipment; oil, and lubricants; munitions; and clothing. With the SBSS, logisticians, customers, and commanders can track every item in the supply system through standardized programs and procedures.

Sustainment: Activities that sustain systems during the operations and support phases of the system life cycle. Sustainment activities include any investigative test and evaluation (T&E) which extends the useful military life of systems or expands the current performance envelope or capabilities of fielded systems. Sustainment activities also include T&E for modifications and upgrade programs and may disclose system or product deficiencies and enhancements that make further acquisitions necessary. The T&E conducted during sustainment follows the same guidance in this AFI as for the DT&E conducted during the acquisition process.

System Outage: A malfunction which degrades program support capability of equipment utilized by the network.

Systems Administration: Software Systems Administration includes the management of critical data backups, the assignment and tracking of system entry passwords and general support of the software system.

Technical Order (TO): A military order containing comprehensive technical instructions and information to produce, configure, install, maintain, modify, and operate systems and equipment so they work safely and effectively.

Technical Solution: A detailed description of the hardware, data, connectivity, logistics support, and other resources necessary to provide the most cost-effective solution to a deficiency or shortfall in mission capability. It includes the recommended acquisition method and strategy, estimates of all one-time and recurring costs, identification of manpower requirements (additional or savings estimate), and a schedule of events.

Technical Support: Detailed information given by qualified personnel to assist in developing new or corrective cost-effective procedures for performance of a mission function.

Technical Training: Training which is developed and maintained after initial training on new equipment, software, and procedures has been provided.

Time Compliance Technical Order: Instructions for modifying military systems or commodities within specified time limits, initiate special “one time” inspections, or impose temporary restrictions on systems or commodities; used to document all permanent modifications, update changes, and retrofit changes to standard Air Force systems and commodities.

Touch Maintenance: a term used to describe Tier 1 level maintenance on IT hardware that can be conducted by qualified IT technicians under contract to perform this effort.

Transfer of Operations (TOOP): The act of virtually transferring the SBM production (on-line) operations between the OKC and MECH GBS CDC SBMs on a quarterly basis in order to perform patching and maintenance on an (off-line) system with the intent on avoiding some unnecessary ASIs.

Unprogrammed Depot Level Maintenance: Depot Maintenance requirements that could not be forecasted or programmed. These activities are required to restore equipment to an operational status when such repair is beyond the capability of site operations and maintenance personnel.

This includes Emergency, Urgent and Routine maintenance.

- Emergency Depot Level Maintenance Those requirements of such an urgent nature that repair cannot be delayed. Equipment or systems are inoperative and active missions have failed.

- Urgent Depot Level Maintenance (UDLM) Those requirements which are unexpected in nature and could not be foreseen. The main difference between this requirement and an emergency is response time. Although the equipment is not inoperative, a mission impacting problem exists that must be resolved in a timely manner.

- Routine Depot Level Maintenance Non-time critical maintenance requirement that has no immediate or projected mission impact but requires a depot maintenance response.

Unscheduled Maintenance: A maintenance requirement that could not be forecasted or projected.

Visitor Group Security Agreement: A contractor operation on an Air Force installation requiring access to information, usually for more than 90 consecutive days. It operates under the direct control of the Air Force. It differs from a cleared facility by its close interactive relationship with the local Air Force mission. Unlike a cleared facility, a visitor group cannot store classified information independently.

Work Center: The area provided by the government or contractor for employees to perform assigned work as required for contractor operations.

Work Order: A document used to authorize work not otherwise authorized by the Preventive Maintenance Program and standing Work Orders. Support costs incidental to construction projects shall be included on all work orders. Work orders are used to authorize the following specific types of work:

- Work involving the removal, addition or replacement of Real Property (RP) or RPIE.

- Work in RP/RPIE covered by an “of value” warranty or guarantee.

# Appendix D Publications and Forms

GENERAL INFORMATION: Publications are listed in the Master Document Listing and Bidders’ Library (CDRL A108 – Master Document List (MDL) / Bidder’s Library (BL)) are generally referenced in the PWS. The Directive/Guidance (D/G) column indicates the extent of contractor compliance with the publication that is prescribed by the PWS. A “D” in the column means the publication is a “directive publication”, that requires mandatory compliance by the contractor. A “G” in the column means the publication is a “guidance publication”. These publications provide guidance for the contractor to perform particular tasks or operations in a manner compatible with Air Force procedures. However, the contractor is required to comply with the intent of the publication. Additionally any procedures developed shall be compatible with the intent of the “G” publications.

Publication Revisions. The publications listed are continually revised. Review all revisions, changes, and supplements for any cost impact (increase/decrease in work) and advise the CO within thirty (30) days of revisions. Upon such notification, the Government will review the changed requirement(s) and provide direction. Changes that are merely administrative in nature shall be accepted and performed accordingly. When revisions, changes or supplements involve safety, classified control or mission degradation, notify the CO by message within twenty-four

(24) hours of receipt of the publication.

Notes: The inclusion of a paragraph denotes the inclusion of all sub-paragraphs unless explicitly stated otherwise. Certain publications are classified or CUI and will be made available upon request through the CO.

# Appendix E

# Mission Essential Contractor Services

| Chapter | Title | Paragraph | Title |
| --- | --- | --- | --- |
| 2 | Program Management |  |  |
|  |  | 2.3 | Mission Essential Contractor Services |
|  |  | 2.8 | Local Purchase |
|  |  | 2.8.1 | Special Purpose Vehicles and Equipment |
| 3 | Operations |  |  |
|  |  | 3.1 | Global Broadcast Service Operations |
|  |  | 3.2 | Broadcast Management |
|  |  | 3.3 | End User Support |
|  |  | 3.4 | GBSOC Service Desk |
|  |  | 3.5 | System Administration |
|  |  | 3.6 | Configuration Management |
|  |  | 3.7 | Transfer of Operations (TOOP) |
|  |  | 3.8 | GBSOC Contingency Operations |
| 4 | Mission Systems Maintenance and Reporting |  |  |
|  |  | 4.1 | Maintenance |
|  |  | 4.2 | Standard Maintenance Practices |
|  |  | 4.3 | Corrective Maintenance |
|  |  | 4.4 | Preventive Maintenance |
|  |  | 4.5 | Corrosion Control |
|  |  | 4.10 | Cannibalization |
|  |  | 4.12 | Electro-Magnetic Interference |
|  |  | 4.13 | Crypto Equipment (All sub- paragraphs) |
|  |  | 4.14 | Self-Sufficiency Program |
|  |  | 4.15 | Maintenance Operations Center Mission System Reporting |
|  |  | 4.16 | Global Broadcasting Service |
|  |  | 4.17 | Wideband Global Satcom |
|  |  | 4.18 | Certification of Special Equipment |
|  |  | 4.24.1 | Systems Maintenance |
|  |  | 4.26 | Mission Impact Reporting (MIR) |
| 5 | Supply and Logistics Support |  |  |
|  |  | 5.1 | Supply Support |
|  |  | 5.1.1 | After Hours Support |
|  |  | 5.1.2 | Supplies and Equipment (need to id sub paras) |
|  |  | 5.1.2.1 | Supply Support |

| Chapter | Title | Paragraph | Title |
| --- | --- | --- | --- |
|  |  | 5.1.2.3 | MICAP and back-shop supply actions |
|  |  | 5.2.1.1 | Critical Spares |
| 6 | Power Management Support |  |  |
|  |  | None |  |
| 7 | Wing Cybersecurity and Information/ Communication Systems Support |  |  |
|  |  | None |  |
| 8 | Systems Safety |  |  |
|  |  | None |  |
| 9 | Environmental Requirements Support |  |  |
|  |  | All | Uninterruptable Power Supply |
| 10 | Space Digital Integrated Network (SDIN) and Global Command and Control System (GCCS) |  |  |
|  |  | 10.1 | General |
|  |  | 10.2 | SDIN and GCCS COOP |
|  |  | 10.3 | Work Status Reports |
|  |  | 10.6 | Support Services / Maintenance Contracts |
|  |  | 10.7 | Integration |
|  |  | 10.8 | Asset Management |
|  |  | 10.9 | SDIN and GCCS Network Services |
|  |  | 10.10 | GCCS |
|  |  | 10.11 | SDIN |
|  |  | 10.12 | Information Assurance Operations |
| 11 | Operational Systems Management |  |  |
|  |  | None |  |
| 12 | Technical Control Facility | All |  |
| 13 | Other Support Requirements |  |  |
|  |  | 13.11.3 | Entry Procedures |
|  |  | 13.12 | Resource Protection |
|  |  | 13.14 | Electro-Magnetic Frequency (EMF) Radiation |
| 14 | Government Furnished Equipment (GFE), Property (GFP) and Information (GFI) |  |  |
|  |  | None |  |