US Department of Agriculture (USDA)

Statement of Work

# STATEMENT OF WORK

# Background

- The USDA, ARS, Sugarcane Research Unit is currently in need of replacement of existing actuators with new actuators for Exhaust Fans 6 and 7 at the Ardoyne farm headhouse.  This work will be completed at 501 Bull Run Road, Schriever, LA  70395.

# Scope

- The contractor shall provide all labor, parts and materials for replacement of existing actuators with new actuators.

# Tasks or Requirements

- The contractor shall replace (2) existing actuators with new actuators for Exhaust Fans 6 and 7.

  - Actuators part number: GNP191.1P and GCA-166.U

    - GNP191.1P or equal-- Fail Safe, 53 lb-in, 2-sec., 24Vac/dc, universal control signal (two-position, floating, 0-10Vdc/2-10Vdc, 4-20mA/0-20mA, Siemens proprietary FHC/LRC pulsed), shaft diameter is 1/4 to 3/4 in. round or 1/4 to 1/2 in square

    - GCA-166.U or equal -- OpenAir GCA Series Electric Damper Actuator, rotary, spring return, 160 lb-in (18 Nm), 24 Vac/dc, 0 to 10 Vdc control, 90 sec run time, dual auxiliary switches

    - 546-00492 or equal-- HORIZ UNI-TRAK TOP ASSY 52 IN

- Arrive and check in with POC

- LOTO main shut off power

- Disconnect electrical for (2) existing actuators

- Remove (2) existing actuators

- Install (2) new Siemens actuators

- Reconnect electrical for (2) new Siemens actuators

- Check Desigo to make sure everything is correctly communicating on the front-end

- Verify operation of (2) new Siemens actuators

- Clean material and trash from site after completion of work

- Services will be completed in accordance with all applicable federal laws and regulations.

- Country of Origin: United States

- Manufacturing Location: Siemens Industry OpenAir manufacturing facility in Buffalo Grove, Illinois

# Deliverables or Delivery Schedule

- All services shall be between the hours of 7:30 a.m. and 4:00 p.m. Monday through Friday

# Place of Performance

- The work will be performed at the USDA, ARS, Sugarcane Research Unit, Ardoyne farm, 501 Bull Run Road, Schriever, LA  70395.

POC

Richard Johnson

richard.johnson@usda.gov

985-870-4188