REQ DOC NO. -	6028V202 – 6028V205, 6040V314      														ATTACHMENT _________

CONTINUATION SHEET	    CONTRACT NO. -	_______________	PAGE ___ OF ___

NAME OF OFFER OR/CONTRACTOR

ITEM #		SUPPLIES/SERVICES			QTY			U/I

______________________________________________________________________________

1	1.5 IN ID X 82 IN LONG; HOSE ASSEMBLY		     2			   EA

2	1.5 IN ID X 76 IN LONG; HOSE ASSEMBLY		     2			   EA

3	2 IN ID X 8.5 FT LONG; HOSE ASSEMBLY		     4			   EA

4	2 IN X 36 FT LONG; HOSE ASSEMBLY			     2			   EA

5	2  IN X 75 FT LONG; HOSE ASSEMBLY			     1			   EA

C. DESCRIPTION/SPECIFICATIONS

C.1. ONE-POINT-FIVE-INCH (1.5”) ID HOSE ASSEMBLIES:

C.1.1. QTY TWO (2): 1.5-INCH ID X 82-INCH LONG; MANUFACTURER’S P/N: 24-CLN+-82IN-XX

C.1.2. QTY TWO (2): 1.5-INCH ID X 76-INCH LONG; MANUFACTURER’S P/N: 24-CLN+-76IN-XX

C.2 TWO-INCH (2.0”) ID HOSE ASSEMBLIES:

C.2.1. QTY FOUR (4): 2.0-INCH ID X 8.5-FT LONG; MANUFACTURER’S P/N: 32-CLN+-102IN-XX

C.2.2. QTY TWO (2): 2.0-INCH ID X 36-FT LONG; MANUFACTURER’S P/N: 32-CLN+-432IN-XX

C.2.3. QTY ONE (1): 2.0-INCH ID X 75-FT LONG; MANUFACTURER’S P/N: 32-CLN+-900IN-XX:

C.2.3.1. NON-LINED FLANGES

C.2.3.2. STAINLESS STEEL WIRE REINFORCED

C.2.3.3. SMOOTH-EPDM, PROTECTIVE RUBBER COVER

C.3. ALL HOSES TO MEET THE FOLLOWING REQUIREMENTS:

C.3.1. 100% PTFE LINER WITH NO ADDITIVES; SMOOTH BORE

C.3.2. ANSI B16.5 CLASS 300 FLANGED END CONNECTIONS; 316 STAINLESS STEEL

C.3.3. AFLEX CORROLINE+, GP SERIES

C.3.4. PERFORM A HYDROSTATIC PROOF TEST OF ALL ASSEMBLIES TO 600 PSIG MINIMUM FOR 30 MINUTES.

C.4. DATA REQUIREMENTS PER EXHIBIT A (DD-1423) & ATTCHMENT #1.

C.4.1.	CERTIFICATION OF HYDROSTATIC PROOF TEST; EXHIBIT A SEQUENCE A002AA, A004AA, A006AA, A008AA, and A010AA.

C.4.1.1	ACCEPTANCE CRITERIA: NO LEAKAGE

C.5. PACKAGING AND MARKING REQUIREMENTS:

C.5.1. AFTER HYDROSTATIC PROOF TEST, HOSES TO BE FREE OF DAMAGE/NICKS/CUTS/SOFT SPOTS/BULGES ETC AND CLEANED/DRIED AND PLUGGED FOR SHIPMENT.

C.5.2 EACH HOSE IS TO BE AFFIXED WITH A TAG INDICATING THAT A HYDROSTATIC PROOF TEST HAS BEEN PERFORMED. THE TAG SHALL CLEARLY IDENTIFY THE HOSE ASSEMBLY UPON WHICH THE PROOF TEST WAS PERFORMED.