STATEMENT OF WORK

Title: Galvanized Steel Stair Installation – Building 2 Mechanical Room

I. BACKGROUND INFORMATION

The Boulder Facilities Maintenance Division (BFMD) manages and provide services at the National Institute of Standards and Technology (NIST) in support of research, technical, and administrative programs. Due to the age and condition of 2 sets of stairs in the building 2 mechanical room these stairs need to be replaced to be brought up to code.

II. PURPOSE

Provide all labor, materials, equipment, tools, and supervision necessary to demolish and legally dispose of one (1) existing 5-riser wooden staircase and one (1) existing 9-riser wooden staircase located in the lower-level mechanical room of Building 2.

Furnish, fabricate, and install new galvanized steel stair assemblies to replace the removed stairs. New stairs shall include slip-resistant (no-slip) treads and shall be field-fabricated and installed in place, including all required on-site welding, due to restricted access that prevents prefabricated installation.

All work shall be performed in strict accordance with all applicable local, state, and federal codes, regulations, and industry standards. Contractor shall ensure proper protection of adjacent surfaces and systems, coordination with other trades, and safe execution of work within the mechanical room environment.

PLACE OF PERFORMANCE

National Institute of Standards and Technology at 325 Broadway, Boulder, CO 80305.

Regular Business Hours

Regular business hours are Monday through Friday, 7:00am to 5:00pm Mountain Time, excluding Federal holidays and NIST closures.

PERIOD OF PERFORMANCE

The contractor will have 120 days to perform the work from the notice to proceed.

- SPECIFIC REQUIREMENTS

### 1. Codes & Standards

Stair system shall be designed, fabricated, and installed to comply with the latest adopted editions of:

- International Building Code (IBC)

- OSHA standards (including OSHA 1910 and 1926, as applicable)

- Local building and fire codes

- Manufacturer installation requirements

### 2. Materials

- Structural stair components fabricated from carbon steel with hot-dip galvanized finish for corrosion resistance.

- Stair treads to be non-slip, heavy-duty industrial type (e.g., serrated bar grating or abrasive safety tread), suitable for mechanical room conditions.

- All fasteners to be galvanized or stainless steel, compatible with galvanized assemblies.

### 3. Fabrication

- Dimensions, rise/run, headroom, and landing requirements shall meet code requirements.

- Stair width and load ratings shall be suitable for mechanical room access and maintenance activities.

### 4. Installation

- Install stairs plumb, level, and securely anchored to existing structure (concrete or steel) per engineered requirements.

- Provide code-compliant landings, handrails, and guardrails as required.

- Ensure all walking surfaces are uniform, stable, and slip resistant.

- Coordinate installation to avoid interference with existing mechanical, electrical, and plumbing systems.

- No sharp edges or protrusions; all cut edges dressed smooth.

- Verify tread slip resistance and secure attachment prior to turnover.

- Maintain safe access to the mechanical room during construction.

### 5. Cleanup & Closeout

- Remove all debris, scrap materials, and packaging from the work area.

- Deliver completed stair installation ready for inspection and use.

OTHER PROJECT REQUIREMENTS

Safety on the NIST Campus

NIST is committed to maintaining a work environment that safeguards the public and the environment, as well as personnel and property.

If the award requires construction, demolition, excavation, renovation, or other work for which the contractor directs and oversees contractor personnel (including subcontractors) at NIST-owned and operated sites, the contractor shall:

Provide a detailed, site-specific Safety Plan to the contracting officer within 7 days after award that describes how the contractor will maintain a safe work environment and comply with applicable Occupational Safety and Health Administration (OSHA) standards, as well as with applicable state and local laws, standards, and codes.

The Safety Plan shall specify how the company’s policies and practices will comply with each of the applicable standards, laws, and codes.

The contracting officer will provide written comments regarding the plan to the contractor within seven (7) days of receipt and will require the contractor to submit a revised safety plan within seven (7) days after that to the contracting officer for their review. This cycle may be repeated, if necessary, until the contracting officer determines the safety plan to be acceptable.

Reference:

See NIST’s Contractor Safety Program for required elements of Contractor Safety Plans.

Safety-Related Equipment

Provide all safety-related equipment required to perform the work under this award, including personal protective equipment (PPE) and other equipment that is required or appropriate to protect contractor personnel and other personnel on the NIST campus. Contractor-provided personal protective equipment shall include, at a minimum, hard hats, safety shoes, safety glasses (prescription or standard), respiratory protection (powered air purifying respirator or full-face mask respirator), high visibility clothing and gloves that meet applicable OSHA standards for PPE.

Training

Provide safety-related training required by applicable OSHA, state and local regulations or standards to affected contractor personnel (including subcontractor personnel) that perform work under this award. Such training may include, for example, training for proper use of PPE, respirator use, fall protection, crane operation, forklift operation; electrical safety, or other safety-related topics.

Maintain records of safety training provided to contractor personnel (including subcontractor personnel) that perform work on this award. The contractor shall make those records available to the contracting officer, contracting officer’s representative for the award, or other authorized personnel within 7 days of receipt of their written request, and annually thereafter or upon request.

Require contractor personnel (including subcontractors) to attend   required NIST-specific health or safety training that is required by the award.

Health Hazard Assessments and Medical Evaluations

Provide required health hazard exposure assessments, medical evaluations and screenings, and exposure monitoring required by applicable OSHA requirements.

Ensure PII information is protected.

Homeland Security Presidential Directive (HSPD)-12

The Contractor (and/or any subcontractor) and its employees who require physical access to NIST facilities or logical access to NIST IT networks or systems must comply with Homeland Security Presidential Directive (HSPD)-12, Policy for a Common Identification Standard for Federal Employees and Contractors; OMB M-05-24; OMB M-19-17; FIPS 201, Personal Identity Verification (PIV) of Federal Employees and Contractors; NIST HSPD-12 policy; and Executive Order 13467, Part 1 §1.2.

5. Roster

The Contractor (and/or any subcontractor) must submit a roster by name, position, e-mail address, phone number and responsibility, of all staff working under this acquisition where the Contractor will develop, have the ability to access, or host and/or maintain a government information system(s).

The roster must be submitted to the COR and/or CO within 14 days of the effective date of this contract. Any revisions to the roster because of staffing changes must be submitted within 14 days of the change. The COR will notify the Contractor of the appropriate level of investigation required for each staff member. If the employee is filling a new position, the Contractor must provide a position description, and the Government will determine the appropriate suitability level.

Position Sensitivity Designations Requirement

All Contractor (and/or any subcontractor) employees must obtain a background investigation commensurate with their position sensitivity designation that complies with Parts 1400 and 731 of Title 5, Code of Federal Regulations (CFR).

Inspection & Acceptance

In addition to the inspection and acceptance terms articulated in the specific FAR clause that allows the Government reserves the right to perform such performance tests and evaluations as defined below to verify specified system performance. Such tests and evaluations, if performed, shall be conducted within the environment that the system is to be operated. The Contractor has the right to be present during the tests and evaluations, if performed, at the Contractor’s expense.

- GENERAL REQUIREMENTS

Safety:  The Contractor employee must be responsible for knowing and complying with all installation safety prevention regulations.  Such regulations include, but are not limited to, general safety, fire prevention, and waste disposal.

Security: NIST is a restricted campus.  An identification badge is required for access for entry into buildings and is shown to the armed Security Police when entering the campus.

Identification Badges: Contractor employees must comply with NIST identification and access requirements.  Each Contractor employee must wear a visible identification badge provided by the NIST Security Office.

V.  SCHEDULE OF DELIVERABLES

| Deliverable Number | Description | Format | Due Date | Quantity |
| --- | --- | --- | --- | --- |
| 1 | Material submittal | Word or PDF | Before the start of installation | 1 |
| 2 | Schedule | Word or Excel | Before the start of work | 1 |
| 3 | Site specific safety plan | Word or PDF | Before the start of work | 1 |

Standards of Acceptance: The NIST POC or COR shall review all the above deliverables and respond with an acceptance or request for revision email to the Contractor Point of Contact (POC) within 3 days of receipt of deliverable. This section applies to deliverables which must be reviewed and approved for acceptances (e.g., draft plans, drawings, etc.)

PAYMENT SCHEDULE

The Contractor will be paid, in accordance with the payments clause in the contract and as otherwise noted in this document, upon receipt of a proper invoice.