San Juan-Chama Project, New Mexico - Colorado

Rio Chama Trail Staircase

Rio Arriba County, New Mexico

FOREWORD

The Rio Chama staircase is located at the Heron trailhead located at Latitude 36º 39’ 40.5” and Longitude 106º 41’ 53.8”.  The following project description shall define the major aspects of work tasks:

- Site Visit observations and discussions:

  - Review of site access difficulties and restraints.

  - Level of effort to remove the following:

    - Existing Rio Chama Trail staircase and railing.

    - Existing Rio Chama Trail cableway bridge.

  - Inventory of Government Furnished Materials and Hardware located at the Chama Field Office.

  - Confirm existing field dimensions against new design drawing dimensions includes hand railing located on each side of the staircase.

- Project activities:

  - Coordination with New Mexico Park Service and Reclamation’s Chama Field Office:

    - New Mexico State Parks for access, construction water, and portable toilet facilities.

    - Chama Field Office for access to staircase and railings and to coordinate delivery of Government Furnished Products.

  - Do not damage existing concrete foundation when removing old staircase.  Inspect all existing and document changes to installation of new staircase and anchor locations.

  - Fabricate staircase and new handrailing.

  - Do not damage existing vegetation and infrastructure when setting up equipment.

  - Remove existing staircase and handrailing and dispose offsite.

  - Install new staircase and handrailing.

  - Install new kiosk.

  - Remove existing cableway bridge and dispose offsite.

PRE-PROPOSAL SITE VISIT: A PREPROPOSAL SITE VISIT WILL BE HELD ON FRIDAY, APRIL 10, 2026; ENTRANCE TO THE RIO CHAMA TRAIL; BEGINNING AT 11 AM MST.  PROSPECTIVE CONTRACTORS SHOULD MAKE ARRANGEMENTS BY CONTACTING CORD EVERETTS AT TELEPHONE NUMBER (505) 462-3619.

FOR DATE AND PLACE OF PROPOSAL DUE DATE, SEE "SOLICITATION, OFFER, AND AWARD," STANDARD FORM 1442.

FOR INFORMATION REGARDING THE BUREAU OF RECLAMATION'S PUBLICATION "RECLAMATION SAFETY AND HEALTH STANDARDS" INCLUDING REVISIONS AT http://www.usbr.gov/ssle/safety/RSHS/rshs.html) WHICH IS APPLICABLE TO WORK UNDER THIS CONTRACT, SEE CLAUSE AT "WAR 1452.223-81 SAFETY AND HEALTH."

SECTION C - DESCRIPTION / SPECIFICATIONS

TABLE OF CONTENTS

DIVISION 01 - GENERAL REQUIREMENTS

01 11 00	Summary of Work

01 14 10	Use of Site

01 31 22	Coordination Meetings

01 32 10	Construction Program

01 33 00	Submittals

01 35 10	Safety Data Sheets

01 35 20	Safety and Health

01 35 30	Contractor’s Onsite Safety Personnel

01 42 10	Reference Standards

01 46 00 	Quality Procedures

01 51 00	Temporary Utilities

01 55 00	Vehicular Access and Parking

01 56 10	Protection of Existing Installations

01 57 20	Environmental Controls

01 57 30	Water Pollution Control

01 57 35	Oil Spill Prevention

01 57 50	Tree and Plant Protection

01 60 00	Product Requirements

01 64 40	Government-Furnished Products

01 71 20	Surveying

01 74 00	Cleaning and Waste Management

01 78 30	Project Record Documents

DIVISION 02 - EXISTING CONDITIONS

02 41 10	Staircase Demolition

02 41 16	Cableway Bridge Demolition

DIVISION 03 - CONCRETE

03 30 53	Miscellaneous Cast-In-Place Concrete

03 62 20	Non-shrink Grout for Equipment and Metalwork

DIVISION 05 - METALS

05 12 00	Structural Steel Framing

05 50 00	Metal Fabrications

DIVISION 09 - FINISHES

09 97 00	Metalizing and Coatings

DIVISION 10 - SPECIALTIES

10 18 00	Informational Kiosks

DIVISION 31 - EARTHWORK

31 23 02	Compacting Earth Materials

31 23 11	Earthwork for Structures

DIVISION 32 – EXTERIOR IMPROVEMENTS

32 92 20	Seeding and Soil Supplements

DIVISION 51 - INFORMATION AVAILABLE TO OFFERORS

51 00 00	Information Available to Offerors

DIVISION 52 - DRAWINGS

52 00 00	Drawings

END OF CONTENTS

SECTION 01 11 00

SUMMARY OF WORK

GENERAL

LOCATION

Work is located near Heron Reservoir on the Rio Chama Trail approximately 10.5 miles southwest of Los Ojos, New Mexico on State Highway 95 in Rio Arriba County at latitude 36° 39’ 40.5”N, longitude 106° 41’ 53.8” W.

PRINCIPAL COMPONENTS OF WORK

Pickup Government Furnished Products (GFP) consisting of miscellaneous metalwork and hardware currently storage at the Chama Field Division located at 193 North Pinon Drive, P.O. Box 426, Chama, New Mexico 87520.

Field verified Government’s design for the replacement staircase and railing panels.

Fabricate staircase and railing panels.

Using GFP to fabricate staircase and side hand railing panels when installed land on the existing concrete abutments or pads sites.

Contractor to furnish and fabricate entrance hand railing panels from similar materials to GFP allowing a consist appearance when installed.

Remove existing staircase, hand railings, and entrance hand railing panels.

Install new staircase, hand railings, and entrance hand railing panels.

Furnish and install new anchor bolts to fasten the new staircase and hand railing panels to existing CIP foundations.

Furnish and install new kiosk.  Location will be determined in the field between Bureau of Reclamation and New Mexico State Parks personnel.

Remove existing cableway suspension bridge located in Rio Arriba County, New Mexico on the Rio Chama right embankment at latitude 36° 39’ 40.5”N, longitude 106° 41’ 53.8” W and left embankment at latitude 36° 39’ 40.5”N, longitude 106° 41’ 53.8” W.  Disposal of removed materials according to local requirements.

When possible, recycle miscellaneous metalwork, plastics, and wooden items.

SPECIFICATIONS REQUIREMENTS

Requirements in Division 1, General Requirements, apply to Divisions 2 through 52.

Imperative statements in these specifications are Contractor requirements, unless otherwise stated.

Where specifications are written in streamlined form, words “shall be” are included by inference where a colon (:) is used within sentence or phrase.

DEFINITIONS

When specifications use a word or term defined in Federal Acquisition Regulations (FAR), definition of the word or term shall be in accordance with FAR sections in effect at the time solicitation was issued.

ACRONYMS

The following acronyms apply to specifications Divisions 1 through 52:

CO -	Contracting Officer.

COR -	Contracting Officer’s Representative.

ACOR. -	Alternate Contracting Officer Representative

OGR -	Onsite Government Representative.

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

SECTION 01 14 10

USE OF SITE

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

REFERENCE STANDARDS

Bureau of Reclamation (USBR)

Cleaning Manual-2021	Inspection and Cleaning Manual for Equipment and Vehicles to Prevent the Spread of Invasive Species (Technical memorandum No. 86-68220-07-05) 2012 Edition available online at: http://www.usbr.gov/mussels/prevention

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 14 10-1, Land Use and Landscape Rehabilitation Plan:

For each Contractor use site on Government land.

Show use location and extent of impact.  Uses include but are not limited to the following:

Buildings and service areas include offices, shops, warehouses, storage areas, fuel and oil storage areas, and fabrication yards.

Parking areas, temporary roads, and haul routes.

Utilities include air, power, and water lines; fire hydrants; and compressor station.

First-aid and medical facilities.

Areas for processing, storing, and disposing of waste materials from construction operations.

Temporary fences.

Describe methods to preserve, protect, and repair, vegetation (such as trees, shrubs, and grass) and other landscape features on or adjacent to jobsite, which are not to be removed, and which do not interfere with work required under this contract.  Include methods to mark work area limits, protect disturbed areas, and prevent erosion.

Describe methods to protect, and repair if damaged, existing improvements and utilities at or near jobsite.

Describe methods for removing temporary structures and facilities, cleanup, and rehabilitating site after completion of construction activities.

Submit revised drawings of changes in use of Government land made during design and erection stages or after use of Government land is in operation.

PROJECT CONDITIONS

Government land as shown in drawings and described may be used for required construction facilities.

When private land is used for construction facilities, or other construction purposes, make necessary arrangements associated with use of private land.

Location, construction, operation, maintenance, and removal of construction facilities on Government land will be subject to approval of COR.

Do not interfere with work of other contractors or the Government in vicinity, or with reservations made by the Government for use of such land.

Housing for construction personnel will not be permitted on Government land, except housing for guards or watchmen as may be approved by COR.

PRODUCTS

Not Used

EXECUTION

CLEANING

Construction equipment:

Before bringing on site, clean construction equipment to remove dirt, vegetation, and other organic material to prevent introduction of noxious weeds, and invasive plant and animal species.

Contractor cleaning procedures shall result in equipment being cleaned as well or better than the procedures described in Cleaning Manual.

COR will inspect construction equipment following procedures described in Cleaning Manual before allowing equipment onsite.

RESTORATION

Restore Contractor use areas to pre-construction condition.

After completion of work, regrade and scarify Government land used for construction purposes and not required for completed installation so that surfaces blend with natural terrain and are in a condition that will facilitate revegetation, provide proper drainage, and prevent erosion.

Seed disturbed areas of Government land used for construction purposes and not required for completed installation in accordance with Section 32 92 20 - Seeding and Soil Supplements, with species of native plants, or other approved perennial species.

END OF SECTION

This page intentionally left blank.

SECTION 01 31 22

COORDINATION MEETINGS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

SUBMITTALS

Submit in accordance with Section 01 33 00 - Submittals.

RSN 01 31 22-1, Coordination Meeting Notification:

Draft Agenda

Include place, date, time, proposed attendees list, and at minimum the following topics:

Schedule(s)

Safety Plan and Job-Hazard-Analysis (JHA)

Staircase, hand railings, and entrance hand railing panels fabrication drawings, field verifications, etc.

Mobilization and removal of existing staircase and hand railings

Installation of new staircase and hand railing panels

Removal of existing cableway bridge

Site cleanup and disposal

SPECIFICATION DRAWINGS

At the Government’s discretion 2023 AutoCAD drawings files can be made available to Contractor within 14 days after written request.

COORDINATION MEETINGS

Project (kick-off) construction meeting.

Date: Approximately 14 days after receipt of Notice to Proceed.

Before scheduling, notify the Government of meeting date, time, and location a minimum of 5 business days, include a draft Agenda.

Location options:

Cham Field Division at 193 North Pinon Drive, P.O. Box 426, Chama, New Mexico 87520.

Albuquerque Area Office at 555 Broadway, Blvd., N.E., Suite 100, Albuquerque, New Mexico 87102.

Purpose:

Review contract administration requirements.

Review contract specifications, drawings, and related technical requirements of specifications.

Discussing site access and safety include review of safety plan and JHAs.

Discuss site access and coordination with New Mexico State Parks personnel.

Attendees:

Contractor’s representatives shall include, at least, 1 person from these groups:

Contractor: Owner, Project Manager, or Site Superintendent.

Crane operator.

Government representatives shall include, at least, 1 per form these groups:

Contracting Officer, Contracting Specialist, and Contracting Office Technical Representative.

Facility Support: CFD, Manager and Heron Dam Tender.

Project Support: Northern Project Manager or SJC Project Manager.

Within 30 calendar days after conclusion of meeting, the Government will furnish the Contractor minutes (email) regarding items discussed and if required list of outstanding action items.

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

SECTION 01 32 10

CONSTRUCTION PROGRAM

GENERAL

MEASUREMENT AND PAYMENT

Costs:

Developing Baseline Schedule: Include in lump sum price offered in the Prices Schedule for Mobilization and Preparatory Work.

Updating and Using Construction Program: Include as part of Contractor’s overhead.

DEFINITIONS

Project Schedule and Calendar(s):

Schedule show award, kick-off meeting, submittals, pickup of Government Furnished Materials/Supplies, fabrication, mobilization to project site, removal of staircase and hand railing panels, installation of staircase and hand railing panels, removal of cableway bridge, and final punch list, and cleanup.

Cross reference of numerical workdays with calendar days.  Project calendars serve as the basis for day/date conversion and assign workdays and non-workdays.

Resources: Equipment, labor or crews, materials, subcontractors, fabricators, manufacturers, and consultants.

Out of Sequence Work: An activity that starts before its predecessor activities are completed.

REFERENCE STANDARDS

Associated General Contractors of America (AGC)

AGC Manual	Construction Planning and Scheduling Manual, 2004

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

Furnish database files in format compatible with 2016 MS Project.

Furnish schedule and analysis data on approved media.

Upon request, provide information and data used to develop and maintain the Construction Program to the CO.

RSN 01 32 10-1, Baseline Schedule:

Include:

Definition of project calendars.

Gantt chart (Bar chart) for project on (ANSI B) 11-inch by 17-inch print(s) and in portable document format (.pdf).

Activity report including logic constraints consisting of predecessors, successors, and constraint dates.

Narrative explanation of project schedule development.  Include impacts of resource limitations and weather conditions on project schedule development.   Include planned number of work hours per day and hours per day for subcontractors.

RSN 01 32 10-2, Updated Schedule Reports:

This submittal is required when milestone dates change by greater than 30 calendar days.

Include:

Definition of project calendars if revised from baseline calendars.

Gantt chart for project on 11 by 17-inch (ANSI B) print(s) and in portable document format (.pdf).

QUALIFICATIONS

Representative: Experienced in developing and maintaining construction schedules and knowledgeable of activities and progress on-site to develop and maintain accurate and reliable schedules.

GENERAL

Develop, maintain, and use Construction Program to plan, monitor, evaluate, and report accomplishment of work.

SCHEDULES

Prepare construction schedules using Critical Path Method outlined in AGC Manual. Use a computer software program to perform mathematical analysis of scheduling data.

Prepare schedule based on required sequence and interdependence of activities.  Logically sequence activities.

Prepare detailed activity network for accomplishing required work organized by Work Breakdown Structure (WBS).  Utilize WBS in lieu of activity codes for organization of schedule.  If activity codes are utilized in addition to WBS, use unique activity code names assigned as project codes rather than global codes.

Activities except “Award” shall have predecessor activities and activities except “Contract Complete” shall have successor activities.

Meet contract requirements; milestone(s) in accordance with the clause at FAR 52.211-10, Commencement, Prosecution, and Completion of Work; and relevant clauses and specification sections.  Include interim milestone dates, Government interface dates, contract completion date, and other time or seasonal constraints specified in contract documents.

Include work of subcontractors, Government interfaces, and contract milestones.

Adjust Construction Program/schedule for seasonal weather conditions.  Provide a narrative explaining the expected weather and plan for incorporation in schedule.  Consider work influenced by high or low ambient temperatures or precipitation.  Utilize 2016 MS Project calendars to depict historical weather days that impact work.  Assign weather calendars to work impacted by weather.

Define activities to level of detail resulting in their durations being no greater than 20 workdays.

Durations for administrative activities (e.g., submittals and reviews) fabrication, manufacturing, and typical level of effort activities (e.g., dewatering, storm water management) will not be subject to workday duration limitations.

Include separate activities for submittal preparation and submittal reviews when submittal addresses work that involves significant quantities, long lead times, is on the critical path or near critical path, or as requested by the COR.  Assign Government submittal review activities to a seven-day calendar with durations as specified in Section 01 33 00 - Submittals.  Include Required Submittal Number (RSN) in activity description.

Include separate activities for fabrication or manufacturing when work involves significant quantities, long lead times, is on the critical path or near critical path, or as requested by the COR.

Include contract title, contract number, and Contractor’s name on transmittal cover sheet and each sheet of Gantt chart.

For each activity on Gantt chart, display activity identification number, activity description, planned or original duration, start date, finish date, total float, and calendar identification.  Do not display early start and late start fields.

Include table of abbreviations used in the schedule, listed and defined alphabetically.

Use finish to start logical relationships between activities.   Do not utilize start to start, finish to finish, or start to finish logic relationships.  Do not use negative lead or lag times.

Use durations in units of whole workdays.

Provide best estimate of time required to complete activity considering quantity of work, work conditions, location of work, and planned resources for activity.

Equate durations of Government reviews and other identified actions to maximum number of calendar days specified in their respective paragraphs.

Establish workday calendar(s) and use these in the schedule to translate activity's workday duration into calendar dates.  Use unique calendar names; do not use software default calendar names.  Save calendars as project calendars, not global calendars.

Baseline Schedule:

Represents Contractor’s as-planned approach to accomplishing the work.  Do not include actual start dates, percentage completes, or actual finish dates.

Updated Schedule:

Meet monthly with COR at Government’s project office, or at a location approved by the COR, to review progress made to end date of progress payment period.  Establish dates that activities were started and completed and remaining duration for each activity started but not completed during the period.

Discuss and mutually agree upon changes to the schedule.

Out-of-sequence activities are not allowed.  Revise logic to reflect change in work plan.

Finalize updated Construction Program database with mutually agreed upon changes.

Following receipt of executed contract modification, incorporate activity data stipulated in modification into current schedule for inclusion in next scheduled progress update.  Provide appropriate logical relationships for revised activities.

Assign a unique project file name for each schedule update.

TIME IMPACT ANALYSIS

Provide time impact analysis for contract changes (e.g., change order, proposed modification, or value engineering change proposal) to support a claim or request for equitable adjustment to the contract due to delay or accelerated schedule.

The CO may use time impact analysis to determine if time extension or reduction to contract milestone dates is justified.

Time impact analysis is required for contract changes whether the Contractor’s current schedule milestone dates are the same as, earlier than, or later than, those required under the contract.

Changes, additions, or deletions to activities; activity durations; activity time frames; or activity predecessors and successors will not automatically determine that extension or reduction of contract time is warranted or due the Contractor.

Time extensions for performance will be considered only to the extent that the Contractor’s current scheduled milestone dates exceed contract milestone dates.

Float is not for exclusive use by or benefit of either the Government or the Contractor.

Perform time impact analyses using data in most recent approved schedule update prior to change or delay event.

Prepare proposed revised schedule and narrative description describing and highlighting changes or delays.

Prepare summary comparing results of two schedule analyses: One using current schedule data from last approved updated schedule prior to event requiring analysis, and one using proposed schedule data incorporating changes or delays.

Show contract milestones and activities whose periods of performance have shifted as result of change which affects production and/or manufacture schedules, material orders, construction seasons, and labor and/or equipment utilization.

For activities directly affected by change or delay, include the current and proposed items:

Activity description.

Types and quantities of major pieces of equipment, principal manpower, and pacing materials (materials that affect activity start, duration, or finish).

Activity duration.

Narrative containing rationale used in developing the proposed logic relationships and activity data.

Data date for impacted schedule used in comparison shall be the same as approved updated schedule data date.

Base schedule comparisons on status of work and available float at time the CO directs or proposes change to the work, the Contractor submits a value engineering change proposal, or when a delay occurs as shown in approved updated schedule.

REVIEW AND EVALUATION

Baseline Schedule:

Within 21 calendar days after receipt of baseline schedule:

CO will approve or not approve proposed baseline schedule.

Upon request from the CO and COR meet for a joint review of proposed baseline schedule.

If schedule is not approved, revise and resubmit within 7 calendar days following date of rejection letter.

Do not proceed with onsite work, except mobilization and surveying, until baseline schedule has been approved by the Government.

Updated schedules:

Submit updated Construction Program schedule monthly.

Submit updated schedule at least seven days prior to the submission of each invoice request for progress payment.

If updated schedule is not approved, revise and resubmit updated schedule within 7 calendar days following date of rejection letter.

The COR will schedule pre-submittal meeting with the Contractor’s representative each month to review draft updated schedule prior to the Contractor submitting RSN 01 32 10-3, Updated Schedule.

Failure to include elements of the work in schedules will not release Contractor from completing required work under the contract.

Performance will be evaluated by the Government using approved CPM schedules.

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

SECTION 01 33 00

SUBMITTALS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

REFERENCE STANDARDS

American Society of Mechanical Engineers (ASME)

ASME Y14.1-2020	Drawing Sheet Size and Format

National Institute of Building Sciences (NIBS)

NIBS NCS-2014	United States National CAD Standard, Version 6

DEFINITIONS

Days: Calendar days.

Required Submittal Number (RSN): Identifies items to be submitted together as a complete submittal.

Submittal Types, as listed in Table 01 33 00A - List of Submittals:

A - Action:

Government will respond as to adequacy of submittal.

Action submittals are considered to be “shop drawings” within terms of clause at FAR 52.236-21, Specifications and Drawings for Construction.

I - Informational:

Government will acknowledge receipt of Informational submittals.

Government may reject an Informational submittal when submittal does not satisfy contract requirements.  Contractor shall correct mistakes or deficiencies in rejected Informational submittals and resubmit.

Informational submittals are considered to be “shop drawings” within terms of clause at FAR 52.236-21, Specifications and Drawings for Construction, except that approval by Government is not required.

SUBMITTAL REQUIREMENTS

In case of conflict between requirements of this section and requirements included elsewhere in these specifications, requirements included elsewhere take precedence.

General:

Prepare in English.

Label with contract number and title, and RSN.

Measurement Units: US Customary Units.

Provide a unique transmittal number for each submittal.

Drawings:

Minimum Identification in Title Block:

Contract number and title.

Contractor’s or supplier’s title and drawing number.

Date.

Reserve 3- by 3-inch space next to title block for review stamps.

Size: D size (22 inches by 34 inches) as defined by ASME Y14.1.

Draw to scale using computer drafting or drafting equipment, unless otherwise specified.

Computer drafted drawings:

In accordance with NIBS NCS.

Electronic file format: Compatible with AUTOCAD, Version 23.1s (2020) or later.

Compile using “eTransmit” utility in AUTOCAD.

Drawing prepared with drafting equipment, when allowed: Lettering shall be neat.

Drawings Designated as “Government Format” in Specifications:

Computer drafted.

Government will provide electronic AutoCAD format template.

Title block and sheet format:

As shown on standard drawing 40-D-7102.

Government will supply templates.

Government will supply specific title block information to be used.

Final Drawings:

Computer drafted.

Government will supply electronic AutoCAD format template.

Show as-built changes, including revision dates, made during installation.  Indicate changes by clouding.

Electronic Files: On Government approved media.

Product Data:

Mark manufacturer's data for commercial products or equipment, such as catalog cut sheets.

Identify manufacturer's name, type, model, size, and characteristics.

Illustrate that product or equipment meets requirements of specifications.

Mark items to be furnished in a manner that will photocopy (no highlighter).

Strike through items that do not apply.

Certifications:

Certifications by a registered professional: Signed and sealed by registered professional.

Manufacturer’s certifications: Signed by authorized representative of manufacturer.

Samples and Color Selection Submittals:

Label with complete manufacturer's product and color identification.

Include type and quantity of materials specified in the referenced section in each “set” of samples.

Samples: Representative of product to be installed.  Xerographic sheets for color selection are not acceptable.

Color Chips: Sample paint chips.  Ink color reproductions are not acceptable.

Label each sample, sample kit, set of color chips, or color chart with contract number and title.

The Government will select the architectural color and patterns after product approval.

SUBMITTALS PROCEDURES

Submit only checked submittals.  Submittals without evidence of Contractor's approval will be returned for resubmission.

Submit complete sets of required materials for each RSN as specified in “Submittals Required” column in Table 01 33 00A - List of Submittals.  A complete set includes all listed items for RSNs with multiple parts.

Submit number of sets specified in “No. of sets to be sent to:” columns in Table 01 33 00A - List of Submittals.

Include the following information in transmittal letters:

Contract number and title.

RSN for each attached submittal.

Responsible code.

Number of sets for each RSN.

Identify submittal as initial or resubmittal.

Resubmittal of submittals not approved:

Mark changes such that they are readily identifiable and show revision date.

Describe reasons for significant changes in transmittal letter.

Resubmit returned submittals within 14 days after receiving comments, unless otherwise directed.

Requirements for initial submittals apply to resubmittals.

More than 1 RSN may be submitted under a transmittal letter, provided responsible code is same.

REVIEW OF SUBMITTALS

Time Required:

Submittal review will require 21 days for review of each submittal or resubmittal, unless otherwise specified.

Time required for review of each submittal or resubmittal begins when complete sets of materials required for a particular RSN are received and extends through return mailing postmark date.

Time in Excess of Specified:

CO may extend contract completion date to allow additional time for completing work affected by excess review time.

Time extension will be to extent that excess review time caused delay to contract completion date.

Time extension will not exceed time used in excess of specified number of days for review of submittals or resubmittals.

Concurrent days of excess review time resulting from review of 2 or more separate submittals or resubmittals will be counted only once in extending contract completion date.

No time extension will be allowed if Contractor fails to make complete action submittals in sequence and within time periods specified.

Adjustment for delay will be made only to extent that:

Approval was required under contract, and.

Requests for approval were properly and in a timely submitted and were approved.

Adjustment will be subject to terms of paragraphs (b) and (c) of the clause at FAR 52.242-14, Suspension of Work; however, no such delay shall be deemed to be a “suspension order” as the term is used in that clause.

Return of Submittals:

Action Submittals:  One set of submittals required for action will be returned either approved, approved with comments, or not approved.

Revise and resubmit submittals not approved.

Do not change designs without approval of CO after drawings, documentation, and technical data have been approved.

Informational Submittals: Government will acknowledge Informational submittals.

Informational submittals will not be returned when they satisfy contract requirements.

Informational submittals that do not satisfy contract requirements may be returned for resubmittal, or additional information may be requested.

TRANSMITTAL

Addresses for codes listed in Table 01 33 00A - List of Submittals:

Kristen A. Ruiz Jimenez, Contracting Officer, Bureau of Reclamation, 555 Broadway Blvd., N.E. Suite 100, Albuquerque, New Mexico 87102.

Cord Everetts, Contracting Officer Representative, Bureau of Reclamation, 555 Broadway Blvd. N.E., Suite 100, Albuquerque, New Mexico 87102

Send original transmittal letter with appropriate number of sets to office listed in “Responsible Code” column in Table 01 33 00A - List of Submittals.

Responsible codes starting with “86-6” are located in Technical Service Center.  Send these submittals to TSC address shown above.

Send copy of transmittal letter with appropriate number of sets to offices that are not responsible code but show “No. of sets to be sent to” in Table 01 33 00A - List of Submittals.

When “No. of sets to be sent to” is 0, send a copy of transmittal letter to that office.

Submittals required by specifications, but not listed in Table 01 33 00A - List of Submittals:

Submit in accordance with this section.

Submit to COR, unless otherwise specified.

PRODUCTS

Not Used

EXECUTION

Not Used

| Table 01 33 00A. - List of Submittals * Submittal Types: A - Action, I - Information ** CO indicates Contracting Officer and COR indicates Contracting Officer’s Representative. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSN | Clause or Section Title | Submittals required |  | Due date or delivery time | Type* | Responsible code | No. of sets to be sent to: ** |  |
|  |  |  |  |  |  |  | CO | COR |
| I-1 | Schedules for Construction Contracts (52.236-15) | Practicable Schedule: Blackline Prints |  | Within 10 days after receipt of Notice of Award | A | COR | 1 | 1 |
| I-2 | Schedules for Construction Contracts (52.236-15) | Annotated Schedule Showing Actual Progress: Blackline Prints |  | As directed by the CO | A | COR | 1 | 1 |
| I-3 | Schedules for Construction Contracts (52.236-15) | Supplementary Schedule or Schedules: Blackline Prints |  | As deemed necessary by the CO. | A | COR | 1 | 1 |
| I-4 | Safety and Health (WBR 1452.223-81) | Safety Program |  | Submitted and accepted before commencing onsite work. See section 3 of RSHS | A | COR | 1 | 1 |
| I-5 | Safety and Health (WBR 1452.223-81) | Monthly Accident Summary Report |  | First day of each month. See paragraph 3.8 of RSHS. | I | CO | 1 | 1 |
| 01 14 10-1 | Use of Site | Land Use and Landscape Rehabilitation Plan |  | At least 28 days before use of Government land | A | COR | 1 | 1 |
| 01 31 22-1 | Coordination Meeting | Draft Agenda |  | Within 14 days after receipt of Notice of Award | A | COR | 1 | 1 |
| 01 32 10-1 | Construction Program | Baseline Schedule |  | Within 14 days after receipt of Notice of Award | I | COR | 1 | 1 |
| 01 32 10-2 | Construction Program | Update Schedule(s) |  | With monthly requests for progress payments. | A | COR | 1 | 1 |
| 01 35 10-1 | Safety Data Sheets | Complete LHM And SDS |  | At least 14 days before jobsite delivery of hazardous material | I | COR | 0 | 1 |
| 01 35 10-2 | Safety Data Sheets | Updated LHM And SDS |  | At least 14 days before jobsite delivery of hazardous material not previously listed | I | COR | 0 | 1 |
| 01 35 20-1 | Safety and Health | Safety Program |  | Within 21 days after receipt of Notice of Award | A | COR | 0 | 1 |
| 01 35 20-2 | Safety and Health | Job Hazard Analysis (JHA)s |  | Within 21 days after receipt of Notice of Award | A | COR | 0 | 1 |
| 01 35 20-2 | Safety and Health | Monthly Accident Summary Report |  | With monthly requests for progress payments. | I | COR | 1 | 1 |
| 01 35 30-1 | Contractor‘s Onsite Safety Personnel | Qualifications |  | Prior to employment | I | COR | 0 | 1 |
| 01 56 10-1 | Protection of Existing Installations | Plan for protecting existing installations |  | Within 21 days after receipt of Notice of Award | A | COR | 0 | 1 |
| 01 57 20-1 | Environmental Controls | Dust abatement plan |  | Within 21 days after receipt of Notice of Award | A | COR | 0 | 1 |
| 01 57 30-1 | Water Pollution Control | Water Management Plan |  | At least 28 days before commencing on site construction work | A | COR | 0 | 1 |
| 01 57 30-2 | Water Pollution Control | Pollution Prevention Plan (> 1 acre) |  | At least 28 days before delivery or storage of oil | A | COR | 0 | 1 |
| 01 57 30-3 | Water Pollution Control | Spill Prevention Control and Countermeasure Plan (SPCC) |  | At least 28 days before delivery or storage of oil | A | COR | 0 | 1 |
| 01 57 50-1 | Tree and Plant Protection | Protection Plan |  | At least 28 days before commencing on site construction work | A | COR | 0 | 1 |
| 01 64 40-1 | Government Furnished Products | Shortage Report |  | At least 2 days after inspection and acceptance of Government Furnish Products | I | COR | 1 | 1 |
| 01 64 40-2 | Government Furnished Products | Damages or loss report |  | At least 2 days after occurrence of damage or acknowledgement of loss of Government Furnish Products | I | COR | 1 | 1 |
| 01 71 20-1 | Surveying | Surveying Plan |  | At least 21 days before beginning survey work | A | COR | 0 | 1 |
| 01 71 20-2 | Surveying | Resumes |  | At least 21 days before beginning survey work | I | COR | 0 | 1 |
| 01 71 20-3 | Surveying | Accuracy Check Results |  | Within 2 days of completing and reducing notes | I | COR | 0 | 1 |
| 01 71 20-4 | Surveying | Completed and Reduced Survey Notes |  | Within 5 days of completing and reducing notes | I | COR | 0 | 1 |
| 01 71 20-5 | Surveying | Workday’s survey notes |  | Within 2 days of completing and reducing notes | I | COR | 0 | 1 |
| 01 74 00-1 | Cleaning and Waste Management | Waste Production and Disposal Plan |  | At least 28 days before starting onsite work | I | COR | 0 | 1 |
| 01 74 00-2 | Cleaning and Waste Management | Waste Production and Disposal Records |  | Within 7 days of waste disposal | I | COR | 0 | 1 |
| 01 78 30-1 | Project Record Documents | As-built Drawings |  | Within 14 days of completion of work | I | COR | 0 | 1 |
| 01 78 30-2 | Project Record Documents | As-built Record of Materials |  | Within 14 days of completion of work | I | COR | 0 | 1 |
| 02 41 10-1 | Staircase Demolition | Demolition Plan |  | At least 14 days prior to starting onsite work | A | COR | 0 | 1 |
| 02 41 10-1 | Staircase Demolition | JHA |  | At least 14 days prior to starting onsite work | A | COR | 0 | 1 |
| 02 41 10-1 | Staircase Demolition | Demolition Pre-Meeting |  | At least 3 days prior to starting onsite work | I | COR | 0 | 1 |
| 02 41 16-1 | Cableway Bridge Demolition | Demolition Plan |  | At least 14 days prior to starting onsite work | A | COR | 0 | 1 |
| 02 41 16-2 | Cableway Bridge Demolition | JHA |  | At least 14 days prior to starting onsite work | I | COR | 0 | 1 |
| 02 41 16-3 | Cableway Bridge Demolition | Demolition Pre-Meeting |  | At least 3 days prior to starting onsite work | A | COR | 0 | 1 |
| 03 30 53-1 | Miscellaneous Cast-In-Place Concrete | Approval Data |  | At least 28 days before starting onsite work | A | COR | 0 | 1 |
| 03 30 53-2 | Miscellaneous Cast-In-Place Concrete | Test Reports |  | Verbally at time of test. Include hard copies with monthly requests for progress payments. | A | COR | 0 | 1 |
| 03 62 20-1 | Non-shrink Grout for Equipment and Metalwork | Package non-shrink grout product data |  | At least 28 days before starting onsite work | A | COR | 0 | 1 |
| 05 12 10-1 | Structural Steel Framing | Dimensional Drawings |  | At least 21 days before starting fabrication work | A | COR | 0 | 1 |
| 05 12 10-2 | Structural Steel Framing | Detail Drawings and Data |  | At least 21 days before starting fabrication work | A | COR | 0 | 1 |
| 05 12 10-3 | Structural Steel Framing | Test Reports |  | Verbally at time of test. Include hard copies with monthly requests for progress payments. | I | COR | 0 | 1 |
| 05 12 10-4 | Structural Steel Framing | Final Drawings and Data |  | Within 14 days of completion of work | I | COR | 0 | 1 |
| 09 97 00-1 | Metalizing and Coatings | Approval Data including color |  | At least 21 days before finalizing approval data | A | COR | 0 | 1 |
| 09 97 00-2 | Metalizing and Coatings | Final Approval Data |  | At least 14 days before starting work | A | COR | 0 | 1 |
| 09 97 00-3 | Metalizing and Coatings | Qualifications |  | At least 14 days before starting work | I | COR | 0 | 1 |
| 09 97 00-4 | Metalizing and Coatings | Quality Control Plan |  | At least 14 days before starting work | A | COR | 0 | 1 |
| 10 18 00-1 | Informational Kiosks | Approval Data |  | At least 28 days before ordering | A | COR | 0 | 1 |
| 31 23 02-1 | Compacting Earth Materials | Test Results |  | Verbally at time of test. Include hard copies with monthly requests for progress payments. | I | COR | 0 | 1 |
| 32 92 20-1 | Seeding and Soil Supplements | Seeding Plan |  | At least 28 days before starting onsite work | A | COR | 0 | 1 |
| 32 92 20-2 | Seeding and Soil Supplements |  | Certifications | At least 28 days before starting onsite work | I | COR | 0 | 1 |

END OF SECTION

SECTION 01 35 10

SAFETY DATA SHEETS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

DEFINITIONS

LHM: List of Hazardous Materials.

SDS: Safety Data Sheet.

Referred to as Material Safety Data Sheets in the clause at 52.223-3, Hazardous Material Identifications and Material Safety Data - Alternate 1.

APPLICATION

For the purposes of this contract, “delivered under this contract” in paragraph (b) of the clause at FAR 52.223-3, Hazardous Material Identification and Material Safety Data - Alternate 1, includes:

Materials delivered to Government.

Materials incorporated into work.

Materials used by the Contractor during contract performance at the jobsite.

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 35 10-1, Complete LHM and SDS.

RSN 01 35 10-2, Updated LHM and SDS:

Comply with paragraph (e) of clause at FAR 52.223-3, Hazardous Material Identification and Material Safety Data - Alternate 1.

In addition to requirements in Table 01 33 00A, submit copies of updated LHM and SDS to COR at least 14 days before delivering hazardous materials to job site.

DELIVERY

Do not deliver hazardous materials to jobsite which are not included on original or previously updated LHM and SDS before receipt of updated LHM and SDS by COR.

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

SECTION 01 35 20

SAFETY AND HEALTH

GENERAL

MEASUREMENT AND PAYMENT

      - Cost:

        - Include in the prices offered in the Price Schedule for other items of work.

REFERENCE STANDARDS

      - Bureau of Reclamation (USBR)

RSHS 2024	Reclamation Safety and Health Standards including revisions posted at: https://www.usbr.gov/safety/rshs/index.htm

FIST 1-1 (2012) 	Hazardous Energy Control Program available at: www.usbr.gov/power/data/fist_pub.html

Applicable Tribal and State Safety and Health Regulations for Construction.

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 35 20-1, Safety Program:

Written safety program in accordance with of RSHS.

Cover aspects of on-site and applicable off-site operations and activities associated with this contract.

Follow the outline in Appendix B of RSHS

Will not be accepted for review by the COR unless it addresses, in order, lettered and numbered per Appendix B, a narrative for each applicable item in the outline.  For items in the outline that do not apply to this contract write “Not applicable to Contract” next to the number/letter.

A generic company safety program is not acceptable. Safety Program must be site specific to this contract.

Submitted and accepted prior to commencing onsite work, including mobilization.

RSN 01 35 20-2, Job Hazard Analysis List:

Provide a list of JHA’s that shall be submitted throughout the project.

RSN 01 35 20-3, Monthly Accident Summary Report:

In accordance with RSHS.

DOCUMENTATION AND RECORDS

Prepare and retain all safety plans, programs, training content, and training records that are applicable to the scope of the work and make them available to the COR unless they are already included in the written safety program.  Some examples include but are not limited to:

Emergency Action Plans Written Program and Training Records.

Fall Protection Written Program and Training Records.

Fire Protection and Prevention Written Program and Training Records.

Flammable and Combustible Liquids Written Spill Control Plan.

General Safety and Health Provisions, Written Safety Program, and Regular Safety Inspection Records.

Hazard Communication Written Program and Training Records.

Ladder Written Compliance Program and Training Records.

Bloodborne Pathogens Exposure Control Plan, Training Records.

Occupational Noise Exposure Written Program and Training Audiometric Testing Records.

Personal Protective Equipment (PPE) Hazard Assessments and Training Records.

Power Operated Hand Tools Training Records.

Respiratory Protection Written Program and Training Records.

Safety Training and Education Written Program and Training Records.

Steel Erection Training Records, if applicable.

Welding, Cutting, and Brazing Written Program and Training Records.

SAFETY AND HEALTH

Provide and maintain a work environment and procedures that:

        - Safeguard the public and Government's personnel, and Contractor employees exposed to Contractor operations and activities.

Avoid interruptions of site operations and delays in project completion dates.

Control costs in contract performance.

Do not begin on site work, including mobilization, until the COR accepts the Safety Program.

Participate in Contractor Safety Program Review meeting prior to mobilization.

Minimum crew shall consist of no less than two (2) people, unless approved by COR.

Develop Job Hazard Analyses for each distinct phase of work under the contract.

Do not begin a phase of work until a JHA is acceptable to COR and shared with construction employees.

Activities involving hazardous materials shall have the appropriate Safety Data Sheet(s) attached to the JHA.

Comply with RSHS, OSHA 29 CFR 1926, and FAR 52.236-13.

The RSHS and revisions in digital format are accessible at https://www.usbr.gov/safety/rshs/index.html

Construction Safety and Health Standards promulgated by the Secretary of Labor may be obtained from any regional or area office of the Occupational Safety and Health Administration of the U.S. Department of Labor.

Correct safety and health violations identified by the CO or the COR.

When the Contractor fails or refuses to correct a compliance directive, the CO  may issue an order to stop all or part of the work.

When satisfactory corrective action is taken, an order to resume work will be issued.

The Contractor shall not be entitled to extension of time or to claim for damage or to additional compensation by reason of either the directive or stop order.

Failure of the CO to order discontinuance of the Contractor’s operations shall not relieve the Contractor’s responsibility for the safety of personnel and property.

Maintain accurate record of and report to the CO the following occurrences during performance of this contract:

Death.

Occupational disease.

Traumatic injury to employees or the public.

Property damage is excess of $2,500.

The rights and remedies of the Government provided in this section are in addition to any other rights and remedies provided by law or under this contract.

In the event there is a conflict between requirements contained in RSHS, specification paragraphs, Contractor’s approved Safety Program, referenced safety and health codes, and standards, or the U.S. Department of Labor Construction Safety and Health Standards, promulgated under Section 107 of the Contract Work Hours and Safety Standards Act (40 U.S.C. 327 et seq.), as amended, the more stringent requirement shall prevail.

Provide appropriate safety barricades, signs, and warnings around field welding operations.

The Contractor shall be responsible for providing medical and emergency assistance in a timely manner consistent with recognized emergency protocols.

In instances where local emergency responders are not readily available, the responsibility to provide emergency services shall default back to the Contractor.

Perform all training as required by federal, state, and local regulations prior to any designating emergency responders.

Training records must be submitted to the COR upon request.

In no case shall an employee perform work until all required training is complete.

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

SECTION 01 35 30

CONTRACTORS ONSITE SAFETY PERSONNEL

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

REFERENCE STANDARDS

Bureau of Reclamation (USBR)

RSHS-2024	Reclamation Safety and Health Standards, including revisions posted at https://www.usbr.gov/safety/rshs/index.html

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 35 30-1, Qualifications:

Contractor’s Onsite Safety Represent.

QUALIFICATIONS

Contractor’s Onsite Safety Representative:

Competent supervisory employee with current safety and health related training, experience and duties on at least three projects of similar nature, size, complexity, and work to be performed as the current project

OSHA Construction 30-Hour Training Course including applicable elective subjects, for example Concrete and Masonry Construction; Cranes, Hoists; Excavations; Fire Protection and Prevention; Materials Handling, Storage, Rollover Protective Structures and Overhead Protection; and Signs; Signals and Barricades; Powered Industrial Vehicles; Safety and Health Programs; Scaffolds; Steel Erection; Tools - Hand and Power; Welding and Cutting. Current first aid CPR certification.

Include resume with current telephone numbers of references, description of safety representative responsibilities, and copies of training certifications.

APPLICATION

Designate an employee as the Contractor’s Onsite Safety Representative prior to start of construction.

QUALITY ASSURANCE

Contractor’s Onsite Safety Representative:

The effectiveness of the Contractor’s Onsite Safety Representative in prosecuting the safety program will be subject to continued review and approval by the COR.

Should the Contractor’s onsite safety representative’s effort be deemed insufficient the Contractor may be required to provide the services of a qualified, full-time safety and health professional at no additional cost to the Government.

PRODUCTS

Not Used

EXECUTION

END OF SECTION

SECTION 01 42 10

REFERENCE STANDARDS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

REFERENCE STANDARDS

Referenced editions of standard specifications, codes, and manuals form a part of this specification to the extent referenced.

These specifications take precedence when conflicting requirements occur between specifications and referenced standards.

JOBSITE REFERENCE STANDARDS

Maintain at fabrication site, access to referenced standard specifications, codes, and manuals required for work in progress at fabrication site.  Make available for use by the Government.

Maintain onsite, access to referenced standard specifications, codes, and manuals required for onsite work in progress.  Make available for use by the Government.

AVAILABILITY

Code of Federal Regulations (CFR):

Available online, authorized by the National Archives and Records Administration (NARA) and the Government Printing Office (GPO), at http://www.gpo.gov/fdsys/search/home.action.

Federal Specifications, Standards, and Commercial Item Descriptions; and Military Specifications:

Copies of Federal Specifications, Standards, and Commercial Item Descriptions may be obtained from GSA Federal Supply Service, see the provision at FAR 52.211-1, Availability of Specifications Listed in the GSA Index of Federal Specifications, Standards and Commercial Item Descriptions, FPMR Part 101-29.

Copies of Military Specifications may be obtained from Department of Defense, see the provision at FAR 52.211-2, Availability of Specifications, Standards, and Data Item Descriptions Listed in the Acquisition Streamlining and Standardization Information System (ASSIST).

Bureau of Reclamation Documents:

Reclamation Safety and Health Standards (RSHS) may be downloaded at https://www.usbr.gov/safety/rshs/index.html.

Hard copies of RSHS, stock number 024-003-00204-6, may be purchased from The Superintendent of Documents at the U.S. Government Printing Office (GPO), phone number 202-512-1800.  Hard copies of RSHS are subject to revisions posted on the site shown above.

GPO online bookstore: http://bookstore.gpo.gov/actions/GetPublication.do?stocknumber=024-003-00204-6.

Bureau of Reclamation Standard Specifications are designated with an M- number.  Copies of individual standards may be obtained from Bureau of Reclamation, Attn: 86-68510, P.O. Box 25007, Denver CO 80225-0007.  Specify standard(s) needed when requesting a copy.

Bureau of Reclamation manuals and other publications including significant scientific, technical, and engineering works are available from the National Technical Information Service (NTIS).  Information regarding availability and pricing may be obtained by contacting NTIS at the following address:

United States Department of Commerce

National Technical Information Service

5285 Port Royal Road

Springfield, VA  22161

Telephone: 703-487-4650 or 1-800-553-6847

Industrial and Governmental Documents:

When a reference has a joint designation (e.g. ANSI/IEEE) these specifications generally cite the proponent organization (e.g. IEEE).

Addresses for obtaining industrial and governmental (other than Federal and Bureau of Reclamation specifications and standards) specifications, standards, and codes are listed in Table 01 42 10A - Addresses for Specifications, Standards, and Codes.

| Table 01 42 10A - Addresses for Specifications, Standards, and Codes |  |  |
| --- | --- | --- |
| Acronym | Name and Address | Telephone |
| ACI | American Concrete Institute 38800 Country Club Dr. Farmington Hills, MI 48331-3439 https://www.concrete.org/home.aspx” | 248-848-3700 |
| AGC | Associated General Contractors of America 333 John Carlyle Street, Suite 200 Alexandria, VA 22314 www.agc.org | 703-548-3118 |
| AISC | American Institute of Steel Construction One East Wacker Drive, Suite 3100 Chicago, IL 60601-2001 www.aisc.org | 312-670-2400 |
| ANSI | American National Standards Institute 1819 L. Street, NW. Washington, DC 20036 www.ansi.org | 202-293-8020 |
| APA/EWA | APA-The Engineered Wood Association P.O. Box 11700 Tacoma, WA 98411-0700 www.apawood.org | 253-565-6600 |
| ASTM | ASTM International P.O. Box C700 100 Barr Harbor Drive West Conshohocken, PA 19428-2959 www.astm.org | 610-832-9585 |
| AWS | American Welding Society 550 NW LeJeune Road Miami, FL 33126 www.amweld.org | 800-443-9353 305-443-9353 |
| ICRI | International Concrete Repair Institute 1000 Westgate Drive, Suite 252 ¦ St. Paul, Minnesota 55114 http://www.icri.org/ | 651-366-6095 |
| NACE | NACE International 1440 South Creek Drive Houston, TX 77084 www.nace.org | 281-228-6200 |
| NFPA | National Fire Protection Association One Batterymarch Park P.O. Box 9101 Quincy, MA 02269-9101 www.nfpa.org | 800-344-3555 617-770-3000 |
| NIBS | National Institute of Building Sciences 1090 Vermont Avenue, NW., Suite 700 Washington, DC 20005-4905 www.nibs.org | 202-289-7800 |
| SSPC | SSPC: The Society for Protective Coatings 40 24th Street, 6th Floor Pittsburgh, PA 15222-4656 www.sspc.org | 800-837-8303 412-281-2331 |

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

SECTION 01 46 00

QUALITY PROCEDURES

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

DEFINITIONS

Government Contract Quality Assurance:  (from FAR 46.101) Various functions, including inspection, performed by the Government to determine whether a contractor has fulfilled the contract obligations pertaining to quality and quantity.

Contractor Quality Assurance/Quality Control (QA/QC): Activities performed by the Contractor to ensure work conforms to contract requirements.

The clause at FAR 52.246-12 - Inspection of Construction, requires the Contractor to establish an inspection system to ensure quality.

Contractor QA/QC includes activities in addition to specified Contractor Quality Testing to ensure work conforms to contract requirements.

Contractor Quality Testing: Specified tests shall be performed by the Contractor.

The Government will evaluate results of these tests when determining acceptability of work.

The Contractor may use the test results as part of Contractor QA/QC.

The Government anticipates that these tests will be part of the Contractor’s QA/QC program, however the tests do not relieve the Contractor of maintaining adequate quality system in accordance with the clause at FAR 52.246-12 - Inspection of Construction.

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

This page intentionally left blank.

SECTION 01 51 00

TEMPORARY UTILITIES

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the price schedule for other items of work.

REFERENCE STANDARDS

Institute of Electrical and Electronics Engineers (IEEE)

IEEE C2-2023	National Electrical Safety Code (NESC)

TEMPORARY ELECTRICITY

Provide electric power required for construction.

Provide generators, transmission lines, distribution circuits, transformers, and other electrical equipment and facilities required for obtaining power and distributing power to points of use.

Comply with IEEE C2 clearances and spacing for temporary communications and supply lines.

TEMPORARY WATER

Water for construction purposes is available from Heron Reservoir within a 2-mile round trip using on-highway equipment.

Make arrangements with COR for use of water.

Government will designate locations from which water may be obtained.

No charge will be made for water obtained from this source.

Contractor shall record volume of water used for construction purposes and report to COR monthly.

If Government water source becomes unavailable, Contractor shall provide water for construction purposes.

Use water which meets specified requirements for water used in concrete, soil-cement, masonry, grouting, and other permanent work.

Provide means of conveying water to points of use.

SANITARY FACILITIES

Contractor shall provide sanitary facilities.

PRODUCTS

Not Used

EXECUTION

REMOVAL

Remove temporary equipment and facilities upon completion of work under this contract.

END OF SECTION

SECTION 01 55 00

VEHICULAR ACCESS AND PARKING

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

REGULATORY REQUIREMENTS

Meet requirements established by jurisdictional authority for use of existing roadways and haul routes; including seasonal or other limitations or restrictions, payment of excess size and weight fees, and posting of bonds conditioned upon repair of damage.

Comply with applicable regulations for haul routes over public highways, roads, or bridges.

SITE CONDITIONS

Rights-of-way for access to work from existing roads will be established by Government.

In accordance with clause at FAR 52.236-10, Operations and Storage Areas, use only established roadways, parking areas, and haul routes.

Subject to clause at FAR 52.249-10, Default (Fixed-Price Construction), unavailability of transportation facilities or limitations thereon shall not become a basis for claims for damages or extension of time for completion of work.

PRODUCTS

MATERIALS

Materials to maintain and repair existing roadways, parking areas, and haul routes: In accordance with requirements of jurisdictional authority.

Materials to construct, maintain, and repair temporary roadways, parking areas, and haul routes: As approved by COR.

Materials to maintain roadways and parking areas constructed under this contract and used by Contractor for construction work: In accordance with specified requirements for construction of those roadways and parking areas.

EXECUTION

EXAMINATION

Investigate conditions of available public or private roads for clearances, restrictions, bridge-load limits, bond requirements, and other limitations that affect or may affect access and transportation operations to and from jobsite.

ESTABLISHED ROADWAYS AND PARKING AREAS

Established roadways and parking areas are available for Contractor's use subject to existing restrictions and approval of the COR.

Existing Park roads to project site may be used for construction traffic.  Tracked vehicles are not allowed.

Designated areas of existing parking facilities may be used by construction personnel.

TEMPORARY ROADWAYS AND PARKING AREAS

Roadways:

Construct temporary channel crossings to allow for unimpeded operations, if approved by COR.

Parking Areas:

Locate as indicated and approved by the COR.

HAUL ROUTES

Perform work on rights-of-way established by Government as necessary to construct and maintain any roads, bridges, or drainage structures required for establishment and use of haul routes for construction operations.

Use existing available public highways, roads, or bridges as haul routes subject to applicable local regulations.

Minimize interference with or congestion of local traffic.

Providing barricades, flaggers, and other necessary precautions for safety of public were haul routes that cross the public highways or roads.

MAINTENANCE

Maintain roadways, parking areas, and haul routes in a sound, smooth condition.

Maintain roadbed, side slopes, structures, and surfacing of roads, trails, and parking areas until completion and acceptance of all work under this contract.  As approved by COR, defer until latest practicable date within specified completion period, placement of surfacing on roads or parking areas subject to heavy and deteriorating use by Contractor's construction operations or equipment.

Maintain surfacing of gravel-surfaced roads and parking areas in a smooth condition until completion and acceptance of work under this contract.

Snow removal for convenience of Contractor or to facilitate work operations of Contractor is considered to be normal required maintenance.

REPAIR

Promptly repair ruts, broken pavement, potholes, low areas with standing water, and other deficiencies to maintain road surfacing and drainage in original or specified condition.

REMOVAL

Remove materials used to construct temporary roadways, parking areas, and haul routes prior to contract completion.  Recycle salvageable materials as approved by COR.

Remove temporary underground work to a depth of (2) feet, backfill in lifts of 6-inches and compacted earthen materials to grade, and regrade site to provide drainage as specified.

END OF SECTION

This page intentionally left blank.

SECTION 01 56 10

PROTECTION OF EXISTING INSTALLATIONS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work, except as specified.

Costs for repair of installations damaged by the Contractor's operations are the Contractor's responsibility.

SUBMITTALS

Submit the following in accordance with Section 01330 – Submittals:

RSN 01 56 10-1, Plan for protecting existing installations.

PROJECT CONDITIONS

Drawings included in these specifications show items of existing materials and equipment but may not show all equipment and materials existing at the jobsite.

Obtain the location of embedded conduit, pipe, cable, ground mat, and other buried items before performing any drilling or cutting of concrete.

PRODUCTS

Not Used

EXECUTION

PROTECTION

Provide protection for personnel and existing facilities from harm due to the Contractor's operations.  Protection shall be subject to approval of the Government.

Arrange protective installations to permit operation of existing equipment and facilities by the Government while work is in progress.

Prevent dust from entering ventilating systems.

ENCLOSURES

Enclose work area to prevent dust, spalls, chips, grit, and other foreign material from endangering personnel and contaminating or damaging equipment.

Provide enclosures to confine the Contractor's operations to immediate work area.

Enclosures shall be subject to approval of the Government.

REMOVAL OF PROTECTIVE INSTALLATIONS

Remove protective installations after purpose has been served.  Materials furnished by the Contractor to provide protection remain property of the Contractor.

REPAIR

Repair, at Contractor's expense, damage to existing installations due to Contractor's operations or Contractor's failure to provide proper protection.  At the Government's option, damage may be repaired by the Government, and the Contractor will be back-charged repair costs.

END OF SECTION

SECTION 01 57 20

ENVIRONMENTAL CONTROLS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work, except as specified.

Costs for damages and work stoppage are the Contractor's responsibility.

See Section 31 02 10 - Water for Dust Abatement, for measurement and payment for applying water used for dust abatement within the rights-of-way

REFERENCE STANDARDS

Bureau of Reclamation (USBR)

RSHS-2024	Reclamation Safety and Health Standards, including revisions posted at http://www.usbr.gov/ssle/safety/RSHS/rshs.html

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 57 20-1, Dust abatement plan:

Provide equipment and source location for water used in dust abatement.

REGULATORY REQUIREMENTS

Comply with Federal, State, and local laws and regulations.

Comply with RSHS.

Conform to most stringent requirements in cases of conflict between specifications, regulatory requirements, and RSHS.

Contractor shall be responsible for damages resulting from dust originating from Contractor operations in accordance with clause at FAR 52.236-7, Permits and Responsibilities.

The CO may stop construction activity in violation of Federal, State, or local laws and additional expenses resulting from work stoppage will be responsibility of Contractor.

DUST CONTROL

Provide dust control and abatement during performance of work.

Prevent, control, and abate dust pollution on rights-of-way provided by Government or elsewhere during performance of work.

Provide labor, equipment, and materials, and use efficient methods to prevent dust nuisance or damage to persons, property, or activities, including, but not limited to, wildlife habitats, dwellings, recreational activities, traffic, and similar conditions.

LIGHT CONTROL

Direct stationary floodlights shall shine downward at an angle less than horizontal.

Shield floodlights so that floodlights will not be a nuisance to surrounding areas.

Direct lighting so that dwellings are not in direct beam of light.

Correct lighting control problems when they occur as approved by the COR.

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

SECTION 01 57 30

WATER POLLUTION CONTROL

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

REFERENCE STANDARDS

Bureau of Reclamation (USBR)

RSHS-2024	Reclamation Safety and Health Standards, including revisions posted at http://www.usbr.gov/ssle/safety/RSHS/rshs.html

Code of Federal Regulations (CFR)

40 CFR, Part 112	Oil Pollution Prevention

Public Law

Sections 311, 402, and 404	Clean Water Act (Public Law 92-500, as amended)

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 57 30-1, Water Management Plan (Less than 1 acre):

Detailed Water Quality Management Plan for construction activities that involve less than 1 acre of land in the vicinity of any stream, flowing or dry watercourse, lake, wetland, reservoir, or underground water source.

Name of person who will be responsible for implementing and carrying out plan.

Relationship of methods and descriptions herein to conditions of required permits specified in article titled "Contractor Responsibilities."

Precautions which will be taken to avoid discharge or accidental spills of pollutants into a river, stream, watercourse, or lake.

Demonstrated compliance with State and local waste disposal, sanitary sewer, or septic regulations.

Methods of handling and treating wastewater, including drawings or maps indicating locations for evaporation or settling ponds, treatment facilities, best management practices to prevent water pollution, and discharge points.  Provide estimates of amount of wastewater which may be handled and treated at each location.

Methods for preventing or controlling runoff and erosion for construction sites, both during and after construction, including:

Access and haul roads,

Stockpile, borrow, and waste areas,

Construction plant and equipment yards,

All excavated surfaces,

Areas containing slurry ponds or water treatment facilities,

Buffer zones; and

Other impacted areas.

Information on vegetative practices, structural control, silt fences, straw dikes, sediment and operator controls, stormwater controls, and solid waste controls.  Address stormwater controls for appropriate stormwater management measures including velocity dissipators.  Address solid waste controls for building materials and offsite tracking of sediment.

RSN 01 57 30-2. Pollution Prevention Plan (Greater than 1 acre):

As required by the stormwater permit for discharges from construction sites.

Include copy of permit[s].

RSN 01 57 30-3, Spill Prevention, Control, and Countermeasure (SPCC) Plan:

Submit when SPCC Plan is required in accordance with 40 CFR, Part 112.

SPCC Plan is required where release of oil and oil products could reasonably be expected to enter or float upon navigable waters of the United States or adjoining shorelines in quantities that may be harmful (40 CFR, Part 110), and aggregate on site oil storage capacity is over 1,320 gallons.  Only containers with capacity of 55 gallons and greater are included in determining on site aggregate storage capacity.

Reviewed and certified by a registered professional engineer in accordance with 40 CFR, Part 112, as required by section 311 of the Clean Water Act (Public Law 92-500 as amended).

REGULATORY REQUIREMENTS

Construction Safety Standards:

Comply with sanitation and potable water requirements of section 7 of RSHS.

Laws, Regulations, and Permits:

Perform construction operations to comply, and ensure subcontractors comply, with:

Applicable Federal, State, and local laws, orders, regulations, and Water Quality Standards concerning control and abatement of water pollution; and terms and conditions of applicable permits issued by permit issuing authority.

If conflict occurs between Federal, State, and local laws, regulations, and requirements, the most stringent shall apply.

Contractor Violations:

If noncompliance should occur, immediately (verbally) report noncompliance to the CO.  Submit specific written information within 2 days.

Violation of applicable Federal, State, or local laws, orders, regulations, or Water Quality Standards may result in the CO stopping site activity until compliance is ensured.

The Contractor shall not be entitled to extension of time, claim for damage, or additional compensation by reason of such a work stoppage.

Corrective measures required to bring activities into compliance shall be at the Contractor's expense.

REQUIRED PERMITS

Wastewater Discharge Permit:

Application:

The Bureau of Reclamation has made application for a permit to discharge pollutants as required under section 402 of the Clean Water Act (Public Law 92-500 as amended).

Terms and Conditions: Comply with terms and conditions as stated in the permit.

Monitoring and Treatment:

Provide monitoring and water treatment, if necessary, to achieve compliance with permit conditions.

Provide recordkeeping required of the section 402 permittee, as stated in the section 402 permit.

Sampling:  Include sampling in monitoring required of the Contractor to meet section 402 requirements, as well as required laboratory tests to determine effluent characteristics.

Monitoring Results:

Provide monitoring results to the CO 2 weeks prior to submittal deadline to appropriate State and/or Environmental Protection Agency (EPA) Regional Administrator.

If Reclamation transfers the section 402 permit to the Contractor, transmit required reports and monitoring results directly to EPA and/or the State.

Send copies of all information transmitted to EPA and/or the State to the CO.

Section 402 Permit:

Contact COR for availability to review permit(s).

If the Bureau of Reclamation holds the Section 402 wastewater discharge permit (permittee), Reclamation will make known the conditions of permit to the Contractor and then may transfer the permit to the Contractor.

Dredge and Fill Permit:

The Bureau of Reclamation has made application for a permit to discharge dredged or fill material into waters of the United States (including wetlands) as required under section 404 of the Clean Water Act (Public Law 92-500 as amended).

If the Bureau of Reclamation is the section 404 dredge and fill permit holder (permittee), Reclamation will make known the conditions of permit to the Contractor and then may transfer the permit to the Contractor.

Stormwater Discharge Permit Associated with a Construction Site:

Notice of Intent (NOI):

Both the Bureau of Reclamation and the Contractor shall sign the NOI to obtain coverage under a stormwater general permit to control stormwater discharges from the construction site as required under section 402 of the Clean Water Act (Public Law 92-500, as amended).

Pollution Prevention Plan:

The Contractor shall prepare a Pollution Prevention Plan as required by the permit.

Comply with terms and conditions to obtain and maintain this stormwater discharge permit.

Monitoring and Water Treatment:

Provide monitoring and water treatment, if necessary, to achieve compliance with applicable Water Quality Standards.

Provide the recordkeeping required by the stormwater discharge permit associated with construction activity.

CONTRACTOR RESPONSIBILITIES

Permits:

Any permits obtained by the Bureau of Reclamation are exceptions to the clause at FAR 52.236-7, Permits and Responsibilities, which requires the Contractor to obtain necessary licenses and permits.

Monitoring:

Conduct monitoring in order to meet the requirements of the permits which may include:

Sampling,

Site inspections, and

Required laboratory tests to determine effluent characteristics.

Reporting Results:

The Government will report required monitoring results to appropriate agencies.  The section 402 wastewater discharge permit has specific reporting requirements for the permittee for noncompliance when effluent limitations are exceeded.

Recordkeeping:

Retain records and data required by permits.

PRODUCTS

Not Used

STRAW BALES

Straw bales, if used: Certified weed free.

EXECUTION

POLLUTION CONTROLS

Control pollutants by use of sediment and erosion controls, wastewater and stormwater management controls, construction site management practices, and other controls including State and local control requirements.

Sediment and Erosion Controls:

Establish methods for controlling sediment and erosion which address vegetative practices, structural control, silt fences, straw dikes, sediment controls, and operator controls as appropriate.

Institute stormwater management measures as required, including velocity dissipators, and solid waste controls which address controls for building materials and offsite tracking of sediment.

Wastewater and Stormwater Management Controls:

Pollution prevention measures:

Use methods of dewatering, unwatering, excavating, or stockpiling earth and rock materials which include prevention measures to control silting and erosion, and which will intercept and settle any runoff of sediment-ladened waters.

Prevent wastewater from general construction activities such as drain water collection, aggregate processing, concrete batching, drilling, grouting, or other construction operations, from entering flowing or dry watercourses without the use of approved turbidity control methods.

Divert stormwater runoff from upslope areas away from disturbed areas.

Turbidity prevention measures:

Use methods for prevention of excess turbidity which include, but are not restricted to, intercepting ditches, settling ponds, gravel filter entrapment dikes, flocculating processes, recirculation, combinations thereof, or other approved methods that are not harmful to aquatic life.

Wastewater discharged into surface waters shall meet conditions of section 402, the National Pollutant Discharge Elimination System (NPDES) permit.

Do not operate mechanized equipment in waterbodies without having first obtained a section 404 permit, and then only as necessary to construct crossings or perform the required construction.

Construction Site Management:

Contractor construction operations:

Perform construction activities by methods that will prevent entrance, or accidental spillage, of solid matter, contaminants, debris, or other pollutants or wastes into streams, flowing or dry watercourses, lakes, wetlands, reservoirs, or underground water sources.

Pollutants and waste include, but are not restricted to refuse, garbage, cement, sanitary waste, industrial waste, hazardous materials, radioactive substances, oil and other petroleum products, aggregate processing tailings, mineral salts, and thermal pollution.

Stockpiled or deposited materials:

Do not stockpile or deposit excavated materials or other construction materials near or on, stream banks, lake shorelines, or other watercourse perimeters where they can be washed away by high water or storm runoff or can in any way encroach upon the watercourse.

Petroleum product storage tanks management:

Place oil or other petroleum product storage tanks at least 20 feet from streams, flowing or dry watercourses, lakes, wetlands, reservoirs, and any other water source.

Do not use underground storage tanks.

Construct storage area dikes at least 12 inches high or graded and sloped to permit safe containment of leaks and spills equal to storage tank capacity located in the area plus sufficient freeboard to contain the 25-year rainstorm.

Line-diked areas with an impermeable barrier at least 50 mils thick.

Areas for refueling operations: Lined with impermeable barrier at least 10 mils thick covered with 2 to 4 inches of soil.

END OF SECTION

This page intentionally left blank.

- SECTION 01 57 35

- OIL SPILL PREVENTION

  - GENERAL

    - MEASUREMENT AND PAYMENT

      - Cost:

        - Include in prices offered in the Price Schedule for items of work that require oil spill prevention measures.

        - Contractor is responsible for damage resulting from oil spills resulting from work performed under this contract.

    - REFERENCE STANDARDS

      - ASTM International (ASTM)

        - ASTM D 2103-10	Polyethylene Film and Sheeting

    - REGULATORY REQUIREMENTS

      - Comply with Federal, State, and local regulations for oil-spill and control regulations.

  - PRODUCTS

    - MATERIALS

      - Polyethylene Sheeting: ASTM D 2103.

        - Thickness: 6 mils, minimum.

  - EXECUTION

    - ONSITE SPILL PREVENTION

      - Contractor is responsible for spills or leaks during performance of this contract which occur as result of or are contributed to by the actions of the Contractor’s agents, employees, or subcontractors.

      - Perform following before draining and transferring oil or removing bushing operations:

        - Place polyethylene sheeting below barrels, hoses, and connections used for draining oils.

        - Overlap, triple-fold, and tape adjoining sheets to trap spilled oil and to not contaminate surfaces.

        - Inspect hoses and gaskets used for pumping equipment for holes, cracks or deterioration.  Inspect fittings for proper connections.  Repair or replace damaged hoses, gaskets, and fittings to COR’s satisfaction.

        - Place drip pans under pumps and connections.

        - Place pumps and hoses so they are not a tripping hazard.

        - Have available onsite sorbent materials and cleanup equipment.  Includes but not limited to the following:

          - Shovels.

          - Brooms.

          - Department of Transportation recovery drums.

          - Rags.

          - Vermiculite, minimum: 50 pounds.

          - Sorbent booms and sorbent pads.

          - Solvents.

      - Always monitor pumping operations.

    - SPILLS OR LEAKS

      - Stop source of spill or leak.

      - Stop migration of spill or leak.

      - Place berm of sorbent material around perimeter of spill.

      - Solidify free standing oil.

    - REPORTING

      - Report spills or leaks, regardless of quantity, to COR within 4 hours of their discovery.

      - Make follow-up written report to COR not later than 7 days after initial report.  Include in a narrative form, as a minimum, the following:

        - Description of item spilled.  Include identity, quantity, and manifest number.

        - Whether or not amount spilled is EPA or State reportable.  If so, when it was reported.

        - Exact time and location of spill including a description of the area involved.

        - Containment procedures initiated.

        - Summary of communications with media or Government officials other than COR.

        - Description of clean-up procedures employed or to be employed at the site.

      - Disposal location and methods.

    - CLEANUP

      - Clean up spills or leaks to satisfaction of COR and in a manner that complies with applicable Federal, State, and local laws and regulations.

      - Pump spills are not practical for solidifying into Department of Transportation drums.  Label and date the drums.

      - Place contaminated porous solids, such as, but not limited to, soil, asphalt, wood, and paper into Department of Transportation removable head drums.  Label and date the drums.

      - Decontaminate non-porous materials using a solvent, such as xylene, toluene, hexane, or kerosene.  Place rags and solvents used in decontamination in Department of Transportation removable head drums.  Label and date the drums.

      - If spill exceeds onsite control and cleanup capabilities, hire outside subcontractor specializing in environmental cleanup to perform cleanup.

      - For cleanup operations, document extent of spill, effectiveness of cleanup operations, and point in time when the environment has been decontaminated as approved by COR.

      - Perform cleanup at no additional cost to Government.

    - DISPOSAL

      - Handle and dispose of spilled or leaked materials contaminated with 50 ppm or greater polychlorinated biphenyls in accordance with Section 02 84 30 - Removal and Disposal of Polychlorinated Biphenyls (PCBs).

      - Handle and dispose of spilled or leaked materials not contaminated or contaminated with less than 50 ppm polychlorinated biphenyls in accordance with applicable Federal, State, and local regulations.

- END OF SECTION

This page intentionally left blank.

SECTION 01 57 50

TREE AND PLANT PROTECTION

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work, except as specified.

Costs for repair or treatment of injured vegetation and replacement of trees or shrubs are the Contractor's responsibility.

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 57 50-1, Protection Plan:

Description of protective barriers or other methods used to protect vegetation from damage or injury caused by construction operations.

PRODUCTS

REPLACEMENT TREES AND SHRUBS

Species: Same as removed tree or shrub or other species approved by the COR.

Size: Same size as removed tree or shrub, or maximum practicable size that can be planted and sustained in the particular environment as approved by the COR.

EXECUTION

PRESERVATION AND PROTECTION

Preserve natural landscape and preserve and protect existing vegetation not required or otherwise authorized to be removed.

Submit requests to remove vegetation not specifically required to be removed to the COR.

Conduct operations to prevent unnecessary destruction, scarring, or defacing of natural surroundings in the vicinity of the work.

Move crew and equipment within the rights-of-way and over routes provided for access to the work in a manner to prevent damage to grazing land, crops, or property.

Protect vegetation from damage or injury caused by construction operations, personnel, or equipment by using protective barriers or other methods approved by the COR.

Minimize, to the greatest extent practicable, clearings and cuts through vegetation.  Irregularly shape authorized clearings and cuts to soften undesirable aesthetic impacts.

Do not use trees for anchorages except in emergency cases or as approved by the COR.

For such use, wrap the trunk with a sufficient thickness of approved protective material before rope, cable, or wire is placed.

Submit requests to use trees for anchorage, except for emergencies.  Include description of protective material.

Use safety ropes where tree climbing is necessary; do not use climbing spurs.

REPAIR, TREATMENT, OR REPLACEMENT

The Contractor is responsible for injuries to vegetation caused by Contractor operations, personnel, or equipment.

Employ the services of an experienced arborist or licensed tree surgeon to direct repair, treatment, and replacement of injured vegetation.  Submit qualifications of experienced arborist or licensed tree surgeon to COR prior to employment.

Repair or treat injured vegetation without delay and as recommended by and under direction of an experienced arborist or licensed tree surgeon.

Remove and dispose of trees or shrubs not required or otherwise authorized to be removed that, in the opinion of the COR, are injured beyond saving.

Replace removed tree or shrub with tree or shrub approved by the COR.

Plants that are protected species require multiple plants of the same species to be replaced for each plant removed.  See Section 01 57 60 - Protected Species.

For a period of one growth year, support each plant as required, water, and maintain replacement trees and shrubs.

Remove and replace planted trees or shrubs that die within the one growth year period.  Maintain such replacements for a period of one growth year from the date of replacement.

END OF SECTION

SECTION 01 60 00

PRODUCT REQUIREMENTS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

When the Price Schedule includes a separate item for furnishing a material, includes cost of furnishing, hauling, storing, and handling in the price offered in the Price Schedule for the item.

When the Price Schedule does not include a separate item for furnishing a material, include cost of furnishing, hauling, storing, and handling in the price offered in the Price Schedule for work for which the material is required.

DEFINITIONS

Essential Characteristics: As used in these specifications, the term "essential characteristics" is synonymous with the term "salient characteristics."

Salient Characteristics: Those qualities of an item that are essential to ensure that the intended use of the item can be satisfactorily realized.

REFERENCE STANDARDS

American Society of Mechanical Engineers (ASME)

ASME B1.1-2003(2008)	Unified Inch Screw Threads, (UN and UNR Thread Form)

ASME B1.20.1-2013	Pipe Threads, General Purpose, Inch

Bureau of Reclamation (USBR)

RSHS-2024	Reclamation Safety and Health Standards, including revisions posted at http://www.usbr.gov/ssle/safety/RSHS/rshs.html

DELIVERY, STORAGE, AND HANDLING

Transport and handle manufactured products in accordance with manufacturer's instructions.

Store and protect manufactured products in accordance with manufacturer's instructions and RSHS.  Obtain instructions from the manufacturer before delivery of materials to jobsite.  Maintain a copy of instructions at jobsite.

Protect materials from adverse effects of moisture, sunlight, ultraviolet light, or weather during storage at jobsite.

Remove and replace damaged items with new items.

Store curing compounds, sealants, adhesives, paints, coatings, sealers, joint compounds, grouts, and similar products at the temperature and environmental conditions recommended by the manufacturer.

PRODUCTS

MATERIALS

Provide materials required for completion of work except Government-furnished materials specified in Section 01 64 40 - Government-Furnished Products.

Provide type and quality described in these specifications. Make diligent effort to procure specified materials from all available sources.

Furnish new materials conforming to referenced standards unless otherwise specified.

For materials not covered by these or referenced specifications, furnish materials of standard commercial quality.

If materials to be used deviate from or are not covered by recognized specifications and standards, submit, for approval, justification for and exact nature of the deviation, and complete specifications for materials proposed for use.

Make parts accurately to standard gauge where possible.

Use unified screw threads conforming to ASME B1.1 or B1.20.1 for threads, including but not limited to those of bolts, nuts, screws, taps, pipes, and pipefittings.

For internal connections only, the Contractor may deviate from ASME standards, provided a complete set of taps and dies are furnished as required to facilitate repair or replacement.

Permanently mark fasteners with a symbol identifying the manufacturer and with symbol(s) indicating grade, class, type, and other identifying marks in accordance with reference or applicable standard.

SUBSTITUTIONS

If materials required by these specifications become unavailable, because of Government priorities or other causes, substitute materials may be used.

Obtain written approval to use substitute materials from the CO.  State in the request for approval the amount of the adjustment, if any, to be made in favor of the Government.

The Government's determination as to whether substitution will be permitted and as to what substitute materials may be used, shall be final and conclusive.

If approved substitute materials are of less value to the Government or involve less cost to the Contractor than specified material, a contract adjustment will be made in favor of the Government.  Where the amount involved or the importance of substitution warrants, a deductive modification to the contract will be issued.

No payments in excess of prices offered in the Price Schedule will be made because of substitution of one material for another or because of use of one alternate material in place of another.

WORKMANSHIP

Accurately manufacture and fabricate materials in accordance with best modern practice and requirements of these specifications, notwithstanding minor errors or omissions therein.

Use liberal factors of safety and adequate shock-absorbing features in designs, especially for parts subjected to variable stress or shock, including alternating or vibrating stress or shock.

Include provisions which prevent components from loosening for shock-absorbing features and parts subject to vibration.

GOVERNMENT CONTRACT SOURCE QUALITY ASSURANCE

Materials will be subject to inspection in accordance with clause at FAR 52.246-12 “Inspection of Construction” at following locations, as determined by the CO:

At place of production or manufacture.

At jobsite.

To allow sufficient time to provide for inspection, submit at time of issuance, copies of purchase orders, including drawings and other pertinent information, covering material on which inspection will be made as advised by the CO, or submit other evidence if such purchase orders are issued verbally or by letter.

Inspection of materials at locations specified above or waiving of inspection shall not be construed as being conclusive as to whether materials and equipment conform to contract requirements under the clause at FAR 52.246-12 “Inspection of Construction, nor shall the Contractor be relieved thereby of the responsibility for furnishing materials meeting the requirements of these specifications.

Acceptance of materials will be made only at the jobsite.

EXECUTION

INSTALLATION

Install products in accordance with manufacturer’s recommendations unless otherwise specified.

END OF SECTION

SECTION 01 64 40

GOVERNMENT-FURNISHED PRODUCTS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Except as specified below for backcharges and expenses to the Contractor, include cost of inspecting, loading, hauling, unloading, storing, protecting, and caring for products furnished by the Government in prices offered in the Price Schedule for Government Furnished Products (GFP).

Back-charges and Expenses to the Contractor:

The Contractor will be charged for, or required to replace, any product lost or damaged after delivery and inspection.

The Contractor will be backcharged for products lost or damaged beyond repair after delivery and inspection or for products not incorporated in the work and not returned to the Government.  The backcharge amount will be the initial Government product cost at point of delivery or the Government replacement cost at point of delivery, whichever is higher.  This amount will include reasonable charges for Government warehousing and handling.

When directed by the CO to replace products lost or damaged by the Contractor, such replacement shall be at the Contractor's expense.

Repair of products damaged by the Contractor shall be at the Contractor's expense.

Estimated Costs:

Estimated costs listed in Table 01 64 40A - Government-Furnished Products are provided so that offerors may include in their offers applicable Federal, State, and local taxes and duties on Government-furnished products.  These costs are not guaranteed, however, and the Government obligates itself only in that the costs are the Government's best estimates at time offers are received.

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 64 40-1, Shortage report.

Written report of shortage in or damage to Government-furnished products.

RSN 01 64 40-2, Damage or loss report

Written report of damage or loss of GFP to Government within 24 hours of becoming aware of such loss.

DELIVERY, STORAGE, AND HANDLING

In Table 01 64 40A - Government-Furnished Products are provided so that offerors may develop a tentative construction program.  Pickup of GFP will be coordinated with the COR and Chama Field Division, Manager and occur between Monday through Friday, less federal holiday during standard office hours of 8 a.m. to 4 p.m. MDT.  The Government obligates itself only to make reasonable efforts to secure and provided pickup of GFP to assist in sequence to permit completion of work within time allowed under a reasonable and orderly construction program.

Load and haul GPP from point of delivery to fabrication or jobsite locations.

If directed by the CO during progress of work, replace products lost or damaged by the Contractor, with equivalent grades of products as approved by the CO and deliver products to the jobsite as directed.

Repair products damaged after delivery to the Contractor, which in the opinion of the CO can be repaired satisfactorily.

Government-furnished products may be furnished with spare parts and other extra products.  Load, haul to jobsite, unload, and protect spare parts and deliver them to the Government at the location directed.

Provide warehouses or other means of protection for products which require storage or protection.

Provide indoor storage for (hardware) items 9 through 23, listed in Table 01 64 40A -Government-Furnished Products.  Miscellaneous metalwork items 1 through 8 can be storage outside area free of direct sunlight and damage.  Approximately 4 square feet of indoor storage and 120 square feet of outdoor storage will be required.

Provide protection for products that require storage including protection after being removed from indoor storage and before installation.

Protect products from distortion, dust, water, or other damage.

Place products on timber blocking furnished by the Contractor, if necessary, for proper protection of products.

Protect painted and coated surfaces from damage during transportation, inspection, handling, and storage.  Protect exposed prime coats of paint or coatings from exposure to direct sunrays and from the weather.

Protect surfaces of products which are furnished unpainted.

The Government reserves the right to direct the Contractor to provide means of protection for Government-furnished products which reasonably might be required for their storage and care; however, the exercise of or failure to exercise this right shall not be deemed to relieve the Contractor of primary responsibility for protecting Government-furnished products

Return unused GFP to the Chama Field Division, as directed.

ACCEPTANCE AT SITE

Inspect, confirm quantities, load, transport, unload, and fabricate new products and materials into staircase and hand railing sections.  Schedule a joint inspection of products with the COR to determine existing condition.  The Contractor shall be responsible for damage sustained by products after this inspection.

After joint inspection, recrate or repackage products.

Report to the CO in writing, within 24 hours after unloading, any shortage in or damage to products when delivered.

PRODUCTS

PRODUCTS

The Government will furnish products listed in Table 01 64 40A - Government-Furnished Products.

| Table 01 64 40A - Government-Furnished Products |  |  |  |  |
| --- | --- | --- | --- | --- |
| Table No. | Description | Estimated Quantity | Estimated Weight (Lbs.) | Estimated Costs |
| 1.) | Choice Steel Company: A36 Steel, 3/16-inch x 1-1/4-inch, length 10 feet strip | 1.0 | 7.8 | $20 |
| 2.) | Choice Steel Company: A36 Steel, 3/16-inch x 1-1/2-inch x 20 feet, length 20 feet strip | 3.0 | 57.4 | $76 |
| 3.) | Choice Steel Company: A36 Steel, 2-1/2-inch x 2 1/2-inch x 3/16-inch, length 20 feet angle | 1.0 | 61.4 | $86 |
| 4.) | Choice Steel Company: A500 Steel, 12-inch x 2-inch x 0.188-inch wall thickness, length 20 feet rectangular tube | 5.0 | 1,708.0 | $2,665 |
| 5.) | Choice Steel Company: A500 Steel, 12-inch x 2-inch x 0.25-inch wall thickness, length 20 feet rectangular tube | 4.0 | 1,792.0 | $4,984 |
| 6.) | Choice Steel Company: A588 Corten steel 1/2-inch x 6-inch, length 20 feet flat bar | 1.0 | 204.0 | $235 |
| 7.) | Choice Steel Company: A588 Corten steel 3/4-inch x 6-inch, length 20 feet flat bar | 1.0 | 306.0 | $362 |
| 8.) | Choice Steel Company: A588 Corten steel 3/4-inch x 12-inch, length 20 feet flat bar | 2.0 | 1,224.0 | $1,642 |
| 9.) | Choice Steel Company: A588 Corten steel 3-inch x 3-inch x 3/16-inch, length 20 feet angle | 3.0 | 222.6 | $261 |
| 10.) | Choice Steel Company: A847 Corten steel 1 1/2-inch x 0.188-inch wall thickness, length 24 feet flat bar | 20.0 | 305.6 | $780 |
| 11.) | Choice Steel Company: A847 Corten steel 4-inch x 0.25 wall thickness, length 24 feet square tube | 2.0 | 586.2 | $658 |
| 12.) | Choice Steel Company: A847 Corten steel 6-inch x 4-inch x 0.25 wall thickness, length 24 feet rectangular tube | 1.0 | 374.9 | $420 |
| 13.) | Lawson Item No. 91611 3/4” x 10” Galv. wedge anchor bolts with lock washers and nuts | 50.0 | 62.5 | $771 |
| 14.) | Lawson Item No. BC502B 5/16” Thru-Hardened steel flat washer | 250.0 | 2.6 | $33 |
| 15.) | Lawson Item No. 1593215 5/16” Plain split-lock washer | 300.0 | 0.4 | $43 |
| 16.) | Lawson Item No. 640 3/8”-16 x 3” Tuff-Torque Hex Cap screw | 50.0 | 5.2 | $45 |
| 17.) | Lawson Item No. 709 5/8”-10 x 5-1/2” Tuff-Torque hex cap screw | 100.0 | 54.6 | $426 |
| 18.) | Lawson Item No. 725 3/4”-10 x 5” Tuff-Torque hex cap screw | 40.0 | 29.3 | $224 |
| 19.) | Lawson Item No. 88447 3/8" Thru-hardened steel flat washer USS | 100.0 | 1.6 | $23 |
| 20.) | Lawson Item No. 88451 5/8" Thru-hardened steel flat washer USS | 100.0 | 8.2 | $99 |
| 21.) | Lawson Item No. 88452 3/4" Thru-hardened steel flat washer USS | 50.0 | 5.2 | $68 |
| 22.) | Lawson Item No. A530 3/8" Alloy steel split lock washer, Zinc | 100.0 | 3.1 | $9 |
| 23.) | Lawson Item No. A534 5/8" Alloy steel split lock washer, zinc | 100.0 | 2.5 | $46 |
| 24.) | Lawson Item No. A535 3/4" Alloy steel split lock washer, zinc | 100.0 | 4.0 | $57 |
| 25.) | Lawson Item No. 91606 1/2" x 7" Wedge anchor bolt | 10.0 | 3.6 | $60 |
| 26.) | Lawson Item No. 1160611 5/16"-18 x 1-1/2" Button head cap screw | 250.0 | 8.5 | $155 |
| 27.) | Lawson Item No. 1151422 5/16"-18 Grade 2 hex head, finish Black oxide | 300.0 | 4.8 | $50 |
| 28.) | McNichols Item No. 60C4999999 Accessory, galvanized type CB saddle clip fastener for 1-13/16” on center bar grating | 18.0 | 2.0 | $17 |
| 29.) | McNichols Item No. 3603250041 Wire mesh, woven, inter-crimp weave, 3”x3” opening, 0.250” (1-3/4 gauge) thickness, 48-inches width by 120 feet length | 7.0 | 345.0 | $1,529 |
| 30.) | McNichols Item No. 4001140440 Accessory, U-Edging, 11-gauge, type 401 18/” opening 1” width and 120” length | 32.0 | 263.0 | $1,054 |
| 31.) | McNichols Item No. 6604310232 Bar grating welded GW-125 19-W-4 spacing, 1-1/4” height by 3/16” bar thickness rectangular bearing bars spaced at 1-3/16” on centers with 1” clear space and cross bars at 4” on centers | 2.0 | 1,092.0 | $2,154 |
| 32.) | McNichols Item No. 660431B212 Stair tread welded checker plate nosing, GW 125 Welded 1-1/4” x 3/16” bearing bars spaced at 1-3/16” on centers and cross bars spaced at 4” on centers 12.125” width and 36” length with standard end plates | 12.0 | 302.0 | $1,206 |
| 33.) | McNichols Item No. 660431B212 Stair tread, welded checker plate nosing, GW 125 welded 1-14” x 3/16” bearing bars spaced at 1-3/16” on center serrated surface, cross bars spaced at 4” on centers 12.125” width at 42” length with standard end plates | 12.0 | 385.0 | $1,358 |
| 34.) | McNichols Item No. 6014180010 Flat bar 1/8” gauge thickness having 1-1/4” width for frame edging and 120” length | 10.0 | 53.0 | $196 |
| 35.) | McNichols Item No. 6604310232 Bar grating GW-125 19-W-4 spacing, 1-1/4” height by 3/16” thickness rectangular bearing bars spaced at 1-3/16” on centers with cross bars spaced 4” on centers, 24” width at 120” length | 1.0 | 182.0 | $645 |
|  | Totals |  | 9,670.0 | $22,457 |

The items listed in Table 01 64 40A - Government-Furnished Products currently shipped with pallets and boxes which increase weight to approximately 9,670 pounds or 4.8 tons.  The GFP estimated current value which includes costs for materials and shipping is rounded to $22,500.

EXECUTION

Not Used

END OF SECTION

SECTION 01 71 20

SURVEYING

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include costs in the Price Schedule for surveying.

DEFINITIONS

GPS: Global Positioning System.

GNSS: Global Navigation Satellite Systems.

LS: Laser/Lidar Scanning.

TPS: Terrestrial Positioning Systems  (i.e. total stations and automatic levels).

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 71 20-1, Surveying Plan:

Describe methods used to confirm Governments drawings and layout of staircase and railing panels.

Describe work layout and survey methods.

Surveying schedule.

Example of field records format on electronic data collection devices.

RSN 01 71 20-2, Resumes:

Construction surveyor.

RSN 01 71 20-3, Accuracy Check Results:

Accuracy check of Government-established primary control.

RSN 01 71 20-4, Completed and Reduced Survey Notes:

Copy of completed and reduced survey notes (electronic or paper) for a survey or portion of survey. Include ASCII coordinate files.

Field records on electronic data collection devices: Include electronic files and paper copies of notes.

RSN 01 71 20-5, Workday's survey notes:

Copies when requested by Government.

PRIMARY CONTROL

The Government has established primary control to be used for establishing work lines and grades.

Primary control consists of benchmarks and horizontal control points in work vicinity as shown on the drawings.

Survey data is based on the following datums (reference frames):

Horizontal:  State Plane NAD 83, U.S. Feet.

Vertical:  NAVD 88, U.S. Feet.

Check and verify primary control and resolve discrepancies with Government before beginning work.

Preserve and maintain primary control points until otherwise authorized.  Government may reestablish damaged or destroyed primary control points and backcharge reestablishment cost to the Contractor.

QUALIFICATIONS

Responsible Surveyor:

Professional and Registered Land Surveyor in New Mexico.

When GPS/GNSS is utilized, shall have working knowledge of geodesy, GNSS/GPS capabilities.

Construction Surveyors:

Under supervision and direction of Professional Land Surveyor.

Minimum 2 years’ experience in charge of construction surveys for construction projects similar in nature to that required by this contract.

In charge of construction surveys for at least 2 projects similar in nature to that required by this contract.

PRODUCTS

SURVEYING MATERIALS AND EQUIPMENT

Provide materials required for surveying work, including, but not limited to, stakes, spikes, steel pins, templates, platforms, and tools.

Except as required to be incorporated in work or left in place, surveying materials remain property of Contractor.

Instruments:

GNSS/GPS: Dual or multi-frequency survey grade receivers using accepted standards of practice.

TPS: Check total station level calibration, vertical index and horizontal collimation monthly as recommended by instrument manufacturer.  Include pre-measurement checks in surveyor reports.

Check and adjust optical plummets, tribrachs, tripods, and leveling bubbles daily and after harsh treatment or long-term storage.  For barometers and thermometers, check regularly for accuracy.

EXECUTION

LAYOUT OF WORK SURVEYS

Establish lines and grades for work layout from Government-established primary control points.

Establish measurements required for work execution to specified tolerances.

Provide stakes, markers, and other survey controls necessary to control, check, and guide construction.  Place and mark controls so COR can monitor progress without the use of survey equipment.

QUANTITY SURVEYS

Perform surveys and computations to determine quantities of work performed or placed during each progress payment period.

Perform surveys necessary for the Government to determine final quantities of work in place.  Final payment quantities will be based on the Government's original terrain data and submitted survey notes and computations.

Perform quantity surveys in presence of an authorized Government representative, unless specifically waived.  Notify the Government at least 24 hours before performing a quantity survey.

Use Government provided AutoCAD Civil 3D 2023 drawing template (DWT), Figure Prefix Database, Linework Code Set to import survey points file and to import survey data and generate 3D surfaces.  Naming utilities for original ground and final ground surfaces include project year, project I.D., abbreviated project name, OG (Original Ground), FG (Final Ground) (e.g., xx-xxx NDM OG_01 or xx-xxx NDM FG_01).

Use Government provided codes and special codes from Figure Prefix Database Manager and Linework Code Set when collecting topographic data points and importing survey data into drawing file.  Refer to Government provided Linework Code Set for Special Codes, Line Segment Codes and Curve Segment Codes.

Survey data points (Numeric Point ID, Northing, Easting, Elevation, and Description) shall be provided to the Government as a comma delimited ASCII text format only (csv). When importing survey point data (project coordinates only) into AutoCAD Civil 3D 2023.  Survey Data Base (SDB), the point file format shall be PNEZD (comma delimited).  See surveyor’s report as shown on drawings for geodesy details.

Geo-reference drawing file in accordance with Government furnished geodesy.

Include in each drawing all metadata (i.e. surveyors report) including but not limited to, control points table associated with specific surveying event, survey methodology, equipment, datums, units, control points used, check shot Northing, Easting and Elevation differences, geodesy, and post processing survey software and version.

Control point table in AutoCAD Civil 3D 2023 survey drawing shall display point number, project coordinates, orthometric heights (elevations), and description or point name. Northings, Eastings and orthometric heights shall be shown to the nearest 0.05 feet. Use point table style “Survey Control Points” provided by Government when inserting survey control table in survey drawing files.

SURVEY REQUIREMENTS

Alignment Staking: 50 feet on tangent and 10 feet on curves.

Slope Staking: 50 feet on tangent and 10 feet on curves and elevation slopes.

Structures: Stake out of structures and checkouts before and during construction.

Laser/Lidar scanning of existing and final structure conditions.

Set additional stakes as required by COR.

As-builts: As required for structures and other features of work.

ACCURACY

Degree of Accuracy:

Alignment of Tangents and Curves: Within 0.1 foot.

Structure Points: Within 0.01 foot, except where installation or operation considerations require tighter tolerances.

DTMs and slope stakes: Within 0.10 foot, horizontally and vertically.

Horizontal Control Surveys: Precision shall be dictated by expected use of control point utilizing accepted standards of practice.

Vertical Control Surveys: Close within 0.05-foot times the square root of the circuit length in miles.

Beginning and ending of each survey shall include check measurement on known survey point(s).

FIELD RECORDS

Record field notes, computations, and other surveying data on electronic data collection devices or in field books.  Field records shall be complete and accurate record of survey.

Record survey data in accordance with recognized professional surveying standards.

Notes or data not in accordance with standard formats will be rejected.

Electronic files of notes on electronic data collection devices: In approved format.

Field books:

Illegible notes or data or erasures in a field book will be sufficient cause for rejection of part or all of field book.

Corrections by ruling or lining out errors will be permitted.

Copied notes or data will not be permitted.

Rejection of part or all of a field book may necessitate resurveying.

Field notes shall include as a minimum:

Daily title page.

Date of survey.

Names and duties of survey crew members.

Pertinent atmospheric conditions, such as temperature, pressure and ppm settings.

Explanatory notes about conditions(s) that might affect accuracy or result of survey.

Instrument data: Manufacturer, Model, Year, and if applicable antenna type(s), equipment setting printout.

Care and adjustment of instruments including pegging the leveling instrument, level rod length checks, use of rod levels, collimation of total stations, prism model and offsets, and other pertinent information.

Survey location.

Diagrams or sketches.

Pertinent record information and references.

Original “raw” data values (without mathematical manipulations and without corrections for errors) of distance, angle, and elevations.

Monuments found or set; with complete descriptions (e.g., found 5/8-inch rebar with 1 ½-inch yellow plastic cap, including markings), and description of stampings.

Consecutively number pages in upper right-hand corner.

END OF SECTION

SECTION 01 74 00

CLEANING AND WASTE MANAGEMENT

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the schedule for other items of work except as specified.

Cost of environmental site assessments are the Contractor’s responsibility.

REFERENCE STANDARDS

Bureau of Reclamation (USBR)

RSHS-2024	Reclamation Safety and Health Standards, including revisions posted at http://www.usbr.gov/ssle/safety/RSHS/rshs.html

Code of Federal Regulations (CFR)

40 CFR 261.3	Definition of Hazardous Waste

49 CFR 171-179	Transportation - Hazardous Waste Regulations

DEFINITIONS

Hazardous waste: Defined as hazardous by 40 CFR 261.3; or by other Federal, State, or local laws or regulations.

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 74 00-1, Waste production and disposal plan:

For each type of waste, list estimated quantity and planned disposal location.

RSN 01 74 00-2, Waste production and disposal records.

For each type of waste, list quantity and disposal location.

Include certifications that waste was properly disposed.

Hazardous wastes manifests.

QUALIFICATIONS

Environmental consultant: Minimum 2 years’ experience in conducting environmental site assessments for similar construction.

REGULATORY REQUIREMENTS

Comply with Federal, State, and local laws and regulations.

Comply with RSHS.

Conform to most stringent requirements in cases of conflict between specifications, regulatory requirements, and RSHS.

PROJECT CONDITIONS

Report waste materials discovered at jobsite to COR.

Cease work in areas where waste may be hazardous until waste materials are investigated by the Government.

If waste is hazardous, the CO may order delays in time of performance or changes in work, or both.

If such delays or changes are ordered, an equitable adjustment will be made in the contract in accordance with applicable clauses of the contract.

PRODUCTS

Not Used

EXECUTION

PROGRESS CLEANING

Keep work and storage areas free from accumulations of waste materials and rubbish.

FINAL CLEANUP

Remove temporary plant facilities, buildings, concrete footings and slabs, rubbish, unused materials, concrete forms, and other similar materials which are not part of permanent work.

Clean equipment and fixtures to a sanitary condition.

Extraordinary Cleanup Requirements:

Removal of cables, bridge timbers, etc. from the Rio Chama after and during the removal of the cableway pedestrian Rio Chama Trail bridge.

DISPOSAL

Nonhazardous waste materials:

Disposal by Removal:

Reuse or recycle waste materials whenever possible.

Dispose of nonhazardous waste materials that are not reused or recycled at appropriately permitted disposal facilities.

Do not burn waste materials.

Do not bury waste materials.

Hazardous Waste Disposal:

Dispose by removal from jobsite.

Dispose of hazardous waste materials that are not recycled at appropriately permitted treatment or disposal facilities.

Transport hazardous waste in accordance with 49 CFR 171-179.

Certification: Certify that waste is disposed of in accordance with Federal, State, and local regulations.

SITE ASSESSMENT

Upon completion of work, perform site assessment at following areas for work done under these specifications:

Hazardous waste accumulation areas.

Petroleum dispensing and storage areas where aggregate storage of petroleum at jobsite was over 110 gallons.

Hazardous material storage areas.

Employ qualified environmental consultants to perform assessments.

Demonstrate and document by appropriate analytical sampling that site contamination is less than State action cleanup levels.

RECORDS

Keep records of types and amounts of waste materials produced.

Keep records of waste material disposal.

END OF SECTION

This page intentionally left blank.

SECTION 01 78 30

PROJECT RECORD DOCUMENTS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for other items of work.

UPDATED RECORD DRAWINGS

Update as-built drawings to document work or services performed for progress payments in accordance with the clauses at FAR 52.232-27 - Prompt Payment for Construction Contracts.

At a minimum, meet monthly with COR to review and approve progress made to full-sized record drawings prior to the submittal for progress payment.  Meeting location to be approved by COR.

Verify with the COR that mark-ups to the drawings are current with the progress of the work prior to submitting invoice for progress payment.

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 01 78 30-1, As-built Drawings:

Certified marked sets: As-built drawings that have been certified by Contractor as up to date and accurate in the description of work.

Submit to COR for approval prior to commissioning, training, and/or Government possession.

For Information Only (FIO) Drawings: Revision date and details.

RSN 01 78 30-2, As-built Record of Materials:

Record of materials used to complete a structure or work.

RECORD DRAWINGS

Maintain 2 sets of full-size prints of contract drawings marked to show accurate and complete records of as-built conditions.

Keep drawings at the jobsite and mark as work progresses.

If For Information Only (FIO) drawings require changes, mark with revisions.

Use the following to represent changes.

Supplementary notes, legends, and details necessary to clearly portray as-built construction.

In lieu of reference, incorporate clarifications from other documents such as RFI’s, where As-Built construction is identified.

Include pictures, sketches or other representations to further explain as-built construction.   Do not provide in lieu of as-built drawings.

Mark and dimension to show variations between actual construction and that indicated or specified in contract documents.

Include buried or concealed construction and utilities.

Include existing items, topographic features, and utility lines revealed during construction which differ from those shown on contract drawings.

Mark to define construction actually provided when choice of materials or methods is permitted in specifications, or where variations in scope or character of work from that of the original contract are authorized.

Mark as-built drawings in the following colors:

Red: Additions.

Green: Deletions.

Blue: Notations necessary for explanation of markings.

Always provide access to drawings for Government’s review.

Following successful commissioning of each unit or Government possession, submit As-builts with changes resulting from commissioning and training to COR for review and approval.

Upon completion of work, sign marked as-built drawings.

        - Sign and date each as-built drawing as certified correct.  If no revisions were necessary to illustrate as-built conditions mark the drawing with “As-Built, No Changes.”

Only certify For Information Only (FIO) drawings which require changes.

AS-BUILT RECORD OF MATERIALS

Where several manufacturers' brands, types, or classes of items have been used in the project, designate specific areas where each item was used.  Key designations to the areas and spaces depicted on contract drawings.

Provide record of materials used in the following format:

Table 01 78 30 - As-Built Record of Materials

| Specification Section | Materials Used (manufacturer's designation and description) | Manufacturer | Location of Use |
| --- | --- | --- | --- |

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

This page intentionally left blank.

SECTION 02 41 10

STAIRCASE DEMOLITION

GENERAL

MEASUREMENT AND PAYMENT

Demolition:

Payment:   Lump sum price offered in the Price Schedule.

DEFINITIONS

Demolition refers to removal and disposal of the existing staircase featuring stringers, bullnose, treads, balusters, handrails, newel posts, attachment/mounting brackets, and anchor bolts.  Do not remove or damage existing cast-in-place (CIP) concrete foundation without notifying and receiving written notice of decision from the Government.

REFERENCE STANDARDS

American National Standards Institute (ANSI):

ANSI A10.1-2024	Pre-Project & Pre-Task Safety and Health Planning

ANSI A10.6-2006 (2016)	Safety and Health Program Requirements for Demolition Operations

Bureau of Reclamation (USBR)

RSHS-2024	Reclamation Safety and Health Standards, including revisions posted at http://www.usbr.gov/ssle/safety/RSHS/rshs.html

SUBMITTALS

Submit the following in accordance with Section 01 33 00 – Submittals.

RSN 02 41 10-1, Demolition Plan:

Submit a comprehensive demolition plan, describing the proposed sequence, methods, and equipment for demolition removal and disposal of exiting staircase wooden members, steel brackets, and hardware such as anchor bolts while minimizing impacts to the cast-in-place concrete foundations.

Define methods and procedures to minimize debris from falling during demolition efforts and removal of anchor bolts.

Include information as related to salvage and recycling of materials.

RSN 02 41 10-2, JHA:

Recognize safety risks and provide best practices to minimize this risk during removal of the existing staircase and railing panels.

RSN 02 41 10-3, Demolition Pre-Meeting:

Contractor shall notify the Government and schedule an onsite meeting to discuss the removal of staircase, hand railing, and rail panels a minimum of 3 days prior to performance of work.

PRODUCTS

NOT USED

EXECUTION

PREPARATION

Erect and maintain temporary barriers and security devices include warning signs and related safety measures for the protection of personnel and public during demolition activities.

ANCHOR BOLT REMOVAL

Removal of anchor bolts shall not damage existing CIP concrete foundation.

If the new mounting locations for steel staircase do not align with the existing anchor bolts or the existing anchor bolts are removed or damage during removal, replace with equal and patch any voids from the removal.

DISPOSAL OF REMOVED MATERIALS AND DEBRIS

Dispose of removed materials, waste, trash, and debris in a safe, acceptable manner, in accordance with local laws.

Burning or burying disposal materials is not allowed onsite.

Removal of trash and debris from the site frequently as to not impact the progress of other work.

Removed materials, trash, and debris shall become the property of the Contractor and shall be removed from the project site and be disposed of in a legal manner. Location of disposal site and length of haul shall be the Contractor's responsibility.

Daily Site Cleanup

Always provide a clean and orderly site.

END OF SECTION

This page intentionally left blank.

SECTION 02 41 16

CABLEWAY BRIDGE DEMOLITION

GENERAL

MEASUREMENT AND PAYMENT

Demolition:

Payment:   Lump sum price offered in the Price Schedule.

DEFINITIONS

Demolition refers to removal and disposal of the existing Cableway Bridge superstructure featuring cables, camps, wooden support members, and hardware.  Do not remove existing substructure consisting of existing cast-in-place concrete anchors.

REFERENCE STANDARDS

American National Standards Institute (ANSI):

ANSI A10.1-2024	Pre-Project & Pre-Task Safety and Health Planning

ANSI A10.6-2006 (2016)	Safety and Health Program Requirements for Demolition Operations

Bureau of Reclamation (USBR)

RSHS-2024	Reclamation Safety and Health Standards, including revisions posted at http://www.usbr.gov/ssle/safety/RSHS/rshs.html

SUBMITTALS

Submit the following in accordance with Section 01 33 00 – Submittals.

RSN 02 41 16-1, Demolition Plan:

Submit a comprehensive demolition plan, describing the proposed sequence, methods, and equipment for demolition removal and disposal of exiting wooden members, cables, clamps, and hardware while minimizing impacts to the Rio Chama.

Define methods and procedures to minimize debris from falling into the Rio Chama during removal.

Include information as related to salvage and recycling of materials.

RSN 02 41 16-2, JHA:

Recognize safety risks and provide best practices to minimize this risk during removal of the existing cableway bridge.

RSN 02 41 16-3, Demolition Pre-Meeting:

Contractor shall notify and schedule an onsite meeting to discuss schedule and methods for removal of existing cableway bridge a minimum of 3-days prior to performance of work.

Meeting can be combined with Staircase Demolition Pre-Meeting.

PRODUCTS

NOT USED

EXECUTION

PREPARATION

Erect and maintain temporary barriers and security devices include warning signs and related safety measures for the protection of personnel and public during demolition activities.

DISPOSAL OF REMOVED MATERIALS AND DEBRIS

Dispose of removed materials, waste, trash, and debris in a safe, acceptable manner, in accordance with local laws.

Burning or burying disposal materials is not allowed onsite.

Removal of trash and debris from so that their presence will not impact the progress of other work.

Removed materials, trash, and debris shall become the property of the Contractor and shall be removed from the project site and be disposed of in a legal manner. Location of disposal site and length of haul shall be the Contractor's responsibility.

Daily Site Cleanup

Work site shall always be clean and orderly.

END OF SECTION

SECTION 03 30 53

MISCELLANEOUS CAST-IN-PLACE CONCRETE

GENERAL

PURPOSE

The Section 03 30 53 – Miscellaneous Cast-In-Place Concrete is included in these specifications, if damage occurs to the existing foundation and the contractor needs to replace or repair the existing foundations.

MEASUREMENT AND PAYMENT

Structural Concrete:

Measurement: Volume of concrete measured to structure neat lines as agreed to by the Contracting Officer for shared repairs.

Volume of openings, recesses, embedment’s, and metalwork larger than 100 square inches in volume will be deducted.

Payment: Cubic feet price offered in the Price Schedule.

Includes cost of labor, materials, formwork, cement, aggregates, additives, reinforcement, mixing, finish work for cast-in-place concrete.

ACRONYMS

NRMCA. -	National Ready Mixed Concrete Association

DEFINITIONS

Supplementary Cementitious Materials (SCM): Cementitious materials other than Portland cement.

REFERENCE STANDARDS

American Concrete Institute (ACI)

ACI 117 – 10(2015)	Tolerances for Concrete Construction and Materials

ACI 201.2R – 16	Guide to Durable Concrete

ACI 301-16	Structural Concrete, Part 1 and 2

ACI 304R-00(2009)	Guide for Measuring, Mixing, Transporting, and Placing Concrete

ACI 305.1-14(2020)	Hot Weather Concreting

ACI 306.1-16	Cold Weather Concreting

ACI 318-19(2022)	Building Code Requirements for Structural Concrete

ASTM International (ASTM)

ASTM C31/C31M-24c	Making and Curing Concrete Test Specimens in the Field

ASTM C33/C33M-24a	Concrete Aggregates

ASTM C39/C39M-24	Compressive Strength of Cylindrical Concrete Specimens

ASTM C42/C42M-20	Obtaining and Testing Drilled Cores and Sawed Beams of Concrete

ASTM C94/C94M-24d	Ready-Mixed Concrete

ASTM C114-24	Chemical Analysis of Hydraulic Cement

ASTM C117-23	Materials Finer than 75-µm (No. 200) Sieve in Mineral Aggregates by Washing

ASTM C136/C136M-19	Sieve Analysis of Fine and Coarse Aggregates

ASTM C138/C138M-24a	Density (Unit Weight), Yield, and Air Content (Gravimetric) of Concrete

ASTM C143/C143M-20	Slump of Hydraulic-Cement Concrete

ASTM C150/C150M-24	Portland Cement

ASTM C157/C157M-24e1	Length Change of Hardened Hydraulic-Cement Mortar and Concrete

ASTM C171-20	Sheet Materials for Curing Concrete

ASTM C231/C231M-24	Air Content of Freshly Mixed Concrete by the Pressure Method

ASTM C260/C260M-24	Air-Entraining Admixtures for Concrete

ASTM C309-19	Liquid Membrane-Forming Compounds for Curing Concrete

ASTM C494/C494M-24	Chemical Admixtures for Concrete

ASTM C595/C595M-24	Blended Hydraulic Cements

ASTM C618-23e1	Coal Fly Ash and Raw or Calcined Natural Pozzolan for Use in Concrete

ASTM C989/C989M-24	Slag Cement for Use in Concrete and Mortars

ASTM C1064/C1064M-23	Temperature of Freshly Mixed Hydraulic-Cement Concrete

ASTM C1074-19e1	Estimating Concrete Strength by the Maturity Method

ASTM C1260-23	Potential Alkali Reactivity of Aggregates (Mortar-Bar Method)

ASTM C1293/C1293M-23a	Determination of Length of Change of Concrete Due to Alkali-Silica Reaction

ASTM C1315-19	Liquid Membrane-Forming Compounds Having Special Properties for Curing and Sealing Concrete

ASTM C1567-23	Determining the Potential Alkali-Silica Reactivity of Combination of Cementitious Materials and Aggregate (Accelerated Mortar-Bar Method)

ASTM C1581/C1581M-24	Determining Age at Cracking and Induces Tensile Stress Characteristics of Mortar and Concrete under Restrained Shrinkage

ASTM C1602/C1602M-22	Mixing Water Used in the Production of Hydraulic Cement Concrete

ASTM D1752-21	Preformed Sponge Rubber Cork and Recycled PVC Expansion Joint Fillers for Concrete Paving and Structural Construction

U.S. Army Corps of Engineers (COE)

COE CRD-C 662-10	Determining the Potential Alkali-Silica Reactivity of Combinations of Cementitious Materials, Lithium Nitrate Admixture and Aggregate (Accelerated Mortar-Bar Method)

International Concrete Repair Institute (ICRI)

ICRI 310.2-2013	Guide for Selecting and Specifying Concrete Surface Preparation for Sealers, Coatings, and Polymer Overlays (formerly No. 03732)

Bureau of Reclamation (USBR)

USBR M-47	Guide to Concrete Repair, Second Edition available at https://www.usbr.gov/tsc/techreferences/mands/mands-pdfs/Guide2ConcreteRepair2015_Final.pdf

USBR Concrete Manual	Concrete Manual, Eighth Edition, Revised Reprint, 1981

NSF International (NSF)

NSF 61-2020	Drinking Water System Components – Health Effects

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 03 30 00-1, Approval Data:

Mix Design: For each concrete mix.

Mixture proportions.

Material sources.

Name and manufacture of each cementitious material.

Name of aggregate source(s).

Product name and manufacturer of admixtures to be used in mix.   Certify those admixtures:

Contain no purposefully added chlorides.

Chloride ion limits in accordance with ACI 201.2R.

Government reserves the right to require submission of samples of concrete materials for testing before or during use in concrete.

Physical properties:

Compressive strength:

Test data: ACI 301, paragraph 4.2.3.4.

Field test data: Performed within past 24 months.

Trial mixtures:

Incorporate admixtures that will be used in production mixes into trial mixes.

Results from trial batches made within past 6 months.

Trial mix test results, three six-inch diameter cylinders each at 1, 7, and 28 calendar days.

Average compressive strength of trial batch cylinders at specified design age.

Resubmit mix design for change in material source or type.

Name and manufacturer of curing compounds, and joint filler.

Include application instructions for curing compound.

Certifications and test reports:

Sealed by Professional Engineer.

Certifications and test reports:

Cementitious materials manufacturer.  Less than 3 months old.

Aggregate producer for:

ASTM C33 physical properties

ASR testing reports for each aggregate source.

Less than 12 months old.

Mixing water: ASTM C1602.

Submittal of certifications and test reports shall not relieve Contractor of responsibility for furnishing materials meeting specified requirements.

RSN 03 30 00-2, Test Reports

Concrete test reports as required by Table 03 30 00E- Contractor Field Quality Testing.

QUALIFICATIONS

ACI Strength Testing Technician: Currently certified ACI Strength Testing Technician.

DELIVERY, STORAGE, AND HANDLING

Furnish legible and digitized batch ticket with each batch of concrete in accordance with ASTM C94.  Deliver ticket to COR at jobsite during batch delivery.

Sponge rubber joint filler storage:

Store in protected area at temperature of 70 degrees F or less.

Do not expose sponge rubber joint filler to direct sun.

PRE-PLACING MEETING

At least 7 calendar days before first concrete placement for pier or slab foundation modifications, meet with OGR and COR for pre-placement meeting.

Coordinate time and place of meeting with OGR and COR at least 7 calendar days before meeting.

Contractor attendees shall be at least the following:

Contractor onsite supervisor.

Concrete forming, placing, and finishing onsite supervisor(s); Contractor or subcontractor(s) employee as applicable.

PRODUCTS

CEMENTITIOUS MATERIALS

Portland cement:

ASTM C150, Type V, unless aggregates have a low ASR risk.

Meet equivalent alkalies requirements of ASTM C150 - Table 2.

Low-alkali limitation for portland cement may be waived when tests of concrete aggregate source show that low-alkali cement is not required for ASR mitigation.  See Concrete Aggregate Materials article (2.03).

Meet false-set requirements of ASTM C150 - Table 4.

Blended hydraulic cement:

ASTM C595, Type HS.

Meet equivalent alkalies requirement of ASTM C595, Table 2, Option G. or Table 3.

SCM:

Pozzolan:

ASTM C618, Class F

Except:

Sulfur trioxide, maximum: 4.0 percent.

Loss on ignition, maximum: 2.5 percent.

In addition:

Meets Effectiveness in Controlling Alkali-Silica Reaction in Table 3 Supplementary Optional Physical Requirements of ASTM C618.

Calcium oxide, maximum: 8.0 percent.

Pozzolan “R” factor less than 2.5.  Pozzolan with this “R” factor shall not decrease sulfate resistance of concrete.

R = (C-5)/F

C: Calcium oxide content of pozzolan in percent determined in accordance with ASTM C114.

F:  Ferric oxide content of pozzolan in percent determined in accordance with ASTM C114.

Slag Cement:

ASTM C 989, Grade 100.

WATER

ASTM C1602, including optional requirements of Table 2.

AGGREGATE MATERIALS

Fine aggregate: ASTM C33

Percent material passing No. 200 sieve: Less than 3 percent.

Coarse aggregate:  ASTM C33, Class 4S, Size No. specified in Table 03 30 00A – Concrete Mixes.

Alkali Silica Reaction (ASR):

Test fine and coarse aggregates in accordance with ASTM C1260 for potential deleterious ASR.

For ASTM C1260, and other tests when required, continue readings for 28 days after the zero readings.

Acceptance criteria specified below are based on 28-day readings after the zero readings.

Expansion is no greater than 0.10 percent: Aggregates are acceptable.

Expansion is greater than 0.10 percent:

Test aggregates according to ASTM C1567 using components (e.g. coarse aggregate, fine aggregate, cementitious materials, and ASR inhibiting admixtures) in proportions proposed for mixture design

For mixes using lithium admixtures use test procedure COE CRD-C 662.

Expansion of proposed mixture design test specimens, tested in accordance with ASTM C1567, does not exceed 0.10 percent:

Aggregates are acceptable.

Expansion of proposed mixture design test specimens is greater than 0.10 percent:

Aggregates are not acceptable unless adjustments to mixture design can reduce expansion to less than 0.10 percent or testing by ASTM C1293 indicates aggregates will not experience deleterious expansion.

Use tested materials.  Materials may be rejected if they do not match tested materials.

ASTM C1293 test results may be substituted for ASTM C1260 test results.

Average ASTM C1293 concrete prism expansion less than 0.04 percent at one year:  Aggregates acceptable.

Appropriate for use in accordance with ASTM C 295.

ADMIXTURES

Air-entraining admixture: ASTM C260.

Chemical admixtures:

Do not use chemical admixtures which contain more than 0.1 percent chloride, by weight.

Admixtures shall be compatible with each other.

Allowable chemical admixtures:

ASTM C494, Type A, D, or S.

ASTM C1017, Type I or II.

ASTM C494, Type C and E, provided they do not contain chlorides.

Specialize chemical admixtures:

When batch plant has not previously used a specialized chemical admixture, admixture manufacturer shall provide an on-site representative to assist with mix design and to trial batch plant personnel in dispensing and mixing operations.

Do not use specialized chemical admixtures which contain more than 0.1 percent chloride, by weight.

Extended set control admixture:

Delvo Stabilizer manufactured by BASF Construction Chemicals, Inc. www.basf-admixtures.com; or equal, with the following essential characteristics:

Meets ASTM C494, Type B.

Retards setting.

Does not reduce concrete strength.

Use within manufacturer’s time limits.

Include admixture on batch ticket.

Admixture quantity required to stabilize concrete shall be pre-determined using jobsite materials.  Initial concrete setting time shall be monitored and adjusted during project by qualified concrete technician.

Viscosity Modifying Admixture:

V-Mar 3 manufactured by Grace Concrete Products, www.na.graceconstruction.com; or equal with the following essential characteristics:

Meets ASTM C494 Type S.

Designed to increase viscosity of concrete while still allowing concrete to flow without segregation.

Does not contain chlorides.

Does not significantly affect slump, air content or time of set.

For controlling rheology of concrete mixtures: Incorporated into concrete mix at batch plant in manufacturer’s recommended dosage.

CURING MATERIALS

Water: ASTM C1602, including optional requirements of Table 2.

Curing compound: ASTM C309:

Capable of meeting moisture retention at manufacturer’s application rate.

Meet Federal, state, and local regulations for VOCs.

Sheet materials:

Polyethylene film: ASTM C171, white opaque.

ACCESSORIES

Sponge rubber joint filler:

ASTM D1752, Type I, except:

Test specimen compression load: 50 to 150 lb/in2.

Joint filler adhesive: Nonbituminous adhesive recommended by filler manufacturer.

Elastomeric sealant: ASTM C920, polyurethane, Use M, Grade NS, Class 25.

Evaporation control:

MasterKure ER 50 manufactured by BASF Construction Chemicals, Inc., www.basf-admixtures.com; or equal having the following essential characteristics:

Monomolecular film forming compound applied to exposed concrete slab surfaces for temporary protection from rapid moisture loss.

For application after finishing and prior to applying curing compound.

For use when the evaporation rate is high.

Do not use as finishing aid.

MIX

The Contractor shall design and adjust concrete mix.

The Government reserves the right to adjust mix proportions when need for adjustment is indicated by results of materials testing.

When required, adjustment of mix proportions by the Government will be in accordance with USBR Concrete Manual.

Cementitious materials options:

Specified portland cement plus specified pozzolan by percent weight specified in Table 03 030 00A - Concrete Mixes.

Specified portland cement plus specified slag cement by percent weight specified in Table 03 03 00A - Concrete Mixes.

Blended Hydraulic:  ASTM C595 provided, specified portland cement with percent of specified pozzolan or specified slag cement specified in Table 03 03 00A - Concrete Mixes.

Ternary blends.

Design concrete mixes in accordance with Table 03 30 00A – Concrete Mixes.

General concrete mix shall be used for concrete unless otherwise specified.

Net water-cementitious materials ratio (w/c) is maximum, by weight.  Cementitious material weight is cement plus SCM.

Slump at point of placement: In accordance with ASTM C143, except SCC.

SCC slump flow at point of placement, in accordance with ASTM C1611.

Air Entrainment:  Percent air by volume of concrete as discharged at point of placement, in accordance with ASTM C231.

| Table 03 30 00A – Concrete Mixes |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mix No | Feature | f’c (lb/in2) | Max w/c* | NMSA** | Percent SCM*** A: Class F Pozzolan B: Slag Cement C: Silica Fume | Slump or Slump flow (in) | Air Content (%) | Notes |
| 1 | General Concrete | 4,500 at 28 days | 0.45 | No. 457, 57, 467, or 67 | A or B: 20 ± 5 | 2 to 4 | 4.5 to 7.5 | 3, 4 |
| 2 | Structural Concrete | 4,500 at 28 days | 0.45 | No. 457, 57, 467, or 67 | A or B: 20 ± 5 | 2 to 4 | 4.5 to 7.5 | 1, 2 |
| * Maximum water/cementitious materials ratio. ** Nominal Maximum Size Aggregate. *** SCM as percent of total cementitious material, by weight. NOTES: - Ternary blended cementitious materials which meet the specifications may be submitted for approval. - Concrete with ASTM C1017, Type I or II plasticizing admixtures, ASTM C494 Type F high-range water-reducing admixtures, or Type G high-range water-reducing and retarding admixtures: - Admixture shall be incorporated into trial batch or historical data. - Use slump appropriate for placing conditions. - Aggregate gradation for SCC concrete may deviate from ASTM C33. - For SCC, deviating from the table may be acceptable provided resulting mixture meets performance targets for fresh and hardened SCC and deviations are approved by COR. |  |  |  |  |  |  |  |  |

Submit design mixes for each type and strength of concrete substantiated by either laboratory trial batch or field performance methods as specified in ACI 301.  For trial batch method, mix shall be proportioned and stamped by a professional engineer.

Concrete trial mixes:

Average compressive strength of trial batch cylinders at design age: Design strength plus 1,200 lbs./in2 for concrete between 3,000 and 5,000 psi.

Admixtures to be used in mix shall be incorporated into mix design submitted for approval.

Air content: Within 1 percent of top of specified range.

Slump: Within 1 inch of top of specified range.

BATCHING, MIXING, AND TRANSPORTING

Batch plant: Currently NRMCA certified with automatic digital recording of cementitious materials, aggregate, water, and chemical admixtures

Manufacture and deliver in accordance with ASTM C94 and ACI C304R.

Prepare batch ticket in accordance with ASTM C94 for every batch of concrete.

Cold weather:  When air temperature has fallen to or is expected to fall below 40 degrees F, prepare ingredients and mix in accordance with ACI 306.1.

Do not use frozen materials or materials containing ice or snow.

Uniformly heat water and aggregates before mixing to obtain concrete mixture temperature of not less than 50 degrees F.

Hot Weather: When precautions are necessary, prepare ingredients and mix in accordance with ACI 305.1.

Cool ingredients before mixing to maintain specified maximum concrete temperature at time of placement

Mixing water may be chilled or chopped ice may be used to control temperature, provided water equivalent of ice is calculated to total amount of mixing water.  Ice replacing batch water shall be melted prior to discharge

Using liquid nitrogen to cool concrete is Contractor’s option.

Prevent appreciable segregation of ingredients.

Place concrete within 90 minutes from introduction of cement to water or aggregates.

For placing times exceeding 90 minutes, extended set control admixtures may be used when approved by COR.

CONCRETE TEMPERATURE

Concrete temperature at placing: shall be within 50 to 90 degrees F.

CONTRACTOR QUALITY TESTING

Contractor’s Independent testing agency shall perform sampling, testing, and reporting as required in Table 03 30 00A – Concrete Mixes.

Personnel conducting tests: Qualified as ACI Aggregate Testing Technician, Level 1; or equal.

Notify COR immediately of test results showing failure of materials to meet specifications.  Provide passing test to COR within 24 hours.  Submit reports of test results as specified.

EXECUTION

PREPARATION

Remove standing water, mud, and debris from foundation surfaces to be covered by concrete.

Prepare rock surfaces free from oil, objectionable coatings, and loose, semidetached, and unsound fragments.  Immediately before placement of concrete, wash rock surfaces with air-water jet and dry to uniform surface-dry condition.

Prepare earth foundations free from frost or ice.

Thoroughly moisten surfaces of absorptive foundations to be covered with concrete so that moisture will not be drawn from fresh concrete.  Keep subgrade moisture uniform without puddles or dry areas.

Clean, roughen, and surface dry surfaces of construction joints and existing concrete to be covered with fresh concrete.

Remove laitance, loose, or defective concrete, coatings, sand, curing compound, and other foreign material.

Wash surface thoroughly and surface dry immediately before placement of adjoining concrete.

PLACING

Notify COR at least 24 hours before placing concrete.

Do not place concrete without approval of the COR.

Place concrete in presence of COR.

Placing concrete requires ACI Concrete Flatwork finisher or a minimum of 3 years’ experience finishing concrete of similar work.

Allow at least 7 days between adjacent placements, or as approved by COR.

Do not use aluminum pipes and chutes for placing or pumping concrete.

Adding water to concrete batch at site will be allowed only once and only when approved by the COR.

Additional water shall be added before concrete is discharged.

Do not exceed specified water to cement ratio.

After water is added, concrete shall be mixed for at least 30 revolutions of mixer drum at mixing speed.

Record added water on the batch ticket to nearest gallon.

Adding air entraining admixtures to concrete batch at site will be allowed only once when approved by the COR.

After air entraining admixture is added, concrete shall be mixed for minimum of 30 revolutions of mixer drum at mixing speed.

Take slump and air content after air-entraining admixture addition and additional revolutions.

Record added air entraining admixture on batch ticket to nearest ounce.

Do not use concrete which has become so stiff that concrete cannot be properly placed.

Place formed concrete in continuous, approximately horizontal layers.  Do not exceed 20 inches in depth of layers.

Vibrate concrete until concrete has been consolidated to maximum practical density, is free from pockets of coarse aggregate, and closes snugly against surfaces of forms and embedded materials.

Hot Weather:

Place concrete in accordance with ACI 305.1.

Protect reinforcing steel so that steel temperature does not exceed ambient air temperature immediately before placing concrete.

Fog spray forms, reinforcing steel, and subgrade just before placing concrete.  Keep subgrade moisture uniform without puddles or dry areas.

Cold weather:

Place concrete in accordance with ACI 306.1.

Do not place concrete on frozen subgrade or subgrade containing frozen materials.

FINISHING

Notify COR before finishing concrete.

Finish concrete in presence of Government inspector unless inspection is waived in each specific case.

Finishing shall be performed under direct supervision of Craftsman certified as ACI Concrete Flatwork Finisher.

Finish surfaces as specified in Table 03 30 00B – Formed Surfaces and Table 03 30 00C – Unformed Surfaces.

Where finishes are not specified or shown on drawings for a particular surface, finish concrete as specified for similar work.

Formed surfaces:

Finish F3:

Applies to formed surfaces with special appearance requirements, such as surfaces exposed to view and as specified in Table 03 30 00F - Formed Surfaces.

After required patching and correction of imperfections has been completed, sack rub surface as follows:

Thoroughly wet surface and sack rub while surface is still damp.

Use mortar consisting of 1 part cement; 2 parts, by volume, of sand passing No. 16 screen; and enough water so that mortar has consistency of thick cream.  Blend standard cement with white cement as necessary to obtain color which will match surrounding concrete surface.

Thoroughly rub mortar over area with clean burlap or sponge rubber float to fill pits, bugholes, and other defects.

While mortar in pits is still plastic, rub surface with dry mix of above proportions and material to remove excess plastic material and place enough dry material in pits to stiffen and solidify mortar so that filling will be flush with surface.  Remove material remaining on surface except material within pits.

Continue curing surface as specified.

Unformed surfaces:

Do not use dry portland cement or additional water during finishing.

Do not use “jitterbugs” or other tools to force coarse aggregate away from surface.

Finish U3 troweled finish:

Applies to unformed surfaces where appearance and porosity is considered by Government to be of special importance as specified in Table 03 30 00C – Unformed Surfaces.

Begin steel troweling after bleed water has disappeared and floated surface has sufficiently hardened to prevent excess of fine material from being drawn to surface.

Trowel with firm pressure to flatten sandy texture of floated surface.

Trowel to a dense uniform surface free from blemishes and trowel marks.  Do not excessively trowel surface.

Slope interior surfaces for drainage as shown on drawings or as directed by COR.  Slope surfaces exposed to weather for drainage as directed by COR.

Slope narrow surfaces, such as tops of walls and curbs, approximately 3/8-inch per foot of width, unless use of other slopes or level surface is indicated on drawings or is directed by the COR.

Slope broader surfaces, such as walks, platform, and decks; approximately 1/4-inch per foot unless use of other slopes or level surfaces is indicated on drawings or is directed by the COR

JOINTS AND EDGES

Construction joints (CJ):

Construction joints are joints which are purposely placed in concrete to facilitate construction, reduce initial shrinkage stresses and cracks, allow time for installation of embedded metalwork, or allow for subsequent placing of other concrete.

Bond is required at construction joints regardless of whether or not reinforcement is continuous across joint.

Locate construction joints as shown on drawings. Relocation, addition, or elimination of construction joints will be subject to approval by the COR.

Clean, roughen, and surface dry surfaces of construction joints to be covered with fresh concrete.  See Preparation article.

Do not use a mortar layer on construction joints.

Control joints (Ct.J):

Control joints are joints placed in concrete to provide for control of initial shrinkage stresses and cracks of monolithic units.

Construct control joints so no bond exists between concrete surfaces forming the joint.

Construct control joints by placing concrete on one side of joint and allowing it to set before concrete is placed on other side of joint.

Coat surface of concrete first placed at control joint with curing compound that prevents bond before placing concrete on other side of joint.

Reinforcement is continuous across tooled or saw cut control joints.

Expansion joints (EJ):

Cut sponge rubber joint filler to size and shape of joint surface to receive filler.

Adhere filler to concrete in accordance with adhesive manufacturer's recommendations.

Butt sections of filler with tight-fitting butt joints to prevent mortar from seeping through joint.

Edges:

Permanently exposed concrete, except slabs and top edges of curbs: Chamfer edges with 45-degree bevel 3/4-inch by 3/4-inch; unless otherwise shown on drawings.

Exposed edges of slabs and top edges of curbs:  Tool to radius of 1/4 inch.

Preformed joints consisting of plastic or metal strips not allowed.

STRUCTURAL DEVIATIONS AND SURFACE TOLERANCES

Structural deviations are defined as allowable variations from specified lines, grades, and dimensions.

Surface tolerances are defined as maximum allowable magnitude of surface irregularities.

Specified structural deviations and surface tolerances are consistent with modern construction practice and governed by effects that permissible variations may have upon a structure.  COR reserves the right to diminish specified structural deviations and surface tolerances where such variations impair structural action, operational function, or architectural appearance of a structure or portion of structure.

Construct concrete within stated variations even though more than one may be specified.

Specified variation for one element of a structure will not apply when it will permit another element of same structure to exceed its allowable variation.

Where variations are not specified or shown on drawings for a particular structure, variations shall be those specified for similar work.  As an exception to clause at FAR 52.236-21 “Specifications and Drawings for Construction,” specific tolerances shown on drawings in connection with dimension shall govern.

Structural deviations:

Check variations from specified lines, grades, and dimensions in hardened concrete to determine that structures are within tolerances specified in Table 03 30 00C - Deviations from Specified Lines, Grades, and Dimensions.

Variation is distance between actual position of structure or element of structure and specified position in plan for structure or particular element.

Plus or minus variations, shown as (), indicate a permitted actual position up or down and in or out from specified position in plan.

Variations not designated as (+) or (-) indicate maximum deviation permitted between designated successive points on completed element of construction.

Specified position in plan are defined as lines, grades, and dimensions described in these specifications, shown on drawings, or prescribed by COR.

| Table 03 30 00B - Deviations from Specified Lines, Grades, And Dimensions |  |
| --- | --- |
| A. STRUCTURES |  |
| 1. Footings: (a) Variation in length and width dimensions from those specified (b) Horizontal misplacement or eccentricity: (c) Reduction in thickness from that specified | -1/2 inch +2 inches 2 percent of footing width in direction of misplacement, but not more than 2 inches 5 percent of specified thickness, not to exceed 1 inch |
| 2. Variation from plumb or specified batter for lines and surfaces of piers, stems, and walls: (a) When overall height of line or surface is: Less than 10 feet 10 feet or more (b) For any two successive intermediate points on the line or surface separated by: 10 to 20 feet, inclusive More than 20 feet | ±1/4 inch ±3/8 inch 1/4 inch 3/8 inch |
| 3. Variation from level or specified grades for slabs: (a) When overall length of line or surface is: Less than 10 feet 10 to 20 feet, inclusive More than 20 feet (b) For any two successive intermediate points on the line or surface separated by: 10 to 20 feet, inclusive More than 20 feet | ±1/4 inch ±3/8 inch ±3/4 inch 1/4 inch 3/8 inch |
| 4. Variation in cross-sectional dimensions of those specified in thicknesses and width | -1/4 inch +1/2 inch |
| 5. Variation from specified elevation for top of concrete for foundations | ±1/2 inch |

Surface irregularities:

Bulges, depressions, and offsets are defined as surface irregularities or roughness.

Surface irregularities are classified as “abrupt” or “gradual” and allowable tolerances are specified in Table 03 30 00E - Surface Tolerances.

A surface tolerance is designated by a capital “T” followed by a number 1 through 5.

Surface tolerance designations are separate from surface finishes and structural deviations.

Abrupt surface irregularities:

Abrupt surface irregularities are defined as offsets such as those caused by misplaced or loose forms in which maximum dimension of irregularity perpendicular to surface is greater than maximum dimension of irregularity in plane of surface.

Abrupt surface irregularities include isolated surface irregularities which exceed specified gradual irregularities.

Gradual surface irregularities:

Gradual surface irregularities are defined as bulges and depressions resulting in gradual changes on surface.

Gradual surface irregularities are further defined as isolated undulations on surface.  Maximum dimension of undulation perpendicular to surface is small relative to maximum dimension of undulation in plane of surface.

Check magnitude of surface irregularities of formwork and finished surfaces to ensure that surfaces are within specified tolerances.

Surface tolerances:

| Table 03 30 00C - Surface Tolerances |  |  |
| --- | --- | --- |
| Concrete surface | Maximum allowable surface irregularity tolerance |  |
|  | Abrupt | Gradual |
| T1 | 1 inch | 1/4 inch/inch |
| T2 | 1/2 inch | 1/8 inch/inch |
| T3 | 1/4 inch | 1/16 inch/inch |
| T4 | 1/8 inch | 1/32 inch/inch |
| T5 | 1/32 inch | 1/120 inch/inch |

Repair of hardened concrete not within specified tolerances:

Repair hardened concrete which is not within specified tolerances to bring it within those tolerances.

Perform repair after consultation with Government inspector regarding method of repair.  Notify COR as to time when repair will be performed.

Repair concrete which will be exposed to view in manner which will result in concrete surface with uniform appearance.

When grinding surfaces exposed to view, limit depth of grinding such that no aggregate particles are exposed more than 1/16 inch in cross section at finished surface.

Where grinding has caused or will cause exposure of aggregate particles greater than 1/16 inch in cross section at finished surface, repair concrete by excavating and replacing concrete.

Field verification of surface tolerances:

Determine compliance of surface with specified surface tolerances.

Evaluate surface roughness.

Measure roughness height or depth and check for compliance with values specified in Table 03 30 00D - Surface Tolerances and Table 03 30 00C - Deviations from Specified Lines, Grades, and Dimensions.

When measured height or depth of roughness is less than value in abrupt tolerance specification and height or depth of roughness does not cause structure to exceed applicable value specified in Table 03 30 00C - Deviations from Specified Lines, Grades, and Dimensions, surface roughness is acceptable.

When roughness height or depth exceeds abrupt tolerance specification, determine roughness slope for comparison to gradual tolerance specification.

Measure roughness length and determine roughness slope by dividing roughness height or depth by roughness length (see Figure 1).

When roughness slope is greater than slope specified by gradual tolerance specification, surface roughness is unacceptable.

When roughness slope is less than gradual slope specified and gradual roughness does not cause structure to exceed allowable structural deviations, surface roughness is acceptable.

Measuring surface roughness:

Examples below illustrate how to make necessary surface measurements for typical roughness.

Case 1 - Roughness Protruding above Surface:

Measure roughness protruding above surface with straightedge that is at least 20 times longer than roughness height being measured.

Position straightedge with one end resting on top of roughness, as shown in Figure 2 (case 1).

Determine roughness height by measuring maximum gap that occurs normal to straight edge.

Note position on the straightedge from which normal distance is measured.

To determine roughness length, measure distance along straightedge from point where the height was measured to point of contact between straightedge and top of roughness.

Roughness slope is defined as ratio of roughness height to roughness length.

As roughness is seldom symmetric, moving position of straightedge about roughness may be necessary to locate point where maximum height and slope exists.

Case 2 - Roughness Extending below the Surface:

A roughness occurring as an indentation to surface is measured by placing straightedge across indentation, as shown in Figure 2 (case 2).

Measure maximum gap between straightedge and surface and note location of measurement on straightedge.

From point of depth measurement, measure along straightedge in both directions to point of contact with surface.

Use shortest length measured as roughness length.

Divide roughness depth by roughness length to determine roughness slope.

Prevention of repeated failure to meet tolerances:

When concrete placements result in hardened concrete which does not meet specified tolerances, submit to COR an outline of preventive actions such as modifications to forms, modified procedure for setting screeds, and different finishing techniques to be implemented to avoid repeated failures.  Submit when requested by COR.

Government reserves the right to delay concrete placements until preventive actions which have been approved by COR are implemented.

CURING

Water curing:

Keep concrete surface wet for 14 days, minimum, from time concrete has attained sufficient set to prevent detrimental effects to surface.

Cure methods:

Water-saturated material.

System of perforated pipes, mechanical sprinklers, or porous hose.

Other methods which will keep surfaces wet.

Subject to approval by COR.

Curing with curing compound:

Apply to concrete surface to provide water-retaining film.  Reapply as necessary to maintain continuous, water-retaining film on surface for 28 days.

Thoroughly mix compound and spray apply in one coat to provide continuous, uniform film over surface.

Do not exceed coverage rate recommended by curing compound manufacturer.  Decrease coverage rate on rough surfaces as necessary to obtain required continuous film.

Ensure ample coverage on edges, corners, and rough surfaces.

Use spray equipment recommended by curing compound manufacturer.

Sheet material curing:

Includes curing with polyethylene film.

Thoroughly moisten concrete surface by lightly spraying with water as soon as concrete has hardened sufficiently to prevent damage.

Completely cover concrete surface with sheet material to provide airtight, water-retaining film over entire surface.

Lap edges of sheet material to seal adjacent sheets.

Place tightly against concrete surface at extreme edge of curing area.

Secure sheet material to withstand wind and prevent circulation of air inside sheet material.

Keep surface covered for 14 days, minimum.

CONTRACTOR FIELD QUALITY TESTING

Contractor furnished Independent testing agency shall perform sampling, testing, and reporting as required in Table 03 30 00D - Field Testing.

Personnel conducting concrete specimen tests: Qualified as ACI Concrete Strength Testing Technician; or equal.

Perform tests at least as often as frequencies specified in Table 03 30 00D – Field Testing.

Notify COR immediately of test results showing failure of materials to meet specifications.  Notify COR within 2 hours of test results showing materials meet specifications.  Submit reports of test results as specified.

FIELD TESTING

| Table 03 30 00D- Field Testing |  |  |  |
| --- | --- | --- | --- |
| TESTS OF | TEST STANDARD | STANDARD TITLE | TESTING FREQUENCY |
| Fresh Concrete Properties - tests performed at site | ASTM C143 | Slump of Hydraulic-Cement Concrete | 1 set of tests per load for first two loads. When tested concrete meets specifications, 1 set of tests each day of placement for each mixture for first 50 or less cubic yards, and 1 set of tests for each additional 100 cubic yards of concrete. Minimum of 1 set of tests per hour during placements. When concrete does not meet specifications, test each load until 2 consecutive loads meet specifications, then resume testing frequency specified above. |
|  | ASTM C231 | Air Content of Freshly Mixed Concrete by the Pressure Method (alternative to ASTM C138 gravimetric method) |  |
|  | ASTM C1064 | Temperature of Freshly Mixed Hydraulic-Cement Concrete |  |
|  | ASTM C138 | Density (Unit Weight), Yield, and Air Content (Gravimetric) of Concrete |  |
| Compressive Strength, | ASTM C31 ASTM C39 | Making and Curing Concrete Test Specimens in the Field Compressive Strength of Cylindrical Concrete Specimens | 1 set of samples (6-inch by 12 inches for each day of placement for each mixture for the first 50 or less cubic yards, and 1 set of samples for each additional 100 cubic yards of concrete. A minimum of 5 samples for strength testing shall be made each time strength samples are collected. 2 additional field cured test cylinders during placement in adverse (hot or cold) weather. Cure these samples on jobsite under same conditions as concrete the cylinders represent for minimum of 7 days, then transfer to testing laboratory until testing at strength design days. Test 2 cylinders each at 7 days age and 2 cylinders at strength design age. Maintain last cylinder for testing in event that the strength design age test results fall below the required strength. |
| Concrete Cores | ASTM C42 | Obtaining and Testing Drilled Cores and Sawed Beams of Concrete | At discretion of the Government when cylinder strengths fail to meet minimum requirements. The Contractor shall obtain core specimens in accordance with ASTM C42 at locations directed by COR, at no additional cost to the Government. The Contractor shall repair the core holes in accordance with USBR M-47 as directed by COR. |

Acceptance criteria:

Cylinder compressive strength:

In accordance with ASTM C94, except as follows:

90 Percent of test cylinders exceed specified compressive strength at design age.

Average compressive strength of six consecutive test cylinders exceeds specified compressive strength at design age.

No individual strength test falls below specified compressive strength by more than 500 lb/in2.

Drilled concrete cores:

Concrete in placement represented by core tests will be considered structurally adequate when average compressive strength of three cores is equal to at least 85 percent of specified compressive strength and no single core has a compressive strength of less than 75 percent of specified compressive strength.

PROTECTION

Protect concrete from damage until final acceptance by Government.

Do not load, remove forms or shoring, or backfill against concrete until concrete has gained sufficient strength to safely support its weight and imposed loads.

Protect fresh concrete against erosion from rain, hail, sleet, or snow; contamination from foreign materials; and damage from foot traffic until the concrete has hardened.

Protect concrete from heavy foot traffic and other construction activities by covering with plywood or other suitable material.  Remove and dispose of temporary covering when no longer required.

Protect concrete when freezing temperatures are imminent:

Maintain concrete at a temperature of 50 degrees F or greater for 72 hours, minimum, after placement.  Vent heater and prevent concrete from drying where artificial heat is employed.

REPAIR

Repair concrete in accordance with USBR M-47.

Use repair or replacement method directed by COR.

FINISH, SURFACE TOLERANCES, AND CURING SCHEDULES

| Table 03 30 00E - Formed Surfaces |  |  |  |
| --- | --- | --- | --- |
| Surface | Finish | Maximum Allowable Tolerances | Acceptable Curing Methods |
| Surfaces exposed prominently to view or where appearance is important | F3 | T5 | ASTM C1315 Class A Clear curing compounds, water-emulsified resin-base if outdoors |

| Table 03 30 00G - Unformed Surfaces |  |  |  |
| --- | --- | --- | --- |
| Surface | Finish | Maximum Allowable Tolerances | Acceptable Curing Methods |
| Surfaces to be covered by fill material or concrete | U1 | T1 | White ASTM C309 Class A or B curing compound for horizontal surfaces, and White ASTM C309, Class B curing compound for vertical or sloped surfaces |
| Surfaces to be covered by grout | U1 | T3 | Water, White ASTM C309 Class A or B curing compound for horizontal surfaces, and White ASTM C309, Class B curing compound for vertical or sloped surfaces |
| Outdoor entrance slabs, and walks and stoops | U2 | T3 | ASTM C1315 Class A Clear curing compounds, water-emulsified resin-base, Polyethylene film or White Burlap-polyethylene sheet |
| Tops of piers or foundations | U2 | T3 | Water White wax-base or white water-emulsified resin-base curing compound |
| Subfloor surfaces not otherwise listed | U2 | T5 | Water Polyethylene film or White Burlap-polyethylene sheet |
| Stair treads and landings | U3 | No abrupt irregularities allowed. Maximum allowable gradual irregularity: 1/240 inch/inch. | Polyethylene film or White Burlap-polyethylene sheet |

END OF SECTION

This page intentionally left blank.

SECTION 03 62 20

NON-SHRINK GROUT FOR EQUIPMENT AND METALWORK

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered in the Price Schedule for items of work requiring non-shrink grout.

REFERENCE STANDARDS

ASTM International (ASTM)

ASTM C150/C150M-19a	Portland Cement

ASTM C1107/C1107M-17	Packaged Dry, Hydraulic-Cement Grout (Nonshrink)

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 03 62 20-1, Packaged non-shrink grout data:

Manufacturer’s product data.

Manufacturer’s certification that product meets specifications.

Manufacturer’s instructions:

Surface preparation, mixing, installation, finishing, curing, and protection recommendations.

Include specific recommendations based on anticipated jobsite temperatures and environmental conditions.

Include manufacturers’ recommended methods for float finishing grout.

Include manufacturer’s recommended minimum temperature and protection period for protecting grout from freezing and damage after placement.

PRODUCTS

PACKAGED NONSHRINK GROUT

Pre-mixed commercial grout mixture:

ASTM C1107.

Cement in mixture: ASTM C150, Type V.

Water:

Clean.

Free of detrimental quantities of silt, organic matters, salts, or other impurities.

As recommended by grout manufacturer.

Mix: In accordance with manufacturer’s instructions to consistency recommended by manufacturer for application.

EXECUTION

PREPARATION

Clean and prepare base concrete surface to which non-shrink grout will be bonded in accordance with manufacturer’s instructions or as directed by COR:

Roughen surface.

Remove dirt, laitance, loose, or defective concrete, curing compound, coatings, and other foreign material.

Wash with water.

Lubricate concrete surface by washing with water immediately before placing non-shrink grout when delay occurs between washing and placing non-shrink grout.

PLACING

Place non-shrink grout in accordance with manufacturer’s instructions to completely fill space to be grouted.

Use forms, where required, to confine non-shrink grout.

CURING

Cure exposed surface of packaged non-shrink grout by method recommended by manufacturer.

Do not apply loads sooner than 72 hours after placement and only after non-shrink grout has attained compressive strength of at least 3,000 lb/in2.

END OF SECTION

SECTION 05 12 00

STRUCTURAL STEEL FRAMING

GENERAL

MEASUREMENT AND PAYMENT

Steel Structures:

Measurement: Weights shown on final approved Contractor-furnished bills of material.  Base calculated weights for shapes other than rectangular on the actual detailed dimensions shown on final detail shop drawings.

Payment: Per unit weight as offered in the Price Schedule.

REFERENCE STANDARDS

American Association of State Highway and Transportation Officials (AASHTO)

AASHTO-SSHB-02	Standard Specifications for Highway Bridges, 17th Edition.

American Concrete Institute (ACI)

ACI 355.2-24	Post-Installed Mechanical Anchors in Concrete – Qualification Requirements and Commentary

ACI 355.4-24	Post-Installed Adhesive Anchors in Concrete – Qualification Requirements and Commentary

American Institute of Steel Construction (AISC)

AISC 316-89	Manual of Steel Construction, Allowable Stress Design, 9th Edition

AISC RCSC-2020	Research Council of Structural Connections (RCSC) Specification for Structural Joints Using High-Strength Bolts

American Society of Mechanical Engineers (ASME)

ASME B 18.2.1-96(2005)	Square and Hex Bolts and Screws, Inch Series

ASTM International (ASTM)

ASTM A36/A 36M-19	Carbon Structural Steel

ASTM A53/A53M-24	Pipe, Steel, Black and Hot-Dipped, Zinc-Coated Welded and Seamless

ASTM A108-24	Steel Bar, Carbon and Alloy, Cold Finished

ASTM A123/A123M-24	Zinc (Hot-Dip Galvanized) Coating on Iron and Steel Products

ASTM A153A153M-23	Zinc Coating (Hot-Dip) on Iron and Steel Hardware

ASTM A307-21	Carbon Steel Bolts and Studs, 60,000 psi Tensile Strength

ASTM A354-17e2	Quenched and Tempered Alloy Bolts, Studs, and Other Externally Threaded Fasteners

ASTM A394-08(2024)	Steel Transmission Tower Bolts, Zinc-Coated and Bare

ASTM A500/A500-23	Cold-Formed Welded and Seamless Carbon Steel Structural Tubing in Rounds and Shapes

ASTM A501/A501M-21	Hot-Formed Welded and Seamless Carbon Steel Structural Tubing

ASTM A513/A513M-24	Electric-Resistance - Welded Carbon and Alloy Steel Mechanical Tubing

ASTM A563-07a	Carbon and Alloy Steel Nuts

ASTM A586-04a	Zinc-Coated Parallel and Helical Steel Wire Structural Strand and Zinc-Coated Wire for Spun-In-Place Structural Strand

ASTM A653/A653M-23	Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process

ASTM A668/A668M-23	Steel Forgings, Carbon and Alloy, for General Industrial Use

ASTM A759-21	Carbon Steel Crane Rails

ASTM A780/A780M-20	Repair of Damaged and Uncoated Areas of Hot-Dip Galvanized Coatings

ASTM A992/A992M-22	Structural Steel Shapes

ASTM A1008/A1008M-24	Steel, Cold-Rolled, Carbon, Structural, High-Strength Low-Alloy and High-Strength Low-Alloy with Improved Formability

ASTM A1011/A1011M-23	Steel, Sheet and Strip, Hot-Rolled, Carbon, Structural, High-Strength Low-Alloy and High-Strength Low-Alloy with Improved Formability

ASTM E165/E165M-23	Liquid Penetrant Examination

ASTM E709-21	Magnetic Particle Examination

ASTM F436/F436M-24	Hardened Steel Washers Inch and Metric Dimensions

ASTM F844-19(2024)	Washers, Steel, Plane (Flat), Unhardened for General Use

ASTM F1554-20	Anchor Bolts, Steel, 36, 55, 105-ksi Yield Strength

ASTM F3125/F3125M-23	High Strength Structural Bolts and Assemblies, Steel and Alloy Steel, Heat Treated, Inch Dimensions 120 ksi and 150 ksi Minimum Tensile Strength, and Metric Dimensions 830 MPa and 1040 MPa Minimum Tensile Strength

American Welding Society (AWS)

AWS D1.1/D1.1M-2020	Structural Welding Code - Steel

Commercial Item Description (CID)

CID A-A-1923A	Shield, Expansion (Lag, Machine and Externally Threaded Wedge Bolt Anchors)

Society of Protective Coatings (SSPC)/NACE International (NACE)

SSPC-SP6/ NACE 3-2007	Commercial Blast Cleaning

SSPC SP7/NACE 4-2007	Brush-Off Blast Cleaning

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

Except as specified, submit drawings and data for each type of steel structure.

RSN 05 12 10-1, Dimensional Drawings:

Show geometric arrangements and controlling dimensions including anchor bolt dimensions.

Obtain approval before detailing steel structures.

RSN 05 12 10-2, Detail Drawings and Data:

As an exception to Section 01 33 00 - Submittals, detail drawings and data may be submitted as they are completed for a particular type of structure, rather than as a single package covering all structure types.

Erection Drawings:

Include controlling dimensions, marking and position of each member in structure, number, size, and length of bolts, size, type, and number used for each joint.

Keep member designations to minimum practicable, for example, designate members of identical size and detail with same designation regardless of position in structure.

Shop Drawings:

Detail members and connections

Include anchor bolts and embedded materials.

Detail members in place where practicable.  Drawings consisting totally of individual members are not acceptable.

Establish "Y" dimensions before submittal of detail drawings and data.

Show controlling dimensions for each structure and type and grade of material and type of finish and edge planning and preparation details for all shop and field welds.

Bill of Materials:

Show number of pieces required in structure; description of each piece including mark number, size, length, and weight; and drawing number for shop detail of that piece.

Summarize weights into a subtotal for structural components; a subtotal for bolts, nuts, locknuts, and fills; and a total weight for structure.

Submit new or original positive reproducible detail drawings and data.  Sepia reproducible are not acceptable.

RSN 05 12 10-3, Test Reports:

Certified “mill” test reports of chemical analyses and physical tests of structural steel and bolts for Government review and use.

Show customer and mill order and weights or quantities of materials furnished represented by tests.

RSN 0510-4, Final Drawings and Data:

Final drawings and data consisting of detail shop drawings, erection diagrams, and bills of material.

Show revisions and changes made up to time fabrication is completed.

Sepia reproducible are not acceptable.

GOVERNMENT FURNISHED DRAWINGS

Within 30 days after Contract Award, Government will furnish to Contractor one reproducible set of drawings listed in Section 52 00 00 Drawings.

QUALIFICATIONS

Qualify welds in accordance with AWS D1.1.

Qualify welding process and welding operators in accordance with AWS qualification procedures.

DELIVERY, STORAGE, AND HANDLING

Transport and handle in a manner which prevents bending or damage.

Prevent damage to coatings.

Materials bent in handling may be used when material can be straightened without injury to coating.  Replace pieces bent beyond repair.

Redip materials on which zinc coating has been damaged unless damage is local and can be field repaired as specified.

Place steel items on timber blocking to prevent ground or surface water contact during storage.

PRODUCTS

DESIGN REQUIREMENTS - BOLTED STEEL STRUCTURES

Embedded Materials: Provide embedded material of shapes, connection details, and structure setting dimensions as shown on drawings.

Splices:

Splice at or directly above panel points.

Splice Angle: Thickness of heavier member.

Grind splice angles at heel to fit fillets of outside angles.

Do not use fillers between splice angles and main members.

Bolts: Minimum of six bolts, three in each leg.

Connections:

Stressed Web Members: Connect by not less than two bolts.

Secondary Members:

Use two bolts at one end, where possible without impairing main member strength.

Otherwise, use a one-bolt connection located on inner gauge line.

Gusset Plates:

Use minimum practicable number with due consideration given to joint eccentricity.

Connect compression web members to main members with at least one bolt.

Stressed Members: Gusset plates at least 1/16-inch thicker than connected web member.

Diagonal tension web members: May be connected entirely on a gusset to avoid use of fills.

Provide one-piece diagonals and connect diagonals at point of intersection by at least one bolt.  Provide ring fills when distance between two members, at intersection, exceeds 1/16-inch per foot of length of one of members.

Reinforce stressed members where outstanding leg is blocked out.  Do not strip outstanding angle legs without approval.

Clip corners of web members to eliminate or reduce eccentricity.

Cut ends of outstanding legs of web member angles 2-1/2 inches and greater at 45 degrees where practicable.

Use a single-plate fill instead of ring fills where fills are required at two or more adjacent holes.

Double-Angle Web Members fastening criteria:

Tension Members: Fasten together at 3-foot-maximum intervals.

Compression Members:

Fasten together at intervals such that the L/r ratio for one angle does not exceed the L/r ratio for the double-angle member.

Maximum interval: 2 feet.

Fasten members with stitch bolts at a minimum of two locations between panel joints.

Connected legs 4 inches or smaller: Fasten members with one stitch bolt and ring fills on inner gauge line.

Connected legs larger than 4 inches: Fasten members with two stitch bolts and one filler plate with one bolt on each gauge line.

Tension Member Lengths:

Detail members carrying tension only, or designated on drawings for shorter than theoretically required length as follows:

Members 10 feet or less in length: Detail 1/8-inch short.

Unspliced members more than 10 feet long: Detail short by 1/16 inch for each 10 feet of length, or major fraction thereof, plus 1/16 inch, up to a maximum of 1/4 inch.

Spliced members more than 10 feet long: Detail short by 1/16 inch for each 10 feet of length or major fraction thereof, plus 1/16 inch, up to a maximum of 1/4 inch plus an additional 1/16 inch for each lap splice, or 1/8 inch for each butt splice.

Connection Bolts:

Length shown on drawings.

Provide with locknuts.

Provide with beveled washers for bolts in sloped flanges.

ASTM A 394 bolts: No threads in shear plane.

ASTM A 325 bolts: Threads may be in shear plane.

Same type and grade for a complete structure.

Connection Bolt holes:

Minimum distance between member edge and bolt hole center and minimum center-to-center bolt hole spacing:  In accordance with AISC M016.

In detailing gauge lines, make allowance for mill tolerances to ensure compliance with required minimum edge distance.

Where practicable, space holes not less than 3 diameters center-to-center.

Where practicable, position hole from member face to permit use of socket wrench.

Ladder bars, Step Bolts, and Eyebolts:

Step Spacing: Approximately 15 inches apart with maximum allowable spacing of 17 inches.

Space steps to avoid climbing interference with outstanding leg of structure web members.

Locate holes for steps on inner gauge of leg angle.

Do not use diagonal or horizontal members for steps.

Details shown on drawings.

DESIGN DETAILS - WELDED STEEL STRUCTURES

Details and Settings Dimensions:

In detailing structures, use details and setting dimensions for embedded materials as shown on drawings.

Details for connections shown on drawings are typical and may be varied slightly.

Splices:

Use splice details shown on drawings.

MATERIALS

Government inspection of materials at mill will be waived subject to Section 01 60 00 - Product Requirements.

Standard-strength structural steel: ASTM A36.

High-strength structural steel: ASTM A992, Grade 50.

Hollow Structural Shapes (HSS):

Tubing 2 inches and larger: ASTM A500 or A501.

Tubing less than 2 inches: ASTM A513.

Steel pipe:

ASTM A53.

Sheet steel: ASTM A1008 or ASTM A1011 as required by site location.

Galvanized sheet steel: ASTM A653.

ACCESSORIES

Bolts: ASTM A307 except as specified or indicated on drawings.

High-strength and erection bolts: ASTM A325.

Anchor bolts:

ASTM A36.

Provide length of bolt threads in accordance with ASME B 18.2.1, Class 2 free bit, American National coarse thread series.

Nuts: ASTM A563.

Eyebolts: ASTM A668, Class C, forged steel.

Tower connection bolts, step bolts, and nuts:

ASTM A394 except as specified.

Tensile Strength of Bolts and Proof Load on Nuts:

16,950 pounds, minimum, for 5/8-inch bolts based on 75,000 pounds per square inch minimum ultimate unit tensile strength.

24,400 pounds, minimum, for 3/4-inch bolts based on 75,000 pounds per square inch minimum ultimate unit tensile strength.

Provide bolts and recessed hex nuts conforming to drawing 40-D-5672.

ASTM A325, Type I, bolts may be used as approved by the COR.  Submit a bolt length and grip table to be substituted for table on drawing 40-D-5672 for approval by COR.

Stud bolts:

Standard commercial quality.

Provide stud bolts with flux-filled ends suitable for arc welding to steel with automatically timed stud-welding equipment.

Washers:

ASTM F844 for ASTM A307 bolts.

ASTM F436 for ASTM A325 bolts.

Galvanize or Zinc washers.

Expansion Anchors:

CID A-A-1923, Type 4.

Bolt Length:   3-1/2 inches, minimum, embedment length except as indicated on drawings.

Galvanized or Zinc steel.

Locknuts:  Stamped or bowed sheet-metal-type locknuts conforming to drawing 40-D-5672.

Splice Bars, Bolts, and Accessories for Crane Rails:

Commercial quality free from scale and rust which can be removed by firm rubbing with burlap or equivalent treatment.

Punch splice bars for tight rail joints.

Filler metal and shielding gases or fluxes:

AWS D1.1.

Use filler metal and shielding gases suitable for base materials, positions, and other conditions.

Use filler metal with a minimum tensile strength of 70,000 lb/in2 for steel.

FABRICATION

Where details of design or fabrication are not specified or indicated on drawings, fabricate in accordance with AISC 316.

Edge plane or flame-cut plates of built-up members with mechanically guided torches and grind smooth.

Provide square and true plates after the above operation.

Edges of web plates may be flame-cut and ground where edge planning is indicated or noted on drawings.

Fabricate straight and free from sharp kinks and bends.

Remove rust and dirt.

Do not injure metal when straightening is necessary.

Shearing and Cutting:

Shear and cut accurately.

Neatly finish work to be exposed to view.

Fillet copes and reentrant cuts before cutting.

Holes:

Holes not shown on drawings or approved shop drawings are not permitted.

Structural steel less than 13/16-inch thick: Holes may be punched to full size except as noted on drawings.

Structural steel 13/16-inch or more in thickness: Drill or subpunch and ream.

Holes shown on drawings as drilled holes: Drill or subpunch and ream.

Holes:

Cylindrical.

Perpendicular to member.

Clean-cut without torn or ragged edges.

Remove burrs resulting from reaming or drilling with a tool making a 1/16-inch bevel.

Punching:

Full size punching:

Punch diameter: Nominal diameter of hole.

Die diameter: Not more than 1/16 inch larger than diameter of punch.

Subpunching:

Punch diameter: 1/4 Inch smaller than nominal diameter of hole.

Die diameter: Not more than 3/32 inch larger than diameter of punch.

Use subpunch for reamed work so that no punched surface will appear in periphery of hole after reaming.

Accuracy of Punching, Reaming, and Drilling:

Space holes accurately in accordance with drawings and locate on gauge lines.

Do not exceed 1/32-inch variation in hole spacing from spacing indicated on drawings for bolt holes.

Welding:

Perform welding and related work in accordance with AWS D1.1.

Interpret welding terms in accordance with AWS definitions.  Use welding symbols shown on drawings that comply with AWS symbols.

Process: Shielded arc-welding.

Welds:

As shown on drawings.

Continuous unless otherwise noted.

In such a manner that residual shrinkage stresses will be reduced to a minimum.

Full penetration through material except as indicated on drawings.

Stress-relieving treatment: Not required.

Measuring:

Use a steel tape or rule.

Make measurements when steel members and steel tape or rule are within 10 degrees F of ambient temperature.

Marking:

Mark anchor bolts and embedded materials with designations on drawings.

Mark other individual pieces with designations on approved erection diagrams.

Stamp markings before galvanizing.

Numerals or letters:

Height, minimum: 1/2-inch.

Clearly legible after galvanizing or final coating system.

Anchor bolts may be identified by tagging.

Baseplates:

Fabricate straight and true with square cuts.

Grind edges cut by torch straight and smooth.

Finish:

Where finish type is not specified, use the type of finish most suitable for the part to which it applies [and average or rough as defined herein.

Average Finish:

For surfaces in permanent contact where a tight joint is required.

Produce smooth surfaces.

Slight toolmarks are permitted.

Rough Finish:

For machine surfaces not requiring average finish.

Rough machine sufficient to produce a plane surface true to dimensions.

Where finish is indicated on drawings, finish members true to line and free from twists, bends, and open joints, in accordance with noted degree of finish.

Compression Joints: Face surfaces of compression joints depending on contact bearings so as to have full contact bearing when aligned and welded.

SHOP ASSEMBLY

Shop assemble one structure of each type to ensure correct fit of parts, adequate bolt lengths, and proper field erection.

Do not ream or elongate mismatched holes.

Reasonable amount of drifting is permitted.  "Reasonable amount of drifting" is defined as drifting without causing hole elongation or deformation of members.

Repair galvanizing damaged during drifting as specified below.

Dismantle shop-assembled parts before shipment.

Designate members and bolts used in shop assembly by sizes and lengths indicated on approved erection diagrams.

Should any part prevent proper assembly, determine correction required and submit revised detail drawings and data.

GALVANIZING AND COATINGS

Cleaning:

After shopwork completion, clean materials of rust, loose scale, dirt, oil, grease, slag from welded areas, and other foreign substances.

Clean surfaces of metalwork in contact with or embedded in concrete or grouting mortar in accordance with NACE 3.

Anchor bolt, embedded material, plate, and shapes:

Galvanize anchor bolts, embedded material, plates, and shapes designated on the drawings for galvanizing after fabrication.

After cleaning, zinc coat materials in accordance with ASTM A123.

Where member lengths prevent dipping in one operation, exercise care to prevent warping.

Provide finished compression members with lateral variations not greater than 1/1000 the axial length between the points which are to be supported laterally.

After galvanizing, remove excess spelter from holes.

Hardware:

Galvanize bolts, nuts, washers, and locknuts in accordance with ASTM A153.

Remove excess spelter by centrifugal spinning.

Do not galvanize erection bolts and nuts.

Externally threaded reinforcing bar anchor bolts may be metalized in lieu of galvanized.

Perform metallizing in accordance with AWS C2.2.

Metallizing zinc-coating thickness, minimum: 0.0033 inch.

Straightening After Galvanizing:

Straighten items which have been warped by the galvanizing process by rerolling or pressing.

Do not hammer or otherwise straighten material in a manner that will damage the protective coating.

Material which has been bent beyond yield point, or which has warped to the extent that it cannot be straightened without damage to the galvanized coating will be rejected.

Galvanizing repair using a two-component epoxy zinc spray coating.

CONTRACTOR SOURCE QUALITY TESTING

Inspect component parts and welds to determine conformance to drawings, procedures, overall workmanship, weld contour, weld size, and any other pertinent items.

Inspect component parts for dimensional compliance.

Matchmark mating parts.

Shop welds:

Perform inspections and tests in presence of COR and in accordance with AWS D1.1.

All welds: Visually inspect entire length in accordance with AWS D1.1.

Complete-joint-penetration welds:

Test entire length.

Ultrasonic or radiographic testing: AWS D1.1.

Partial-joint-penetration welds and fillet welds:

Test entire length of welds that fail visual inspection.

Magnetic particle or dye penetrant:

Magnetic particle testing technique and procedure: ASTM E709.

Dye penetrant technique and procedure: ASTM E165

Standards of acceptance: AWS D1.1.

When welds are found to be defective, repair welds in accordance with AWS D1.1 and reexamine welds to ensure adequacy of repairs.

EXECUTION

ERECTING STEEL STRUCTURES

Erect steel structures as specified and as shown on the drawings.

Erect by bolting, and in accordance with approved erection diagrams and bills of material.

Reasonable amount of drifting allowed.

Reaming or elongating of mismatched holes: Not permitted.

Holes more than 1/8-inch off in concentric after the connection is temporarily assembled shall be reported to the COR and approval of method of correction shall be obtained.

Place bolts with tightened and locked nuts within the shortest practicable time after members have been placed.

Where incomplete bolting is determined to be a major factor in structure damage, repair or replace the structure.

If shop errors in structure materials are discovered, submit revised shop drawings for approval of proposed field correction or return materials to the manufacturer for correction or replacement.

Galvanizing Repair:

Repair damage to galvanizing with two-component zinc epoxy spray coating.

Anchor bolts, embedded material, stubs, and footings:

Anchor embedment depths shall comply with ACI 355.2 and ACI 355.4.

Reuse existing anchor holes, if not suitable for new installation patch hole with grout mixture and drill new holes.

Set and hold anchor bolts, embedded material, stub angles, or tees in correct position and alignment during the placing and setting of the concrete in concrete footings.

Protect contact faces and bolt holes from damage and thoroughly clean them of any contamination prior to assembly.

Do not erect any structure on concrete footings until 7 days after the last placement of concrete in those footings, or until all backfill has been placed and compacted where and as required around those footings.

Unless specified otherwise, use metal braces, supports, and other items to position and align embedded metalwork, and which will also be embedded in concrete.  Do not use wooden braces, supports, or other items to position and align embedded metalwork.

Construct suitable blockouts in concrete where required for installation of the metalwork.  After installation of the metalwork, fill blockouts with concrete or grout as shown on the drawing.

Drill, or drill and tap as required, holes in metalwork required for installation of metalwork.

Where shown on the drawing, fill spaces under metalwork completely as required with nonsettling grouting mortar or] grouting mortar in accordance with Section 03 62 20 - Grouting Mortar for Equipment and Metalwork.

Erection:

Structures may be erected by assembling sections on the ground and hoisting successive sections into place, or by being built up in place with individual members.

If erected in sections, perform initial bolting adequate for dead and live loads and erection stresses, but not so tight as to prevent aligning and fitting adjacent sections or members.

Correct misalignment or misfit attributable to the method of erection corrected by adjusting erection methods as necessary to eliminate the problems.

In the completed structure, enter connection bolts to the head.

Tightening torque:

5/8-inch-diameter bolts: 125 to 165 pound-feet

3/4-inch-diameter bolts: 180 to 235 pound-feet.

Lock structural nuts in place:

Bowed sheet metal-type locknuts: Tighten sufficiently to flatten concave face into full contact against structural nut.

Stamped sheet metal-type locknuts: Turn one-third turn beyond contact with structural nut.

Do not use wrench which may deform nuts or cut or flake the galvanizing.

Perform field welding, including welding processes and welding operators, conforming to the requirements of AWS D1.1.

Cleaning: Clean structures of foreign matter that will tend to permanently adhere, with special attention to corrosive salts or substances.

CONTRACTOR FIELD QUALITY TESTING

Field welds:

Perform inspections and tests in presence of COR and in accordance with AWS D1.1.

All welds: Visually inspect entire length in accordance with AWS D1.1.

Complete-joint-penetration welds:

Test entire length.

Ultrasonic or radiographic testing: AWS D1.1.

Partial-joint-penetration welds and fillet welds:

Test entire length of welds that fail visual inspection.

Magnetic particle or dye penetrant:

Magnetic particle testing technique and procedure: ASTM E709.

Dye penetrant technique and procedure: ASTM E165

Standards of acceptance: AWS D1.1.

When welds are found to be defective, repair welds in accordance with AWS D1.1 and reexamine welds to ensure adequacy of repairs.

END OF SECTION

- SECTION 05 50 00

- METAL FABRICATIONS

  - GENERAL

    - MEASUREMENT AND PAYMENT

      - Miscellaneous metalwork:

        - Payment: Lump-sum price offered in the schedule.

    - COST

      - Include cost of furnishing and installing staircase, hand railing, and entrance hand railing panels, including cost of excavation, placing and compacting backfill, replacing gravel surfacing, anchor bolts, and materials and labor in the applicable prices offered in the schedule for required work.

    - REFERENCE STANDARDS

      - American Institute of Steel Construction (AISC)

        - AISC 316-89	Manual of Steel Construction - Allowable Stress Design - 9th Edition

      - American Society of Mechanical Engineers (ASME)

        - ASME B18.2.1-1996(2005)	Square and Hex Bolts and Screws, Inch Series

      - ASTM International (ASTM)

        - ASTM A36/A36M-19	Carbon Structural Steel

        - ASTM A53/A53M-24	Pipe, Steel, Black and Hot-dipped, Zinc-Coated Welded and Seamless

        - ASTM A108-24 	Steel Bars, Carbon, Cold-Finished, Standard Quality

        - ASTM A123/A123M-24	Zinc (Hot-Dip Galvanized) Coatings on Iron and Steel Products

        - ASTM A153/A153M-23	Zinc Coating (Hot-Dip) on Iron and Steel Hardware

        - ASTM A307-21	Carbon Steel Bolts and Studs, 60,000 psi Tensile Strength

        - ASTM A354-17e2	Quenched and Tempered Alloy Steel Bolts, Studs, and Other Externally Threaded Fasteners

        - ASTM A385/A385M-22	Providing High-Quality Zinc Coatings (Hot-Dip)

        - ASTM A500/A500M-23	Cold-Formed Welded and Seamless Carbon Steel Structural Tubing in Rounds and Shapes

        - ASTM A501/A501M-21	Hot-Formed Welded and Seamless Carbon Steel Structural Tubing

        - ASTM A513/A513M-24	Electric-Resistance-Welded Carbon and Alloy Steel Mechanical Tubing

        - ASTM A563/A563M-24	Carbon and Alloy Steel Nuts

        - ASTM A653/A653M-23	Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process

        - ASTM A668/A668M-23	Steel Forgings, Carbon and Alloy, for General Industrial Use

        - ASTM A780/A780M-20	Repair of Damaged and Uncoated Areas of Hot-Dip Galvanized Coatings

        - ASTM A786/A786M-15(2001) Hot-Rolled Carbon, Low-Alloy. High Strength Low-Alloy, and Alloy Steel Floor Plates

        - ASTM A992/A992M-22	Structural Steel Shapes

        - ASTM A1008/A1008M-24	Steel, Cold-Rolled, Carbon, Structural, High-Strength Low-Alloy and High-Strength Low-Alloy with Improved Formability

        - ASTM A1011/A1011M-23	Steel, Sheet and Strip, Hot-Rolled, Carbon, Structural, High-Strength Low-Alloy and High-Strength Low-Alloy with Improved Formability

        - ASTM D1056-20	Flexible Cellular Materials - Sponge or Expanded Rubber

        - ASTM F436/F436M-24	Hardened Steel Washers Inch and Metric Dimensions

        - ASTM F844-19(2024)	Washers, Steel, Plain (Flat), Unhardened for General Use

        - ASTM F1267-18(2023)	Metal, Expanded, Steel

        - ASTM F1554-20	Anchor Bolts, Steel, 36, 55, and 105 ksi Yield Strength

        - ASTM F3125/F3125M-23	High Strength Structural Bolts and Assemblies, Steel and Alloy Steel, Heat Treated, Inch Dimensions 120 ksi and 150 ksi Minimum Tensile Strength, and Metric Dimensions 830 MPa and 1040 MPa Minimum Tensile Strength

      - American Welding Society, Inc. (AWS)

        - AWS D1.1-2020	Structural Welding Code – Steel

      - Commercial Item Description (CID)

        - CID A-A-1923A	Shield, Expansion (Lag, Machine and Externally Threaded Wedge Bolt Anchors)

      - Federal Specifications (FS)

        - FS FF-S-85D	Screw, Cap, Slotted and Hexagon Head

        - FS RR-C-271D(1)	Chains and Attachments, Welded and Weldless

        - FS RR-T-650E	Treads, Metallic and Nonmetallic, Skid Resistant

      - National Association of Architectural Metal Mfrs. (NAAMM)

        - NAAMM 510-92	Metal Stairs Manual

        - NAAMM 531-24	Metal Bar Grating Manual

      - Society of Protective Coatings (SSPC)/NACE International (NACE)

        - SSPC SP7/NACE 4-07	Brush-Off Blast Cleaning

    - DEFINITIONS

      - Miscellaneous metalwork: Where either shown on the drawings or specified elsewhere in this Section or these specifications means metal fabrications as used in this Section.

    - QUALIFICATIONS

      - Qualify welders in accordance with AWS D1.1 using procedures, materials, and equipment of the type required for the work.

    - DELIVERY, STORAGE, AND HANDLING

      - Protect from corrosion, deformation, and other types of damage.

      - Store items in an enclosed area free from contact with soil and weather.

  - PRODUCTS

    - MATERIALS

      - Arc-Welding Electrodes:

        - Filler metal and required shielding gases or fluxes: AWS D1.1.

        - Filler metal for steel, minimum tensile strength: 70,000 psi.

      - Bolts, Nuts, And Washers:

        - Eyebolts: Forged steel, ASTM A 668, class C.

        - Nuts: ASTM A 563.

        - Capscrews:  FS FF-S-85.

        - Washers:

          - For use with ASTM A 325 bolts: ASTM F 436

          - Unhardened for general use: ASTM F 844.

        - Bolts:

          - ASTM A 307, except anchor bolts.

          - Anchor bolts as shown on drawings

          - ASTM A 36 steel

          - Length of bolt threads: ASME B18.2.1.

          - Thread class:  2 free-fit, American National coarse-thread series.

      - Chain:  FS RR-C-271, type 1, class 1, 2, or 4.

      - Expanded Metal: ASTM F 1267, Type I, Class 2, Grade A; or Type II, Class 2, Grade A.

      - Grating: NAAMM 531 galvanized steel.

        - Where safety grating is required, provide serrated-edge grating.

        - Banding bar: Same size as bearing bars, where required.

      - Metal Safety Treads:

        - FS RR-T-650, composition D, type 5, 4 inches wide.

        - One-piece treads having suitable anchors for anchoring to concrete.

      - Sheet Steel: ASTM A 1008, or ASTM A 1011 galvanized sheet steel flat.

      - Steel Floor Plate: ASTM A 786, pattern 1, 2, 3, or 5, with material conforming to ASTM A 36.

      - Steel Pipe: ASTM A 53, type E or S, grade B.

        - Unless otherwise shown on drawings, provide standard-weight, black, steel pipe.

        - Government inspection at the mill and hydrostatic tests will not be required.

      - Structural steel:

        - Shapes except wide flange sections: ASTM A 36.

        - Wide flange sections: ASTM A 992/A 992 M

      - Square Hollow Structural Shapes (HSS):

        - HSS 2 inches and larger: ASTM A 500 or A 501

        - HSS less than 2 inches: ASTM A 513.

      - Round Hollow Structural Shapes (HSS): ASTM A500, Grade B.

    - SAFETY RAIL

      - Design:

        - Working load: 250 pounds.

        - Permit user to ascend or descend hands free.

        - Maximum possible fall length: 6 inches.

      - Main support rail:

        - Continuous.

        - Notchless.

        - Mount permanently to ladder.

      - Slide or safety sleeve: Equipped with locking mechanism for making connection between rail and safety belt.

      - Components: Non-corrosive, i.e. fiberglass, aluminum, or stainless steel.

    - ANCHORS

      - Adhesive anchors: All-thread anchor rod, nut, washer, and adhesive capsule manufactured by HILTI, PO Box 21148, Tulsa OK 74121; Rawlplug Company, Two F.B. Powers Square, New Rochelle, NY 10802; Williams Form Engineering Corp., PO Box 7389, Grand Rapids, MI 49510, or equal, having the following essential characteristics:

        - Rods with rolled threads: ASTM A 307, grade A.

        - Resin adhesive capsules with hardener.

        - Minimum allowable tensile strength 20,000 pounds for 5/8-inch diameter rods when installed in concrete with 4,000 pounds-per-square-inch compressive strength.

        - Minimum bolt embedment length: One capsule length, unless indicated otherwise on drawings.

      - Expansion Anchors:

        - CID A-A-1923, Type 4, galvanized steel, except stainless steel anchors when submerged in water.

        - Bolt length: As shown on drawings.  If not shown, provide bolt length with 3- 1/2- inch minimum embedment.

      - Stud Anchors: ASTM A 108, deformed bar anchors and stud anchors.

        - Flux-filled ends suitable for end welding to steel with automatically timed stud-welding equipment.

    - FABRICATION

      - Fabricate metalwork in accordance with AISC 316, AA ADM-105, and these specifications.

        - Perform welding and related work in accordance with AWS D1.1.

        - Grind all welds on ladders, grab bars, pipe guardrails, wall rails, and handrails smooth.

      - If straightening is necessary, use methods that will not injure the metal.

      - After shop work completion and before galvanizing, if required, clean material of rust, loose scale, dirt, oil, grease, slag from welded areas, and other foreign substances.

      - Railings, fabricate from:

        - Standard weight pipe with diameter shown on drawings, or

        - Round HSS of the same outside diameter and minimum wall thickness of 0.132-inches.

      - Ladders and grab bars, fabricate from:

        - Standard weight pipe with diameter shown on drawings, or

        - Round HSS of the same outside diameter and minimum wall thickness of 0.132-inches, or

        - ASTM A 36 steel shapes.

      - Galvanizing:

        - Galvanize items of metalwork as specified or shown on drawings.

        - Galvanize bolts, nuts, washers, and locknuts in accordance with ASTM A 153.  Remove excess spelter by centrifugal spinning.

        - Galvanizing Repair:

          - Repair by two component epoxy zinc primer.

          - If the galvanized coating becomes damaged after being dipped twice, the material will be rejected.

          - Where local repair is authorized:

            - Repair damage to galvanizing using a two-component zinc epoxy spray coating.

  - EXECUTION

    - PREPARATION

      - Where locations and dimensions of miscellaneous metalwork shown on drawings are dependent upon existing equipment and/or equipment furnished, confirm locations and dimensions prior to fabrication of miscellaneous metalwork.

    - INSTALLATION

      - Blockouts:

        - Construct suitable blockouts in concrete where required for installation of metalwork.

        - After installation of metalwork, fill blockouts with concrete or grout as shown on drawings.

      - Embedded Metalwork:

        - Accurately locate metalwork to be embedded in concrete.  Hold metalwork in correct position and alignment and protect metalwork from damage and displacement during placing and setting of concrete.

        - Install safety treads and risers in a rigid and substantial manner.  After installation is completed, clean safety threads of foreign material.

        - Unless otherwise specified, use only metal braces, supports, and other items to position and align embedded metalwork, which will be embedded in concrete.  Do not use wooden braces, supports, or other items to position and align embedded metalwork if they will also be embedded in concrete.

        - Clean surfaces of metalwork to be in contact with or embedded in concrete grouting mortar in accordance with NACE 3.

      - Anchors:

        - Contractor may use suitable adhesive or expansion anchors meeting requirements of this Section in lieu of embedded concrete inserts shown on drawings.

        - Drill holes for anchors straight and true and of diameter recommended by anchor manufacturer.

        - Install anchors in accordance with manufacturer's recommendations.

        - Follow manufacturers recommendations when embedded steel or reinforcement is encountered during drilling for anchors.

        - When drilling water is used, clean surfaces of concrete to remain exposed immediately to prevent discoloration.

      - Galvanizing Repair:

        - Repair by two component epoxy zinc primer.

      - Grout:

        - Where shown on drawings, fill spaces under metalwork completely as required with nonsettling grouting mortar or] grouting mortar in accordance with Section 03 62 20 - Grouting Mortar for Equipment and Metalwork.

        - Clean surfaces of metalwork to be in contact with or embedded in concrete or grouting mortar.

      - Holes in Metalwork:

        - Drill, or drill and tap as required, holes in metalwork required for installation.

- END OF SECTION

SECTION 09 97 00

METALIZING AND COATINGS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Include in prices offered schedule for other items of work.

REFERENCE STANDARDS

ASTM International (ASTM)

ASTM A 588-24	High Strength Low-Alloy Structural Steel, up to 50 ksi (345 MPa) Minimum Yield Point, with Atmospheric Corrosion Resistance

ASTM A 780-09(2015)	Repair of Damaged and Uncoated Areas of Hot Dip Galvanized Coatings

ASTM B 833-20	Zinc and Zinc Alloy Wire for Thermal Spraying (Metallizing) for the Corrosion Protection of Steel

ASTM D 522-17(2021)	Mandrel Bend Test of Attached Organic Coatings

ASTM D 1002-10(2019)	Apparent Shear Strength of Single-Lap-Joint Adhesively Bonded Metal Specimen by Tension Loading (Metal-to-Metal)

ASTM D 4285-24	Indicating Oil or Water in Compressed Air

ASTM D 4541-22	Pull-Off Strength of Coatings Using Portable Adhesion Testers

Federal Standards (Fed Std)

Fed Std 595C(1)-08	Colors Used in Government Procurement

Military Specifications (MIL) and Quality Products List (QPL)

MIL-PRF-16173E-06	Corrosion Preventive Compound, Solvent Cutback, Cold-Application Qualified products listed by Qualified Data Set (QDS) in the Qualified Products Database (QDS)

The Association for Materials Protection and Performance (AMPP) includes Society for Protective Coatings (SSPC) and National Association of Corrosion Engineers International (NACE)

NACE SP 0287-16	Field Measurement of Surface Profile of Abrasive Blast Cleaned Steel Surfaces Using a Replica Tape

SSPC-AB 1-15(2019)	Mineral and Slag Abrasives

SSPC AB 3-17	Newly Manufactured or Re-Manufactured Steel Abrasives

SSPC-CS-23.00/AWS C2.23M/NACE 12-16 Application of Thermal Spray Coatings (Metallizing) of Aluminum, Zinc, and Their Alloys and Composites for the Corrosion Protection of Steel

SSPC-PA 2-15(2018)	Measurement of Dry Paint Thickness with Magnetic Gages

SSPC SP 1-15(2016)	Solvent Cleaning

SSPC-SP 2-18	Hand Tool Cleaning

SSPC-SP 3-18	Power Tool Cleaning

SSPC-SP 5/NACE 1-07	White Metal Blast Cleaning

SSPC-SP 7/NACE 4-07	Brush-Off Blast Cleaning

SSPC-VIS3-04	Visual Standard for Power- and Hand-Tool Cleaned Steel

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 09 97 00-1, Approval Data:

For each coating material:

Manufacturer’s designated product name, product data, application, and MSDS sheets.

Supplier’s name, address, phone number, purchase order number, and batch number(s) for each coating material.

Identification of “Items to be coated” including sub-letter and sub-number listed in Coating Tabulations.

Manufacturer’s certification that material meets Federal or SSPC specification requirements.

Color: Provide samples that match Corten rusted metal appearance.

RSN 09 97 00-2, Final Approval Data:

For each coating material and thinner.

Purchase orders including order number, date, manufacturer’s designated product name, batch number(s), and quantity purchased.

Color: Final government approved finish.

RSN 09 97 00-3, Qualifications:

Coating applicator:

Current Coating Application Specialist (CAS) or Plural Component Application Certification (PCAC).

List of not less than 3 comparable jobs for each coating type including owner’s name, address, and telephone number.

RSN 09 97 00-4, Quality Control Plan:

Quality Control Manager:

Name and title.

Quality Control Procedure:

Quality control plan.

Procedure for addressing identified non-conformities.

QUALIFICATIONS

Coating Applicator’s Qualifications:

General Coatings:  The Contractor shall employ trained, experienced applicators with the AMPP, NACE, and/or SSPC certifications.

Metalizing Coatings: The Applicator shall have been regularly engaged in metallizing work for the past 3 years.

DELIVERY, STORAGE, HANDLING

Deliver materials to jobsite in original, undamaged, unopened containers labeled with manufacturer’s name, designated product name, batch number, date of manufacture, and any special instructions.

Deliver materials in containers not larger than 5 gallons as packaged by manufacturers unless suitable equipment is provided at jobsite to handle and thoroughly mix materials in larger containers.

Store coating materials and thinners in accordance with the manufacturer's recommendations, and as a minimum, store off the ground, under cover, and at 50 to 95 degrees F.

Do not use coating material which has exceeded manufacturer’s specified storage stability period (shelf life).

AMBIENT CONDITIONS

Maintain environmental conditions to meet coating manufacturer’s requirements during surface preparation, coating application, and curing period.  Provide heating or cooling, and dehumidification as needed.  Complete curing before placing the coating systems into service.

Coatings shall be applied when the substrate surface temperature is 5 degrees F or more above the dew point temperature and for at least 8 hours after application.

PRODUCTS

EQUIPMENT

Surface preparation and spray application equipment:

Sized in accordance with coating manufacturer’s recommendations.

Provide:

Pressure gauges and pressure regulators.

Air supply lines free from oil and moisture.

ABRASIVES

Mineral and slag abrasives:  Meets SSPC-AB1, type I (natural minerals) and type II (slags), class A, except the following are not permitted:

Flint minerals.

Ferrous metallic abrasives shall meet SSPC-AB2 for recycled cleanliness and SSPC AB3, Class I (steel) or II (iron) for angular shaped grit.

Do not exceed toxicity threshold limit for hazardous materials.

Zinc Metal:

ASTM B 833: 99.99 Zinc (Z13005)

Caulk: ASTM C 920, Type S, Grade NS, Class 35, and Use I.

COATINGS

Specified in Coating Tabulations:

Apply only one coating system per option in Coating Tabulations.

Apply compatible products of same manufacturer for coating system components.

Volatile Organic Compounds (VOC): Do not exceed maximum permitted by Federal, State, and local air pollution control regulations as supplied in container or by addition of thinner material.

Factory color or tint only.  Do not color or tint at jobsite.

Use manufacturer’s recommended thinners for coating materials.

Do not use accelerator products unless approved by COR.

EXECUTION

CONTAINMENT SYSTEM

Design and construct containment systems to control emissions to the environment in conformance with Federal, State, and local regulations and the following criteria:

Do not exceed ambient air quality standards.

Prevent contamination of soil.

Design and construct containment system for following:

Suitable for surface preparation and application method specified.

Containment and ventilation components in accordance with SSPC Guide 6.

Perform the following activities within containment system:

Coating removal,

Surface preparation,

Waste and debris containerization,

Activities which may spread contamination outside contained area.

Provide signs and barriers at each point of entry to containment area to prevent unauthorized personnel access.

PROTECTION OF ADJACENT SURFACES, EQUIPMENT AND NEWLY COATED SURFACES

Protect items or surfaces not to be coated and adjacent to surfaces to be cleaned and coated from contamination and damage during cleaning and coating operations.

Examples include protecting mating and machined surfaces, mechanical and electrical equipment (open or enclosed), instruction and similar plates, and wet and newly coated surfaces.

Remove, mask, or otherwise protect items or surfaces not coated from contamination and damage during cleaning and coating operations.

REPAIR OF CONTRACTOR-DAMAGED SURFACES

Repair items, equipment, or surfaces which are damaged or contaminated as determined by Contracting Officer.

SURFACE PREPARATION

Remove or repair weld spatter, slag burrs, porosity, sharp edges, pits, laminations, crevices, or other objectionable surface irregularities before specific surface preparation.

SURFACE PREPARATION

Remove or repair surface irregularities and impart profile as specified:

Welds:

Welds: Smooth and continuous.

Weld spatter, slag, buckshot, laminations, and slivers: remove and grind smooth.

Undercuts and porosity: Fill with weld material and grind smooth.

Projections, sharp edges, high points, fillets, and corners: Grind smooth to a radius of at least 1/16-inch.

Substrate defects:

Pitting, gouges, scratches, porosity, and similar:

Grind smooth.

Defects exceeding 20 percent reduction in wall thickness require weld repairs.

Grind full length of edges and corners smooth to a radius of at least 1/16-inch.

Pre-Cleaning:

SSPC-SP WJ-4/NACE WJ-4.

Water: Potable quality and may be heated.

After washing, allow surface to dry before subsequent cleaning and coating.

Surfaces: Visually free of dust, dirt, animal waste, oil, grease, and other debris after pressure washing.

Surface preparation method:

See Coating Tabulations.

Method A: SSPC-SP1.

Method B: SSPC-SP6/NACE 3.

Methods B-1:

Repair of defective or damaged coated areas.

For metal substrate:

SSPC-SP6/NACE 3.

SSPC-SP11 where abrasive blasting is impractical.

For surrounding intact coating:

Nominal 2 to 4-inch margin.

SSPC-SP1.

Feather abrupt edges and around repair area by hand or power tool with non-woven pad into adjacent tightly bonded coatings.

Roughen or abrade surface in accordance with manufacturer’s recommendations to achieve matted or lusterless finish.

SSPC-SP1.

Method C: SSPC-SP10/NACE 2.

Method C-1: Repair of defective or damaged coated areas.

For metal substrate:

SSPC-SP10/NACE 2.

SSPC-SP11 where abrasive blasting is impractical.

For surrounding intact coating (nominal 2 to 4-inch margin):

SSPC-SP1.

Feather abrupt edges and around repair area by hand or power tool with non-woven pad into adjacent tightly bonded coatings.

Roughen or abrade surface in accordance with manufacturer’s recommendations to achieve matted or lusterless finish.

SSPC-SP1

Method H: SSPC-SP5/NACE 1.

Method I (galvanized): Repair of defective or damaged galvanized surface. Use following:

To steel substrate:

SSPC-SP7/NACE 4.

SSPC-SP3, where abrasive blast cleaning is impractical.

Galvanized surface:

Method A and treat with conditioner specific for galvanized substrate in accordance with manufacturer’s instructions.  Rinse surface with potable water.

Sweep blast.

Compare prepared steel surfaces to following visual reference photographs for allowable visible contaminants and stains:

SSPC-VIS1 for abrasive blast cleaning.

SSPC-VIS3 for power and hand tool cleaning.

Assessment of dust contaminants:

Perform clear tape test method in accordance with ISO 8502-3.

Perform three tests per shift.

Acceptance Criteria: Degree of cleanliness shall meet Class 3.

Surface profile:

Prepare as specified in Coating Tabulations.

Where not specified, prepare in accordance with manufacturer’s instructions for metallic substrates or existing coating surfaces and service environment.

Where manufacturer’s instructions do not specify a surface profile, prepare blasted surfaces to following profile:

Atmospheric Service Environments: 1 mil or greater angular profile and less than specified millage of first applied coat.

Burial and Immersion Service Environments: Angular profile 2 mil or greater and less than specified millage of first applied coat.

Inspect surface profile in accordance with NACE RP 0287:

Use replica tape suitable for surface profile depth range.

Perform at least two tests per 1,000 square feet.

Re-clean or perform additional surface preparation of contaminated surfaces before coating application.

APPLICATION EQUIPMENT

Air compressor and spray application equipment:

Provide appropriate size and type air compressor and spray application equipment adequately sized, in proper working order, and equipped with pressure gauges and regulators.

Compressed air quality:

Perform tests in accordance with ASTM D4285.

Perform test at the beginning of every shift.

Spray equipment supply lines free from oil and moisture during operations and perform tests in accordance with oil and moisture inspection procedures.

COATING APPLICATION

Apply in accordance with manufacturer’s instructions and Coating Tabulations.

Metallized coatings applications shall be performed only on thoroughly clean and dry surfaces.  Coatings that are damaged by the elements shall be replaced.

Prevent overspray or dry spray.  When such conditions occur, clean surfaces prior to application of subsequent coats.

Re-clean contaminated surfaces between coats.

Coating Adhesion:

Test in accordance with ASTM D4541 at the direction of the COR.

CONTRACTOR FIELD QUALITY TESTING

Equipment: Inspect air supply lines on air compressors for oil and moisture in accordance with ASTM D 4285.  Remove oil or water before proceeding with work.

Surface Profile: Inspect the surface profile in accordance with NACE RP 0287 for compliance with specified requirements.

Use replica tape suitable for surface profile depth range.

Visual Comparison of Prepared Surfaces:

Compare prepared steel surfaces to following visual reference photographs for allowable visible contaminants and stains:

SSPC-VIS1 for abrasive blast cleaning.

Inspection Equipment:

Calibrate inspection devices and equipment in accordance with the manufacturer’s instructions.

Verify calibration prior to every use.

Qualified personnel shall conduct tests and sign reports.

Perform tests in presence of the COR:

Provide rigging for COR to observe inspections.

Make daily reports for review by the COR, including:

Date of work.

Description of areas and work performed.

Environmental conditions.

Results from the following quality control tests:

Surface profile.

Visual comparison of prepared surfaces.

Assessment of dust contaminants.

Compressed air quality.

Dry film thickness (DFT).

Coating adhesion testing (when directed by the COR).

Final inspection will take place after repairs, but before entering into service.

REPAIR AND REMEDIAL WORK

Repair defective or damaged areas of coatings in accordance with coating manufacturer’s recommendations.

COATING TABULATIONS

Tabulation No. AE-11

Items to be coated:

- Gratings - Handrails and safety railings - Platforms - Damaged galvanized surfaces not otherwise tabulated

Repair damaged galvanized surfaces in accordance with ASTM A 780. Repair materials containing cadmium and lead are not permitted.

Tab AE-11.DOCX

| Tabulation No. AE-12 |  |  |
| --- | --- | --- |
| Items to be coated: |  |  |
| - Machine-finished surfaces of ferrous metalwork, including screw threads |  |  |
| Notes: 1. Remove corrosion prevention compound before installation of equipment. 2. Do not abrade or etch metal surfaces during application or removal. |  |  |
| Coating materials | Number and thickness of coats | Surface preparation method |
| Corrosion prevention compound: Meets: MIL-PRF-16173E*, Class II, Grade I, hard film, and QPL-16173* approved | 1 heavy coat | Unpainted surfaces: SSPC-SP1 Previously painted surfaces: Remove all coatings with commercial paint striper |
| *Military Specification MIL-PRF-16173E and QPL-16173: Prepared by Naval Sea Systems Command (Ship Systems), Commander, Naval Sea Systems Command – Command Standards Executive Program Office, 1333 Isaac Hull Avenue, Southeast, Washington Navy Yard DC 20376, (202) 781-4563 |  |  |
| Tab AE-12.DOCX |  |  |

| Tabulation No. Metal |  |  |
| --- | --- | --- |
| Items to be coated: |  |  |
| - Exterior surfaces: Steel work platform supports, stairs, and handrails if not constructed using ASTM A 588 standards - Expansion bolts, nuts, and hardware |  |  |
| Coating material – Thermal Sprayed Zinc (Metallized) Coating | Number and thickness of coats | Surface preparation method |
| For ferrous surfaces: Zinc Metal 99.99 (Z13005) | Apply at 8-10-mil DFT, minimum. | Method A followed by Achieve 3-5 mil angular surface profile, by: Method H In accordance with SSPC-CS 23.00/ AWS C2.23M/ NACE No. 12 |
|  |  | 099CT_Metal.DOC |

COLOR SCHEDULE

Colors and glosses of finished coats:

Corten rusty, final look to be selected by the COR.

END OF SECTION

This page intentionally left blank.

- SECTION 10 18 00

- INFORMATIONAL KIOSKS

  - GENERAL

    - MEASUREMENT AND PAYMENT

      - Informational kiosks bulletin board panels:

        - Payment: Lump sum price offered in the Price Schedule.

    - REFERENCE STANDARDS

      - ASTM International (ASTM)

        - ASTM B 209-14	Aluminum and Aluminum-Alloy Sheet and Plate

        - ASTM B 221-20	Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes

        - ASTM B 632/B632M-18	Aluminum-Alloy Rolled Tread Plate

        - ASTM D 256-10(2018)	Determining the Izod Pendulum Impact Resistance of Plastics

        - ASTM D 1929-20	Determining Ignition Temperature of Plastics

        - ASTM D 2394-17	Simulated Service Testing of Wood and Wood-Based Finish Flooring

        - ASTM D 4060-19	Abrasion Resistance of Organic Coatings by the Taber Abraser

        - ASTM D 4329-13	Fluorescent Ultraviolet (UV) Lamp Apparatus Exposure of Plastics

        - ASTM D 6108-19	Compressive Properties of Plastic Lumber and Shapes

        - ASTM D 6109-19	Flexural Properties of Unreinforced and Reinforced Plastic Lumber and Related Products

        - ASTM D 6111-19a	Bulk Density and Specific Gravity of Plastic Lumber and Shapes by Displacement

        - ASTM D 6117-18	Mechanical Fasteners in Plastic Lumber and Shapes

        - ASTM D 6341-21	Determination of the Linear Coefficient of Thermal Expansion of Plastic Lumber and Plastic Shapes Between -30 and 140˚F (-34.4 and 60˚C)

        - ASTM E 84-21	Burning Characteristics of Building Materials

      - Society of Protective Coatings (SSPC) / NACE International (NACE)

        - NACE3 / SSPC-SP6-15	Commercial Wet Blast Cleaning

        - NACE 4 / SSPC-SP7-15	Brush-Off Wet Blast Cleaning

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 10 18 00-1, Approval Data:

Manufacturer’s product data and installation recommendations.

Manufacturer’s maintenance recommendations and operational manual:

  - PRODUCTS

    - INFORMATIONAL KIOSKS

      - Kiosk Sign Panel(s) manufactured by Rockart Signs and Markers, Inc. 3006 N. Maple, Mesa Arizona 85215 or equal having the following essential characteristics:

        - Phone No. 480) 854-3400

        - Web Site:  http://www.rockartsigns.com

        - The Informational Kiosk intrinsic characteristics are as follows:

          - Frame: Aluminum or composite plastic

            - Swing up door with glazing

            - Weatherproof with long-life rubber gaskets for exterior use

            - Swinging doors shall have a tamper proof locking system

          - Background: Synthetic cork or other Government approved material

          - Color: As approved by the Government

          - Glazing: Heavy duty polycarbonate

          - Non-Illuminated

        - Sign-Panel Size: 72-Inches (W) by 48-Inches (H) by 4-Inches (D) featuring fully enclosed double-sided panel.

          - Graphic Insert Size: 66-Inches (W) by 40-inches (H) by 3-inches (D)

          - Dimension Variations:

            - Width (W): 2-inches

            - Height (H): 2-Inches

            - Depth (D): ±2-Inch

Hardware: Tamper-resistant features

        - Vertical Support Posts:

Posts options:

Steel:  Powder coated steel 6-inches (W) by 6-inches (D) by 120-inches (L) having a minimum wall thickness of 0.188-inch (3/16”-inch).  Color of powder coating shall be approved by the Government.

Wood or Composite: 5.5-inches (W) by 5.5-inches (D) by 120-inches (L) solid core:

Wood shall be pressure-treated southern pine.

Composite shall be 98-percent recycled plastic materials.  Color and texture shall provide a cedar grain finish.

        - Mounting:

          - Screws: Noncorrosive screws of type and size recommended by sign manufacturer for required mounting application.

  - EXECUTION

    - INSTALLATION

      - Mount as shown on drawing no. 465-512-60017.

      - Install level and secure at approved location in the field.

Location of kiosk will be determined between the Bureau of Reclamation and New Mexico State Parks personnel.

- END OF SECTION

This page intentionally left blank.

- SECTION 31 23 02

COMPACTING EARTH MATERIALS

GENERAL

MEASUREMENT AND PAYMENT

Cost:

Including cost of compacting earth materials in prices offered in the Price Schedule for compacting backfill and for other items of work where earth materials are required to be compacted.

Includes furnishing water and moistening materials.

Includes Contractor Quality Testing.

DEFINITIONS

Use definitions from ASTM D653.

Control Fraction: The portion of a soil sample consisting of particles smaller than a designated sieve size.  The fraction is used to compare in-place unit weight with standard laboratory unit weight.  The control sieve size depends on the laboratory test used to determine laboratory maximum density.

C-Value:  The ratio expressed as a percentage of (1) in-place unit weight at fill moisture content to (2) the wet unit weight of a laboratory-compacted specimen prepared at fill moisture content as determined by the rapid method of construction control in accordance with ASTM D5080.  The C-Value is a comparison of compactive effort of field compaction equipment to standard laboratory compactive effort.

D-value: The ratio expressed as a percentage of (1) in-place wet unit weight at fill moisture content to (2) laboratory maximum wet unit weight as determined from a compaction curve constructed at fill moisture content as determined by the rapid method of construction control ASTM D5080.  The D-value is the equivalent of percent compaction in accordance with ASTM D698.

Percent Compaction: The ratio expressed as a percentage of the in-place unit dry unit weight to the laboratory maximum dry unit weight.

Special compaction: Compaction close to structures or in spaces not accessible by standard width rollers.

REFERENCE STANDARDS

ASTM International (ASTM)

ASTM D653-24a	Terminology Relating to Soil, Rock, and Contained Fluids

ASTM D698-12(2021)	Laboratory Compaction Characteristics of Soil Using Standard Effort (12,400 ft-lbf/ft3 (600 kN-m/m3))

ASTM D1140-17	Amount of Material in Soils Finer than the No. 200 (75-µm) Sieve

ASTM D1556/D1556M-24	Density and Unit Weight of Soil in Place by the Sand-Cone Method

ASTM D2216-19	Laboratory Determination of Water (Moisture) Content of Soil and Rock by Mass

ASTM D2487-17e1	Classification of Soils for Engineering Purposes (Unified Soil Classification System)

ASTM D2488-17e1	Description and Identification of Soils (Visual-Manual Procedure)

ASTM D4318-17e1	Liquid Limit, Plastic Limit, and Plasticity Index of Soils

ASTM D4546-21	One-Dimensional Swell or Collapse of Cohesive Soils

ASTM D4647/D4647M-13(2020)	Identification and Classification of Dispersive Clay Soils by the Pinhole Test

ASTM D4718/D4718M-15(2023)	Correction of Unit Weight and Water Content for Soils Containing Oversize Particles

ASTM D4914/D4914M-16	Density of Soil and Rock in Place by the Sand Replacement Method in a Test Pit

ASTM D5030/D5030M-21	Density of Soil and Rock in Place by the Water Replacement Method in a Test Pit

ASTM D5080-20	Rapid Determination of Percent Compaction

ASTM D6572-21 	Determining Dispersive Characteristics of Clayey Soils by the Crumb Test

ASTM D6913/D6913M-17	Particle-Size Distribution (Gradation) of Soils Using Sieve Analysis

ASTM D6938-23	In-Place Density and Water Content of Soil and Soil-Aggregate by Nuclear Methods (Shallow Depths)

ASTM D7382-20	Determination of Maximum Dry Unit Weight and Water Content Range for Effective Compaction of Granular Soils Using a Vibrating Hammer

Bureau of Reclamation (USBR)

USBR Guidelines for Earthwork Construction Control Testing of Gravelly Soils, Earth Sciences and Research Laboratory, Technical Service Center, Bureau of Reclamation, Denver, Colorado, September 2008.

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 31 23 02-1, Test Results:

Results:

Notify COR within 1 hour if testing does not meet specified requirements.

Provide written results within 24 hours of field test(s).

Include test date, time, location and person performing tests.

AMBIENT CONDITIONS

Do not place and compact soil under following conditions:

Rain that creates puddles in clayey or silty materials.

Heat or wind or both that dries material below specified moisture conditions.

Ice, frost, or snow pockets are visible in soil being placed or in underlying materials.

Ambient air temperature below freezing may make placement and compaction operations difficult.

Minimize interruptions of operations.

Continuous operations may be required to eliminate freezing of materials.

Obtain COR approval, before performing operations when ambient air temperatures are below freezing.

Cease operations when placed materials are freezing or below freezing.

PRODUCTS

CLASSIFICATION

When required, classify earth materials using the Unified Soil Classification System (USCS) according to ASTM D2487 or ASTM D2488.

Gradation tests for classification: ASTM D9913 or D1140.

Atterberg limits testing for classification: ASTM D4318.

SOIL TYPES

Clean Fill:

Any soil except soils with classifications Peat (PT), Organic Silts and Organic Clays (OL and OH), or Elastic Silt (MH) per ASTM D2487.

Do not use soils that exhibit expansive or dispersive characteristics in accordance with ASTM D4546, ASTM D4647, or ASTM D6572.

Free of roots, stumps, limbs, vegetation, organic matter, and ice.

Does not contain construction debris, scrap materials, refuse, man-made wastes, or chemical or hydro-carbon contamination.

Do not use frozen soil.

Special Gradations and Plasticity.

In some cases, such as embankment materials, embedment for buried pipe, special gradations and/or plasticity characteristics may be required.  These requirements are in the applicable Section.

DESIGNATION OF SOILS FOR COMPACTION

Requirements for lift thickness, method of compaction, and method of determining degree of compaction depends on whether soil is considered to be silty or clayey, cohesionless, or cohesionless containing some silt and clay.

Silty or Clayey Soils:

Contains appreciable amounts of fines (more than 15 percent plastic fines or 30 percent non-plastic fines).

Cohesionless Soils:

Contains few fines (less than 5 percent fines).

Cohesionless Soils Containing Some Clay and Silt:

Contains some clay and silt (between 5 percent fines and 15 percent plastic fines or 30 percent non-plastic fines).

MAXIMUM PARTICLE SIZE

Soil for compacted backfill and embankment:

Maximum particle size limitations described in appropriate Sections for backfill against specific structures and zoned embankment materials.

Otherwise, no cobbles larger than 5 inches or boulders.

EXECUTION

SURFACE PREPARATION

Clear, grub, and strip.

Prepare surface so that first compacted lift will be placed on firm, stable base.  Compact surface to specified compaction, if necessary.

For water-retaining compacted fill, scarify and moisten surface to provide satisfactory bonding surface before placing layer of material to be compacted.

SOIL MOISTURE CONTENT

Moisten or aerate material, as necessary, to provide moisture content that will readily facilitate obtaining specified compaction.  Add water to soil only in increments that will permit moisture content to be uniform and homogenous throughout each layer after mixing.

Silty and Clayey Soils and Cohesionless Soils Containing Some Silt and Clay:

Add no more than 2 percent water to fill by sprinkling just prior to compaction when fill is clayey and contains dry clods of clay.

If clayey borrow soil is more than 2 percent below optimum moisture, pre-conditioning and curing may be required to obtain uniform and homogenous distribution of moisture in the clods.

Use of disks, harrows, or rakes may be required to blend moisture in the borrow area.

Moisture content during compaction:

Soils controlled by impact compaction

Not greater than 2 percentage points wet or not less than 4 percentage points dry of optimum moisture content.

Soils controlled by the vibrating hammer.

Within the water content range for effective compaction as determined by ASTM D7382.

Cohesionless Soils:

Add water during compaction, as necessary, since these soils are free draining.

Determine moisture with one of the following:

ASTM D1556, D4914, D5030, D5080, D2216, or D6938.

Correct moisture from the nuclear gauge for gauge error for the specific soils tested and the moisture content of the total material may require adjustment for the control fraction (ASTM D4718).

PLACEMENT

Place soils to be compacted in horizontal layers.

If necessary, blend materials so that compacted fill is homogenous and free from lenses, pockets, streaks, voids, laminations, or other imperfections.

COMPACTION

Compact material with following methods and techniques appropriate to type of soil.

Silty or clayey material:

Compact with mechanical impact tampers, tamping rollers, vibrating pad foot rollers, rubber tire rollers, other suitable compaction equipment, or equipment travel.

Uniformly distribute equipment passes.

Compact in horizontal layers to compacted thickness of 6 inches or less.

Special compaction: Compact with handheld impact tampers, or small tamping equipment.

Uniformly distribute effort.

Compact in horizontal layers to compacted thickness of 4 inches.

Density:

Percent Compaction, minimum 98%, or

D-value:

Minimum 95 percent with moisture content +/-2 percent of optimum content.

Minimum 100 percent with moisture content 2 to 4 percent dry of optimum.

Cohesionless free-draining material:

Compact with crawler-type tractors, vibrating drum rollers, surface plate vibrator, or similar equipment

Uniformly distribute equipment passes.

Compact horizontal layers to compact thickness of 12 inches or less.

Special compaction: Compact with handheld impact tampers, vibrating plate tampers, or small tamping equipment.

Uniformly distribute effort.

Compact horizontal layers to compact thickness of 6 inches.

Density:

Percent Compaction, minimum: 98 percent.

Cohesionless Soils Containing Some Silt and Clay:

Compact in accordance with either procedure above.

Density:

Percent Compaction, minimum:98 percent, or

D-value:

Minimum 100 percent with moisture content +/- 4 percent.

Demonstration:

Lift thicknesses may vary depending on equipment and methods.  Before changing requirements in this Section, demonstrate that required density will be obtained.

MEASURE OF COMPACTION

Determine unit weight of soils in-place using one of the following methods:

Silty or Clayey and Cohesionless Soils:

ASTM D1556, D4914, D5030, or D6938.

Determine soil compaction by one of the following.

Silty or clayey soils:

Percent Compaction:

Rapid Method: ASTM D5080.

Laboratory Compaction Test: Comparison of in-place dry unit weight of minus no. 4 sieve size control fraction to laboratory maximum dry unit weight as determined by ASTM D698, Procedure A.

Cohesionless soils:

Percent Compaction: In-place dry unit weight compared to the maximum dry unit weight determined by ASTM D7382.

Cohesionless Soils Containing Some Silt and Clay:

Use whichever testing procedure results requires higher in-place dry density.

Adjustment:

For soils with 5 to 30 percent oversized particles: In-place unit weight of minus no. 4 size control fraction determined by screening gravel, washing, and determining mass and volume by assuming surface saturated dried moisture as outlined in ASTM D4718.

For soils containing greater than 30 percent oversized particles:   Required D ratio or Percent Compaction may be adjusted in accordance with appropriate curve on Figure 3 in USBR Guidelines for Earthwork Construction Control Testing of Gravelly Soils.

CONTRACTOR FIELD QUALITY TESTING

Testing:

Independent testing laboratory shall perform sampling, testing, and reporting as required in Table 31 23 02A - Contractor Materials Testing Requirements and Frequency.

Independent testing laboratory shall meet requirements specified in Section 01 46 20 - Testing Agency Services.

Notify the independent testing laboratory and Government 24 hours before compaction work begins and 24 hours before significant change in compaction operations (major change in equipment or procedure used).

Notify the independent testing laboratory and Government immediately of equipment change due to breakdown, or re-deployment.

Testing Frequency:

At a minimum, perform test at frequencies specified in Table 31 23 02A - Contractor Materials Testing Requirements and Frequency.

Greater frequency of testing is normally performed at beginning of new work, new work crew, or new equipment.

After a successful work operation pattern is established, testing frequency may be performed at the minimum guidelines.

Perform additional tests at sites considered questionable by the Government, such as suspected incomplete compaction, surfaces that may have become excessively wet or dry since compaction, compacted surfaces torn up by subsequent equipment travel, or other similar circumstances.  Frequency of additional testing is at discretion of the Government.

| Table 31 23 02A – Contractor Materials Testing and Frequency |  |  |  |
| --- | --- | --- | --- |
| PROCEDURE | TEST STANDARD | STANDARD TITLE | MINIMUM FREQUENCY OF TESTING |
| Soil Classification | ASTM D2487 | Classification of Soils for Engineering Purposes | As necessary to classify material to meet specification requirements or to index material for determining density by nuclear method. |
|  | ASTM D2488 | Description and Identification of Soils |  |
| Moisture Content | ASTM D2216 | Laboratory Determination of Water (Moisture Content of Soil and Rock by Mass. | With in-place density or as required to index material for determining density by nuclear method. |
| Laboratory Compaction (Standard Effort) | ASTM D698, Procedure A. | Laboratory Compaction Characteristics of Soil Using Standard Effort | One test every 500 yd3 Minimum 1 test per day per compacted backfill operation, regardless of amount of material placed. |
| Laboratory Compaction (Vibratory Hammer) | ASTM D7382 | Standard Test Methods for Determination of Maximum Dry Unit Weight and Water Content Range for Effective Compaction of Granular Soils Using a Vibrating Hammer | One test every 500 yd3 Minimum 1 test per day per compacted backfill operation, regardless of amount of material placed. |
| Oversized Correction | ASTM D4718 | Correction of Unit Weight and Water Content for Soils Containing Oversize Particles | When oversize material is between 5 to 30 percent |
| Rock Factor |  | USBR Guidelines for Earthwork Construction Control Testing of Gravelly Soils | When oversize material is greater than 30 percent |
| Nuclear Method | ASTM D6938 | In-Place Density and Water Content of Soil and Soil-Aggregate by Nuclear Methods (Shallow Depth) | One test every 100 yd3 Minimum 1 test per day per compacted backfill operation, regardless of amount of material placed. Minimum 1 test per foundation prior to concrete placement. |

Contractor Support:

Provide timely access to areas for density testing and excavate and level an area in compacted material to provide a surface for testing.

Test fills compacted by sheepsfoot rollers one or two lifts below surface.

Government may select location of testing.

When density is being measured by a sand-cone device (ASTM D1556), cease construction activity in immediate vicinity of testing.

Dig test pits as requested to examine compacted soil against structures or pipe.

Backfill test pits to original requirements.

Provide warning lights, flags, or other safety devices as needed by testing personnel.

Provide adequate lighting for performing test if required because of darkness.

GOVERNMENT CONTRACT FIELD QUALITY ASSURANCE

The Government may perform tests as required to verify contractor quality testing.  If the Government performs quality assurance testing, provide support as required for contactor quality testing.

END OF SECTION

SECTION 31 23 11

EARTHWORK FOR STRUCTURES

GENERAL

MEASUREMENT AND PAYMENT

Common Excavation for Structures:

Measurement: Made to excavation paylines shown on the drawings or to paylines directed by the COR.

Regardless of quantities removed, measurements for payment will be made to the prescribed paylines.

For safety or other reasons, the Government reserves the right to require the Contractor to excavate beyond the prescribed paylines.

Measurement for payment for additional excavation will be made to limits directed by COR.

Cost: Include in price offered in the Price Schedule for furnishing and installation of kiosk.

Includes cost of labor and materials necessary to maintain excavations in good order during construction; removing such temporary construction, where required, and disposing the excavated material.

Overexcavation performed beyond specified or directed paylines and backfill and compaction of backfill for such overexcavation shall be at the expense of the Contractor.

The cost of furnishing and applying water to surfaces of shale shall be included in the prices offered in the Price Schedule for other items of work.

Backfill About Structures:

Measurement: Made to structure lines and excavation paylines shown on the drawings.

Costs: Include costs of earthwork in applicable prices offered in the Price Schedule for items requiring earthwork.

Includes haul of necessary material.  Material from required excavation used for backfilling will be paid for both as excavation when removed from original position and as backfill when placed.

Where additional material is obtained from approved borrow pits and used for backfilling about structures, payment will be made for backfill only.

Refill of excavation performed outside the established paylines for excavation for structures shall be at the expense of the Contractor.

Compacting Backfill about Structures:

Measurement: Made to structure lines, excavation lines, and compacted backfill lines shown on the drawings.

Costs: Include costs for compacting earthwork in applicable prices offered in the Price Schedule for items requiring compacted earthwork.

Includes cost of furnishing water and moistening materials.

Compacting refill outside of excavation paylines shall be at the expense of the Contractor.

DEFINITIONS

Additional Excavation: Excavation beyond specified lines as directed by the COR to remove unsuitable foundation material.

Overexcavation: Excavation performed for the convenience, fault, or operation of the Contractor beyond specified or directed additional excavation lines.

Cover: Distance between the top surface of buried cables, conductors, or conduits and the finished grade.

PRODUCTS

EXCAVATED MATERIALS

The Contractor's operations in excavations shall be such that excavations will yield as much suitable material for use in permanent construction required under these specifications as practicable.

Place excavated materials which are too wet for immediate compaction temporarily in stockpiles until moisture content is reduced sufficiently to permit them to be placed in embankments.

MATERIAL FOR BACKFILL

Type and amount of material used for backfilling, and manner of depositing material shall be subject to approval.

Obtain backfill material from required excavations.  If sufficient suitable material is not available from this source, obtain additional material from approved borrow pits or sources.

The Government makes no guarantee that specified backfill materials are contained in or can be processed from materials excavated.

Do not place material in backfill when either the material or the surfaces on which it is to be placed are frozen.

Do not use material removed in stripping for backfill material.

Do not use expansive clays and shales for backfill material.

Except as otherwise provided below, backfill material to be compacted shall contain no stones larger than 3 inches in diameter.

MATERIAL FOR COMPACTING BACKFILL

Selected material containing no stones larger than 3 inches in diameter, approved by the COR, and obtained from required excavation or approved borrow pits.

EXECUTION

EXCAVATION, GENERAL

Excavate structure foundations to elevations shown on the drawings or established by the COR.

The Government reserves the right, during progress of work, to vary slopes, grades, and dimensions of excavations from those specified.

The Government does not represent that excavation performed under these specifications can be made to or maintained at paylines shown on the drawings or described in these specifications.

Perform excavation in the dry.

Do not excavate in frozen materials without written approval.

Where excavating in backfill and embankment placed under these specifications, excavate in accordance with applicable provisions for excavation.

Blasting: Not allowed.

Preserve material below and beyond established lines of excavation in the soundest possible condition.

Damage to work due to the Contractor's operations shall be repaired by and at the expense of the Contractor.

Material beyond required or prescribed excavation lines which are loosened by the Contractor's operations shall be removed at the expense of the Contractor.

PREPARATION OF STRUCTURE FOUNDATIONS

Prepare foundations at structure sites by methods which will provide firm foundations for structures.

Finish bottom and side slopes of excavation, upon or against which the structure is to be placed, to prescribed dimensions.

Moisten and compact prepared surfaces with suitable tools to form firm foundations upon or against which to place the structure.

Where unsuitable material is encountered in the foundation for a structure, the COR will direct performance of additional excavation to remove unsuitable material.

Refill additional excavation with compacted backfill in accordance with Section 31 23 02 - Compacting Earth Materials.

DISPOSAL OF EXCAVATED MATERIALS

Dispose of excavated materials which are unsuitable for or are in excess of backfill or other earthwork requirements, as determined by the COR.

PLACING BACKFILL

Place, backfill to the lines and grades shown on the drawings, or as directed by the COR.

Place, backfill carefully and spread in uniform layers so that all spaces about rocks and clods will be filled.

COMPACTING BACKFILL

Place, moisten, and compact materials in backfill as provided in Section 31 23 02 - Compacting Earth Materials.

PROTECTION

To provide adequate protection for compacted backfill about a structure, the Government reserves the right to direct the Contractor to place a sufficient amount of backfill or embankment material over compacted backfill within 72 hours after completion of compacting backfill.

END OF SECTION

SECTION 32 92 20

SEEDING AND SOIL SUPPLEMENTS

GENERAL

MEASUREMENT AND PAYMENT

Seeding and Soil Supplements:

Measurement: Area surfaces where disturbance has occurred during construction activities that will not be disturbed in the future from normal use and as directed by the COR.

Payment: Square Yard price offered in the Price Schedule.

DEFINITIONS

Pure live seed content: Weight of seed times percent purity times percent germination.

SUBMITTALS

Submit the following in accordance with Section 01 33 00 - Submittals.

RSN 32 92 20-1, Seeding Plan:

Method and equipment used for seeding.

Date for seeding.

Mulch and fertilizer type and application equipment.

Qualifying experience for person responsible for supervision of seeding, for approval.

Names, addresses, and telephone numbers of references.

Name and address of seed suppliers.

RSN 32 92 20-2, Certifications:

Origin of seed.

Percent purity and germination.

Prohibited and restricted weed seed content.

DELIVERY STORAGE AND HANDLING

Seed containers:

Sealed.

Labeled:

Identify seed origin on label.

Intrastate shipping: In accordance with State Seed Laws and Regulations.

Interstate shipping:  In accordance with U.S. Department of Agriculture Rules and Regulations under the Federal Seed Act.

AMBIENT CONDITIONS

Do not seed or fertilize when ambient temperature is below 38 degrees F without approval of the COR.

Do not seed or fertilize when ground is covered with snow.

Do not seed, fertilize, or mulch, or hydroseed when wind velocities prevent uniform application of materials or would drift materials.

PRODUCTS

SEED

Weed seeds classified by State Seed Department:

Prohibited noxious weeds: None

Restricted noxious weeds: 0.5 percent maximum, by weight.

Seed mixture:

Purity, minimum: 85 percent.

Germination, minimum: 85 percent.

Germination test: Less than 1 year old at time of seeding.

Uniform mixture shown in Table 32 92 20A - Seed Mixture.

| Table 32 92 20A - Seed Mixture |  |  |
| --- | --- | --- |
| Common Name | Scientific Name | Seeding Rate (Pounds pure live seed per acre) |
| Sideoats Gramma | Bouteloua Curtipendula Var. Vaughn | 2.7 Lbs. |
| Blue Grama | Bouteloua Gracilis Var. Lovington | 1.1 Lbs. |
| Mountain Brome | Bromus Marginatus Var. Whitmar | 5.0 Lbs. |
| Canada Wildrye | Elymus Canadensis Var. Native | 3.0 Lbs. |
| Slender Wheatgrass | Elymus Trachyculus Var. An Louis | 2.2 Lbs. |
| Western Wheatgrass | Pascropyrum Smithii Var. Arriba | 4.8 Lbs. |
| Beardless Wheatgrass | Pseudoroegneria Spicata Spp. Inermis, Var. Bromar | 4.5 Lbs. |

FERTILIZER

Agricultural grade nitrogen fertilizer and phosphate fertilizer.

Nitrogen fertilizer: Urea (46-0-0).

Phosphate fertilizer: as recommended by seed supplier.

STRAW MULCH

Wheat or barley straw.

Free of mold or other evidence of decomposition.

Free from weed seed.

HYDROMULCH

      - Mat-Fiber Plus, manufactured by Mat Inc.,12402 Hwy 2, Floodwood, MN 55736, 1 -888-477-3028; Spray Mulch X-80 manufactured by Pacific Wood Fibers, PO Box 2109, Redmond WA 98052; or equal, having the following essential characteristics:

Wood cellulose fiber.

No germination or growth inhibiting factors.

Dyed appropriate color to allow visual metering of application.

Evenly dispersed and suspended when agitated in water.

Forms blotter like ground cover that readily absorbs water and allow infiltration to underlying soil.

TACKIFIER

Mixture of at least three specially blended compatible hydrocolloids.

One hydrocolloid will act as a slippery agent during suspension.

Will form loose, long-chain-like film on drying

No growth or germination inhibiting factors.

Hydrates and disperses in circulating water to form homogeneous slurry.

Equilibrium air dry moisture content at time of manufacture of 8 percent, plus or minus 2 percent.

Minimum water holding capacity: 6-1/2 times weight of dry material.

EXECUTION

SEEDBED PREPARATION

Complete prior to seeding, and mulching or hydro mulching.

Scarify or harrow and rake topsoil to minimum depth of three inches.

Remove stiff clods, lumps, roots, litter, stones, and other foreign materials greater than 6 inches in size from the surface. Dispose of removed materials in accordance with Section 01 74 10 – Cleaning.

Fill or smooth top-soil surface to remove rills, gullies and depressions.

Protect prepared top-soil surfaces from erosion and washouts.  Repair damaged surfaces as required.

FERTILIZING

Apply nitrogen fertilizer uniformly at a rate of 30 pounds of nitrogen content per acre.

Apply phosphate fertilizer uniformly at a rate of 50 pounds of P2O5 content per acre.

When fertilizer is applied before seeding, disc or harrow fertilizer into top three inches of soil.

SEEDING

Apply seed and mulch by one of the following methods:

Broadcast seeding followed by mulching or hydro mulching.

Apply seed mixture at rate specified in Table 32 92 20A - Seed Mixture.

Seed only between July 1 and September 1 of each year.

BROADCAST SEEDING

Broadcast seed only in areas not accessible for drilling or hydroseeding.

Apply seed and fertilizer separately.

Mechanical broadcasting:

Equipment:

Centrifugal type.

Pull type similar to fertilizer spreader.

Designed and regulated to apply seed uniformly at proper rate per acre.

Hand Broadcasting:

By hand broadcaster.

By hand.

Uniformly applied.

Cover seed with soil to depth of 1/4-inch to 1/2-inch immediately after broadcasting.

Use hand rake or float.

Do not use log chain or similar device.

MULCHING

Spread within 2 days of spreading seed.

Rate: 2 tons per acre uniformly spread

Anchor with threader.

Operate crosswise to slope.

Depth: 3 to 4 inches.

Interval: 6 to 12 inches across slope.

END OF SECTION

This page intentionally left blank.

- SECTION 51 00 00

- INFORMATION AVAILABLE TO OFFERORS

GENERAL

ORDER OF PRECEDENCE

In case of differences between sections in Division 51 - Information Available to Offerors and sections in other divisions of the specifications, the requirements in the other divisions will govern.

The Government assumes no responsibility for deductions, interpretations, or conclusions made by the Contractor based on information made available by the Government, in accordance with the clause at FAR 52.236-3 “Site Investigations and Conditions Affecting the Work.”

PRODUCTS

Not Used

EXECUTION

Not Used

END OF SECTION

This page intentionally left blank.

SECTION 52 00 00

DRAWINGS

GENERAL

DISCREPANCIES, ERRORS, OR OMISSIONS

Inform the CO of discrepancies discovered on drawings in accordance with clause at FAR 52.236-21, Specifications and Drawings for Construction.  Promptly inform the CO of discrepancies discovered on the drawings.  The CO will make a determination in writing.  Any adjustment by the Contractor without such a determination shall be at its own risk and expense.

In accordance with clause at FAR 52.236-21, Specifications and Drawings for Construction, in case of discrepancies, written specifications take precedence over drawings unless otherwise specified.

Anything mentioned in the written specifications and not shown on the drawings or shown on the drawings and not mentioned in the written specifications, shall be of like effect as if shown or mentioned in both.

In case of differences between drawings and written specifications, the written specifications shall govern.

PROJECT CONDITIONS

Where there are differences as determined by the CO between details and dimensions shown on drawings and details and dimensions of existing features at jobsite, use details and dimensions of existing features at jobsite.

SPECIFICATION DRAWINGS

Some drawings show details of fabrication or other details and specifications which are not a part of work under this contract.  Disregard specifications and details shown on these drawings which are not applicable to work under this contract.

Reference drawings referred to on specification drawings and not considered necessary for contract purposes are not included in specifications.

EXISTING INSTALLATION DRAWINGS

Drawings marked with "Existing Installation" show existing condition of installations to be modified under this contract.  The revised versions of these drawings show the required modifications to be performed under this contract.

ADDITIONAL OR REVISED DRAWINGS

Except as provided in these specifications for drawings to be furnished by Contractor, specifications drawings will be supplemented by additional or revised general and detail drawings as necessary or desirable as work progresses.

Do not perform work without proper drawings and instructions.

Additional or revised general and detail drawings will show dimensions and details necessary for construction purposes more completely than are shown on these specifications’ drawings for features of work and for equipment or metalwork not yet purchased.

Perform work in accordance with additional general and detailed drawings or revisions at applicable prices offered in Price Schedule for such work.

INFORMATION DRAWINGS

Drawings marked "For Information Only" in the drawing list are included to show existing features about which knowledge is required to perform work under this contract.  These drawings do not show work to be performed under this contract.

If there are differences as determined by the CO between details and dimensions shown on these drawings and those of existing features at jobsite, use details and dimensions of existing features at jobsite.

STANDARD DRAWINGS

Standard drawings may show details which are not a part of work under this contract.  Disregard details shown on these drawings which are not applicable to work under this contract.

COPIES OF DRAWINGS

One set of full-size (ANSI D) 22 inches by 34 inches drawings will be furnished to the Contractor upon request.

One set of half-size (ANSI B) 11 inches by 17 inches drawings will be furnished to the Contractor, if requested.

LIST OF DRAWINGS

Drawings listed in Table 52 00 00A - List of Drawings are made a part of Section C - Description/Specifications.

PRODUCTS

Not Used

EXECUTION

Not Used

U.S. Department of the Interior

Bureau of Reclamation

San Juan-Chama Project, New Mexico - Colorado

Rio Chama Division

Heron Reservoir Recreation Area

Rio Chama Trail

| Table 52 00 00A - List of Drawings |  |  |
| --- | --- | --- |
| Sheet No. | Drawing No. | Title |
| Structural: |  |  |
| 1. | 465-512-60016 | New Staircase Installation Lower Platform and Connection Details |
| 2. | 465-512-60017 | New Staircase Installation Elevations, Plans, and Details |
| 3. | 465-512-60018 | New Staircase Installation Hand Railing and Baluster Panels Elevations |
| 4. | 465-512-60019 | New Staircase Installation Elevations, Plans, and Details |
| 5. | 465-512-60020 | New Staircase Installation Lower Platform and Connection Details |
| 5. | 465-512-60122 | New Staircase Installation Elevations, Plans, and Details |
| 7. | 465-512-60123 | New Staircase Installation Elevations, Plans, and Details |
| 8. | 465-512-60124 | New Staircase and Kisok Installation Location Map, Site Plan, and Sections |
| Information Drawings: |  |  |
| 9. | Sheet No. 1 | Chama Footbridge |
| 10. | Sheet No. 2 | Chama Footbridge Site Layout - Plan and Profile |
| 11. | Sheet No. 3 | Chama Footbridge Details for Cables |
| 12. | Sheet No. 4 | Chama Footbridge Details for Single - Bay Layout |
| 13. | Sheet No. 5 | Chama Footbridge Details for Abutments |
| Standard Drawings: |  |  |
| 14. | 40-D-7102 | Standard and Typical Designs Government Drawing Format Drawing Border, Sample Title Block and Signature Lines |
| 15. | 40-D-60003 | Standard and Typical Designs Concrete Outline and Reinforcement General Notes for Concrete Outline and Reinforcement |
| 16. | 40-D-60004 | Standard and Typical Designs Concrete Outline and Reinforcement Minimum Requirements for Detailing Reinforcement |

END OF SECTION