B523 Rooms 201&202 JWICS/SIPR VTC Upgrade (ITR 10956)

Statement of Objectives (SOO)

DATE: 27 March 2026

# DESCRIPTION:

- System Description:  The 17th Training Support Squadron (17 TRSS) requires a technical refresh of the Building 523 Rooms 201 & 202 JWICS/SIPRnet Video Teleconference (VTC) suites, as they have reached End of Life (EOL). The performance of this contract will be inside a Sensitive Compartmented Information Facility (SCIF) and the company(s) authorized to perform this work must have a Top Secret Facility Clearance Letter (FCL).

  - Scope of Work:  The contractor shall provide all equipment listed in attachment 1, and perform all tasks necessary to accomplish the installation and configuration of new VTC suite equipment to include displays, signage, switching equipment, codecs, audio equipment etc, listed in attachment 1 in building 523, rooms 201 & 202. It is vital to ensure that both end user VTC operator experiences  and VTC maintenance technician support operations remain consistent across all VTCs in use across the training environment. Therefore, we are requesting that all Graphical User Interfaces (GUIs) visually appear and operate similar and programming be consistent with previously refreshed suites, eliminating additional costs associated with retraining local personnel who have already been trained on equipment operation.  These tasks are outlined in section 9, General Tasks and section 11, Specific Tasks below.  As an overview, the contractor must:

    - Utilize this Statement of Objectives (SOO) to create and deliver a Statement of Work, SOW) detailing how the contractor will meet all objectives within.  The SOW will be utilized by the Government to determine acceptability of the proposal.

    - Perform an inventory of equipment

    - Stage and prepare equipment for installation in stated datacenter locations.

    - Perform requirements and systems analysis

    - Develop implementation plan and schedule

    - Install and configure all VTC equipment listed in Attachment A in building 523 rooms 201 & 202.

    - Remove and dispose of all equipment packing materials.

    - Perform test calls to ensure full VTC functionality.

  - Location: The equipment will be located in building 523, rooms 201 & 202.

  - Purpose:  This project is to replace EOL equipment with new, supported hardware on the JWICS and SIPRnet networks.

- PERSONNEL: Contractor personnel shall present a neat appearance and be easily recognized.  This shall be accomplished by wearing distinctive clothing bearing the name of company or by wearing appropriate badges that contain the company name and employee name. Employees must have SCI-DCID 6/4 eligibility and be indoctrinated for SI/TK access. In accordance with Department of Defense Manual (DoDM) 5105.21, volume 3, employee eligibility must be verified using the Joint Personnel Adjudication System (JPAS) (or successor system) (reference Enclosure 2, section 3, paragraph f of DoDM 5105.21, volume 3).  Scattered Castles (SC) may also be used for employees deemed eligible by the non-DoD Intelligence Community (NSA, CIA, NGA, et al).  The employee may be indoctrinated locally if needed as long as the employee has the appropriate eligibility. The company must possess a current Facility Clearance Letter (FCL). In accordance with DoDM 5105.21, volume 3, FCL requirements for access to SCI. Contractor personnel may not be permitted access to SCI based on an interim TOP SECRET FCL.  The DoD Component or agency must have a valid contractual requirement to sponsor the contractor for a TOP SECRET FCL for access to SCI.  The contractor company must possess a final TOP SECRET FCL.  The CSSO must meet SCI eligibility requirements and be indoctrinated for SCI.  Contractor employees, to perform work on SCI contracts, must meet SCI eligibility requirements and be indoctrinated for SCI.  The contract representative must appoint a local staff member as the local representative as liaison between the company and the Special Security Office.  Contractor shall ensure personnel are compliant with REAL ID Act of 2005 for the purpose of gaining access to Goodfellow Air Force Base.

  - Suitability Investigations.  Contractor personnel shall successfully complete, as a minimum, a National Agency Check with Written Inquiries (NACI), before operating government furnished workstations, being given government information, or being given access to multiple government buildings.   These investigations shall be submitted by the government at no additional cost to the contractor.  The contractor shall comply with the DoD 5200.2-R, Personnel Security Program.

  - Listing of Employees.  The contractor shall maintain a current listing of employees.  The list shall include employee's name, social security number, and date of investigation if contract work involves unescorted entry to Air Force restricted or other sensitive areas designated by the installation commander.  The list shall be validated and signed by the company Facility Security Officer (FSO) and provided to the contracting officer and ISPM prior to the contract start date. Updated listings shall be provided when an employee's status or information changes.

  - Freedom of Information Act Program (FOIA).  The contractor shall comply with DoD Regulation 5400.7/Air Force Supplement, DoD Freedom of Information Act Program, requirements.

  - Additional Security Requirements.  In accordance with DoD 5200.01 and DAFI 16-1404, the contractor shall comply with AFMAN 33-282, Computer Security (COMPUSEC); AFSSI 7700, Emission Security (EMSEC); applicable AFKAGs, DAFIs, and AFSSIs for Communication Security (COMSEC); and DAFI 10-701, Operations Security (OPSEC).

  - Controlled Unclassified Information (CUI) Requirements Controlled Unclassified Information (CUI) Requirements. The contractor shall comply with DoDI 5200.48_DAFI 16-1403 regulation which sets policy and procedures for the marking, handling, transmitting, safeguarding, and destruction of Controlled Unclassified Information (CUI). The contractor is required to protect controlled unclassified information and/or indicators that may reveal classified information, sensitive or critical information or information that can be collected in aggregate to identify vulnerabilities. Specifically, the contractor is prohibited from disclosing, without express permission from the United States Air Force, any floor plans, schematics, or any other documents containing information or information gleaned from said documents on any government facilities or its contents regardless of whether it is part of the infrastructure or otherwise.

  - OPSEC Requirements. The contractor is required to protect controlled unclassified information and/or indicators that may reveal classified information, sensitive or critical information or information that can be collected in aggregate to identify vulnerabilities. Specifically, the contractor is prohibited from disclosing, without express permission from the United States Air Force, any structured process that identifies critical information, analyzes friendly actions, integrates threat analysis and risk assessments that helps personnel apply protective measures to mitigate unacceptable risk. The basis for the OPSEC program is AFI 10-701.

# QUALITY CONTROL:  The contractor shall insure that all work performed under this (SOO) meets the quality established in section 9, General Tasks, and section 11, Specific Tasks.

# PHYSICAL SECURITY:

  - Vehicle Passes:  The Contractor shall obtain vehicle passes for each vehicle used on Goodfellow AFB (GAFB) during the performance period.  The necessary applications for vehicle passes can be obtained by contacting the Government Project Manager (GPM).  Before starting the work, the Contractor shall provide a list of all personnel that will work on GAFB, (include Social Security number and driver’s license number and state), to permit security screening.  Any workers not cleared will be identified, and not permitted to work on GAFB.

  - Government Property:  The Contractor shall be responsible for safeguarding Government property associated with the required work. Facilities, equipment, and materials shall be secured at the close of each work period.

# HOURS OF OPERATION: The contractor personnel must be available to perform installation tasks during normal duty hours.   Normal duty hours are defined as 0730 to 1630 Central Standard Time, Monday through Friday, except Federal Holidays. Requests for changes or exceptions to this work schedule may be submitted to the GPM as required.  These requests will be evaluated and approved or declined by the GPM, based on impact to the cost, scope, and schedule of the project.

# CONSERVATION OF UTILITIES: The Contractor shall instruct employees in utilities conservation practices and shall be responsible for operating under conditions, which preclude the waste of Government utilities.

# GOVERNMENT FURNISHED UTILITIES AND SERVICES:  The Government shall provide without costs to the Contractor, the utilities necessary (limited to water & electricity) to accomplish the tasks specified in this work description.

# CONTRACTOR FURNISHED EQUIPMENT, PARTS, MATERIALS, and SERVICES:  Except for the utilities, services and equipment specifically stated to be Government furnished, the contractor shall provide all personnel, equipment, tools, transportation, and supervision required.

# GENERAL TASKS:

  - The Contractor shall confine operations at the site to areas permitted by law, ordinances, permits, and contract documents and shall not encumber the site with any materials or equipment without prior written authorization for exact location of each.  Any deviation from or changes to the terms outlined in the contract (including this SOO) are subject to the approval of the CO or their designee (i.e., Contracting Officer Representative (COR) or GPM).

  - The Contractor shall plan operations so as to keep temporary work from blocking access to completed work.  If, however, conflict with normal traffic access occurs, the contractor shall provide temporary bypass routing until such temporary work is completed.

  - During the performance of the work within the requirements of this contract and until final completion and acceptance thereof, the Contractor shall exercise the utmost care to avoid accident or injury to persons or property.  The contractor shall place and maintain all necessary barriers and safeguards, including watchmen, if necessary, about the work site for the prevention of accidents and shall take all precautions required to protect personnel and property to include the general public and properties adjacent to the work site.

  - The Contractor shall provide a full-time quality control function to assure himself and the Government that all materials and workmanship are in strict accordance with industry standards and this SOO.  Installation and materials shall conform to the requirements of applicable codes.  The Contractor shall maintain adequate quality control capability regardless of the volume of workload.  All work performed by the Contractor shall be inspected and approved by the Government before final acceptance by the Government.

  - Upon completion of all work the Contractor shall request a final inspection.  The Government shall inspect all work before acceptance.  All work identified during the final inspection not meeting the criteria specified by this statement of work shall be repaired by the Contractor at no additional cost to the Government.

# SPECIFICATIONS:  It is the contractor’s full responsibility to inventory equipment to ensure the correct material and amount required to perform the stated requirements under the terms of this statement of work and technical exhibits, PRIOR TO INSTALLATION BEGINNING.  This product will be installed with the manufacturers’ recommended methods and must be installed by a professional who is knowledgeable of this specific product to prevent exclusion of manufacturer’s warranty.

# SPECIFIC TASKS:  The contractor shall provide all equipment, hardware, software, material, labor, documentation, tools, test plans, transportation, lodging, and staff needed to complete this project. Any existing equipment or components removed during contractor execution shall remain the property of the government.  The contractor shall accomplish all tasks required to complete this system expansion, including overall Project Management, Installation, and Testing phases.

- Project Management

- Phase 1:  Installation

- Phase 2:  Testing

Project Management:  The contractor shall assign a dedicated Contractor Project Manager (CPM) to work with the Government Project Manager (GPM), will be identified when the contract is awarded, to provide oversight for on-site project planning and execution.

    - The CPM shall:

- (1)	Schedule and host at Goodfellow AFB, TX a project kick-off meeting with the GPM, System Administration personnel, and the contractor project team.

- (2)	Develop and deliver to the GPM a project management plan and detailed schedule using Microsoft Project.

- (3)	Schedule and host weekly onsite or via telephone conference project status meetings with the GPM and the contractor project team.

- (4)	Provide written status reports to the GPM prior to and during project execution on a weekly basis.

- (5)	Complete project “punch list” with GPM prior to final acceptance.

  - Project Management Documentation:  At the conclusion of the project execution, provide final revisions of design and review document.

Phase I - Installation:  The contractor shall travel to Goodfellow AFB, TX to complete installation of all equipment per location as follows:

#### Room 1 - Sidewinder - Room 201 For Sidewinder - Room 201 the following describes the system details as related to the associated MEL... ### Destination Configuration Contractor will provide a video system with the following destinations. Each device will be installed in the defined location with the listed method. Signal to each device is defined below... QTY; Destination; Location; Mounting. 2; 75" Display; Front Wall; Chief Wall Mount. 2; Dual Channel VTC Codec; Equipment Rack; Rack Mount. 1; WebVTC Codec; Equipment Rack; Rack Mount... ### Source Configuration Contractor will provide a video system with the following sources. Each source will be installed in the location defined with the method listed. Signal from each source is defined below... QTY; Source; Location; Mounting. 4; Dual Workstation Outputs; Equipment Rack; Rack Mount. 2; Dual Channel VTC Codec; Equipment Rack; Rack Mount. 1; Camera; Front Wall; Wall Mount... ### Video Routing Contractor will provide Crestron NVX AV over IP matrix switching platform to provide routing from the above listed inputs to the listed outputs. The matrix switch will be sized appropriately for the listed IO.... ### Room Control Contractor will provide a control system for the space consisting of a Crestron based system. This system will provide the user with a simple to use interface for control of the use of the room. The programming and end user GUI interface shall replicate what is currently installed in all other VTC suites recently upgraded at Goodfellow AFB, providing a nearly identical end user and maintenance technician experience. The user interface will consist of :.. QTY; Interface; Location. 10" Desktop Control Panel; Operators Desk.. The control system will give you the following capabilities at a minimum:.. - Turn on/off displays. Raise/lower screens (if applicable)... - Route appropriate sources to destination devices within the switching matrix... - Initiate and receive VTC calls - Share content from workstations to VTC Call.. - Manipulate camera PTZ and select presets.. - Ability to select audio source for the room. - Ability to control speaker volume in the room... - Ability to manipulate the digital signage in and around the room... Contractor Programmers will also modify the code for room 145 to include the two new codecs installed there.. ### Audio System Contractor will provide a(n) BIAMP Tesira that will give the ability to route the appropriate audio source to the correct destination. The audio system will be controlled by the Control System. The audio system will consist of the following destinations and sources:.. ### Audio Sources QTY; Input Type; Note. 1; Matrix Output. 4; Ceiling Microphone.. ### Audio Destinations QTY; Output Type; Note. 6; Ceiling Speakers. 2; VTC Codec. 1; webVTC Codec... ### VTC System Contractor will provide a video conferencing solution for the space. This system will consist of cameras mounted in the room and a number of codecs as defined below... ### Cameras QTY; Camera Type; Location. 1; PTZ; Between Displays.. ### VTC Codecs QTY; Codec Type; Classification. 1; Cisco Webex PRO; Top Secret. 1; Cisco Webex PRO; Classified. 1; Extron webVTC Interface; Based on Connected PC.. Audio to/from the codecs will be provided and routed by the audio Digital Signal Processor (DSP). All room microphones will be routed to the codecs to ensure the far end can properly hear the in-room participants... For all listed webVTC interfaces a USB extension will be provided to the appropriate host PC. This will allow the users the ability to use the in room camera and audio system with any soft codec application such as TEAMS, Zoom or WebEx... In a multi-enclave environment with multiple codecs, the system will be engineered to prevent cross contamination of signals. This will be accomplished by preventing sources of higher classification to be routed to codecs of lower classification. In addition, codecs of lower classification will be powered off or disconnected from source signaling when calls at higher classifications are happening.... ### KVM System Contractor will provide a KVM system based around the Crestron NVX that will give the operator the ability to control multiple remotely located PCs from a single user interface. The KVM system will provide extension of the USB control to the source PC and the video from the source PC to the local monitor(s)... ### KVM Sources QTY; Source; Note. 4; Dual Output PC.. ### KVM Destinations QTY; Destination; Note. 1; KM Control Only at Conference Table... ### Digital Signage Contractor will provide a room status signage system consisting of a number of displays located in and around the room. The size and location of the displays is listed below... QTY; Display Size; Location; Content. 1; 48" Panoramic; Above Displays; Room Status/Mic Status/Time Zone. 1; 27" Panoramic; Entry Door; Outside Room Status... ### Equipment Enclosure Contractor will provide the following AV equipment enclosures for the room. This will house all the needed equipment for the system. Power distribution and UPS units will be mounted in the equipment rack if applicably listed on the MEL. The Electrical Contractor is to provide the required power at the rack location... Qty; Furniture; Location; Notes. 1; 38 RU Equipment Rack; Adjacent Equipment Room....

#### Room 1 - Rattler Room 202 For Rattler Room 202 the following describes the system details as related to the associated MEL... ### Destination Configuration Contractor will provide a video system with the following destinations. Each device will be installed in the defined location with the listed method. Signal to each device is defined below... QTY; Destination; Location; Mounting. 2; 86" Display; Front Wall; Chief Wall Mount. 1; 75" Display; Rear Wall; Chief Wall Mount. 2; Dual Channel VTC Codec; Equipment Rack; Rack Mount. 1; WebVTC Codec; Equipment Rack; Rack Mount... ### Source Configuration Contractor will provide a video system with the following sources. Each source will be installed in the location defined with the method listed. Signal from each source is defined below... QTY; Source; Location; Mounting. 3; Dual Workstation Outputs; Equipment Rack; Rack Mount. 2; Dual Channel VTC Codec; Equipment Rack; Rack Mount. 1; Camera; Front Wall Between Displays; Wall Mount. 1; Camera; Rear Wall; Wall Mount... ### Video Routing Contractor will provide Crestron NVX AV over IP matrix switching platform to provide routing from the above listed inputs to the listed outputs. The matrix switch will be sized appropriately for the listed IO.... ### Room Control Contractor will provide a control system for the space consisting of a Crestron based system. This system will provide the user with a simple to use interface for control of the use of the room. The programming and end user GUI interface shall replicate what is currently installed in all other VTC suites recently upgraded at Goodfellow AFB, providing a nearly identical end user and maintenance technician experience. The user interface will consist of :.. QTY; Interface; Location. 1; 10" Desktop Control Panel; Operators Desk.. - The control system will give you the following capabilities at a minimum:.. - Turn on/off displays. Raise/lower screens (if applicable)... - Route appropriate sources to destination devices within the switching matrix... - Initiate and receive VTC calls - Share content from workstations to VTC Call.. - Manipulate camera PTZ and select presets.. - Ability to select audio source for the room. - Ability to control speaker volume in the room... - Ability to manipulate the digital signage in and around the room.... ### Audio System Contractor will provide a(n) BIAMP Tesira that will give the ability to route the appropriate audio source to the correct destination. The audio system will be controlled by the Control System. The audio system will consist of the following destinations and sources:.. ### Audio Sources QTY; Input Type; Note. 1; Matrix Output. 4; Ceiling Microphone.. ### Audio Destinations QTY; Output Type; Note. 10; Ceiling Speakers. 2; VTC Codec. 1; webVTC Codec... ### VTC System Contractor will provide a video conferencing solution for the space. This system will consist of cameras mounted in the room and a number of codecs as defined below... ### Cameras QTY; Camera Type; Location. 1; PTZ; Between Displays. 1; PTZ; Wall Mount Rear.. ### VTC Codecs QTY; Codec Type; Classification. 1; Cisco Webex PRO; Top Secret. 1; Cisco Webex PRO; Classified. 1; Extron webVTC Interface; Unclass.. Audio to/from the codecs will be provided and routed by the audio DSP. All room microphones will be routed to the codecs to ensure the far end can properly hear the in-room participants... For all listed webVTC interfaces a USB extension will be provided to the appropriate host PC. This will allow the users the ability to use the in room camera and audio system with any soft codec application such as TEAMS, Zoom or WebEx... In a multi-enclave environment with multiple codecs, the system will be engineered to prevent cross contamination of signals. This will be accomplished by preventing sources of higher classification to be routed to codecs of lower classification. In addition, codecs of lower classification will be powered off or disconnected from source signaling when calls at higher classifications are happening.... ### KVM System Contractor will provide a KVM system based around the Crestron NVX that will give the operator the ability to control multiple remotely located PCs from a single user interface. The KVM system will provide extension of the USB control to the source PC and the video from the source PC to the local monitor(s)... ### KVM Sources QTY; Source; Note. 3; Dual Output PC.. ### KVM Destinations QTY; Destination; Note. 1; Dual Output Desktop... ### Digital Signage Contractor will provide a room status signage system consiting of a number of displays located in and around the room. The size and location of the displays is listed below... QTY; Display Size; Location; Content. 1; 48" Panoramic; Above Displays; Room Status/Mic Status/Time Zone. 1; 27" Panoramic; Entry Door; Outside Room Status... ### Equipment Enclosure Contractor will provide the following AV equipment enclosures for the room. This will house all the needed equipment for the system. Power distribution and UPS units will be mounted in the equipment rack if applicably listed on the MEL. The Electrical Contractor is to provide the required power at the rack location... Qty; Furniture; Location; Notes. 1; 38 RU Equipment Rack; Adjacent Equipment Room....

Phase II – Testing

    - Contractor shall complete a minimum of 2 test calls per network classification, to ensure functionality of system(s). Contractor shall demonstrate all signage works properly, and system control functionality aligns with room and display layouts, to include camera, microphone and sound amplification functionality.

- End state of this project shall be the technical refresh of all JWICS/SIPRnet VTC equipment in building 523 rooms 201 & 202 with current, warrantied equipment.  All VTC capabilities shall be operational upon completion of project.

- PERIOD OF PERFORMANCE:

All aspects of this contract must be completed within 180 days from award of contract. (with exception to manufacturer-caused delivery delays out of the contractor's control). The contractor is responsible for analyzing the tasks outlined in these installation instructions and developing a plan to complete the work after the project kick-off meeting. All work defined in these installation instructions will be conducted during the hours of operation stated above and during normal working days. Exceptions to this constraint will be submitted to the Contracting Officer and GPM in writing no later than ten (10) business days prior to the event. All activities, including delivery of final system documentation must be completed within the Period of Performance.

ACRONYMS

AFB – Air Force Base

AFMAN – Air Force Manual

CIFS – Common Internet File System

CO – Contracting Officer

COR – Contract Office Representative

COMSEC – Communications Security

CPM - Contractor Project Manager

CUI - Controlled Unclassified Information

DAFI – Department of the Air Force Instruction

DISA – Defense Information Systems Agency

DNS – Domain Naming Service

DoD – Department of Defense

DoDM – Department of Defense Manual

EMSEC – Emissions Security

EOL – End of Life

FAS – Fabric Attached Storage

FCL – Facility Clearance Letter

FOIA – Freedom of Information Act

FSO – Facility Security Officer

GAFB – Goodfellow Air Force Base

GPM – Government Project Monitor

JPAS - Joint Personnel Adjudication System

NACI - National Agency Check with Written Inquiries

NFS – Network File System

NGA – National Geospatial-Intelligence Agency

NSA – National Security Agency

CIA – Central Intelligence Agency

SCIF – Sensitive Compartmented Information Facility

SCI-DCID – Sensitive Compartmented Information – Director of Central Intelligence Directive

SI/TK – Special Intelligence / Talent Keyhole

SOO – Statement of Objectives

STIG – Security Technical Implementation Guide

TRSS – Training Support Squadron

VTC – Video Teleconference

Attachment 1 – equipment list

| Description | Manufacturer | Model | Part Number | QTY |
| --- | --- | --- | --- | --- |
| VTC Codecs |  |  |  |  |
| CISCO WEBEX PRO CODEC KIT [SIPR/TS/TS-SCI] | CISCO |  | CS-CODEC-PRO-K9++ | 2 |
| ESS WITH 8X5XNBD CISCO WEBEX PRO CODEC KIT | CISCO |  | CON-ECDN-CSCOREC9 | 2 |
| PWR CORD US 1.8M GREY YP-12 TO YC-12 | CISCO |  | PWR-CORD-USA-B | 2 |
| CISCO WEBEX PRO CODEC | CISCO |  | CS-CODEC-PRO-NR++ | 2 |
| WEBEX PRO RACK EARS - FOR AUTO EXPAND ONLY | CISCO |  | CS-CPRO-RACKEARS | 2 |
| NETWORK CABLE ETHERNET 8M | CISCO |  | CAB-DV10-8M+ | 1 |
| Cisco Webex Room Navigator - Table stand version | CISCO |  | CS-T10-TS-L-K9 | 1 |
| ESS WITH 8X5XNBD Cisco Touch 10 inch | CISCO |  | CON-ECDN-T10TSK9 | 1 |
| WEBEX PRO MULTISITE SOFTWARE OPTION | CISCO |  | L-KITPRO-MS | 2 |
| ELECTRONIC DELIVERY PAK FOR CISCO SPARK ROOM SERVICE | CISCO |  | L-ROOM-PAK | 2 |
| MEDIA CONVERTER (US) | DMSI |  | MF31855 | 2 |
| 10ft LC-LC OM4 MM Patch cable | Fiberstore | OM4LCDX | 40233 | 2 |
| 10ft LC-ST OM4 MM Patch cable | Fiberstore | OM4LCSTDX | 74369 | 2 |
| 10ft LC-SC OM4 MM Patch cable | Fiberstore | OM4LCSCDX | 40272 | 2 |
| 4K HDMI and Audio to USB Scaling Bridge | EXTRON | MEDIAPORT 300 | 60-1873-01 | 1 |
| Cameras |  |  |  |  |
| i12 Intelligent PTZ Camera, 12x Optical Zoom, Moon Gray | CRESTRON | IV-CAM-I12-B | 6513204 | 1 |
| Custom camera box for use with a GPO DK75N utilizing a XTM1U mount. | AV3 | GPO-DK75N-XTM1U | GPO-DK75N-XTM1U | 1 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 1 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 1 |
| 6' Ultra Flexible Premium High Speed HDMI Cables | EXTRON | HDMI Ultra/6 | 26-663-06 | 1 |
| Digital Video Routing |  |  |  |  |
| DigitalMedia™ Card Chassis for DM-NVX-C & DMCF, 8 Slots | CRESTRON | DMF-CI-8 | 6507968 | 1 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder Card | CRESTRON | DM-NVX-360C | 6511007 | 8 |
| 6' Ultra Flexible Premium High Speed HDMI Cables | EXTRON | HDMI Ultra/6 | 26-663-06 | 8 |
| Displays |  |  |  |  |
| 75" 4K/UHD LCD Monitor, TAA compliant | GPO | DK75N | DK75N | 2 |
| Fusion Mount For 60" And Above LCD | Chief | XTM1U | XTM1U | 2 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 2 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 2 |
| 6' Ultra Flexible Premium High Speed HDMI Cables | EXTRON | HDMI Ultra/6 | 26-663-06 | 2 |
| Signage |  |  |  |  |
| Digital Graphics Engine 100 | CRESTRON | DGE-100 | DGE-100 | 2 |
| Vaddio LT PoE++ 60W Midspan Power Injector | VADDIO | 802-3303 | 802-3303 | 2 |
| 48" Power over Ethernet Display Panoramic | GPO | POE48D | POE48D | 1 |
| 27" Power over Ethernet Display Panoramic | GPO | POE27D | POE27D | 1 |
| Ultra Flexible Premium High Speed HDMI Cables, 3' | EXTRON | HDMI Ultra/3 | 26-663-03 | 2 |
| Operators Position |  |  |  |  |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 1 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 1 |
| USB HUB | STARTECH | ST4200USBM | ST4200USBM | 1 |
| IR Remote Control Kit | Celadon | 02591-VTC-001 | 02591-VTC-001 | 1 |
| Single port CAT 5e/6/7 100m USB 2.0 extender | ICRON | Ranger 2301 | Ranger 2301 | 2 |
| Computer Interfaces |  |  |  |  |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 8 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 8 |
| 6' Ultra Flexible Premium High Speed HDMI Cables | EXTRON | HDMI Ultra/6 | 26-663-06 | 8 |
| USB-RS232 interface | Vetra | USB-335 | USB-335 | 4 |
| Audio |  |  |  |  |
| Fixed I/O DSP with 12 analog inputs, 8 analog outputs, 8 channels configurable USB audio, AVB | BIAMP | TesiraFORTE AVB CI | TesiraFORTE AVB CI | 1 |
| X-Series Amplifier, 300 W | Crestron | AMP-X300 | 6510866 | 1 |
| 5-port expansion device | BIAMP | TesiraCONNECT TC-5 | TesiraCONNECT TC-5 | 1 |
| Parle TCM-x Ceiling Microphone, white | BIAMP | TCM-X White | TCM-X White | 2 |
| Parle TCM-xex Ceiling Microphone Expansion, white | BIAMP | TCM-XEX White | TCM-XEX White | 2 |
| 8" Two-Way SoundField Ceiling Tile Speaker (pair) | EXTRON | SF 228T Plus | 60-1862-03 | 3 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 1 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 1 |
| Control/Network System |  |  |  |  |
| 4 SERIES PROCESSOR | CRESTRON | CP4 | CP4 | 1 |
| 10.1 in. Wall Mount Touch Screen, Government Version, Black Smooth | CRESTRON | TSW-1070-GV-B-S | TSW-1070-GV-B-S | 1 |
| Equipment Racks |  |  |  |  |
| Araknis 420 Series PoE+ Network Switch - 48 Port | ARAKNIS | AN-420-SW-F-48-POE | AN-420-SW-F-48-POE | 1 |
| 38RU BGR Series Equipment Rack (32" Deep) | MID ATLANTIC | BGR-38SA-32LRD | BGR-38SA-32LRD | 1 |
| BGR Series Casters | MID ATLANTIC | CBS-BGR | CBS-BGR | 1 |
| Plexi-Glass Front Door | MID ATLANTIC | BPFD-38 | BPFD-38 | 1 |
| Equipment Cooling Rack Fan | MID ATLANTIC | BGR-276FT-FC | BGR-276FT-FC | 1 |
| Rack Fan Vent Cover Kit | MID ATLANTIC | VBK-BGR-SA | VBK-BGR-SA | 1 |
| Vertical Power Strip - 24 Outlet | MID ATLANTIC | PD-2420SC-NS | PD-2420SC-NS | 1 |
| Rackscrews, 10-32, Trim-Head, 500 pc. | MID ATLANTIC | HW500 | HW500 | 1 |
| 1 RU Utility Rack Shelf | MID ATLANTIC | U1 | U1 | 4 |
| Blank Panel, 1 RU, Steel, Flanged (Box of 12) | MID ATLANTIC | EB1-CP12 | EB1-CP12 | 3 |
| Lace Bar 2 Inch Offset, L-Shaped (10-pack) | MID ATLANTIC | LBP-2A | LBP-2A | 2 |
| Energy Monitoring Power Conditioner & Controller 300 | CRESTRON | PC-300 | 6507244 | 2 |
| TAA SmartPro 120V 1.5kVA 1.35kW Line-Interactive Sine Wave UPS, 2U, Extended Run, Network Card Options, LCD, USB, DB9 | TRIPPLITE | SM1500RMXL2UTAA | SM1500RMXL2UTAA | 1 |
| Multi-Mount PDU, 20 Outlet & 15A | MID ATLANTIC | PD-2015R-NS | PD-2015R-NS | 2 |
| Spares |  |  |  |  |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder Card | CRESTRON | DM-NVX-360C | 6511007 | 2 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 2 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 2 |
| Miscellaneous |  |  |  |  |
| Black Commercial Grade General Purpose 16 AWG 2 Conductor Plenum Cable Reel (1000') | LIBERTY | 16-2C-P-BLK | 16-2C-P-BLK | 1 |
| 1000' Black Category 5e U/UTP EN Series 24 AWG 4 Pair Unshielded Cable Reel | LIBERTY | 24-4P-P-L5-EN-BLK | 24-4P-P-L5-EN-BLK | 2 |
| 1000' Black Category 5e F/UTP EN Series Plenum 24 AWG 4 Pair Shielded Cable Reel | LIBERTY | 24-4P-P-L5SH-BLK | 24-4P-P-L5SH-BLK | 2 |
| 6ft Cat6 Snagless Unshielded UTP Ethernet Network Patch Cable - Blue | C2G | CG03975 | CG03975 | 6 |
| 12ft Cat6 Snagless Unshielded UTP Ethernet Network Patch Cable - Blue | C2G | CG03978 | CG03978 | 6 |
| 6ft Cat6 Snagless Shielded STP Ethernet Network Patch Cable - Gray | C2G | CG00779 | CG00779 | 6 |
| 12ft Cat6 Snagless Shielded STP Ethernet Network Patch Cable - Gray | C2G | CG00784 | CG00784 | 6 |
| DisplayPort Male to HDMI Female Active Adapter Cable – 6” | EXTRON | DPM-HDF/0.5 4K PLUS | 26-713-01 | 6 |
| OFR Overfloor Raceway Base and Cover | WIREMOLD | OFRBC-8 | OFRBC-8 | 8 |
| OFR Overfloor Raceway Enterance End Fitting | WIREMOLD | OFR10A | OFR10A | 1 |
| OFR Overfloor Raceway Two Gang Box | WIREMOLD | OFR48-2 | OFR48-2 | 1 |
| Description | Manufacturer | Model | Part Number | QTY |
| VTC Codecs |  |  |  |  |
| CISCO WEBEX PRO CODEC KIT [SIPR/TS/TS-SCI] | CISCO |  | CS-CODEC-PRO-K9++ | 2 |
| ESS WITH 8X5XNBD CISCO WEBEX PRO CODEC KIT | CISCO |  | CON-ECDN-CSCOREC9 | 2 |
| PWR CORD US 1.8M GREY YP-12 TO YC-12 | CISCO |  | PWR-CORD-USA-B | 2 |
| CISCO WEBEX PRO CODEC | CISCO |  | CS-CODEC-PRO-NR++ | 2 |
| WEBEX PRO RACK EARS - FOR AUTO EXPAND ONLY | CISCO |  | CS-CPRO-RACKEARS | 2 |
| NETWORK CABLE ETHERNET 8M | CISCO |  | CAB-DV10-8M+ | 1 |
| Cisco Webex Room Navigator - Table stand version | CISCO |  | CS-T10-TS-L-K9 | 1 |
| ESS WITH 8X5XNBD Cisco Touch 10 inch | CISCO |  | CON-ECDN-T10TSK9 | 1 |
| WEBEX PRO MULTISITE SOFTWARE OPTION | CISCO |  | L-KITPRO-MS | 2 |
| ELECTRONIC DELIVERY PAK FOR CISCO SPARK ROOM SERVICE | CISCO |  | L-ROOM-PAK | 2 |
| MEDIA CONVERTER (US) | DMSI |  | MF31855 | 2 |
| 10ft LC-LC OM4 MM Patch cable | Fiberstore | OM4LCDX | 40233 | 2 |
| 10ft LC-ST OM4 MM Patch cable | Fiberstore | OM4LCSTDX | 74369 | 2 |
| 10ft LC-SC OM4 MM Patch cable | Fiberstore | OM4LCSCDX | 40272 | 2 |
| 4K HDMI and Audio to USB Scaling Bridge | EXTRON | MEDIAPORT 300 | 60-1873-01 | 1 |
| Cameras |  |  |  |  |
| i12 Intelligent PTZ Camera, 12x Optical Zoom, Moon Gray | CRESTRON | IV-CAM-I12-B | 6513204 | 2 |
| Custom camera box for use with a GPO DK86N utilizing a XTM1U mount. | AV3 | GPO-DK86N-XTM1U | GPO-DK86N-XTM1U | 1 |
| Custom camera box shelf for PTZ camera. | AV3 | PTZ-WALL-BOX | PTZ-WALL-BOX | 1 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 2 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 2 |
| 6' Ultra Flexible Premium High Speed HDMI Cables | EXTRON | HDMI Ultra/6 | 26-663-06 | 2 |
| Digital Video Routing |  |  |  |  |
| DigitalMedia™ Card Chassis for DM-NVX-C & DMCF, 8 Slots | CRESTRON | DMF-CI-8 | 6507968 | 1 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder Card | CRESTRON | DM-NVX-360C | 6511007 | 8 |
| 6' Ultra Flexible Premium High Speed HDMI Cables | EXTRON | HDMI Ultra/6 | 26-663-06 | 8 |
| Displays |  |  |  |  |
| 86" 4K/UHD LCD Monitor, TAA compliant | GPO | DK86N | DK86N | 2 |
| 75" 4K/UHD LCD Monitor, TAA compliant | GPO | DK75N | DK75N | 1 |
| Fusion Mount For 60" And Above LCD | Chief | XTM1U | XTM1U | 3 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 3 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 3 |
| 6' Ultra Flexible Premium High Speed HDMI Cables | EXTRON | HDMI Ultra/6 | 26-663-06 | 3 |
| Signage |  |  |  |  |
| Digital Graphics Engine 100 | CRESTRON | DGE-100 | DGE-100 | 2 |
| Vaddio LT PoE++ 60W Midspan Power Injector | VADDIO | 802-3303 | 802-3303 | 2 |
| 48" Power over Ethernet Display Panoramic | GPO | POE48D | POE48D | 1 |
| 27" Power over Ethernet Display Panoramic | GPO | POE27D | POE27D | 1 |
| Ultra Flexible Premium High Speed HDMI Cables, 3' | EXTRON | HDMI Ultra/3 | 26-663-03 | 2 |
| Operators Position |  |  |  |  |
| 27” IPS UHD 4K Monitor (3840x2160) | LG | 27BN85UN-B | 27BN85UN-B | 2 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 2 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 2 |
| USB HUB | STARTECH | ST4200USBM | ST4200USBM | 1 |
| IR Remote Control Kit | Celadon | 02591-VTC-001 | 02591-VTC-001 | 1 |
| Single port CAT 5e/6/7 100m USB 2.0 extender | ICRON | Ranger 2301 | Ranger 2301 | 2 |
| Computer Interfaces |  |  |  |  |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 8 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 8 |
| 6' Ultra Flexible Premium High Speed HDMI Cables | EXTRON | HDMI Ultra/6 | 26-663-06 | 8 |
| USB-RS232 interface | Vetra | USB-335 | USB-335 | 4 |
| Audio |  |  |  |  |
| Fixed I/O DSP with 12 analog inputs, 8 analog outputs, 8 channels configurable USB audio, AVB | BIAMP | TesiraFORTE AVB CI | TesiraFORTE AVB CI | 1 |
| X-Series Amplifier, 300 W | Crestron | AMP-X300 | 6510866 | 1 |
| 5-port expansion device | BIAMP | TesiraCONNECT TC-5 | TesiraCONNECT TC-5 | 1 |
| Parle TCM-x Ceiling Microphone, white | BIAMP | TCM-X White | TCM-X White | 2 |
| Parle TCM-xex Ceiling Microphone Expansion, white | BIAMP | TCM-XEX White | TCM-XEX White | 2 |
| 8" Two-Way SoundField Ceiling Tile Speaker (pair) | EXTRON | SF 228T Plus | 60-1862-03 | 5 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 1 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 1 |
| Control/Network System |  |  |  |  |
| 4 SERIES PROCESSOR | CRESTRON | CP4 | CP4 | 1 |
| 10.1 in. Wall Mount Touch Screen, Government Version, Black Smooth | CRESTRON | TSW-1070-GV-B-S | TSW-1070-GV-B-S | 1 |
| Equipment Racks |  |  |  |  |
| Araknis 420 Series PoE+ Network Switch - 48 Port | ARAKNIS | AN-420-SW-F-48-POE | AN-420-SW-F-48-POE | 1 |
| 38RU BGR Series Equipment Rack (32" Deep) | MID ATLANTIC | BGR-38SA-32LRD | BGR-38SA-32LRD | 1 |
| BGR Series Casters | MID ATLANTIC | CBS-BGR | CBS-BGR | 1 |
| Plexi-Glass Front Door | MID ATLANTIC | BPFD-38 | BPFD-38 | 1 |
| Equipment Cooling Rack Fan | MID ATLANTIC | BGR-276FT-FC | BGR-276FT-FC | 1 |
| Rack Fan Vent Cover Kit | MID ATLANTIC | VBK-BGR-SA | VBK-BGR-SA | 1 |
| Vertical Power Strip - 24 Outlet | MID ATLANTIC | PD-2420SC-NS | PD-2420SC-NS | 1 |
| Rackscrews, 10-32, Trim-Head, 500 pc. | MID ATLANTIC | HW500 | HW500 | 1 |
| 1 RU Utility Rack Shelf | MID ATLANTIC | U1 | U1 | 4 |
| Blank Panel, 1 RU, Steel, Flanged (Box of 12) | MID ATLANTIC | EB1-CP12 | EB1-CP12 | 3 |
| Lace Bar 2 Inch Offset, L-Shaped (10-pack) | MID ATLANTIC | LBP-2A | LBP-2A | 2 |
| Energy Monitoring Power Conditioner & Controller 300 | CRESTRON | PC-300 | 6507244 | 2 |
| TAA SmartPro 120V 1.5kVA 1.35kW Line-Interactive Sine Wave UPS, 2U, Extended Run, Network Card Options, LCD, USB, DB9 | TRIPPLITE | SM1500RMXL2UTAA | SM1500RMXL2UTAA | 1 |
| Multi-Mount PDU, 20 Outlet & 15A | MID ATLANTIC | PD-2015R-NS | PD-2015R-NS | 2 |
| Spares |  |  |  |  |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder Card | CRESTRON | DM-NVX-360C | 6511007 | 2 |
| Wall Mount Power Pack, 24 VDC, 1.25 A, 2.1 mm, Universal | CRESTRON | PW-2412WU | 6507485 | 2 |
| DM NVX® 4K60 4:4:4 HDR Network AV Encoder/Decoder | CRESTRON | DM-NVX-360 | 6511006 | 2 |
| Miscellaneous |  |  |  |  |
| Black Commercial Grade General Purpose 16 AWG 2 Conductor Plenum Cable Reel (1000') | LIBERTY | 16-2C-P-BLK | 16-2C-P-BLK | 1 |
| 1000' Black Category 5e U/UTP EN Series 24 AWG 4 Pair Unshielded Cable Reel | LIBERTY | 24-4P-P-L5-EN-BLK | 24-4P-P-L5-EN-BLK | 2 |
| 1000' Black Category 5e F/UTP EN Series Plenum 24 AWG 4 Pair Shielded Cable Reel | LIBERTY | 24-4P-P-L5SH-BLK | 24-4P-P-L5SH-BLK | 2 |
| 6ft Cat6 Snagless Unshielded UTP Ethernet Network Patch Cable - Blue | C2G | CG03975 | CG03975 | 6 |
| 12ft Cat6 Snagless Unshielded UTP Ethernet Network Patch Cable - Blue | C2G | CG03978 | CG03978 | 6 |
| 6ft Cat6 Snagless Shielded STP Ethernet Network Patch Cable - Gray | C2G | CG00779 | CG00779 | 6 |
| 12ft Cat6 Snagless Shielded STP Ethernet Network Patch Cable - Gray | C2G | CG00784 | CG00784 | 6 |
| DisplayPort Male to HDMI Female Active Adapter Cable – 6” | EXTRON | DPM-HDF/0.5 4K PLUS | 26-713-01 | 6 |