# Specification Sheet

The Government requires commercially available equipment meeting the specifications provided below.

Item 1 - Camera

Quantity: 4

| Specification | Value |
| --- | --- |
| Description | CAMERA, IMX661 SENSOR |
| Resolution | 127.4MP |
| Sensor Type | 56.7MM CMOS |
| Max Frame Rate | 15FPS (12-BIT) |
| Cell Size | 3.45 x 3.45 MICRON |
| Lens Mount | M82X0.75 |
| Shutter Type | Global Shutter |
| Bit Depth | 12-BIT |
| GPIO/Triggering | 3 IN/3 OUT SOFTWARE/EXTERNAL (PULSE OR EDGE) |
| Exposure/Integration | 1 MICROSECOND – 1 SECOND |
| Dynamic Range | >66DB |
| Color | Monochrome |
| Operating System | WINDOWS 64-BIT |
| Power Supply | 12W 12V (INCLUDED) |
| Dimensions | 95 X 95 X 53MM |
| Weight | 800G |

NOTE: The requirement can be met by manufacturer Emergent Vision Technologies, part number HB-127-S-M-E2, or an “or equal” commercially available product.

Item 2 - Laser

Quantity: 4

| Specification | Value |
| --- | --- |
| Description | LASER SYSTEM, LAMP PUMPED ND:YAG |
| Energy per Pulse | 370MJ AT 532NM WAVELENGTH |

| Repetition Rate | 10HZ |
| --- | --- |
| Pulse Width | <15NS AT 532NM |
| Energy Stability | <2% RMS AT 532NM |
| Beam Diameter | <12MM AT 532NM |
| Divergence | 3MRAD AT 532NM |
| Transverse Mode | Multiple |
| Resonator Configuration | Stable |
| Polarization Direction | VERTICAL AT 532NM |
| Spectral Purity | >98% AT 532NM |
| Power | 200-240VAC 50/60HZ <10A |
| Computer Interface | RS232 |
| Cable Length | 3M |

NOTE: The requirement can be met by manufacturer Lumibird, part number DRL 1430523-10, or an “or equal” commercially available product.