Drivable Aluminum Monument for Wetlands

Drivable aluminum monument, designed for use in wetlands, swamps, riverbanks and lake shores - anywhere a substantial monument is needed and digging a hole is impossible due to constant water cover

- 3 ¼" diameter aluminum survey cap

- 2" diameter aluminum pipe, 30” long

- Cast aluminum base point

- Cone-shaped point is fluted for easy installation

- Point tapers out to a diameter larger than the diameter of the pipe to deter removal

- Cap and base are held to the pipe with aluminum rivets

- Permanent ceramic magnet located in the survey cap

Drivable Steel Monument with Bronze Cap

Drivable monument with stainless steel rod and bronze cap

- 3 ½" diameter forged bronze survey cap

- 9/16" diameter stainless steel rod, 30” long with pointed tip

- Powerful magnet installed on top of rod

- Stainless steel flange that anchors the monument firmly in the ground welded to the rod

Aluminum Drive-in Survey Monument

- 3¼" (83mm) diameter series 6000 aluminum survey cap

- 1" (50mm) diameter aluminum pipe, 30" long with pointed tip.

- Cast aluminum base point

- Cap and base are held to the pipe with aluminum rivets

- Permanent ceramic magnet located below the break-off point in the base.

- Cap should be stamped as shown below

  - 3/16" Letters

  - Optional stamped center point

2" diameter Aluminum Cap for 5/8" Rebar

- Survey cap for rebar forged from 6000 series aluminum which is highly resistant to chipping, cracking and breaking.

- Feature SureGripT plastic insulator that locks the cap tight on to the rebar or pipe and helps prevent dissimilar metal corrosion.

- Feature a deep hex socket that won't "wobble" when tapped on to the rebar.

- Cap should be stamped as shown below

  - 1/8" Letters

  - Optional stamped center point

2" diameter Aluminum Cap for 3/4" Rebar

- Survey cap for rebar forged from 6000 series aluminum which is highly resistant to chipping, cracking and breaking.

- Feature SureGripT plastic insulator that locks the cap tight on to the rebar or pipe and helps prevent dissimilar metal corrosion.

- Feature a deep hex socket that won't "wobble" when tapped on to the rebar.

- Cap should be stamped as shown below

  - 1/8" Letters

  - Optional stamped center point

Steel Nail Marker with SURVEY MARK imprint

- Strong solid steel and drives easily into blacktop.

- "SURVEY MARK" is imprinted on top of nail for easy identification as a survey mark.

- 3" in length with a 1" (25mm) convex top and conical recessed center chamfer for pinpoint accuracy.