Contractor Name:

Notice ID:

Date:

NAICS Code(s):

Business Size (Small, Large, Disadvantages (specify), 8(a):

| Question | (Circle One) |  | Comments |
| --- | --- | --- | --- |
| Does contractor agree to perform the work using material that is corrosion resistant steel, conforming to the chemical composition of UNS S44003 or UNS S44004 in accordance with ASTM A276? | Yes | No |  |
| Does the contractor agree to meet Grade 10 material? | Yes | No |  |
| Does the contractor agree to manufacture the ball bearings in the United States or Canada? | Yes | No |  |
| Does the contractor agree to meet the harness requirement of 58-65 HRC or equivalent? | Yes | No |  |
| Does the contractor agree to meet the basic diameter of 0.125000", with an allowable ball diameter variation of 10 millionths of an inch (.00001)? | Yes | No |  |
| Does the contractor agree to meet the basic diameter of .125000", with an allowable spherical form deviation of 10 millionths of an inch (.00001)? | Yes | No |  |
| Does the contractor agree to meet the materail density of .277 pound per cubic inch? | Yes | No |  |
| Does the contractor agree to meet the a ball diameter of 10 millionths of an inch. (.00001)? | Yes | No |  |
| Does contractor agree to have correspondence by email with CAAA personnel in a timely manner (within 24 hours) of first contact? | Yes | No |  |
| Does the contractor agree to provide the Certificate of Analysis for CAAA approval on all lots PRIOR to shipment? | Yes | No |  |
| Does the contractor agree to provide test results referencing material, hardness, surface roughness, material density and part number? | Yes | No |  |
| Does the contractor agree to identify deliveries by identifiable / traceable by a lot number? | Yes | No |  |
| Does the contractor agree to provide the delivery's in multiples of 1000, with 1000 packaged per bag? | Yes | No |  |
| Does the contractor understand that CAAA will reject any delivery that does not meet the specifications of Drawing 11713 | Yes | No |  |
| Final remarks for consideration for the Contracting Team: |  |  |  |