Statement of Requirements

Studio Camera Upgrade, 42 ABW/PA

1.0   PURPOSE: The purpose of this requirement is to procure three (3) new studio-quality cameras for the Public

Affairs office. These cameras will replace outdated equipment and enhance video production and broadcasting

capabilities to support the installation's communication mission.

2.0   DESCRIPTION OF ITEMS: The following items are required for performance under this contract: Three (3) Sony FR7 Cinema Line PTZ camera Kits with 28-135mm Zoom Lens (MFR ILME-FR7K). The camera kits must each meet the following specifications:

2.1 Imaging

2.1.1 Must have a full-frame interchangeable Lens mount

2.1.2 Lens Mount: Sony E

2.1.3 Sensor Resolution/Actual: 12.9 Megapixel

2.1.4 Senor Resolution/ Effective: 10.3 Megapixel

2.1.5 Lens Communication: Yes, with autofocus support

2.1.6 Focus Control: Autofocus

2.1.7 Built in Neutral Density filter

2.2 Exposure Control

2.2.1 Shutter Speed: 1/8000 to 1 Second

2.2.2 ISO Sensitivity Range: 800 to 12,800 (Extended: 409,600)

2.2.3 Gain: -3 to 30dB

2.2.4 Flip/Mirror Support: Image Flip

2.2.5 White Balance/Range: 2000 to 15,000K

2.3 Video Capture

2.3.1 External Recording Modes: HDMI:

2.3.1.1 4096 x 2160 at 23.98/24.00/25/29.97/50/59.94 fps

2.3.1.2 3840 x 2160 at 23.98/25/29.97/50/59.94 fps

2.3.1.3 1920 x 1080 at 23.98/24.00/25/29.97/50/59.94 fps

2.3.2 External Recording Modes: Raw 15-Bit via SDI/BNC

2.3.2.1 4096 x 2160 at 23.98/24.00/25/29.97/50/59.94 fps

2.3.2.2 3840 x 2160 at 23.98/25/29.97/50/59.94 fps

2.3.2.3 1920 x 1080 at 23.98/24.00/25/29.97/50/59.94 fps

2.3.3 Internal Recording Modes: 4:2:2 10-Bit

2.3.3.1 DCI 4K (4096 x 2160) at 24.00/25/29.97/50/59.94 fps [240 to 600 Mb/s VBR]

2.3.3.2 UHD 4K (3840 x 2160) at 25/29.97/50/59.94 fps [240 to 600 Mb/s VBR]

2.3.3.3 1920 x 1080 at 25/29.97/50/59.94 fps [89 to 222 Mb/s]

2.3.4 Internal Recording Modes: 4:2:0 8-Bit

2.3.4.1 UHD 4K (3840 x 2160) at 25/29.97/50/59.94 fps [100 to 150 Mb/s VBR]

2.3.4.2 1920 x 1080 at 25/29.97/50/59.94 fps [35 to 50 Mb/s VBR]

2.3.5 Audio Recording: 4-Channel 24-Bit 48 kHz LPCM Audio

2.3.6 Broadcast Output: NTSC

2.3.7 IP Streaming: 640 x 360 to 4096 x 2160 (0.5 to 80.00)

2.4 Control

2.4.1 Pan: 340 degrees (-170 to 170 degrees)

2.4.2 Tilt: 225 degrees (-30 to 195 degrees)

2.4.3 Tally Light: Yes

2.5 Interface

2.5.1 Media/Memory Car Slot: Dual Slot: CFexpress Type A and SDXC

2.5.2 Audio I/O: 1x XLR 5-Pin Mic/Line (+48 V Phantom Power) Input

2.5.3 Other I/O: 1x RJ45 (LAN)

2.6 Power

2.6.1 PoE Support: PoE++ 802.3bt

2.7 Lens

2.7.1 Focal Length: 28-135mm

2.7.2 Maximum Aperture: f/4

2.7.3 Minimum Aperture: f/22

2.7.4 Lens Mount: Sony E

2.7.5 Lens Format Coverage: Full-Frame

2.7.6 Angle of View:

2.7.6.1 Full-frame: Horizontal: 75 to 18°

2.7.6.2 APS-C (1.5x): Horizontal: 54 to 12°

2.7.7 Image Stabilization: Yes

2.7.8 Electronic Communication: Yes

2.7.9 Zoom Ratio: 4.8x

2.7.10 Minimum Focus Distance: 1.3’ / 40 cm (Wide) to 3.12’ / 95cm (Tele)

2.7.11 Magnification: 6.66:1 Macro Reproduction Ration

0.15x Magnification

3.0  DELIVERY REQUIREMENTS:

3.1 Items are to be delivered to 600 Chennault Circle, Maxwell AFB, AL 36112.  The facility has a loading dock that accommodates medium to small sized delivery trucks.  Semi-trucks will not fit in loading dock area.

3.2 The facility number is 1405, Room 130 located on the first floor.

4.0 STANDARD WARRANTY: The contractor shall provide any Original Equipment Manufacturer

(OEM) standard commercial warranty applicable to the products being purchased at no additional cost

to the Government.

4.1 Prior to project acceptance, the contractor shall provide the customer with all manufacturer warranties as needed for any component incorporated into this project such as equipment manuals, literature, CDs and DVDs and other documentation on all equipment.

4.2 Prior to project acceptance, the contractor shall provide written procedures, points of contact within company, and required information for all warranty services prior to project acceptance. This information shall include written confirmation of the warranty period, phone numbers and contact information/procedures for technical support, troubleshooting assistance, and replacement of faulty equipment.

5.0   GENERAL INFORMATION: Items must be brand new. Refurbished items are not authorized.