PERFORMANCE WORK STATEMENT (PWS)

Training Support Center Stuttgart – Range 7 Full Gran-Tex Material Exchange/Panzer Kaserne Range Complex, RTSD-West

18 FEB 2026

Part 1

General Information

1.  General:  This is a non-personal service(s) contract under which the personnel rendering the service(s) are not subject, either by the contract’s terms or by the manner of its administration, to the supervision and control usually prevailing in relationships between the Government and its employees.

1.1  Description of Services/Introduction:  This is a non-personal service(s) contract to provide range maintenance and lead/residue remediation by conducting a full material exchange for Panzer Range Complex – Range 7, which incorporate the use of granulated rubber (commonly known as Gran Tex®) backstop material or rubber blocks at target emplacements enhancing the operational safety of this range while reducing the probability of ricochets. The Government shall not exercise any supervision or control over the contract service providers performing the services herein. Such contract service providers shall be accountable solely to the Contractor who, in turn is responsible to the Government.

1.2  Background: As a cost saving effort, USAREUR-AF Range and Training Land Program (RTLP) started using a granulated rubber material called Gran Tex®. As part of it’s modernization responsibilities, RTLP began installing this material at selected 25 Meter live fir erangesin 2004. Since 204, RTLP has replaced sand and installed Gran Tex® at 13 USAREUR ranges. Current costs for remediation at these ranges have saved the U.S. Army significant money per bullet catch and substantially reduced the previous costs incurred for sand replacement and lead remediation.

Seventh U.S. Army Training Command (7ATC), Training Support Activity Europe (TSAE) is responsible for managing and executing the USAREUR-AF Sustainable Range Program (SRP) IAW AR 350-19 (Army Sustainable Range Program) and DA-G3 guidance. One of the major core components of the SRP is RTLP which is responsible for the operation and modernization of USAREUR-AF’s live fire ranges and training facilities. In executing this mission RTLP services and conducts periodic lead remediation on USAREUR-AF ranges. The purpose of this contract is to provide lead remediation at specified Gran Tex® (or similar ballistic rubber within the specifications) equipped ranges throughout USAREUR-AF.

1.3  Scope:  The Contractor shall provide non-personal services for range maintenance to include inspection and repair of the reclining Gran Traps and rubber coverings. The contractor shall accomplish remediation of lead, reinstallation of Gran Tex® (or similar ballistic rubber within the specifications) material, and installation of rubber blocks for designated range target pits. The Contractor shall provide all personnel, equipment, supplies, transportation, tools, materials, supervision, and other items necessary to perform services as described in the PWS and associated contract documents. The Contractor shall perform to the standards and acceptable quality levels identified in this PWS and associated contract documents.

1.4  Objectives:

- Conduct full Gran Tex® (or similar ballistic rubber within the specifications) material exchange.

- Conduct lead remediation during/after Gran Tex removal from bullet catch.

- Inspect, repair/replace, and re-install rubber matting to cover bullet catch.

The Government shall not exercise supervision or control over the contract service providers performing the services herein. Such contract service providers shall be accountable solely to the Contractor who, in turn is responsible to the Government.

1.5 General Information

1.5.1  Quality Control Plan (QCP): The Contractor shall develop and maintain a QCP to ensure services are performed in accordance with (IAW) this PWS.  The Contractor shall develop and implement procedures to identify, prevent, and ensure non-recurrence of defective services.  The Contractor's QCP is the means by which it assures that the work provided complies with the requirements of the contract.

The Contractor’s Proposed QCP shall be submitted to the Contracting Officer (KO) through the Contracting Officer’s Representative (COR) for review within 10 working days after date of contract award (Deliverable #1).

The Government will review and either notify the Contractor in writing of acceptance of the plan or return their comments to the Contractor within 10 working days after receipt.  If the Government has provided comments, the Contractor shall then have 3 working days to submit a Final QCP.  After receipt of the Final QCP, the Contractor may receive the Contracting Officer’s acceptance in writing.  Any proposed changes to the accepted QCP are required to be resubmitted for acceptance by the Contracting Officer no later than 30 days prior to the anticipated change and before implementation by the Contractor.  The timeline noted above will apply for review and acceptance for proposed changes.  At a minimum, the QCP must include and answer the following to be acceptable:

a. A chart showing the organizational structure and lines of authority, the names, qualifications, duties, responsibilities, and classification of each member of the Contractor's Quality Control Team.

b. How will the Contractor monitor work to ensure that performance complies with all deliverables (etc. timelines, deadlines, and goals)?

c. How will the Contractor monitor work to ensure that performance complies with all specifications and requirements of the contract, including clauses?

d. How will the Contractor monitor and ensure staff qualifications remain current and valid including DOCPER processes/approvals throughout contract performance?

e. How will the Contractor ensure all keys issued will remain controlled items (Paragraph 1.7 Key Control)?

f. How will the Contractor inventory and track maintenance of all Government Provided Equipment / Materials?

g. How will the Contractor identify, investigate, and correct any non-conforming performance and prevent similar deficiencies in the future?

h. How will the Contractor file and save all Quality Control related documents for the life of the contract plus 5 years?

1.5.1.1 An Analysis of Quality Control and Timeliness of Submittals (i.e. deliverables) shall be provided to the COR within 2 working days upon completion of each option, period, or Task Order (Deliverable #2).

1.5.1.2  The Contractor shall inform the COR of issues or potential issues that might affect the performance within 2 working hours of identifying those issues.  Verbal reports shall be followed up by written reports within 1 working day (Deliverable #3).

1.5.2  Quality Assurance:  The Government will evaluate the Contractor’s performance under this contract in accordance with the Quality Assurance Surveillance Plan (QASP).  This plan is primarily focused on what the Government must do to ensure that the Contractor has performed in accordance with the performance standards.  It defines how the performance standards will be applied, the frequency of surveillance, and the acceptable quality levels (performance thresholds).

1.5.3  Recognized Holidays:  The following provides information on recognized holidays for the purpose of the PWS.  If submittal of any documentation (e.g. deliverables, submittals, etc.) deadlines fall on a holiday, the closest work day prior to the holiday will apply as the deadline for submittal.

1.5.3.1  U.S. Holidays: Work shall not be performed on U.S. holidays occurring during the normal workweek unless otherwise directed by the Contracting Officer. When a U.S holiday occurs on a Saturday or a Sunday, the holiday is observed on the preceding Friday or following Monday, respectively. The contractor is not required to perform services on holidays.

The Recognized American national holidays are:

New Year’s Day January 1st			M. L. King Memorial Day 3rd Monday in January

Presidents Day 3rd Monday in February		Memorial Day last Monday in May

Independence Day July 4th				Labor Day 1st Monday in September

Columbus Day 2nd Monday in October		Veteran’s Day November 11th

Thanksgiving Day 4th Thursday in November	Christmas Day December 25th

1.5.3.2  Host Nation Holidays:  Work shall be performed on local Host Nation holidays occurring during the normal workweek unless otherwise directed by the Contracting Officer.

The German national and local holidays are:

1st January	New Year’s Day (Neujahr)

6th January	Three King’s Day (Heilige Drei Könige)

Varies yearly	Good Friday (Karfeitag)

Varies yearly	Easter Monday (Ostermontag)

1st May	Labor Day (Tag der Arbeit)

Varies yearly	Ascension Day (Christi Himmelfahrt)

Varies yearly	Whit Monday (Pfingstmontag)

Varies yearly	Corpus Christi (Fronleichnam)

15th August	Assumption Day (Mariä Himmelfahrt)

3rd October	Day of German Unity (Tag der Deutschen Einheit)

1st November	All Saints’ Day (Allerheiligen)

25th December	1st Christmas Day (1. Weihnachtsfeiertag)

26th December	2nd Christmas Day (2. Weihnachtsfeiertag)

1.5.4  Operating Hours:  The Government facility office hours, facility operating hours, and the Contractor support hour requirements often coincide, however, they may differ. Please refer to the following.

1.5.4.1  Government Facility Office Hours:  The Government facility office hours are 0730-1630 Monday through Friday except U.S. Holidays identified in paragraph 1.5.3.1 above or when the Government facility is closed due to local or national emergencies, administrative closings, or similar Government directed facility closings.

1.5.4.2  Government Facility Operating Hours:  The Government facility operating hours occur between the hours of 0730-1630 Monday through Friday.  The Government facility is closed for operations during the recognized U.S. Holidays identified in paragraph 1.5.3.2 above.  The Government facility may close due to local or national emergencies, administrative closings, or similar Government directed facility closings and will be communicated by the Contracting Officer to the Contractor.

1.5.4.3  Contractor Support Hours:  The Contractor shall provide support between the hours of 0730-1630 Monday through Friday except U.S. Holidays identified in paragraph 1.5.3.1 above or when the Government facility is closed due to local or national emergencies, administrative closings, or similar Government directed facility closings. The Contractor shall at all times maintain an adequate workforce for the uninterrupted performance of all tasks defined within this PWS when the Government facility is not closed for the above reasons.  When hiring personnel, the Contractor shall keep in mind that the stability and continuity of the workforce are essential.

1.5.4.3.1  Contractor Work Plan: RESERVED

1.5.5  Training Holidays:  USAREUR has historically granted military personnel training holidays as compensation for time that they spent in field training. These are typically combined with regular holiday leave schedules (for example, Christmas half days or day on day off schedules).  Training holidays may affect access to military operated facilities.  If scheduled work or access to facilities is required, coordination with occupants will be necessary or the work in affected facilities shall be rescheduled after the training holiday.  However, announced training holidays or exodus of military personnel may provide the Contractor with an opportunity for unrestricted access to facilities and areas.  The Contractor shall take full advantage of such opportunities by planning maintenance or actions that are better accomplished when such areas are vacant.

1.5.6  Place of Performance:  The work to be performed under this contract will be performed on Range 7 at the Panzer Range Complex (PRC) located in Boeblingen (Stuttgart), Germany.

1.6  Security Requirements:  The following information is provided on security related matters.

1.6.1  Security Program: RESERVED

1.6.2  Personnel Security Clearance Requirements: RESERVED

1.6.3  Installation Access:  Access to U.S. installations and controlled areas is limited to personnel who meet security criteria and are authorized by Host Nation law to work in that country. Failure to submit required information/data and obtain required documentation or clearances in accordance with Army in Europe (AE) Regulation 190-16 (AE Reg 190-16) Installation Access Control or (AE) Regulation 604-1 for Germany specific contracts, will be grounds for denying access to U.S. installations and controlled areas. The Contractor shall ensure that any subcontractors used in performance of this contract complies with these requirements and that all employees, of both the Contractor and any subcontractor utilized by the Contractor, are made aware of and comply with these requirements. Should anything in this paragraph conflict with AE Reg 190-16 or (AE) Regulation 604-1, the regulation takes precedent.

The Contractor shall be aware of and comply with the requirements associated with Installation Access Control. The Government is not liable for any costs associated with performance delays due solely to a firm’s failure to comply with Installation Access Control System (IACS) processing requirements.

The Contractor shall return installation passes to the issuing IACS office when the contract is completed or when a Contractor employee no longer requires access.

AE Reg 190-16 (and AE 190-16-G German translation) can be found on the following website:

https://media.defense.gov/2017/Apr/27/2001921546/-1/-1/0/AER190-16.pdf

https://media.defense.gov/2017/Apr/27/2001921547/-1/-1/0/AER190-16-G.pdf

1.6.3.1  In addition, the Contractor shall provide the Contracting Officer, via the COR, an Initial Installation Pass Log within 3 calendar days after contract award (Deliverable #4).  The log must at a minimum contain the following information employee name, date of birth, nationality, and primary identification document number.  The log shall be updated as employees are added or removed.  The Updated Installation Pass Log shall be current and available at any time upon request by the Government and submitted to the COR within five (5) working days after any changes to the log (Deliverable #5).

1.6.3.2  The Contractor shall collect the installation access passes the same day employment of an individual has expired or has been terminated and shall return them to the issuing office within 2 working days or last working day prior to start of  weekend (Deliverable #6).

1.6.3.3  Upon termination or cancellation of this contract, the Contractor shall collect all outstanding installation access passes and return them to the issuing office within 2 working days or last working day prior to the start of weekend (Deliverable #7).

1.6.3.4  Common Access Cards (CAC):  RESERVED

1.6.4  Background Checks:  Army in Europe (AE) Regulation 604-1 provides guidance pertaining to background checks for U.S. and Host Nation Contractors. A digital copy of a background check at the national level is required to process the request for installation access during period of performance.

1.6.4.1  U.S. Citizen:  Contractors that work with or teach youth under the age of 18 years are subject to local and stateside background checks. The Requiring Activity Manager will determine the extent of background checks required. The Contractor employee may begin performance of the contract after receiving a favorable local background check. The Contractor agrees to replace the employee should results of the stateside background check, if required, be unfavorable as determined by the Requiring Activity Manager. The Contractor’s replacement of the employee with the unfavorable background check shall be within a timeframe provided by the Requiring Activity Manager. The Contractor shall also replace employees whose background check, during their tenure of employment, renders them with an unfavorable local or stateside background check, as determined by the Requiring Activity Manager. The Contractor shall forward a copy to the Contracting Officer via the COR and the Requiring Activity Manager of each favorable background check within 2 working days of receipt by the Contractor, identifying such with the appropriate contract number (Deliverable #8). The Contractor shall forward a copy to the Contracting Officer via the COR and the Requiring Activity Manager of each unfavorable background check within 2 working days of receipt by the Contractor, identifying such with the appropriate contract number (Deliverable #9). When the Government awards a non-personal services contract directly to an individual, the failure of the individual to provide a favorable background check would be grounds for termination.

1.6.4.2  Local Nationals and Third Country Nationals:  Local national and third country national Contractors that work with or teach youths under the age of 18 years are subject to background checks as determined by the Requiring Activity Manager. As a minimum prior to beginning performance, Contractors employing local national and third country nationals shall furnish the Contracting Officer via the COR and the Requiring Activity Manager a Polizeiliches Fuehrungszeugnis, issued within the previous four months, no later than 5 working days after contract award (Deliverable #10). Forwarding correspondence from the Contractors shall identify the associated contract number. The Requiring Activity Manager will determine the favorability of the Polizeiliche Fuehrungszeugnis and notify the Contractor. The Contractor shall forward a copy of unfavorable Polizeiliche Fuehrungszeugnis to the Contracting Officer via the COR and the Requiring Activity Manager within 24 hours of receipt, identifying such with the appropriate contract number (Deliverable #11).  Local nationals and third country nationals agree to the same employee termination provisions as stated for U.S. Citizens (see paragraph 1.6.4.1 above, in particular, same requirements to replace the employee (and within timeframe noted) when results of background check are unfavorable).

1.6.5   Physical Security:  The Contractor shall safeguard all Government equipment, information, and property provided for Contractor use. At the close of each work period, Government facilities, equipment, and materials shall be secured IAW the Army Physical Security Program (AR 190-13).

1.6.6  Operations Security (OPSEC) Requirements: Contractor personnel shall adhere to facility security policies and restrictions.  The Contractor shall immediately report suspicious activities to security personnel.

1.7   Key Control:  RESERVED

1.8  Post Award Conference/Periodic Progress Meetings:  The Contractor shall attend any post award conference convened by the contracting activity or contract administration office in accordance with FAR Subpart 42.5.  The Contracting Officer, COR, and other Government personnel, as appropriate, may meet periodically with the Contractor to review the Contractor's performance.  At these meetings the Contracting Officer will apprise the Contractor of how the Government views the Contractor's performance and the Contractor shall apprise the Government of problems, if any, being experienced.  Appropriate action shall be taken to resolve outstanding issues.  These meetings shall be at no additional cost to the Government.

1.8.1 The Contractor shall attend, participate in, and furnish input to scheduled and unscheduled meetings, conferences, and briefings that relate to the functions and services herein as required by the Government to provide effective communication and impart necessary information.  The Contract Manager or designated representative shall attend meetings as requested by the Government.  Meeting attendees shall at times include Contractor managerial, supervisory, and other personnel knowledgeable of the subject matter. Meetings may start or end outside of regular duty hours.

1.9  Contracting Officer’s Representative (COR):  Refer to Part 2 of this PWS for the definition of a COR.  As determined by the Contracting Officer, a COR will be appointed and identified by letter of designation, a copy of which will be provided to the Contractor by the Contracting Officer. The designation letter states the responsibilities and limitations of the COR, especially with regard to changes in cost or price, estimates, or changes in delivery dates. The COR is not authorized to change any of the terms and conditions of the resulting order. The COR monitors all technical aspects of the contract and assists in contract administration. The COR is not authorized to obligate the Government. If the work is not written in the contract, the COR is not authorized to request new work. The COR is authorized to perform the following functions: assure that the Contractor performs the technical requirements of the contract, perform inspections necessary in connection with contract performance, maintain written and oral communications with the Contractor concerning technical aspects of the contract, issue written interpretations of technical requirements, including Government drawings, designs, specifications, monitor Contractor's performance and notifies both the Contracting Officer and Contractor of any deficiencies, coordinate availability of Government property, and coordinate site entry of Contractor personnel.

1.10  Key Personnel:  RESERVED

1.11   Identification of Contractor Employees:  All Contractor personnel attending meetings, answering Government telephones, and working in other situations where their Contractor status is not obvious to third parties must identify themselves as such to avoid creating an impression in the minds of members of the public that they are Government officials.  The Contractor shall ensure that all documents or reports produced by Contractors are suitably marked as Contractor products or that Contractor participation is appropriately disclosed.

1.12.1   Contractor workspace (staging site, on-site office, desk or work area, etc.) shall contain a sign signifying the space is occupied by Contractor employee(s) to ensure that Government employees and the public know that they are not Government employees. The proposed sign shall be submitted to the Contracting Officer via the COR for review and acceptance 5 working days after contract award (Deliverable #13). Workspace sign location shall be coordinated with the COR. Contractor employees shall identify themselves by name and company affiliation when answering the telephone, presenting briefings, conducting or attending meetings, and seminars. All Contractor correspondence (written, facsimile, and email display) shall include their company name.  Requirements for Contractor vehicles is addressed under Part 4 paragraph 4.4.

1.12.2   Contractor Identification Badges:  Contractor personnel shall be easily identifiable through the display of badges IAW Contractor Identification (AE Reg. 27-715). (For services at Child, Youth, and School Services where children are present, recommend personnel wear identification when children are present.) (Deliverable #14)

1.13   Contractor Travel:  RESERVED

1.14  Other Direct Costs (ODCs):  RESERVED

1.15  Data Rights: The Government has unlimited rights to all documents and materials produced under this contract.  All documents and materials, to include the source codes of software, produced under this contract shall be Government owned and are the property of the Government with all rights and privileges of ownership and copyright belonging exclusively to the Government.  These documents and materials may not be used or sold by the Contractor without written permission from the Contracting Officer.  All materials supplied to the Government shall be the sole property of the Government and may not be used for any other purpose.  This right does not abrogate any other Government rights.

1.16  Non-Disclosure Requirements: Performance under this contract may require the Contractor to access data and information proprietary to a Government agency, another Government Contractor, or of such nature that its dissemination or use other than as specified in this work statement would be adverse to the interests of the Government or others.  The Contractor and Contractor personnel, shall not divulge or release data or information developed, or obtained under performance of this PWS, except to authorized Government personnel or upon written approval of the Contracting Officer.  The Contractor shall not use, disclose, or reproduce proprietary data, which bears a restrictive legend, other than as specified in this PWS.  All documentation showing individual names or other personal information shall be controlled and protected under the provisions of the Privacy Act of 1974, Public Law 93-579, 5 United States Code (U.S.C.) Section 552a.

1.17 The use of propaganda violates DoD Commercial Use of Imagery Guidelines stated at http://www.defenseimagery.mil/products/dodimagery/commercialuse.html  The Contractor shall not cite any information (e.g., contract information, pictures, locations, etc.) obtained through this contract on any marketing tools to include its company website.

1.18  Non-Disclosure Statements: The Contractor shall provide signed non-disclosure agreements to the Government no later than 14 calendar days (Deliverable #15) prior to commencement of work under the contract.  Disclosure of information by Contractor personnel may result in Contractor personnel removal from performance under this contract.

1.19  Organizational Conflict of Interest (OCI):  Contractor and subcontractor personnel performing work under this contract may receive, have access to or participate in the development of proprietary or source selection information (e.g., cost or pricing information, budget information or analyses, specifications or work statements, etc.) or perform evaluation services which may create a current or subsequent OCI as defined in FAR Subpart 9.5.  The Contractor shall notify the Contracting Officer immediately whenever it becomes aware that such access or participation may result in any actual or potential OCI and shall promptly submit a plan to the Contracting Officer to avoid or mitigate any such OCI (Deliverable #16).  The Contractor’s OCI Mitigation Plan will be determined to be acceptable solely at the discretion of the Contracting Officer.  In the event the Contracting Officer unilaterally determines that any such OCI cannot be satisfactorily avoided or mitigated, the Contracting Officer may affect other remedies as he or she deems necessary, including prohibiting the Contractor from participation in subsequent contracted requirements which may be affected by the OCI.

1.20   Phase In /Phase Out Period:  RESERVED

1.21  Safety:  The following provides information pertaining to Safety and Accident Prevention: All work completed under this contract is performed on live-fire training facilities. Therefore, the contractors will receive a safety briefing by range personnel prior to any maintenance services being performed.

1.21.1   Host Nation Safety Laws and Regulations:  The Contractor shall comply with FAR 52.236-13, Accident Prevention (November 1991), the references contained therein (e.g. U.S. Army Corps of Engineers Safety and Health Requirements Manual (EM 385-1-1)), and applicable Host Nation Safety Laws and Regulations.

The requirements under FAR 52.236-13 and specifically of the EM 385-1-1 include, but are not limited to the following:

(a) Before initiation of work at the job site, an Accident Prevention Plan, written by the Contractor for the specific work and hazards of the contract and implementing in detail the pertinent requirements of EM 385-1-1; will be reviewed for acceptance by designated Government personnel (Deliverable #17).  Specific requirements for development of the Accident Prevention Plan are found in 01.A and Appendix A of EM 385-1-1. EM 385-1-1 is available online at www.usace.army.mil.

(b) Before beginning each activity involving a type of work presenting hazards not experienced in previous project operations or where a new work crew or subcontractor is to perform work, activity hazard analysis (AHA) shall be prepared by the Contractor performing the work activity (See paragraph 01.a.09 of EM 385-1-1).

(c) The Contractor shall require subcontractors to submit their plan of operations showing methods they propose to use in accomplishing major phases of work.

(d) The Contractor shall discuss the plans in conferences convened by the Contracting Officer prior to starting work on each major phase of operation. Plans shall include all pertinent information such as layout of haul roads, access roads, storage areas, electrical distribution lines, methods of providing minimum exposure to overhead loads, and methods of access to work areas. The plan for accomplishing the initial work phase shall be submitted within fifteen (15) calendar days after award of the contract. Plans for subsequent major phases of work shall be submitted no later than fifteen (15) calendar days prior to initiation of work on each major phase.

(e) In all areas where construction, demolition, alteration, building, or similar related activities take place, the Contractor shall have the following minimum personal protective clothing and equipment requirements in place for any person working on or visiting the site:

(i) Short sleeve shirt.

(ii) Long trousers.

(iii) Steel-toed safety boots.

(iv) Hard hat.

1.21.1.1  In addition to other requirements for accident prevention specified in this contract, the Contractor shall comply with all Federal and State requirements governing safety and health protection on construction sites and other applicable occupational safety codes. In particular, within the requirements of the German Safety and Health Protection Regulation for Construction Sites, Baustellenverordnung (BaustellV) (current version in effect), the Contractor shall coordinate all work at the site, including development of plans and providing advance notices.

The Contractor shall indicate its plan for compliance considering conditions of the work described by the design, technical proposal, operational activities of the Government or others in the area of the work, identification of the coordinator, and implementation of all other accident prevention provisions of this contract. For the compliance with host nation laws and regulations, the Contractor shall comply with the non-exclusive list of duties described in this paragraph. If the Contractor determines that additional activities are required to fully comply, the Contractor shall ensure that compliance with the laws is complete. The duties include:

(a) Conditions of construction and operational activities to be considered include but are not limited to:

(i) For all construction activities meeting the following criteria: the work is scheduled to last longer than 30 working days and more than 20 workers are occupied simultaneously, or the volume of work is scheduled to exceed 500 person days, or the work presents particular tasks, e.g. excavations deeper than 5 meters, falls from a height of greater than 7 meters, exposures to hazardous chemical or biological substances, work at less than 5 meters from high voltage lines, underground earth works or tunnels, danger of drowning, diving, use of compressed air, work with ionizing radiation, explosives, or work involving assembly or dismantling of heavy prefabricated components (10 tons or greater).

(b) If the Contractor determines that the BaustellV, dated 10 June 1998, applies, the Contractor shall prepare a Safety and Health Protection Plan (SIGE) as required under that regulation. The SIGE shall be submitted two weeks prior to mobilization. In addition, the Contractor shall appoint a qualified Safety Coordinator who will ensure that safety measures are implemented during construction. The Safety Coordinator must meet minimum training and experience requirements required under Host Nation laws.

(c) The SIGE will be written in a comprehensive form and explained in language the workers understand and shall contain the following minimum information:

(i) Maintenance of tools and equipment.

(ii) Provisions for the storage and disposal of materials and waste, particular hazardous materials and substances.

(iii) Modifying performance periods for such work, taking into consideration the site conditions.

(iv) Organizing cooperation between different employers and Contractors without employees.

(v) Interaction between work at the site and other operational activities in the area, on which or near which the first identified work will be performed, and taking into consideration the instructions and advice of the coordinator as well as safety and health plans.

1.21.1.2  The Contractor shall complete and provide safety notifications required by host nation authorities and will provide documented proof to the Contracting Officer via the COR, no later than 10 working days (Deliverable #18) that such notifications were made.

1.22   Environmental Compliance:  The following provides information on Environmental Compliance.

1.22.1   Environmental Compliance Plan and Program: The Contractor shall ensure all heavy metals, lead, ammunition residue, and unserviceable granulated rubber material resulting from live fire operations on ranges maintained herein shall be removed and disposed of in accordance with applicable European Host Nation hazardous material environmental laws/standards. Additionally, it is the responsibility of the Contractor to possess the proper permits for transporting and disposing of lead, heavy metal contaminates, and contaminated rubber granulated materials, and ensure contract personnel handling or disposing of hazardous materials have received appropriate levels of training in the handling, use, storage, and disposal of hazardous material. The Government shall not be held responsible nor liable for Contractor’s negligence, or inability to obtain required host nation certifications for transport of disposal of hazardous materials. In addition, the Contractor shall ensure that all Contractor controlled activities on US installations where ranges are supported are conducted in accordance with the installation Spill Prevention and Response Plan, Solid Waste Policy (Deliverable #19).

The Contractor shall ensure that all of its operations and activities conform to the USAG Stuttgart Hazardous Waste Management Plan. The Contractor shall ensure that all contract personnel have received appropriate levels of training in the handling, use, storage, and disposal of hazardous material. In addition, the Contractor shall ensure that all Contractor controlled activities on USAG Stuttgart installations are conducted in accordance with the USAG Stuttgart Spill Prevention and Response Plan, Solid Waste Management Plan, Final Governing Standards-Germany, and applicable German environmental laws/standards.

Prior to contract completion, the Contractor shall remove all hazardous materials and dispose of all hazardous waste (HW) generated in accordance with Final Governing Standards Germany and applicable German environmental laws/standards.  A copy of the HW disposal documents must be forwarded to the HW program manager at the DPW Environmental Division.

The Contractor shall clean up at its own expense any contamination, including soil and ground water contamination caused by hazardous materials/substances brought onto the garrison, or by his activities. The Contractor must immediately report any hazardous substance spills to the Garrison Fire Department at CIV 0711-729-112.

The Contractor shall contact the COR or the USAG Stuttgart Directorate of Public Works-Environmental Division (DPW-ED), DSN 596-6135, CIV 09641-70596-6135, Bldg. 3318, RM 216, Kelley Barrcaks for a copy of the Garrison Hazardous Waste Management Plan, Spill Prevention and Response Red Plan, Garrison Solid Waste Management Plan and German Final Governing Standards.

1.23   Required Training:  The following provides information on training requirements.

1.23.1   Anti-Terrorism (AT) Level I Training: All Contractor employees, including subcontractor employees, who are employed under the contract, shall complete AT Level I Training within 10 working days(Deliverable #3) after contract start date or effective date of incorporation of this requirement into the contract, whichever applies, or 10 working days after employment of new personnel.  Contractor personnel travelling overseas shall complete area of responsibility (AOR) specific AT awareness training for U.S. based Contractor employees and associated subcontractor employees.  The Contractor shall submit certificates of completion (Deliverable #20) for each affected Contractor employee and subcontractor employee to the COR (or to the Contracting Officer, if a COR is not assigned) within 2 working days after completion of this training by all employees and subcontractor personnel.  AT Level I awareness training is available at https://jkodirect.jten.mil.  Contractor personnel shall complete refresher training (Deliverable #21) every twelve (12) months except AOR specific AT Level I training which shall be completed as required by local AT policies

If training cannot be conducted on the website, the Contractor shall coordinate with the COR who will contact the local AT Officer at their assigned location for classroom training.  Contractor personnel completing training in the classroom will receive a certificate or the training attendance roster signed by the AT Officer. Verification of the training shall be provided to the COR within 2 working days after completion of the training (Deliverable #22).

1.23.2   OPSEC Training: All Contractor employees, including subcontractors, shall complete Level I OPSEC training within 10 working days of employment under this contract (Deliverable #23).  Verification of the training shall be provided to the COR within 2 working days after completion of the training (Deliverable #24).  OPSEC Level I training is available at http://www.cdse.edu/catalog/elearning/GS130.html.  Contractor personnel must complete refresher training every 12 months (Deliverable #25).  Verification of the training shall be provided to the COR within 2 working days after completion of the training (Deliverable #26).

If training cannot be conducted on the website, the Contractor shall coordinate with the COR, who will contact the local OPSEC Officer at their assigned location for classroom training.  Contractor personnel completing training in the classroom will receive a certificate or the training attendance roster signed by the OPSEC Officer.  Verification of the training shall be provided to the COR within 2 working days after completion of the training (Deliverable #27).

1.23.3   iWATCH Training: The Contractor and all associated subcontractors shall brief all employees on the local iWATCH program (training standards provided by the requiring activity ATO).  This locally developed training will be used to inform employees of the types of behavior to watch for and instruct employees to report suspicious activity to the COR. This training shall be completed within 10 working days after the contract is awarded and within 10 working days after new employees commence contract performance, with the results reported to the COR within 2 working days after the contract is awarded and within 2 working days after new employees commence contract performance (Deliverable #28).  Verification of the training shall be provided to the COR within 2 working days after completion of the training (Deliverable #29).  Training slides can be found on the Army OneSource we site:

http://www.myarmyonesource.com/familyprogramsandservices/iwatchprogram/default.aspx.

1.23.4   Information Assurance (IA) Training: RESERVED

1.23.5  Personally Identifiable Information (PII): RESERVED

1.24  Authorization to Perform Services in Germany

1.24.1 Contractors performing services in the Federal Republic of Germany (FRG) shall comply with German law.  The Contractor shall determine whether performance requires registration with German authorities or authorization to do business in Germany and, if so, shall comply with all requirements.  Whether or not registration or authorization to do business is required, the Contractor shall determine what documents or authorization its employees and any subcontractor employees must possess to work in Germany.  The Contractor shall ensure affirmatively that its employees and subcontractor employees possess such documents or authorizations.

1.24.2 Contractor employees who: (a) are not nationals of Germany or other European Union countries; (b) are not members of the force, the civilian component or their dependents; or c) do not have assimilated status under Articles 71, 72, or 73 of the Supplementary Agreement to the NATO SOFA shall possess work and residence permits.

1.24.3 By acceptance of and performance under this contract, the Contractor affirms that it has complied with the requirements above.  Compliance with this requirement and German law is a material contract requirement.  Noncompliance by the Contractor or subcontractor at any tier shall be grounds for issuing a negative past performance evaluation and terminating this contract for default.

1.25  Notice to Proceed: The notice to proceed will be provided as indicated below. (Contracting Office must identify one)

The effective date of contract will constitute the date of notice to proceed

A notice to proceed letter will be issued

The date of the letter transmitting the contract will constitute the date of the notice to proceed

1.26   Electromagnetic Compatibility Directive: The Contractor certifies that all electrical and electronic equipment and appliances furnished are in compliance with the Electromagnetic Compatibility Directive 2014/30/EU.  For radio transmitting and telecommunication devices furnished in the Federal Republic of Germany, the Contractor certifies that all such items have been approved by the Bundesamt fuer Zulassungen.  Any penalties for noncompliance imposed by the Government of the European country where the equipment or appliances are delivered or product liability claims caused by interference generated by furnished appliances or equipment are solely the responsibility of the Contractor.

1.27  Government Property and Services:  RESERVED

1.28   Conventional Forces Europe (CFE) Treaty Verification Inspection:  Contractor-owned or controlled structures and containers located on U.S. Government installations are subject to short-notice inspections by CFE treaty inspection teams.  The Government will endeavor to provide the Contractor as much notice as possible in the event of such inspections. The contractor shall provide access to such structures and containers at the request of the Government.  The Contractor shall report such inspections to the KO through the COR in writing within ten (10) calendar days of completion of the inspection.  Contractor-owned or controlled structures and/or containers on U.S. installations may have to be inspected to comply with the CFE treaty even if Contractor personnel are unavailable to provide access.  Any costs borne by the Contractor or any impact on contract performance as a result of the inspection will be a matter of negotiation between the Contractor and the KO.

PART 2

DEFINITIONS & ACRONYMS

2.  Definitions and Acronyms

2.1   Definitions:  Although not inclusive of every term used within this PWS, the following provides a list of definitions used throughout this PWS and commonly used in the acquisition field.

Contracting Officer – means a person with authority to enter into, administer, and or terminate contracts, and make related determinations and findings on behalf of the Government.  Note: The only individual who can legally bind the Government.

Contracting Officer's Representative (COR) – As defined in DFARS 202.101, means an individual designated and authorized in writing by the Contracting Officer to perform specific technical or administrative functions.  Department of Defense Instruction (DoDI) 5000.72, Part II Definitions states the following when defining a COR:  “Defined in subpart 202.101 of Reference (f).  Any individual delegated responsibilities pursuant to subpart 1.602-2 of Reference (e), regardless of local terminology, must be certified in accordance with this instruction.  For example, local terminology can be COR, contracting officer’s technical representative, technical point of contact, technical representative, alternate COR, administrative COR, assistant COR, line item manager, task order manager, quality assurance personnel, quality assurance evaluator, or COR management.”  In addition, Army Regulation 70-13, Chapter 2, paragraph 2-2g, states, in part, the following when providing support personnel to assist the COR, “…These other surveillance support personnel may serve as on-site representatives of the COR in performance of actual contract surveillance if they meet all COR requirements and have been appointed by the Contracting Officer as alternate CORs.”

Contractor – means a supplier or vendor awarded a contract to provide specific supplies or service to the Government.  The term used in this contract refers to the prime.

Contractor-acquired property - means property acquired, fabricated, or otherwise provided by the Contractor for performing a contract, and to which the Government has title.

Day – means, unless otherwise specified, a calendar day.

Defective Service – means service output that does not meet the standard of performance associated with the Performance Work Statement.

Deliverable – means anything that can be physically delivered, but may include non-manufactured things such as meeting minutes or reports.

Government-furnished property - means property furnished to the contractor the Government and subsequently furnished to the Contractor for performance of a contract. Government-furnished property includes, but is not limited to, spares and property furnished for repair, maintenance, overhaul, or modification. Government-furnished property also includes Contractor-acquired property if the Contractor-acquired property is a deliverable under a cost contract when accepted by the Government for continued use under the contract.

Government property - means all property owned or leased by the Government. Government property includes both Government-furnished property and Contractor-acquired property. Government property includes material, equipment, special tooling, special test equipment, and real property. Government property does not include intellectual property and software.

Property Administrator - means an authorized representative of the Contracting Officer appointed in accordance with agency procedures, responsible for administering the contract requirements and obligations relating to Government property in the possession of a Contractor.

High Level Objective (HLO) – means a key overarching result-based objective for a project necessary to achieve the project’s vision. HLOs are similar to Level 2 in a Work Breakdown Structure.  Each HLO may contain several statements to flesh out the areas necessary to meet the objective.

Physical Security – means that part of security concerned with physical measures designed to safeguard personnel; to prevent unauthorized access to equipment, installations, material, and documents; and to safeguard against espionage, sabotage, damage, and theft.

Quality Assurance – (or Government contract quality assurance) means the various functions, including, inspection, performed by the Government to determine whether a Contractor has fulfilled the contract obligations pertaining to quality and quantity.

Quality Assurance Surveillance Plan (QASP) – means the key Government-developed surveillance process document, and is applied to Performance-Based Service Contracting (PBSC). The QASP is used for managing Contractor performance assessment by ensuring that systematic quality assurance methods validate that Contractor quality control efforts are timely, effective, and are delivering the results specified in the contract or task order. The QASP directly corresponds to the performance objectives and standards (i.e., quality, quantity, timeliness) specified in the Performance Work Statement (PWS). It provides specific details on how the Government will survey, observe, test, sample, evaluate, and document Contractor performance results to determine if the Contractor has met the required standards for each objective in the PWS.  The QASP, with very few if any exceptions, is an internal to Government document.

Quality Control – means all necessary measures taken by the Contractor to assure that the quality of an end product or service shall meet contract requirements.

Statement – means the specific results-based activities required to satisfy HLOs. A statement contains a result, the context of the statement, and the required action(s). Statements focus on “what” is to be accomplished; however they are not prescriptive in describing “how” the outcome is to be achieved. Each HLO may have several statements to flesh out the areas necessary to meet the objective. Statements are similar to Level 3 in a Work Breakdown Structure.

Subcontractor – means one that enters into a contract with a prime Contractor.  The Government does not have privity of contract with the subcontractor.

Work Day - The number of hours per day the Contractor provides services in accordance with the contract.

Work Week - Monday through Friday, unless otherwise specified.

2.2   Acronyms:  Although not inclusive of every term used within this PWS, the following provides a list of acronyms used throughout this PWS or commonly used in the acquisition field.

ACOR			Alternate Contracting Officer's Representative

AE 			Army in Europe

AFARS			Army Federal Acquisition Regulation Supplement

AHA			Activity	Hazard Analysis

AOR			Area of Responsibility

AR			Army Regulation

AT			Anti-terrorism

ATCTS			Army Training Certification Tracking System

CAC			Common Access Card

CCE			Contracting Center of Excellence

CFE 			Conventional Forces Europe

CFR			Code of Federal Regulations

CONUS			Continental United States (excludes Alaska and Hawaii)

COR			Contracting Officer’s Representative

COTS			Commercial-Off-the-Shelf

DA			Department of the Army

DD250			Department of Defense Form 250 (Receiving Report)

DD254			Department of Defense Contract Security Requirement List

DFARS			Defense Federal Acquisition Regulation Supplement

DMDC			Defense Manpower Data Center

DoD			Department of Defense

FAR			Federal Acquisition Regulation

FRG			Federal Republic of Germany

GFP			Government Furnished Property

HLO			High Level Objective

HIPAA			Health Insurance Portability and Accountability Act of 1996

IA			Information Awareness

IAC			Installation Access Control

IACO			Installation Access Control Office

ID			Identification

IGCE			Independent Government Cost Estimate

IT			Information Technology

JTR			Joint Travel Regulation

JPAS			Joint Personnel Adjudication System

KO			Contracting Officer

OCI			Organizational Conflict of Interest

OCONUS		Outside Continental United States (includes Alaska and Hawaii)

ODC 			Other Direct Costs

OPSEC			Operations Security

PA			Property Administrator

PII			Personally Identifiable Information

PIPO			Phase In/Phase Out

POC			Point of Contact

PRS			Performance Requirements Summary

PWS			Performance Work Statement

QA			Quality Assurance

QAP			Quality Assurance Program

QASP			Quality Assurance Surveillance Plan

QC			Quality Control

QCP			Quality Control Program

RA			Requiring Activity

RCO			Regional Contracting Office

RTSD-W		Regional Training Support Division-West

SIGE			Safety and Health Protection Plan

TE			Technical Exhibit

TSAE			Training Support Activity Europe

TSC			Training Support Center

USAREUR		U.S. Army Europe
UOM 			Unit of Measure

PART 3

GOVERNMENT PROPERTY (GP) AND SERVICES

3.  Government Property and Services

3.1   Services: The Government (TSC/RTSD-West) will provide the coordination of Installation Access Control measures to ensure contractor personnel are able to access the site as needed during the performance of work associated with contract requirements.

3.2   Facilities: The Government will provide bathroom facility as needed when available on-site during the performance of work.

3.3   Utilities: The Government will provide electrical outlets when available on-site as required during the scope of work. The Contractor shall instruct employees in utilities conservation practices.  The Contractor shall operate under conditions that preclude the waste of utilities, which include turning off the water faucets or valves, light switches, etc. after using the required amount to accomplish tasks requiring the use of the utilities.

3.4   Equipment: RESERVED

3.5   Special Tooling and Test Equipment: RESERVED

3.6   Materials: The Government will provide no additional materials unless pre-positioned stock is on-site for required work to reduce additional costs.

3.7   Training: The Government will provide the following training in the event that previously prescribed methods are unavailable: AT Level 1 Awareness, OPSEC, iWATCH, etc.

3.8  Common Access Cards (CAC): RESERVED

PART 4

CONTRACTOR FURNISHED ITEMS AND SERVICES

4.  Contractor Furnished Property and Services

4.1  General: The Contractor shall furnish all personnel, equipment, supplies, facilities, transportation, tools, materials, supervision, and other items necessary to perform services as described in the PWS and associated contract documents, except for those items specified in Part 3 Government Property and Services.

4.2   Secret Facility Clearance: RESERVED

4.3   Materials: The Contractor shall provide any and all material such as GranTex or similar rubber granulate, sand, dura block, rubber covering material, or wood needed to replenish the bullet catch to design standards that mitigate ballistic safety hazards. The Contractor shall also provide any Personal Protective Equipment, PPE, Office supplies and materials such as, paper, printer cartridges, and all other office supplies and materials needed in the performance of this contract; automated data processing equipment (ADPE) such as computers, copy, printer, and fax machines. All ADPE connecting to the Government network shall be reviewed and approved by the Government prior to purchase by the Contractor; office furniture; cellular and desktop telephones, etc. The Contractor may request a letter of authorization from the Contracting Officer to utilize Government supply sources such as Government Services Administration (GSA), UNICOR, etc.

4.4   Vehicles and Equipment: The Contractor shall provide all necessary vehicles (no GSA vehicles provided to Contractor for use on this contract) and equipment to perform the work described in this PWS. All Contractor provided vehicles and equipment shall have the company name prominently displayed on both sides of the vehicle or equipment and present a neat, professional appearance. Contractor's vehicles and equipment used in the performance of this contract shall meet all local and Host Nation traffic, safety, and environmental requirements. Vehicles and equipment found to be unsafe shall be removed from the installation and replaced. The Contractor shall not use any Government-owned tools, materials, or parts to maintain Contractor vehicles and equipment without prior written approval of the Contracting Officer. The COR may inspect the Contractor’s vehicles and equipment at any time.

4.5  Background Checks):  The Contractor shall provide Background Checks in accordance with this PWS and as determined by the Government.

PART 5

SPECIFIC TASKS

5.1   Basic Services:  The Contractor shall provide services for the complete material exchange of range 7 at the Panzer Range Complex identified in this PWS by performing the removal of all material, lead remediation, inspection for damage, and the complete replacement of material ensuring that the bullet catch has a minimum depth of 1 meter of impact media.

5.2.  Tasks

5.2.1  Contractor Resources Management

5.2.1.1 Work Order Reception:  Contractor shall establish a Work Reception System for tracking all requests for recurring and preventative maintenance work or services and maintaining active and historical workload records.

a) STD: The Work Reception Desk and telephone shall be attended continuously during normal working hours including lunchtime. After duty hours emergency coverage shall be provided 24/7. Work Reception shall be responsible for all functions IAW Department of the Army Pamphlet (DA PAM) 420-6 Directorate of Public Works (DPW) Resource Management System (RMS).

AQL: 100% Compliance, Zero Deviation from standard.

b) STD: The Work Reception Desk shall assign classification to orders upon receipt IAW priority definitions in Part 2 of the PWS.

AQL: 95% Compliance; No more than 5 deficiencies per month.

5.2.1.2 Contractor Project Management. Contractor shall manage all assigned projects from initial receipt until project completion. The Contractor shall ensure all projects are completed IAW the Contractor's approved schedule and project cost as noted below.

a) STD:  The Contractor shall complete the project within +/- 5% of the approved schedule completion date.

AQL:  100% Compliance, Zero Deviation from standard.

b) STD:  The Contractor shall complete the project within +/- 5% of the initial detailed cost estimate approved by 7ATC, TSAE.

AQL:  100% Compliance, Zero Deviation from standard.

5.2.2  Bullet Catch Material Exchange

5.2.2.1  Material Removal and Replacement: The Contractor shall remove all sand/rubber granulate to separate fired projectiles (i.e. lead, copper, or other metal alloys) from the impact media.

a) STD: The Contractor shall prepare range floor in a manner that prevents the removed impact media from mixing with the existing range floor substrate.

AQL: 100% Compliance, Zero Deviation from standard.

b) STD: The Contractor shall properly dispose of all removed substances as outlined by Host Nation Hazardous Material control measures and Garrison Environmental Policy.

AQL: 100% Compliance, Zero Deviation from standard.

c) STD: The Contractor shall inspect interior of the bullet catch walls to identify damages that may need to be corrected.

AQL: 100% Compliance, Zero Deviation from standard. (Range staff will be present when conducting inspection.)

d) STD: The Contractor shall replace the removed impact media with equal or better new material adhering to industry standards maintaining at least a 1 meter ballistic rubber granulate depth sloped at a 45 degree reclining surface angle.

AQL: 100% Compliance, Zero Deviation from standard.

e) STD: The Contractor shall install rubber matting on the surface of the ballistic rubber granulate to hinder the material from shifting downward causing spillage onto range floor by affixing it to the front and back walls of the bullet catch with material that will not cause a ricochet or errant projectile.

AQL: 100% Compliance, Zero Deviation from standard.

5.3.  Service Contract Reporting

5.3.1 System for Award Management (SAM) Service Contract Report (SCR): The Contractor shall report ALL Contractor labor hours (including subcontractor labor hours) required for performance of services provided under this contract for the 7ATC, Training Support Activity Europe (TSAE), Range and Training Land Program (RTLP) Range Maintenance Support and Lead Remediation via a secure data collection site. The Contractor is required to completely fill in all required data fields through the following web address: www.SAM.gov. Reporting inputs will be for the labor executed during the period of performance during each Government FY, which runs from October 1 through September 30. While inputs may be reported any time during the FY, all data shall be reported no later than October 31 of each calendar year. Contractors may direct questions to the help desk by clicking “View Assistance for SAM.gov” which is located at the top of the SAM.gov website. From there, you can the select “Contact Our Service Desk” which will allow you to contact SAM.gov directly. If contract period of performance ends prior to September 30, the Contractor has 30 calendar days from end date of the contract to complete the SAM SCR requirement.

Steps for Submitting a Service Contract Report (SCR)

- Go to www.sam.gov and log in.

- Select Entity Registrations and then select Service Contract Reporting.

- SAM displays your entities which have service contracts and meet the reporting criteria. Select View by entity to see the service contracts for each entity.

- Next, select Add for the service contract against which you want to create a Service Contract Report. Each service contract which meets the FAR Subpart 4.1703 reporting thresholds is displayed.

- You will be taken to the Complete Service Contract Report page. SAM displays the contract details and allows you to report. You are required to enter the following information:

- Total Amount Invoiced: Total dollar amount invoiced for services performed during the previous Government fiscal year under the contract (this amount should include the prime and any subcontract amount).

- Prime Contractor Hours Expended: Prime contractor direct labor hours expended on the services performed during the previous Government fiscal year. The amount you enter is automatically divided by 2,080 hours to calculate a Full Time Employee (FTE) equivalent, displayed under the Prime Contractor Hours Expended as Prime Contractor FTEs.

- Report any required Tier 1 subcontractor information by selecting the Add Tier 1 Subcontract Information button.

- When you are ready to submit the report, select Submit. This saves your report and returns you to the Select Service Contract page where you can create other SCRs or edit an existing SCR.

Steps for Editing a Submitted Report

- Follow steps 1-4 above. Any previously submitted SCR will have a View/Edit button instead of an Add button.

- Select View/Edit. You will be able to view the current SCR, edit available fields, and resubmit the record. You can also delete previously entered information altogether.

STD:  All information provided by the Contractor shall be accurate, complete, and not exceed suspense dates noted in corresponding paragraph above.

AQL: 100% Compliant

(Deliverable #30)

PART 6

APPLICABLE PUBLICATIONS

6.   Applicable Publications (Current Editions):  The following publications, manuals, regulations, etc. are mentioned in this PWS and are listed below.

6.1.1	Department Of Defense Contract Security Classification Specification (DD Form 254)

6.1.2	Department of Defense Security Agreement (DD Form 441)

6.1.3	National Industrial Security Program Operating Manual (DoD 5220.22-M)

6.1.4	Installation Access Control (AE Reg 190-16)

6.1.5	The Army Physical Security Program (AR 190-13)

6.1.6	Contractor Identification (AE Reg 27-715)

6.1.7	U.S. Army Corps of Engineers Safety and Health Requirements Manual (EM 385-1-1)

6.1.8	Information Assurance Workforce Improvement Program (DoD 8570-M)

6.1.9	Information Assurance Training Certification and Workforce Management Directive (DoDD 8570.01)

6.1.10	Information Assurance (AR 25-2)

6.1.11	Electromagnetic Compatibility Directive (2004/108/EC)

6.1.12	Policies and Procedures for Property Accountability (AR 735-5)

6.1.13	The Army Sustainable Range Program (AR 350-19)

6.1.14	Local National Screening Program in Germany (AER 604-1)

6.1.15	Baustellenverordnung (BaustellV) – German Safety and Health Protection Regulation for Construction Sites

6.1.16	Raange Safety (AR 385-63)

6.1.17	Unified Facilities Criteria-Small Arms Ranges (UFC 4-179-02)

6.1.18	Bullet Trap User’s Guide-U.S. Army Environmental Center

PART 7

ATTACHMENT AND TECHNICAL EXHIBIT LISTING

7.  Attachment and Technical Exhibit List

7.1  Technical Exhibit A – Performance Requirements Summary

7.2  Technical Exhibit B – Deliverables Schedule

7.3  Attachment 1 – Holiday List

TECHNICAL EXHIBIT A

Performance Requirements Summary (PRS)

| PERFORMANCE OBJECTIVE | STANDARD | PERFORMANCE THRESHOLD ACCEPTABLE QUALITY LEVELS (AQLs) | METHOD OF SURVEILLANCE |
| --- | --- | --- | --- |
| PRS # 1 - PWS Para. 5.2.1.1 Contractor shall establish a Work Reception System for tracking all requests for recurring and preventative maintenance work or services and maintaining active and historical workload records. | The Work Reception Desk and telephone shall be attended continuously during normal working hours including lunchtime. After duty hours emergency coverage shall be provided 24/7. Work Reception shall be responsible for all functions IAW Department of the Army Pamphlet (DA PAM) 420-6 Directorate of Public Works (DPW) Resource Management System (RMS). | 100% Compliance, Zero deviation from standard | Periodic Surveillance, Customer Feedback |
|  | The Work Reception Desk shall assign classification to orders upon receipt IAW priority definitions in Part 2 of the PWS. | 95% Compliance; No more than 5 deficiencies per month | Periodic Surveillance, Customer Feedback |
| PRS # 2 - PWS Para. 5.2.1.2 Contractor shall manage all assigned projects from initial receipt until project completion. The Contractor shall ensure all projects are completed IAW the Contractor's approved schedule and project cost as noted below. | The Contractor shall complete the project within +/- 5% of the approved schedule completion date. | 100% Compliance, Zero deviation from standard | 100% Inspection |
|  | The Contractor shall complete the project within +/- 5% of the initial detailed cost estimate approved by 7ATC, TSAE. | 100% Compliance, Zero deviation from standard | 100% Inspection |
| PRS # 3 - PWS Para. 5.2.2.1 The Contractor shall remove all sand/rubber granulate to separate fired projectiles (i.e. lead, copper, or other metal alloys) from the impact media. | The Contractor shall prepare range floor in a manner that prevents the removed impact media from mixing with the existing range floor substrate. | 100% Compliance, Zero deviation from standard | Periodic Inspection |
|  | The Contractor shall properly dispose of all removed substances as outlined by Host Nation Hazardous Material control measures and Garrison Environmental Policy. | 100% Compliance, Zero deviation from standard | 100% Inspection |
|  | The Contractor shall inspect interior of the bullet catch walls to identify damages that may need to be corrected. | 100% Compliance, Zero deviation from standard (Range Staff will be present when conducting inspection.) | 100% Inspection, Customer Feedback |
|  | The Contractor shall replace the removed impact media with equal or better new material adhering to industry standards maintaining at least a 1-meter ballistic rubber granulate depth sloped at a 45-degree reclining surface angle. | 100% Compliance, Zero deviation from standard. | 100% Inspection |
|  | The Contractor shall install rubber matting on the surface of the ballistic rubber granulate to hinder the material from shifting downward causing spillage onto range floor by affixing it to the front and back walls of the bullet catch with material that will not cause a ricochet or errant projectile. | 100% Compliance, Zero deviation from standard. | 100% Inspection |
| PRS #X - Service Contract Reporting (SCR) PWS 5.3 and CLIN XXXX | The Contractor shall completely fill in all the information in the format using the following web address www.sam.gov. The contractor shall provide the estimated total cost (if any) incurred to comply with this reporting requirement. Reporting period shall be the period of performance not to exceed 12 months ending September 30 of each government fiscal year and must be reported by 31 October of each calendar year. If contract period of performance ends prior to September 30, the contractor has 30 days from end date of the contract to complete the SCR requirement. | Zero Deviation from Standard | 100% Inspection |

TECHNICAL EXHIBIT B

DELIVERABLES SCHEDULE

| Deliverable | Frequency | # of Copies | Medium/Format | Submit To |
| --- | --- | --- | --- | --- |
| 1. Quality Control Plan (paragraph 1.5.1) | Once within 10 days after contract award and no later than 30 days prior to any anticipated changes. | 1 | Electronic delivery via email using PDF | To the Contracting Officer through the Contracting Officer’s Representative (COR) |
| 2. Analysis of Quality Control Plan (paragraph 1.5.1.1) | Once within 2 working days after contract award | 1 | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 3. Written Potential Service Issues (paragraph 1.5.1.2) | As needed within 1 working day upon awareness and verbal notification | 1 | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 4. Installation Pass Log (paragraph 1.6.3.1) | Once within 3 calendar days after contract award | 1 | Electronic delivery via email using PDF | To the Contracting Officer through the Contracting Officer’s Representative (COR) |
| 5. Installation Pass Log Chnages (paragraph 1.6.3.1) | As needed within 5 working days of changes | 1 | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 6. Collected Installation Passes-Staff changes (paragraph 1.6.3.2) | As needed during staff/personnel updates or changes occur | 1 each for all contractor staff as applicable | Original issued item via hand delivery | To the Issuing Office |
| 7. Collected Installation Passes-Contract End/Termination (paragraph 1.6.3.3) | Once within 2 working days after contract end or termination | 1 each for all contractor staff as applicable | Original issued item via hand delivery | To the Issuing Office |
| 8. U.S. Citizen Background Check-Favorable (paragraph 1.6.4.1) | Once within 2 working days after contract award | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer through the Contracting Officer’s Representative (COR) |
| 9. U.S. Citizen Background Check-Unfavorable (paragraph 1.6.4.1) | Once within 2 working days after contract award | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer through the Contracting Officer’s Representative (COR) |
| 10. Polizeiliches Fuehrungszeugnis-Favorable (issued within 4 months) (paragraph 1.6.4.2) | Within 5 working days after contract award | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer through the Contracting Officer’s Representative (COR) |
| 11. Polzeiliches Fuehrungszeugnis-Unfavorable (paragraph 1.6.4.2) | Within 24 hours of receipt | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer through the Contracting Officer’s Representative (COR) |
| 12. Contractor Personnel List (paragraph 1.10) | Within 5 working days after contract award | 1 | Electronic delivery via email using PDF | To the Contracting Officer |
| 13. Contractor Workspace Signage (paragraph 1.12.1) | Within 5 working days after contract award | Varies | Electronic delivery via email using PDF | To the Contracting Officer through the Contracting Officer’s Representative (COR) |
| 14. Contractor Identification Badge List with example (paragraph 1.12.2) | Once prior to the start of work | 1 | Electronic delivery via email using PDF | To the Contracting Officer |
| 15. Non-Disclosure Agreements (paragraph 1.18) | Within 14 calendar days prior to commencement of contract | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the U.S. Government |
| 16. Organizational Conflict of Interest Notification (paragraph 1.19) | Immediately upon notification of conflict of interest | 1 for every occurrence upon awareness | Electronic delivery via email using PDF | To the Contracting Officer |
| 17. Host Nation Safety- Accident Prvention Plan (paragraph 1.21.1a) | Prior to start of work | 1 | Electronic delivery via email using PDF | To the designated Government personnel (Garrison DPW Environmental) |
| 18. Safety Notifications (paragraph 1.21.1.2) | Within 10 working days upon original notification | Varies as needed | Electronic delivery via email using PDF | To the Contracting Officer |
| 19. Environmental Compliance Plan (paragraph 1.22.1) | Once prior to start of work | 1 | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 20. Antiterrorism Level 1 Training Certificates (paragraph 1.23.1) | Once within 10 working days after contract start date | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 21. Antiterrorism Level 1 Training Refresher Certificates (paragraph 1.23.1) | Once within 2 working days every 12 months | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 22. Antiterrorism Level 1 Training In-person Certificates (paragraph 1.23.1) | Once within 2 working days after completion | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 23. Operations Security (OPSEC) Training Certificates (paragraph 1.23.2) | Once within 10 working days after completion | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 24. Operations Security (OPSEC) Training Certificates Verification (paragraph 1.23.2) | Once within 2 working days after completion | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 25. Operations Security (OPSEC) Refresher Training Certificates (paragraph 1.23.2) | Once within 2 working days after completion | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 26. Operations Security (OPSEC) Refresher Training Certificates Verification (paragraph 1.23.2) | Once within 2 working days after completion | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 27. Operations Security (OPSEC) In-person Training Certificates (paragraph 1.23.2) | Once within 2 working days after completion | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 28. iWatch Training Certificate (paragraph 1.23.3) | Once within 10 working days after completion | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 29. iWatch Training Certificate Verification (paragraph 1.23.3) | Once within 2 working days after completion | 1 each for all contractor staff as applicable | Electronic delivery via email using PDF | To the Contracting Officer’s Representative (COR) |
| 30. Simplified Acquisition Management (SAM) Service Contract Report (SCR) (paragraph 5.3.1) | Once upon contract completion or NLT 30 calendar days after Fiscal Year | 1 | Electronic submission via www.SAM.gov | To the Contracting Officer through www.SAM.gov |

ATTACHMENT 1

HOLIDAY LIST

U.S Federal holidays are:

New Year’s Day 					January 1st

M. L. King Memorial Day 			3rd Monday in January

Presidents Day 					3rd Monday in February

Memorial Day 					last Monday in May

Independence Day 				July 4th

Labor Day 					1st Monday in September

Columbus Day 					2nd Monday in October

Thanksgiving Day 				4th Thursday in November

Veteran’s Day 					November 11th

Christmas Day 					December 25th

German national and local holidays are:

1st January	New Year’s Day (Neujahr)

6th January	Three King’s Day (Heilige Drei Könige)

Varies yearly	Good Friday (Karfeitag)

Varies yearly	Easter Monday (Ostermontag)

1st May	Labor Day (Tag der Arbeit)

Varies yearly	Ascension Day (Christi Himmelfahrt)

Varies yearly	Whit Monday (Pfingstmontag)

Varies yearly	Corpus Christi (Fronleichnam)

15th August	Assumption Day (Mariä Himmelfahrt)

3rd October	Day of German Unity (Tag der Deutschen Einheit)

1st November	All Saints’ Day (Allerheiligen)

25th December	1st Christmas Day (1. Weihnachtsfeiertag)

26th December	2nd Christmas Day (2. Weihnachtsfeiertag)