99th Readiness Division, Directorate of Public Works

Environmental Division

PERFORMANCE WORK STATEMENT

FY26 Stormwater Compliance Project Sampling and Monitoring

13 April 2026

# INTRODUCTION

The 99th Readiness Division (RD) Directorate of Public Works (DPW) requires professional services and technical support to complete stormwater sampling and monitoring at one (1) facility in Maryland, two (2) facilities in Massachusetts, one (1) facility in New Hampshire, four (4) facilities in New York, and one (1) facility in Pennsylvania.  The 99th RD does not have staffing or in-house subject matter expertise required to ensure compliance with State mandated permit requirements.

# BACKGROUND

The 99th RD DPW Environmental Division provides regional environmental support for facilities located in thirteen (13) states along the east coast that comprise the 99th RD’s Area of Responsibility (AOR). The 99th RD has an environmental support team at headquarters, located at Joint Base McGuire-Dix-Lakehurst (JBMDL), NJ, and Regional Environmental Protection Specialists (REPS) located throughout the AOR who serve as the Point of Contact (POC) at each facility.

The 99th RD has one (1) facility in Maryland under the Maryland Department of the Environment (MDE) General Permit for Discharges from Stormwater Associated with Industrial Activities.  The facility is currently inactive, because industrial activity is not occurring, but the sampling required under the permit is conducted in case industrial activities resume at the facility in the future.  This permit requires a comprehensive sampling program which includes analytical stormwater sampling, monitoring, and associated reporting to MDE.

The 99th RD has two (2) facilities in Massachusetts and one (1) facility in New Hampshire under the Environmental Protection Agency (EPA) Multi Sector General Permit for Discharges of Stormwater Associated with Industrial Activities (MSGP). This permit requires the 99th to conduct a comprehensive sampling program which includes analytical stormwater sampling, monitoring, and associated reporting to EPA.

The 99th RD has four (4) facilities in New York regulated under the New York State Department of Environmental Conservation’s (NYSDEC) Multi-Sector General Permit for Stormwater Discharges Associated with Industrial Activity (MSGP). The MSGP requires the 99th RD to conduct a comprehensive sampling program which includes analytical stormwater sampling, monitoring and associated reporting to NYSDEC.

The 99th RD has one (1) facility in Pennsylvania under the Pennsylvania Department of Environmental Projection (PADEP) General Permit for Discharges of Stormwater Associated with Industrial Activities (PAG-03). This permit requires the 99th to conduct a comprehensive sampling program which includes analytical stormwater sampling, monitoring, and associated reporting to PADEP.

The option to use representative outfalls is available for some facilities and that information is noted in tables 3.1-3.5. If there are two (2) or more regulated outfalls at a facility that produce almost identical discharges based on the type of industrial activities occurring within the drainage area, the contractor may test from what is considered the, “worst case,” outfall and use the results as representative data for the almost identical outfalls. The contractor shall verify this prior to conducting monitoring.

# SCOPE

## Facilities

Sampling and monitoring programs are required at the facilities listed below in tables 3.1-3.5:

Table 3.1. Maryland Facility

| FACID | Facility | Address | Rep Samples Allowed | Min # of OF’s to be Sampled | Access Needed for Site | REPS Name |
| --- | --- | --- | --- | --- | --- | --- |
| MD024 | Curtis Bay AMSA #83 (M) | 720 E. Ordnance Rd Baltimore, MD 21226 | No | 1 | Yes | Trey Shope |

Table 3.2. Massachusetts Facilities

| FACID | Facility | Address | Rep Samples Allowed | Min # of OF’s to be Sampled | Access Needed for Site | REPS Name |
| --- | --- | --- | --- | --- | --- | --- |
| MA058 | MAJ David Connelly AFRC | 115 Barnum Rd Ayer, MA 01432 | No | 1 | Yes | Dan O’Leary |
| MA062 | Bristol County ARC | 110 County Rd Taunton, MA 02780 | No | 2 | No | Dan O’Leary |

Table 3.3. New Hampshire Facility

| FACID | Facility | Address | Rep Samples Allowed | Min # of OF’s to be Sampled | Access Needed for Site | REPS Name |
| --- | --- | --- | --- | --- | --- | --- |
| NH014 | Londonderry ARC/ AMSA #160 G | 64 Harvey Road Londonderry, NH 03053 | Yes | 1 (out of 3) | No | Dan O’Leary |

Table 3.4. New York Facilities

| FACID | Facility Name | Facility Address | Are Rep Samples Allowed | Min # of OF’s to be Sampled | Access Needed for Site | REPS Name |
| --- | --- | --- | --- | --- | --- | --- |
| NY059 | Rotterdam AMSA #8 (G) | 101 Remsen St Rotterdam, NY 12306 | Yes1 | 1 (out of 3) | Yes | Shane Matuskovic |
| NY070 | MAJ Holleder USARC/ AMSA #76 BMA | 515 Ridge Rd Webster, NY 14580 | No | 1 (out of 1) | Yes | Scott White |
| NY116 | SGT Catalin D. Dima AFRC | 910 Raz Ave New Windsor, NY 12553 | No | 2 (out of 2) | Yes | Shane Matuskovic |
| NY113 | Mattydale ARC | 6791 Townline Road Syracuse, NY 13211 | No | 1 (out of 4) | Yes | Shane Matuskovic |

1 If there is an event that triggers corrective actions at an outfall that represents other substantially identical outfalls, representative outfalls are suspended. Quarterly visual monitoring and benchmark monitoring of all substantially identical outfalls shall commence immediately. Unless otherwise notified by the NYSDEC, the representative outfall again applies when the results of two consecutive monitoring periods reported to the Department show that all outfalls have had no exceedances of benchmark monitoring cut-off concentrations for all parameters and a new Representative Outfall Waiver Form is submitted to the Department. See Part IV(G)(3) of the NYSDEC MSGP.

Table 3.5. Pennsylvania Facility

| FACID | Facility | Address | Rep Samples Allowed | Min # of OF’s to be Sampled | Access Needed for Site | REPS Name |
| --- | --- | --- | --- | --- | --- | --- |
| PA051/ PA171 | Johnstown Aviation Facility/AMSA #104 | 284 Aviation Drive Johnstown, PA 15902 | No | 2 | No | Lorrie Nogrady |

## Benchmark Sampling and Monitoring

Quarterly Benchmark Sampling for MD Facility

The contractor shall conduct required quarterly sampling for the Maryland facility listed in Table 3.1. Sampling is to be conducted for the pollutants listed in Table 5.1.  Representative outfalls are not allowed to be used for the quarterly sampling. It is preferred that the sampling be performed within the first month of the quarter to ensure timely submissions of appropriate reports to EPA. The Quarterly Benchmark Sampling monitoring periods are as follows:

- Quarter 3:  July 1 through September 30; due to MDE by October 28

- Quarter 4: October 1 through December 31; due to MDE by January 28

- Quarter 1: January 1 through March 31; due to MDE by April 28

- Quarter 2: April 1 through June 30; due to MDE by July 28

All sample results must be submitted to the 99th RD no later than 30 days following each sampling event.

Quarterly Benchmark Sampling for MA Facilities:

The contractor shall conduct required quarterly sampling for the Massachusetts facilities listed in Table 3.2. Sampling is to be conducted for the pollutants listed in Table 5.2. Representative outfalls are not allowed to be used for the quarterly sampling. It is preferred that the sampling be performed within the first month of the quarter to ensure timely submissions of appropriate reports to EPA. The Quarterly Benchmark Sampling monitoring periods are as follows:

- Quarter 3:  July 1 through September 30; due to EPA by October 28

- Quarter 4: October 1 through December 31; due to EPA by January 28

- Quarter 1: January 1 through March 31; due to EPA by April 28

- Quarter 2: April 1 through June 30; due to EPA by July 28

All sample results must be submitted to the 99th RD no later than 30 days following each sampling event.

Quarterly Benchmark Sampling for NH Facility

The contractor shall conduct required quarterly sampling for the New Hampshire facility listed in Table 3.3. Sampling is to be conducted for the pollutants listed in Table 5.2.  Representative outfalls are not allowed to be used for the quarterly sampling. It is preferred that the sampling be performed within the first month of the quarter to ensure timely submissions of appropriate reports to EPA. The Quarterly Benchmark Sampling monitoring periods are as follows:

- Quarter 3:  July 1 through September 30; due to EPA by October 28

- Quarter 4: October 1 through December 31; due to EPA by January 28

- Quarter 1: January 1 through March 31; due to EPA by April 28

- Quarter 2: April 1 through June 30; due to EPA by July 28

All sample results must be submitted to the 99th RD no later than 30 days following each sampling event.

Semi-Annual Benchmark Sampling for NY Facilities:

All New York facilities in Table 3.4 require Semi-Annual Benchmark Sampling to be performed for the pollutants listed in Table 5.3. The monitoring period for semi-annual sample requirements is listed below. It is preferred that the sampling be performed within the first four months of the monitoring period to ensure timely submission of appropriate reports to NYSDEC.  As described in Part IV(F)(2) of the NYSDEC Multi-Sector General Permit, the semiannual Benchmark Sampling monitoring periods are as follows:

- Semiannual Period 2:  July 1 through December 31; due to NYSDEC by January 28

- Semiannual Period 1:  January 1 through June 30; due to NYSDEC by July 28.

All sample results must be submitted to the 99th RD no later than 30 days following each sampling event.

Semi-Annual Benchmark Sampling for PA Facility-Air Transportation Sector

The contractor shall conduct required sampling for the Pennsylvania facility listed in Table 3.5. Sampling is required for the Air Transportation Sector during the deicing season which is defined by PADEP as October 1 – March 31. Sampling is to be conducted for pollutants listed in Table 5.4. The monitoring periods for the sampling are listed below. It is preferred that the sampling be performed within the first month of the monitoring period to ensure timely submission of appropriate reports to PADEP.

- Once from October 1 through December 31; due to PADEP January 28.

- Once from January 1 through March 31; due to PADEP April 28.

All sample results must be submitted to the 99th RD no later than 30 days following each sampling event.

# APPLICABLE DOCUMENTS

Applicable documents include the following:

- Maryland Department of the Environment (MDE) General Permit for Discharges from Stormwater Associated with Industrial Activities, Permit No. 20-SW (effective February 1, 2023 to January 31, 2028).

- United States Environmental Protection Agency, National Pollutant Discharge Elimination System (NPDES) 2020 Multi Sector General Permit for Discharges of Stormwater Associated with Industrial Activities (effective January 15, 2021 to February 28, 2026).

- New York State Pollution Department of Environmental Conservation SPDES Multi-Sector General Permit for Stormwater Discharges Associated with Industrial Activity, Permit No. GP-0-23-001 (effective March 8, 2023 to March 7, 2028).

- Pennsylvania Department of Environmental Protection’s National Pollutant Discharge Elimination System (NPDES) General Permit for Discharges of Stormwater Associated with Industrial Activities, PAG-03 (effective March 24, 2023 to March 23, 2028).

- “PAG-03 General Permit Approval,” letter from Christopher Kriley, PADEP, to Jacqueline Siciliano, 99th Readiness Division, dated 13 JUN 2023.

- If any of the above referenced permits are updated, modified or renewed during the period of performance of this contract, the vendor will be notified of changes and is expected to provide support to ensure all requirements of the permit are satisfied. It is anticipated that any changes will not incur additional cost to the Government unless the vendor justifies to the COR and KO additional tasks required to comply with the permit and the KO determines a change order is warranted.

# TECHNICAL REQUIREMENTS

## Kick-off Meeting

Upon award of the contract, the contractor will convene a kick-off meeting with the 99th RD and Contracting Officer Representative (COR) to discuss the tasks to be executed in accordance with this contract. The contractor shall prepare a kick- off meeting agenda and provide it to the 99th RD and the COR at least three (3) days prior to the kick-off meeting. The contractor shall have seven (7) days after the kick-off meeting to provide the 99th and the COR with kick-off meeting minutes.

## Site Visits

Upon award of the contract, the contractor will visit each facility prior to sample collection to determine the best mechanism and location to perform the sampling. While on site, the contractor will identify the POCs and process for accessing the sampling locations.

The contractor shall coordinate with the 99th RD and facility personnel to develop a schedule of site visits. The contractor shall submit the site visit schedule no later than 14 days prior to the first site visit to the 99th RD and the COR for review and approval.

## Work Plan

The contractor shall prepare a brief work plan describing the proposed work for each site listed in this PWS. The work plan shall be submitted to the 99th RD DPW, Environmental Division and the COR for review and comment within 30 days upon completion of the kickoff meeting and before beginning any field activities. Field activities may commence before the work plan is finalized if agreed upon by the 99th RD and the COR. The work plan shall include:

Observations and decisions made during the site visit regarding sampling procedures and mechanisms, who the points of contact are for each site, and what site visit and facility procedures and access will be.

The weather station to be used as verification of a qualifying storm event for each facility shall be identified and then agreed upon by the 99th RD and COR.

Sampling methods and descriptions of techniques the contractor will use to prevent sample contamination. Sampling methods may include the installation of automatic samplers, handheld grab samples, or any other method deemed necessary based on the field conditions.

A description of whether every regulated outfall at each facility will be sampled directly or via a representative sample where allowed. If a representative sample is to be used, the justification should be identified.

The criteria and methods for obtaining and analyzing the samples (e.g. laboratory analysis, holding times, preservatives, bottle type) and completing the required documents for each site as per the individual state agency requirements. The laboratory that will be used for sample analysis shall also be included.

## Sampling Performance

The contractor will monitor the weather to determine when a qualifying storm event will occur.

    - In MA, NH, NY, and PA, a qualifying storm event is defined as any storm resulting in greater than 0.1 inch of precipitation preceded by an interval of at least 72 hours of dry weather.

    - In MD, a measurable (i.e., qualifying) storm event is an event that results in an actual discharge from the site that follows the preceding measurable storm event by at least 72 hours.

The contractor will provide and prepare all sampling equipment.

The contractor is responsible for ensuring that all samples are collected within the first 30 minutes of the qualifying storm event. This can be accomplished by pre-installing an automatic sampler prior to the storm event or by having personnel physically on site during the commencement of the storm event.

Observations about the quality of the samples must be documented on a visual monitoring form and details of the storm that was sampled must be documented on a storm event form. State-specific forms shall be used, if available.

Monitoring and analysis shall be performed in accordance with 40 CFR Part 136. The samples must be delivered to a Licensed State Specific laboratory as applicable to each site.

All samples will be collected using only approved EPA sampling techniques and in a manner that prevents cross contamination of samples. Discharge samples must be analyzed for the following parameters listed in tables 5.1 -5.4.

Table 5.1. Analytical Parameters for MD Facilities

| Pollutant of Concern | When it is Tested | Sample Type | Benchmark Monitoring Cut-Off Concentration |
| --- | --- | --- | --- |
| Total Recoverable Aluminum | Quarterly | Grab | 1.1 mg/L |
| Total Lead (saltwater) | Quarterly | Grab | 0.21 mg/L |
| Total Zinc (saltwater) | Quarterly | Grab | 0.09 mg/L |

Table 5.2. Analytical Parameters for MA & NH Facilities-Land Transportation

| Pollutant of Concern | When it is Tested | Sample Type | Benchmark Monitoring Cut-Off Concentration |
| --- | --- | --- | --- |
| Total Suspended Solids (TSS) | Quarterly | Grab | Report |
| Chemical Oxygen Demand | Quarterly | Grab | Report |
| pH | Quarterly | Grab | Report |
| PAH 1 | Semiannual | Grab | Report |

1 Monitoring is required for the 16 individual PAHs identified at Appendix A to 40 CFR Part 423: naphthalene, acenaphthylene, acenaphthene, fluorene, phenanthrene, anthracene, fluoranthene, pyrene, benzo[a]anthracene, chrysene, benzo[b]fluoranthene, benzo[k]fluoranthene, benzo[a]pyrene, benzo[g,h,i]perylene, indeno[1,2,3-c,d]pyrene, and dibenz[a,h]anthracene.

Table 5.3. Analytical Parameters for NY Facilities

| Pollutant of Concern | When it is Tested | EPA Method | Benchmark Monitoring Cut-Off Concentration |
| --- | --- | --- | --- |
| Oil and Grease | Semi-annually | Method 1664, Revision A | 15 mg/L |
| Chemical Oxygen Demand (COD) | Semi-annually | Method 410.4 | 120 mg/L |
| Benzene | Semi-annually | Method 602 by 624 | 50 µg/L |
| Toluene | Semi-annually | Method 602 by 624 | 50 µg/L |
| Ethylbenzene | Semi-annually | Method 602 by 624 | 50 µg/L |
| Total Xylene | Semi-annually | Method 602 by 624 | 50 µg/L |

Table 5.4. Analytical Parameters for PA Facilities-Air Transportation

| Pollutant of Concern | When it is Tested (deicing season) | Sample Type | Benchmark Monitoring Cut-Off Concentration |
| --- | --- | --- | --- |
| Total Nitrogen 1 | Semi-annually | Calculation | Report |
| Total Phosphorus | Semi-annually | Grab | Report |
| pH (S.U) | Semi-annually | Grab | 9.0 |
| 5-Day Biochemical Oxygen Demand (BOD5) | Semi-annually | Grab | 30 mg/L |
| Chemical Oxygen Demand (COD) | Semi-annually | Grab | 120 mg/L |
| Total Suspended Solids (TSS) | Semi-annually | Grab | 100 mg/L |
| Ammonia-Nitrogen | Semi-annually | Grab | Report |
| Total Dissolved Solids | Semi-annually | Grab | Report |
| Four PFAS: PFOA, PFOS, GenX, PFBS 2 | Semi-annually | Grab | Report |

1  Total Nitrogen is the sum of Total Kjeldahl-N (TKN) plus Nitrite-Nitrate as N (NO2+NO3-N), where TKN and NO2+NO3-N are measured in the same sample.

2  The June 13, 2023 PAG-03 General Permit Approval letter for the Johnstown facility requires semi-annual PFAS sampling in addition to the semi-annual monitoring requirements required by Appendix G of the General Permit.  Samples must be analyzed for the specified PFAS using the EPA-approved method that will achieve the lowest level of quantitation for each pollutant.

# IN-PROGRESS REVIEW MEETINGS

The contractor shall conduct semi-annual in-progress review meetings to discuss work completed, work planned and any issues. The contractor shall provide the 99th and the COR with an agenda at least three (3) days prior to the scheduled meeting. The contractor shall provide meeting minutes to the 99th and the COR within seven (7) days after the meeting.

# SUPPORTING DATA

## Discharge Monitoring Report (DMR):

The 99th facilities listed shall submit DMRs electronically via NetDMR and for the PA facility, via PADEP’s electronic Discharge Monitoring Report (eDMR). The contractor shall, upon approval from the 99th RD, create a NetDMR & eDMR account as a Data Provider and fill out the DMRs electronically. The contractor shall submit a packet of all required information and copies of the electronically entered data to the 99th RD. The following information shall be included with the DMR packets:

- The value of the parameter as well as its units;

- The frequency of analysis (semi-annually/quarterly);

- The type of sample conducted;

- Any additional comments or explanations in accordance with state-specific DMR instructions.

A separate DMR is required for each representative discharge point (outfall) in some States; the contractor shall submit all necessary DMR information as required.

## PADEP’s Supplemental Laboratory Accreditation Form

PADEP’s Supplemental Laboratory Accreditation Form (3800-FM-BPNPSM0189) must be completed and submitted to PADEP with the first DMR (if the certified laboratory changes).

## Records of Monitoring Information

The vendor shall include the raw laboratory data listed below and chain of custody paperwork for each sample. This data must include, but is not limited to:

- The date, exact place (OF and/or storm drain# which can be located on the facility site map), and time the sample was collected;

- The individual(s) who performed the sampling or measurements;

- The date(s) analyses were examined;

- The individual(s) who performed the analyses;

- The analytical techniques or methods used;

- The results of such analyses;

- Quality assurance/quality control documentation.

## Storm Event Data (SED)/ Stormwater Monitoring Report Form (SMR):

The contractor shall record the storm event data on a state-specific storm event data form each time a sampling attempt is made at an outfall, whether or not the attempt is successful. The storm event data must include, but is not limited to:

- Date and duration (in hours) of the measurable (greater than 0.1 inch of precipitation) storm event(s) sampled;

- Rainfall measurements or estimates (in inches) of the storm event that generated the sampled runoff;

- Date of the last measurable storm event (greater than 0.1 inch of precipitation);

- The duration between the storm event sampled and the end of the previous measurable (greater than 0.1 inch of rainfall) storm event (must be an interval of at least 72 hours).

- Any other state specific requirements in regard to the sampling (CT has specific requirements).

- In the absence of a State Form the contractor shall create one that is similar to one used by the other states listed, after approval from the 99th RD.

## Weather Data:

Weather data shall be included to prove that qualifying storm event requirements were met.

## Representative Outfall Waiver (ROW):

The contractor shall fill out this form anytime one outfall is used to represent another. The required information for this form includes, but is not limited to the following:

- The outfall number that is being claimed as a representative outfall as well as the outfall number(s) for which the waiver is being claimed;

- The size (in square feet) and runoff coefficient of the drainage area for each outfall (information available on-site maps)

- Short description explaining why an outfall was chosen as a representative outfall.

## Quarterly Visual Monitoring Form (QVM):

A QVM form shall be filled out by the contractor for each regulated outfall at each facility based upon the observations made during each sampling event. The information needed includes, but is not limited to:

- The date, exact place (OF and/or storm drain# which can be located on the facility site map), and time the sample was collected;

- The individual(s) who performed the sampling or measurements and their title;

- The date and exact time the sample was examined;

- Visual information about the discharge such as color, odor, concerns etc.

- The form shall be signed by the contractor.

In the absence of a State Form the contractor shall create one that is similar to one used by the other states listed, after approval from the 99th RD.

# DATA REPORTING

## Benchmark Exceedances:

If any of the sample results show an exceedance of the benchmark values, the contractor must forward those results to the 99th RD within 24 hours of receipt. All applicable supporting data (storm event data, records of monitoring information, weather data, and quarterly visual monitoring form) must be forwarded within 72 hours so that the RD can initiate mandatory corrective action analyses.

# DELIVERABLES

| 1 | Kick-off Meeting Agenda | Submit to 99th RD and COR at least three (3) days prior to kick-off meeting |
| --- | --- | --- |
| 2 | Kick-off Meeting Minutes | Submit to 99th RD and COR within seven (7) days after kick-off meeting |
| 3 | Site Visit Schedule | Submit to 99th RD and COR no later than 14 days prior to first site visit |
| 4 | Work Plan | Submittal of Work Plan within 30 days of initial site visit schedule approval that addresses all aspects of the work to be performed/approval by the COR and 99th RD. |
| 5 | Collect & Analyze Samples | 100% of the samples collected for 100% of the regulated outfalls, analyzed for 100% of the required parameters at 100% of required facilities within the required monitoring period and time frame 100% of the time. |
| 6 | Completion of State Specific Monitoring Reports and Forms | 100% of the forms completed with 100% of the required data (DMR, SMR, SED, ROW, QVM) within the required timeframe 100% of the time. |
| 7 | Semi-Annual In-Progress Review Meeting Agenda & Minutes | Submit agenda to 99th RD and COR at least three days prior to each scheduled meeting & minutes three days after. |

The contractor shall provide draft copies of all forms and reports in electronic form to the COR and the 99th RD DPW, Environmental Division for approval prior to finalization. The 99th personnel shall review each document and provide comments to the COR who will issue final approval.

# SCHEDULE

All sampling and monitoring at each facility must be completed by the last day of each required monitoring period. It is preferred that the sampling and monitoring events take place well in advance of the last day of each required monitoring period to ensure timely submission to the appropriate regulatory agency.

All supporting data and forms must be returned to the 99th RD electronically.  Data and forms must be returned within a timeframe which ensures that DMRs can be submitted by the 99th within 28 days of the end of the monitoring period, as required by regulation. Procedures identified in Section 8.1 must be followed for all benchmark exceedances.

All regulatory requirements must be followed when scheduling the grabbing of samples: sample grabs must be made during qualifying storm events that are preceded by at least 72 hours of dry weather.

The contractor will attempt to complete site visits to facilities in close geographic proximity to each other on the same trip wherever possible, to reduce costs and expedite project completion time.

# OPTIONAL LINE ITEM TASKS

# 11.1		Optional Line Item 0001: Hydraulic Survey at NY116

The Contractor shall investigate the stormwater system at SGT Catalin D. Dima AFRC. The contractor shall conduct a drain survey to identify noncompliance and deficiencies and verify current stormwater maps. The contractor shall provide recommendations for bringing these systems into compliance with applicable regulations, such as needed repairs and estimated cost, permit or best management practice (BMP) changes and improved operating procedures.

The contractor shall conduct data collection and perform an on-site survey to obtain data and information on facility stormwater management, validate presence and function of stormwater management structures identified on as-built drawings, and to document site conditions at the facility and surrounding properties.

Data shall be collected in a format such that the results of the evaluation can be used to determine negative impacts to the environment and neighboring properties. The contractor shall collect pertinent data using GPS equipment with sub-foot accuracy at a 95% confidence level. The contractor shall develop maps to display the data collected and provide in .APRX (ArcGIS Pro version 3.3.1) and .pdf formats.

The Contractor shall conduct an initial site visit at the facility to survey the stormwater discharge activities. The purpose of the initial site visit is to conduct site reconnaissance and gather all necessary information to successfully conduct the drain surveys. During the initial site visits, the contractor shall assess what activities and equipment will be required at each site in order to conduct the drain survey and will:

- Determine sources, flow pathways and discharge points

- Determine which locations and structures will require testing

- Determine the length of time required for each site investigation

- Determine equipment and other resources needed for the drain survey

- Obtain available stormwater and sewer system related documents or permits

- Collect available drawings and facility information; and

- Interview facility personnel

The Contractor shall gather all pertinent information needed to develop & conduct the drain surveys.

# 11.2		Optional Line Item 0002: Hydrojet/Vac at NY023

The contractor shall perform maintenance on the stormwater system at Ernie Pyle USARC/ AMSA #12.

The contractor will conduct an initial site visit to the facility to perform site reconnaissance and assess what equipment is necessary and what areas work will be conducted from; grassed areas, parking lots, etc, in order to clean the stormwater catch basin and pipe system at Ernie Pyle USARC/ AMSA #12.

The contractor shall, first with a heavy-duty pump truck remove trapped water from the stormwater system, and then with a heavy-duty high velocity Vac-All truck, clean catch basins and water jet interconnected lines to clear pipes of debris blockages and sediment build up.

The contractor shall provide testing of sediment and/or water pumped/ vac’d from the stormwater system to ensure its proper disposal.

The contractor shall obtain any necessary permits to dispose of sediment or water pumped/ vac’d from the stormwater system and will dispose of it accordingly.

The contractor shall provide a final report of the pre- and post- condition of the stormwater system, testing results, disposal information, and future recommendations for maintenance or repair.

# 11.3		Optional Line Item 0003: PFAS Sampling at 3 Sites in EPA Region 1 (MA058, MA062, NH014)

In the event that the USEPA includes PFAS Indicator sampling in the 2026 Multi-Sector General Permit for Stormwater Discharges Associated with Industrial Activity, this option will be exercised in accordance with the conditions of the permit to include quarterly Indicator Monitoring for PFAS with the quarterly Benchmark Monitoring for analytes that currently occurs.

The contractor shall provide all labor and supplies to sample stormwater at MA058, MA062, and NH014. The EPA requires monitoring for the 40 PFAS compounds listed in EPA Method 1633. Samples must be analyzed using EPA Method 1633.

All samples shall be submitted to a laboratory accredited to EPA Method 1633 under a proper chain of custody procedure & results provided to the 99th RD.

# 11.4		Optional Line Item 0004: Drinking Water PFAS Sampling at Muddy Creek

The contractor shall provide all labor and supplies to sample a ground water well at 1123 Perry Highway (Building #2), Muddy Creek Township, PA 16052, in accordance with the applicable Federal or State approved guidance. The DoD requires analysis by EPA Method 533.

Each groundwater well shall be sampled for all analytes included in the most current EPA Method 533 analyte lists.

Prior to sample collection the wells shall be purged IAW with proper sampling procedure. The wells shall be allowed to recover to prevent turbid water at sample collection. All samples shall be submitted to a laboratory accredited to EPA Method 533 by the DoD Environmental Laboratory Accreditation Program (ELAP) under a proper chain of custody procedure & results provided to the 99th RD.

The contractor shall compose a draft and final report detailing the sampling results and any recommendations for further testing.

# ASSUMPTIONS

The contractor shall furnish all labor, materials, appliances, and equipment to complete the work as specified herein.

The contractor shall be responsible for any damage resulting from or in connection with the execution of this project.

# 99th RD POINT OF CONTACT

The technical point of contact for the 99th RD concerning Performance Work Statement (PWS) is Mr. Vincent Poulsen, vincent.c.poulsen.ctr@army.mil, 609-562-1628 (O).

# ADDITIONAL REQUIREMENTS:

## General Security Requirements and Guidance

The security requirements described below apply to all contract personnel (including employees of the prime Contractor [“Contractor”] and all subcontractor employees) supporting the performance requirements of this contract.  The Contractor is responsible for compliance with these security requirements. Questions regarding security matters shall be addressed to the designated Government representative (e.g. Contracting Officer Representative [COR], Requiring Activity [RA] representative, or Contracting Officer [if a COR or other RA representative is not appointed]). Contract personnel are critical to the overall security and safety of US Army Corps of Engineers (USACE) installations, facilities, and activities, and security awareness training contributes to those efforts.  The Department of Defense (DoD) and Army security training requirements specified below, if applicable, are performance requirements; all applicable contract personnel shall complete initial training within 30 days of contract award or the date new contract personnel begin performance on the contract. Within five business days from the completion of training, the Contractor shall provide written documentation (e.g. email or memorandum) to the Government representative. The documentation shall include the names of contract personnel trained and which training they completed; the Contractor shall maintain training records as part of their contract files and be prepared to provide copies of training certificates to the Government representative. Contractor personnel and vehicles are subject to search when entering federal installations. Additionally, all contract personnel shall comply with Force Protection Condition (FPCON) measures, Random Antiterrorism Measures (commonly referred to as “RAMs”), and Health Protection Condition (HPCON) measures. The Contractor is responsible for meeting performance requirements during elevated FPCON and/or HPCON levels in accordance with applicable RA plans and procedures- this includes identifying mission essential and non-mission essential personnel. In addition to the changes otherwise authorized by the changes clause of the contract, should the FPCON or HPCON levels at any individual facility or installation change, the Government may implement security changes that affect contract personnel. The Contractor shall ensure all contract personnel are aware of their security responsibilities, including any site-specific requirements identified in local policies or procedures.

## AT Level I Training

All contract personnel requiring routine access to Army installations, facilities, and controlled access areas, or requiring network access, shall complete initial and annual refresher AT Level I awareness training.  Online AT Level I awareness training is available at https://jko.jten.mil/ (website subject to change).

## Physical Security and Access Control Requirements

All contract personnel requiring physical access to a federal installation or facility shall comply with the access control procedures of that location. Contract personnel requiring unescorted access to meet contract performance requirements on a DoD installation in the US shall be vetted by the installation/facility Provost Marshal/Directorate of Emergency Services/Security Office using the National Crime Information Center-Interstate Identification Index (commonly referred to as “NCIC-III”) and Terrorist Screening Database (commonly referred to as “TSDB”). Contract personnel shall comply with all personal identity verification requirements specified in installation/facility policies and procedures. Contract personnel who do not meet requirements for unescorted access to facilities shall coordinate escorted access with the Government representative, as needed. Contract personnel who receive keys, access cards, or lock combinations that provide access to government-owned property shall comply with key and lock control procedures of the RA.

## Suspicious Activity Reporting Training

All contract personnel shall receive initial and annual refresher training from the RA representative on the local suspicious activity reporting program. This locally developed training provides contract personnel with general information on suspicious behavior, and guidance on reporting suspicious activity to the project manager, security representative or law enforcement entity.

## Pre-screen candidates using E-Verify Program

Contractors shall comply with the requirements set forth in FAR clause 52.222-54 Employment Eligibility Verification and FAR Subpart 22.18 in using the E-Verify Program at https://www.e-verify.gov (website subject to change) to meet the contract employment eligibility requirements. Contractors are encouraged to cooperate with Federal and State agencies responsible for enforcing labor requirements to include eligibility for employment under United States immigration laws in accordance with FAR 22.102-1(i). An initial list of verified/eligible candidates shall be provided to the COR no later than three business days after the initial contract award. When contracts are with individuals, the individuals will be required to complete a Form I-9, Employment Eligibility Verification, and submit it to the Contracting Officer to become part of the official contract file.

# PROGRESS REPORTS/INVOICES

The Contractor shall prepare and submit monthly progress reports to the USACE COR and provide a copy to the 99th RD. Monthly reports will include an accurate, current summary of all work completed and deliverables furnished to the Government by contract line item. Progress reports will be submitted to the Government, along with payment invoices, no later than fifteen (15) calendar days following the end of the month in which work was completed. Records of telephone conversations and significant verbal conservations conducted in support of this project will be provided along with monthly reports, as applicable. The Contractor shall assign sufficient personnel to insure the execution of the work in accordance with the approved work plan and submittal schedule.

The Contractor shall correct all deficiencies identified by the Government or in its own quality control review, relating to completeness or accuracy of the work, compliance with the contract, task order, laws or regulations.

# PERIOD OF PERFORMANCE

The period of performance is from July 1, 2026, to June 30, 2026.  All deliverables and work to be performed shall be completed in accordance with the period of performance of this contract.

The options, if awarded, will be awarded no later than 270 days after the base contract award. Notice of intent to award an option is required within 30 days of exercising the option via contract modification. The period of performance for optional work will end at the end of the base period of performance.