DRAFT * Performance Work Statement (PWS)

for

Electrical Equipment Operation and Maintenance Services Contract

Electrical Maintenance Contract (EMC)

In Support of

Air Force Research Laboratories

FA2396-26-X-B###

16 March 2026

AFRL/PZLEB

# Table of Contents

Table of Contents	2

1	Introduction	4

1.1	General	4

1.2	Background	4

1.3	Vision Statement	4

1.4	Non-Personal Services	5

1.5	Inherently Governmental Functions	5

1.6	Ethics	5

1.7	Scope	5

2	Services Summary (SS)	6

2.1	Service Summary Table	6

2.2	Specific Requirements and Services	8

2.3	Remedies For Less Than Satisfactory Performance	20

2.4	Deliverables	20

2.5	Industry Standards (Technical Standards)	22

2.6	Government Specific Standards (Technical & Operating Instructions)	22

2.7	Scheduled and Unscheduled Work	22

2.8	Workflow for WOs and Work Plans	23

3	Staffing Requirements	23

3.1	Contract Service Personnel	23

3.2	Personnel Qualifications	24

3.3	The Use of Foreign Nationals	24

3.4	Technician Certification Programs	25

3.5	Professional Positions	25

3.6	Key Personnel For Services	29

4	Contract Management	29

4.1	Government Management	30

4.2	Transition Planning	30

4.3	Integration of Effort	30

4.4	Quality of Support	30

4.5	Contractor Facilities, Equipment, Services, Data & Materials	31

4.6	Office Equipment, Software, Support, and Network Access	32

4.7	Shared Equipment	33

4.8	Data	34

4.9	Consumable Materials	35

4.10	Contractor Supplied Property	35

4.11	Management of Personnel	35

4.12	Timeliness	37

4.13	Reperformance of Deficient Services	37

4.14	Support Reviews	38

5	Quality Assurance	38

5.1	Government Inspection	38

5.2	Contracting Officer’s Representative	38

5.3	Administrative Records	39

5.4	Quality Control Plan	39

5.5	Performance Evaluation	39

6	Contract Security	40

6.1	Non-Disclosure Agreements (NDA)	40

6.2	Information Access	40

6.3	Physical Security	40

6.4	Industrial Security	41

6.5	Operational Security (OPSEC)	41

6.6	Communications Security (COMSEC)	42

6.7	Security Clearances	42

6.8	Security Training	42

6.9	In/Out Processing	42

6.10	Security Administration	42

7	Government Furnished Property & Equipment	43

7.1	Use of Personal Equipment	43

7.2	Protection from Loss	43

7.3	Supplies and Equipment	43

Appendix A: Contract Data Requirements List	45

A.1 CDRL A001 Technical and Management Work Plan	45

A.2 CDRL A002 Technical Report-Study/Services -TR-S/S (Reports)	45

A.3 CDRL A003 Conceptual Design Drawings/Models and Associated Lists	45

A.4 CDRL A004 Presentation Material for Briefings/Meetings (B/M)	46

A.5 CDRL A005 Records of B/M (Minutes)	46

A.6 CDRL A006 - Daily Site Activity Schedule (DSAS)	46

A.7 CDRL A007 Acceptance Test Plan (ATP)	46

A.8 CDRL A008 Contractor Employee Qualification & Certification Report.	46

Appendix B:  Use of Government Infrastructure	47

B.1. Use of Government Vehicles	47

B.2. Traffic Laws	47

B.4. Environmental Management System (EMS)	47

B.5. Network Administration	47

Appendix C:  Contract Safety	49

C.1. Safety and Health Requirements	49

C.2. Contractor EMPLOYEES’ Safety Requirements	49

# Introduction

## General

The scope of work contained within this Performance Work Statement encompasses a broad range of engineering, project management, and technical services for electrical equipment and systems, and will be acquired as non-personal services in accordance with (IAW) the provisions of FAR Part 37.

## Background

The United States (US) Air Force Research Laboratory (AFRL) Aerospace Systems Directorate (RQ) Integration and Operations Division (RQO) Research Operations Support Branch’s (RQOO) mission is to provide engineering, project management, Research and Development (R&D) support, facility operation, and other technical services for the planning, acquisition, operation, maintenance, installation, removal, and repair of electrical equipment and systems at the AFRL/RQ Wright Patterson Air Force Base (WPAFB) site.

AFRL is located at WPAFB, Ohio and under Air Force Materiel Command (AFMC). This Performance Work Statement (PWS) describes the operational support required by AFRL to sustain RQ’s research and development.

RQ performs R&D of aircraft propulsion and other air vehicle systems. As part of this responsibility, the directorate provides and maintains low and medium voltage electrical systems and equipment that support RQ’s R&D efforts. RQOO supports operations in about 49 different buildings at WPAFB, area B.

RQ operates and maintains over 100,000 horsepower (HP) of rotating equipment, some of which predates WWII.  There are medium voltage, mostly 6900V, electrical cubicles and equipment included on a list of over 3000 pieces of electrical equipment varying from electromechanical to solid state relays; motors of various age, HP and type; 480V motor control cabinets of various age and size; oil filled and dry type transformers; circuit breakers from molded case to oil filled; and wire and cabling of all types and age.

AFRL/RQ’s research electrical distribution and research electrical loads and support maintenance requirements, sets the foundation for this contract.

This Contract is managed by RQOO. When approved by RQOO, other AFRL Directorates or government organizations may utilize this Contract.

## Vision Statement

The primary objective of this contract is to obtain high-quality, affordable, and responsive InterNational Electrical Testing Association (NETA) certified electrical maintenance and repair of equipment. The contractor shall provide personnel, services, test equipment, tools, materials, vehicles, and subcontracting necessary to perform non-personal services for, engineering, acquisition, operation, maintenance, installation, removal, and repair of electrical equipment and systems supporting RQ. Due to the dynamic nature of research and development, the contract will experience variations in workload. The Contractor shall have sufficient flexibility and depth to accommodate AFRL/RQ requirements in a timely and efficient manner.

RQOO follows proven Electrical Operations and Maintenance (O&M) practices to set strict requirements to guard and protect valuable RQ R&D electrical assets against failure. To meet RQOO’s mission the contractor shall follow a strict InterNational Electrical Testing Association (NETA) standard as explained in this PWS and required by NETA. This requires that both the contractor and technicians working for them are certified and in good standing with NETA.

The majority of services explained in this PWS require personnel qualified by NETA. The Contractor shall be a current NETA Accredited Company (NAC) at the time of proposal submission and shall maintain this accreditation in good standing for the duration of the contract. Because the majority of the work required under this contract must be performed by NETA-certified personnel operating under a NETA Accredited Company’s quality program, the prime contractor shall hold the NETA accreditation.

The Contractor shall provide documentation verifying current NAC accreditation, evidence that the accreditation is in good standing, and a copy of the current NETA accreditation certificate upon request by the Government.

Failure to maintain NAC accreditation throughout contract performance may be considered grounds for termination or other contractual remedies as determined by the Government.

## Non-Personal Services

This is a non-personal services contract. The Government will neither supervise contractor employees nor control the method by which the contractor performs the work. It shall be the responsibility of the contractor to manage its own personnel and to guard against any actions that are of the nature of personal services or give the perception of personal services. If the contractor believes that any action constitutes, or are perceived to constitute personal services, it shall be the contractor’s responsibility to notify the Contracting Officer (CO) immediately.

## Inherently Governmental Functions

The Contractor shall not perform any inherently governmental functions. All decisions relative to research programs supported by the Contractor shall be the sole responsibility of the Government. The Contractor shall not counsel, mentor, judge and/or make discretionary decisions or perform any other activities related to the supervision of Government personnel. If the Contractor believes that any actions constitute or are perceived to constitute inherently governmental functions, the Contractor shall notify the CO immediately.

## Ethics

The Contractor is highly visible to the entire acquisition community as a result of providing assistance to the Government. Consequently, the Contractor shall present an unblemished appearance regarding ethics, discretion,
and protection of information, including the guidelines for electronic mail use listed in AFH 33-337 chapter 12, Air Force Instruction (AFI) 33-322 Records Management, and Internet use of AFI 35-107 Public Web and Social Communications.

## Scope

The contractor shall provide personnel, services, test equipment, tools, materials, vehicles, and subcontracting
necessary to perform non-personal services for planning, engineering, acquisition, operation, maintenance,
installation, removal, unscheduled response, and repair of electrical equipment and systems supporting RQOO. All tasks shall be accomplished IAW generally accepted engineering standards and practices. All government material handling vehicles and equipment used to perform the tasks under this PWS shall be operated IAW Wright Patterson AFB policies and procedures.

The following subsections underline the performance areas for the scope of this project.

### Program Management (NETA)

### Electrical Engineering (NETA)

### Electrical studies (NETA)

### Preventive Maintenance (NETA)

### Install and Removal (NETA)

### Acceptance Testing (NETA)

### Troubleshooting (NETA)

### Drafting and CAD Services

### Purchasing

### Data Records

### Administration Duties (Scheduling, Time Keeping)

- Services Summary (SS)

## Service Summary Table

The Services Summary (SS) table below specifies the Performance Objectives the Government will survey. The absence of any contract requirement from this section shall not detract from its enforceability nor limit the rights or remedies of the Government under any other provision of the contract.

| Performance Objective | PWS Para. | Performance Threshold |
| --- | --- | --- |
| SS-1 Qualified personnel are present for duty as scheduled | PWS Paragraphs 1.2, 1.7, 2.4.5, 3, & 4 (including applicable subsections for each section and subsection called out in this row) | 90% of key and 80% of non-key personnel are present for duty at start of scheduled performance |
| SS-2 Replacement/Substitute personnel are provided in a timely manner | PWS Paragraphs 1.2, 1.7, 2.4.5, 3, & 4 (including applicable subsections for each section and subsection called out in this row) | 90% of replacement/substitute personnel are present for duty such that position vacancies do not exceed fourteen (14) business days. The contractor shall provide evidence within three (3) days of action being taken to fill the position. |
| SS-3 Maintains a stable workforce | PWS Paragraphs 1.7, 2.4.5, 2.4.6, 3.7 (including applicable subsections for each section and subsection called out in this row) | Contractor employee turnover rate less than 10% across all contracted positions per quarter. |
| SS-4 Provide satisfactory client service for each work order (WO) | PWS Paragraphs 1.7, 2.4, 2.7, 2.8 (including applicable subsections for each section and subsection called out in this row) | No more than two (2) Contracting Officer Representative (COR) validated Customer Complaint Reports (CCRs) per year. 90% of services meet objectives and required outcomes for all PWS services |
| SS-5 Manages services to ensure fulfillment of compliance documents | PWS Paragraphs 1.7, 2.2, 4 (including applicable subsections for each section and subsection called out in this row) | 90% of Contractor personnel meet compliance document requirements |
| SS-6 Manages currency of qualifying factors (company and employee certifications, licensure, work experiences and history, schooling certifications and diplomas) | PWS Paragraphs 1.3, 1.7, 2.4.5, 3 (including applicable subsections for each section and subsection called out in this row) | 90% of requirements are renewed before the expiration date 90% all qualifying factors, each employee, are delivered to CORs prior to each contract renewal period or employee start date |
| SS-7 Submits accurate and complete deliverables on time | PWS Paragraphs 2.4 (including applicable subsections for each section and subsection called out in this row) | 90% of all are deliverables are delivered on-time as defined in the Contract Data Requirements List (CDRL) |
| SS-8 Manages currency of (CDRL submittals, work orders (WOs), data management) computer-based applications, computer filing (soft copy) systems, hard copy filing systems and compliance documents (Work Agreements; Federal, State, and Local workplace doctrine; NFPA 70 & 70E, NECA, & NETA standards; and documents and PWS submittals) | Continued on Next Page PWS Paragraphs 2.4, 4.8 (including applicable subsections for each section and subsection called out in this row) | 90% of all WOs and unscheduled work occurrences are Program Manager (PM) or Foreman reviewed with CORs and have CORs acceptance prior to services, managed by Program Manager (PM) or Foreman, accrue billable hours. -90% of all WOs, service reports & test actions, and documents are delivered and filed as per PWS and CDRLs -90% of all documentation for compliance, standards, CDRL submittals and PWS submittals (training, certifications, plans) are current |
| SS-9 Provides timely daily communication (PM to COR(s)) PM shall confirm COR(s) acceptance of all schedule changes prior to implementing changes to the weekly schedule | PWS Paragraphs 2.2.10 (including all subsections for each section and subsection called out in this row) | -90% scheduled is manned to COR accepted weekly schedule -90% of contractor-generated schedules and schedule updates achieve acceptance by COR(s). -90% generated schedule, job, or services issues from previous day are reviewed with CORs |
| SS-10 Manage deliverables to quality control standards | PWS Paragraphs 2.4, 5.5, 5.6, 5.7 (including applicable subsections for each section and subsection called out in this row) | -90% of all deliverables are free of technical, typos, spelling & grammar errors, & are technically correct -90% of all work orders and subtasks are managed and meet COR accepted schedules |
| SS-11 Proactive and Effective Management | All Sections and Subsections | -Zero valid customer complaints per month -COR will decide which customer complaints are considered valid -PM, Engineers and Foreman are prepared for reviews with specific quantitative and qualitative answers as to who, what when, where and with drawings and system knowledge for CORs |
| SS-12 Comply with all applicable site regulations concerning handling, storing, containerizing, positioning, disposing of, and transporting of hazardous materials for disposal by the on-site disposal facility. | PWS Paragraphs 4.9, Appendix D (including applicable subsections for each section and subsection called out in this row) | -Zero infractions per inspection. Infractions shall be corrected immediately after notification by the Government or discovery by the Contractor, whichever comes first. Provide a typed document of steps taken to prevent future infractions to the Government within 72 hours of notification detailing the corrective actions taken |

  - Specific Requirements and Services

Specific requirements, e.g. WOs and services, are those required by the Government to be performed by the
Contractor as detailed in this PWS. Under the direction of the PM, the Contractor personnel shall complete service
requirements while accomplishing their PM assigned tasks to complete WOs according to the Contractor’s Quality Control Plan (QCP).
The PM shall clearly delineate specific tasks for Contractor employees and services assigned for this work in the Technical and Management Work Plan (T&MWP) form sub sections of this section.

    - Program Management Services

The Contractor shall provide a T&MWP with supporting documentation, detailing PM’s management of Contractor personnel. The Contractor through its PM shall be responsible for managing this program, all efforts, as described in the T&MWP.

The Contractor shall:

      - Manage: Ensure timeliness, completeness, and quality of effort meets PM standards and these shall be briefed to the COR in set meetings.

      - Provide timely: Identification of issues, corrective action plans for deviations to the established plan, and effective management of Contractor and if any subcontractor personnel.

      - Provide Coordination:

##### PM shall ensure all Contract personnel work closely with CORs and are aware they only report to and take direction from their company’s management.

        - PM shall timely identify issues, develop and implement corrective action plans for any deviations from the established plan, and provide effective management and oversight of Contractor and subcontractor personnel.

        - PM shall provide timely replies to CO or COR generated e-mails and not delay responses unless a reasonable reason for the delay is provided.

        - PM shall provide a Staffing Plan as part of the T&MWP that aligns Contractor personnel to PWS services and ensure Contractor personnel are providing to the requirements/ tasks detailed in the T&MWP for the services assigned to each Contractor employee.

        - PM shall establish a weekly schedule documenting contractor set work plans, matching services to WOs and scheduled work.

        - PM shall provide instruction to use and ensure all Contract personnel utilize the sign out board.

        - PM shall set meetings and during these meeting inform the CORs of accomplished WOs, plans to accomplish new WOs and progress reports for services and other PWS required work and coordination.

        - PM shall set and provide workflow patterns to ensure Contractor personnel, CO, and COR(s) are able to recognize Contractor’s organizational structure and interfaces.

        - PM shall ensure that all Contractor employees perform work in accordance with the PM’s direction.

        - PM shall ensure Work Order Control (WOC) generated E-mails for each specific WO number, are defined in the T&MWP:

###### Initial WO e-mail from CORs to Contractor opening a new WO number,

          - Status updates as actions are taken,

          - Final e-mail stating WO is to be closed and is from Contractor to CORs.

          - PM shall ensure that all WOs are updated and reviewed prior to closing in WOCs

        - PM shall ensure e-mails from Contractor Personnel to CORs adhere to Contractor-set standards detailed in the T&MWP providing direction for what should appear in the “To”, “Subject”, and main e-mail body windows. If a WO exists in WOCs, the WO number will be included in the “subject” line.

        - PM shall provide a two-week look-ahead Daily Site Activity Schedule (DSAS) weekly and be able to answer questions regarding it in a weekly status report meeting.

        - PM shall document in the T&MWP the Contracting Company’s plan to cover On-Call services for unscheduled maintenance events in accordance with the Contracting Company’s On-Call procedures.

        - PM shall ensure Contractor employees utilize government provided Electronic Storage as part of end of business day filing process and provide workflow patterns to ensure Contractor personnel, CO, and CORs are able to access the data on this storage.

        - PM shall provide narratives to include workflow between Contractor services, PM and CORs, RQOO client’s requirements and organizations outside of RQOO requirements for Scheduled & Unscheduled Work:

#### Core Hours DSAS Workflow – 
PM shall work with the COR(s) as the focal point, RQOO is the Contractor’s customer, and ensure that all client and personnel associated with the Contracts dialog is coordinated through CORs and recorded according to the PWS. Ensure the COR accepted T&MWP matches the COR accepted DSAS and confirm Contractor personnel are providing WOs and services as directed by the PM.

##### PM shall coordinate with CORs for all Contractor edits or changes to COR accepted submittals and ensuring COR acceptance by COR signature prior to releasing Contractor personnel to work the deviated schedule.

##### Contractor shall provide daily attendance e-mails, to CORs, at the start of each workday. Daily attendance e-mails shall account for all Contractor personnel and the Contractor’s visitors scheduled to be on-site. For days that the daily attendance does not agree with the COR accepted schedule the PM shall ensure CORs are informed of the schedule deviation and shall request COR acceptance for the PM schedule edits. The PM shall provide, throughout the workday, timely updates for COR acceptance of any Contractor edits, changes to the schedule being worked, and other requested COR information regarding the DSAS work.

#### Scheduled Weekend WO Work Plans (DSAS Workflow) 
PM shall ensure local oversight works with the COR(s) as the focal point, RQOO is the Contractor’s customer. Local oversight shall ensure that all WO activity and services are coordinated through CORs and recorded according to the PWS. PM shall ensure outage requests and schedules are submitted and accepted prior to the outage. For any changes to the COR accepted schedule, local oversight shall ensure CORs are informed of the schedule change and receive acceptance prior to releasing personnel to work the edited plan. Local oversight shall ensure the COR accepted T&MWP matches the COR accepted DSAS confirming Contractor personnel are working on COR approved WOs and services. 
Local oversight shall provide timely daily dialog with CORs ensuring timely Local oversight to COR for any Contractor edits or changes to COR accepted submittals and ensuring COR acceptance prior to releasing Contractor personnel to work the deviated schedule.
Local oversight shall provide daily attendance e-mails to CORs which shall account for all Contractor personnel and visitors scheduled to be on-site. For days that the daily attendance does not agree with the COR accepted schedule the Local oversight shall ensure CORs are informed of the schedule deviation and shall request COR acceptance for the Local oversight schedule edits prior to Local oversight releasing personnel to work the edited plan. The Local oversight shall provide completion updates, changes to the schedule being worked, and other requested COR information regarding the weekend work.

#### Unscheduled Work Workflow – On Call Services
PM shall ensure Local oversight works with the COR(s) as the focal point, RQOO is the Contractor’s customer. Local oversight shall ensure that all services are coordinated through CORs and recorded according to the PWS.  
Local oversight shall provide daily updates to CORs, ensuring timely Local oversight to COR dialog for all Contractor edits or changes to contractor’s On Call Plan and ensuring COR acceptance prior to releasing Contractor personnel to work the deviated schedule. 
The Local oversight shall provide, throughout the service call, timely updates for COR acceptance to any Contractor edits, changes to the On Call Plan being worked, and other requested COR information regarding unscheduled work.

      - Provide Complete CDRL Submittal & Deliverables

Ensure that submitted documentation is free of spelling, grammar, punctuation, and technical errors; meet technical requirements (technical requirements implying PWS requirements and Government and industrial standards) for the documentation; are properly prepared; and delivered to the COR for acceptance.

      - Designate Alternate Program Manager (APM)

Designate an APM who shall be authorized to assume and perform all duties, authorities, and responsibilities of the PM in the PM’s absence.

      - Provide Supervisory Positions

Provide details concerning the hierarchical role for the Contractor employees.

      - Accomplish Briefings

Schedule and lead briefings with current meeting support documents provided.

      - Provide Planning

Set processes and procedures for the program and job planning. Describe in detail the overall results of this planning for each contract service. Explain each contractor’s employee assigned to tasks or a job(s).

      - Provide Assignment of Duties

Use proven processes, analyses, and best practices to:

##### Select or assign personnel with the appropriate combination of education, training and experience (including supervisory experience if required) and who are otherwise well qualified to perform the required services.

##### Plan and manage jobs.

##### Provide directions to Contractor employees who are working to complete WOs, and ensure that tasks are completed to cost, quality, schedule and meeting technical standards.

      - Accomplish Administrative and Financial Management Functions

Exercise administrative and financial management functions to include managing the services to add or remove resources to meet the schedule of services set by the PM, commitments and expenditures, and making recommendations on funding and funding changes.

      - Track & Schedule Services: EPM, and WOs

Manage services to ensure Electricians are performing EPM (Electrical Preventive Maintenance), WOs, and services required in the PWS, which are tracked and documented according to the PWS.

      - Provide Processes/Procedures, Lists, Templates, and Plans

Create processes/procedures, templates, and lists detailing work and itemizing information for all Contractor employees and services provided. Maintain and manage processes, procedures, templates, and lists for currency and accuracy for inclusion into plans. Review templates, lists, processes and procedures with CORs and ensure services are meeting Contractor’s Quality Control, Safety and Security Plans.

      - Track Training

Track training requirements for contract employees to ensure these requirements are scheduled and met prior to becoming delinquent, record the training requirement for each employee, include employee name, training accomplished, date and time, and submit to COR for acknowledgement and tracking of required training/education. Training shall be tracked on a yearly cycle and shown on schedule 60 days in advance with e-mail to COR.

      - Response Time to On Call Service for Unscheduled Work

PM shall be available by phone during Core Hours with response time of fifteen (15) minutes. After core hours until midnight, and on weekends the PM shall have a response time of one (1) hour over the phone, with COR.  This shall be detailed in the submitted T&MWP.

Local oversight On-Call services responding to the PM’s call shall be from the COR accepted weekly plan, Daily Site Activity Plan. The response time for the Local oversight is one (1) hour, after COR review with PM, to be On-Site and ready to review the Work Plan with the COR.

    - Perform Maintenance on Electrical Equipment and Systems

      - Electrical Equipment Maintenance, Repair and Testing

The NAC shall perform electrical equipment maintenance, testing and repair services on electrical power systems and loads.

The NAC shall write an Acceptance Test Plan (ATP) to detail the latest ANSI/NETA ATS (Acceptance Testing Standard), MTS (Maintenance Testing Standard), and ECS (Electrical Commissioning Standard) to incorporate the EPM schedule, RQOO’s electrical equipment test plan, and the services that are to provide the input for creating, reviewing and complying with the COR accepted ATP. The ATP shall cover RQ’s R&D electrical distribution systems and connected loads. These loads include but are not limited to generators and motors; motor-generator (M-G) sets; switchgear; motor control, protection, metering and monitoring devices, starting systems (rheostats and field application panels) and starters; air, vacuum and oil circuit breakers; general low voltage distribution equipment; transformers; high voltage rectifiers; cooling systems and other specialized equipment to include DC power supplies, batteries, and systems of all varieties. Certain cables are required to be tested as well.

These services shall be NETA compliant unless otherwise stated and specifically include:

        - Acceptance Testing.

        - Maintenance Testing.

        - Electrical Commissioning Specifications.

        - Equipment inspections.

        - Preventive maintenance and repairs.

        - Problem/trouble diagnostics and repair.

        - Equipment calibration.

        - Equipment adjustments.

        - Equipment overhaul.

        - Equipment operation for the purpose of troubleshooting and verification of operability following maintenance.

        - Specialized parts adaptation or minor fabrication (primarily using or customizing old parts to make repairs when manufacturer’s parts are no longer available or for configuring custom applications e.g., copper buss bars, hinged boxes, sheet metal, circuit breaker and bucket assemblies and conduit/wire ways).

        - Control and protection equipment replacement, including removal of old equipment and installation of this new equipment.

        - The Contractor shall perform third-party acceptance testing verifying equipment condition is acceptable to set NETA ATS, MTS and ECS standards prior to obtaining government acceptance.

      - Provide Electrical Test Equipment

##### The Contractor shall provide electrical test equipment that is specialized and intended for use in low and medium voltage applications up to electrical substation level, 15kV maximum, and specific to that detailed in the ANSI/NETA ATS, MTS and ECS for the current year’s standards. The necessary electrical test equipment shall be on site for scheduled work and must be provided within twenty-four hours for Contractor use in the event of an emergency or if normally used shared electrical test equipment is not available. Government shared equipment, shared test equipment, may be made available upon COR acceptance of Contractor provided T&MWP, PWS section for Facilities/Furnishings & Services, Equipment, Data, & Materials and subsection for Shared Equipment.

        - The Contractor shall lease through Doble an up-to-date and properly calibrated Doble M4000 Insulation Analyzer (without M4200 Controller), Resonator (Type C), Doble Test Assistant, C-RIV Coupler, and the M4300 Transport.  The lease shall be yearly for the base and option periods and shall include CORs access to the Doble share site and call center and that training is provided for CORs and Contractor NETA personnel. This equipment shall be utilized for EPMs, stored, and maintained by the Contractor on WPAFB, RQOO Contractor office location, for use only within RQ facilities.

      - Electrical Meter and Relay Repair (NETA Level 3 or Higher)

The Contractor shall perform electrical meter and relay calibration, setting, validation and repairs. This work will be performed on electro-mechanical and solid-state meters and relays. Using current testing and other methods the Contractor shall determine, set, and document electromechanical and electronic protection relays, relay calibration data and metering. The Contractor shall provide a cost-effective means to accomplish bench-level repairs of meters and relays that can be readily modified.

      - Electrical Equipment Testing (NETA Level 2 or Higher)

The Contractor shall perform equipment commissioning, acceptance testing and maintenance testing. The Contractor shall provide an in-depth detailed set of electrical equipment ATPs with test sheets describing scheduling, maintenance actions, and testing interval. This plan shall incorporate WOCs & EPM electronic data storage-based systems and TP for NETA acceptance and maintenance testing with PM set planned review meetings. The EPM application stores and coordinates detail, e.g., location, type and naming convention, test cycle periods and requirements for each test cycle period for electrical or electro-mechanical items, requiring maintenance to ensure a safe and functioning electrical distribution system from electrical source to research loads, that are tracked as part of this contract. Items in EPM are tracked and maintained as per a testing cycle set by the manufacturer, industry standards and/or engineering review.

      - Lead-Covered Cable Splicing

If necessary, the Contractor shall sub-contract splicing of lead-covered cable. The cable shall be tested to NETA, Institute of Electrical and Electronics Engineers (IEEE) and National Electric Code (NEC) standards prior to being put into service. The personnel performing the lead splicing or terminations shall be certified to accomplish this task.

      - Disposal of Electrical Equipment and Materials

The Contractor shall prepare, consolidate, arrange, and organize those materials that are to be disposed of or turned in through government organizations. The Contractor will coordinate these efforts through the COR.

    - Perform Electrical Equipment Installation, Modification & Upgrade

      - The Contractor shall provide complete system modification, upgrade, replacement and installation services for electrical distribution systems, electrical equipment, and specialized electrical systems as listed in this PWS.

        - The Contractor shall also provide protection, metering, relays and controls.

      - The Contractor shall utilize engineering services in support of the maintenance for these systems.

    - Perform Electrical Engineering Services

The Contractor shall provide electrical and power engineering services. These services shall be used for all phases of NETA acceptance and maintenance of Electrical Supply and/or Distribution Systems and Specialized R&D support equipment. Specific detailed support includes evaluation and reporting status of all NETA completed test sheets and keeping current the EPM data base. This support also includes maintaining and updating RQOO’s EPM application(s), modification of RQOO’s electrical equipment testing plans, and results (equipment failure remediation, preparation of schematics, electrical test and checkout and other tasks requiring specialized expertise).

The Contractor shall provide Technical Reports/Studies from the engineering services as generated through work initiated/described in this PWS.

The Contractor shall supply detailed documentation for Commissioning Plans, ATP, acceptance and maintenance testing, and what is to be accomplished by the EE services for support.

The Contractor shall ensure that the ATP include details from EPM, RQOO’s electrical equipment test plan, and NETA documents, and for the Contractor services that will support Electrical Design and Drafting Services for Design Drawings/Models and Associated Lists and what these services are to accomplish when assigned work pertaining to the Test Plan.

The Contractor shall provide licensed professional Electrical Engineering to assist with maintaining research facilities. The intent of this requirement is to establish what NETA deems to be the necessary responsibilities of a PE to ensure quality control. The licensed professional engineer is required by NETA to personally perform or review and approve all power system studies as well as any other engineering analysis for all phases of maintenance and integration and evaluate unusual test results to make recommendations for corrective actions.

Specific detailed support includes evaluation and modification of Commissioning Plans, Acceptance Test Plans and Test Procedures and results oversight, maintenance and modification of included equipment, equipment and system specifications, evaluation and purchasing. This support also includes electrical system studies using Power Tools for Windows by SKM (load studies, fault current, protection coordination, transient motor starting, arc-flash), preparation of schematics, collaboration with Commissioning, Acceptance and Maintenance Electrical Engineer for test and checkout and other tasks requiring this specialized expertise.

    - Technical Report Study/Services

The Contractor shall supply technical reports, studies, notes, specifications, layouts, sketches, drawings, and conceptual design models as needed.

The Contractor shall supply detailed documentation for the ATP, acceptance and maintenance testing, and what is to be accomplished by the EE services for support to NETA standards.

### The Contractor shall ensure that the Acceptance Test Plan and Test Procedures include details from EPM, RQOO’s electrical preventative maintenance device database, and NETA documents, and for the Contractor services that will support Electrical Design and Drafting Services for Design Drawings/Models and Associated Lists and what these services are to accomplish when assigned work pertaining to the Acceptance Test Plan and Test Procedures.

    - Perform Electrical Design and Drafting Services for Drawings, Models, and Associated Lists

These services shall be performed, as required, for electrical power equipment system modifications and installations including the documentation of existing systems. These services shall be performed to a level 3 CAD operator capability and standard.

The Contractor shall supply conceptual design models.

The Contractor shall supply detailed documentation of procedures for supporting data input and storage of electronic and hardcopies of CDRL and WO submittals.

The Contractor shall ensure that the Acceptance Test Plan and Test Procedures include the detail for Contractor services that will support Electrical Design and Drafting Services for Design Drawings/Models and Associated Lists and what these services are to produce for the Acceptance Test Plan and Test Procedures. (A001 - A008)

When performing electrical design and drafting services, the Contractor shall:

#### Produce designs, create and modify documentation that include electrical single line and three-line diagrams, equipment layout and installation drawings, manhole drawings, electrical controls wiring diagrams, circuit diagrams, panelboard schedules, and equipment specifications.

#### Produce designs that are compliant with the most current Industry and Government standards.

#### Produce associated lists including design/drawing criteria, what the design/drawing will contain, and what the Contractor PM and COR will use to ensure that the design/drawing meets the Contractor’s Quality Control Plan. These lists, design and drawing criteria, shall be maintained by the Contractor.

#### Maintain and input new additions to RQOO’s electrical distribution software model.  This model shall describe all electrical distribution systems that are under the scope of this contract.

#### Enter, maintain, and modify data in electronic data system(s), EMC Electronic Storage Location(s) (EESLs) and hardcopy files.

#### Use Government provided hardware and software, where applicable.

#### Use Computer-Aided Design and Computer-Aided Engineering (CAD & CAE) systems compatible with government provided CAD and CAE systems.

#### Use software that conforms to AFRL/RQOO standards for the preparation, revision, and maintenance of drawings and documentation

#### Support the design and creation of electrical system labeling.

    - Management of Electrical Equipment and Systems Data

Unless otherwise stated in this PWS or with prior written COR and CO approval to state otherwise, all documentation, e.g., reports, specifications, material ordering, and other documentation required in this PWS, shall be stored on RQOO electronic data storage areas referred to as the EMC Electronic Storage Location (EESL) or on other electronic systems specifically designed for this data.

The Contractor shall ensure that the EESL is requested and created as defined in the CDRLs within ten (10) business days from contract start date.

The contractor shall store soft/electronic for all services and data, WOs and support material, designs, drawings, tests and reports supplied on this contract. When needed, hard copies may be requested.

The Contractor shall supply technical reports-studies/services and conceptual design models. The Contractor shall supply ATP.

The Contractor shall ensure that the ATP incorporate EPM and RQOO’s existing electrical equipment cycle/periods for testing, include references to ANSI/NETA; ATS, MTS and ECS, management of electrical equipment and systems data, and detail the contract services matched to Contractor personnel matched to task or subtasks that shall support the ATP.

For preparation, delivery and electronic storage into the EESL of submittals on this contract the Contractor shall:

#### Use an RQOO provided temporary location for electronic storage if delivery to the EESL is not directly possible. The Contractor shall have 20 business days to transfer all material from the temporary location to PM set electronic storage and filing system under the EESL file.

      - Utilize RQ’s applications:

        - Work Order Control (WOCs)

        - EMC Cost (EMCC)

        - Electrical Preventative Maintenance (EPM).

      - Utilize and maintain RQ’s hard copy files.

      - Design, utilize, and maintain Contractor set and RQOO approved EESLs file system.

      - Design, utilize, and maintain Contractor set and RQOO approved ATP.

      - Design, utilize, and maintain Contractor set and RQOO approved TMWP.

      - Utilize and Maintain WOCs.

      - Utilize and Maintain the EPM application system for current, new, and added, electrical equipment and systems.

      - Utilize and Maintain the EMCC application system.

      - Manage Historical Maintenance and Testing Data Report-Study/Services

The Contractor shall keep historical maintenance data on every piece of equipment serviced. Depending on the nature of the work this may be in a software application system and a hardcopy input into electrical equipment inventory. The maintenance data resides in the Government’s EMC hard copy files, the EPM system as inventory control records, and the Power DB database. Test sheets are also stored on network locations both of past test and blank future test sheets. Regarding files and this system, the Contractor shall perform the following functions:

        - Manage EPM inventory control records in soft and hard copy.

        - Add data on newly acquired equipment to these systems, including the acceptance test report results.

        - Make system entries to capture all maintenance performed on each piece of equipment.  These entries shall be made as work is performed to keep all records current.

        - Ensure that maintenance data entries indicate what work was performed, discrepancies noted, how discrepancies were resolved, date work was completed, and maintenance test report results.

        - Make data entries based on hardcopy Maintenance and Test Reports completed by the technician performing the work, reviewed by the electrical engineer, signed by the PM, and reviewed by a COR.

        - Maintenance and testing completion shall be indicated on the report. Generate and electronically deliver a Technical Report-Study/Services for each item in the EPM system or WO action completed on an EPM item.

      - Provide Project and WO Planning

The Contractor shall provide EMC project planning to include equipment maintenance, testing schedules and cycles, services, tasks and subtasks assigned to Contractor personnel matched to specific services. Forecasting schedules and charts shall detail Contractor’s tracking of CDRLs and WO due dates set by the PM and derived from purchase orders (POs), CDRLs, WOs, equipment scheduled for EPM routine maintenance and testing, research and research support requirements, Base Civil Engineering (CE) and utility requirements, and Contractor personnel manning this Contract. Include details for follow-up actions on any equipment having an outstanding maintenance discrepancy on equipment skipped during previous month(s) EPMs, EPM items placed on hold for parts or other issues, and WOs.

Preventative maintenance and testing schedule shall be based on the Contractor’s CDRL submittals and RQOO’s approved ATP as detailed in CDRLs and equipment listed in EPM. The PM shall use the latest information from the EPM application and RQOO’s existing electrical equipment test plan to aid with generating ATP to accomplish EPMs. COR acceptance is required for interim and completed ATPs prior to PM requesting COR acceptance of the DSAS. The PM shall ensure workflow procedures are in place for review of submittals that require COR approval and submittal has been approved prior to submitting on the DSAS for acceptance.

WOs shall also be tracked as part of project planning. WO work plans, for individual WOs, shall provide lists for equipment testing, tools, lifts, POs for materials and parts, and any items required to accomplish the PM or Foreman detailed task or subtasks; services matched to individual personnel; and schedules set to PM or Foreman detailed task or subtasks.

These WO work plans shall contain narratives for Contractor personnel. Narratives, provided by the PM and Foreman, shall:

        - Set milestones and goals with specific dates and times for completion of tasks and subtasks to include work breakdown structure for services.

        - Provide updates for delays or interruptions, RQOO customer feedback, and accepted work changes

        - Ensure Contractor’s personnel comply with standards and other contract materials that define work and workplace guidance.

        - The WO work plan shall be approved by the COR prior to placing WOs on the DSAS.

      - Provide T&MWP and DSAS

The PM shall provide, for all services, a Weekly Plan/Weekly Schedules for the DSAS.

The Contractor shall utilize the COR accepted EPM schedules, WOs, CDRLs, plans, defined work-flow patterns and daily management actions to gather information to generate this weekly schedule. Work locations referenced in the report shall correspond to those detailed in the COR accepted associated plan.

PM shall ensure WOs, plans, and workflow procedures are vetted for COR acceptance prior to being placed on weekly schedule or requesting COR acceptance of the DSAS.

      - Manage and Maintain Historical Records of Material

The Contractor shall keep historical records of all materials purchased and received in the performance of this contract. These are hard copy and electronic records. The purchase order forms for materials reside in the Government’s EMCC data system as material control records. Regarding this system the Contractor shall perform management and record keeping of the Electrical Equipment data base material control records in EPM, EMCC and WOCs data systems and EESLs and hard copy files. The Contractor shall track POs for equipment and materials supporting WOs.

      - Manage and Maintain HAZMAT Record Management

The Contractor shall support RQOO in conjunction with other program management functions to oversee and update all hazardous materials (HazMat) used in conjunction with this contract. The Contractor shall follow applicable HazMat management requirements and Base Facility Standards.

      - Manage and Maintain Records

The Contractor shall obtain and keep copies of the following documents:

        - Electrical Equipment information files

        - Maintenance and Test Reports

        - Electrical Design, Study and Analysis Documents

        - Purchase Orders

        - Invoices

        - Packing Lists

        - Warranties

        - Relevant Equipment Catalogs, Maintenance Manuals, and Specification Sheets

      - Providing Record Disposal

Under this contract the Contractor may be required to dispose of outdated records or documents. This shall only be done at the specific written direction signed by the COR and IAW Government records management regulations.

      - Management of Electrical Equipment and Systems Data Plan

The Contractor through the PM shall ensure items provided in this subsection Management of Electrical Equipment and Systems Data and subsections, EMCC, WOCs and EPM are part of a plan for Management of Electrical Equipment and Systems Data detailed in the T&MWP. This plan shall provide procedures for Contractor personnel matched to specific services ensuring Management of Electrical Equipment and Systems Data support criteria is completed as required.

    - Acquisition Support for EMC

The Contractor shall use COR and CO approved procedures to procure equipment, materials, software, spare parts, expendable supplies, and services in support of AFRL/RQOO Electrical Maintenance and support operations described in the other objectives of this PWS. The Contractor shall ensure that the ATP include acquisition support and detail the Contractor services that shall support these procedures in the T&MWP.

      - Purchase Procedures – Material and Subcontracted services

General Procurement Requirements. The following general procurement requirements apply:

        - Contractor’s purchases, Purchase Orders, shall only be made with prior written COR approval and signature.

        - The Contractor shall follow applicable Government compliant purchase policies and procedures when making purchases under this contract.

        - The Contractor shall prepare material procurement specification sheets (prescribed by the Government through EMCC) for inclusion with the Contractor’s proposed purchase approval request.

        - Once approved by the COR, the Contractor is authorized to make purchase up to the estimate stated in their Government approved quote.

        - All purchases become property of the U.S. Air Force.

        - The Contractor shall maintain copies of all purchase orders, packing lists, and supplier invoices.

        - The Contractor shall request reimbursement IAW the invoicing instructions.

      - Receiving & Shipping

The Contractor shall be responsible for receiving, inspecting, testing for serviceability, and obtaining government acceptance of all ordered goods and services. Additionally, the Contractor shall be responsible for verifying the accuracy of third-party invoices and making payment for accepted items or services.

The Contractor shall, in a Receiving & Shipping Plan as part of the T&MWP, detail the services matched to Contractor personnel that shall provide PO invoicing, shipping and receiving, inspecting, testing for serviceability, and provide process and procedures for these Contractor personnel providing these contract efforts.

The Contractor shall perform third party acceptance testing verifying condition of received equipment is acceptable to standards prior to obtaining government acceptance, COR acceptance, as per ANSI/NETA ATS, MTS and ECS standards documented in the ATP. These acceptance testing procedures shall be referenced in the Receiving & Shipping. PM shall ensure that COR has accepted the acceptance testing prior to requesting government payment.

The Contractor shall be responsible for shipping items that pertain to this Contract.

      - Expenditure Report

When requested, the Contractor shall provide documentation to support monthly invoices, detail spending rates, include forecast for future spending, and detail procedures in the T&MWP.

      - Items Purchased

With prior COR approval, COR Signature on EMCC generated PO, the Contractor shall purchase items in the following categories. Individual items more than $5,000 purchased under Time and Materials contract line item number CLINs shall require authorization from the CO.

        - Services:

##### The Contractor shall acquire the equipment and/or services of personnel or other agencies which cannot be performed by the Contractor. Subcontractors shall be vetted for adherence to standards established in this PWS and approved by the CO before starting work.

        - Materials:

##### The Contractor shall purchase materials required to operate and maintain the equipment in this contract. This includes but is not limited to items such as conduits, boxes and conduletes, circuit breakers, fuses, wire, cable, fasteners, and miscellaneous hardware.

        - Equipment:

##### The Contractor shall be required to purchase both standard off-the-shelf and nonstandard types of equipment. This includes but is not limited to diesel and gas generators, Uninterruptible Power Supply (UPS) systems and other types of proprietary equipment. The Contractor shall have the ability to purchase, modify, and utilize standard off-the-shelf equipment that may be more cost effective than specially developed equipment.

        - Expendable Supplies:

##### The Contractor shall purchase expendable supply items such as rags, drill bits, blades (saw and knife), and other items that are fastened to power tools for cutting and or shaping/forming metal, wood or concrete.

        - Spare & Replacement Parts:

##### The Contractor shall identify spare parts and establish the minimum number of electronic or electrical modules/parts, electromechanical parts, connecting materials, electronic components, and mechanical components to assure equipment availability. An absolute minimum number of modules, components, and materials shall be established. The Contractor shall have the capability to provide all the items under emergency conditions and to establish a maintenance support package.

        - Software:

##### Certain devices used to undertake testing require computer software to function and record the testing data. The Contractor shall maintain fully Government licensed copy/copies of this software with the test equipment and upgrade it as new releases are made available by the software manufacturer. In addition, specialized analytical software is used for supporting electrical engineering efforts in the areas of fault current analysis, coordination studies, motor starting analysis and arc flash prediction. This analysis software, PowerTools for Windows by SKM, is government owned and available for the Contractor’s use.

        - Hazardous Materials

##### The Contractor shall purchase HazMat as per this section and Environmental Management Requirements section.

      - Receiving Items and Making Payment

        - Rapid Procurement:

##### The Contractor shall establish purchasing procedures that may be required to procure electrical equipment or materials when system failures or emergencies occur. These procedures should include an internal priority system and a method to expedite the approval process. Expedited purchases are not exempt from COR approval or the application of standard Air Force procurement policies and practices.

        - Manufacturers’ Literature, Drawings, and Software

##### The Contractor shall be responsible for obtaining and storing literature, manuals, drawings, and fully licensed software applications (where applicable) that accompany purchased items or services on behalf of the government.

        - Warranties

##### The Contractor shall obtain, register, and retain all manufacturers’ commercial warranties. The warranties shall be written such that AFRL/RQOO holds the warranty for any and all equipment purchased on this contract. If manufacturers’ warranties are optional or if manufacturers offer optional warranty periods and/or provisions, the Contractor shall obtain prior written authorization from the Government as to which warranty periods and/or provisions to purchase along with verification of funding. The Contractor shall pursue remedies under the warranty if equipment or materials fail during the warranty period.

      - Acquisition Support for Receiving & Shipping Plan

The Contractor through the PM shall ensure items listed in this subsection Acquisition Support for this contract and subsections, EMCC, WOCs and EPM are part of the Receiving & Shipping Plan detailed in the T&MWP, detailing procedures ensuring acquisition support criteria is completed as required

    - Cleanup of Workspaces

The Contractor shall ensure that all electrical maintenance facilities and work areas made available to the Contractor are kept clean. This includes all Electrical Supply and/or Distribution System spaces, Government furnished facilities, areas where contracting services for this PWS are performed and outside approaches (stairwell, steps, walkways and vehicle drives) for entrances and exits listed buildings, spaces and areas, and sinks and storage locations.

      - Cleaning:

#### Defined by where and what is to be cleaned and the type of cleaning to be accomplished includes vacuuming, sweeping, dusting and mopping: floors, walls, doors, windows, windowsills, pedestals, and ceilings. For example, inside floor areas of electrical and mechanical areas shall be vacuumed while all outside spaces swept with a broom. Cleaned implies spills, smudges, dirt and other types of debris have been removed from walk surfaces/floors/steps and other surfaces. Contractor shall ensure all floors are vacuumed rather than swept unless debris is such that it requires floor sweeping and hand-wiping prior to vacuuming. Ensure that trash receptacles are emptied.

##### The Contractor shall clean items to ensure that they are free of dust, dirt, rocks, stones, grease, fingerprints, and other foreign material.

##### Floors are to be dry or wet mopped, vacuumed, scrubbed, or swept to remove dirt, dust, debris, grease or standing water. All floor surfaces include cracks, chips, holes, corners,
edges, steps, curbs, and any type of penetrations shall be vacuumed.

        - Walls are to be wiped down and cracks, crevices, or other nooks and crannies where walls meet the floor, other walls or other structures shall be first swept and then vacuumed. Cobwebs along with any other natural or manmade debris shall be cleaned/removed.

        - Window ledges, pedestals, and steel building structure being vacuumed and then wiped down with a damp cloth or some type of cleaning soap or mixture. Areas holes or/and pockets shall have all debris removed by hand or vacuum and then wiped down.

        - Shop and work light fixtures reachable from a 6 footstep work platform or ladder are to be cleaned.

        - One-time work locations, cleaned by electrician(s) accomplishing the WO, are to be kept broom clean and floors protected from any and all types of spills or foot traffic dirt during work progress, vacuumed, if carpeted area, at the end of each day and fully cleaned to include vacuuming at the conclusion of the job.

        - Personnel shall be appropriately trained and oriented to provide these services in hazardous low, medium, and high voltage areas (Electrical Supply and/or Distribution System spaces, electrical vaults, and industrial areas). The Contractor shall control access to these areas.

        - Outside walkways to the street shall be swept free of all debris including a portion of the street where debris collects at the level where street and curb meet to a distance of 10 feet up the street on both sides of where walkway or ramp meets the street.

        - The Contractor shall ensure that the ATP include cleaning of workspaces, rooms, areas, and pathways for entrances and exits from these locations; detail hazards that are involved with each specific area or room; and detail the Contractor services that shall support this plan.

    - Briefings and Meetings

The Contractor through the PM shall participate, conduct and chair briefings/meetings (B/M).

Nothing discussed in any B/M, or discussions between the Government and the Contractor, shall be construed as adding, deleting, or modifying contractual agreements without written authorization from the CO.

B/M invites, other than scheduled by Contractor PM, shall only be accepted by Contractor personnel with prior COR approval, requested by the PM, and only attended if a COR or CO is in attendance. For staff other than the PM to attend meetings the PM must be in attendance with a COR or CO.

The Contractor PM shall conduct and chair briefings for PWS Required Meetings, providing to attendees current and complete meeting support documents, presentation materials, and ensure meeting minutes are provided in a timely fashion after the meeting.

The Contractor PM shall participate in B/M not set by the Contractor; with current meeting support documents provided, presentation material; ensure a COR is in attendance, and meeting minutes are provided as per PWS.

      - Contractor shall participate in Kickoff Meetings.

      - Contractor shall schedule via Outlook and conduct B/M(s) (Status and Review Meetings)

        - Scheduled B/M - The PM shall hold one B/M a week to review PWS requirements, all work orders, and the two week look ahead Daily site Activity schedule.

        - Ad Hoc B/M(s) - The Contractor shall take the lead for scheduling (date, time, and place) and Chair in Ad Hoc B/M(s). The PM shall brief the Government on results of efforts undertaken and future efforts during these meetings.

        - Additional Contractor set B/M, ensuring PM is proactive with meeting PWS requirements and providing the Government results of efforts undertaken and future Contract direction and efforts during these B/M.- The Contractor shall take the lead for scheduling (date, time, and place) and chair these additional B/M.

      - The Contractor shall develop a standardized B/M agenda form. The form shall be submitted for COR approval prior to being implemented during B/M in addition to presentation material.

      - Contractor personnel shall take minutes in all meetings attended.

      - PM shall develop a standardized B/M minutes form (the form shall be submitted for COR approval prior to being implemented for B/M submittals.

      - The Contractor shall ensure that the ATP, and RQOO’s existing electrical equipment test plan and EPM are included in PM set monthly meetings for appropriate CDRLs with detail that supports the inclusion of RQOO’s existing electrical equipment test plan and EPM into ATP and T&MWP.

      - Additional B/M may be required, if in the Government's opinion, they are needed to facilitate the content of PWS items received and Contractor activity.

    - Subcontract Management

The PM shall be fully responsible for management, control, and performance of any subcontractor used in conjunction with a WO assigned under this contract. Use of a subcontractor shall not relieve the Contractor of responsibility or accountability in the timely and cost-effective execution of the PWS requirements.

Facility modifications that are performed by subcontractors shall be fully documented by the Contractor. In these
cases, the Contractor shall obtain prior approval from the CO of any subcontractor arrangement.

  - Remedies For Less Than Satisfactory Performance

In addition to the contractual remedies available, the government management team will also utilize, but are not limited to, the following remedies for less than satisfactory performance: Corrective Action Requests (CAR), Letter of Concern (LOC), input of validated quarterly Performance Assessment Report (PAR) information into annual Contract Performance Assessment Report (CPAR) ratings.

  - Deliverables

Reports and/or other data required shall be submitted IAW the CDRL.

The Contractor shall provide internal scheduling of activities and milestones; provide status; outlining contractor
activity and progress toward accomplishment of tasks; internal planning, program planning; describing in detail the overall results of the effort.

    - Document Designs

### All designs and related engineering information shall be identified with the appropriate RQOO nomenclature, i.e.,
identification shall consist of task, subtask, and project number as an ancillary support service. In all cases the drawing and engineering information of a project will be identified according to the nomenclature system in use at the time the project begins.

    - Government Access to Internal Data

The Contractor shall provide Government access to any data generated in compliance with the tasks completed
IAW this contract.

    - Contractor’s Employees Workplace Guidance

The Contractor shall provide contractor documentation that states employee organizational standards, charts, rules,
regulations, codes, technical orders, operating instructions, directive and work agreement documents at the local,
|state and federal levels for workplace conduct, performance and behavior. Contractor shall provide documents
federal, state and local government rules and regulations and Work Agreements defining Contractor personnel break
and lunch periods and include these, rules, regulations and Work Agreements by reference (title and numbered, page,
paragraph, and specified periods) as part of the T&MWP.

    - Service Schedules

Services scheduled and performed under this contract shall be accomplished IAW the Contractor’s
provided and COR accepted T&MWP.

    - Staffing Plan

The Contractor as part of the T&MWP shall provide a Staffing Plan. The Staffing Plan shall provide details as to
what position each member of the contractor’s staff holds and their role in fulfilling the PWS.

    - Work Stoppage and Workforce Stability

The Contractor shall maintain a stable workforce while minimizing turnover and managing their personnel in such a way as to minimize the impact of any turnover and/or disruption to the customer and/or mission. The Contractor shall ensure continuation of services during personnel absences due to sickness, leave, and voluntary or involuntary termination from employment such that there is no negative impact to the Government mission schedule.

### Professional Appearance and Demeanor

### Contractor employees while on WPAFB shall present a neat appearance and be easily recognized as Contractor employees, and wear the government provided Common Access Card (CAC) on a lanyard (provided by the government) around the neck on the front of the body where the CAC is always visible. One exception is if the lanyard poses a hazard to the contractor employee and in this specific case the lanyard with CAC shall be carried in a pocket of the contractor employee. Once clear of the hazard it shall be worn as previously described.

      - Conduct themselves in a professional manner per the PWS requirements and Contractor standards.

      - Shoes shall cover the entire foot and shall not be gym shoes, sneakers or tennis shoes.

      - T-shirts of any style or type shall not to be worn by office staff.

      - Worn outer or displayed garments/clothing/ Personal Protective Equipment (PPE) shall be free of any type or style of printed/written words, prints and designs with the exceptions of:

        - Contractor company logo or name

        - Affiliated organizations (organizations with which the Contractor has work agreements) logo or name and free of slogans and other language or prints,

        - Manufactured standards to meet PPE requirements, and/or the manufacture’s logo or name on PPE.

      - Garments/clothing/PPE shall be free of rips and tears.

    - Privacy Act

Work on this contract may require that Contractor personnel have access to information which is subject to the Privacy Act of 1974. Contractor personnel shall adhere to the Privacy Act, Title 5 of the U.S. Code, Section 552a and applicable agency rules and regulations when handling this information. Privacy Act information is considered sensitive and appropriate safeguards shall be implemented by the Contractor. The Contractor is solely responsible for ensuring all Contractor personnel are briefed on and comply with Privacy Act of 1974 requirements.

    - Quality Control Plan

Contractor derived QCP shall define specific quality standards, practices, procedures, and instructions relevant to and applied to services in the PWS and contractor employees performing work detailed in the PWS. To ensure Quality Control (QC), Quality Assurance (QA), and Quality Management (QM), the QCP shall detail Contractor’s
allocation of responsibilities, authority, resources, and how their work fulfills the PWS for each contractor employee.

    - Safety Plan

Contractor Safety Plan shall define standards, practices, procedures, and instructions relevant to and applied to contractor employees performing services and jobs detailed in the PWS. Safety Plan shall detail allocation of responsibilities, authority, and resources for each contractor employee and jobs and services, from the PWS, they are detailed to complete.

    - Security Plan

Contractor Security Plan as part of the T&MWP shall define standards, practices, procedures, and instructions relevant to and applied to contractor employees performing jobs and services detailed in the PWS. Security Plan shall detail allocation of responsibilities, authority, and resources for each contractor employee and jobs and services, from the PWS, they are detailed to complete. Additionally, the Security Plan shall ensure all contractor employees:

      - Are only accessing roads, areas, buildings, floors, rooms, hallways or other spaces detailed as part of their duties.

      - Safeguard provided keys, key cards or any other form of access.

      - Able to identify by sight CORs and all contract employees

      - Ensure no others (persons not employed by the contractor) enter areas, buildings, floors, rooms, hallways or other spaces using means of access gained by Contractor employees.

      - Have all contract documents and training completed according to times stipulated for these items. Reference the RQ security policy.

  - Industry Standards (Technical Standards)

Foremen, managers, electricians, electrical engineers and any member of the contracted staff responsible for
installing, operating, testing, or modifying electrical distribution equipment or responsible for direction of others
performing this work under this contract shall have experience with, fully understand, and apply the most current NETA, NFPA 70 & 70E, and National Electrical Contractors Association (NECA) standards. Planning and implementation of this work will be evaluated based upon general guidelines in standards such as the NECA Manual of Labor Units and NECA reference manuals/documents.

  - Government Specific Standards (Technical & Operating Instructions)

Contractor personnel shall apply and follow Federal Government, United States Air Force, WPAFB, AFMC,
AFRL/RQ, law/manuals and industrial standards. Current UFC’s, the WPAFB IFS and AFRL/RQ OI 32-10 East are included in these standards. Applicable Government regulations, directive, and/or guidance, and contract documents will apply to all work performed on Government facilities.

## Scheduled and Unscheduled Work

All planned work shall be documented in Contractor provided DSAS. Unscheduled maintenance issues may arise during or outside normal duty hours.

### The PM shall provide, as part of the T&MWP, workflow and plans for scheduled and unscheduled work specifying the contract service that each Contractor Employee is to provide. Contractor employees shall utilize contractor-provided, COR accepted schedules, plans, and plan templates.

### The Contractor through the PM shall provide DSAS templates to cover scheduled and unscheduled maintenance.

### The DSAS shall provide the detail for scheduled work derived by the PM from COR accepted plans.

### Unscheduled work occurrences shall be delivered to the PM by a COR. The PM shall review the event and possible solutions with the COR. The Contractor will follow the agreed upon company On-Call service procedure to respond to events that occur after normal duty hours with the appropriate contractor personnel and within the timeline specified below.

### In cases where research is interrupted by a system failure or other higher priority event, RQOO will notify the PM of the requirement of DSAS changes or work outside of Core Hours. The potential always exists that a percentage of DSAS shall be interrupted to accomplish unscheduled maintenance, i.e., work to restore research or research support to planned operations.

    - PM shall provide a DSAS detailing schedule work covering Work Hours

    - The PM shall provide DSAS covering four (4) planned weekends, ten (10) hours for each Saturday and ten (10) hours for each Sunday, scheduled electrical power outages. These outages are to perform planned/programmed maintenance covering EPMs and WO items. The DSAS submitted shall provide to these planned electrical outages detailing process, procedures, and descriptive materials detailing work for a crew on-site that shall include Foreman and eight (8) NETA Electricians, Level 2 or higher. The PM shall provide detail for all stages of the work for all services and for Contractor employees providing these services, prior planning requirements pertaining to what electrical systems will be down and what systems will require back-up power, what is required from the client, RQOO and CORs, and issues with regards to past system performance that requires maintenance or other attention.

    - Requests for scheduled work, outside of Core Hours, shall be COR approved 30 days in advance. The PM shall review, in a PM meeting detailed specifically for the outage, the Contractor’s plan to accomplish the outage. The PM shall have provided the entire outage plan no less than two weeks prior to the set meeting. This meeting shall review only the planned outage. From this meeting shall note COR acceptance or follow-up tasks for the next PM planned review.

    - When unscheduled maintenance requires the Contractor to respond outside of core duty hours, the Contractor shall respond within one hour and be on-site within two hours of Company POC’s and COR’s phone call. This will only be utilized in cases of mission critical impact.

    - The PM or Local Oversight shall maintain communication with COR during unscheduled events and will only proceed with verbal or written approval from COR (phone call, Email, text or other messaging platform). The Local Oversight shall provide a written narrative including Unscheduled Maintenance event facts, equipment status upon arrival, steps taken to make repairs, whether the repairs were successful, amount of time on-site, time of departure, and future recommendations to make repairs or prevent the event from occurring again.

    - Unscheduled Maintenance event during Core Hours:

### The PM or Foreman will notify the COR(s) which activities on the DSAS will be interrupted by the Unscheduled Maintenance event within one (1) hour of reviewing the Unscheduled Maintenance event with the COR(s) and shall request COR’s acceptance signature. The DSAS shall be updated by the next business day.

    - Unscheduled Maintenance event Outside of Core Hours

### The Local Oversight, once on-site, shall assess the situation and inform the COR(s) of the equipment status upon arrival, the work plan, and estimated amount of time required to make repairs within one (1) hour of reviewing the Unscheduled Maintenance event with the COR(s) and then request COR’s acceptance.

    - Contractor Deviations from COR Accepted DSAS

### The Contractor shall provide updates for Contractor’s deviations to COR accepted DSAS. This shall be part of the T&MWP and ensure PM or Local oversight shall provide timely identification of issues, providing a review with corrective action items to established plan deviations, DSAS, and detail COR acceptance from Contractor’s request for deviating from DSAS prior to releasing the edited plan to be worked.

## Workflow for WOs and Work Plans

### As detailed, the Contractor employees shall be working to complete an effort directly derived from one of two options:

#### From PM and Foreman. The PM shall provide acknowledgement of WOs and unscheduled work occurrences. This acknowledgement is obtained from COR(s), during a PM set meeting with PM providing and reviewing a written assessment for each WO and each unscheduled work occurrence. The written assessment shall detail what is required by the Contractor to complete WO’s and work plans.

#### Other than PM and Foreman. COR acceptance of PM’s plan with provided schedules, DSAS, to accomplish WOs or providing On-Call Services’.

### The contractor shall detail these workflow procedures ensuring Contractor employees shall

### provide to COR accepted, Daily Site Activity Schedule, Work Plans and T&MWP.

### The Contractor shall provide PM procedures to follow workflow ensuring DSAS Work Plans and T&MWP are COR accepted. The PM as part of these procedures shall verify Contractor personnel are providing to Contractor personnel set plans, schedule and standards.

# Staffing Requirements

## Contract Service Personnel

The Contractor shall staff personnel accordingly for the following services.

#### Administrative Duties (Purchasing/Security/Office Manager)

#### Electrical Drafting and CAD Services

#### The following Services will require Knowledge of and adhering to Neta Standards:

#### Program Management

#### General Foreman Services

#### Electrical Foreman Services

#### Test Sheet Review Services

#### Electrical Engineering for Electrical Systems Maintenance Support

#### Electrical Engineering for Power Systems Analysis

#### Electrical Technicians

## Personnel Qualifications

### The Contractor shall staff this Contract with employees that have the knowledge, skills, ability, and the requisite training required for efficient and effective delivery of service under this contract. The Contractor shall, before starting work, provide proof that each employee has requisite current and valid professional certifications, degrees, training and work experience detailed in the PWS for the services the employees are to provide.

### Except where explicitly stated in this PWS, the Contractor shall perform this work using employees with the following general qualifications in this subsection and specific qualifications detailed in the subsection of each service.

### The Contractor shall furnish documentation to substantiate compliance for each Contractor employee by providing certifications, diplomas, and experience.

### The Contractor shall certify as to the accuracy and completeness of the furnished documentation.

### Contract employees must in-process and out-process through the AFRL/RQ security office. In-processing must be conducted through the security office before any work can begin. Furthermore, the FSO, must submit a visit request via Joint Personnel Adjudication System (JPAS) for all Contractors working on base. At least once a year in January, a new list must be sent to the Government security office with the names of each employee that will be working on base. After the work is completed or if an employee is terminated the CAC and lanyard must be turned over to the security office.

### Contractor employees requiring access to USAF bases, AFRL facilities, and/or access to US Government Information Technology networks in connection with the work on this contract must be US citizens.

### The Contractor shall not divulge any information regarding files, data, process activities/ functions, user IDs, passwords, or other knowledge that may be gained, to anyone who is not authorized to have access to such information.  Contractor personnel shall abide by all Government rules, procedures, and standards of conduct. Contractors will require access to Government Automated Information Systems (AIS) and shall have background investigations and security awareness training completed prior to the start of contract performance. When the period of performance is complete and/or Contractor personnel leave work on this Contract, they will have two (2) days to terminate all their network user accounts and to return all access cards and base identification badges.

### The Contractor shall ensure all personnel employed (Contractor employees) by the Contractor or subcontractors in the performance of this contract, entering the government installation, shall abide by all security regulations of the installation. Contractor personnel may be required to work in secure facilities without government personnel oversight. The Contractor may be required to open, close and perform end-of-day security checks of the government facilities IAW AF requirements. Government furnished data and information generated by the Contractor as a result of performing these tasks may be sensitive or classified. The Contractor shall be required to handle all sensitive and classified information in a secure manner, IAW prescribed security procedures and regulations.

### Contractor personnel shall not release or remove system documentation, data, or reports generated by or through use of Government systems.

## The Use of Foreign Nationals

Only U.S. Citizens are authorized to work on this effort.

### General Requirements (Knowledge, Skills, and Training):

#### Contractor employees shall have the required skill sets and certifications may not be a full or part-time employee of the US Federal Government or US uniform services ability to fluently read, write, speak, and understand American English.

## Technician Certification Programs

### INTERNATIONAL ELECTRICAL TESTING ASSOCIATION (NETA)

### The intent of the certification levels noted below is to summarize the NETA certification levels. Official breakdowns can be found though NETA’s guide which can be found on their website. To be considered NETA certified you must obtain a minimum of level 2 NETA certification. NETA certification also requires a technician to be employed by a NETA accredited company. To maintain certification, NETA technicians must complete ongoing Continuing Technical Development (CTD) credits to stay updated on industry standards and safety practices.

#### level 1- Trainee Technician: Anyone not yet certified. Basic electrical theory, safety procedures, fundamental testing techniques, requiring a high school diploma or GED and minimal field experience. Directly supervised by a level 3 or 4 as they are not certified technicians

#### level 2 – certified assistant technician: Performs limited testing and service work while generally requiring direct supervision. The individual has sufficient knowledge and experience to be qualified for assuring the safety of him/herself. Safety knowledge includes an understanding of lockout/tagout procedures and requirements, arc-flash and shock hazard analyses, and other facets of hazardous electrical energy control procedures. Requires a minimum of 40 hours of safety and 160 hours of electrical education and training and two years of experience.

#### level 3 – certified technician: Expertise in complex electrical systems, ability to supervise crews, advanced troubleshooting, extensive experience in the field. Capable of supervising NETA Trainee Level I and NETA Certified Assistant Level 2. Typical duties include, but are not limited to, performance and management of routine and moderately complex tasks and projects, record keeping, evaluation of test data, and responsibility for the safety of others. Individual is qualified to provide guidance and is capable of performing electrical power switching. Requires a minimum of 64 hours of safety training, 400 hours of electrical training, and 5 years of experience.

#### level 4 – certified senior technician: Highest level of competency, overseeing large projects, in-depth knowledge of electrical standards and regulations, can work independently, writes reports as needed, and performs complex investigations, tests, and evaluations. Requires a minimum of 104 hours of safety training, 600 hours of electrical training, and 10 years of job experience.

## Professional Positions

### PROOF OF QUALIFICATIONS AND LICENSURE:

### The Contractor shall provide, for all Professional Positions, proof of qualifications and licensure. Proof as being, but not limited to, certifications, diplomas, and licenses.

### PROGRAM MANAGER:

#### Reports to:

##### Contracting Officer’s Representative(s) (CORs).

#### Key Responsibilities:

##### Manage and oversee the electrical maintenance and testing program; planning, scheduling, and coordinating with other departments and teams.

##### Ensure compliance with contract requirements and regulations, including safety and security protocols.

##### Coordinate with and communicate with program managers and stakeholders to ensure that program goals and objectives are met.

##### Develop and manage budgets, costs and spending.

##### Provide leadership and guidance to program staff, including hiring, training, and performance management.

##### Ensure that deliverables are met, including quality control and assurance.

##### Develop and implement program policies and procedures, including change management and continuous improvement.

#### Requirements:

##### Bachelor’s degree in Electrical Engineering from an ABET accredited program.

##### Three or more years of experience in program management, as a government contractor.

##### Excellent communication, leadership, and problem-solving skills.

##### Ability to work independently and as part of a team.

##### Strong analytical and critical thinking skills.

##### Experience with program management software, including MS Project.

##### Familiarity, knowledge, and experience with NETA standards and procedures

##### Have at least five (5) years of experience managing electrical maintenance efforts with a NETA accredited company. Proof of experience in program management is required. This shall be in addition to (5) years of experience managing electrical maintenance and installation efforts pertaining to low and medium voltage ranges

##### Ability to productively use the NFPA 70 & 70E and NECA standards and written documents pertaining to the electrical trade and demonstrate these abilities as they relate to scheduling contract services and completion of tasks.

##### Demonstrated experience managing, coordinating, and facilitating a team’s efforts effectively and efficiently in a Cost or Labor Hour contracted environment within the Department of Defense (DoD).

##### Shall be an employee of the NETA accredited company performing this contract.

##### Knowledge of electrical maintenance and testing principles and practices.

##### Experience with budgeting and financial management software, including Excel.

##### SECRET security clearance or have the background that enables them to obtain a SECRET security clearance.

#### Preferred Qualifications:

##### Master’s degree in electrical engineering, Business Administration, or related field.

##### Professional certification in program management, such as PMP or PgMP.

##### Professional Engineering (PE) license.

### Electrical Engineer – Electrical Systems Support

#### Reports to:

##### Program Manager

#### Key Responsibilities:

##### Develop, and maintain electrical systems and equipment, including power distribution, lighting, and control systems to ensure reliable and efficient operations.

##### Prepare and Review electrical documents, drawings, specifications, and calculations as required to support maintenance, upgrades, and compliance activities.

##### Ensure compliance with relevant codes, standards, and regulations, including the NEC and IEEE standards.

##### Collaborate with other engineers and technicians to ensure that designs are feasible, safe, and meet project requirements.

##### Conduct site visits to gather information, collect data, and verify assumptions to support project development and implementation.

##### Provide technical guidance and support to technicians

##### Prepare and deliver presentations to stakeholders

#### Requirements:

##### Bachelor’s degree in Electrical Engineering from an ABET accredited program.

##### Five or more years of experience in electrical power distribution, preferably in a similar industry or field.

##### Current Professional Engineering (PE) license in Electrical Power or Electronics, Controls and Communications.

##### Shall be an employee of the NETA accredited company performing this contract.

##### Shall have a minimum of five (5) years of experience with the types of equipment and procedures and quality of results used for NETA Acceptance Testing and NETA Maintenance Testing

##### Shall have a minimum of five (5) years of specialized experience in power engineering including the application and maintenance of low, medium and high voltage distribution systems, DC excitation and DC control, rotating equipment above 500 horsepower (HP) including synchronous and induction motors and generators, variable speed control systems and experience with authoring load, fault current, coordination, and arc flash studies.

##### Shall be familiar with and productively use the NFPA 70 & 70E and NECA standards and written documents pertaining to the electrical trade and demonstrate these abilities as they relate to scheduling contract services and completion of tasks.

##### Shall have a minimum of five (5) years of experience performing electrical engineering for modification or installation of equipment and systems in this contract.

##### Excellent communication, technical, and problem-solving skills.

##### Proficiency in using CAD software (AutoCAD/Revit) to create, modify, and update technical drawings.

##### Ability to work independently and as part of a team.

##### SECRET security clearance or have the background that enables them to obtain a SECRET security clearance.

#### Preferred Qualifications:

##### Master’s degree in electrical engineering or related field.

##### Experience with electrical design for industrial or commercial facilities.

##### Familiarity with building information modeling (BIM) and 3D design software.

##### Certification in energy efficient electrical design, such as LEED AP or CEM.

### Electrical Engineer – Power System Analysis

#### Reports to:

##### Program Manager

#### Key Responsibilities:

##### Perform power system assessments, including load flow, short circuit, time-current coordination for protective devices, motor starting, and arc flash analyses.

##### Develop and maintain power system models, including three-phase and single-phase systems.

##### Conduct power system design and analysis for new and existing facilities.

##### Prepare and deliver reports and presentations to stakeholders, including clients, contractors, and project managers.

##### Collaborate with other engineers and technicians to ensure that power system designs are safe, reliable, and meet project requirements.

##### Provide technical guidance and support to other engineers and technicians.

##### Conduct site visits to gather information and verify design assumptions.

##### Develop and implement power system testing and commissioning procedures.

#### Requirements:

##### Bachelor’s degree in Electrical Engineering from an ABET accredited program.

##### Three or more years of experience in power system studies, preferably in a similar industry or field.

##### Professional Engineering (PE) license in Electrical Power.

##### Proficiency in power system analysis software, such as SKM, ETAP and EasyPower.

##### Shall be an employee of the NETA accredited company performing this contract.

##### Excellent communication, technical, and problem-solving skills.

##### Ability to work independently and as part of a team.

##### SECRET security clearance or have the background that enables them to obtain a SECRET security clearance.

#### Preferred Qualifications:

##### Master’s degree in electrical engineering or related field.

##### Experience with power system studies for industrial or commercial facilities.

##### Certification in power system studies, such as CEM or CMVP.

### Electrical Engineer – Test Sheet Reviewer (NETA)

      - Reports To:

        - Program Manager.

      - Key Responsibilities:

##### Review and approve test sheets for electrical equipment and systems, including switchgear, circuit breakers, medium and low voltage distribution equipment, transformers, motors, motor control and protection, metering and monitoring devices, starting systems, cables, DC power supplies, batteries and any other equipment requiring electrical preventative maintenance.

        - Analyze test data and make recommendations for changes or repairs to ensure safe and reliable operation.

        - Conduct technical reviews of electrical equipment and systems to identify potential problems or areas for improvement.

        - Develop and implement testing and commissioning procedures for electrical equipment and systems.

        - Collaborate with other engineers and technicians to ensure that electrical equipment and systems are designed, installed, and tested to meet project requirements and industry standards.

        - Provide technical guidance and support to other engineers and technicians.

        - Develop and deliver training programs for electrical testing and maintenance personnel.

        - Stay current with emerging technologies and industry trends in electrical testing and maintenance.

        - Review test sheets for electrical equipment and systems to ensure that they meet industry standards and project requirements.

        - Verify that test data is accurate and complete.

        - Identify potential problems or areas for improvement based on test data.

        - Make recommendations for changes or repairs to ensure safe and reliable operation.

        - Approve test sheets and provide feedback to testing personnel.

        - Collaborate with other engineers and technicians to ensure that electrical equipment and systems are designed, installed, and tested to meet project requirements and industry standards.

      - Requirements:

        - Completion of NETA training program for electrical testing and maintenance.

        - Extensive Knowledge of NETA MTS and ATS Standards

        - Three or more years of experience in electrical testing and maintenance, preferably in a similar industry or field.

        - Shall be an employee of the NETA accredited company performing this contract.

        - Excellent communication, technical, and problem-solving skills.

        - Ability to work independently and as part of a team.

        - Strong analytical and critical thinking skills.

##### SECRET security clearance or have the background that enables them to obtain a SECRET security clearance

        - Bachelor’s degree in Electrical Engineering from an ABET accredited program

      - Preferred Qualifications:

        - Experience with electrical testing and maintenance for industrial or commercial facilities.

        - Familiarity with electrical testing equipment and software, including Megger, Doble, Dranetz, AEMC, and Fluke.

        - Experience with electrical design and analysis software, including ETAP, SKM, and/or EasyPower.

### Electrical Computer-Aided Drafting & Design Technician

      - Reports to:

        - Electrical Engineer – Electrical Systems Support

        - Program Manager

      - Key Responsibilities:

        - Develop electrical systems and equipment, including power distribution, lighting, and control systems.

        - Utilize electrical design software, including AutoCAD and Revit, for drafting electrical power equipment system modifications and installations, and to update existing and create when needed single-lines diagrams, panel schedules, and wiring schematics.

        - Prepare and review electrical design documents, including drawings, specifications, and calculations.

        - Collaborate with engineers and technicians to enure compliance with relevant codes, standards, and regulations, including the National Electric Code (NEC) and IEEE standards.

        - Collaborate with engineers and technicians to ensure that designs are feasible, safe, and meet project requirements.

        - Conduct site visits to gather information and verify design assumptions.

      - Requirements:

        - Associate’s degree in Drafting, Architecture or Engineering.

        - Three or more years of experience using CAD software for electrical design, preferably in a similar industry or field.

        - Proficiency in electrical design software, including AutoCAD and Revit.

        - Excellent communication, technical, and problem-solving skills.

        - Ability to work independently and as part of a team.

        - SECRET security clearance or have the background that enables them to obtain a SECRET security clearance.

      - Preferred Qualifications:

        - Bachelor’s degree in electrical engineering or related field.

        - Experience with electrical design for industrial or commercial facilities.

        - Familiarity with building information modeling (BIM) and 3D design software.

### Office Manager:

#### Reports To:

##### Program Manager

#### Key Responsibilities:

##### Provide administrative support to the team.

##### Manage and maintain records and files.

##### Coordinate travel and meetings.

##### Prepare and edit documents and reports.

##### Answer phone calls and respond to emails.

##### Manage purchase orders and accounting.

#### Requirements:

##### Associate’s degree in business management, accounting, or finance.

##### Three plus years of experience in administrative support.

##### SECRET security clearance or have the background that enables them to obtain a SECRET security clearance.

##### Excellent communication, organizational, and technical skills.

## Technical Positions

### General Electrical Foreman:

#### Reports To:

##### Program Manager

#### Key Responsibilities:

##### Oversee the work of Electrical Foreman and crews.

##### Develop project plans, schedules, and assign work to Electrical Foreman and crews.

##### Ensure that all electrical work is performed in a safe and efficient manner.

##### Conduct regular inspections of work sites to ensure that all electrical work is being performed to code and in accordance with established procedures.

##### Identify and correct any safety hazards or electrical code violations.

##### Provide training & guidance to electricians & electrical technicians.

##### Coordinate with other departments and teams to ensure that electrical work is integrated with other projects and activities.

##### Develop and implement electrical maintenance and repair plans.

##### Oversees quality control to ensure that all electrical equipment and systems are properly maintained and repaired.

##### Develop and manage budgets for electrical maintenance and repair activities.

##### Participate in walkdowns and site visits.

#### Requirements:

##### High school diploma or equivalent.

##### Three or more years of experience as a Foreman or General Forman.

##### NETA certification (Level 3 or higher).

##### Professional certification in electrical supervision, such as Certified Electrical Supervisor (CES).

##### Shall be an employee of the NETA accredited company performing this contract

##### Excellent communication, leadership, and problem-solving skills.

##### Ability to work independently and as part of a team.

##### Strong analytical and critical thinking skills.

##### Experience with electrical maintenance & repair software (PowerDB)

##### SECRET security clearance or have the background that enables them to obtain a SECRET security clearance.

##### Certification in electrical safety, such as OSHA 30-Hour Construction Industry Outreach Training Program.

##### Experience with budgeting, scheduling, and estimating.

### Electrical Foreman

#### Reports To:

##### Program Manager

#### Key Responsibilities:

##### Supervise the work of electrical technicians.

##### Works closely with General Electrical Foreman to assign work to electrical technicians.

##### Ensure all electrical work is performed in a safe and efficient manner.

##### Conduct regular inspections of work sites to ensure that all electrical work is being performed to code and IAW established procedures.

##### Identify and correct any safety hazards or electrical code violations.

##### Provide on-the-job training & guidance to electricians and electrical technicians.

##### Coordinate with other departments and teams to ensure that electrical work is integrated with other projects and activities.

##### Develop and implement electrical maintenance and repair plans.

##### Ensure that all electrical equipment and systems are properly maintained and repaired.

##### Participate in walkdowns and site visits.

#### Requirements:

##### 3+ years as a foreman on electrical projects.

##### High school diploma or equivalent.

##### NETA certification Level 3 or higher.

##### Professional certification in electrical supervision, such as Certified Electrical Supervisor (CES).

##### Certification in electrical safety, such as OSHA 30-Hour Construction Industry Outreach Training Program.

##### Demonstrated experience utilizing the NECA Manual of Labor Units for all contract services under their purview.

##### Familiarity with NETA standards and procedures.

##### Excellent communication, leadership, and problem-solving skills.

##### Ability to work independently and as part of a team.

##### Strong analytical and critical thinking skills.

##### Shall be an employee of the NETA accredited company performing this contract

##### Experience with electrical maintenance and repair software, including CMMS and EAM.

##### SECRET security clearance or have the background that enables them to obtain a SECRET security clearance.

### Journeyman Electrician:

#### Reports To:

##### General Electrical Foreman

##### Electrical Foreman

#### Key Responsibilities:

##### Perform NETA compliant electrical work, including maintenance, repair, & installation.

##### Perform NETA compliant testing

##### Ensure compliance with contract requirements and regulations.

##### Coordinate with other departments and teams as necessary.

##### Manage project schedules, budgets, and resources.

##### Ensure quality control and assurance.

#### Requirements:

##### NETA Level 2 certification.

##### Two or more years of experience with electrical testing.

##### Four or more years of experience with maintenance & repair of equipment.

##### Excellent communication, technical, and problem-solving skills.

##### Shall be an employee of the NETA accredited company performing this contract

##### SECRET security clearance or have the background that enables them to obtain a SECRET security clearance.

## Key Personnel For Services

Definition of Key Personnel - Key Personnel are defined as those individuals who are so essential to the
work being performed that the Contractor shall not replace them without receiving prior approval from the COR.

### List of Key Personnel

#### The Contractor shall furnish key personnel IAW the Staffing Plan. The following Contractor personnel will be “Key Personnel” under this contract.

#### Program Manager (PM)

#### Electrical Engineer / Alternate Program Manager (APM)

#### Electrical Engineer

#### General Foreman

#### Electrical Foreman

#### Two (2) Journeyman Electricians, each with level 3 NETA certification or higher

### Key Personnel Substitution

The Contractor’s request for a change to key personnel shall be made no later than ten (10) calendar days in advance of any proposed substitution and shall include a justification for the change.  The request shall (1) indicate the labor category or labor categories affected by the proposed change, (2) include credentials and experience of the proposed substitute in sufficient detail to allow the Government to assess their qualifications, experience, and security clearance level, and (3) include a statement addressing the impact of the change on the Contractor’s performance and WO delays. The Contractor shall, before starting work, provide proof that each key personnel has requisite current and valid professional certifications, degrees, training and work experience detailed in the PWS for the services the employees are to provide. The Contractor shall furnish key personnel substitutions IAW COR accepted Staffing Plan.

### Non-Key Personnel Substitutions

The COR shall be immediately informed of any potential vacancy or prior to any staff member being removed, rotated in and out, re-assigned, diverted or replaced. The Contractor shall provide proof to the COR that all proposed replacements (temporary or permanent) meet the security and minimum educational and experience requirements of this PWS. The Contractor shall furnish non-key personnel IAW the Staffing Plan.

### Continuity of Qualified Personnel

It is the Government’s expectation that the Contractor shall do everything reasonable to ensure continuity of effort by using the same personnel, Key Personnel or otherwise, throughout the entire period of contract performance.  However, if an individual leaves the Contractor’s employ before the contract expires, the Contractor shall provide the Government with a candidate for replacement who will meet the qualifications as stipulated in this PWS, Contractor’s Staffing Plan, and has comparable qualifications and security clearances.  During any transition of personnel, the Contractor shall make every effort to prevent a loss of service to the Government.  This may necessitate the use of temporary employees to fill short-term gaps between permanently assigned employees.

# Contract Management

## Government Management

RQOO Contracting Officer Representatives (COR) will be appointed in writing by RQ and the Contracting Officer (CO) will subsequently delegate responsibilities to the COR to perform specified duties related to technical oversight and administration of this specific contract.

The Government COR will evaluate the Contractor’s performance to ensure services are received IAW this PWS.  The COR will evaluate the Contractor’s performance through inspection of deliverables and receipt of validated feedback from RQOO’s clients.

## Transition Planning

The transition period starts on the contract start date and will last no more than 60 days.

The Contractor shall submit applications to obtain Contractor identification badges, vehicle passes, security clearances IAW DoD Form 254, passports and visas, as required, Internet and e-mail connectivity, etc.

The Contractor shall familiarize all Contractor personnel with equipment operation, organizational workflow; to include work in progress, reporting, priorities, forms, documents, scheduling, storage, safety, security, and quality control procedures. The Contractor shall make arrangements with the CORs to access the Government facilities.

### Kick Off Meeting and Continuity of Services

The Contractor shall schedule a kick-off meeting with the CO, PM, and COR(s) within 10 calendar days award. The PM will coordinate and confirm the date, time and location of the kick-off meeting. Notification shall be provided at least five (5) calendar days prior to the scheduled meeting.

In addition to the clause “Continuity of Services” (FAR 52.237-3), Contractor agrees to give support to and cooperate with any successor that may be designated prior to the expiration of this contract. Phase-in assistance to a successor Contractor may be required during the final 60 days of this contract.

At the end of this contract, the Contractor shall ensure maximum support to the follow-on Contractor in the transfer of tasks and workload. If the Contractor is to be replaced by a successor Contractor in the performance of all or part of the work contemplated by this contract, it is mutually understood and agreed that the best interest of the Government will be served through re-employment by the successor of qualified Contractor employees.  Therefore, the Contractor shall cooperate in good faith with any successor to provide reasonable access to employees for recruitment efforts. With respect to any such employees who are offered positions by the successor Contractor, the Contractor further agrees to release, without penalty, any employee who chooses to accept employment by a successor Contractor. This obligation shall not require the Contractor to release an employee prior to the completion date of its performance under this contract if such early release would materially affect Contractor’s ability to perform its obligations under the remainder. Furthermore, this clause shall not be deemed to preclude Contractor from making bona fide offers of alternate employment to its employees involved in the performance of this contract.

## Integration of Effort

The Contractor shall provide program management capabilities and professional business relations to manage this contract. The Contractor shall successfully integrate and coordinate all activities needed to execute requirements, including the timeliness, completeness and quality of problem identification, corrective action plans, timely identification of issues, effective/responsive management of any Sub-Contractors, customer satisfaction, and professional and ethical behavior of Contractor personnel.

## Quality of Support

The Contractor shall provide the necessary resources and infrastructure to manage and administer contract. The Contractor shall provide appropriate, flexible, and dynamic support in response to mission demands and changes in technology and technical requirements.

The Contractor shall accomplish the assigned work by employing and utilizing qualified personnel with the appropriate combination of education, training and experience; matching personnel skills to the work required.

The Contractor shall provide qualified replacement/substitute personnel which meet or exceed the same standards as identified in this PWS. The Contractor shall prepare and submit a quarterly Qualification and Certification Report demonstrating that the workforce possesses specified requirements identified in Sections 3.2, 3.4, and 3.5.

## Contractor Facilities, Equipment, Services, Data & Materials

Through RQ support, the Government will, upon COR acceptance of the Contractor-provided T&MWP parts
detailing care and use of facilities/furnishings and services, office equipment, shared equipment, data, and materials
as provided in this section and its subsections, make available facilities/furnishings and services, office equipment,
shared equipment, data, and materials as detailed in this section.

If the Contractor requires any additional items made available to accomplish the requirements outlined in the PWS, the Contractor may request those items; however, the Contractor is obligated to perform and shall make alternate arrangements if the request is denied. All requests shall be asked in an e-mail, from PM to the CORs.

Once the COR accepts the Care of Facilities and Materials plans, which is part of the Technical and Management Work Plan, the Contractor becomes fully responsible for managing, maintaining, and accounting for all assigned government assets according to those approved procedures.

The Contractor shall note all of the Contract Facilities /Furnishings & Services, Equipment, Data, & Materials are to be used solely on WPAFB Area B within RQ facilities while performing work detailed in the PWS.

The Contractor’s plans for the Care of Facilities, Furnishings, Equipment, Data, and Materials shall be organized into specific subsections that correspond to subsections 4.5.1 through 4.5.4 and define procedures for documentation and inventory management. These plans must explicitly schedule at least two inventories, one at the beginning and one at the end of the contract period and identify the specific services responsible for executing care and maintenance. Within 24 hours of completing any inventory, the PM must report all activities to the  COR, specifically highlighting any missing or unserviceable items. Final documentation for these tasks must be submitted to the COR no later than 24 hours after completion, ensuring all solutions align with the respective subsections and mandated CDRLs.

### The Government will provide furnishings and make facilities available for Contractor offices, meeting space, and work/break areas with limited furniture.

### The Government will make available facilities for contract materials/equipment storage, and electrical maintenance and testing.

### The Government facilities have been inspected for compliance with the Air Force Environmental, Safety, and Occupational Health Standards and Regulations (OSHA). No hazards have been identified. Should hazards subsequently be identified, the government will correct hazards according to base-wide priorities. Compliance with OSHA and other applicable laws and regulations for the protection of employees is exclusively the obligation of the Contractor.

### No alterations to the facilities shall be made by the Contractor.

### Services

#### Utility services:

#### For furnished office spaces, the Government will make available electricity, water, sewage, and HVAC. The Contractor shall comply with all Government utilities conservation practices. The Contractor shall be responsible for operating under conditions that prevent utilities waste, outlined as follows:

##### Lights shall be used only in areas where and when work is actively being performed.

##### Mechanical equipment controls for heating, ventilation, and air conditioning systems shall not be adjusted by Contractor employees unless authorized.

##### Water faucets/valves shall be turned off after the required usage.

#### Postal services:

#### The Government will provide on-base mail distribution and United States Postal Service and United Parcel Service for official Government mail required under the terms of this PWS.

#### Custodial Services:

#### The Government will provide custodial service IAW the Base Custodial Contract(s).

#### Facility Maintenance:

#### The Government will provide maintenance and repair of real property facilities. This implies Heating ventilation air conditioning, hot water, sewage, pest control and grounds maintenance.

#### Telephone:

#### The Government will provide telephone service consisting of a minimum of one Class C telephone with voice mail service. Telephone use is limited to official duties IAW this contract. Phone support shall be provided through the Real Property Building Manager (RPBM).

#### Fire Prevention and Protection:

#### The Government will provide fire prevention and protection, inspection and maintenance of government fire extinguishers and systems.

#### Environment Safety and Occupational Health (ESOH):

#### RQOE will provide Environmental Management services as detailed in the Environmental Management section of this PWS.

#### Security Police:

#### The Government will provide general on-base Security Police service.

#### Emergency Medical Service:

#### The Government will provide emergency medical treatment and emergency patient transportation service for Contractor personnel who are injured or become critically ill during the performance of this contract. The Contractor is liable for the cost of medical treatment and patient transportation service at the current inpatient or outpatient rate, as appropriate.

### The Contractor’s plan for Facilities/Furnishings & Services shall contain:

#### Procedures for managing the facilities/furnishings and services.

#### Procedures for tracking maintenance and making operational.

#### Procedures for inventory to include dates and times and when and what feedback is to be provided to the CORs as to the status of the items inventoried.

#### Procedures for Contractor’s employees use and signing out the items.

#### Procedures for contacting the services detailed in subsections 4.5.5.6; 4.5.5.8; and 4.5.5.9.

#### QCP procedures to ensure Contractor’s employees are managing and maintaining these items.

## Office Equipment, Software, Support, and Network Access

### The government will make available office equipment. Office equipment consists of phones, personal computers (PCs), laptop computers, printers, scanners, CAD/CAE equipment, and software.  This equipment along with software and limited network service shall be used for all contract digital/computer data storage, data collection, drawing files, reports and communications.  The following will be made available:

#### Phones for Contractor personnel shall be made available, upon Contractor request to RPBM, for Contractor personnel with designated offices space.

#### PCs for Contractor personnel with designated offices space.

#### PC & Laptop for electrical maintenance, testing and other WO support pertaining to electricians; (These computers shall have limited access to networks and network data storage)

#### Information Technology support (AFRL.RQ.Helpdesk@us.af.mil)  through AFRL/RQ Helpdesk for the PC and the Local Area Network connection. This Helpdesk will also provide software support for PCs and laptops.

#### Common Access Card (CAC)

##### The CAC is predominately used to gain WPAFB access and

##### The CAC is also used to gain access to e-mail accounts, network access, RQ buildings and other security requirements. An e-mail account accessible with the CAC shall be provided by the Government to Contractor personnel with designated office space.  The PM shall begin the application to apply for Contractor personnel to be issued a DoD CAC. Any CAC shall be surrendered per AFI 31-101, The Air Force Installation Security Program.

#### Training for security and access to government networks

##### Initial training will be provided by the government.

##### Yearly training will be provided by the government.

#### The Contractor’s plan for Office Equipment, Software, Support of Office Equipment & Software, and Network Access shall contain:

##### Procedures for managing these items:

##### Procedures for tracking maintenance and making operational.

##### Procedures for tracking training and issuance of these items.

##### Procedures for inventory to include dates and times and when and what feedback is to be provided to the CORs as to the status of the items inventoried.

##### Procedures for Contractor’s employees use and signing out the items.

##### QCP to ensure Contractor’s employees are managing and maintaining these items.

## Shared Equipment

Shared Equipment Plan is submitted within the T&MWP.

Equipment in this subsection is managed under two distinct operational frameworks:

Equipment under Contractor Control is assigned exclusively to this contract for use within WPAFB Area B RQ facilities. The Contractor is responsible for checking this equipment out through a COR, maintaining an accurate inventory, and ensuring daily security at the end of business. All equipment must be maintained and used in accordance with the approved Care of Facilities/Furnishings & Services, Equipment, Data, & Materials plan.

Equipment under RQOO Control is not specific to this contract and is shared across multiple projects. To use this equipment, the Contractor must schedule it at least 15 days in advance and ensure it is checked out only by contract-employed electricians for the approved timeframe. Because availability is subject to maintenance issues or prior scheduling, the Contractor is required to source alternative equipment if RQOO resources are unavailable at the time of need.

### Tools (hand tool non–electric – Consolidated Tool Kits (CTK) items/individual and electric tools) - RQOO has on hand a large portion of tools required for the performance of this contract. This equipment is specific to this contract.

### RQOO has on hand a large portion of equipment for installation of electrical gear and support systems, and removal and replacement (R&R) of electrical systems required for the performance of this contract. This equipment is specific to this contract and RQOO may make available this equipment if deemed necessary and in the best interest of the Government in performance for This contract’s work.

### Test Equipment (Test and Diagnostic) - RQOO has on hand a large portion of Test Equipment required for the performance of this contract. This equipment is specific to this contract. RQOO may make available this equipment if deemed necessary and in the best interest of the Government in performance for this contract’s work.

### Material Handling Equipment (MHE) & Man Lift Platforms (MLPs)

#### The Government may provide MHE and MLPs for contract performance on a case-by-case basis. Use of this equipment requires proper planning by the Contractor Project Manager (PM). All provided MHE and MLPs must be inspected and signed off as serviceable by the Contractor both before use and at the end of every shift.

#### MHE/MLP NOT specific to this contract’s use (RQOO Controlled) - Some MHE/MLPs are shared resources within the RQ and are not dedicated exclusively to this contract. These must be scheduled in advance through RQOO or the Resource and Program Business Managers (RPBMs).

#### MHE/MLP specific to this contract’s use - listed in the contract inventory.

#### Training

##### Training costs to obtain operator licensing or certification for unique Government owned or any and all other MLPs and MHE shall be the Contractor’s responsibility.

##### The Contractor shall provide operator’s driver’s license and proof of training for all Contractor employees operating MLPs and MHE in support of this contract.

### The Contractor’s Plan for Shared Equipment Shall Contain:

#### Procedures for managing these items.

#### Procedures for tracking maintenance and making operational.

#### Procedures for tracking training.

#### Procedures for tracking issuance for any time period.

#### Procedures for securing within shared facilities.

#### Procedures for scheduling the use of shared equipment

#### Procedures for inventory to include dates and times and when and what feedback is to be provided to the CORs as to the status of the items inventoried.

#### Procedures for Contractor’s employees use and signing out the items.

#### QCP procedures to ensure Contractor’s employees are managing and maintaining these items.

## Data

### RQOO will make available to the Contractor operational and historical electrical equipment data.

### Additional documents/information may be provided on individual WOs.

### The Contractor is responsible for obtaining data necessary to perform each task if that data is in the public domain and is not otherwise furnished by the government.

### The Government shall retain unlimited rights to all data and deliverables developed at Government expense. At any time and per the Government request, any records, documents, and associated papers shall be available for review.

## Consumable Materials

The Government will provide a limited supply of basic electrical system materials for the Contractor's use. The Contractor is responsible for managing these materials effectively to support the completion and progression of all WOs and EPM tasks.

### Contractor Material Management

4.9.1.1 The Contractor must ensure sufficient materials are available to meet WO requirements.

4.9.1.2 Inventory Report: At the conclusion of any contract period, the Contractor shall provide a complete inventory report to the Government. This report must detail the building number, room number, and exact location within the room for all materials on hand.

### Contractor's Plan Requirements

4.9.2.1 The Contractor's plan for using Government-provided materials must include documented procedures for:

Management: Managing and maintaining these consumable items.

4.9.2.2 Inventory and Reporting: Conducting inventories (including dates and times) and providing status feedback to the CORs.

4.9.2.3 Quality Control: QCP procedures to ensure proper management and maintenance of items by all employees.

## Contractor Supplied Property

### The PM shall ensure any and all property brought on to WPAFB is for direct support of this Contract and that all property falls within all applicable Government Rules and Regulations.

### The PM shall ensure the CORs are notified, through email and no less than five (5) business days, prior to all Contractor property brought onto or leaving WPAFB

### The PM shall ensure that all Contractor property is clearly labeled with the Contractor’s name and or logo and address prior to being brought on to WPAFB.

### The Contractor’s plan for Contractor Supplied Property shall contain:

#### Procedures for managing these items.

#### Procedures for tracking maintenance.

#### Procedures for inventory to include dates and times and when and what feedback is to be provided to the CORs as to the status of the items inventoried.

#### Procedures for Contractor’s employees use and signing out the items; and

#### QCP procedures to ensure Contractor’s employees are managing and maintaining these items.

## Management of Personnel

### Workforce Stability.

The Contractor shall maintain a stable workforce in such a way as to minimize the impact of any turnover and/or disruption to the customer and/or mission.  Once the Contractor knows an employee will no longer support a task, the Contractor shall provide written documentation to the COR within one (1) calendar day. In the case of a no-notice departure, the Contractor shall immediately inform the COR. A position shall not remain vacant for more than fourteen (14) business days. The contractor shall provide evidence within three (3) days of action being taken to fill the position.

### Contractor Identification in the Government Workplace.

All Contractor employees shall wear company picture identification badges and/or lanyards to distinguish themselves from Government employees. All Contractor employees shall prominently identify their workstations as a Contractor workstation with their company name. When conversing with Government employees during business meetings, over the telephone, or via electronic mail, Contractor employees shall identify themselves as Contractor employees and who they are employed by to avoid sensitive topics that Government employees should not discuss with Contractors. Contractors shall identify themselves on any attendance sheet or any coordination documents. Electronic mail signature blocks shall identify their company affiliation. Contractor badges shall be worn when Contractor employees are at Government installations or when at another location. Government-issued badges, identification cards, passes, and vehicle registration media are US. Government property to be accounted for, protected, and returned to the Government the next business day when no longer required.

### Access to Worksite

In all cases, if Government personnel are not available in a Government facility to oversee Contractor activities, Contractor personnel shall leave the premises.

#### Base Closure/Delays

All services to be performed under the contract have been determined to be non-essential for performance during a
crisis or severe weather. The contractor is responsible for notifying its employees about base closures and delays. Contractor employees are not to report to the base if it is closed and will adhere to delays, unless otherwise specifically instructed to do so by the COR. This latter case would only be necessary if it is determined that contractor operations are necessary to support crisis operations per Department of Defense Instruction (DoDI)3020.37.

#### Work During Emergency Conditions

#### There may be occasions when WPAFB will not be closed but performance of services must be altered due to emergency conditions declared by WPAFB authority.  In this event an alternate work schedule will be implemented. An alternate work schedule shall be negotiated with the COR and contractor. This alternate work schedule shall be adopted for the duration of the declared emergency, and the contractor shall comply with the provisions of that alternate work schedule.

#### Work During National Crisis Declared

In the event of a crisis declaration, the government may require the contractor to perform services that are deemed essential for performance according to DoDI 3020.37. In such cases the contractor shall continue the performance of services as documented in the PWS.

#### Travel

The government's requirement for travel costs shall be determined by the Joint Travel Regulations (JTR), which specify the costs associated with travel, such as airfare, meals, and lodging. Costs for travel shall be detailed with coordination and approval signature by the contractor’s PM through written authorization from the CORs prior to travel.

#### Government Holidays

Federal law (5 U.S.C. 6103) establishes public holidays for Federal employees, published by the Office of Personnel
Management (OPM), and will be observed under this contract. The Contractor’s employees are not required to
perform services on legal holidays and/or the day of Government observation if legal holiday falls on the weekend.
These holidays may be different than the holidays listed in the IBEW 82 Union Agreement. Should PM request contractor personnel to work on a Government facility on an observed holiday, prior approval must be given by the COR. The PM shall submit holiday work requests for approval at least thirty (30) days before the work is to occur unless the work is for an emergency or an unscheduled electrical outage. The request must include the position and name of the contractor employee(s), the number of hours requested, and details regarding work to be performed.

### Union Agreement

When utilizing union employees any Collective Bargaining Agreement (CBA) shall be adhered to for all local union employees, and it should be understood that the CBA for the IBEW 82 here in Dayton is typically structured for 3 year terms.

### Contractor Personnel Training

#### During the course of this contract, the Government may need to provide specialized training for contractor employees. Contractor employees for whom the training is applicable will be required to participate. The training will allow contractor employees to fulfill the requirements of this PWS.

### Location and Hours:

#### Work Location

The primary location of work is WPAFB, OH unless otherwise stated. The Contractor, with COR approval, may be authorized to attend Directorate, Division, and Branch-level unit functions.

#### Work Hours

Working hours are defined as the period of time the contractor shall complete their day-to-day work without working less than 8 hours a day and no more than 40 hours a week. Working hours are between 0700 – 1700 Monday – Friday unless an Alternate Work Schedule has been approved by the COR.

Alternate Work Schedule (AWS) may be available, as approved by the COR, contingent upon no degradation in mission accomplishment and/or customer service.

The Contractor as part of the T&MWP shall provide a schedule that establishes workdays during Work Hours. Short-term situations may make performance more than 8 hours per day, 40 hours per week necessary on occasion.  The Contractor must ensure that they can provide for this type of contingency. Contractors may be required to work outside of the normal working hours indicated above.

### Overtime

Overtime shall be approved by the COR with written approval prior to performance.

### On-Call Services Outside of work hours

Contractor shall provide support for two (2) types of On-Call Services: Support by Phone and Support on Site

4.11.8.1 Support by Phone: Provided by PM.

4.11.8.2 Support on Site: Local Oversight shall respond and be on-site within one (1) hour of PM’s and COR’s phone call.

## Timeliness

The Contractor shall adhere to all schedule timelines, milestones, delivery schedules and administrative requirements that contribute to or effect schedule variance, to include scheduled on-time delivery or submission of reports, data products,  invoices, staffing of personnel, and action items.

## Reperformance of Deficient Services

The Contractor shall be compliant with all Safety and Health Plan requirements and ensure all their employees are also compliant. The Contractor shall provide their Total Case Incidence Rate (TCIR), their Days Away Restricted and/or Transfer Case Incident rate (DART) rates and their OSHA Form 300A annually to the contracting office for consolidation and submission as part of the installation's annual Voluntary Protection Program (VPP) Safety and Health management report.

The Contractor shall establish efficient and effective processes and assign appropriate resources to effectively administer this contract. The Contractor shall implement and maintain a management structure that can effectively manage a diverse workload across multiple organizations. The structure shall include the capability to respond to requirements across the full range of potential workloads and shall provide the flexibility to support both long-term stable tasks, and short-notice short-duration tasks. The management structure shall provide the capability to rapidly reassign personnel as well as assign personnel to multiple, sometimes diverse tasks, where the Contractor and Government deem such action to be appropriate. Duties include but are not limited to responding to Government requests for contractual actions in a timely fashion; having a single point of contact between the Government and Contractor personnel assigned at the appropriate levels to effectively manage the required tasks; assigning work effort; and maintaining proper and accurate time keeping and cost accounting records of personnel assigned to work on the requirement.

## Procurement of Materials.

The contractor shall purchase materials required to meet the requirement of this Contract and shall obtain COR approval prior to purchase. Delivery shall be within the needed to complete the work as stated in the PWS. All purchased materials/items become the property of the Government. The contractor shall purchase supplies that are needed and required on daily or recurring use with COR approved.

## Support Reviews

The Contractor, at the request of RQOO, shall conduct annual Program Management Reviews (PMR) of the data generated in preparation of the Status Report to address monthly data and other pertinent management information.  The review shall include Government requested information and shall include, but is not limited to:

- Contract management reporting

- Task Progress

- Projected changes in manpower and redistribution based on customer organization needs, manpower and recruiting summary.

- Security issues

- Contractual action items

- Accounting/data documentation

- Comparison of total contract funding to invoiced services.

- Any special interest items requested by the Government or provided at the Contractor’s initiative.

- Requirements Review (as needed)

These reviews may also address, in general, the efforts, challenges, problems, and accomplishments of Contractor personnel in the respective task areas. The Contractor shall provide the PMR agenda and briefing slides three (3) calendar days prior to the meeting and PMR minutes within five (5) calendar days following the meeting. Government program and contract management may require other compilations of data to ensure adequate insight into this Contracts execution. This review shall be held with the PMO and COR at a WPAFB, OH.

# Quality Assurance

The Contractor shall provide and maintain an QCP acceptable to the Government covering the services under this contract. Complete records of all inspection work performed by the Contractor shall be maintained and made available to the Government during contract performance.

The Government has the right to monitor all services called for by the contract, to the extent practicable at all places and times during the term of the contract.

If any of the services performed do not conform to the contract requirements, the Government shall require the Contractor to perform the services again in conformity with contract requirements at no additional cost to the government.

## Government Inspection

The Government will evaluate the Contractor's performance under this contract IAW the QASP. Government methods of inspection other than those listed in the PWS  may occur during the performance period of this contract. Such surveillance will be done according to standard inspection procedures or other contract provisions.

## Contracting Officer’s Representative

CORs are representatives of the CO and will participate in the administration of this contract.

## Administrative Records

All records, documents, and associated papers provided by the Government and generated during the period of performance of this Contract shall remain in place upon contract completion or termination.  During the period of performance of the contract, records, documents, and associated papers shall be available for review.

## Quality Control Plan

The Contractor shall develop a QCP that demonstrates how the Contractor will ensure its services consistently meet quality standards acceptable to the Government covering the services under this contract. The Contractor shall provide the QCP to the Government no later than (NLT) 30 days after contract award date. The Contractor shall also provide updated versions to the Government within 15 days of implementation by the Contractor. The QCP shall demonstrate the Contractor's documented processes and procedures monitor and maintain including but not limited to the following:

- Objectives in the Performance Requirement Summary

- Sub-Contractor relationships, if applicable

- Contractor and Sub-Contractor invoicing

- Non-conformances

- Contractor employee qualifications and certifications

## Performance Evaluation

Contractor performance is subject to surveillance by a Government COR to ensure compliance with this PWS to include performance quality. The Contractor shall comply with the following:

Access The Contractor shall permit the COR or CO to all work areas, records, and data used in the performance of this Contract.

Non-conformances The Contractor shall identify and control non-conformances through root cause analysis, corrective actions and preventive actions. The Contractor shall focus on proactive identification and transparency of issues and on eliminating the cause to prevent reoccurrence. The Contractor shall maintain records of non-conformities and actions taken to remedy them. The Contractor shall correct and provide response to all Government-identified non-conformances. This contract identifies two types of non-conformances: Minor and Major. All records of non-conformance and corrective actions will be documented in PARs; Quarterly Surveillance Reports (QSR); and annual CPARs.

Minor Non-conformance A minor non-conformance is a non-conformance, which by itself does not adversely impact the overall AFRL mission, safety of personnel and/or equipment, performance (quality), schedule (delivery), or cost.  Minor non-conformances are typically low risk and may be communicated through a CAR form with the minor box checked or another documented Government communication method. Minor CARs are normally issued for any identified non-conformance; second notice minor CARs are normally issued for repeat non-conformances or failing to correct issues within a reasonable amount of time. Upon receipt of a minor CAR or another documented Government communication method addressing minor non-conformance, the Contractor shall complete applicable sections and return it to the COR, as specified, within the time specified in the notice. A formal corrective action plan is not required for minor CARs. Minor non-conformances shall be documented in order to be used in support of a QSR, Performance Assessment, or CPAR rating.

Major Non-conformance A major non-conformance is a non-conformance that adversely impacts (or has the potential to impact) mission, safety of personnel and/or equipment, performance (quality), schedule (delivery), or cost. This type of non-conformance increases risk to the Government and therefore moderate to high risk. The COR may communicate major non-conformances with a CAR. A suspense date for the Contractor's corrective action plan will be included as well as a summary of the minor CARs and documented customer complaints, if any, that have preceded this issuance. Major non-conformances shall be documented in order to be used in support of a QSR, PAR, or CPAR rating. The Contractor shall generate a formal corrective action plan for major CARs and other documented Government complaints/concerns that will address as a minimum:

- Action taken to fix the immediate problem.

- Root cause analysis of the problem to determine cause.

- Corrective action on the cause of the problem.

- Actions taken to prevent recurrence.

- CO will determine if a cure notice is required for unresolved non-conformance issues.

# Contract Security

Access to classified national security information up to SECRET may be required under this contract. The Contractor shall comply with the contract security requirements and Contractor access to classified information as specified in the contract attachment DD Form 254, “Security Classification Guide (SCG)” as well as security requirements flowed down and identified in the following subparagraphs:

## Non-Disclosure Agreements (NDA)

The Contractor shall ensure personnel, information, system, property, facility, and international security requirements are met. The Contractor shall comply with Government Security regulations and the installation's supplemental guidance.

The Contractor may need to access other companies' proprietary information in order to perform the duties in this task order. The Contractor and its employees shall enter into appropriate NDA. The Contractor is responsible for obtaining all necessary NDAs with all applicable corporate, supplier, and sub-tier vendors with proprietary information that will be used or accessed during the execution of this task order. The Contractor shall provide a copy of each NDA to the COR. Violation of NDA is considered a major security violation.

## Information Access

The Contractor shall maintain and store internally generated data and shall permit Government access to all documentation which is created in performance of this task order. The Contractor shall also provide the Government access to Contractor computer databases and files containing task order generated programs, management tools, and resource utilization cost information. Presentation material prepared under Contractor activities related to the task order shall be made available for use by Government personnel. All documentation, programs, tools, databases, records, information or other such information or systems created in performance of this TO shall be transferred to the Government at the completion of the contract.

## Physical Security

Safeguarding Government Property.  The Contractor shall safeguard all Government property and controlled forms provided for Contractor use and adhere to the Government property requirements contained in this task order.  At the end of each workday, all facilities, equipment and materials shall be secured, including workstations.

Key Control.  The Contractor shall establish and implement procedures to ensure all keys issued to the Contractor by the Government are not lost or misplaced and are not used by unauthorized persons.  The Contractor shall not duplicate any keys issued by the Government.  The Contractor shall immediately report to the COR any occurrences of lost or duplicated keys within two days of occurrence. In the event keys are lost or duplicated, the Contractor may be required, upon written direction of the CO, to re-key or replace the affected lock or locks with no additional cost to the Government.  The Government may replace the affected lock or locks or perform re-keying and deduct the cost of such from the monthly payment due the Contractor.  The Contractor shall not loan issued keys to any other persons nor allow access by use of issued keys to other persons not associated with performance of work at the contract work site.

Lock Combinations.  The Contractor shall control access to all Government-provided lock combinations to prevent unauthorized entry.  The Contractor is not authorized to record lock combinations without written approval by the COR.  Records with written combinations to authorized secure storage containers, secure storage rooms or certified vaults, shall be marked and safeguarded at the highest classification level as the classified material maintained inside the approved containers.

Weapons, Firearms and Ammunition.  The Contractor shall not possess weapons, firearms or ammunition on themselves or within their Contractor-owned vehicle or privately-owned vehicle while on any installation or any office/working location covered under this contract. Possession of weapons, firearms or ammunitions while on installation is considered a major security violation.

Pass and Identification.  The Contractor shall ensure pass and identification items required for contract performance are obtained for employees and non-Government owned vehicles. The Contractor shall comply with AFMAN 31-113, Installation Perimeter Access Control and the installation's supplemental guidance.  Loss of pass and identification items is considered a minor security violation. The COR must be contacted, and a new pass/identification will need to be reissued.

Emergency Evacuation Procedures.  Contractor employees shall follow the direction of Government employees in regard to emergency evacuation procedures, i.e., fire, tornado, bomb threats. It is the responsibility of the Contractor to provide guidance and establish procedures for responding to emergencies for their employees. It is the Contractor's responsibility to account for their employees following an emergency evacuation.

Controlled/Restricted Areas.  The Contractor shall adhere to local base procedures for entry to Air Force controlled or restricted areas where Contractor personnel shall work. Contractor personnel must have appropriate clearances prior to commencing work on this Contract. Access to the controlled/restricted area without proper escort and by an unauthorized Contractor employee is considered a major security violation.

## Industrial Security

Access to Defense Research and Engineering Network (DREN) will be required to meet performance requirements.

## Operational Security (OPSEC)

OPSEC must be an integral part of our daily activities.  As we maintain security on our future technologies that are vital to national interest, we must recognize and prepare for the threat poised against our technology.  Department of Defense policies mandate a high degree of security throughout the acquisition process.  However, heightened security awareness and threat-based countermeasures are particularly essential during the research and development phase when our technology is most vulnerable to espionage, sabotage, or exploitation.  It is the obligation of each employee or persons involved on this agreement be constantly aware of and strictly adhere to security requirements designed to protect sensitive unclassified and other information and resources produced by the acquisition, research and development, and technological security efforts outlined in this SOW/PWS.  The recipient shall ensure employees receive training and follow appropriate Operations Security (OPSEC) requirements during the performance of the agreement.

The Contractor shall establish, implement, and maintain a security program IAW the Contract Security Classification Specification, DD Form 254. The nature of the contract requires Contractor personnel to possess a high degree of security awareness. The Contractor shall not divulge any information about Government files or any sensitive information to anyone not authorized access to such information. The Contractor shall be required to sign a NDA. The Contractor shall maintain a database of personnel with active security clearances and initiate periodic background updates as required.

DD Form 254: Overarching security requirements and Contractor access to classified information shall be as specified in the basic DD Form 254, which will be further identified in the DD Form 254 for each TOP, as required. All Contractor personnel with access to unclassified information systems, including e-mail, shall have at a minimum a favorable National Agency Check with Inquiries (NACI). The contractor shall request a waiver from the Contracting Officer for contractor personnel to perform work during the NACI investigation.

AFRL/RQOS is the AFRL/RQ Division security point of contact (POC).

## Communications Security (COMSEC)

The Contractor shall use only Government approved secure communications methods and/or equipment to transmit or otherwise transfer classified and/or controlled classified information IAW DoD 5220.22-M.  Applicable equipment shall be safeguarded, maintained, and operated IAW DoD 5220.22-M.  Violation of any COMSEC requirements is considered a major security violation.

Freedom of Information Act (FOIA).  The Contractor shall comply with DoD 5400.7-R/Air Force Manual 33-302, DoD FOIA Program, requirements. This regulation sets policy and procedures for the disclosure of records to the public see DoD 5200.1-R, Information Security, Program, Appendix 3 for marking, handling, transmitting and safeguarding controlled unclassified information.

Controlled Unclassified Information.  The Contractor shall comply with DoDM 5200.01, Volume 4, DoD Information Security Program: Controlled Unclassified Information (CUI), Enclosures 3 & 4, for identification, protection and training requirements of CUI.  The Contractor shall be responsible for training their personnel and accomplishment of the out-processing procedures identified in DoDM 5200.01, Volume 4, Enclosure 4. Protection of unclassified DoD information not approved for public release on non-DoD Information Systems will be protected IAW DoDI 8582.01, Security of Unclassified DoD Information on non-DoD Information Systems, Enclosure 3.

Restricted/Formerly Restricted Data.  The Contractor shall obtain a final U.S. Government clearance at the appropriate level before obtaining access to RESTRICTED DATA and FORMERLY RESTRICTED DATA.

## Security Clearances

The Contractor shall ensure Contractor personnel have appropriate clearances prior to commencing work on this contract. The Contractor shall also complete visit requests for each individual that will be performing work on this contract in the Defense Security Service (DSS) Joint Personnel Adjudication System (JPAS) prior to performance starts. There may also be a requirement that the individual possess a specified security clearance in addition to the requirements listed below. Initial Clearance will be at the Secret Level.

## Security Training

The Contractor shall ensure Visitor Group personnel participate in the Government activity’s initial and reoccurring security training IAW AFI 31-401, Information Security Program Management, Chapter 8; AFI 31-601, Industrial Security Program Management, Chapter 3, AFI 10-701, Operations Security, Chapter 5, and the Visitor Group Security Agreement, Training Section.

## In/Out Processing

The Contractor shall ensure Contractor personnel in-process with the unit security manager/information systems security officer on the first duty day and out-process with the unit security manager/information systems security officer no later than the period of performance end date of this contract or no later than the Contractor personnel’s last duty day, whichever occurs first. This process will facilitate issuance/retrieval of CACs, issuance/termination of computer accounts and building access.

Retrieving Identification Media. The Contractor shall retrieve all identification media, including vehicle passes when the period of performance for this contract ends or from employees who depart for any reason before the contract ends.

## Security Administration

Overarching security requirements and Contractor access to classified information shall be as specified in the basic DD Form 254. All Contractor personnel with access to unclassified information systems, including e-mail, shall have at a minimum a favorable National Agency Check (NAC).

Visitor Group Security Agreement.  With the assistance of the respective Air Base Wing security offices, the Contractor shall enter a Contractor Visitor Group Security Agreement in the JPAS between the Contractor and the AFRL Directorate. The Agreement will outline responsibilities in the following areas: Contractor security supervision; Standard Practice Procedures; access, accountability, storage, and transmission of classified material; marking requirements; security education; personnel security clearances; reports; security checks; security guidance; emergency protection; protection of Government resources; DD Forms 254; periodic security inspections.

Classified Visits.  The Contractor shall process all classified visit requests via the JPAS in support of tasks within DoD. Classified visits outside DoD will require Visit Authorization Letters (VALs) IAW the National Industrial Security Program Operating Manuel (NISPOM).

Classified Documentation Access.  Contractor personnel may be required to receive or transmit classified documents.  Specific security requirements will be defined in the DD254.

Facility Clearances.  Secret.

Information Protection.  The Contractor shall comply with program SCGs and shall mark and protect information IAW program SCGs. Additionally, the Contractor shall pass down the security requirements to all sub-s and team members, as stated in the contract.

Security Incident or Violation.  The Contractor shall immediately notify the Government Security Office of any potential or actual security incident or violation including potential or actual unauthorized disclosure or compromise of classified and/or controlled information.

# Government Furnished Property & Equipment

The Government will provide access to relevant Government organizations, information, documentation, manuals, text briefs, and associated materials as required and available.

The Government will provide access to facilities, office space, supplies and services, to include workstations, computers and phones. Access will be granted to classified and unclassified military local area network (LAN) services, classified military LAN services, LAN support, telephones, and reproduction facilities.  If the Contractor determines additional equipment is required, the Contractor shall notify the Government, in writing, of the applicable information/equipment required.

## Use of Personal Equipment

Personal equipment is not approved for use in the performance of this contract.

## Protection from Loss

Government furnished equipment/property must be reasonably protected from loss, theft, or unauthorized use. The Contractor shall be responsible for understanding security obligations and shall assist in the formulation of adequate procedures for the safeguarding of classified and other protected information that is under the Contractor's control.  Procedures must also be developed for the normal and emergency protection of Government equipment and facilities under the Contractor's control.

When a loss, theft, or compromise of personal information or Government equipment occurs, the Contractor must immediately notify his/her supervisor and the COR.

## Supplies and Equipment

The Contractor shall use AFMC approved knowledge management tools IAW DoD, AF, AFMC, and AFMC information technology instructions, regulations, policies, and procedures, including AFI 33-129.

# Appendix A: Contract Data Requirements List

The Contractor shall provide deliverables as described in this task order. Deliverables shall be specified by the Government.  Format and delivery schedule for deliverables shall be outlined in CDRLs and/or other means.

| Identifier | Name | Frequency |
| --- | --- | --- |
| A001 | Technical and Management Work Plan | By the 10th of each month |
| A002 | Technical Report-Study/Services | As Generated |
| A003 | Design Drawings/Models | As Generated |
| A004 | Presentation Material | Submit at each briefing/meeting/or review. |
| A005 | Record of Meeting/Minutes Report | Within 24 hours of Meeting |
| A006 | Daily Site Activity Schedule | Weekly - IAW PWS Sections 2.2, 2.7, and 2.8 |
| A007 | Acceptance Test Plan | Within the first 30 days |
| A008 | Contractor Employee Qualifications and Certifications Report | Quarterly, IAW PWS Section 4.4 |

CDRL Deliverable Documentation.

The Contractor shall deliver CDRL submittals authored by the Contractor, Contractor provided wording and phrasing (Contractor’s original composition specifying distinctive solutions fitting to this PWS requirements), explicitly establishing Contractor’s approach aligns with contract documents and provides Contractor personnel, CORs, and CO clear understanding of Contractor’s direction meeting contract requirements.

## A.1 CDRL A001 Technical and Management Work Plan

The Contractor through the PM, shall provide a Technical and Management Work Plan (T&MWP) describing the Contractor’s plan to execute the PWS.  The Contractor shall treat the T&MWP as a living document and ensure edits are made to keep the plan current through each monthly submittal validating the Contractor’s personnel are providing to the Service Summary Table and Contractor’s, COR accepted, QCP.

## A.2 CDRL A002 Technical Report-Study/Services -TR-S/S (Reports)

The Contractor through the PM shall provide technical reports/studies/analysis/evaluations (reports) as generated from WOs. EMC services, Contractor personnel, providing the reports shall be defined in the T&MWP.

## A.3 CDRL A003 Conceptual Design Drawings/Models and Associated Lists

The contractor shall provide designs and related drawings developed using the appropriate CAD/CAE software such as Auto CAD design software.  All designs shall comply with applicable codes and standards.  Revisions to existing drawings will be assigned through marked-up drawings with a detailed explanation of the desired results; more complex revisions are produced from sketches of the desired product. The EMC services, Contractor personnel, providing the Conceptual Design Drawings/Models and Associated Lists shall be defined in the T&MWP.  Submittals shall contain:

## A.4 CDRL A004 Presentation Material for Briefings/Meetings (B/M)

The Contractor through the PM shall provide presentation material and set all contract meetings. The EMC services, Contractor personnel, providing B/M presentation material, the PM’s requirements and materials shall be defined in the T&MWP.

Meetings or briefings imply any gathering of more than one person from impromptu conversations to conversations with set time and place.

## A.5 CDRL A005 Records of B/M (Minutes)

The Contractor through the PM shall provide minutes, an accurate written history of the proceedings, from briefings or meetings.

## A.6 CDRL A006 - Daily Site Activity Schedule (DSAS)

The Contractor through the PM shall provide a COR accepted DSAS for all Contractor employees.

## A.7 CDRL A007 Acceptance Test Plan (ATP)

Contractor shall provide an ATP for RQ’s research and research support electrical equipment.

Contractor, through the PM, shall set a monthly meeting, for ATP progress.

## A.8 CDRL A008 Contractor Employee Qualification & Certification Report.

The Contractor shall submit the Qualification and Certification report, quarterly, NLT 15 calendar days after each report period. The report shall address requirements listed in this PWS for each position.

# Appendix B:  Use of Government Infrastructure

## B.1. Use of Government Vehicles

The Contractor may be required to operate Government vehicles in the course of providing task order support.  All such vehicle operations shall be IAW AF Instruction 24-301 and 31-204.  The Contractor shall verify that Contractor personnel required to operate Government vehicles possess a valid driver’s license.  It may be necessary under some circumstances for the Contractor to handle/transport hazardous munitions or munition components using a Government vehicle in the conduct of task order execution.  All such handling/transporting of hazardous munitions or munition components shall be IAW DoD 4145.26-M.

## B.2. Traffic Laws

The Contractor shall ensure all Contractor employees comply with base traffic regulations.

## B.4. Environmental Management System (EMS)

The Contractor shall perform work in a manner that conforms to all appropriate Environmental Management Programs and Operational Controls identified by the agency, organizational, installation, or facility EMS.

- The Contractor shall advise and assist by providing monitoring and measurement information as necessary for the organization to address environmental performance relative to the environmental, energy, and transportation management goals.  In the event an environmental nonconformance or noncompliance associated with the contracted services is identified, the Contractor shall take corrective and/or preventative actions.

- The Contractor assumes no responsibility for maintenance or upgrade to Government-owned facilities, equipment, or operating instructions/policy to bring them into compliance with environmental rules and regulations.

- The Contractor also assumes no responsibility for work-stoppages that may result due to environmental compliance issues of Government-owned facilities. In the case of a noncompliance and/or nonconformance of EMS issues associated with Contractor provided services, the Contractor shall respond and take corrective action based on the time schedule established by the EMS Site Coordinator.

- The Contractor shall provide a list of toxic and hazard chemicals to the Facility Hazardous Materials Cell before bringing any material onto the installation.  This list will not supplant nor duplicate reporting that is done through direct task support work in individual installation organizations.  This list shall consist of the material name, manufacturer name and address, quantity and size of the materials.  This list may be in Microsoft Word or Excel.  Updates of this list shall be submitted quarterly to the 96 CEG/CEVCP.

- All on-site Contractor personnel shall complete the initial installation sponsored environmental training specific to the installation which is titled Environmental Management System Awareness Training (EM 201) or equivalent.  This training must be completed within 90 days of the Contractor employee beginning work on the installation.  The facility training will take approximately thirty minutes to accomplish.  The method of training can be by video, or seminar presentations given by the EMS Coordinator.  Documentation of the training shall be reported to the PMO.

## B.5. Network Administration

The Contractor shall implement network and client system computer security measures IAW Government requirements. The Contractor shall securely configure the server/client hardware and software so as to prevent intruder attacks/unauthorized penetration. The Contractor shall implement all Government required security measures (e.g., Base Information Assurance Office (BIAO) and/or AF Emergency Response Team (AFERT) mandatory compliance notices). The Contractor shall control networked personal computers, printers, and servers.  This includes coordination of all network operations with local Information Technology/Communications requirements; design and implementation of ad-hoc Management Information System (MIS) tools; management of online help desk; distribution and installation of desktop computers; operations of video teleconferencing system and conference room computer systems;  network security; software configuration control on all operating platforms; installation and configuration of computer hardware and software for desktop computers and server platforms, related hardware, and software, and software updates; establishment and maintenance of required data bases; development and implementation of appropriate MIS capabilities in support of business, task order management, and administration requirements; and developing changes for updating the MIS system.

Access to Government Systems.  The Government will provide Contractor personnel with necessary workstations, classified storage, and computer hardware and software for performance of all routine office tasks.  The Contractor shall ensure Contractor personnel who require access to a Government AIS have a DoD National Agency Check Plus Written Inquiries (DNACI) conducted IAW DoD 5200.2-R.

If supporting multiple tools and devices, an IAT should obtain CE certifications for all the tools and devices they are supporting. At a minimum the IAT should obtain a certification for the tool or device he or she spends the most time supporting. For example, if an IAT is spending most of his or her time supporting security functions on a CISCO router, the IAT should obtain a CE certification for that equipment.  This requirement ensures they can effectively apply IA requirements to their hardware and software systems.  All personnel must agree to release their IA baseline certification qualification(s) to the Department of Defense through the Defense Workforce Certification Application (DWCA).

# Appendix C:  Contract Safety

## C.1. Safety and Health Requirements

- The Contractor shall comply with all applicable Occupational Safety and Health Administration (OSHA) and Air Force Occupational Safety and Health (AFOSH) standards, technical orders, regulations and referenced publications both within and outside of the United States.  The Contractor shall comply with the highest degree of safety protection where any disagreements exist.

- The Contractor shall ensure its employees and managers have a comprehensive understanding of OSHA requirements.  Contractors, whether regularly involved in routine site operations or engaged in temporary projects must follow the safety and health rules of the installation.

- The Contractor shall provide their Total Case Incidence Rate (TCIR), their Days Away Restricted and/or Transfer Case Incident rate (DART) rates and their OSHA Form 300A annually to the contracting office for consolidation and submission as part of the installation's annual VPP Safety and Health management report.  TCIR and DART rates are due by the 15th of January of each year.

- The Contractor is responsible for compliance with the OSHA (Public law 91-596).  The Contractor shall submit a Safety and Health Plan and corresponding site safety checklist to the  COR not later than ten (10) workdays after contract award.  Subsequent submissions shall be required for proposed updates and submission for Government approval shall be required as specified in the CDRL.  The Contractor's Safety and Health Plan shall include appropriate measures to ensure the Contractor reacts promptly to investigate, correct and track alleged safety and health violations and uncontrolled hazards in the Contractor work areas.

- The Contractor shall identify the processes and procedures the Contractor shall use to track compliance with the Safety and Health Plan, and the process and procedures that shall be used to correct violations.

- The Contractor's Safety and Health Plan shall demonstrate a management commitment to employee safety and health and shall identify the application of the Safety and Health Plan to Sub-contractors.

- The Contractor's Safety and Health Plan shall identify the roles and responsibilities of the following individuals: management, supervisors, employees and safety coordinator.

- The Contractor's Safety and Health Plan shall identify applicable safety rules and regulations and shall include a worksite hazard analysis to include baseline hazard identification and required control measures.

- The Contractor's Safety and Health Plan shall include a job site analysis to include hazard of tasks required to control measures, identify employee safety and health training requirements and detail the documentation process.

- The Contractor's Safety and Health Plan shall include a workplace inspection frequency to include identifying the individual conducting the inspections and shall include employee hazard and mishap reporting procedures.

- The Contractor's Safety and Health Plan shall identify individual(s) responsible for corrective hazards and shall identify first aid and injury procedures, procedures for accident investigation and reporting, identify emergency response procedures and identify the process for tracking controlled hazards in Contractor work areas.

- The Contractor is responsible for establishing these requirements for all Sub-contractors who qualify as applicable Contractors meaning they are working more than 1,000 hours per quarter cumulatively on a Government installation under the same contract.

## C.2. Contractor EMPLOYEES’ Safety Requirements

Compliance with OSHA and other applicable laws and regulations for the protection of Contractor employees is exclusively the obligation of the Contractor.  Note:  Air Force Occupational Safety and Health Standards (AFI’s) are annotated because many of the Air Force Standards exceed the OSHA standard criteria.  If a conflict is noted, the most stringent requirement takes precedence.  The Government shall assume no liability or responsibility for the Contractor's compliance or non-compliance with such requirements.  The Contractor shall furnish to each of its employees a place of employment, which is free from recognized hazards.  The Contractor shall brief its employees on the safety requirements of this contract and on hazards associated with prescribed tasks.  The Contractor is responsible for compliance with OSHA Public Law and the resultant standards identified within.  In addition, the Contractor is required to flow down the safety requirements and specification to all Subcontractors.  This contract shall in no way require people to work in surroundings or under working conditions which are unsafe or dangerous to their health.  The Contractor must coordinate and perform work so as not to impact the safety of Government employees or cause damage to Government property.  This requires providing personnel with protective equipment and associated safety equipment as necessary.  The Contractor must also protect personnel from hazards generated by the work.  If the Contractor employs bilingual speaking employees, they must post bilingual signs and have written procedures for specific tasks in applicable languages.