900-ton Crane Rental

Statement of Work

For

704th Test Group/846 Test Squadron

10 March 2026

1.0  PURPOSE

The 846th Test Squadron (846 TS), part of the 704th Test Group, operates the Holloman High Speed Test Track (HHSTT) to execute rocket-sled test simulations for critical Department of War systems.

A critical upcoming mission requires the movement and placement of eight large concrete targets. This task is normally performed by the government-owned 200-ton Shuttlelift Gantry Crane, which recently suffered a critical failure and is currently inoperable, halting all related mission activities.

The unavailability of the primary crane creates a direct and immediate risk of delaying critical National Defense programs that depend on the timely completion of this test mission. A temporary solution is required to mitigate this delay and execute the task as scheduled.

The purpose of this requirement is to provide a temporary, short-term mobile crane rental service. This service will provide the necessary heavy-lift capability to move and set concrete targets, ensuring the mission proceeds on schedule and tested systems are delivered to the warfighter without delay.

2.0 SCOPE

The contractor shall provide all personnel, equipment, materials, supervision, and other items necessary to provide and operate a 900-ton class mobile hydraulic crane. The purpose of this service is to move and set eight (8) concrete targets at the Holloman High Speed Test Track (HHSTT).

Period of Performance: The work shall be performed from approximately April 1, 2026, to April 24, 2026.

Location: The work will be performed at the HHSTT, Holloman AFB, NM.

846TS/TGTOT

Attn: Issac Portillo

1521 Test Track Rd, Building 1189

Holloman Air Force Base, NM 88330

Phone: 575-572-8993

3.0 CONTRACTOR REQUIREMENTS

The contractor shall provide all-inclusive, turn-key crane services required to complete this project. This includes providing all necessary equipment, certified personnel, supervision, and related services.

3.1 Equipment Requirements
The contractor shall provide and operate the following:

- A mobile hydraulic crane, such as a Liebherr LTM 1400-7.1 or a fully equivalent model, that meets or exceeds the following minimum performance requirements:

  - Crane Class: 900-ton minimum.

  - Lift Capability: Must safely lift and place a load of 320,000 lbs. at a working radius of 28 ft.

- All necessary support equipment, including but not limited to rigging, slings, shackles, and a full counterweight package (approximately 441,000 lbs.).

3.2 Service Requirements
The contractor's services shall encompass all phases of the crane operation:

- Mobilization: All activities required for the transport, assembly, and setup of the crane and all associated equipment at the designated work site.

- Crane Operation: The operation of the crane for a period not to exceed 80 hours to safely move and set the eight (8) concrete targets.

- Demobilization: The complete disassembly, cleaning, and removal of the crane and all contractor-provided equipment from the government site upon completion of the work.

3.3 Personnel Requirements
The contractor shall provide a dedicated and fully qualified crew for the duration of the project, to include at a minimum:

- One (1) Certified Crane Operator

- One (2) Certified Rigger

- One (1) Licensed Truck Driver

- One (1) Self-Propelled Modular Transporter (SPMT)

All personnel must possess current certifications as required by federal, state, and local regulations (e.g., OSHA, ANSI/ASME).

### 4.0 GOVERNMENT RESPONSIBILITIES

The Government will be responsible for providing clear access to the work site and ensuring the site is prepared for crane operations.