FY27 COMMON MISSILE COMPARTMENT CONTRACT

STATEMENT OF WORK (SOW)

CLIN 0001 – Common Missile Compartment (CMC) Missile System Program Management and Support (US)

The Contractor shall make available and employ its facilities, personnel, and materials at the level of effort to conduct and manage the integration of the D5LE Missile System (MS) into the CMC for COLUMBIA Class submarines. Additionally, the contractor shall provide comprehensive technical and program management for the tasks. The tasks shall include:

- US program management and intergration for all MS elements of the COLUMBIA and CMC Program

- Systems engineering and intergration support

- Shipyard installation test program (SITP) support

CLIN 0002 - Common Missile Compartment (CMC) Missile System Program Management and Support (UK)

The Contractor shall make available and employ its facilities, personnel, and materials, at the level of effort, to conduct and manage the integration of the D5LE MS into the CMC for Dreadnought submarines.  Additionally, the contractor shall provide comprehensive technicaland program management for the tasks.  The tasks sall include:

- UK program management and intergration for all MS elements of the COLUMBIA and CMC Program

- Systems engineering and intergration support

- Shipyard installation test program (SITP) support

CLIN 0003 – CMC Coordinated Shipboard Allowance Equipment  (COSAL) Procurement and Delivery (US)

The Contractor shall procure, test, and deliver a Coordinated Shipboard Allowance Listfor US04, US05, and US06 and deliver ballast cores and ballast cans.

CLIN 0004 – CMC Coordinated Shipboard Allowance Equipment (COSAL) Procurement and Delivery (UK)

The Contractor shall develop and deliver a Coordinated Shipboard Allowance Listfor UK11.

CLIN 0005 - CMC EXPERIMENTAL VEHICLES

The Contractor shall perform the tasks below:

- The Contractor shall perform readiness tests for the VTV Missile in support for the VTV System Installation into US01 The Contractor and provide technical support as needed and will verify system connectivity and performance prior to Government Sea Trials and ensure operational availability.

- The Contractor shall monitor the performance of the VTV system while US01 is underway during Government Sea Trials and DASO 1A.

- The Contractor shall remove the VTV Missile and Data and Power Controller from US01 for inspection and technical support.

- The Contractor shall receive an XPOD kit and install on the US01 DASO 1A shot 2 candidate DASO missile.

- The Contractor shall procure and deliver XPOD and VTV installation spares.

CLIN 0006- CMC REENTRY SYSTEM HARDWARE

The Contractor shall produce and deliver the Navy Experimental Surveillance Test (NEST) flight hardware.

CLIN 0007 – UK CMC EXPERIMENTAL VEHICLES (UK)

The Contractor XPOD team shall receive and install two (2) XPOD kits on UK09 shots 1 and 2 primary flight candidate DASO missiles and conduct an XPOD Integration Readiness Review (XIRR) prior to installation on the missile.

CLIN 0008 – DASO-1A OPERATIONS, SUPPORT, AND RESULTS

The Contractor shall plan, conduct, and close the Flight Safety and Readiness Reviews required to gain Government approval to perform DASO-1A from the COLUMBIA platform.

The Contractor shall perform the pre-DASO, the DASO-1A, and the post-DASO operations as well as perform. the DASO-1A flight test analysis.

Applicable to all CLINS:

- MOSA: 10 U.S.C. 2446a.(b), Sec 805: MOSA - Interfacing development: The contractor and its subcontractors will follow DOD INST 5000.02 (dated August 10, 2017) and 5010.44 (dated October 16, 2019) as to Modular Open Systems Approach (MOSA) to the Major System Interfaces and ensure that the Government will acquire at least Government Purpose Rights (GPR) to data and intellectual property and to minimize proprietary elements to the lowest component level is critical to this effort. This includes all technical data, hardware, computer software, and computer software documentation to all interfaces for the Navy subsystems/elements. When any existing subsystem/elements incorporates other company proprietary information or Intellectual Property, it is up to the contractor and its subcontractors to obtain NDAs from each of those companies.

- Trademarks: “The contractor shall not assert any claim, in any jurisdiction, based on trademark or other name or design-based causes of action that are based on rights the contractor believes it has in the term(s) [contracting officers shall list term(s): SSP (Strategic Systems Programs), PMOSSP (Shipboard Systems), PMOSSP (Flight Systems), SWFPAC (Strategic Weapons Facility Pacific), SWFLANT Strategic Weapons Facility Atlantic), POLARIS, POSIEDON, TRIDENT, TRIDENT II, D5LE2 (Life Extension), NTIP (Naval Treaty Implementation Program), NOTU (Naval Ordnance Test Unit), CPS (Conventional Prompt Strike), FBM Operational Test Support Unit Two, SPUK (Strategic Systems Program Liaison Office), (the “Designation(s)”), against the Government or others authorized by the Government to use the Designation(s) (including the word(s), name, symbol, or design) acting within the scope of such authorization (i.e. claims for trademark infringement, dilution, trade dress infringement, unfair competition, false advertising, palming off, passing off, or counterfeiting). Such authorization shall be implied by the award of a Government contract to any party for the manufacture, production, distribution, use, modification, maintenance, sustainment, or packaging of the products and services identified under this contract, and the scope of such implied authorization is defined as the use of the Designation(s) in performance under such contract by the prime contractor and its subcontractors and suppliers at any tier. In all other cases, the scope of the authorization will be defined by the Government in writing. The contractor shall notify the contracting officer at least 30 days before asserting rights in, or filing an application to register, any one of the Designation(s) in any jurisdiction within the United States. Any such notification shall be in writing and shall identify the Designation(s) (including the word(s), name, symbol, or design), provide a statement as to its intended use(s) in commerce, and list the particular classes of goods or services in which registration will be sought.”