DEPARTMENT OF VETERAN AFFAIRS

STATEMENT OF WORK

General CT Injector (Brand Name or Equal)

Background:

The Bay Pines VAHCS has a requirement for Qty 1 General CT Injector. This is in support of replacing the current injector having past replacement date, end of warranty, end of service, and technological upgrades. The Contractor shall provide all resources necessary to accomplish the deliverables described in this Statement of Work (SOW), except as may otherwise be specified.  The delivery order will be awarded as Firm-Fixed-Price. Medrad Intego (Qty 1) are the basis of design (BOD).

Scope of Work:

The injector unit is designed for use in Nuclear Medicine.

General specifications for CT injector system BOD models (Brand Name or Equal) must include:

BOD: Medrad Intego

- Shall have a maximum weight of 800 lbs

- Shall have patient-specific dosing based on weight and specific PET imaging requirements

- Shall have customizable protocols for different radiopharmaceuticals

- Shall have a combination of dose preparation, quality control, and infusion capabilities

- Shall include touchscreen interface

- Shall include documentation of administered doses for record-keeping

- Shal have shielding capabilities to minimize radiation exposure for healthcare professionals during dose preparation and injection process

- Shall have the ability of automatic dispensing of radiopharmaceuticals

- Shall have equipment that is motor driven

- Shall have integrated dose calibrator

- Shall be within 2% accuracy within measured dose

- Shall have a dose range of at least 1-25 mCi

- Shall have a flow rate of 0.5 mL/set to 1 mL/sec

- Shall provide end-user training

Delivery:

- Items shall be delivered at the delivery address and the approved times listed below. Confirmation of delivery location will be provided post-award.

Delivery address and hours are Monday – Friday, 8am – 3:30pm, excluding Federal Holidays (New Year’s Day, Martin Luther King Day, President’s Day, Memorial Day, Independence Day, Labor Day, Columbus Day, Veterans Day, Thanksgiving Day, and Christmas Day).

Bay Pines VA

10000 Bay Pines Blvd

Bay Pines, FL 33744