Purchase Description

Navy-EXWC

Trailer Stake Breakbulk 34T

7 AUG  2025 Final

Notes to Offerors:

Offeror shall explicitly and completely identify any and all exceptions taken to the requirements of this specification. Drawings, literature, and any other information submitted with an offer do not constitute a stated or implied exception unless specifically identified in writing as an exception and accepted and implemented by an amendment to the solicitation or modification to a contract. Any exception deemed acceptable by the Government shall only apply to the specific item, requirement, etc. cited, and shall not extend to any other requirement.

Offeror offering other than brand name items identified below, when an “or equal” may be acceptable, shall furnish with their offers all technical data information, product descriptions, etc. to ensure that a determination may be made as to the equality of the product(s) offered (see clause titled Brand “Name or Equal” set forth in Section 52.211-6 of the Federal Acquisition Regulations.)

The following shall be provided with the offer.

- Detailed explanation, listed in a separate letter, of any exceptions to the specification, including alternate equipment proposed or a declaration of comprehensive understanding of the requirements with no exceptions taken.

- The offeror shall submit with the offer, preliminary drawings that show the following:

- Layout views of the proposed trailer including all equipment locations and basic dimensions verifying compliance with the requirements herein.

- Exterior front, rear, and side view with overall dimensions.

- Estimated curb weight and Gross Trailer Weight Rating (GVWR)

- Documented evidence of quality assurance procedures or system

e. 	Offeror shall provide access to customer and warranty service, as well as technical support via email, phone service, internet/web, or other method. Offeror shall provide detailed information describing their customer and warranty service access.

- SCOPE and BACKGROUND

Naval Facilities Engineering and Expeditionary Warfare Center (NAVFAC EXWC) has a requirement to replace an aging fleet of trailers based on current mission and support requirements. NAVFAC EXWC has documented requirements to address shortcomings of the current fleet that at a minimum comply with Military (MIL), American Welding Society (AWS), and Society of American Engineer (SAE) standards for tactical and commercial trailers. Compliance to these government and non-government standards introduces commonality with other military, Department of Defense (DOD), and civilian trailers, allowing fleet and joint operations to utilize the capabilities to their fullest. The trailers will also support operational readiness and functionality in austere conditions.

2.0 APPLICABLE DOCUMENTS

2.1 FEDERAL STANDARDS AND CODES

Code of Federal Regulations 49 CFR 393 (FMCSR) and 571 (FMVSS)

Application for copies of DOT publications should reference the code of Federal Regulations, 49 CFR, and the Federal Register and should be addressed to the Superintendent of Documents, U.S. Government Printing Office, Washington,

D.C. 20402. DOT standards may be accessed on the internet at http://www.access.gpo.gov.

MIL-STD- 209K, Interface Standard for Lift and Tie Down Provisions.

MIL-STD-1366E, Interface Standard for Transportability Criteria.

MIL-STD-1791D_CHG1, Department of Defense Interface Standard for Internal Aerial Delivery in Fixed Wing Aircraft

## 2.2 SOCIETY OF AUTOMOTIVE ENGINEERS, INC

J163 - Low Tension Wiring and Cable Terminals and Splice Clips.

J429 - Mechanical and Material Requirements for Externally Threaded Fasteners.

J551 - Measurement of Electromagnetic Radiation from Motor Vehicles and Devices.

J553 - Circuit Breakers.

J560 - Primary and Auxiliary Seven Conductor Electrical Connector for Truck-Trailer Jumper Cable.

J561 - Electric Terminals-Eyelet and Spade Type.

J575 - Tests for Motor Vehicle Lighting Devices & Components.

J576 - Plastic Materials, For Use in Optical Parts Such as Lenses and Reflectors of Motor Vehicle Lighting Devices.

J578 - Color Specification for Electric Signal Lighting Devices.

J682 – Front and Rear Wheel Splash and Stone Throw Protection.

J1128 - Low Tension Primary Cable.

J1292 -Automobile, Truck, Truck-Tractor, Trailer, and Motor Coach Wiring.

AMS-STD-595A- Colors Used in Government Procurement

Applications for copies of SAE publications should be addressed to SAE, Inc., 400 Commonwealth Drive, Warrendale, PA  15096

2.3 INTERNATIONAL STANDARDS ORGANIZATION (ISO)

ISO 668- Shipping Containers

ISO standards may be accessed on the internet at https://iso.org

2.4 NATO (STANAG)

NATO (STANAG) 4009 - Interface Geometry, Tractors and Semi-Trailers detailing the tow vehicle/trailer interface. Reference larger swing radius, 90.5 inches.

NATO standards may be accessed on the internet at https://nso.nato.int/nso/nsdd/listpromulg.html

2.5 OTHER STANDARDS

RP 803-Pre-Service Trailer Inspection.

Published by the Technical and Maintenance Council of the American Trucking Association.

2200 Mill Road, Alexandria, VA 22314.

TMC Recommended Practices may be accessed on the internet at https://tmcconnect.trucking.org

2.6 ORDER OF PRECEDENCE

In the event of a conflict between the text of this purchase description and the references cited above, the most stringent requirements, as determined by the government, take precedence. Nothing in this purchase description, however, supersedes applicable state and federal safety laws and regulations.

- GENERAL REQUIREMENTS

Materials shall be as specified herein. When materials are not specified, the trailer and all parts thereof shall be furnished to provide the intended function, durability, safety, and maintain good long-term appearance. All materials shall be new, shall be suitable for the intended purpose, and shall be free of any characteristics or defects in material and workmanship, which may affect the performance, function, durability, and serviceability of the finished trailer, or detract from its appearance. The government reserves the right to make the final determination of the suitability of all components and their arrangement on and in the trailer.

All items shall be of good quality, in safe operating condition. All safety devices shall be fully functional; to include all manual and automatic emergency shut-off devices. All safety guards, warning labels, and safety related instruction labels shall be properly always affixed.

The trailers shall be compatible with the following tractors, Government Owned Vehicles:

Note: EXWC shall briefly describe their commercial and tactical tractors and salient features of each type including:

Make/Model

Mack Granite 6X4, 160K Tractor, 2500 psi, 20 gpm

Series: Mack GR64BTX

GCW: 66,000 Lbs.

GCWR: 176,000 Lbs.

FAWR: 22,000 Lbs. but limited to 20,000 Lbs. due to tires

RAWR for Tandem: 46,001 Lbs. each axle

Wheelbase:  Center of front axle to center of tandem spread 204.5”

5th wheel Height: 53.5”

Rear swing/clearance area Center of 5th: 75”

Front swing/clearance area center of 5th wheel: from 5th wheel to front most obstruction on cab 66.5” / from 5th wheel to rear most of tractor 75” (with adjustability of 5th wheel left at 6” toward the rear of tractor left and 17.5” inches toward the cab of tractor left.

Tractor wet line kit rating: 2500 psi. @ 20 GPM. With ¾” quick connect coupler 30-gallon reservoir

Heavy Truck Tractor (HTT),

Series: Navistar Heavy Truck Tractor

GCW: 130,000 Lbs.

GCWR: 166,000 Lbs.

FAWR: 20,000 Lbs.

RAWR for Tandem: 84,000 Lbs.

Wheelbase Center of Front Axle to center of intermediate axle 197.5” and then from center of front axle to center of most rear axle 258”; spacing between rear duals is 60” front center of intermediate to center of rear dual.

5th wheel Height: 52” – 59” adjustable

Rear swing/clearance area Center of 5th: 73”

Front swing/clearance area center of 5th wheel: 58” (with adjustability of 5th wheel left at 15.5” toward the rear of tractor left and 30” inches toward the cab of tractor left.)

Tractor wet line kit rating: 3000 psi. @ 57 GPM. With 1 ¼” quick connect coupler

Heavy Interim Truck Tractor (HITT);

Series: International Work Star Model 7600 SBA 6X6.

GCW: 62,000 lbs.

GCWR: 26,280 lbs.

FAWR: 16,000 Lbs.

RAWR for Tandem: 46,000 Lbs.

Wheelbase Center of front axle to center of tandem spread 217”

5th wheel Height 50 “- 52”

Rear swing/clearance area Center of 5th wheel to rear most point on tractor 99” (However 5th wheel is adjustable)

Front swing/clearance area center of 5th wheel to forward most point of obstruction 79.5 “ (with adjustability of 5th wheel left 41” toward the rear of tractor and 14” inches toward the cab of tractor left)

Tractor wet line kit rating: 13-46 GPM @2500 psi with 1 ¾” female coupler

MTVR Tractor,

Series:MK31

GCW:62,000 Lbs.

GCWR:46,000 Lbs.

FAWR:16,000 Lbs.

RAWR for Tandem: 46,000 Lbs.

Tandem Spread: 56.4 “Centerline to centerline of both rear axle hubs

Wheelbase length: 212.2 “

5th wheel type: Fixed

5TH wheel height:  52” - 59” adjustable suspension.

Rear swing/clearance area Center of 5th wheel to rear most point on tractor 69”.

Front swing/clearance area center of 5th wheel to forward most point of obstruction 62.5”

Tractor wet line kit rating: N/A

  - CONFIGURATION REQUIREMENTS

Trailer Stake Breakbulk 34 Ton to transport containerized freight, breakbulk cargo, and construction supplies evenly distributed across the load space.

The trailer must be able to transport the following:

Six (6) 6 ft. 8 In. TRICON containers to be configured as a single load, with each container to be individually locked to deck.

Two (2) 20 ft. ISO containers to be configured as a single load with each container to be individually locked to deck.

One (1) 40 ft. ISO container to be configured as a single load and locked to deck.

A combination of TRICON, ISO-20ft, and 40ft sized shipping containers, and/or containerized freight and breakbulk cargo, palletized supplies, and/or miscellaneous construction supplies configured in a single load with each container to be individually locked to deck.

Placement of ISO locks must be submitted for review/approval by USN (E.g., due to interference with wheel configuration/axle spacing or other).

BODY:

Steel Frame.

CAPACITY:

70,000 lbs. minimum distributed

50,000 lbs. minimum or greater concentrated in 20 ft. or less distributed evenly across the load space

DIMENSIONS:

Overall Length: Not to exceed 48 ft.

Overall Width: Not to exceed 8.5 ft. (102 in.)

DECK:

Steel/Composite decking, 1 ¾ inch Rumber or similar.

All decking materials utilized must be impervious to fluids, insects, mud, oil, and UV rays.

Decking to have proper cross member spacing to support decking/load without surface deviation.

Unloaded Deck Height: Trailer shall remain level and shall not to exceed 59 Inches when connected to a tractor at 5th wheel height of 51 inches.

When deck loaded, loaded trailer shall not exceed DOT Standard 13 ft. 6 inches total height.

Bolt on removable front steel headache rack/headboard.

Two forklift pockets on center (4-inch (H) x 10 inches (W), front to back) located at top of headache rack.

Spacing between Fork Pockets approximately 43 inches apart.

DOT reflectors, and lockable chain rack

Work lighting protected under top beam. Controls recessed and protected on rack, driver side, to allow user to switch off manually.

Swing radius: 84 Inches (Minimum)

A means to disconnect the wiring installed on the headache rack shall be provided and must have enough slack in the wire so that it is not taunt and is in a position to be easily unpluggable and require no tools to get to it or access it.

All hydraulic and or air hoses will have a sling available for use to protect lines from getting damaged when the trailer is hooking and unhooking from tractor/prime mover

All the hydraulic system or Air systems must be compatible with all the Navy prime movers listed here in.

LANDING GEAR:

2-speed landing gear, handle on driver side

Landing gear must not interfere with commercial or military tractor variants to prevent Tractor/Trailer or landing gear damage. Landing gear crank handle shall include a positive locking feature to secure during transit

ELECTRICAL:

6–28-volt DC Electrical / LED lighting system.

NATO intravehicular plug, 12 pin/24V connection and must be centered in any opening if it is placed into one to allow the thickened plug area to be able to plug into port.

Commercial Intravehicular plug, 7-way, 12V connection.

ABS light, driver side rear.

Tail lamps, stop lamps, and reflectors shall be capable of operation throughout all operating conditions and shall meet Department of Transportation (DOT) standards.

All running lights shall have proper weatherproofing.

All wiring harnesses shall be modular, sealed using weatherproof connectors, enclosed in protected jacket/cover, including grommets to prevent wiring damage, and be secured with hard brackets and corrosion resistant steel bolts and nuts.

All wiring connections shall be waterproof and protected where it terminates at fixtures. All wiring harnesses shall be modular, sealed using weather proof connectors, enclosed in protected jacket/cover, include grommets to prevent wiring damage, and all electrical to be secured with bolt through clam shell brackets with rubber inserts and enough of them to prevent electrical drooping or movement of electrical while in operation and must be fashioned to trailer in a way that does not allow it to be caught of brush or other surfaces when operation in an off highway scenario; no other electrical securing options such as zip ties, tape, wire, hangers etc. will be accepted.

All hoses must be secured to chassis with hard brackets and corrosion resistant steel bolts and nuts not attached to axle.

SUSPENSION/AXLES

Air ride 25,000 lbs. (nominal).

Air ride suspension dump valve (Hold, raise, lower with auto dump), located at rear, recessed in to back of trailer.

Capable of backing up 15°-degree ramp angle without degradation to asset performance or damage to suspension or frame.

Rear tires and rear wheel suspension should not strike trailer frame.

Automatic slack adjusters.

Mechanical Hub-odometer mounted on drivers’ side front axle (NO Digital hub meter will be accepted.)

Oil bath hubs with visible oil level indicators.

BRAKES:

Anti-Lock Braking System (ABS), Commercial Standard. ABS system/wiring should be adequately protected and mounted so as not to interfere with prime mover connection or operation of the trailer.

Mechanical air tank draining control, easily accessible from side or rear.

WHEELS/RIMS

Solid steel single piece wheels with lugs, dual wheels (super singles are prohibited).

Wheels/Rim size information shall be provided.

All wheels used must have easy access to air valves to air up or down, additionally only analog hub meters will be used.

Refer to Paint and Marking Section for further guidance/information.

TIRES:

New, non-retread, Commercial Highway, Pneumatic Rubber radial tires, rated for trailer at full load weight capacity.

All tires must be the same, brand matching.

Tire size information shall be provided.

The manufacturer date on all tires shall be no greater than one year at time of delivery of equipment. (No double coins)

Refer to Paint and Marking Section for further guidance/information. Tire pressure shall be stenciled/painted black in 1-inch black figures on visible area of frame /side beam above each tire.

(E.g. above fender “TP-25").  Alternative use of decals by OEM in place of stencil/paint must be submitted for review/approval by USN.

Valve extensions shall be provided on inner and outer tires and must be easily accessible

PERFORMANCE:

Trailer shall be:

Commercial design, conforming to 50 State Department of Transportation (DOT) requirements and Federal highway and bridge laws.

Exception:  Permitting may be required for transport in certain U.S. states or across state lines.

Operable cross country on highways, paved secondary, gravel, unimproved, or cobblestone roadways.

Able to traverse on, through, over, and across multiple on-and-off road terrain and surfaces, for transfer, transport, or on/offloading of cargo, equipment, or material.

Capable of operating in a wide variety of climatic conditions and environments, able to function in operating temperatures ranging from -25 degrees to 130 degrees Fahrenheit, must be capable of 10 hours run time continuously with a hour maintenance between periods.

Air, brake, hydraulic lines, and fittings shall be free from leaks and mounted to prevent damage during vehicle operation.

Ground Clearance: Should be sufficient to prevent damage to the axles or frame when trailer in operation on unimproved terrain while loaded.

Capable of backing up a 15°-degree ramp angle without degradation to asset performance or damage to suspension or frame.

All welding, to include welded, bolted, and riveted construction utilized, shall be in accordance with the highest standards of industry and must conform to Structural Welding Code for Steel and Stainless Steel IAW American Society of Mechanical Engineers (ASME) and American Welding Society (AWS) Standards.

All welds shall be free of excess slag and splatter.

All welds shall be uniform, have adequate penetration, and be free of voids.

All rubber components shall be ozone and UV resistant.

AUXILIARY:

Glad hand covers affixed with chain to glad hand, covers shall be positioned to not interfere with coupling to tractor and interfering with tractor while hooking and unhooking or turning while in operation. Dummy plugs to protect disconnects from debris.

Two sealed/weatherproof, lockable steel toolboxes/storage compartments with lockable lid latches and reinforced door hinge(s) to hold chains, binder, electrical pigtails, intermodal connectors/ISO container locks, cargo straps, tools, etc. (Positions of boxes recommended by OEM to be approved by USN).

One (1) new, spare tire/wheel with lugs to match other tires/wheels on trailer. Mounted spare should not interfere with prime mover connection, loading to aircraft or ship via ramp, lift or tie down, or operation/loading of the trailer.

ISO pockets with removable, interchangeable locks.

Must be capable of accommodating a combination of various sized shipping containers and/or containerized freight configured as a single load, with each container to be individually locked to deck (see objective description).

Placement of ISO locks and spacing must be submitted for review/approval by USN.

Reinforced outboard and inboard flanges (as applicable).

MILITARY MODIFICATIONS

The trailer shall be able to interface with standard and tactical prime mover models with 49 to 57” inches 5th wheel height.

Load Securement:

Steel Sliding winch track (Roadside) with twelve (12) minimum, 4-inch heavy duty, storable, sliding winches, 4-inch (W) x 25 ft. (L) (minimum) cargo straps with flat hook (5,500 lbs. capacity minimum for each individual winch and strap) Working Load Limit (WLL) each to secure loads.

Steel Rub Rail (Curbside) mounted to bottom of side channel.

ISO Shipping Container Twist Lock; Twenty-Four (24) ea. Twelve (12) per side.

ISO Shipping Container Cone Twist Lock Handle Tool Bar: Two (2) ea. One per side.

Stake Pockets: 24” on center.

Chain/Key slots on top of each side outer trailer bed deck frame, big enough to be able to pass the hook of the chain that is to be used for securing.

4-Way Center Chain/Key slots to complement outer chain key slots, big enough to be able to pass the hook of the chain that is to be used for securing.

Flag holders Two (2); one (1) pair each at front and back of trailer.

Tools: to include breaker bar, lug wrench handle, and socket tool to remove wheels.

Two (2) ea. keys per toolbox.

ISO Cone Twist Lock Handle Tool Bar: (2) ea (One per side).

Heavy duty black rubber mud guards without markings.

Reflective Tape NHTSA (FMVSS) DOT C2.

MILITARY MODIFICATIONS:

Trailer shall be able to interface with standard and tactical prime mover models with 49-to-57-inch 5th wheel height.

AIR TRANSPORT:

The trailer shall be capable of air transport by C-130, C-17, and C-5 Aircraft. Ref to MIL-STD-1366 and MIL-STD-1791D_CHG-1.

(See Air Transport/Air Transportability Test Loading Agency (ATTLA) paragraph for further guidance).

Transportability shall be maximum curb weight of trailer.

4540.07 CHG-3 Operation of the DoD Engineering for Transportability and Deploy ability Program.

MIL STD-1366E Interface Standard for Transportability Criteria

MIL-STD-1791D_CHG-1 Designing for Internal Aerial Delivery in Fixed Wing Aircraft.

MIL-STD-209K Interface Standards for Lifting and Tie down Provisions

Process for ATTLA Air Transportability Certification

Equipment or ground vehicles that require USAF Fixed Wing Air Transportability certification via C130E/H/J, C17, or C5 shall be in accordance with the following:

DODI 4540.07 CHG-3 Operation of the DoD Engineering for Transportability and Deploy ability Program.

MIL STD-1366E Interface Standard for Transportability Criteria

MIL-STD-1791D_CHG-1 Designing for Internal Aerial Delivery in Fixed Wing Aircraft.

MIL-STD-209K Interface Standards for Lifting and Tie down Provisions

The following sequence of events shall occur:

The General Services Agency (GSA), Defense Logistics Agency (DLA), or other Acquisition source will:

Notify the purchasing customer/Government Agency sponsor of contract award.

Forward a copy of the GSA, DLA, or other Acquisition source award order/contract to the purchasing customer/Government Agency and awarded OEM vendor/contractor.

a) (GSA, DLA, or other Acquisition Source) Order number or contract to include all contract options ordered

b) Copy of the Motor Vehicle Delivery Order (MVDO) (as applicable)

c) Purchasing customer/Government Agency Sponsor contact information (name/phone/email).

d) OEM vendors / contractors contact information (name/phone/email)

The purchasing customer/Government Agency Sponsor will:

Act as the OEM vendor/contractor’s Government Agency Sponsor to submit an ATTLA package for any equipment or ground vehicles that require Air Transportability certification.

Submit a Sponsor Letter within 10 working days post contract award to request to have the Air Transportability Test Loading Agency (ATTLA) open a project for an Air Transportability Certificate.

Sponsor requests to be sent by the purchasing customer/Government Agency to the

Air Transportability Test Loading Agency (ATTLA) at Wright-Patterson Air Force Base at (ATTLA@us.af.mil).

Provide ATTLA with the following information as reference for the ATTLA project case file:

a) Product: (insert Product name here i.e. Truck Tractor 20T, Trailer Low Bed RGN 60T, etc.)

b) Contract: (GSA, DLA, or other Acquisition Source) Order number to include all contract options ordered

c) Copy of the Motor Vehicle Delivery Order (MVDO) (as applicable)

d) Purchasing customer/Government Agency Sponsor contact information (name/phone/email).

e) OEM vendors / contractors contact information (name/phone/email)

Provide awarded OEM vendor/contractor a checklist of items required by ATTLA that they must complete and submit to the purchasing customer/Government Agency upon completion of the product build as part of the ATTLA Transportability package submission.

Confirm receipt of OEM vendor/contractor ATTLA package submittal to purchasing customer/Government Agency Sponsor.

Verify final package submitted to ATTLA is complete including all required information.

Track status of the ATTLA certification review including the date of first submittal, any requests for information by ATTLA, any vendor resolution of nonconformance to meet ATTLA requirements, or other items as required.

Forward copies of final ATTLA certification to EXWC EX branches (EX2/EX4/EX5 as applicable)

During production, the purchasing customer/Government Agency Sponsor shall:

Collaborate with OEM vendor/contractor to review work in progress: the OEM Finite Element Analysis (FEA) and other engineering data, drawings, photos, etc.

The following angles, dimensions, and descriptions should be checked against data and drawings (see ATTLA Universal Data Spreadsheet for instructions/information) as part of the ATTLA technical data package submittals:

a) Angle of approach

b) Ramp breakover angle

c) Angle of departure

d) Height, longitudinal location and identification of highest component on the equipment/vehicle

e) Dimensions and locations of any significant projections on truck

f) Curb weight of each axle

g) Wheelbase

h) Front overhang

i) Rear overhang

j) Articulation of rear suspension, unloaded, each axle (curb weight)

k) Rear axle spacing

l) Axle rating, front, and comparison to 1 ¼ times (curb weight) load

m) Axle rating, rear, and comparison to 1 ¼ times (curb weight) load

n) Suspension rating, front, and comparison to 1 ¼ times (curb weight) load

o) Suspension rating, rear, and comparison to 1 ¼ times (curb weight) load

p) If axle stops are to be removed for ramp loading on aircraft, verification that the driveline

remains intact when cresting maximum ramp slope. Provide guidance and recommendations on changes needed to correct any issues identified where the certification request may be denied, or the information package is incomplete.

q) Configuration and directional capability of equipment/ground vehicle tie down provisions (Reference MIL-STD-1791-D_CHG1, 5.3.3.2)

Delivered/ Fielded Equipment with NO ATTLA Certification:

In cases where Equipment or Ground Vehicles were delivered to a customer without an Air Certification letter, the purchasing customer/Government Agency Sponsor shall provide the OEM vendor/contractor with guidance on the ATTLA process.

Working with the Acquisition Source (GSA, DLA, other) the purchasing customer/Government Agency Sponsor will direct the OEM to either retrieve the Equipment / Ground Vehicle delivered for needed modifications or arrange for a local shop to perform any needed modifications to satisfy ATTLA requirements to obtain transportability certification. Upon completion of modification work, the OEM vendor/contractor shall advise the purchasing customer/Government Agency Sponsor.

The Purchasing customer/Government Agency Sponsor will review OEM corrections of any issues and then submit a technical data package on behalf of the OEM/vendor for ATTLA review for certification.

Air Transportability Test Loading Agency (ATTLA) will:

Contact the purchasing customer/Government Agency Sponsor to acknowledge request and to confirm an ATTLA project has been opened/established. Other information/data/analysis could be required.

(Pending action by the purchasing customer/Government Agency to submit the vendor/contractor’s completed package for ATTLA review, analysis, and certification of for Air Transportability).

Upon receiving the submission of the data package (i.e. characteristics of the equipment, 3-view dimensional drawings, proposed tiedown pattern) ATTLA will then conduct a technical analysis of the transportability at no cost to the requesting federal agency.

ATTLA will compare the OEM vendor/contractor data and all weight estimates against the rated capacities (E.g. Equipment, axle, suspension, etc.) to determine compliance with MIL-STD-1791D_CHG-1 Department of Defense Interface Standard for Internal Aerial Delivery in Fixed Wing Aircraft.

Should submitted package requirements be incomplete, or should an item fail to qualify for airlift transport, ATTLA will provide an explanation stating reasons for rejection to the purchasing customer/Government Agency and make recommendations for changes that will allow an item to meet ATTLA certification requirements.

Upon approval, ATTLA will provide a transportability certification memo to the purchasing customer/Government Agency sponsor stating the conditions of approval / certification of

transportability within 60 days of receiving a complete package with all proper requirements met.

Note: For items in development, ATTLA may offer design assistance through the applicable procurement office to ensure that the vendor produces an item that ultimately meets all airlift objectives. Because developmental items are subject to continual changes, ATTLA personnel may wish to participate in design reviews and other technical interchanges during the evolution of a design. However, funding must be provided for ATTLA personnel to attend onsite activities at an OEM vendor production site.

Teams or video teleconference capability may be offered by ATTLA at no charge to the purchasing customer/Government Agency or OEM at any phase of equipment/vehicle production. (Meetings must be requested/scheduled with ATTLA)

The OEM vendor/ contractor will:

Upon completion of the build of the equipment/vehicle, submit the following technical data as a package to the Mailing Address of the purchasing customer/Government Agency Sponsor (as provided on the Motor Vehicle Delivery Order (MVDO) or contract, or other mailing address identified/specified by the purchasing customer).

a) ATTLA Universal Data Spreadsheet (appropriate tab sheets completed - See Instructions and General Information tab sheet for guidance on completion)

a. Identify equipment type needing Air Transportability Certification.

b. Name of the item from the data plate (E.g. crane, tracked vehicle, truck tractor, trailer, etc., with naming format as: Truck Stake 2-Door 20 Foot Bed 30T, Trailer Lowbed HYD RGN 60T, etc.

b) Identify type of USAF aircraft (C-130E/H/J, C-17, or C-5) (provided by the purchasing customer/government agency sponsor). ATTLA will identify the USAF aircraft required if different from the submittal package request.

c) The purchasing customer/government agency sponsor will provide the OEM/vendor with a technical data package template recommended as a tool/ resource to ensure all requirements are provided in the submittal for transportability certification. This template will help the OEM identify MIL-STDs and requirements needed for the specific variant of equipment or vehicle to be certified.

d) Provide a technical data package as per MIL-STD-1791-D para-6.4 Air Transport Certification Process. Package should include physical characteristics of the equipment, structural strength with substantiating information (for analysis of tiedown capability), and proposed tiedown pattern. Dimensions may be submitted using drawings, certain CAD files, the ATTLA data sheet, or annotated photographs. The structural analysis should list assumptions, material properties, and force diagrams in addition to calculations and conclusions.

e) .. The contractor shall supply the ATTLA Approved Certification memo to the purchasing customer/Government agency after receipt of the first production equipment/vehicle.

A copy of the ATTLA certification memo shall be forwarded to the GSA IOA when it is available.

The contractor shall present to the GSA IOA a certified weight ticket of the completed trailer including curb weights of all axles. The GSA IOA shall then confirm and validate that the actual loaded axle weights are equal to or less than 80% of their ratings in accordance to the ATTLA certification.

ATTLA (The Air Transportability Test Loading Agency) is located at Wright Patterson Air Force Base and can be contacted via e-mail at (ATTLA@us.af.mil).

The equipment/ground vehicle shall be inspected to determine that it conforms to the approved certification memo the contractor received from Air Transportability Test Loading Agency (ATTLA).

Lift and Cargo/Equipment Tie Down Provisions:

The contractor shall be required to provide engineering analysis or other documentation, to include design data plate with specifications and drawings mounted to trailer, to validate Cargo and Equipment tie down and lift provision testing and certification to include maximum Cargo and Equipment tie down and lifting/sling weight capacity in accordance with MIL-STD-209K.

Pull Test:

If analysis alone cannot positively determine the transportability of the Equipment / Ground Vehicle, then a test loading may be required.

Test loadings are done when the transportability of an item is uncertain.

Test loads require a formal test report and are usually conducted as Special Airlift or Transport Missions.

The purchasing customer/Government Agency Sponsor shall be responsible for requesting a test loading as well as all costs associated when it is determined to be required.

SDDCTEA

Engineering structural analysis.

Prior to all testing, the OEM Vendor/contractor may provide dimensional and material design data on the provisions and their surrounding structure. This data will be used to perform a structural analysis of the provisions. In lieu of providing the required design information, the contractor can perform a structural analysis and provide the results of the analysis for review and concurrence. Either the required data or the contractor's structural analysis results should be provided at least 90 days prior to scheduled provision testing. The purpose of the structural analysis is to assess the risk of testing, thereby reducing the number of failures or possibly eliminating the test in acceptable cases. This will result in lower overall acquisition costs to materiel developers.

Removal or relocation of mechanically attached (non-welded, non-riveted, etc.) components with common tools, requiring not more than 1 man-hour total to remove, relocate and tie down, and not more than 1 man-hour total to return the vehicle to its original, as opposed to reduced, configuration; shall be acceptable.

a. The self-mobility of the vehicle shall not be affected by reducing its configuration. Tie downs for removed or relocated equipment shall be furnished.

b. For Trailers, the rated capacity of the axles and suspension system shall be not less than 1.25 times the load imposed on each by the curb weight of the trailer.

c. The vehicle shall be transportable as described above without any other special provisions and without any shoring unless unavoidable without significant alteration to the vehicle.

d. The vehicle shall not be delivered to the Government in its reduced configuration.

For Rail or pull test certification contact: the Military Surface Deployment and Distribution Command Transportation Engineering Agency (SDDCTEA) for modes of transportability.

SDDCTEA is in Newport News, VA, and can be contacted by e-mail at “dpemail@tea.army.mil” or by phone at (757) 599-1113.

A transportability report form for SDDCTEA can be obtained at http://www.tea.army.mil/pubs/nr/deploy/transinstruction/DI-PACK-80880B.pdf” \t “_new”. References, publications, assistance and engineering services are available from SDDCTEA. Testing information can be obtained from the Aberdeen Test Center (ATC). ATC can be contacted through their website at “http://www.atc.army.mil

LIFT AND TIE DOWN PROVISIONS:

The trailer shall be equipped with lifting provisions and tie down provisions to ensure interoperability between transported equipment and lifting and tie down devices commonly used in the transportation environment. If there is any drainage holes utilized in pockets of tie down or lift ring areas will require protection from chains or other foreign objects to not contact part of the chain or load from striking anything but pocket and lift ring area or decking surface area, the reason is to prevent any damage to other parts of trailer; i.e.. Valve bodies, hoses, electric lights etc.

Lifting:

Lift provisions (such that no spreader bar is required) shall meet the Interface Standards and requirements for transportability in accordance with MIL-STD-1366, and shall permit the lifting of the trailer, without payload, in accordance with MIL-STD-209K.

Total lift capacity, to include maximum sling lift weight, shall be reflected via data plate.

All design loads for lift provisions shall be for maximum Curb Weight of trailer.

Total Lift Ring “Lift Rating” capability shall be reflected via data plate and on the D-Ring. The capacity of each tie down D-Ring shall be a Raised Casting located on the body of the D-Ring and mounted so that the capacity information is facing outward.

All lift provisions must be seated, properly welded to frame, and rated for capacity of load for visibility and if there is any drainage holes utilized in pockets that they have protection from chains or other foreign objects not part of the trailer from striking anything but pocket and lift ring.

Refer to Paint and Marking Section for further guidance/information.

Tie Down:

Tie Down provisions and hardware shall meet the Interface Standards and requirements for transportability in accordance with MIL-STD-1366 to permit transport of the trailer, with or without payload, in accordance with MIL-STD-209K and MIL-STD-1791D CHG-1.

All design loads for tie down shall be for full load capacity of trailer.

Total Tie Down “Rating” capacity shall be reflected via data plate and on the D-Ring. The capacity of each tie down D-Ring shall be a Raised Casting located on the body of the D-Ring and mounted so that the capacity information is facing outward for visibility.

All tie down provisions must be seated, properly welded to frame, and rated for capacity of load for visibility.

Refer to Paint and Marking Section for further guidance/information.

PAINT AND MARKINGS

PRIMER, PAINT, FINISH, CORROSION CONTROL

Preparation:

Trailer to be primed/painted prior to installation of air, decking, brake, electrical, and hydraulic lines.

Rumber or composite decking shall be installed post paint/coatings so that no unsightly paint or coating overspray is evident on any portion of the decking material.

Chassis frame, undercarriage, underbody, and all base materials (E.g. I-beams, axles, wheels, etc.) should be free of cracks, burrs, sharp edges, and any weld slag, splatter, rust or drill through and driving of fasteners into cross members weakening them and leaving metal exposed that may affect the corrosion performance and coatings adhesion to frame, undercarriage, and wash down areas.  Exposed fasteners shall be painted flat lusterless green.

Before painting, all steel/metal surfaces shall be cleaned to remove any dirt, grease, oil, moisture, silicone, rust, sanding residue, scale, wax, or other foreign matter that may have accumulated, and surface prepared properly to ensure coating adhesion.

Primer/Base:

Two-part Epoxy NON-CARC corrosion resistant coat or zinc rich primer shall be > or = to 4 MILS Minimum dry-film thickness (DFT).

Paint:

Color:  Flat lusterless green #34094, in accordance with SAE AMS-STD-595A. (Matte, no gloss)

Finish:

Poly Urethane NON-CARC corrosion resistant coat shall be > or = to 4 MILS Minimum dry-film thickness (DFT)

Finish shall be suitable for outdoor/UV exposure without undue fading of colors or pigment, or chalking.

Combined total:

Primer, paint, and finish coats shall be no less than 8 MILS Minimum dry-film thickness (DFT).

A non-skid application (E.g., Applied grit coating, grip tape, other) should be applied to top of all horizontal frame surfaces for safety. All Product information to be submitted to USN for review/approval.

Undercoat Corrosion Protection:

A corrosion resistance and wear protection preventive undercoating to prevent corrosion.

(E.g., Tectyl, LINE-X, or equivalent) shall be applied evenly to top flanges and outer webs, and all surfaces, frame down, including wash down areas for protection from corrosion and road debris.

Undercoat shall be dry to touch (not tacky) and reflect no excessive running, sagging, or build-up. Any overspray shall be removed.

All Product information to be submitted to USN for review/approval.

VENDOR SUITABLE PRIMER/PAINT/FINISH/UNDERCOAT ALTERNATIVES:

Vendors may propose ALTERNATIVE primer/paint/finish and undercoat applications to the USN.

Manufacturer must supply the USN with product chip samples and product technical data sheets (TDS) for primer, paint, finish, and undercoat coatings, (to include product numbers, application, viscosity, certifications/testing documentation (E.g. salt spray, gravel, hardness, etc.), and MIL thickness minimums for review and approval by the USN prior to OEM application.

Alternate coatings shall meet or exceed the performance requirements stated above, including:

(1) Be capable of withstanding 240 hours of salt spray in accordance with ASTM B117/D1654 without rusting or blistering on un-scribed area and rust creepage rating of 5 or higher on scribed specimen

(2) Chipping resistance in accordance with ASTM D3170.

Suitable primer/paint/finish and undercoat coating alternatives must be compatible and shall be field repairable.

A non-skid application (E.g., Applied grit coating, grip tape, other) should be applied to top of all horizontal frame surfaces for safety.

All Product information to be submitted to USN for review/approval.

REGISTRATION/MARKINGS/LABELS:

U.S. Navy stenciled/painted identification and registration markings shall be applied to all trailers IAW NAVFAC requirements consisting of:

- Agency identification, “U.S. NAVY.”

- U.S. Navy registration number assigned (E.g., U.S. NAVY XX-XXXXX).

Alternative use of decals, laser engraving, or riveted, or welded plates by OEM in place of stencil/paint must be submitted for review/approval by USN.

The Government will provide a listing of the USN Registration Numbers to be applied to sides and rear of trailer.

Markings shall be in the following Font, Format, Size, and Location:

Side:

Centered on each side of the front quarter of the trailer.

- Font Format: Stencil.

- Size and Stroke: 3-inches high, ½-inch stroke.

- Dash: ½-inch high, 1-inch long.

- Color: Matte Black.

- Example:  U.S. NAVY 97-12345

Rear:

The agency identification and military registration number (USN) shall be readily identifiable from the rear (E.g., rear or side of rear ramp/beavertail, etc.) and arranged in a single line equidistance from the vehicles outside edges. “U.S. Navy” on the left (driver side rear) followed by the USN registration number on the right (passenger side rear).

- Font Format: Stencil.

- Size & Stroke: 3-inches high, ½-inch stroke.

- Dash: ½-inch high, 1-inch long.

- Color: Matte Black.

- Example – U.S. NAVY                       			97-12345

When small surfaces are unsuitable or space is limited for marking sizes shown (E.g., at rear), the OEM shall review and confirm use of a shortened military registration name (E.g., “USN”) with the U.S. Navy to suit the space to include location and placement.

- Example – USN 97-12345

Alternative use of decals, laser engraved, or riveted, or welded plates by OEM in place of stencil/paint must be submitted for review/approval by USN.

Any trailers requiring concealed markings shall be furnished with a name/data plate only, to be permanently attached to a concealed but readily accessible interior surface of the trailer, to the underside of the lid of a lockable toolbox, or in another concealed area.

WHEEL/RIM:

Wheels/Rims shall be prepped, painted, and finished based on industry standard (Matte black or flat lusterless green #34094 per Paint color specification requirements).

TIRE PRESSURE MARKING:

Tire pressure shall be stenciled/painted matte black on visible area of frame/side beams each side between the 1st and 2nd axle wheel well.

Marking shall be in the following Font Format, Size, and Stroke:

- Font Format: Stencil.

- Size and Stroke: 1-inch high 5/32-inch stroke.

- Dash: ½-inch high, 1-inch long.

- Color: Matte Black

- Example:  tp – 120 PSI.

Alternative use of decals, or laser engraved, riveted, or welded plates by OEM in place of stencil/paint must be submitted for review/approval by USN.

LIFT AND TIE DOWN PROVISION MARKING:

LIFT POINT MARKING

Lift provision markings shall be as follows.

Lift provisions and hardware (for air, sea, and highway) shall be painted flat lusterless green #34094, in accordance with SAE AMS-STD-595A (No gloss).

Lift Provisions shall be identified/marked via laser engraved/stencil/painted matte black on a visible area of frame or side beam next to the lift provision.

Lift devices shall be identified by the words LIFT HERE.

Markings shall be in the following Font, Format, and Size:

- Font Format: Stencil.

- Size and Stroke: 1-inch high ½-inch stroke.

- Color: Matte Black.

- D-Ring Color: Flat lusterless green #34094.

- Example:  LIFT HERE

Alternative use of decals, laser engraved, or riveted, or welded plates by OEM in place of stencil/paint must be submitted for review/approval by USN.

TIE DOWN MARKING

Tie Down provision markings shall be as follows.

Tie-down provisions and hardware (for air, sea, and highway) shall be painted flat lusterless green #34094, in accordance with SAE AMS-STD-595A (No gloss).

Tie down markings shall be identified via data plate and laser engraved/stencil/paint, shall also clearly indicate the points/attachments are which it is intended for.

Tie Down devices shall be identified by the words TIE DOWN.

Markings shall be in the following Font, Format, and Size:

- Font Format: Stencil.

- Size and Stroke: 1-inch high by ½-inch stroke.

- Color: Matte Black.

- Metal Bracket / D-Ring Color: Flat lusterless green #34094.

- Example:  TIE DOWN

Alternative use of decals, laser engraved, or riveted, or welded plates by OEM in place of stencil/paint must be submitted for review/approval by USN.

MARKING EXAMPLES:

DATA PLATES:

All data, shipping, transportation, and Instruction plates shall be metal, non-ferrous material, and mounted on all trailers using screws, rivets, or other mechanical devices IAW NAVFAC requirements.  The data plates shall be inscribed or etched.

No sticker type data plates shall be acceptable.

Safety decals must be of high quality for long-lasting durability.

The following information shall be embossed /engraved onto the plates:

Nomenclature: As applicable to the type of vehicle specified in the contract (E.g., Trailer Stake Breakbulk).

Manufacturer name, make, and model:

Manufacturer’s serial number:

U.S. Navy registration number (USN):

National Stock Number (NSN):

Vehicle curb weight:  _______________________________ 	kg (pounds).

Payload, maximum:   ______________________________ 	kg (pounds).

Gross vehicle weight rating (GVWR) max. __________kg (pounds).

Gross combination weight rating, max. ______________kg (pounds).

Date of delivery: (month and year).

(Date of delivery shall be left blank until the vehicle is accepted by the Government inspector.

Date of delivery shall not be prior to date of acceptance indicated on form DD 250.)

Contracting Agency:

Contract number:

Warranty:  ____________________ mo. __________________ km (miles).

U.S. Property

TECHNICAL SUPPORT:

The offeror shall provide access to customer and warranty service, as well as technical support via email, phone service, internet/web, or other method.

An initial response for technical support from the offeror or a certified dealer shall be with-in three (3) business days.

An after-hours emergency phone number shall be made available to provide first response assistance to help resolve emergency service /maintenance issues.

Parts supportability shall be provided utilizing commercial-off-the-shelf (COTS) parts to the maximum extent possible

OPERATOR, MAINTENANCE, PARTS, SERVICE MANUALS, AND SUPPORT

All printed and computer materials shall be written in English.  and must be able to be recreated by government for government use only and cannot be proprietary in the use with government only

Operator and Maintenance instructions manuals and original equipment manufacturer parts listings shall be provided in the following formats:

PDF reader / search capable on a CD or web-based access.

Traditional paper copies.

Operator's manuals shall contain procedures for the safe operation of all system and sub-system components along with associated safety cautions and warnings applicable to equipment operation.

Operator’s manual to include 1(800) phone number, email, or other method for access.

Service Maintenance Manual(s) shall contain:

Original equipment manufacturer's preventive maintenance schedules, procedures, and safety precautions applicable to each maintenance task.

Instructions and procedures for inspecting, testing, calibration, service, repair, replacement, and overhaul of all installed components, equipment systems, and sub-systems (e.g., trailer, brakes, engine, electrical system, etc.)

Parts Manuals shall be an illustrated parts breakdown, include military modification items, and shall contain:

A pictorial representation (i.e., exploded view) of all system and sub-system components, indentured to the item level, identified by individual item numbers.

Shall list complete item names, original equipment manufacturer part numbers, quantity per end item, and the quantity per assembly.

Shall provide manufacturer name, cage code, and part numbers for all replacement parts.

Supplemental Manuals for all work tool attachments shall be included as applicable.

Operator/Maintenance/Parts manuals to include Point of Contact phone number, email, or other method to respond to customer questions/requests for manual support.

The following shall be delivered 30-45 days After Receipt of Order (ARO)

One (1) Standard Commercial off the Shelf (COTS) Product Manual, with parts list, for past build or similar equipment/product configuration to allow customers to begin Product Support Development.

The following shall be delivered 150-180 days After Receipt of Order (ARO)

Draft:

One (1) set of Tech Manuals (PDF reader / search capable with editing capabilities on a CD or web-based access)

The contractor shall provide details of the major systems and components in the draft, (i.e.: axles, brakes, wheels, pony engine, etc.)

Prior to submission of the DRAFT Technical Manual, individual component cut sheets may be offered or provided by OEM as they are finalized. Final as Built Tech Manuals shall include same component cut sheets if submitted.

To:

Naval Facilities Engineering Command (NAVFAC)

Engineering and Expeditionary Warfare Center (EXWC)

1000 23rd Avenue, Bldg. 1100

Port Hueneme, CA  91343

Attn:  Ground Vehicles (ExPO EX22)

(POC to be provided).

The final versions of the manuals shall be completed and delivered after completion of 1st Article review and testing at final destination location but a current version although not the final needs to be onsite at the time of the 1st Article review for referencing; If any items are found at the time of the 1st Article review that need correcting or require reconfiguration of any sort that affects the manuals data will; the manuals will need to be updated and delivered first in order to receive a final acceptance.

Final As Built:

One (1) set of Tech Manuals (PDF reader / search capable with editing capabilities on a CD or web-based access)

To:

Naval Facilities Engineering Command (NAVFAC)

Engineering and Expeditionary Warfare Center (EXWC)

1000 23rd Avenue, Bldg. 1100

Port Hueneme, CA  91343

Attn: Ground Vehicles (ExPO EX22)

(POC to be provided).

The following shall be delivered with each trailer:

One (1) set of Tech Manuals (CD or web-based access and paper copy format).

OEM Quality control prints out to ship with each trailer.

OEM End of line report to be provided with manuals.

Onboard Tools (E.g., Breaker bar, Lug wrench handle and socket, keys for toolbox).

The CD-ROMs or web-based access will incorporate a navigation page with electronic links to the Operator’s Manual, Service Manual, Parts Manual, and Warranty information, as well as instructions on how to use the manual. The copy will include a table of contents with links to the specified documents or illustrations.

The OEMs shall allow for reproduction and distribution of Proprietary and/or Copyrighted Data, for government purposes and include a parts list (E.g., bolts, filters, gaskets) and a list of Collateral Items as applicable (E.g., Spare tire, spare rim, cargo straps, chains, crank handles, etc.). Parts and collateral lists shall be included as part of the Technical Manual.

Provide a Build Sheet or line setting ticket for each trailer, listing VIN, component part numbers, serial numbers, and UID tag numbers.

3.8 GSA FORM 1398

Unless otherwise specified, the Successful offeror shall affix one copy of GSA Form 1398, GSA Purchased Trailer, after final inspection is made. Copies of GSA Form 1398 are available from the Contracting Officer.

3.9 WORKMANSHIP

A. Trailers shall be free from defects that may impair their serviceability or detract from appearance.

B. All bodies, systems, equipment, and interfaces with the chassis shall be done in accordance with the OEM’s Body Builders Book. Whenever dissimilar metals are used, they shall be insulated against corrosive action.

C. All components shall be new. Defective components shall not be furnished. Parts, equipment, and assemblies, which have been repaired or modified to overcome deficiencies, shall not be furnished without the approval of the purchaser. The purchaser reserves the right to make the final determination of the suitability of all components and their arrangement on and in the trailer. Component parts and units shall be manufactured to definite standard dimensions with proper fits, clearances, and uniformity. All welding, to include Welded, bolted, and riveted construction utilized shall be in accordance with the highest standards of industry and must conform to Structural Welding Code for Steel and Stainless IAW American Society of Mechanical Engineers (ASME) and American Welding Society (AWS) Standards.

All welds shall be free of excess slag and spatter.

All welds shall be uniform, have adequate penetration, and be free of voids.

All rubber components shall be ozone and UV resistant.

D.	All components shall be suitable for the intended purpose and shall be free of any characteristics or defects in material and workmanship, which may affect the performance, function, durability, and serviceability of the finished trailer, or detract from its appearance.

E. 	All items shall be of good quality, in safe operating condition. All safety devices shall be fully functional; to include all manual and automatic emergency shut-off devices. All safety guards, warning labels, and safety related instruction labels shall be properly affixed at all times.

The General appearance of the trailer shall not show any evidence of poor workmanship.

The following shall be the reason for rejection:

1. Rough, sharp, or unfinished edges, burrs, seams, corners, and joints.

2. Non-uniform panels. Edges that are not radiused, beveled, etc.

3. Improper preparation of steel, frame, or other parts and equipment to prevent rust or corrosion

4. Paint runs, sags, streaks, wrinkles, pin holes, craters, grit, seeds, orange peel, “fisheyes”, etc., and any other imperfection, non-uniformity, improper application, or lack of complete coverage of primers, paints, or coatings.

5. Body panels or components that are uneven, unsealed, or contain cracks, dents or have voids.

6. Misalignment of body fasteners, glass, viewing panels, light housing, other items with large or uneven gaps, spacing etc. such as doors, body panels and hinged panels.

7. Improperly fabricated and routed wiring or harnesses, and electrical connections. Wiring should be adequately protected and mounted so as not to interfere with prime mover connection or operation of the trailer.

8. Improperly supported or secured hoses, wiring harnesses, mechanical controls etc., including interference with other components. Cables must flex between moving parts, with supporting clips securely mounted and cables secured in a permanent manner. Zip tied cables to hold ABS or other wiring in place are not acceptable.

9. Interference of chassis components, body parts, doors, etc.

10. Leaks of any gas, vacuum, or fluid lines (air conditioning, coolant, oil, etc.).

11. Noise, panel vibrations etc.

12. Inappropriate or incorrect use of hardware, fasteners, components, or methods of construction.

13. Incomplete or improper welding, riveting, or bolting, not IAW ASME and AWS standards.

14. Lack of uniformity and symmetry where applicable.

15. Loose, vibrating, abrading body parts, components, subassemblies, hoses, wiring harnesses or trim.

16. Improper body design or interface between prime mover and the chassis that could cause injury during normal use or maintenance, and which fail to provide access to perform routine or mandatory repairs or maintenance on trailer electrical and mechanical systems. In addition, the improper combination of options which by their combination and installation are inherently incompatible with regard to function or safety.

17. Incomplete or incorrect application of rust proofing, corrosion prevention coatings, or undercoat to all parts, equipment, and applicable surfaces.

18. Visual deformities and equipment malfunctions.

19. Unsealed appurtenances or other body components, gaskets, etc.

20. In addition, any deviation from specification requirements or any other item, whether or not stipulated herein, that affects form, fit, function, finish, durability, reliability, safety, performance, or appearance shall be cause for rejection.

21. The purchaser reserves the right to make the final determination of the suitability of all components and their arrangement on          and in the trailer.

3.10 VERIFICATION REQUIREMENTS

The contractor shall host a design review meeting onsite to include GSA and the government in attendance.

The contractor shall verify compliance to the requirements herein prior to first trailer production release. Verification may be in the form of contractor production line bill of material, drawings, instructions, test results, vendor documentation, certifications, or design analytical data.

The contractor shall not proceed with production before receiving agreement from the Government that verification of compliance is satisfactory. Acceptance of designs by the Government does not relieve the contractor from the contractor’s responsibility to meet all the requirements and specifications herein, whether reviewed by the Government or not.

It shall be the contractor’s responsibility to assure all production documentation results in the production of trailers that are in compliance with these requirements and specifications.

PAC	Post Award Conference			Receipt of Order + 5 Days

VDR	Verification Design Review 			Receipt of Order + 60 days

FDR 	Final Design Review				Receipt of Order +90 Days

VI		Trailer Inspection

BOM	Bill of Material

The type of verification to be submitted for acceptance is listed below:

ET	    Engineering Test – to be performed by the contractor and results accepted by the Government. Engineering tests are tests resulting in data output from engineering instruments used to take measurements of parameters such as temperatures, sound levels, etc. Verification may be in the form of an engineering report, test result sheet, or test summary with results documented.

COC 	    Confirmation of Compliance – Validation by analysis or production builds documentation to be performed by the contractor and submitted for acceptance by the Government. Verification may be in the form of analysis summaries, build documentation, purchase orders, or similar written documentation.

QAPVT   Quality Assurance Performance and Verification Testing – Testing to be performed and verified by the contractor to the Government. QAPVT tests are functional tests that do not involve instrumentation. The results are observable and recorded as pass/fail, i.e., trailer functions as intended; lights function as intended, etc. Systems or components that fail shall be annotated with the corrective action taken and retested until passed, however final acceptance and 1st article operational testing will be validated by the government prior to any acceptance of 1st article.

The contractor shall, within 45 days after receipt of order, submit detailed plans for the performance of all ET tests. These plans, at a minimum, shall include all instruments to be used, their calibration certification, and the procedures to be followed.

Government acceptances of verification and designs shall constitute design freezes and enable the contractor to proceed with production activities relative to the accepted designs. The design freeze shall be effective for the duration of the production of the entire quantity of trailers being procured.

3.11 OPTIONAL REQUIREMENTS

3.11.1 GOOSENECK NATO STANAG 4009

When this option is ordered, a gooseneck with a minimum swing clearance of 90.5” shall be provided. The intent and purpose of this option is to assure compatibility with tow tractors in accordance with NATO STANAG 4009

3.11.2 TRAINING/NEW EQUIPMENT TRAINING

(No Training requested)

4 TESTING, INSPECTION, ACCEPTANCE

4.1 TRAILER WEIGHT

The completed trailer shall be weighed on a certified scale, to determine the curb weight and axle weights.

4.2 PRE-SERVICE INSPECTION

The successful offeror shall perform a pre-service inspection of the production trailers based, as a minimum, on TMC RP 803- Pre-Service Trailer Inspection. All items listed in RP 803 shall be inspected and verified as satisfactory with all adjustments noted and verifications that all needed repairs have been made. The successful offeror’s own form may be used in lieu of the form in RP 803 if the successful offeror’s inspection form includes, as a minimum, all the inspection criteria in RP 803 applicable to the trailer. A copy of this completed inspection form shall be made available to any government inspector and shall be kept in the trailer as part of the documentation shipped with the trailer.

4.3 VOCATIONAL TEST

All equipment shall be functionally tested and verified to operate in accordance with the manufacturer’s operating instructions.

4.4 OPERATIONAL TESTING

- The First Production Vehicle shall be shipped to the Navy’s facility in Gulf Port, MS. The Navy will conduct operational testing to verify compliance with the operational and performance requirements referenced within this Purchase Description with all listed Navy Prime movers which are listed in section 3.0 here in.

The operational testing conducted by the Navy shall include:

4.4.1. Verification of form, fit and function with no obstructions or interface operability issues with the specified Prime movers in section 3.0.  1st article operational testing will have the OEM send the trailer to testing site and provided it passes testing it will be accepted. If testing is not passed it will be the responsibility of the contracted agent to send the trailer to facility to get items corrected at their cost and then sent back for operational testing until the trailer passes 1st article operational testing.  After the 1st trailer is accepted and passes operational testing, all follow on trailers will be required to meet the same standard as the 1st article trailer at time of delivery to the end user and if not the contracted agent is responsible for correcting to get to that standard at the contracting agents cost to include all shipping back and forth until shipped and accepted by end user. Note final acceptance will not be given until ATTLA certification is provided by contracted agent.

4.4.2. Verification to transport a combination of various sized ISO 668 shipping containers and/or containerized freight, breakbulk cargo. equipment attachments, palletized supplies, and/or miscellaneous construction supplies, configured in a single load, distributed evenly across the load space, as specified in section 3.1. Additionally testing will be conducted with all prime movers to test wet line kit interface, electrical and prime mover heights and swing clearance along with braking system hookups as the trailer may or may not be equipped as per requirements in this spec.

    - Verification of performance as referenced in section 3.1, 4.4.1. and 4.4.2. including:

4.4.3.1 Operation on cross country on highways, paved secondary, gravel, unimproved, or cobblestone roadways

Ability to traverse on, thru, over, and across multiple on-and-off road terrain, surfaces, roads, streets, parking aprons, runways, airfield shoulders and overruns, parking and open storage areas, for transfer transport, or on/offloading of cargo, equipment, or material .

4.4.3.2 Capability of operating in a wide variety of climatic conditions and environments, able to function in operating temperatures ranging from -25 degrees to 130 degrees Fahrenheit.

4.4.3.3 Ground Clearance: Should be sufficient to prevent damage to the axles or frame or any other trailer surface when trailer is in operation on unimproved terrain while loaded.

Capability of loading to dock height of 60 Inches (not for outsized equipment)

Verification of final weights to complete the ATTLA certification process and due before trailer delivery to end user

The contractor and their suppliers are welcome to attend and witness the operational testing activities conducted by the Navy

The estimated time to complete the operational testing is 5 business days or until testing is complete due to any delay’s dependent on weather and unforeseen circumstances i.e.. Prime mover breakdown

The Navy shall notify the Contracting Officer of the results of their operational testing.

Pending successful results, the Contracting Officer will provide notification of the Navy ‘s final acceptance of the First Production Vehicle. The Contracting Officer will provide notification to proceed with production of the balance of orders.

Pending unsuccessful results, the Contractor and their supplier(s) shall remedy any non-conformances prior to government’s acceptance. Retesting may be required.

- WARRANTY

The successful offeror shall provide a 60-month comprehensive (bumper to bumper) warranty.

The warranty shall include the trailer manufacturer’s commercial warranty, including the body and all other parts and components required herein, against parts failure or malfunction due to design, construction or installation errors, defective workmanship, and missing or incorrect parts, for a minimum period of 60 months from date of delivery of destination.

If the successful offeror receives from any supplier or subcontractor additional warranty on the whole or any component of the trailer, in form of time, including any prorated arrangements, or the successful offeror generally extends to its commercial customers greater or extended warranty coverage, the government shall receive corresponding warranty benefits.

The warranty coverage shall begin when the Government accepts the trailer from the successful offeror at destination.

The offeror shall provide access to warranty service via email, phone service, internet/web, or other method.

Initial response for on-site warranty repairs from the offeror or a certified dealer shall be within three business days.

Cost for parts and shipping for warranted repairs shall be the responsibility of the selected vendor. This shall include 3rd party Original Equipment Manufacturer (OEM) selected vendors for corrective action or warranty issues on the trailer if applicable.

If service or repair is needed, the OEM will provide an authorized location within the Continental United States (CONUS), to allow for staged equipment to be repaired and will supply parts and labor for all items under standard warranty agreement.

OEM replacement parts shall be shipped via the most expedient shipping method (E.g., FedEx, DHL, UPS, etc.). to CONUS Government locations for distribution.

The successful offeror shall furnish the Government with warranty certificates showing evidence that the warranty requirements above have been met.

A warranty registration card shall also be provided as applicable.

6.0 POST AWARD MEETINGS

The successful offeror shall participate in a post award review meeting (PAC/VDR/FDR). Attendees typically include, but are not limited to, GSA engineering personnel, Navy and the end user representative(s), and GSA contracting personnel. The meetings will be scheduled by the GSA as soon after the award of contract as practicable.

6.1 DELIVERY LOCATIONS

The trailers shall be delivered and accepted ONLY at the (FOB Destination) to the locations identified on the applicable GSA Motor Trailer Delivery Order.

All drivers shall possess a valid form of federal identification (license, TWIC card, Veterans ID card, etc.). If the drivers’ Commercial Driver License (CDL) states “NOT FOR FEDERAL IDENTIFICATION” the driver will not be allowed on Federal Property. The drivers shall show proof of insurance.

The contractor may stack trailers (wheels down only) for transport to destination; the contractor shall be responsible to offload and unstack if delivered stacked and present the trailers for final destination inspection with wheels on ground. The Navy shall inspect for damage in transit prior to acceptance at destination and shall submit claims in accordance with the offeror’s proposed damage in transit terms and conditions. IT IS THE DELIVERY CONTRACTORS RESPONSIBILITY TO OBTAIN BASE PASS ACCESS BY GOING THROUGH THE APPROPRIATE SECURITY CHANNELS AND MEETING THE REQUIREMENTS, ADDITIONALLY ALL SCHEDULED DELIVERIES NEED TO BE COMMUNICATED THE NAVY PROJECT MANAGER 2 WEEKS IN ADVANCE; FAILURE TO MEET ANY OF THESE MAY HINDER THE ACCEPTANCE OF THE ASSET AND COST THE CONTRACTOR MONEY THAT THE NAVY IS NOT RESPONSIBLE FOR.

The contractor shall contact the NAVFAC EXWC point of contact no later than 30 days prior to delivery to validate DELIVERY ADDRESS information:

MESG-1  4 EA. N57092

MESG-1

1498 13th St. Nolf IB Bldg. 165

Imperial Beach, CA 91932

POC: Dave Gray, N4

Comm: 619-437-9547

MESG-2  5 EA. N4365A

MESG-2

2465 Guadalcanal Rd

Virginia Beach VA 23459

POC: David Sanders

Office- 757-492-8787

NMCB  5 EA. N55752

Naval Construction Group ONE

Track 13 Road, Bldg. 806

Port Hueneme, CA 93043-4301

POC: Jerry Petry

Comm: 805-982-2014

NAVELSG  13 EA. N81464

Naval Expeditionary Logistics

Support Group

593 Mayfield Street Cheatham Annex

Williamsburg, VA 21385-5831

POC: Jonathan Pruett

Cell: 757-343 1174

NSWC  3 EA. W8Y0AA

NSW OPSTOCKS

106 Jacobson Drive

Poca, WV 25159

Mark For: NSW-OPSTOCK01

POC: Paul D Parsons

Comm:304-561-6885

Point of Contact is:

Naval Facilities Engineering Command (NAVFAC)

NAVFAC Engineering and Expeditionary Warfare Center (NFEXWC)

1000 23rd Ave, Bldg. 1100,

Port Hueneme, CA 93043

Attn: Ground Vehicles (ExPO EX22)

Adam Ward  (805)890-5750

(POC to be provided).

See Separate Shipping Instructions

6.2 NAVY HOURS OF OPERATIONS

Delivery hours are Monday through Thursday 0800 AM to 1430 PM local time.

No Friday or after-hours availability. Closed Federal holidays.

The contractor shall schedule all delivery activities with the identified delivery points of contact.