# STATEMENT OF WORK (SOW)

Aircraft Repair, Modification Oversight, and Integration Management Support
U.S. Navy VXS-1

# 1. BACKGROUND AND PURPOSE

Scientific Development Squadron ONE (VXS-1), a subordinate command of the Naval Research Laboratory (NRL), conducts airborne Science & Technology (S&T) experimentation and advanced technology development. To conduct this type of work, the squadron flies and maintains several aircraft at the Naval Air Station (NAS) Patuxent River, MD (PAX). The squadron is in the process of replacing the retiring NP-3C aircraft with up to three, Navy owned, Beechcraft King Air 350 Extended Range (KA350ER, MC-12S) aircraft for the airborne S&T mission.  The first of the three aircraft is nearing completion and will be delivered to VXS-1 in July 2026. The two aircraft addressed under this Statement of Work (SOW) are currently inducted at an FAA-certified Part 145 Repair Station where extensive work has already been completed to remove the prior US Army special mission modifications. The aircraft are currently in a non-airworthy status.

This SOW defines the requirements for a continuation of current efforts to return the aircraft to a baseline configuration and install S&T modifications specified. The requirement is to complete aircraft repairs, perform maintenance, install S&T modifications, maintain oversight, and provide integrated management of the process.  The Contractor will lead the continuing integration effort in support of VXS-1 under this SOW and will be responsible for the proper repair, modification, maintenance, management, ground and flight testing of two KA350ER research aircraft until restorations and modifications are complete and the aircraft delivered to VXS-1 located at NAS Patuxent River, Maryland.

# 2. SCOPE OF WORK

The Contractor shall provide comprehensive program management, technical oversight, integration support, aircraft maintenance, repair and modification activities on two KA350ER in support of VXS-1’s requirements. This includes continuation of the oversight of return to baseline modifications, specific Supplemental Type Certificate (STC) and other repairs; modification STC incorporation, configuration management, regulatory compliance monitoring and progress reporting. Two contract options, if exercised, will provide the same level of management, oversite and direction for the S&T modifications required to be performed on the baseline configured aircraft, as directed by VXS-1

# 3. IDENTIFIED TASKS FOR RESTORATION AND OPTIONS

The following baseline table represents the known work needed to restore each of the two aircraft to a baseline airworthy configuration.  The additional Option 1and Option 2 tables represent the desired work to incorporate specific modifications necessary to create configurations to support S&T projects in the KA350ERs and take advantage of STCs, engineering designs and other modification work done on the first aircraft.

| KA-350ER Tail 2 & 3 Baseline Completion |
| --- |
| Structural Restoration and Repair |  |
| System Restoration and Repair Includes: |  |
| Mission Interior |  |
| VHF Antenna Relocation |  |
| Oxygen System Replacement |  |
| O2 Mask |  |
| New A/C Battery |  |
| ADC Maintenance |  |
| Nose Landing Gear Overhaul |  |
| Nose Landing Gear brake lines |  |
| Landing Gear Extension Hoses |  |
| Battery Cap Check |  |
| Aft Wing Spar Inspection |  |
| Replacement Winglets L + R |  |
| Winglet Cover Lenses |  |
| Winglet Engineering |  |
| Starlink Antenna Install |  |
| Management Costs |  |
| Unscheduled Mx or Repair |  |
| Project Mod Prep |  |

Option 1 Task Table:

| Option 1 KA-350ER Tail 2 Mods | STC # |
| --- | --- |
| S&T Mods on Tail #2 |  |
| Extra Large Belly Radome | SA04581CH |
| Belly Rails and Hard Points | SA01902WI |
| 4x Additional Belly Hardpoints |  |
| Passthroughs- Belly | SA01902WI |
| Antenna relocations on Belly |  |
| Passthrough -Tail |  |
| Extended Nose | SA04581CH |
| Special Mission Alternator | SA 03698AT |
| Project DC Power Distro Systems |  |
| Inboard Wing Hard Points | SA04178NY |
| Underwing Wire fairings |  |
| Wing Pylons and BRU racks |  |
| Approval to carry STC Stores | SA0419NY |
| Analysis to carry the additional Stores | SA0419NY, plus analysis |
| 18” Drop Hatch in Aft Belly | SA2429CE |
| Full aft Baggage area platform |  |
| Install GFE ARC-210 & Antenna |  |
| ACFT Paint-flashes and logos |  |
| ACFT Paint-support |  |
| Engr Support and PM |  |
| Additional Antenna hardpoint-top (ea) |  |
| Starlink/Starshield Antenna |  |
| TACAN install & Antennas |  |
| Maintenance during Mods |  |
| Two Top Zenith ports |  |
| Additional Antenna: Project GPS |  |

Option 2 Task Table:

| Option 2 KA-350ER Tail 3 Mods | STC # |
| --- | --- |
| S&T Mods on Tail #3 |  |
| Belly Rails and Hard Points | SA01902WI |
| 4x Add'l Belly Hardpoints |  |
| Passthroughs- Belly | SA01902WI |
| Antenna relocations on Belly |  |
| Passthrough -Tail |  |
| Project DC Power Distro Systems |  |
| Inboard Wing Hard Points | SA04178NY |
| Underwing Wire fairings |  |
| 18” Drop Hatch in Aft Belly | SA2429CE |
| Full aft Baggage area platform |  |
| Install GFE ARC-210 & Antenna |  |
| ACFT Paint-flashes and logos |  |
| ACFT Paint-support |  |
| Engr Support and PM |  |
| Add'l Antenna hardpoint-top (ea) |  |
| Starlink/Starshield Antenna |  |
| TACAN install & Antennas |  |
| Maintenance during Mods |  |
| Add’l Antenna: GPS |  |

# 4. AIRCRAFT IDENTIFICATION AND CONFIGURATION

The following aircraft are covered under this SOW:

- King Air B300, S/N FL-450

- King Air B300, S/N FL-388

Aircraft shall be managed from the current as-configured baseline documented at the time of Government transition. Configuration control shall be established and maintained for the duration of this effort. Configuration and maintenance records and logbooks will be provided to VXS-1 with aircraft delivery.

# 5. ROLES AND RESPONSIBILITIES

The Contractor shall serve as the primary program integrator responsible for overseeing execution of maintenance and modification activities, coordinating with the Part 145 Repair Station, managing schedules and priorities, reviewing work packages, and ensuring compliance with approved technical data.

The Government (VXS-1) shall provide mission priorities, approve major technical decisions, and serve as final authority for acceptance of completed work.

# 6. MAINTENANCE AND MODIFICATION MANAGEMENT

The Contractor shall manage and oversee maintenance actions, baseline aircraft modifications, structural and avionics changes, and mission system integration as directed by VXS-1. The Contractor shall verify return-to-service approvals for all work is supported by compliant documentation from a Part 145 Repair Station and provided to NAVAIR Engineering Class Desk supporting PMA-207.

# 7. SUPPLEMENTAL TYPE CERTIFICATES (STCs)

The Contractor shall manage STC-related activities including coordination with STC holders, ensuring availability of approved data, and tracking STC applicability. STC ownership and approval authority shall remain with the appropriate approval holders.

# 8. PROGRAM MANAGEMENT AND REPORTING

The Contractor shall provide program execution planning, integrated schedules, technical status reports, risk and issue tracking, and recurring program reviews with the Government.

# 9. AIRWORTHINESS, QUALITY, AND COMPLIANCE

The Contractor shall support continued airworthiness by monitoring compliance with FAA regulations, NAVAIR airworthiness policy, OEM maintenance requirements, and applicable OPNAV instructions. The Contractor shall support Government audits and airworthiness reviews as required.

# 10. TECHNICAL DATA AND CONFIGURATION MANAGEMENT

The Contractor shall maintain configuration status accounting, manage technical records, control approved data, and ensure traceability between modifications and authorization bases. Technical data including drawings and analysis reports will be provided to the NAVAIR and VXS-1 for review, concurrence and reference in support to NAVAIR airworthiness policy.

# 11. GOVERNING DIRECTIVES AND REFERENCES

- NAVAIR Airworthiness and Safety Policy Manuals

- FAA 14 CFR Part 145

- Applicable STCs and Certification Bases

# 12. SECURITY REQUIREMENTS

All Contractor personnel shall comply with DoD and Navy security requirements, installation access controls, and controlled information handling procedures.

# 13. CHANGE MANAGEMENT

All changes to scope, responsibilities, or deliverables shall be accomplished through formal contract modification approved by the Government Contracting Officer.

# 14. ACCEPTANCE

Services and deliverables shall be accepted by the Government upon verification of completeness, accuracy, and compliance with this SOW.