Statement of Work (SOW)

BLOOD GAS INSTRUMENT PLACEMENT

- Description of the work to be performed:

Contractor will enter into a 5- year instrument placement /reagent rental agreement with Chinle laboratory. Contractor will provide 2 Radiometer ABL 90 Flex Plus analyzers. These analyzers will replace Radiometer blood gas analyzers, which will be traded in.

2 ABL 90 Flex Plus analyzers

Warranty service for 1 year and service coverage for 4 years

Supplies and reagents sufficient to operate analyzers on a continuous basis, under normal operating conditions

Installation and user training

Full Validation kit

Analyzers are covered by depot service

INSTRUMENT SPECIFICATIONS:

Required measured parameters-pH, pCO2, pO2, K+, Na+, Ca+, Cl-, Glu, Lactate,  SO2, tHb, and oximetry parameters such as O2, tHb, MetHb, FO2Hb, FCOHb

Closed system requiring only 2 or less solution packs and/or testing cassette

Built-in barcode reader

Capability for automatic QC, customizable QC and calibration schedule

Small sample volume requirement for testing <100 uL

Capability for direct specimen testing from syringe, test tubes, capillary tubes without adapter

Has programmable printer display options on analyzer

Small tabletop analyzer that can be portable, if necessary.

II. Location of work: Chinle Comprehensive Health Care Facility

III. Period of Performance: 04/01/2026-03/31/2027

IV. Deliverable schedule:

Contractor will ship/deliver instrumentation upon receipt of PO and schedule training on mutually acceptable date.

V. Performance standards:

Contractor will provide proper invoice for billing.

Instrumentation will be replaced/repaired at manufacturer’s discretion following their internal protocols.

VI. Contracting Officer Representative: Colleen Ben, Laboratory Supervisor; (928)-674-7101; email: colleen.ben@ihs.gov