NAVAIR LAKEHURST

STATEMENT OF WORK 
for the Production of

CH-53K Peculiar Support Equipment (PSE)

Prepared by:

Rotary Wing PSE Branch

SE Engineering Division

Naval Air Warfare Center – Aircraft Division

Joint Base McGuire-Dix-Lakehurst, New Jersey 08733

March 2026

DISTRIBUTION STATEMENT A: APPROVED FOR PUBLIC RELEASE; DISTRIBUTION IS UNLIMITED.

DESTRUCTION NOTICE. FOR UNCLASSIFIED, LIMITED DOCUMENTS, DESTROY BY ANY METHOD THAT WILL PREVENT DISCLOSURE OF CONTENCTS OR RECONSTRUCTION OF THIS DOCUMENT.

Table of Contents

1.0 Scope	3

2.0 Applicable Documents	3

2.1 Government Handbooks, Standards and Guidelines	3

2.2 Government Drawings	3

3.0 Requirements	3

3.1 General	3

3.2 Calibration	3

3.3 Proof Load	3

3.4 Non-Destructive Inspection (NDI)	3

3.5 Data Deliverables	4

3.5.1 Certifications	4

3.6 Quality Assurance	4

3.7 Government Inspection and Acceptance	4

3.8 Item Unique Identification (IUID) Marking of US Military Property	4

3.9 Packaging, Preservation, and Shipping	5

# 1.0 Scope

This Statement of Work (SOW) for the Firm Fixed Price production and delivery of the Peculiar Support Equipment (PSE) (listed in Attachment 1) for the CH-53K Aircraft Domestic and FMS platforms. The PSE includes various slings, maintenance stands, fixtures and carts. This PSE is all critical for the scheduled maintenance operations at the O-Level of maintenance for the CH-53K Aircraft. The PSE is needed for initial outfitting of CH-53K squadrons.

# 2.0 Applicable Documents

The following documents are incorporated by reference into the contract.

## 2.1 Government Handbooks, Standards and Guidelines

The following documents are to be used as guidance only, unless otherwise specified in the Statement of Work.

| Document Number | Document Title |
| --- | --- |
| MIL-STD-130 | IDENTIFICATION PLATE, SUPPORT EQUIPMENT |

## 2.2 Government Drawings

The contractor shall manufacture all end items in accordance with (IAW) the Government furnished technical data packages (TDPs) as referenced in Attachment 1. Any request for deviation will need approval

# 3.0 Requirements

## 3.1 General

The Contractor shall manufacture and deliver the PSE identified in Section 1.0, above, in accordance with (IAW) this SOW and the requirements of the contract.

## 3.2 Calibration

When applicable, the end items shall be delivered with a certificate of calibration traceable to the National Institute of Standards and Technology (NIST). The certificate verifies that the component has been tested against a NIST-certified gauge and falls within a required tolerance. The certificate shall be delivered with the end item as part of the Certifications deliverable specified in paragraph 3.5.2 (CDRL A005).

## 3.3 Proof Load

## The Sling Assemblies listed in 1.0 shall be proof load tested in the configuration shown in the drawing. The load shall be applied in a smooth manner and held for a period not less than 3 minutes. Any evidence of slipping, loosening of hardware, permanent structural deformation, or other signs of failure shall be cause for rejection. Operator performing the test shall verify successful proof load and provide CDRL: A006 specified in paragraph 3.5.2. Upon successful completion of the proof load Test, attach the proof load Tag to the sling. The proof load Tag shall be IAW NAVAIR Part Number called out in the drawing. After successful proofload test, a Foil Inspection Tag shall be affixed to the rear of the Proofload Tag. The Foil Inspection Tag shall in accordance with drawing 4SE00726-1, (NSN 9905-01-023-4226) with all information filled in as necessary.

## 3.4 Non-Destructive Inspection (NDI)

When NDI is required as called out on the applicable drawing, a certificate shall be delivered with each delivered end item IAW CDRL A007. This certificate shall document the inspection methods, equipment, and successful results and shall be delivered as part of the Certifications specified in paragraph 3.5.2.

## 3.5 Data Deliverables

## 3.5.1 Request for Information

Should the contractor require further information or clarification on the TDP provided to perform the fabrication process they may submit a Request for Information IAW CDRL A001 as soon as possible to avoid manufacturing delays. The Government shall respond within two (2) business days after the contractor’s submission.

### 3.5.2 Certifications

## The Contractor shall obtain and provide all certification documentation for each PSE end item delivered traceable to each serial number IAW the corresponding CDRL. This submittal shall include: raw material certifications (which must be DFARS Compliant) (CDRL: A002), plating/coating certifications (CDRL: A003), dimensional quality checklist (CDRL: A004), calibration certification (as necessary) (CDRL: A005), proof load certifications (as necessary) (CDRL: A006), non-destructive inspection (NDI) (as necessary) (CDRL: A007).Certification documentation shall be delivered in Adobe Portable Document Format (PDF) formats.

## 3.6 Quality Assurance

The Contractor shall have an AS9100D Certification at the time of award and throughout the performance of the contract This will ensure that manufactured configuration items are built to print, functional, and reliably made IAW controlling technical drawings, specifications and standards.  Each item manufactured as part of this contract shall have passed a Government final inspection prior to DD-250.

## 3.7 Government Inspection and Acceptance

## 3.7.1 Government Inspection and Acceptance – Domestic CLINs

Government Inspection and Acceptance of all production units shall be accomplished at DESTINATION by a Government representative. The PSE end items identified in Section 1.0 and all accompanying certification documentation shall be delivered no later than 200 calendar days after contract award.

## 3.7.2 Government Inspection and Acceptance – FMS CLINs

Government Inspection and Acceptance of all production units shall be accomplished at ORGIN by a Government representative. The PSE end items identified in Section 1.0 and all accompanying certification documentation shall be delivered no later than 200 calendar days after contract award.

## 3.8 Item Unique Identification (IUID) Marking of US Military Property

In accordance with DFARS Clause 252.211-7003 Item Identification and valuation, the Contractor is required to mark and register all end items and their components meeting the requirements called out in MIL-STD-130.  These include components for which the Governments unit acquisition cost is $5,000 or greater, or any component, subassembly or embedded part that is serially managed, mission essential or a controlled inventory item.

### 3.8.1 Construct and Method                                                                                                          The Contractor shall follow Construct and Method requirements as outlined in UID Construct #1 in MIL-STD-130N.

### 3.8.2 UID Location and Marking

The locations and marking methods selected should bear no impact on the performance of the part and minimal configuration change(s) to the part.  The UID (including 2D Matrix) should be incorporated onto the data plate in accordance with NAVAIR Drawing 3909AS9999 (IDENTIFICATION PLATE, SUPPORT EQUIPMENT).

3.8.3 Serial Numbers                                                                                                                                The contractor shall assign a serial number for each PSE end item. A unique four-digit alpha numeric serial number is required for each item beginning with 0001.

### 3.8.4 Permanency and Legibility

The UID marking and identification plates, tags, or labels when used on equipment, parts, assemblies, subassemblies, units, sets, or groups shall be permanent during the normal life expectancy of the item and be capable of withstanding the environmental test and cleaning procedures specified for the item.  Legibility shall be as required for ready readability as per MIL-STD-130.

### 3.8.5 UID Registration

The prime Contractor has the responsibility to furnish Item Unique Identification (IUID) data to the IUID registry. Data submission of IUID data and acquisition cost should be via Wide Area Work Flow (WAWF), IUID XML file, IUID flat file or web entry.  Additional information regarding data submission as well as the actual marking can be found at <http://www.acq.osd.mil/dpap/UID/>.

### 3.8.6 Records

The Contractor shall maintain an accurate, current list of UIDs for all manufactured items on this contract and supply to the Government upon request.  The list shall include P/N, Serial #, CAGE, UID Location, Construct used, how it was marked and the registration method used.

## 3.9 Packaging, Preservation, and Shipping

Packaging and preservation for shipping shall be IAW best commercial practices for long term preservation. Shipments shall be packaged and transported (FOB Destination) via safe, secure, reliable, and traceable means of transportation and provide protection from normal climatic/weather conditions and incidental mishandling during shipping to prevent any damage or loss during shipment.

All exterior shipping containers/packaging shall at a minimum be marked with the following information: Part Number; Nomenclature; Quantity; Serial number(s); Contract Number; National Stock Number; and Manufacturer’s Name, Address, and CAGE Code.

## 3.10 Cybersecurity Maturity Model Certification (CMMC) Program

Cybersecurity is a top priority for the Department of Defense (DoD). The defense industrial base (DIB) faces increasingly frequent and complex cyberattacks. To strengthen DIB cybersecurity and better safeguard DoD information, the DoD developed the Cybersecurity Maturity Model Certification (CMMC) Program to assess DoD cybersecurity requirements pertaining to Federal Contract Information (FCI) and Controlled Unclassified Information (CUI).

The items in this acquisition have been deemed to fall into CMMC Level 2- Self-Assessment. CMMC Level 2 requires implementation of the 110 security requirements set forth in NIST SP 800–171 Rev. 2 to protect CUI in contractor information systems. Level 2 assessments are conducted every three years. CMMC Level 2 self-assessment scores are entered by the OSA into SPRS