SECTION 01026

SUMMARY OF WORK

PART 1 GENERAL

1.1 WORK COVERED BY CONTRACT DOCUMENTS

1.1.1 Project Description

Furnish all labor, equipment, materials, and all incidental related services to perform Replacement of Repair of Floor Covering and Carpeting of Miscellaneous Buildings in accordance with the technical provisions prepared by the Directorate of Public Works, U.S. Army Garrison, at various Army Installations on the Island of Oahu, Hawaii. The scope of work covered by this contract does not cover floor covering and or carpet work, which might arise as part of the contract to repair or renovate a building or group of buildings. The work covered by this contract does not include projects awarded by The U.S. Army Engineering District, Honolulu, or the Medical Facilities under Tripler (TAMC), to be designated during the contract period.

1.2 ESTIMATED QUANTITIES

The estimated quantities of work, indicated herein, are for information of the prospective Contractors. The actual quantities ordered may be more or less than the estimated quantities.

LOCATIONS

The location of work sites shall be as follows:

Fort Shafter Areas

Fort Shafter, Fort DeRussy, Tripler Army Medical Center, (Except for the main Medical Bldg.) Aliamanu Military Reservation

Schofield Barracks Area

Schofield Barracks (includes East Range & Leilehua Golf Course, Wheeler Army Airfield, Helemano Military Reservation)

The exact locations will be shown by the Contracting Officer.

1.3 ITEMS OF WORK

1.3.1 Base Year Estimated quantities, item numbers, units of measure, unit prices and amounts are as follows:

Repair (K) Work: Repair/Replace Existing.

0001: (NON-ASBESTOS) VINYL AND VINLY COMPOSITION TILE removal and disposal. Excludes stairway Treads and Risers. Remove and dispose of existing single-layer floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 12,000 SF

0002: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on stairway Treads and Risers. Remove and dispose of existing single-layer floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

0003: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on floors for multi-layered floor covering. Remove and dispose of existing multi-layered floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

0004: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on stairway Treads and Risers for multi-layered floor covering. Remove and dispose of existing multi-layered floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

0005: VINYL TILE installation on floors. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 3,000 SF

0006: VINYL TILE installation on stairway Treads & Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 100 SF

0007: VINYL COMPOSITION TILE installation on floors. Excludes stairway Treads and Risers.  Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 30,000 SF

0008: VINYL COMPOSITION TILE installation on stairway Treads & Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 100 SF

0009: POLISH VINYL COMPOSITION TILE FLOORING. Polish pre-waxed/no wax flooring in various buildings following installation. Estimated at: 30,000 SF

0010: SAMPLING & TESTING OF ASBESTOS (NON-FRIABLE) IN VINYL, VINYL COMPOSITION TILE, AND ADHESIVE. Collection and laboratory analysis of samples.

CLARIFICATION: One unit (EA) consists of one sample retrieved at 	the site w/ one laboratory test. Samples may or may not be in the	same building. For example: 10 samples may be required in 10	different buildings in the Fort Shafter area or all 10 samples	may be needed in one building.	Estimated at: 5 EA

NOTE: A requirement for submittal of accreditation and medical	surveillance for each worker has been included under Section	028214.

0011: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal from floors. Excludes stairway Treads and Risers. Remove and dispose of existing single layer floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 5,000 SF

0012: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal from stairway Treads and Risers. Remove and dispose of existing single layer floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

0013: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal of multi-layered tiles from floors. Excludes stairway Treads and Risers. Remove and dispose of existing multi-layered floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

0014: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal of multi-layered tiles from stairway Treads and Risers. Remove and dispose of existing multi-layered floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

0015: Resilient Sheet Vinyl Floor removal and dispose from floors. Excludes stairway Treads and Risers. Remove and dispose of existing floor covering in various buildings down to underlayment of bare concrete. Estimated at: 1,000 SF

0016: Resilient Sheet Vinyl Floor removal and disposal from Stairway Treads and Risers. Remove and dispose of existing floor covering in various buildings down to underlayment of bare concrete. Estimated at: 50 SF

0017: Resilient Sheet Vinyl (with backing) Floor installation. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 1,000 SF

0018: Resilient Sheet Vinyl (with backing) Floor installation on Stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 50 SF

0019: Resilient Sheet Vinyl (without backing) Floor installation. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 100 SF

0020: Resilient Sheet Vinyl (without backing) Floor installation on Stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 20 SF

0021: Rubber Floor Tile installation on Floor. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 2,000 SF

0022: Polish Resilient Sheet Vinyl Flooring. Polish pre-waxed/no wax flooring in various buildings following installation. Estimated at: 40 SF

0023: Ceramic Tile Floor and Mortar Bed Removal and Disposal. Remove Ceramic Tile and Mortar Bed in various buildings down to underlayment or bare concrete. Estimated at: 1,000 SF

0024: Ceramic Tile Floor Installation.	(Mosaic 1”X1”, 1”X2”, or 2”X2”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 250 SF

0025: Ceramic Tile Floor Installation.	(Mosaic 6”X6”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 250 SF

0026: Ceramic Tile Floor Installation.	(Mosaic 8”X8”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 100 SF

0027: Ceramic Tile Floor Installation.	(Mosaic 12”X12”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 4,000 SF

0028: Quarry Tile Floor and Mortar Bed Removal and Disposal. Remove Quarry Tile and Mortar Bed in various buildings down to underlayment or bare concrete. Estimated at: 500 SF

0029: Quarry Tile Floor and Mortar Bed Installation. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Quarry Tile Floor and Mortar Bed in various buildings. Estimated at: 500 SF

0030: Carpet Removal and Disposal, Excludes Stairway Treads and Risers. Remove carpet in various buildings down to underlayment or bare concrete. Estimated at: 8,000 SY

0031: Carpet Removal and Disposal from Stairway Treads and Risers. Remove carpet in various buildings down to underlayment or bare concrete. Estimated at: 100 SY

0032: Commercial “Severe” Grade Carpet Installation Excludes Stairway Treads and Risers. Clean Floor surface of all foreign materials prior to installation of new carpet. Includes Urethane carpet cushion installation. Installation process in various buildings includes stretching in over cushion (tack down). Estimated at: 150 SY

0033: Commercial “Severe” Grade Carpet Installation on Stairway Treads and Risers. Clean Floor surface on all foreign materials prior to installation of new carpet. Includes Urethane carpet cushion installation. Installation process in various buildings includes stretching in over cushion (tack down). Estimated at: 50 SY

0034: Exterior “Heavy” Grade Carpet Installation Excludes Stairway Treads and Risers. Clean floor surface of all foreign materials prior to installation of new carpet. Installation is direct glue down between carpet and floor (no cushion). Estimated at: 50 SY

0035: Remove and Dispose of Existing Particle Board or Plywood Floor Underlayment in various buildings. Estimated at: 500 SF

0036: Provide and Install Plywood Floor Underlayment in various buildings. Estimated at: 500 SF

0037: Remove and Dispose of Existing Membrane Waterproofing in various buildings. Estimated at: 40 SF

0038: Install New Waterproofing Membrane in various buildings. Estimated at: 2,000 SF

0039: Remove and Dispose of Existing Wood Base in various buildings. Estimated at: 1,500 FT

0040: Remove and Dispose of Existing Resilient Base in various buildings. Estimated at: 10,000 FT

0041: Reinstall Existing Wood Base in various buildings. Estimated at: 50 FT

0042: Install New Wood Base in various buildings. Estimated at 50 FT

0043: Install New 4-inch Resilient Base in various buildings. Estimated at: 15,000 FT

0044: Install New 6-inch Resilient Base in various buildings. Estimated at: 1,500 FT

0045: Install New 10—inch Resilient Stringer Base in various buildings. Estimated at: 50 FT

0046: Trim bottom of Doors in various buildings. Estimated at: 20 EA

0047: Paint Existing 6” Concrete Wall Base in various buildings. Estimated at 50 SF

0048: Install New Vinyl Edge Strips in various buildings. Estimated at: 1,000 FT

0049: Install New Aluminum Binder Edge Strips in various buildings. Estimated at: 200 FT

0050: Install New Aluminum / Vinyl Stair Nosing in various buildings. Estimated at: 200 FT

0051: Install New Rubber Molded Stair Tread and Nosing in various buildings. Estimated at: 200 FT

0052: Install New Rubber Molded Cover Stair Nosing in various buildings. Estimated at: 200 FT

0053: Install Concrete Sealant in various buildings. Estimated at: 100 SF

0054: Prepare Concrete Floor, Moderate (includes grinding, sanding & scraping) in various buildings. Estimated at: 30,000 SF

0055: Prepare Concrete Floor, Heavy (includes “shot blasting” or similar methods). Estimated at: 500 SF

0056: Installation of flooring outside of normal working hours as requested by the government.  Job to be determined on each task order. CLIN to be used in conjunction CLINS for removal and installation of flooring during normal working hours.  For example CLINS 0001, 0007, and 0056 would be included in a project to be performed after hours or on weekends.  Estimated at: 1,000 SF

0057: Remove and Re-install Bathroom Fixtures and Partitions and Miscellaneous Labor. Estimated at: 20 EA

0058: “Function Modular” or approved equal, 1/10 gauge Modular Tufted Textured Patterned Loop, solution dyed nylon, 24”X24” Carpet Squares with Severe Traffic CRI rating, vinyl, modular backing system. Flammability conforming to ASTM 648 Class 1 (Glue down), Smoke Density conforming to ASTM 662 less than 450. Static Propensity conforming to AATCC-134 under 3.5 KV. IAQ Green label certified. Clean floor surface of all foreign materials prior to installation. Installation is direct glue down between carpet and floor. Estimated at: 2,000 SY

0059: Provide Carpet with Pad attached at various buildings. Estimated at: 50 SY

0060: Install Carpet with Furniture in place. Lift Jack system to lift modular furniture or temporarily relocate furniture and replace after installation of carpet. Estimated at: 15,000 SF

0061. Provide moving furniture prior to and after installation of flooring. Includes general, desks, partitions, storage boxes, tables, file cabinets (full), chairs, and bookshelves. Quantity to be plan space the furniture covers.  Estimated at: 12,500 SF

0062: Plush Commercial "Heavy" Grade Carpet Installation Includes Stairway Treads and Risers or flat office areas. Clean Floor surface of all foreign materials prior to Installation of new Carpet. Installation is direct glue down between carpet and floor (no cushion). Estimated at: 50 SY

1.3.2 First Option Year Estimated quantities, item numbers, units of measure, unit prices and amounts are as follows:

Repair (K) Work: Repair/Replace Existing.

1001: (NON-ASBESTOS) VINYL AND VINLY COMPOSITION TILE removal and disposal. Excludes stairway Treads and Risers. Remove and dispose of existing single-layer floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 12,000 SF

1002: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on stairway Treads and Risers. Remove and dispose of existing single-layer floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

1003: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on floors for multi-layered floor covering. Remove and dispose of existing multi-layered floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

1004: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on stairway Treads and Risers for multi-layered floor covering. Remove and dispose of existing multi-layered floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

1005: VINYL TILE installation on floors. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 3,000 SF

1006: VINYL TILE installation on stairway Treads & Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 100 SF

1007: VINYL COMPOSITION TILE installation on floors. Excludes stairway Treads and Risers.  Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 30,000 SF

1008: VINYL COMPOSITION TILE installation on stairway Treads & Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 100 SF

1009: POLISH VINYL COMPOSITION TILE FLOORING. Polish pre-waxed/no wax flooring in various buildings following installation. Estimated at: 30,000 SF

1010: SAMPLING & TESTING OF ASBESTOS (NON-FRIABLE) IN VINYL, VINYL COMPOSITION TILE, AND ADHESIVE. Collection and laboratory analysis of samples.

CLARIFICATION: One unit (EA) consists of one sample retrieved at 	the site w/ one laboratory test. Samples may or may not be in the	same building. For example: 10 samples may be required in 10	different buildings in the Fort Shafter area or all 10 samples	may be needed in one building.	Estimated at: 5 EA

NOTE: A requirement for submittal of accreditation and medical	surveillance for each worker has been included under Section	028214.

1011: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal from floors. Excludes stairway Treads and Risers. Remove and dispose of existing single layer floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 5,000 SF

1012: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal from stairway Treads and Risers. Remove and dispose of existing single layer floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

1013: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal of multi-layered tiles from floors. Excludes stairway Treads and Risers. Remove and dispose of existing multi-layered floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

1014: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal of multi-layered tiles from stairway Treads and Risers. Remove and dispose of existing multi-layered floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

1015: Resilient Sheet Vinyl Floor removal and dispose from floors. Excludes stairway Treads and Risers. Remove and dispose of existing floor covering in various buildings down to underlayment of bare concrete. Estimated at: 1,000 SF

1016: Resilient Sheet Vinyl Floor removal and disposal from Stairway Treads and Risers. Remove and dispose of existing floor covering in various buildings down to underlayment of bare concrete. Estimated at: 50 SF

1017: Resilient Sheet Vinyl (with backing) Floor installation. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 1,000 SF

1018: Resilient Sheet Vinyl (with backing) Floor installation on Stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 50 SF

1019: Resilient Sheet Vinyl (without backing) Floor installation. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 100 SF

1020: Resilient Sheet Vinyl (without backing) Floor installation on Stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 20 SF

1021: Rubber Floor Tile installation on Floor. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 2,000 SF

1022: Polish Resilient Sheet Vinyl Flooring. Polish pre-waxed/no wax flooring in various buildings following installation. Estimated at: 40 SF

1023: Ceramic Tile Floor and Mortar Bed Removal and Disposal. Remove Ceramic Tile and Mortar Bed in various buildings down to underlayment or bare concrete. Estimated at: 1,000 SF

1024: Ceramic Tile Floor Installation.	(Mosaic 1”X1”, 1”X2”, or 2”X2”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 250 SF

1025: Ceramic Tile Floor Installation.	(Mosaic 6”X6”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 250 SF

1026: Ceramic Tile Floor Installation.	(Mosaic 8”X8”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 100 SF

1027: Ceramic Tile Floor Installation.	(Mosaic 12”X12”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 4,000 SF

1028: Quarry Tile Floor and Mortar Bed Removal and Disposal. Remove Quarry Tile and Mortar Bed in various buildings down to underlayment or bare concrete. Estimated at: 500 SF

1029: Quarry Tile Floor and Mortar Bed Installation. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Quarry Tile Floor and Mortar Bed in various buildings. Estimated at: 500 SF

1030: Carpet Removal and Disposal, Excludes Stairway Treads and Risers. Remove carpet in various buildings down to underlayment or bare concrete. Estimated at: 8,000 SY

1031: Carpet Removal and Disposal from Stairway Treads and Risers. Remove carpet in various buildings down to underlayment or bare concrete. Estimated at: 100 SY

1032: Commercial “Severe” Grade Carpet Installation Excludes Stairway Treads and Risers. Clean Floor surface of all foreign materials prior to installation of new carpet. Includes Urethane carpet cushion installation. Installation process in various buildings includes stretching in over cushion (tack down). Estimated at: 150 SY

1033: Commercial “Severe” Grade Carpet Installation on Stairway Treads and Risers. Clean Floor surface on all foreign materials prior to installation of new carpet. Includes Urethane carpet cushion installation. Installation process in various buildings includes stretching in over cushion (tack down). Estimated at: 50 SY

1034: Exterior “Heavy” Grade Carpet Installation Excludes Stairway Treads and Risers. Clean floor surface of all foreign materials prior to installation of new carpet. Installation is direct glue down between carpet and floor (no cushion). Estimated at: 50 SY

1035: Remove and Dispose of Existing Particle Board or Plywood Floor Underlayment in various buildings. Estimated at: 500 SF

1036: Provide and Install Plywood Floor Underlayment in various buildings. Estimated at: 500 SF

1037: Remove and Dispose of Existing Membrane Waterproofing in various buildings. Estimated at: 40 SF

1038: Install New Waterproofing Membrane in various buildings. Estimated at: 2,000 SF

1039: Remove and Dispose of Existing Wood Base in various buildings. Estimated at: 1,500 FT

1040: Remove and Dispose of Existing Resilient Base in various buildings. Estimated at: 10,000 FT

1041: Reinstall Existing Wood Base in various buildings. Estimated at: 50 FT

1042: Install New Wood Base in various buildings. Estimated at 50 FT

1043: Install New 4-inch Resilient Base in various buildings. Estimated at: 15,000 FT

1044: Install New 6-inch Resilient Base in various buildings. Estimated at: 1,500 FT

1045: Install New 10—inch Resilient Stringer Base in various buildings. Estimated at: 50 FT

1046: Trim bottom of Doors in various buildings. Estimated at: 20 EA

1047: Paint Existing 6” Concrete Wall Base in various buildings. Estimated at 50 SF

1048: Install New Vinyl Edge Strips in various buildings. Estimated at: 1,000 FT

1049: Install New Aluminum Binder Edge Strips in various buildings. Estimated at: 200 FT

1050: Install New Aluminum / Vinyl Stair Nosing in various buildings. Estimated at: 200 FT

1051: Install New Rubber Molded Stair Tread and Nosing in various buildings. Estimated at: 200 FT

1052: Install New Rubber Molded Cover Stair Nosing in various buildings. Estimated at: 200 FT

1053: Install Concrete Sealant in various buildings. Estimated at: 100 SF

1054: Prepare Concrete Floor, Moderate (includes grinding, sanding & scraping) in various buildings. Estimated at: 30,000 SF

1055: Prepare Concrete Floor, Heavy (includes “shot blasting” or similar methods). Estimated at: 500 SF

1056: Installation of flooring outside of normal working hours as requested by the government.  Job to be determined on each task order. CLIN to be used in conjunction CLINS for removal and installation of flooring during normal working hours.  For example CLINS 0001, 0007, and 0056 would be included in a project to be performed after hours or on weekends.  Estimated at: 1,000 SF

1057: Remove and Re-install Bathroom Fixtures and Partitions and Miscellaneous Labor. Estimated at: 20 EA

1058: “Function Modular” or approved equal, 1/10 gauge Modular Tufted Textured Patterned Loop, solution dyed nylon, 24”X24” Carpet Squares with Severe Traffic CRI rating, vinyl, modular backing system. Flammability conforming to ASTM 648 Class 1 (Glue down), Smoke Density conforming to ASTM 662 less than 450. Static Propensity conforming to AATCC-134 under 3.5 KV. IAQ Green label certified. Clean floor surface of all foreign materials prior to installation. Installation is direct glue down between carpet and floor. Estimated at: 2,000 SY

1059: Provide Carpet with Pad attached at various buildings. Estimated at: 50 SY

1060: Install Carpet with Furniture in place. Lift Jack system to lift modular furniture or temporarily relocate furniture and replace after installation of carpet. Estimated at: 15,000 SF

1061. Provide moving furniture prior to and after installation of flooring. Includes general, desks, partitions, storage boxes, tables, file cabinets (full), chairs, and bookshelves. Quantity to be plan space the furniture covers.  Estimated at: 12,500 SF

1062: Plush Commercial "Heavy" Grade Carpet Installation Includes Stairway Treads and Risers or flat office areas. Clean Floor surface of all foreign materials prior to Installation of new Carpet. Installation is direct glue down between carpet and floor (no cushion). Estimated at: 50 SY

1.3.3 Second Option Year Estimated quantities, item numbers, units of measure, unit prices and amounts are as follows:

Repair (K) Work: Repair/Replace Existing.

2001: (NON-ASBESTOS) VINYL AND VINLY COMPOSITION TILE removal and disposal. Excludes stairway Treads and Risers. Remove and dispose of existing single-layer floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 12,000 SF

2002: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on stairway Treads and Risers. Remove and dispose of existing single-layer floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

2003: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on floors for multi-layered floor covering. Remove and dispose of existing multi-layered floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

2004: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on stairway Treads and Risers for multi-layered floor covering. Remove and dispose of existing multi-layered floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

2005: VINYL TILE installation on floors. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 3,000 SF

2006: VINYL TILE installation on stairway Treads & Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 100 SF

2007: VINYL COMPOSITION TILE installation on floors. Excludes stairway Treads and Risers.  Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 30,000 SF

2008: VINYL COMPOSITION TILE installation on stairway Treads & Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 100 SF

2009: POLISH VINYL COMPOSITION TILE FLOORING. Polish pre-waxed/no wax flooring in various buildings following installation. Estimated at: 30,000 SF

2010: SAMPLING & TESTING OF ASBESTOS (NON-FRIABLE) IN VINYL, VINYL COMPOSITION TILE, AND ADHESIVE. Collection and laboratory analysis of samples.

CLARIFICATION: One unit (EA) consists of one sample retrieved at 	the site w/ one laboratory test. Samples may or may not be in the	same building. For example: 10 samples may be required in 10	different buildings in the Fort Shafter area or all 10 samples	may be needed in one building.	Estimated at: 5 EA

NOTE: A requirement for submittal of accreditation and medical	surveillance for each worker has been included under Section	028214.

2011: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal from floors. Excludes stairway Treads and Risers. Remove and dispose of existing single layer floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 5,000 SF

2012: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal from stairway Treads and Risers. Remove and dispose of existing single layer floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

2013: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal of multi-layered tiles from floors. Excludes stairway Treads and Risers. Remove and dispose of existing multi-layered floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

2014: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal of multi-layered tiles from stairway Treads and Risers. Remove and dispose of existing multi-layered floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

2015: Resilient Sheet Vinyl Floor removal and dispose from floors. Excludes stairway Treads and Risers. Remove and dispose of existing floor covering in various buildings down to underlayment of bare concrete. Estimated at: 1,000 SF

2016: Resilient Sheet Vinyl Floor removal and disposal from Stairway Treads and Risers. Remove and dispose of existing floor covering in various buildings down to underlayment of bare concrete. Estimated at: 50 SF

2017: Resilient Sheet Vinyl (with backing) Floor installation. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 1,000 SF

2018: Resilient Sheet Vinyl (with backing) Floor installation on Stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 50 SF

2019: Resilient Sheet Vinyl (without backing) Floor installation. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 100 SF

2020: Resilient Sheet Vinyl (without backing) Floor installation on Stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 20 SF

2021: Rubber Floor Tile installation on Floor. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 2,000 SF

2022: Polish Resilient Sheet Vinyl Flooring. Polish pre-waxed/no wax flooring in various buildings following installation. Estimated at: 40 SF

2023: Ceramic Tile Floor and Mortar Bed Removal and Disposal. Remove Ceramic Tile and Mortar Bed in various buildings down to underlayment or bare concrete. Estimated at: 1,000 SF

2024: Ceramic Tile Floor Installation.	(Mosaic 1”X1”, 1”X2”, or 2”X2”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 250 SF

2025: Ceramic Tile Floor Installation.	(Mosaic 6”X6”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 250 SF

2026: Ceramic Tile Floor Installation.	(Mosaic 8”X8”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 100 SF

2027: Ceramic Tile Floor Installation.	(Mosaic 12”X12”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 4,000 SF

2028: Quarry Tile Floor and Mortar Bed Removal and Disposal. Remove Quarry Tile and Mortar Bed in various buildings down to underlayment or bare concrete. Estimated at: 500 SF

2029: Quarry Tile Floor and Mortar Bed Installation. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Quarry Tile Floor and Mortar Bed in various buildings. Estimated at: 500 SF

2030: Carpet Removal and Disposal, Excludes Stairway Treads and Risers. Remove carpet in various buildings down to underlayment or bare concrete. Estimated at: 8,000 SY

2031: Carpet Removal and Disposal from Stairway Treads and Risers. Remove carpet in various buildings down to underlayment or bare concrete. Estimated at: 100 SY

2032: Commercial “Severe” Grade Carpet Installation Excludes Stairway Treads and Risers. Clean Floor surface of all foreign materials prior to installation of new carpet. Includes Urethane carpet cushion installation. Installation process in various buildings includes stretching in over cushion (tack down). Estimated at: 150 SY

2033: Commercial “Severe” Grade Carpet Installation on Stairway Treads and Risers. Clean Floor surface on all foreign materials prior to installation of new carpet. Includes Urethane carpet cushion installation. Installation process in various buildings includes stretching in over cushion (tack down). Estimated at: 50 SY

2034: Exterior “Heavy” Grade Carpet Installation Excludes Stairway Treads and Risers. Clean floor surface of all foreign materials prior to installation of new carpet. Installation is direct glue down between carpet and floor (no cushion). Estimated at: 50 SY

2035: Remove and Dispose of Existing Particle Board or Plywood Floor Underlayment in various buildings. Estimated at: 500 SF

2036: Provide and Install Plywood Floor Underlayment in various buildings. Estimated at: 500 SF

2037: Remove and Dispose of Existing Membrane Waterproofing in various buildings. Estimated at: 40 SF

2038: Install New Waterproofing Membrane in various buildings. Estimated at: 2,000 SF

2039: Remove and Dispose of Existing Wood Base in various buildings. Estimated at: 1,500 FT

2040: Remove and Dispose of Existing Resilient Base in various buildings. Estimated at: 10,000 FT

2041: Reinstall Existing Wood Base in various buildings. Estimated at: 50 FT

2042: Install New Wood Base in various buildings. Estimated at 50 FT

2043: Install New 4-inch Resilient Base in various buildings. Estimated at: 15,000 FT

2044: Install New 6-inch Resilient Base in various buildings. Estimated at: 1,500 FT

2045: Install New 10—inch Resilient Stringer Base in various buildings. Estimated at: 50 FT

2046: Trim bottom of Doors in various buildings. Estimated at: 20 EA

2047: Paint Existing 6” Concrete Wall Base in various buildings. Estimated at 50 SF

2048: Install New Vinyl Edge Strips in various buildings. Estimated at: 1,000 FT

2049: Install New Aluminum Binder Edge Strips in various buildings. Estimated at: 200 FT

2050: Install New Aluminum / Vinyl Stair Nosing in various buildings. Estimated at: 200 FT

2051: Install New Rubber Molded Stair Tread and Nosing in various buildings. Estimated at: 200 FT

2052: Install New Rubber Molded Cover Stair Nosing in various buildings. Estimated at: 200 FT

2053: Install Concrete Sealant in various buildings. Estimated at: 100 SF

2054: Prepare Concrete Floor, Moderate (includes grinding, sanding & scraping) in various buildings. Estimated at: 30,000 SF

2055: Prepare Concrete Floor, Heavy (includes “shot blasting” or similar methods). Estimated at: 500 SF

2056: Installation of flooring outside of normal working hours as requested by the government.  Job to be determined on each task order. CLIN to be used in conjunction CLINS for removal and installation of flooring during normal working hours.  For example CLINS 0001, 0007, and 0056 would be included in a project to be performed after hours or on weekends.  Estimated at: 1,000 SF

2057: Remove and Re-install Bathroom Fixtures and Partitions and Miscellaneous Labor. Estimated at: 20 EA

2058: “Function Modular” or approved equal, 1/10 gauge Modular Tufted Textured Patterned Loop, solution dyed nylon, 24”X24” Carpet Squares with Severe Traffic CRI rating, vinyl, modular backing system. Flammability conforming to ASTM 648 Class 1 (Glue down), Smoke Density conforming to ASTM 662 less than 450. Static Propensity conforming to AATCC-134 under 3.5 KV. IAQ Green label certified. Clean floor surface of all foreign materials prior to installation. Installation is direct glue down between carpet and floor. Estimated at: 2,000 SY

2059: Provide Carpet with Pad attached at various buildings. Estimated at: 50 SY

2060: Install Carpet with Furniture in place. Lift Jack system to lift modular furniture or temporarily relocate furniture and replace after installation of carpet. Estimated at: 15,000 SF

2061. Provide moving furniture prior to and after installation of flooring. Includes general, desks, partitions, storage boxes, tables, file cabinets (full), chairs, and bookshelves. Quantity to be plan space the furniture covers.  Estimated at: 12,500 SF

2062: Plush Commercial "Heavy" Grade Carpet Installation Includes Stairway Treads and Risers or flat office areas. Clean Floor surface of all foreign materials prior to Installation of new Carpet. Installation is direct glue down between carpet and floor (no cushion). Estimated at: 50 SY

1.3.4 Third Option Year Estimated quantities, item numbers, units of measure, unit prices and amounts are as follows:

Repair (K) Work: Repair/Replace Existing.

3001: (NON-ASBESTOS) VINYL AND VINLY COMPOSITION TILE removal and disposal. Excludes stairway Treads and Risers. Remove and dispose of existing single-layer floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 12,000 SF

3002: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on stairway Treads and Risers. Remove and dispose of existing single-layer floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

3003: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on floors for multi-layered floor covering. Remove and dispose of existing multi-layered floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

3004: (NON-ASBESTOS) VINYL AND VINYL COMPOSITION TILE removal and disposal on stairway Treads and Risers for multi-layered floor covering. Remove and dispose of existing multi-layered floor tile covering in various down to wood underlayment or bare concrete. Estimated at: 100 SF

3005: VINYL TILE installation on floors. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 3,000 SF

3006: VINYL TILE installation on stairway Treads & Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 100 SF

3007: VINYL COMPOSITION TILE installation on floors. Excludes stairway Treads and Risers.  Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 30,000 SF

3008: VINYL COMPOSITION TILE installation on stairway Treads & Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to new tile installation. Estimated at: 100 SF

3009: POLISH VINYL COMPOSITION TILE FLOORING. Polish pre-waxed/no wax flooring in various buildings following installation. Estimated at: 30,000 SF

3010: SAMPLING & TESTING OF ASBESTOS (NON-FRIABLE) IN VINYL, VINYL COMPOSITION TILE, AND ADHESIVE. Collection and laboratory analysis of samples.

CLARIFICATION: One unit (EA) consists of one sample retrieved at 	the site w/ one laboratory test. Samples may or may not be in the	same building. For example: 10 samples may be required in 10	different buildings in the Fort Shafter area or all 10 samples	may be needed in one building.	Estimated at: 5 EA

NOTE: A requirement for submittal of accreditation and medical	surveillance for each worker has been included under Section	028214.

3011: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal from floors. Excludes stairway Treads and Risers. Remove and dispose of existing single layer floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 5,000 SF

3012: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal from stairway Treads and Risers. Remove and dispose of existing single layer floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

3013: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal of multi-layered tiles from floors. Excludes stairway Treads and Risers. Remove and dispose of existing multi-layered floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

3014: Asbestos Containing Material (ACM).	Vinyl and Vinyl Composition tile removal and disposal of multi-layered tiles from stairway Treads and Risers. Remove and dispose of existing multi-layered floor tile covering in various buildings down to wood underlayment or bare concrete. Estimated at: 100 SF

3015: Resilient Sheet Vinyl Floor removal and dispose from floors. Excludes stairway Treads and Risers. Remove and dispose of existing floor covering in various buildings down to underlayment of bare concrete. Estimated at: 1,000 SF

3016: Resilient Sheet Vinyl Floor removal and disposal from Stairway Treads and Risers. Remove and dispose of existing floor covering in various buildings down to underlayment of bare concrete. Estimated at: 50 SF

3017: Resilient Sheet Vinyl (with backing) Floor installation. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 1,000 SF

3018: Resilient Sheet Vinyl (with backing) Floor installation on Stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 50 SF

3019: Resilient Sheet Vinyl (without backing) Floor installation. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 100 SF

3020: Resilient Sheet Vinyl (without backing) Floor installation on Stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 20 SF

3021: Rubber Floor Tile installation on Floor. Excludes stairway Treads and Risers. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation. Estimated at: 2,000 SF

3022: Polish Resilient Sheet Vinyl Flooring. Polish pre-waxed/no wax flooring in various buildings following installation. Estimated at: 40 SF

3023: Ceramic Tile Floor and Mortar Bed Removal and Disposal. Remove Ceramic Tile and Mortar Bed in various buildings down to underlayment or bare concrete. Estimated at: 1,000 SF

3024: Ceramic Tile Floor Installation.	(Mosaic 1”X1”, 1”X2”, or 2”X2”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 250 SF

3025: Ceramic Tile Floor Installation.	(Mosaic 6”X6”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 250 SF

3026: Ceramic Tile Floor Installation.	(Mosaic 8”X8”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 100 SF

3027: Ceramic Tile Floor Installation.	(Mosaic 12”X12”). Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Ceramic Tile Floor and Mortar Bed in various buildings. Estimated at: 4,000 SF

3028: Quarry Tile Floor and Mortar Bed Removal and Disposal. Remove Quarry Tile and Mortar Bed in various buildings down to underlayment or bare concrete. Estimated at: 500 SF

3029: Quarry Tile Floor and Mortar Bed Installation. Repair cracks, holes, rough areas, depressions and other surface defects prior to installation of Quarry Tile Floor and Mortar Bed in various buildings. Estimated at: 500 SF

3030: Carpet Removal and Disposal, Excludes Stairway Treads and Risers. Remove carpet in various buildings down to underlayment or bare concrete. Estimated at: 8,000 SY

3031: Carpet Removal and Disposal from Stairway Treads and Risers. Remove carpet in various buildings down to underlayment or bare concrete. Estimated at: 100 SY

3032: Commercial “Severe” Grade Carpet Installation Excludes Stairway Treads and Risers. Clean Floor surface of all foreign materials prior to installation of new carpet. Includes Urethane carpet cushion installation. Installation process in various buildings includes stretching in over cushion (tack down). Estimated at: 150 SY

3033: Commercial “Severe” Grade Carpet Installation on Stairway Treads and Risers. Clean Floor surface on all foreign materials prior to installation of new carpet. Includes Urethane carpet cushion installation. Installation process in various buildings includes stretching in over cushion (tack down). Estimated at: 50 SY

3034: Exterior “Heavy” Grade Carpet Installation Excludes Stairway Treads and Risers. Clean floor surface of all foreign materials prior to installation of new carpet. Installation is direct glue down between carpet and floor (no cushion). Estimated at: 50 SY

3035: Remove and Dispose of Existing Particle Board or Plywood Floor Underlayment in various buildings. Estimated at: 500 SF

3036: Provide and Install Plywood Floor Underlayment in various buildings. Estimated at: 500 SF

3037: Remove and Dispose of Existing Membrane Waterproofing in various buildings. Estimated at: 40 SF

3038: Install New Waterproofing Membrane in various buildings. Estimated at: 2,000 SF

3039: Remove and Dispose of Existing Wood Base in various buildings. Estimated at: 1,500 FT

3040: Remove and Dispose of Existing Resilient Base in various buildings. Estimated at: 10,000 FT

3041: Reinstall Existing Wood Base in various buildings. Estimated at: 50 FT

3042: Install New Wood Base in various buildings. Estimated at 50 FT

3043: Install New 4-inch Resilient Base in various buildings. Estimated at: 15,000 FT

3044: Install New 6-inch Resilient Base in various buildings. Estimated at: 1,500 FT

3045: Install New 10—inch Resilient Stringer Base in various buildings. Estimated at: 50 FT

3046: Trim bottom of Doors in various buildings. Estimated at: 20 EA

3047: Paint Existing 6” Concrete Wall Base in various buildings. Estimated at 50 SF

3048: Install New Vinyl Edge Strips in various buildings. Estimated at: 1,000 FT

3049: Install New Aluminum Binder Edge Strips in various buildings. Estimated at: 200 FT

3050: Install New Aluminum / Vinyl Stair Nosing in various buildings. Estimated at: 200 FT

3051: Install New Rubber Molded Stair Tread and Nosing in various buildings. Estimated at: 200 FT

3052: Install New Rubber Molded Cover Stair Nosing in various buildings. Estimated at: 200 FT

3053: Install Concrete Sealant in various buildings. Estimated at: 100 SF

3054: Prepare Concrete Floor, Moderate (includes grinding, sanding & scraping) in various buildings. Estimated at: 30,000 SF

3055: Prepare Concrete Floor, Heavy (includes “shot blasting” or similar methods). Estimated at: 500 SF

3056: Installation of flooring outside of normal working hours as requested by the government.  Job to be determined on each task order. CLIN to be used in conjunction CLINS for removal and installation of flooring during normal working hours.  For example CLINS 0001, 0007, and 0056 would be included in a project to be performed after hours or on weekends.  Estimated at: 1,000 SF

3057: Remove and Re-install Bathroom Fixtures and Partitions and Miscellaneous Labor. Estimated at: 20 EA

3058: “Function Modular” or approved equal, 1/10 gauge Modular Tufted Textured Patterned Loop, solution dyed nylon, 24”X24” Carpet Squares with Severe Traffic CRI rating, vinyl, modular backing system. Flammability conforming to ASTM 648 Class 1 (Glue down), Smoke Density conforming to ASTM 662 less than 450. Static Propensity conforming to AATCC-134 under 3.5 KV. IAQ Green label certified. Clean floor surface of all foreign materials prior to installation. Installation is direct glue down between carpet and floor. Estimated at: 2,000 SY

3059: Provide Carpet with Pad attached at various buildings. Estimated at: 50 SY

3060: Install Carpet with Furniture in place. Lift Jack system to lift modular furniture or temporarily relocate furniture and replace after installation of carpet. Estimated at: 15,000 SF

3061. Provide moving furniture prior to and after installation of flooring. Includes general, desks, partitions, storage boxes, tables, file cabinets (full), chairs, and bookshelves. Quantity to be plan space the furniture covers.  Estimated at: 12,500 SF

3062: Plush Commercial "Heavy" Grade Carpet Installation Includes Stairway Treads and Risers or flat office areas. Clean Floor surface of all foreign materials prior to Installation of new Carpet. Installation is direct glue down between carpet and floor (no cushion). Estimated at: 50 SY

1.4 EXISTING WORK

In addition to "FAR 52.236-9, Protection of Existing Structures, Equipment, Utilities, and Improvements":

a. Remove or alter existing work in such a manner as to prevent injury or damage to any portions of the existing work which remain.

b. Repair or replace portions of existing work which have been altered during construction operations to match existing or adjoining work, as approved by the Contracting Officer. At the completion of operations, existing work shall be in a condition equal to or better than that which existed before new work started.

PART 2 PRODUCTS

Not used.

PART 3 EXECUTION

Not used.