STATEMENT OF WORK

Construction Services

for

B1 Wing 6 Seal Above Hallway Doors Project

- BACKGROUND INFORMATION

The Department of Commerce (DOC), National Institute of Standards and Technology (NIST) campus is located in Boulder, Colorado and is a research & development facility.  DOC also consists of National Oceanic Atmospheric Administration (NOAA) and National Telecommunications and Information Administration (NTIA) agencies on the NIST campus. The Office of Facilities and Property Management (OFPM) at NIST is responsible for maintaining and renovating spaces which consist of laboratories, office space and facilities maintenance areas, except the David Skaggs Research Center.

- PURPOSE

The objective of this acquisition is to provide the minimum requirements for the

The B1 Wing 6 Seal Above Hallway Doors Project. This project is to include extending the Wing 6 to Spine demising wall vertically upward to terminate against the bottom of the structural concrete deck above. It also includes sealing all penetrations and abandoned penetrations in the wall, where this wall separates the adjacent Women’s restroom from the Spine corridor. This acquisition also provides for an option (Option #1) to reconstruct storage room #4089 and seal all penetrations through the continuation of the W6/Spine demising wall that would be inside of the reconstructed storage room #4089.

- PLACE OF PERFORMANCE

Work will be performed at the US Department of Commerce (DOC) Boulder Laboratories campus at 325 Broadway, Boulder, CO in and around Building 1, Wing 6.

- SPECIFIC REQUIREMENTS

- The contractor shall coordinate with the Government Contracting Officer’s Representative (COR) all activities required throughout the duration of this task order.  The Contractor shall communicate with the COR and the Contracting Officer (CO) on all matters pertaining to the task order.

- Contractor will provide and install temporary chain link fencing and safety barriers, as required to secure the site.

Task 1 – Extend the B1W6 Demising Wall and Seal Demising Wall Penetrations (Base Scope – CLIN 001)

- The contractor will extend the Wing 6 to Spine Corridor demising wall vertically upward to terminate against the concrete deck above and/or horizontal structural surface above the wall. Currently the Wing 6 to Spine Corridor wall does not fully extend to the horizontal surface above, leaving a 2ft to 3.5ft gap. Extending this wall will result in thermally and mechanically isolating Wing 6 from the Spine corridor thermal/mechanical space.

- Additionally, at locations where this demising wall separates the Women’s restroom from the Spine corridor, the contractor will infill holes in gypsum sheathing at both sides of the wall and will seal any penetration gaps with fire rated sealants. All construction debris shall be disposed of by the contractor.

- The contractor shall provide for and/or install all materials necessary to achieve task 1 item 1 and item 2 above, including construction debris and construction zone barriers to control the impact of construction activities on all adjacent spaces and to all building occupants.

Task 2 – Reconstruct Storage Room #4089 & Seal all Penetrations Within (Option #1 – CLIN 002)

- Construct a new wall to reestablish previously existing storage room #4089.

- Provide hollow metal frames and hollow metal doors centered in the newly constructed wall system, with NIST campus standard required security hardware. Double hollow metal doors and frame provided will accommodate two 3’-0” x 7’-0” door slabs.

- Infill any open holes or gaps in the gypsum wall board that exist, with like-kind gypsum wall system materials. At all penetrations, provide fire rated sealant to seal all gaps around existing penetrations through the wall.

- Provide any code required additional fire sprinkler system sprinkler pipe(s) and head(s) necessary to protect the newly reconstructed storage room space.

- Provide interior room signage for the reconstructed storage room, per NIST campus standard interior signage requirements.

- Provide interior lighting.

- Coordinate lock and core with NIST.

- The contractor shall provide for and/or install all materials necessary to achieve task 2 item 1 through 5 above, including construction debris and construction zone barriers to control the impact of construction activities on all adjacent spaces and to all building occupants.

Safety on the NIST Campus

NIST is committed to maintaining a work environment that safeguards the public and the environment, as well as personnel and property.

If the award requires construction, demolition, excavation, renovation, or other work for which the contractor directs and oversees contractor personnel (including subcontractors) at NIST-owned and operated sites, the contractor shall:

Provide a detailed, site-specific Safety Plan to the contracting officer within 7 days after award that describes how the contractor will maintain a safe work environment and comply with applicable Occupational Safety and Health Administration (OSHA) standards, as well as with applicable state and local laws, standards, and codes.

The Safety Plan shall specify how the company’s policies and practices will comply with each of the applicable standards, laws, and codes.

The contracting officer will provide written comments regarding the plan to the contractor within seven (7) days of receipt and will require the contractor to submit a revised safety plan within seven (7) days after that to the contracting officer for their review. This cycle may be repeated, if necessary, until the contracting officer determines the safety plan to be acceptable.

Reference:

See NIST’s Contractor Safety Program for required elements of Contractor Safety Plans.

Safety-Related Equipment

The Contractor shall:

Provide all safety-related equipment required to perform the work under this award, including personal protective equipment (PPE) and other equipment that is required or appropriate to protect contractor personnel and other personnel on the NIST campus. Contractor-provided personal protective equipment shall include, at a minimum, hard hats, safety shoes, safety glasses (prescription or standard), respiratory protection (powered air purifying respirator or full-face mask respirator), high visibility clothing and gloves that meet applicable OSHA standards for PPE.

Training

The Contractor shall:

Provide safety-related training required by applicable OSHA, state and local regulations or standards to affected contractor personnel (including subcontractor personnel) that perform work under this award. Such training may include, for example, training for proper use of PPE, respirator use, fall protection, crane operation, forklift operation; electrical safety, or other safety-related topics.

Maintain records of safety training provided to contractor personnel (including subcontractor personnel) that perform work on this award. The contractor shall make those records available to the contracting officer, contracting officer’s representative for the award, or other authorized personnel within 7 days of receipt of their written request, and annually thereafter or upon request.

Require contractor personnel (including subcontractors) to attend   required NIST-specific health or safety training that is required by the award.

Health Hazard Assessments and Medical Evaluations

The Contractor shall:

Provide required health hazard exposure assessments, medical evaluations and screenings, and exposure monitoring required by applicable OSHA requirements.

Ensure PII information is protected.

Homeland Security Presidential Directive (HSPD)-12

The Contractor (and/or any subcontractor) and its employees who require physical access to NIST facilities or logical access to NIST IT networks or systems must comply with Homeland Security Presidential Directive (HSPD)-12, Policy for a Common Identification Standard for Federal Employees and Contractors; OMB M-05-24; OMB M-19-17; FIPS 201, Personal Identity Verification (PIV) of Federal Employees and Contractors; NIST HSPD-12 policy; and Executive Order 13467, Part 1 §1.2.

5.

The Contractor (and/or any subcontractor) must submit a roster by name, position, e-mail address, phone number and responsibility, of all staff working under this acquisition where the Contractor will develop, have the ability to access, or host and/or maintain a government information system(s).

The roster must be submitted to the COR and/or CO within 14 days of the effective date of this contract. Any revisions to the roster as a result of staffing changes must be submitted within 14 days of the change. The COR will notify the Contractor of the appropriate level of investigation required for each staff member. If the employee is filling a new position, the Contractor must provide a position description, and the Government will determine the appropriate suitability level.

Position Sensitivity Designations Requirement

All Contractor (and/or any subcontractor) employees must obtain a background investigation commensurate with their position sensitivity designation that complies with Parts 1400 and 731 of Title 5, Code of Federal Regulations (CFR).

- GENERAL REQUIREMENTS

Include all requirements that are not specific tasks but are applicable to the overall contract and the work that will be performed. For example, IT Security, Government Property, Public Affairs Clearance, OMB clearance, printing, data rights, etc.

REQUIREMENTS FOR CONTRACTOR PERSONNEL

Identify all qualifications that the Contractor and/or their personnel working on this contract must possess.

Safety:  The Contractor employee must be responsible for knowing and complying with all installation safety prevention regulations.  Such regulations include, but are not limited to, general safety, fire prevention, and waste disposal.

Security: NIST is a restricted campus.  An identification badge is required for access for entry into buildings and is shown to the armed Security Police when entering the campus.

Identification Badges: Contractor employees must comply with NIST identification and access requirements.  Each Contractor employee must wear a visible identification badge provided by the NIST Security Office.

- DELIVERABLES

            - Quality Control Plans and Site-Specific Safety Plans shall be submitted no later than 30 days after award.

            - Notice to Proceed will not be issued until the contractor’s Site-Specific Safety Plan has been accepted.

            - If not included in the proposal, a schedule of values shall be submitted no later than 30 days after award.

Standards of Acceptance: The NIST POC or COR shall review all the above deliverables and respond with an acceptance or an email with request for revision to the Contractor Point of Contact (POC) within 15 business days of receipt of a deliverable.

- The Contractor shall provide a submittal log and submittals in formats approved by the COR.

- Contractor is not to include any contact information in any project drawings, specifications or related information.

- The Contractor shall refer to all NIST provided drawings, specifications and information provided as part of this project.

- The Contractor shall review, and field verify all existing facility documentation provided by NIST.

- PAYMENT SCHEDULE

The Contractor will be paid, in accordance with the payments clause in the contract and as otherwise noted in this document, upon receipt of a proper invoice.

- 100% after installation and acceptance by the POC of fully installed system, AND

- After the successful completion of the testing requirements set forth in this document under section set forth in this document, AND

- SITE MOBILIZATION

      - NIST shall provide the Contractor with the area for construction and staging activities shown in the attached staging drawing. Additional storage areas on the NIST campus may be available to the Contractor if approved by NIST. Refer to NIST Boulder General Contractor Requirements.

- SUBMITTALS

Contractor shall provide the following submittals to the COR for approval. NIST shall have up to 14 calendar days to review and return the submittal to the contractor.

Submittals as required in the contract documents from the following divisions:

  - Division 00 - Procurement and Contracting Requirements

    - Project CPM Schedule

    - Construction daily reports – submitted to NIST daily during the progression of the project.

  - Division 01 - General Requirements

    - Refer to Attachment 01 for General Requirements Submittals

  - Division 02 - Existing Conditions

    - Pre-construction photographs showing the existing conditions of the area around the project extents

    - Submit proposed protection measures, including drawings, that indicates the measures proposed for protecting individuals and property. Indicate proposed locations and construction of barriers.

  - Division 03 – Concrete (NOT USED)

  - Division 04 - Masonry (NOT USED)

  - Division 05 – Metals

    - Product data and/or shop drawings for metal studs, bracing, fasteners and other metal framing products required to construct and/or extend demising walls and/or storage room walls.

    - Product data and/or shop drawings of all fasteners to be used in any wall construction/ framing extension.

  - Division 06 - Wood, Plastics, Composites (NOT USED)

  - Division 07 - Thermal and Moisture Protection

    - 2hr minimum rated fire sealant for use at penetrations.

    - Any alternate applicable 2hr fire rated product for use at penetrations.

  - Division 08 – Openings

    - Product data sheets and shop drawings sheets for hollow metal frames.

    - Product data sheets and shop drawings sheets for hollow metal doors.

    - Hollow metal frame and door information can be submitted together.

  - Division 09 – Finishes

    - Product data sheets and shop drawings for gypsum wall board and accessories.

    - Product data sheets and shop drawings sheets for all applicable paint primers and paint finish products to meet NIST campus standard type, finish/sheen and color requirements.

  - Division 10 – Specialties

    - Product data and shop drawings for NIST campus standard specified interior room signage.

  - Division 11 - Equipment (NOT USED)

  - Division 12 - Furnishings (NOT USED)

  - Division 13 - Special Construction (NOT USED)

  - Division 14 - Conveying Equipment (NOT USED)

  - Division 21 - Fire Suppression

    - Product data sheets, shop drawings, and fire suppression calculations for added fire sprinkler suppression in storage room #4089.

  - Division 22 - Plumbing (NOT USED)

  - Division 23 - Heating, Ventilating, and Air Conditioning (HVAC) (NOT USED)

  - Division 25 - Integrated Automation (NOT USED)

  - Division 26 - Electrical

    - Product data sheets, shop drawings and information related to interior lighting fixtures, per attached NIST specifications.

  - Division 27 – Communications

    - Cable routing drawings

    - Product data sheets

  - Division 28 - Electronic Safety and Security

    - Product data for security door hardware.

  - Division 31 - Earthwork (NOT USED)

  - Division 32 - Exterior Improvements (NOT USED)

  - Division 33 – Utilities (NOT USED)

- Contractor shall provide submittals through a NIST approved, secure electronic site accessible by NIST.

- SECURITY REQUIREMENTS

- No Government-furnished tools, equipment, or devices shall be removed from the NIST site.

- The contractor (including subcontractors) shall be responsible for properly protecting all information used, gathered, or developed as a result of work under this contract.

- The data shall not be used for any other purpose other than that specified in the contract. Any information made available to the contractor by the Government shall be used only for the purpose of carrying out the requirements of this contract.

- If the contractor (including subcontractors) anticipates using external information system services provided by third-party entities, the contractor shall ensure that external information system meets the security standards as described in NIST Special Publication 800-53 and shall be maintained throughout the duration of the contract.

- At the completion of the contract, all Government data shall be returned to NIST, or if return is not feasible, destroy and do not retain any copies of Government data such as backup copies created at the contractor (including subcontractors) site (and certify in writing to NIST that such return or destruction has been completed).

- Product and Execution Requirements

Refer to NIST site standards attachment.

Quality Assurance

- Comply with all applicable requirements from federal, local, and state regulations and ordinances concerning shoring, bracing, or sloping of excavations and safety of workers.

- All inspections and testing, including special inspections, required to complete the scope of work will be performed by an independent testing and inspection agency employed by the contractor and approved by the contracting officer as having the competence necessary to perform the work.

Attachments:

Attachment 1 - General Instructions for Conducting Work at the NIST Boulder Campus

Attachment 2 - Safety Plan Checklist