Statement of Need

RMR Regional VA Medical Center

1700 N. Wheeling St.

Aurora, Colorado 80045

- Background: Rocky Mountain Regional Medical Center has a requirement for Behavioral Health Beds for inpatient Mental Health Units who require specialized medical care.  Replacement Patient Beds are required for the Inpatient Mental Health Ward as the existing furniture is moveable, has paint that is chipping and is rusting. VA Central Office requires VA Mental Health Bedroom furniture that cannot be moved for Suicide Prevention Safety and diminished risk of danger to others.

- Requirement:

      - (32) ARC100 - Arcadian Platform Bed Color: Dove

      - (32) PRD-RR06K - Removable Restraint Ring Set (6 Rings)

##### Salient Characteristics:

##### Beds must meet the following requirements for patient safety and interior design needs.

##### MH-01	Behavior Health Platform Bed Quantity: 32

- 80” Bed Deck Length – Maximum 83” Bed Deck Length (no longer than 84” overall length

- 36” Bed Deck Width – Maximum 40” Bed Deck Width (no wider than 41” overall length)

- Height 16.9”

- Overall length 82”

- Overall width 38”

- Must fit 36” x 80” mattress

- Rotationally molded high impact polyethylene materials utilized

- Seamless one-piece construction ligature resistant with no internal openings for additional safety and security

- Inverted internal rib structure supports overall strength which allows for minimal surface movement

- Raised outer edges to keep fluids contained with gently crowned top to allow fluids to drain towards corners

- Removable restraint ring kit available

- Chemically resistant to blood, vinegar, urine, feces, salt solution and chlorine solution

- Seamless, one-piece construction that is ligature resistant with no internal openings

- Proprietary anchoring system with concealed hardware – 6 ring set attached

- Completely waterproof construction with hygienic wipe-clean surfaces

- Performance tested to 1,500lbs static load design

- Certified GREENGUARD and GREENGUARD Gold for low chemical emissions

- ASTM E1537 test method for fire testing of real scale upholstered furniture

- NFPA 261 cigarette ignition resistance of upholstered furniture

- Made in USA

##### Delivery Requirements:

  - Delivery Location:

RMR VA Medical Center

C/O Interior Design

1700 N. Wheeling St.

Aurora, CO 80045

Delivery and installation hours are Monday thru Friday, 7:30 am – 4:00 pm.

  - All furniture must be delivered to the loading dock.

##### Furniture Requirements

  - Provide warranty information, maintenance, or any other product manuals for each item listed below if applicable.

  - All finished surfaces shall be free of scratches, marks, dents, or blemishes, and withstand staining, and must not exhibit flaking, cracking, or loss of adhesion.