The Department of War

Radioactive Waste Program

Basic Ordering Agreement (BOA)

An Issuance of the DoW Lead Agent

for Low-Level Radioactive Waste

Version 6 May 2026

Table of Contents

| Section | Title | Page |
| --- | --- | --- |
| 0 | Program and Policy | 4 |
| 1 | Overview, Limitations, and Evaluations | 4 |
| 2 | Performance Work Statement Overview | 8 |
| 3 | Tiers of this Agreement | 8 |
| 4 | How to Participate in this BOA | 9 |
| 5 | The Approval and Removal Process | 12 |
| 5.1 | Technical Approval | 12 |
| 5.2 | On-Ramping | 12 |
| 5.3 | Reasons for Removal | 13 |
| 5.4 | Performance Requirements Summary | 13 |
| 5.5 | Complaints | 14 |
| 5.6 | Off-Ramping | 14 |
| 6 | Basic Performance Work Statement (PWS-B) | 14 |
| 6.1 | Safety and Licensing | 14 |
| 6.2 | Waste Shipper | 15 |
| 6.3 | Radiation Detectors | 15 |
| 6.4 | Communications | 15 |
| 6.5 | Inventory and Characterization | 15 |
| 6.6 | Cost Proposals | 16 |
| 6.7 | Technical Proposals | 16 |
| 6.8 | Travel | 16 |
| 6.9 | Schedule | 17 |
| 6.10 | Packaging | 17 |
| 6.11 | Shipping | 17 |
| 6.12 | Tasks | 17 |
| 6.13 | Transportation | 18 |
| 6.14 | Disposal | 19 |
| 6.15 | Regulatory Compliance | 19 |
| 6.16 | Materials, Equipment, and Supplies | 20 |
| 6.17 | Contractor Personnel | 20 |
| 6.18 | Invoices | 20 |
| 6.19 | Permits or Petitions | 21 |
| 6.20 | Authorizations | 23 |
| 6.21 | Reports and Deliverables | 23 |
| 6.22 | Ozone-Depleting Substances | 25 |
| 7.0 | Supplemental Requirements by Tier | 25 |
| 7.1 | Tier 1 – Low Level Radioactive Waste | 26 |
| 7.2 | Tier 2 – Low Level Mixed Waste | 26 |
| 7.2.23 | Generator Identification Number | 27 |
| 7.3 | Tier 3 – Decommissioning Support | 27 |
| 7.3.24 | Document Revisions | 31 |
| 7.3.25 | Final Print Copy | 31 |
| 7.4 | Tier 4 – Site Disposal Operations | 31 |
| 7.4.26 | Radiation Control Area | 35 |
| 7.4.27 | Discrete Sources | 36 |
| 7.5 | Tier 5 – Technical Radiation Protection, N.O.S. | 36 |
| 7.6 | Tier 6 – Emergency Response | 36 |
| 7.6.28 | Domestic Mobilization | 36 |
| 7.6.29 | Overseas Mobilization | 37 |
| 8 | In Scope Determination | 37 |
| 9 | Availability of Funds | 37 |
| 10 | Annual Review of this BOA | 37 |
| 11 | Requirements for Federal Contractors | 38 |

0.0.  PROGRAM AND POLICY

The U.S. Army is the Department of War Lead Agent (DOWLA) for disposal of low-level radioactive waste (LLRW). It provides oversight and guidance for the Department of War (DoW) LLRW Program. Its mission is safe, compliant, and cost-effective LLRW disposal. It is DoW policy that all LLRW be handled and disposed of in a manner that is protective of human health and the environment and complies with DoW issuances and applicable statutory and regulatory requirements.

Responsibility of the DoW LLRW Program is assigned in Army Regulation (AR) 385-10 to the U.S. Army Joint Munitions Command (JMC), Environmental and Health (Physics) Division to provide LLRW disposal services in accordance with Department of Defense Instruction (DODI) 4715.27.  (JMC has been operationally ordered by Department of the Army to integrate its workforce with Army Sustainment Command [ASC] to create a single command amongst the implementation of this BOA. The authority of the LLRW Program will reside with the DOWLA, regardless of command alignment, after the integration.)

The DoW LLRW Program relies on qualified contractors to carry out parts of this world-wide mission. Contractors shall travel to locations to prepare shipments of excessed or unwanted radioactive materials for ultimate transfer, recycle, or disposal (as waste). (Transfer of ownership, recycle, and return to manufacturers are only authorized on a case-by-case basis.  These paths are not considered paths for waste). Other requirements of the mission may include disposing of excessed or unwanted radioactive materials (as waste) generated by other private contractors on behalf of the Government or developing detailed work plans to characterize material or assist the Government in the planning future disposal of excessed or unwanted radioactive materials (as waste).

1.0.  OVERVIEW, LIMITATIONS, AND EVALUATIONS

In accordance with Federal Acquisition Regulation (FAR) 16.504(a)(4)(v), the only activity authorized to issue orders against this Basic Ordering Agreement (BOA) is the Army Contracting Command - Rock Island (ACC-RI).  The Contracting Officer (KO) for this BOA is:

Army Contracting Command - Rock Island

Attention: Ryan J. Buhman, Contracting Officer

3055 Rodman Avenue

Rock Island, IL  61299-8000

Comm: (309) 782-0248

E-mail:  ryan.j.buhman.civ@army.mil

Each Task Order will appoint a Contracting Officer’s Representative (COR) to act as the KO’s authorized representative to assist in administering a contract. The source and authority for a COR is the KO. COR limitations are contained in the written letter of appointment.  Namely, the COR is not empowered to make any contractual commitments or to authorize any contractual changes on the Government’s behalf.  Any changes that the Contractor deems may affect contract, price, terms, or conditions will be referred to the KO for action.

The Government reserves the right to determine the limitations of this BOA at any time to meet mission requirements or the demands of the DoW. The awards under this BOA will be competed in a fair environment to businesses with the 8(a) Small Business Designation. The 8(a) must be certified by the Small Business Administration (SBA).

In general, excessed or unwanted radioactive materials are designated radioactive waste (waste) when the generator determines whether the items are non-serviceable or condemned and has intent to discard it by land burial, except for low-level mixed waste (LLMW) which explicitly specifies the date of generation on an accumulation label for a specific container.  All waste under this BOA will either contain source, special nuclear, or byproduct material that is acceptable for disposal in a land disposal facility, in accordance with the U.S. Nuclear Regulatory Commission (NRC) regulations: Title 10, Code of Federal Regulations (CFR).  If waste is not acceptable for disposal in a land burial facility, it will be noted in the Task Order. Radioactive waste may also include low-level mixed waste (LLMW), naturally occurring radioactive material waste (NORM), and naturally occurring and accelerator-produced radioactive material waste (NARM) intended for land burial. It does not include:

- Waste resulting from activities falling under Sections 2121 and 2140 of Title 42, United States Code (U.S.C.) (also known as the “Atomic Energy Act of 1954”). These activities include, but are not limited to, waste:

  - Generated by the Naval Nuclear Propulsion Program in accordance with Executive Order 12344 and Section 7158 of Title 42, U.S.C.

  - Related to nuclear weapons programs.1

  - Generated by decommissioning former Army Deactivated Nuclear Power Program facilities.

- Decay in storage, release in effluents, disposal by release into sanitary sewage, treatment or disposal by incineration, and disposal of specific wastes in accordance with Sections 20.2002-20.2005 and 20.2108 of Title 10, CFR.

1 For purposes of this agreement only, 91(b) waste

is deemed as a waste eligible for disposal under this BOA.

In accordance with Federal Acquisition Regulation (FAR) 16.703, this Basic Ordering Agreement (BOA) provides a framework for future orders. Before placing an order, the Government will provide each 8(a) BOA-holder a fair opportunity to be considered. The specific procedures for placing an order will be detailed in the Request for Quote (RFQ) or Request for Proposal (RFP) for that order and will adhere to the following:

(a) Orders At or Below the Simplified Acquisition Threshold (SAT): The Government will solicit responses and award orders in accordance with the procedures outlined in FAR Part 13, Simplified Acquisition Procedures.

(b) Orders Exceeding the SAT: The Government will solicit proposals and award orders in accordance with the source selection procedures outlined in FAR Part 15, Contracting by Negotiation.

The Government intends to award orders to the contractor whose offer provides the best value. Unless otherwise specified in the order's solicitation, the Government will use a comparative evaluation methodology.

- Comparative Evaluation: As described in FAR 13.106-2(b)(3) for acquisitions under the SAT and consistent with best value principles under FAR Part 15, a comparative evaluation involves comparing offers to each other to select the contractor that is best suited for the requirement.

- Evaluation Factors: The specific evaluation factor(s) for the award of each order will be defined within the individual solicitation for that order and are at the Government’s discretion.

- Alternative Methodologies: If the Government intends to use an evaluation method other than comparative evaluation for a specific order, that method will be explicitly defined in that order's solicitation.

This BOA uses Task Orders to manage the DoW’s excessed or unwanted radioactive materials. Each Task Order is an individual project, and each project is assigned to a specific tier. There are six tiers of work which vary in scope and complexity. Each tier contains language found in a Basic Performance Work Statement (PWS-B) found in Section 6.0 – Basic Performance Work Statement (PWS-B), which is the basic language and requirements that guide the contractor to perform the work and (when applicable) additional supplemental language specific to each tier found in Section 7.0 – Supplemental Requirements by Tier.

Task Orders awarded on this BOA are non-severable because they involve shipping waste materials. Non-severability begins when the waste is shipped (physically moved) from the generator’s location and is accompanied by a manifest.

While Follow-On Task Orders for a particular site or location are not guaranteed, the Government retains the right to find those additional services to be non-severable based on its own discretion and award those Task Orders to the performing contractor if the technical risk and cost savings of the work are determined to be in the Government’s best interest (See Section 8.0. – In Scope Determination).

Modification of Task Orders are given priority to an already existing Task Order for a particular or near-by location. For instance, a customer at a particular location may accumulate more excessed or unwanted radioactive materials prior to mobilization. The Government reserves the right to add inventory to an existing Task Order, rather than competing the additional inventory over again. The Government may also determine that other existing Task Orders offer regional efficiencies (travel under one mobilization or use of a single conveyance in a certain region) as a strategy for cost avoidance.  The Government reserves the right to determine when these types of efficiencies exist as a reason to not compete the additional excessed or unwanted radioactive materials.

Mobilization of the contractor’s personnel or equipment or supplies can be severable. The Government reserves the right to terminate a Task Order between mobilization and shipping of waste materials. The Government, and its customers, are not responsible for recovering equipment or supplies sent ahead for projects that are terminated.  Periodically, the contractor may be required to coordinate travel accompanied by Government personnel. Severability is not contingent upon completing a trip with (or without) Government personnel; it is contingent upon shipping waste materials.

In the event of termination for convenience by the Government, the contractor is authorized to present a Settlement Proposal within 60 calendar days of issuance of the letter of termination. The Settlement Proposal shall not exceed the value of the original Task Order and its modifications.  The settlement window will close after 60 calendar days.

All Task Orders are firm-fixed price. Each Task Order will have an on-site period of performance date and a final period of performance date.

- The on-site period of performance date is the date that the inventory of excessed or unwanted radioactive materials must be shipped from the generator’s location.

- The final period of performance date is the date that the contractor must submit all deliverables and a final invoice for the project.

Prices presented at the time of award must be valid for the duration of both periods of performance.  A delay beyond the final period of performance will be handled using determination. The COR will determine whether the project was delayed by the inventory holder or caused by the contractor. The KO will review and approve the COR’s position. The contractor will be issued a modification that includes the determination made by the COR as to which entity (inventory holder or contractor) is primarily responsible for the inexcusable delay. In the event of inventory holder delay, the contractor is authorized to review their original bid and update the pricing for the revised schedule. Delays of any other sort will result in a “no cost” modification and terminate or extend the final period of performance. There could also be cases where the Government seeks consideration in these instances. Project delays caused by the contractor will result in unsatisfactory or marginal performance on the contractor’s Contractor Performance Assessment Reporting System (CPARS). The contractor cannot protest the COR’s determination related to delays in project schedule.

The contractor shall not take directions from the inventory holders or generators.  Only the KO can make commitments on behalf of the Government. Contractors will assume risk for carrying out commitments made with any other entity than the KO.  However, the DOWLA can give approval for discussions with inventory holders or generators on a case-by-case basis to determine potential courses of action to address anything that is not covered in the Task Order.

If the contractor is supplied Government-furnished equipment (predominately loading equipment) or manpower, the contractor maintains full responsibility of safe loading, shipment, and disposal of any waste. It is the contractor’s responsibility to coordinate, and direct, the entity providing the additional equipment or manpower.  The contractor is prohibited from using any government-furnished equipment that is not explicitly listed in the PWS-A, Section 16.

2.0.  PERFORMANCE WORK STATEMENT OVERVIEW

Project-specific details (a requirements package) will be issued to the contractor as a Task Order Performance Work Statement (PWS), hereafter referred to as an Addendum Performance Work Statement (PWS-A). The contractor shall adhere to the PWS-B, the applicable Supplemental Requirements by Tier, and the PWS-A (for the specific project). Participants in this BOA acknowledge that all work shall be performed in accordance with these documents, however, should a contradiction appear between this document (PWS-B) and the Addendum PWS (PWS-A) then precedence of the imperatives will be given to the PWS-A level.

3.0.  TIERS OF THIS AGREEMENT

The PWS-B describes the general scope of the administrative requirements, equipment, and manpower requirements. PWS-A’s, in general, require labor, travel, surveys, sampling, identification, packaging, rental, consumables, transportation, processing, disposal, permits, and subcontracts to dispose of excessed or unwanted radioactive materials (as waste) which may be defined as mixed waste with hazardous materials. There are six tiers of work that vary in complexity. The functions for a specific tier in Section 7.0. – Supplemental Requirements by Tier must be completed in addition to the PWS-B if they apply.  Implementation of the supplemental requirements by tier is a principle of propagation, where the PWS-B and the supplemental requirements of the specific tier are combined. The exact structure of these requirements for each tier is detailed in Section 7.0.  A description is included:

TIER 1 (T1) - Low-Level Radioactive Waste (Minimum Requirement) Pack, ship, and dispose of licensed and non-licensed radioactive materials.

TIER 2 (T2) - Low-Level Mixed Waste Pack, ship, and dispose of licensed and non-licensed radioactive materials also characterized as a hazardous waste.

TIER 3 (T3) - Decommissioning Support Planning, conducting, evaluating, and documenting building surface and surface soil final status radiological surveys for demonstrating compliance with dose or risk-based regulations or standards.

TIER 4 (T4) - Site Disposal Operations Long-term mobilization of personnel and equipment to support remediation projects with characterization, packing, shipping, and disposal of excessed or unwanted radioactive materials.

TIER 5 (T5) – Technical Radiation Protection - Not Otherwise Specified Any requirement for the program related to the safe, compliant, and cost-effective management and handling of the DoW’s radioactive materials.

TIER 6 (T6) – Emergency Response Incidents outside of normal operations involving DoW-owned radioactive materials that will become waste or support for the Army Contaminated Equipment Retrograde Team (A.C.E.R.T.).

4.0.  HOW TO PARTICIPATE IN THIS BOA

Any Small Business Administration-certified 8(a) contractor that would like to receive awards under this BOA must provide qualifications to be considered for T1 work:

- Evidence that the company has access to a waste shipper (broker) via broker qualifications **

- Evidence that the company holds a Federal or Agreement State radioactive material license that is suitable for waste brokerage and transportation.

** Broker qualification requirements and radioactive material license conditions are described in Addendum 1:

** The Government does not dictate who the contractor hires for brokerage. However, the company must have access to a broker to utilize. The contractor’s business dealings with individuals for market advantages are between the individual and the company and any discrepancies in pricing are considered risks that must be analyzed and mitigated by the contractor as they see fit. Regardless, all brokers must be approved annually by DOWLA in writing prior to performance of work. Upon submission of qualifications, the DOWLA will issue a memorandum either approving or declining the broker.

** If the company no longer has access to the broker they have previously utilized, then inform the DOWLA within 5 business days of termination/exit. Failure to do so could result in off-ramping from this BOA.  See Section 5.3 – Removal and Section 5.4 – Off-Ramping.

To be considered for T2 through T4, the Small Business Administration-certified 8(a) contractor may also submit:

- Evidence that the company has access to a senior broker (waste shipper) via broker qualifications

- Evidence of the company or member of the company having past performance or experience supporting radioactive decontamination and decommissioning projects (i.e. Final Status Survey Plan or Final Status Survey Report)

- Evidence in a (maximum) 2-page Statement of Qualifications that describes the company’s prior experience in disposing of waste from demolition and/or excavation site(s).

T5 and T6 share the same qualifications as T1.  Contractors who enter this BOA will have the opportunity to compete for T5 and T6 work.  These tiers encompass non-standard technical support and emergency response actions. Accordingly, the definitive requirements will be set forth on a case-by-case basis within the PWS-A for each specific task.  However, the contractor must adhere to the PWS-B, Supplemental Requirements by Tier (if applicable), and the PWS-A.

All companies should submit the maximum amount of support practical, and the Government will make the final determination on the highest tier of work they may perform on this BOA. All determinations are final without the option to protest. The contractor is responsible for updating their broker’s qualifications and experience through-out the duration of this working relationship using the processes described in Section 5.2 – On-Ramping of this BOA. The DOWLA will send a Memorandum for Record (MFR) showcased in Addendum 2 qualifying their broker. This letter will give them the authority to direct the preparation of shipments of the Government’s excessed or unwanted radioactive materials on the Government’s behalf and sign shipping documentation, profile sheets, permit applications, and other paperwork to meet regulatory requirements for Federal, State, regional compacts, local, and other criteria.  (However, it does not allow brokers to sign the Environmental Protection Agency’s [EPA] uniform hazardous waste manifests [UHWM].  See Section 7.2.23):

Small Business Administration-certified 8(a) may also submit a statement of qualifications or cover letter that certifies they can:

- Travel to military installations and defense sites worldwide

- Adhere to the project schedules established by the Government

- Prepare and maintain shipping records

- Track waste through disposal and readily report on its status and location

- Prepare waste profiles and maintain disposal records

- Share documents securely with Government customers

- Invoice for payment using Government systems

- Maintain communications with the Government

The Tiers of work, and qualifications for those Tiers of work, are in Addendum 3. T1, T5 and T6 share the same base level of qualifications. However, the Government retains the right to select the Tier that best fits the complexity of the project. The Tiers start with the most simple requirements and increase in complexity:

All Task Orders are to be invoiced within 365 days of award.  Figure 1, below, provides a notional timeline of the key activities and their durations required to meet this one-year completion requirement.  The timeline is only to be used as a guide:

NON-SEVERABILITY

FIGURE 1

5.0.  THE APPROVAL AND REMOVAL PROCESS

5.1.  TECHNICAL APPROVAL.  Interested contractors shall submit their qualifications via email at any time to the following mailbox:

usarmy.ria.jmc.mbx.amsjm-sfr@army.mil

The subject line shall include the capital letter words “FOR CONSIDERATION” to be valid.

The DOWLA will confirm receipt of a valid email submission within seven (7) calendar days. Technical approval of these submissions will occur at the Government’s discretion, but no fewer than one time per year. This minimal annual review and approval shall occur between 1 November through 30 November each year. However, the Government may review a contractor’s qualifications submission at any time. For an interested contractor to ensure their submission is reviewed within a calendar year’s 1 November through 30 November review period, qualifications should be submitted no later than 30 September of that calendar year. Any qualifications submitted after this date may not be reviewed until the following calendar year’s review period of 1 November through 30 November.

The outcome of the above-mentioned review will result in either an official “Technical Approval” or a “Technical Rejection” for the contractor’s ability to continue to pursue participation in this BOA; to include the Tiers in which proposals on future work will be accepted by the Government. The Government reserves the right to decide which contractors may participate in the BOA and the Tiers of work they qualify for.  All determinations are final for the calendar year’s review period.

Technical Approval does not ensure acceptance into the BOA. Technical Approval does, however, allow the contractor to enter the On-Ramping Process with ACC-RI, outlined in Section 5.2 – On-Ramping of this BOA. An example of this Technical Approval Letter is enclosed in Addendum 4:

5.2.  ON-RAMPING.  The Government reserves the right to decide which contractors, and what Tiers of work, may participate in this BOA.  All determinations are final.

- PROMOTION TO HIGHER WORK.  The contractor is responsible for notifying the DOWLA when their broker has achieved the qualifications for senior broker. It is not the Government’s responsibility to track the level of experience of a particular employee. The contractor shall submit the promotion following the instructions in 5.1. – Technical Approval.

5.3.  REASONS FOR REMOVAL.  Participating contractors may be removed from this BOA for marginal or unsatisfactory CPARS.  However, timely response to Corrective Actions Reports (CARs) can remedy removal from this BOA in these instances. Other reasons for removal may include, but are not limited to breach, mutual consent, evidence of waste, evidence of fraud, inactivity**, trafficking in persons, unlawful acts, criminal sanctions, or notice of violations related to regulatory compliance. The contractor will be notified in written communication that they have been removed from this BOA and its implementation will be effective immediately upon transmittal.

** Inactivity is defined as failing to submit a proposal on any Task Order for a period of two consecutive calendar years.

5.4.  PERFORMANCE REQUIREMENTS SUMMARY.  These are the three most important performance objectives and are the minimum requirements to perform at the “Satisfactory level” on a performance evaluation. This section is intended to summarize and highlight each performance objective. These objectives are what the DOWLA deems the acceptable level to sustain participation in the contract awards process associated with this BOA. This BOA also has its own Quality Assurance and Surveillance Plan (QASP) to surveil important components of maintaining participation in this BOA.  The QASP is not provided to the contractor.  For transparency, the contractor should be aware that they will be evaluated annually on the status of their radioactive material license (Paragraph 6.1) , the access to a broker (Paragraph 6.2), and their overall performance in the areas of disposal (Paragraph 6.14).

The table below focuses on standards of project management because the requirements of this BOA are all similar in nature.  This review may happen at any time across all Task Orders award to any one contractor.

Each Task Order requires the contractor to schedule, deliver, and invoice to complete the project. The COR will evaluate the contractor in CPARS under these areas.  However, they may also inspect other areas, as they determine necessary.  Here are the three most important performance objectives:

| PERFORMANCE OBJECTIVE | STANDARD | PERFORMANCE THRESHOLD ACCEPTABLE QUALITY LEVEL | METHOD OF SURVEILLANCE |
| --- | --- | --- | --- |
| PWS-B, Para 9. SCHEDULE | Contractor shall schedule the work with the COR within 28 calendar days of Task Order award.* *The contractor will not be penalized for any failed attempts to schedule the work because of coordination issues, lack of response, or unavailability of Government personnel. | Zero Deviation from the Standard. | 50% Inspection. |
| PWS-B, Para 18. INVOICES | Contractor shall submit invoices in accordance with Sections i-vi. | Greater than 85%. | Random Sampling. |
| PWS-B, Para 21. REPORTS AND DELIVERABLES | Contractor shall submit reports and deliverables in accordance with the PWS-B and the Supplemental Requirements by Tier. | Zero Deviation from the Standard. | Random Sampling. |

5.5. COMPLAINTS. Should the contractor feel they are unfairly evaluated against this criterion and no longer receive awards based on any evaluation prepared by the Government, the contractor may submit a complaint by email to the KO requesting continued participation in the BOA. The complaint must include formal position of why the evaluation prepared by the Government does not fairly address their performance. Complaints must be submitted within 30 calendar days of receipt of the unfavorable CPARS. The KO reserves the right to make a final decision on the contractor’s level of performance.  The complaint window will close after 30 calendar days.

5.6.  OFF-RAMPING.  The Government reserves the right to off-ramp contractors at any time from this BOA.  All determinations are final.

Basic Performance Work Statement (PWS-B)

6.0. TECHNICAL PERFORMANCE REQUIREMENTS

The requirements below must be met to perform LLRW disposal Task Orders, also described as T1:

6.1.  SAFETY AND LICENSING.  Contractor shall possess a Nuclear Regulatory Commission (NRC) or Agreement State radioactive materials license with a safety plan that includes health physics support, hazardous and radiological laboratory support, dosimetry, personnel protective equipment and clothing, and hazardous constituents and radiation monitoring equipment. The license must be valid at the time of the on-ramping and throughout the performance of projects issued on this agreement.  The contractor will acquire reciprocity in agreement states, and with the NRC (as appropriate), to handle waste depending on its origin and physical location.

6.2.  WASTE SHIPPER.  Contractor shall employ a person (not necessarily a Health Physicist) trained, educated and/or with a minimum of 3 years of experience in waste brokering as a significant activity.  (This person is commonly referred to as a “broker”.)  The broker has the authority to sign shipping papers, profiles, and manifests for the Government, with the exception of signing as the generator on an EPA UHWM.

6.3.  RADIATION DETECTORS.  Contractor shall have access to radiation detectors.  (Equipment must be in good working order and calibrated.)

- SURVEYS.  Contractor shall conduct surveys to determine radiation levels and removable contamination on package(s), when applicable.

- RADIONUCLIDE IDENTIFICATION.  Contractor shall identify radionuclides and activities or concentrations of radioactive material, when required.  (The inventory holder is expected to provide generator knowledge about the radionuclides of concern and their associated activities for each row of inventory; however, this capability may be needed in situations where the inventory holder requires assistance with identification.)

6.4.  COMMUNICATIONS.  Contractor shall coordinate all aspects related to this agreement through the KO.  (The KO ensures performance of all necessary actions for effective contracting, ensures compliance with the terms of the agreement and safeguards the interests of the United States in the contractual relationship.)

  - TECHNICAL OVERSIGHT.  Contractor shall include the COR on all communications related to this contract from the date award through close-out.  (CORs are responsible for technical administration of the project and ensure proper Government surveillance of the Contractor’s performance. The COR is not empowered to make any contractual commitments or to authorize any contractual changes on the Government’s behalf. Any changes that the Contractor deems may affect agreement, price, terms, or conditions will be referred to the KO for action.  Contractors can speak to inventory holders or generators with, both, the KO and COR’s written approval.)

  - PROJECT NUMBER.  Each project holds a unique identification number. Contractor shall include the project number on all correspondence and all deliverables supplied by the contractor for the duration of the project.

6.5.  INVENTORY AND CHARACTERIZATION.  Reserved.  This section of the PWS-B is reserved as all inputs will be found in the corresponding PWS-A.

6.6.  COST PROPOSALS.  Contractor shall respond to a Requests for Proposal (RFPs) with a document showing the cost, cost elements, and other units related labor, travel, consumables, equipment, rentals, transportation/shipping, processing, disposal, permits, other expenses, and subcontracts.  The cost proposal shall be prepared using Addendum 5:

- SUBCONTRACTS.  If the contractor needs to subcontract certain elements of the project, the contractor’s cost proposal shall break out all costs, to include all subcontracted costs. (This information not only informs the COR of actual cost for processing, disposal, etc., but it allows the contractor to incrementally invoice for periodic progress of this project rather than having to wait for project completion prior to invoicing.  This information also allows the COR to provide accurate feedback to the customer. Include a brief explanation in accordance with Section 6.18.ii – Format.)

- FIFTY-ONE PERCENT.  Contractor shall submit a cost proposal showing that most of the labor costs are being performed by the 8(a) contractor who is preparing the proposal.  To be considered an 8(a) contractor, the company must perform at a minimum 51% of the labor costs so that subcontracted labor costs do not exceed 49% of the Task Order.  Any subcontracted labor costs planned for this requirement must be approved by the Government before award.

6.7.  TECHNICAL PROPOSALS.  Contractor shall respond to RFPs with a document describing the technical approach.  The technical proposal shall be prepared using Addendum 6:

6.8.  TRAVEL.  Contractor shall travel to various Government or commercial sites in the Continental United States (CONUS) and sites Outside the Continental United States (OCONUS).  (The Government is not responsible for a failure to obtain or a revocation of site access.  This also includes general delays in obtaining site access.)

- SITE ACCESS.  Contractor shall coordinate on-site schedules and site access for themselves and their subcontractors with the site points of contact (POCs) supplied in the Task Order prior to travel (Upon the award of the Task Order contract, other site POCs information will be given to the contractor.  Typically, this is 30 calendar days before mobilization.) Contractor shall provide the following information:

      - Date and time of arrival at the site.

      - Number and names of contractor personnel.

      - Reason for visit.

      - Length of stay.

      - Any special instructions required for performance of the project, etc.

- ESCORT. Contractor shall anticipate no escort for themselves or drivers onto military or contractor-operated Government installations and facilities.  (For contractor personnel that do not have a Common Access Card [CAC], coordinate with the site POC for base access procedures prior to travel.)

- DEFENSE TRAVEL REGULATIONS.  Contractor shall follow the requirements of the Defense Travel Regulations, including providing the required level of receipt for cost-reimbursable items.  (The DOWLA reserves the right to reject invoices for unauthorized expenses.)

6.9.  SCHEDULE.  Contractor shall schedule the work with the COR within 28 calendar days of award of the Task Order contract.  (Failure to do so could result in off-ramping from this BOA.  See Section 5.3 – Removal and Section 5.4 – Off-Ramping.)

6.10.  PACKAGING.  Contractor shall know domestic, international, and commercial facilities LLRW laws, regulations, and procedures for the inspection, storage, handling, packaging, transportation, processing/treatment, disposal, and licensee/permittee acceptance of the waste materials.  (Contractors may be required to package waste/materials and/or inspect previously packaged wastes/materials to verify contents and ensure proper regulatory compliance and packaging requirements are met.)

6.11.  SHIPPING.  Contractor shall prepare and load packaged waste in accordance with Federal regulations, including Nuclear Regulatory Commission (NRC), Department of Transportation (DOT), Occupational Safety and Health Administration (OSHA), International Air Transport Association (IATA), International Maritime Dangerous Goods Code (IMDG) when shipping by vessel, disposal facility and other receiving facility laws and criteria for transportation CONUS and OCONUS, to include correct preparation of all shipping papers and manifests.

6.12.  TASKS.  Contractor shall complete basic packing and shipping functions:

- DOCUMENTATION.  Contractor shall complete all documentation required by the recycling, processing, treatment, and/or disposal facilities.

- COORDINATION.  Contractor shall coordinate with recycling, processing, treatment, and/or disposal facilities for shipping and receiving dates and times.

- LOADING PLAN.  Contractor shall develop a loading plan and arrange an appropriate shipping method for the inventory.

- INSPECTION.  Contractor shall visually inspect the excessed or unwanted radioactive materials and verify inventories.

- REPACKAGING.  Contractor shall package or repackage waste.

- LABELING AND MARKING.  Contractor shall appropriately label or mark packages of waste in accordance with Title 10 and 49 CFR and other federal, state, and local regulations.

- SHIPPING PAPERS.  Contractor shall prepare shipping papers in accordance with Title 10 and 49 CFR and other federal, state, and local regulations.

- BLOCKING AND BRACING.  Contractor shall load, block, and brace packages of waste for shipment.

- PACKAGE INVENTORY AND SURVEY DOCUMENTS.  Contractor shall document all final package inventories, surveys, and samples performed of packages, including any photographs taken of inventory, or packages, for purposes of tracking the items to their interim and final destinations.

6.13.  TRANSPORTATION.  Contractor shall propose the most cost-effective transportation method(s) for the inventory.  (The proposed carriers shall be approved to carry hazardous materials.)

- REGULATIONS.  Contractor shall prepare and make the shipments of waste in accordance with Title 10 and Title 49 CFR.

- OFFEROR.  Contractor shall use a Government-approved broker and transporter to make all shipments of hazardous materials.

- DURATION.  Contractor shall ensure waste delivery meets the timeline provided in the PWS-A.

- STATEMENT OF LIABILITY.  Contractor’s waste shipper shall sign shipping paperwork in accordance with Section 6.2 of this PWS-B.

- CARRIER CHANGES.  Contractor shall immediately notify the COR in writing about any changes related to the transportation or conveyance method that was technically accepted.  (The DOWLA does not authorize contractors to change transporters mid-shipment or utilize a single carrier for multiple efforts, unless otherwise specified and accepted in advance.  Carriers are not authorized for storage of waste and are strictly used for movement to an intended destination.)

6.14.  DISPOSAL.  Contractor shall propose the most safe, compliant, and cost-effective recycling, processing, and/or disposal facility for the waste.  (The contractor shall develop and enclose a roadmap for each line of the inventory with a destination and include it within the technical proposal.)

- VARIATIONS.  Contractor shall immediately notify the COR in writing if the disposal roadmap changes.  Any changes after award to the final destinations within the proposal must be approved, in writing, by the KO and the COR prior to execution.

- DIRECT DISPOSAL. Contractor shall, when able, present direct permanent disposal of waste.  (i.e. shipments directly to a disposal facility without use of a processor.)

- PROCESSING OR TREATMENT.  Contractor shall ensure processing and/or treatment of waste does not exceed 270 calendar days from the date of drop-off at the processor.  (The Government considers processing and treatment essential steps in the disposal progression. Therefore, the contractor must propose an expedited schedule to meet DoW objectives.  Failure to meet this deadline may result in unfavorable CPARS and/or financial considerations.)

- AUTHORITY.  The DOWLA reserves the right to make final determinations for disposal. Contractor shall adhere to the DOWLA’s determinations for disposal.

- STATEMENT OF LIABILITY.  Contractor shall certify that the waste is properly classified, described, packaged, marked and/or labeled, and is in proper condition for transportation according to the applicable regulations.  (Contractor’s waste shipper shall sign shipping paperwork in accordance with Section 6.2 of this PWS-B.)

6.15.  REGULATORY COMPLIANCE.  Contractor shall comply with all DoW, Federal, State, and local laws and regulations when performing project tasks.

- FINES.  Contractor shall assume fiscal responsibility for monetary fines levied against the DoW, from regulatory agencies, resulting from the contractor’s field activities.

- LICENSED RADIOACTIVE WASTE.  Contractor shall dispose of licensed radioactive material at a licensed radioactive waste disposal facility.  (The waste must be properly profiled and must meet the Waste Acceptance Criteria [WAC] for the chosen disposal facilities.) 1

¹ The contractor may apply for approval of an alternative disposal method in accordance with 10 CFR 20.2002. This process allows for the disposal of certain low-activity radioactive waste at facilities other than a licensed radioactive waste disposal facility, provided the contractor can demonstrate to the NRC the proposed method will be protective of public health and safety.  However, the Government must technically accept this approach.

- PERMITS AND AUTHORIZATIONS.  Contractor shall acquire all permits and authorizations required for the field work prior to executing it.  (See Section 6.19. Permits and Section 6.20. Authorizations.)

6.16.  MATERIALS, EQUIPMENT, AND SUPPLIES.  Contractor shall supply all services, equipment, materials and supplies necessary to complete the PWS-B, unless otherwise specified.

6.17.  CONTRACTOR PERSONNEL.  Contractor shall supply all personnel to complete the PWS-B, unless otherwise specified.  (Contractor hours may be periodically surveilled.  Any labor hours, and labor categories, will be scrutinized if the Government surveillance perceives discrepancies from the technically accepted proposal.  This is considered fraud, which is a reason for removal from this BOA in Section 5.3. – Reasons for Removal.)

    - COMPETENCY TRAINING.  Contractor shall not allow personnel to perform any activities and/or tasks on installations without proper and adequate qualifications or job competency training. In the event of any identified noncompliance, the Contractor shall, if requested, provide proof of contractor personnel training or qualification (individual name, training/qualification type, training/qualification certificate, and date of training/qualification) to perform those agreement activities associated with the identified noncompliance.

6.18.  INVOICES. Upon completion of the work requirements, contractor shall provide invoice(s) to the COR for approval.

  - WORK PERFORMED.  Contractor shall invoice only for work performed.

  - FORMAT.  Contractor shall state on the invoice the Project Number and a brief explanation of the work performed within dates specified for the performance.  (Failure to provide this will result in non-payment of the invoice(s) until revision is made.  All invoices shall be submitted electronically through Wide-Area Work Flow (WAWF) – see Clause 252.232-7006 in the Task Order Contract for procedures.  For any invoice that includes transportation or recycle, processing, and/or disposal charges, the contractor shall include shipping papers showing receipt of the material and certificates of disposal, as appropriate.  The contractor shall include a statement within or on these invoices certifying that they have distributed the shipping papers and/or disposal certificates to the generating customer and the COR.)

  - PROPRIETOR.  Contractor shall submit invoices with the employer (prime contractor’s name) on the invoice.  (The DOWLA will reject invoices with subcontractor’s name and all statements submitted by individual employees.)

  - NOTIFICATION.  Contractor shall ensure the COR is immediately notified in writing that an invoice has been submitted for inspection and acceptance.  (The desired method of notification system integrated into WAWF immediately after submitting an invoice.  It sends an email to a customer distribution list.  The contractor shall ensure the COR is included on the notification.)

  - INVOICE PERIOD.   Contractor shall invoice within 30 calendar days of the issuance of the final deliverables.

  - RECEIPTS.  Contractor shall only include receipts for authorized expenses.  (When possible, the contractor shall provide an itemized list of expenses with supporting receipts.  Official receipts must be dated and meet all Defense Travel Regulations.  Itemization of official receipts shall be presented on a document with official company letterhead – See Section 6.18.iii - Proprietor.)

6.19.  PERMITS OR PETITIONS.  Contractor shall obtain required permits or petitions from the Federal, State, or municipal agencies at least 30 calendar days prior to mobilization, as required.

    - WASTE COMPACT RULES.  Contractor shall follow all low-level radioactive waste interstate compact rules.  (It is the contractor’s responsibility to know which compact they are performing work in and adhere to all rules.)

    - SYNCHRONIZATION.  Contractor shall ensure that all shipments conform to the rules, regulations, and requirements of the State/Compacts. (The contractor must make sure that the designated Permit POC and the COR are informed.  This does not absolve the contractor from their sole responsibility in adhering to the permit or petition and its required conditions.)

    - CIVIL PENALTIES.  Contractor shall settle any civil penalties between State/Compact and the generator or the DOWLA or DoW Component.  (Any civil penalty issued by the compacts related to any project will be settled between the State/Compact and the contractor. The contractor shall obtain export permits, petitions, export authorizations, waste tracking permits, and disposal permits for each project.)

    - ADHERANCE.  Contractor shall adhere to all terms, timelines, and conditions of the permits or petitions.  (The contractor is responsible for obtaining the permit or petitions and completing all required conditions required by the export permits, export authorizations, waste tracking permits, petition, and disposal permits.)

    - APPLICATIONS, NOTIFICATIONS, AND REPORTS.  Contractor shall make necessary notification(s) to the States/Compacts.  (The contractor shall complete all actions required to maintain compliance, to include, but not limited to, create applications, sign applications, submit applications, amend applications, extend applications before expiration, submit notifications of export, submit interim disposal reports, submit progress reports, submit notification of disposal, and any other related reporting requirements not otherwise specified.)

    - PAYMENTS.  Contractor shall propose the required fees for permits or petitions and submit payments to obtain all permits or petitions.

    - SIGNATORY.  To acquire a generator’s signature on the Government’s behalf, contractor shall coordinate with designated Permit POC.

    - CORRESPONDENCE.  Contractor shall include the Permit POC and the COR on all correspondence concerning the permits or petitions obtained; to include export authorizations, waste tracking permits or petitions, and disposal permits or petitions.

    - OTHER DISPOSAL PERMITS. Contractor shall request, at least 30 calendar days prior to mobilization, the site use permit(s) for Republic Services-Richland from the Permit POC and the COR, when required.  (The DOWLA maintains some Low-Level, NARM and Exempt Waste site-use permits. They are available for use for any approved wastes profiles at Republic Services-Richland.  The site-use permits are issued by the State of Washington Department of Health.  These permits are acquired annually from the State of Washington Department of Health and expire so they must be verified for availability.)

    - OTHER TRANSPORTATION PERMITS.  Contractor shall request, at least 30 calendar days prior to mobilization, the transportation permit for the State of Mississippi from the Permit POC and the COR, when required.  (The DOWLA maintains transportation permits for each branch.  They are available when waste is transported into, through, or out of the State of Mississippi.)

6.20.  AUTHORIZATIONS.  Contractor shall obtain authorizations from the DoW, Federal, State, or municipal agencies, as required.

- MORRIS CONSOLIDATION FACILITY.  Contractor shall coordinate with the COR at least 30 calendar days prior to mobilization, to request an authorization letter for materials to be sent to the Morris Consolidation Facility (MCF), located at Rock Island Arsenal.

      - The MCF can take many different items.  However, it predominately takes licensed tritium devices and generally licensed chemical/explosive detection equipment for transfer or recycle.

      - The COR and the facility’s Radiation Safety Officer (RSO) reserve the right to determine which items are acceptable for delivery to the MCF.

- AIR FORCE RADIOACTIVE RECYCLING AND DISPOSAL (AFRRAD).  Contractor shall coordinate with the COR at least 30 calendar days prior to mobilization, to request an authorization letter for materials to be sent to the consolidation facility at Wright-Patterson Air Force Base (WPAFB).

      - AFRRAD accepts a variety of instruments and articles containing radioactive material for recycling and waste minimization services, including but not limited to: static eliminators, radioluminescent devices, vacuum tubes, chemical agent detectors, and ion-generators.

      - The COR and the Air Force liaison reserve the right to determine which items are acceptable for delivery to WPAFB.

6.21.  REPORTS AND DELIVERABLES.  Contractor shall prepare reports, and other possessions, and deliver them to the Government.

    - DAILY REPORTS.  For any day that the contractor is on-site, contractor shall develop and submit an electronic daily report that summarizes weather, site conditions, work performed, work planned, personnel assigned, equipment utilization, and challenges.  Injuries or illnesses will also be summarized in the report.  (The report shall be provided within one business day of completion.)

    - MONTHLY REPORTS.  On the first of each month, contractor shall develop and submit an electronic monthly report using Addendum 7:

    - FINAL REPORTS.  Upon issuance of the final invoice, contractor shall include an annual report using Addendum 8 with the final invoice:

    - PROPERTY RIGHTS.  Contractor shall release reports as Government property upon issuance.  (All contractor-prepared documents and drafts, developed as part of a project, shall become the property of the U.S. Government. The Government reserves the right to distribute and reproduce all or part of the documents without restriction.)

    - PACKAGE INVENTORIES.  Contractor shall provide all final package inventories (itemized list of contents) for purposes of tracking the items to their interim and final destinations.

    - SHIPPING PAPERS.  Contractor shall provide all shipping papers with acknowledgement of receipt and acceptance by the transportation contractor for each shipment from the installation, within 10 business days of acceptance.

    - MANIFESTS.  Contractor shall, when applicable, provide all manifests with acknowledgement of receipt for each shipment from the installation, within 10 business days of acceptance.

    - CERTIFICATES OF DISPOSAL.  Contractor shall provide, from each waste disposal facility, a copy of the Certificate(s) of Disposal (CODs) for each waste shipment within 30 calendar days after the date of disposal.

- TRANSFER PAPERWORK.  Contractor shall, when applicable, provide receipt of material transferred to a specific licensee within 10 calendar days of acceptance.

6.22.  OZONE-DEPLETING SUBSTANCES.  In accordance with the National Defense Appropriations Act (NDAA), the contractor, in performing all Task Orders as a project, shall in no way construe the Government’s direction as supporting, suggesting, or directing the use of ozone-depleting substances.  Contractor shall specifically bid on all proposals and perform all contractual efforts in compliance with this Act.

6.23. – 6.30.  Reserved.

7.0.  SUPPLEMENTAL REQUIREMENTS BY TIER

The functions below must be completed in addition to the PWS-B, when stated. Implementation of the supplemental requirements by tier is a principle of propagation, where the PWS-B and the supplemental requirements, of the specific tier, are combined.  This is only applicable for the first four tiers of work.  For instance:

- T1 requires the language in the PWS-B, Section 7.1 – Low-Level Radioactive Waste, and the PWS-A.

- T2 requires the language in the PWS-B, Section 7.1 – Low-Level Radioactive Waste, Section 7.2 – Low-Level Mixed Waste, and the PWS-A.

- T3 requires the language in the PWS-B, Section 7.1 – Low-Level Radioactive Waste, Section 7.2 – Low-Level Mixed Waste, Section 7.3 – Decommissioning Support, and the PWS-A.

- T4 requires the language in the PWS-B, Section 7.1 – Low-Level Radioactive Waste, Section 7.2 – Low-Level Mixed Waste, Section 7.3 – Decommissioning Support, Section 7.4 – Site Disposal Operations, and the PWS-A.

The contractor shall adhere to the PWS-B (for the designated tier) and the PWS-A (for the specific project). Participants in this BOA acknowledge that all work shall be performed in accordance with these documents, however, should a contradiction appear between this document (PWS-B) and the PWS-A then precedence of the imperatives will be given to the PWS-A level.

T5 and T6 are slightly different.  They shall also adhere to the PWS-B, the Supplemental Requirements for that specific tier (if applicable), and the PWS-A.  We expect these requirements to be outside of “normal operations” but are considered integral to the veracity of the overall program.  It is likely they will have their own unique set of specifications defined almost entirely in the specific PWS-A for that task.  Contractors should, however, not overlook the requirements explained in this document.

Areas that are reserved for the remainder of this document default to the corresponding section of the PWS-B.

7.1. TIER 1 - LOW-LEVEL RADIOACTIVE WASTE

Reserved.  Requirements for T1 are exclusively outlined in the PWS-B.  This area is reserved for additional requirements or revisions, as they become known.

7.2. TIER 2 - LOW-LEVEL MIXED WASTE

In addition to the PWS-B and Section 7.1 – Low-Level Radioactive Waste, the functions below must be met to perform LLMW disposal Task Orders, described as T2:

7.2.1.  Reserved.

7.2.2.  WASTE SHIPPER.  Contractor shall employ a person (not necessarily a Health Physicist) trained, educated and/or with a minimum of 5 years of experience in waste brokering as a significant activity. The senior broker shall prepare EPA UHWMs, but it is not authorized to sign them on the generator’s behalf.

7.2.3. – 7.2.7.  Reserved.

7.2.8.  TRAVEL.  Contractor shall prioritize travel for LLMW projects due to regulatory waste accumulation schedules.

7.2.9.  SCHEDULE.  Contractor shall adhere to all waste accumulation schedules and must transfer all waste to the appropriate treatment, storage and disposal facility (TSDF) prior to the expiration of the accumulation dates of the generator to maintain regulatory compliance.  (Specific compliance deadlines will be specified in the PWS-A.)

7.2.10.  PACKAGING.  Contractor shall know domestic, international, and commercial facilities requirements to dispose of LLMW including wastes generated under the Resource Conservation and Recovery Act (RCRA) and/or Comprehensive Environmental, Response, Compensation, and Liability Act (CERCLA).

7.2.11.  SHIPPING.  Contractor shall prepare and load packaged waste in accordance with Federal, including Environmental Protection Agency (EPA), OSHA, DOT, IATA, IMDG when shipping by vessel, disposal facility and other receiving facility laws, regulations, and criteria for transportation CONUS and OCONUS, to include correct preparation of all shipping papers and manifests.

7.2.12.  TASKS.  Contractor shall complete these additional advanced packing and shipping functions:

- HAZARD IDENTIFICATION.  When generator knowledge is not presented, the contractor shall identify, classify, and profile hazardous components of waste materials.

- HAZARDOUS SAMPLING.  Contractor shall develop sampling and analysis plans for Government review, if needed.  Contractor shall perform analytical analysis by a DoW Environmental Laboratory Accreditation Program (ELAP) certified analytical laboratory to characterize the wastes.  Sampling will achieve comprehensive waste profiles for the generator that also meet the requirements of the disposal facility.

- LIQUIDS.  Contractor shall compliantly manage liquids prior to disposal.

- LABELING AND MARKING.  Contractor shall appropriately label or mark packages of LLMW in accordance with Title 10, 40, and 49 CFR and other Federal, State, and local regulations.

- SHIPPING PAPERS.  In addition to other required shipping papers, contractor shall prepare shipping papers in accordance with 40 CFR.

7.2.13.  Reserved.

7.2.14.  DISPOSAL. Contractor shall ensure the waste is disposed of at a properly permitted RCRA disposal facility, when applicable, or other appropriate facilities.

7.2.15.  REGULATORY COMPLIANCE.  Contractor shall comply with all applicable Federal, State, and local environmental laws, statutes, regulations, permits, Army regulations (with supplements), as well as Major Subordinate Command (MSC) and installation regulation, policy, Host Tenant Agreement, Interagency Service Support Agreement, or Status-of-Forces Agreement. Contractor shall immediately report any conflicts between applicable federal, state, local environmental laws, statutes, and provisions of AR 200-1, and any specifications within this agreement or subsequent Task Order contract to the COR.

7.2.16. – 7.2.22.  Reserved.

7.2.23.  GENERATOR IDENTIFICATION NUMBER.  Contractor shall coordinate with the site POC to identify local Environmental POC to sign for their associated EPA Generator ID Number on a UHWM required for the hazardous constituents of the waste at least 30 calendar days prior to mobilization.

7.2.24. – 7.2.30.  Reserved.

7.3.  TIER 3 – DECOMISSIONING SUPPORT

In addition to the PWS-B, Section 7.1. – Low-Level Radioactive Waste, and Section 7.2. – Low-Level Mixed Waste, the functions below must be met to perform Decommissioning Support Task Orders, described as T3:

7.3.1. – 7.3.2.  Reserved.

7.3.3.  RADIATION DETECTORS.  Based on the potential radionuclide contaminants, their associated radiation type and the types of residual contamination categories (e.g., soil, structure surfaces) to be evaluated, contractor shall have access to radiation detectors or other instruments that are commonly used in decommissioning.  (Equipment must be in good working order and calibrated. Choose reliable instruments that are suited to the physical and environmental conditions at the site and capable of detecting the radiation types at the appropriate minimum detectable concentration (MDC) and/or minimum detectable level (MDL)).

- WASTE, SOIL, AIR, VEGATATION, AND WATER SAMPLING AND ANALYSIS.  Contractor shall collect waste, soil, air, vegetation, and water samples and provide scientific analyses and reports on the samples.  Results shall be provided to the COR.

- SURVEYS OF STRUCTURES.  Contractor shall use appropriate detectors to collect direct measurements in survey units on structures and land areas.

- SCANS.  Contractor shall use appropriate detectors to perform radiation scans in survey units on structures and land areas.

7.3.4. – 7.3.11.  Reserved.

7.3.12.  TASKS.  Contractor shall complete these additional decontamination and decommissioning functions:

- PRELIMINARY ASSESSMENT, SITE RECONNAISSANCE, AND FEASIBILITY STUDY.  Contractor shall assess, inspect, investigate, and study sites contaminated with radioactive, hazardous, or mixed waste to conduct actions, under NRC, CERCLA, RCRA and EPA.

- HISTORICAL SITE ASSESSMENT.  Contractor shall perform actions (review historical facility records, conduct interviews, etc.) to determine the extent of potential impact of radioactive contamination at the site.

- RECIPROCITY.  Contractor shall perform under reciprocity with the NRC or equivalent Agreement State regulatory agency.  (Specific guidance regarding the technical approach for the release of the facility should be coordinated with the regulatory agency of the licensee prior to conducting site work.)

- SITE ACCESS CONTROLS.  Contractor shall get consent to perform surveys in designated areas and ensure the site is secure and has appropriate signage or other means to prevent access in or through all survey unit boundaries.

- REMEDIAL ACTIONS.  Contractor shall transfer, recycle, or dispose of any excessed or unwanted radioactive materials prior to conducting scoping or characterization surveys.  (This includes performing remedial actions to prepare areas to become survey units.)

- SCOPING SURVEYS AND CHARACTERIZATION.  Contractor shall conduct surveys and sampling with enough information to design the Final Status Survey Plan for compliance, demonstration and free release.  Results shall be provided to the COR.

- DECONTAMINATION.  Contractor shall decontaminate structures, equipment, materials, wastes, or areas for free release and certify the results.  Results shall be provided to the COR.

- RISK ASSESSMENT.  Contractor shall perform the correct risk assessments (concentration or dose or risk criterion) method to demonstrate unrestricted release criteria for radioactive, hazardous, and mixed wastes.

- DATA QUALITY OBJECTIVES.  Contractor shall establish Data Quality Objectives (DQOs) and periodically assess them throughout the Data Life Cycle.

- MARSSIM.  Contractor shall perform all surveys in accordance with NUREG 1557, Multi-Agency Radiation Survey and Site Investigation Manual (MARSSIM) to statistically confirm or deny the presence of radioactive contamination.

      - Identify Contaminants.  Contractor shall identify contamination and/or radionuclides of concern (ROCs) prior to performing any surveys.

      - DCGLs.  Contractor shall develop site-specific Derived Concentration Guideline Levels (DCGLs) for the ROCs on surface and structures, as required.

      - Area Classifications.  Contractor shall develop maps delineating the survey units and their classification, as required.

- SURVEY DESIGN.  Contractor shall design a survey that investigates the potential contamination pathways and describes the instrumentation, sample frequency, sample analysis with the required Quality Assurance (QA)/Quality Control (QC) protocols.

- FINAL STATUS SURVEY.  Upon approval of the survey design and plan, contractor shall mobilize to the site and set up all labor, equipment, and materials to conduct the survey.

      - BACKGROUND REFERENCE AREA.  Contractor shall establish the appropriate background references for the survey design.

      - FIXED RADIATION MEASUREMENTS.  Contractor shall take the appropriate number of fixed (static readings) in each survey unit.

      - SCANNING.  Contractor shall perform the correct level of scanning for each survey unit.

      - DEMOBILIZATION.  Contractor shall clean up the work areas, survey and release all lay down areas, and demobilize all personnel and equipment.

- FINAL STATUS SURVEY REPORT.  Contractor shall review all Final Status Survey data, draw conclusions, then issue the Final Status Survey Report that incorporates all data, samples, and QA/QC protocols that meet the criteria for the decommissioning or termination of a license/permit.  This report shall be provided to the COR and any other POCs designated in the task order.

7.3.13. – 7.3.16.  Reserved.

7.3.17.  CONTRACTOR PERSONNEL.  Contractor shall provide a person trained, educated, and experienced in the applications of health physics techniques, to include decommissioning, and having a minimum four-year degree in a science field and 5 years’ experience in health physics work.

7.3.18. – 7.3.20.  Reserved.

7.3.21.  REPORTS AND DELIVERABLES.  Contractor shall prepare reports, and other possessions, and deliver them to the Government.

- FINAL STATUS SURVEY PLAN.  Contractor shall classify and specify boundaries of survey units in the form of a final status survey plan at least 60 calendar days after demobilization from remedial actions, scoping, and characterization surveys.

- FINAL STATUS SURVEY REPORT.  Contractor shall issue a final status survey report that provides data to demonstrate that all radiological parameters satisfy the established guideline values and conditions within 60 calendar days of completing the final status survey.

7.3.22. – 7.3.23.  Reserved.

7.3.24.  DOCUMENT REVISIONS.  Contractor shall issue all documents as a draft, draft final, and final versions for approval.  Contractor shall submit the draft versions either via e-mail or secure data sharing website.  Contractor shall incorporate comments from the Government prior to issuing a draft final version.  Contractor shall submit the draft final version to the installation and outside regulatory agencies as deemed appropriate by the Government for external review.

7.3.25.  FINAL PRINT COPY.  Contractor may be requested to print, bind, mail, and deliver one print copy of the Final Status Survey Report to the COR.  (The PWS-A will clearly state if this is required.)

7.3.26. – 7.3.30.  Reserved.

7.4.  TIER 4 – SITE DISPOSAL OPERATIONS

In addition to the PWS-B, Section 7.1 – Low-Level Radioactive Waste, Section 7.2 – Low-Level Mixed Waste, Section 7.3 – Decommissioning Support, the functions below must be met to perform Site Disposal Operations Task Orders, described as T4:

7.4.1. SAFETY AND LICENSING.  Contractor shall acquire all required radioactive material licenses to perform work sites under Exclusive Federal Jurisdiction, Concurrent Jurisdiction, or Proprietary Jurisdiction. (The Government can provide the correct jurisdiction of any site for the contractor prior to preparing their proposal.)

    - MAINTENANCE.  Contractor shall invoke and maintain their license(s) (as necessary) to cover all required activities for the required duration of the on-site work.  Copies of these licenses shall be provided to the COR.

    - MEMORANDUM OF UNDERSTANDING.  Contractor shall establish a Memorandum of Understanding (MOU) with other site contractor(s) (as applicable) who have invoked their license(s) at the site and are performing radiological work.  (This MOU will clearly define areas of responsibility and areas of control for the various radioactive material licenses and contractors involved.  The contractor shall show areas of control on a map to be included in the MOU.)  A copy shall be provided to the COR.

    - NOTIFICATIONS.  Contractor shall make all necessary notifications to regulatory agencies at the beginning and end of site disposal operations.  A copy of notification(s) shall be provided to the COR.

    - INSPECTIONS.  Contractor shall participate in regulatory inspections and share outcomes of these inspections with COR within 10 business days of receipt.

7.4.2. – 7.4.8.  Reserved.

7.4.9.  SCHEDULE.  Contractor shall prepare to maintain permanent presence at a designated site not to exceed one year (365 days) unless options are presented that exceed one year, or a shorter duration, are agreed to between the Government and contractor during negotiations in response to an RFP.  (Contract follow-ons and options are only exercised when a formal Task Order modification is issued.)

7.4.10.  PACKAGING.  Contractor shall provide hard-sided containers for waste: up to 20 to 25 cubic yards nominal capacity intermodal or roll-off type containers with hard lids, capable of being locked.  Contractor shall ensure that all containers that contain waste are locked when the contractor is not working on them.  (Soft-sided packaging is generally not acceptable for large volumes of DoW waste (soil and debris), however, soft-sided packaging may be acceptable if, it will be explicitly stated in the PWS-A, Section 10.)

- SCALES.  Contractor shall provide a means to weigh the contents of the containers, which must be in calibration.

- MOVEMENT.  Contractor shall supply a means to move containers around the site, as needed, giving preference to locally sourced options.  (If movement of container will be provided by another means, it will be listed in the PWS-A, Section 16.)

7.4.11.  SHIPPING.  Once containers are filled and the waste is characterized, contractor shall mobilize an approved broker to the site to prepare shipping papers, load, and ship the contents for disposal no more than every thirty days.  Shipping papers shall be provided to the COR.  (There may be allowances made to mobilize at extended intervals, beyond thirty days, if there is not enough waste generated. These allowances must be approved by the KO in writing.)

7.4.12.  TASKS.  Contractor shall complete these additional site-specific functions:

- PROJECTIONS.  Contractor shall prepare waste profiles for site-specific waste projections.  (At times, the Government may forecast or estimate the total volume or weight of debris, soil, or other media to be removed from a site by demolition or excavation.  The contaminants may also be assumed.  The contractor shall prepare proposals based on Government projections described in the PWS-A but verify that the actual characterization is acceptable for the intended waste profile.  Immediately notify the Government of any changes.)

- DECONSTRUCTION, DEMOLITION, AND EXCAVATION.  Contractor shall remove structures, equipment, materials, and soils or de-water and segregate contaminated from non-contaminated materials.

- CONTAINER FLEET.  Contractor shall acquire, mobilize, maintain, and prepare (for use), a fixed number of intermodal or roll-off type containers at the site to support site remediation efforts. (These containers should be in good repair, fully functional, and lockable and sealed to prevent rainwater intrusion.  The Government may order a specific number of containers depending on size of the designated lay down area.)

- CONTAINER INSPECTION.  Contractor shall perform radiological receipt baseline surveys and a quality receipt inspection for damage upon receipt of each intermodal or roll-off type container.  (If a supplied bin is received and unsuitable to receive and transport of waste materials, the contractor shall replace the bin within one business day of failure.  The contractor shall ensure that all seals and components of the bin are in proper working condition to prevent rainwater intrusion.)

- SITE ACCESS.  Contractor shall ensure site security and site access requirements are met for their personnel (and subcontractors) for the duration of the site disposal operations.

- SAMPLING.  Contractor shall take QA/QC samples of soil, at an interval determined by the customer.

7.4.13.  TRANSPORTATION.  Contractor shall issue shipping papers for the disposed contents.  (Shipping papers shall be tied to a specific project and use integers to correspond with the shipments for a specific Task Order.)

- SAMPLE TRACKING.  Contractor shall track samples taken of soil.

- WASTE TRACKING.  Contractor shall track all containers from empty in the Radiation Control Area (RCA), to issuance in the field, to the dates sampled, with the characterization included, waste shipped, waste disposed, and date returned empty.

7.4.14.  DISPOSAL.  Contractor shall ensure that the waste is properly characterized and profiled, meets the proposed processor/disposal facility’s WAC and is approved for processing/disposal.  (Contractor shall complete all documentation that is required by the processing/disposal facilities prior to all shipments.  Contractor shall coordinate with the processing/disposal facilities for shipping and receiving times.)

7.4.15.  REGULATORY COMPLIANCE.  Contractor shall work under the appropriate licensing per the requirements of the site’s jurisdiction.

7.4.16.  MATERIALS, EQUIPMENT, AND SUPPLIES.  Contractor may be required to provide operational equipment (such as quarters, restroom, etc.) for long-term assignment of personnel.  Operational equipment shall meet the 29 CFR 1910.120 and 29 CFR 1926.65 requirements for worker’s safety during the duration of the project.

7.4.17.  CONTRACTOR PERSONNEL.  Contractor shall ensure all on-site personnel will be 40-hour OSHA qualified and have current 8-hour refresher training. (Contractor shall maintain training records onsite of 40-hour OSHA training qualification and current 8-hour OSHA refresher training.  OSHA regulations do not require the 8-hour refresher training if the worker completed the 40-hour training within the last year.)

7.4.18.  INVOICES. Upon completion of the work requirements, contractor shall provide dated invoice(s) to the COR for approval on a monthly interval, at minimum.

- CONTAINER RENTAL.  Contractor shall submit receipts for any containers rented.

- SAMPLES.  Contractor shall submit sample data for any samples taken.  (Reports must be dated.)

- TRANSPORTS.  Contractor shall attach the dated manifest or shipping paper as support for the transportation units being paid.

- WEIGHT TICKETS.  Contractor shall submit dated weight tickets for each disposal.  (The weight ticket must designate if the contents are debris or soil.  List the gross, tare, and net weight for each container).

- SOIL.  Contractor shall submit the weight or volume of soil waste with total number of units of soil being invoiced.   (The Government will only accept net tonnage or net volume as the pricing terms for payment.)

- DEBRIS.  Contractor shall submit the weight or volume of debris waste with the total number of units of debris being invoiced.   (The Government will only accept net tonnage or net volume as the pricing terms for payment.)

- COST CONTROL.  For any fixed-unit-rate (FUR) CLINs on the task order contract, contractor shall submit appropriate documentation (weight tickets, field reports, turn-in paperwork, etc.) to validate the number of units being invoiced for.  For multiple waste types, the contractor shall clearly mark the type of waste being disposed of on the documentation.  The contractor must ensure FUR CLINs are not exceeded.

- NOTIFICATIONS.  Contractor shall monitor the number of units being used against the units available on the task order for the duration of the project.  Contract shall immediately notify the Government when they reach or exceed 50%, 75%, and 90% of any FUR CLINs on the task order contract.  (The contractor releases Government from all claims if the contractor proceeds with utilizing FUR CLINs more than the units available on the Task Order contract.  Contractor shall not complete any work beyond 100% of the units on the task order contract.)

- RESPONSIBILITY.  Contractor has sole responsibility to monitor spending, invoice monthly, make timely government notifications, and ensure that all FUR CLINs (or options not yet in place) on the task order contract are not exercised or exceeded.

7.4.19. – 7.4.20.  Reserved.

7.4.21.  REPORTS AND DELIVERABLES.  Contractor shall track waste and samples and deliver their reports to the Government:

- WASTE TRACKING REPORT.  Contractor shall submit a waste tracking report to the COR on the first of each month.

- SAMPLE TRACKING REPORT.  Contractor shall submit a sample tracking report to the COR on the first of each month.

7.4.22. – 7.4.24.  Reserved.

7.4.26.  RADIATION CONTROL AREA (RCA). Contractor shall establish a perimeter fence or utilize an existing structure (with the site’s agreement), as a secure radiation control area (RCA).  (The RCA is used as a secure and locked lay down area to stage empty containers and store full containers of waste as it is generated.)

- OUTGOING.  Contractor shall maintain a written inspection log with container numbers and inspect each container’s overall site-worthiness prior to issuance to the field.  Namely, contractor shall ensure that contents do not leak out of the container and that locked lids do not allow rainwater intrusion.  (Contractor shall replace containers that do not pass inspection within one business day of failure.)

- LINERS. Contractor shall insert an appropriate liner to prevent spillage or loss of material into all containers prior to issuance to the field.

- FILL LINE.  Contractor shall participate in the filling process and ensure containers are filled to not exceed weight capacity for transportation by the intended conveyance, whether road or rail for domestic shipments.  Additionally, contractor shall ensure that void space is minimized, and containers are loaded as efficiently as possible.

- INCOMING.  Contractor shall radiologically survey incoming containers with waste to ensure contamination levels and radiation levels meet requirements for transport.

- ABSORBENT.  Contactor shall add absorbent to containers (as required) to ensure no free-standing liquids are acquired during storage or transport to disposal facilities.

7.4.27.  DISCRETE SOURCES.  Contractor shall provide smaller containers to collect discrete sources recovered or excavated from the site.  Contractor shall log the item, identify the nuclide of concern, estimate activity, bag and label the items for additional handling, and log the item in a container log sheet.

7.4.28. – 7.4.30.  Reserved.

7.5. TIER 5 - TECHNICAL RADIATION PROTECTION, N.O.S.

Reserved.  Requirements for T5 are outlined in the PWS-B.  This area is reserved for additional requirements or revisions, as they become known.

7.6. TIER 6 – EMERGENCY RESPONSE

In addition to the PWS-B, the functions below must be met to perform Emergency Response Task Orders, described as T6:

7.6.1. – 7.6.27.  Reserved.

7.6.28.  DOMESTIC MOBILIZATION.  Contractor, on verbal orders from the KO, shall provide rapid response capabilities to respond to an emergency situation involving radioactive materials.  (Such capabilities can mean that the Contractor shall provide personnel, equipment, materials and/or procedures to resolve a critical situation.  The response time will be within 48 hours for the continental United States (CONUS).  If an emergency response team requirement is used, the KO will confirm his/her verbal orders with a formal delivery order.)  Contractor personnel are required to have all applicable travel documentation required for domestic travel.

7.6.29.  OVERSEAS MOBILIZATION.  Contractor, on verbal orders from the KO, shall provide rapid response capabilities to respond to an emergency situation involving radioactive materials.  (Such capabilities can mean that the Contractor shall provide personnel, equipment, materials and/or procedures to resolve a critical situation.  The response time will be within 72 hours for the other than continental United States (OCONUS).  If an emergency response team requirement is used, the KO will confirm his/her verbal orders with a formal delivery order.)  Contractor personnel are required to have all applicable travel documentation required for international travel.

7.6.30.  Reserved.

8.0. IN SCOPE DETERMINATION

During the performance of work under this BOA, if additional radioactive material is identified on-site prior to the completion of the Task Order Contract, the Government reserves the right to determine that such material is within the scope of a current Task Order Contract. This determination is based on the principle that addressing the additional material under the existing Task Order Contract is the most cost-effective and time-efficient method for waste removal.

The inclusion of additional radioactive material shall be executed through a supplemental agreement or with the mutual consent of the parties involved. The contractor and the Government shall negotiate the terms, including any necessary adjustments to cost, schedule, or performance requirements, to ensure compliance with applicable regulations and the efficient completion of the project.

9.0. AVAILABILITY OF FUNDS

All solicitations and follow-on order awards issued under this BOA are always subject to the availability of funds. The Government reserves the right to cancel any solicitation or proposed award if funding becomes unavailable or insufficient to meet the requirements of the solicitation. This provision ensures the Government's ability to manage fiscal resources responsibly and maintain compliance with applicable regulations.

10.0. ANNUAL REVIEW OF THIS BOA

In accordance with FAR 16.703(c)(2), a BOA shall be reviewed annually before the anniversary of its effective date and revised as necessary to conform to the changing requirements of the FAR and other applicable statutes and regulations.  BOAs shall be changed only by modifying the agreement itself and not by individual orders issued under it.  Modifying a BOA shall not retroactively affect orders previously issued under it.  The terms of the BOA may need to be revised before the annual review due to changes in mandatory requirements.

11.0. REQUIREMENTS FOR FEDERAL CONTRACTORS

In connection with the performance of work under this agreement, all participants agree to the following:

- The contractor will not engage in any racially discriminatory diversity, equity, and inclusion (DEI) activities, as defined in section 2 of the Executive Order of March 26, 2026 (Addressing DEI Discrimination by Federal Contractors);

- The contractor will furnish all information and reports, including providing access to books, records, and accounts, as required by the contracting agency pursuant to the Executive Order of March 26, 2026 (Addressing DEI Discrimination by Federal Contractors), for purposes of ascertaining compliance with this clause;

- In the event of the contractor’s or a subcontractor’s noncompliance with this clause, this contract may be canceled, terminated, or suspended in whole or in part, and the contractor or subcontractor may be declared ineligible for further Government contracts;

- The contractor will report any subcontractor’s known or reasonably knowable conduct that may violate this clause to the contracting department or agency and take any appropriate remedial actions directed by the contracting department or agency;

- The contractor will inform the contracting department or agency if a subcontractor sues the contractor and the suit puts at issue, in any way, the validity of this clause; and

- The contractor recognizes that compliance with the requirements of this clause are material to the Government’s payment decisions for purposes of section 3729(b)(4) of title 31, United States Code (False Claims Act).”.

-  END OF AGREEMENT  -