APPENDIX A

- Contract #:  TBD

Task Order #:  TBD

ACQR #:  6145032

Fund Type:  ENVBRAC

Project Title/Location:  EV35, Limited Munitions Surface Clearance, Nomans Land Island, Chilmark, MA

Attachments:	(a) Scope of Work dated 05 March 2026

- Project Points of Contact (POC’s):

NAVFAC MIDLANT Contracting Officer’s Representative (COR)

Derek Pinkham

BRAC PMO East

4911 South Broad Street

Philadelphia, PA  19112

215-200-5182

derek.j.pinkham.civ@us.navy.mil

The Contracting Officer has authorized the COR to perform general oversight and technical administration of the negotiated contract.  In that position the COR may provide in-scope direction to the Contractor and assures the terms of the negotiated services.  The COR administers the scope and outside agency interface and coordinates criteria and technical oversight within the IPT.

NAVFAC MIDLANT Contract Specialist

Heather Dennis, Code CON41

9324 Virginia Ave., Bldg. Z-140, 2nd Floor

Norfolk, VA  23511

757-341-1518

heather.m.dennis13.civ@us.navy.mil

The Contract Specialist is responsible for all contract terms, changes, or deviations requiring contract adjustments.  No changes to the contract scope will be made or additional work authorized without the prior approval of the Contracting Officer.

- Project Scope:  See Attachment (a)

- Project Milestones:

Task Order Completion Date	31 July 2029

- Deliverables

In addition to the Records Management requirements, deliverables shall be provided via email (or other electronic document delivery method) per the schedule below (no hard copies should be necessary).

| Item # | Deliverable | PWS Reference Paragraph(s) | RPM | NOSSA | CSO | MADEP | USFWS | Due Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ESS | Section 2.2.1 | Draft Draft Final Final | Per NOSSAINST 8020.15E | Draft Draft Final Final | - | - | 30 days after kickoff meeting |
| 2 | QAPP | Section 2.2.2 | Draft Draft Final Final | - | Draft Draft Final Final | Draft Draft Final Final | Draft Draft Final Final | 60 days after kickoff meeting |
| 3 | PCR | Section 2.8.4 | Draft Draft Final Final | - | Draft Draft Final Final | Draft Draft Final Final | Draft Draft Final Final | 45 days after fieldwork complete |
| 4 | AAR | Section 2.8.5 | Draft Draft Final Final | Per NOSSAINST 8020.15E | Draft Draft Final Final | - | - | 45 days after fieldwork complete |

- Project Submittal Requirements and Distribution

MAILING ADDRESSES:  DIRECT DISTRIBUTION TO EACH ADDRESSEE BY CONTRACTOR

NAVFAC MIDLANT RPM / COR

Derek Pinkham

BRAC PMO East

4911 South Broad Street

Philadelphia, PA  19112

215-200-5182

derek.j.pinkham.civ@us.navy.mil

U. S. Fish and Wildlife Service

Grace Bottitta

Eastern Massachusetts Wildlife Refuge Complex

73 Weir Hill Road

Sudbury, MA  01776

978-579-4026

grace_bottitta@fws.gov

Massachusetts Department of Environmental Protection

Randi Augustine

Bureau of Waste Site Cleanup

1 Winter Street

Boston, MA  02108

617-292-5788

randi.augustine@state.ma.us

BRAC Environmental Coordinator

Christopher Harding

BRAC PMO East

4911 South Broad Street

Philadelphia, PA  19112

215-897-4904

christopher.l.harding4.civ@us.navy.mil

SCOPE OF WORK

EV35, Limited Munitions Surface Clearance, Nomans Land Island, Chilmark, MA

05 March 2026

- GENERAL

This Statement of Work solicits services for performing munitions response program support at Nomans Land Island, Chilmark, MA. The initial task includes a Limited Munitions and Explosives of Concern (MEC) Surface Clearance.   Navy Base Realignment and Closure (BRAC) Program Management Office (PMO) East will administer this contract.  This action will be performed in accordance with the Comprehensive Environmental Response, Compensation, and Liability Act (CERCLA); Executive Order 12580; the National Oil and Hazardous Substances Pollution Contingency Plan (NCP); and the Massachusetts Contingency Plan (MCP).

Under this PWS, the following services will be required: Conduct all work required to complete a Limited instrument-aided visual MEC Surface Clearance of accessible areas and inspect, certify, verify, demilitarize, and dispose offsite all MEC items. Details of this scope are further defined in Section 2. All work must be performed following applicable and appropriate Department of Defense (DOD) guidance and policy for the Navy Environmental Restoration Program (ERP), to include the Munitions Response Program (MRP), and consider all site documentation and reports to date.

Primary Objective

The primary objective of this project is to provide a Limited MEC Surface Clearance of accessible areas and inspect, certify, verify, demilitarize, and dispose offsite all MEC items at Nomans Land Island. The project scope shall include all of the necessary labor and equipment associated with meeting the objective.

The Contractor shall obtain final approval of the work required by BRAC PMO East and NAVFAC Mid-Atlantic upon completion of these performance objectives as specified in Section 5.

Background

Nomans Land Island is located about 3 miles from Martha’s Vineyard, MA. The Navy used the island as a practice bombing range from 1943 to 1996. The Navy conducted removal actions in 1998 to remove USTs, a fuel pipeline, surface scrap metal and ordnance debris. As part of ongoing O&M requirements, the Navy must conduct recurring MEC surface clearances to remove munitions items that have surfaced due to erosion and frost heave. Previous surface clearances have been conducted in 2003, 2008, 2014, and 2021. The property has been transferred to the USFWS for use as an unstaffed wildlife refuge and is closed to the public.

Nomans Land Island is not listed on the CERCLA National Priority List. Navy is the lead agency under CERCLA, with input from US Fish and Wildlife Service (USFWS) and Massachusetts Department of Environmental Protection (MassDEP). A CERCLA Record of Decision for the site was finalized in November 2021.

- WORK ELEMENTS

The Contractor shall furnish all labor, materials, tools, supplies, services, equipment, project management, permits, contract/procurement administration, and other appurtenances necessary to perform all work in accordance with the PWS. The Contractor shall conduct all work in accordance with all pertinent state, local and federal regulations and approved plans. Work shall also be consistent with the Record of Decision and the associated ARARs for the site.

The Contractor’s work areas and activities at the site shall be subject to inspection without announcement by the Navy, their representatives, as well as representatives from the U.S. Environmental Protection Agency (USEPA), the Massachusetts Department of Environmental Protection (MassDEP), and U.S. Fish and Wildlife Service (USFWS).

The Contractor’s proposal should be presented in a format that follows the general structure below, including the following work elements:

- Work Element 1 – Project Management and Administrative Support

- Work Element 2 – Project Plans

- Work Element 3 – Mobilization and Setup

- Work Element 4 – Fieldwork – Limited MEC Surface Clearance

- Work Element 5 – Transportation and Disposition of Munitions Debria

- Work Element 6 – Site Restoration and Sign Installation

- Work Element 7 – Demobilization

- Work Element 8 – Post Construction Deliverables

  - Work Element 1 - UST Testing and Inspections

This element includes personnel and resources for the management and control of project activities, such as scoping, planning, estimating, executing, tracking, controlling, analyzing, coordinating, and closure of the project.  The element also includes direct management of the project, as well as the support for administrative functions needed for successful project management.

The project manager is expected to focus on managing the performance of the CTO including technical and financial performance, interacting with the Navy project team, financial tracking, communicating with team members, preparing management reports, attending management meetings, coordinating contract issues with the project teams, and providing required project support functions.

The Contractor will submit a brief monthly progress report, in writing, detailing, at a minimum, what has been accomplished during the period since last reporting, what is expected to be accomplished during the next month, schedule, and costs incurred during the month.  The schedule shall include dates for deliverables and the sequence of tasks needed to reach the objectives for this SOW.  The Contractor shall follow the schedule dates reflected in Table A.  During fieldwork the Contractor will provide daily progress updates via email.

If there are delays in the project schedule or limited work during a month, the project manager will discuss with the Navy RPM to determine if a monthly progress report is necessary.  The Project Manager shall include project management hours for appropriate personnel in the proposal commensurate with the task to be prepared.  Project delays and/or an extension in the period of performance will not provide for an unforeseen condition that would require additional funding for project management.

  - Work Element 2 – Project Plans

2.2.1.	Explosive Safety Submission (ESS)

The Contractor shall prepare an ESS in accordance with NOSSA Instruction (NOSSAINST) 8020.15E as required for completion of the activities described in this PWS.

2.2.2	QAPP Development

The objective of the Quality Assurance Project Plan (QAPP) is to ensure that sufficient detail is provided to evaluate site conditions and provide for the safe and efficient completion of a limited detector-aided visual MEC surface clearance at Nomans Land Island. The surface clearance shall encompass beaches to mean low tide, roads, trails, and work areas that are easily accessible to trespassers. This includes approximately 50 acres of beaches and 14 acres or roads and trails. Roads and trails are unpaved and are periodically mowed by USFWS. These areas can be identified based on documents prepared for the previous clearance in 2022.

Documents will be provided in both hardcopy and on CD in accordance with the NAVFAC CERCLA Environmental Restoration Program Recordkeeping Manual. The Contractor shall prepare formal Response-to-Comments (RTCs) for Navy Chemist and regulator comments received on the draft QAPP. The Contractor shall provide the RTCs for Navy review prior to submittal.

The Contractor shall prepare and submit a Draft, Draft-Final and Final QAPP, with the required appendices. The Plan will define project objectives and associated data needs to reach project closeout and describe Data Quality Objectives. The Plan will follow the IDQTF QAPP template and describe the general methodology for performing the site MEC work, including at a minimum:

- Site preparation

- Location surveys and mapping

- Data Quality Objectives (DQOs)

- Description of anomaly avoidance procedures

- Details of the QC program

- Geographical Information Systems (GIS) and data management

Consistent with the requirements of the basic contract, the plan will identify, by name, key personnel responsible for data processing and quality control (QC) and will include a description of their experience and qualifications to perform the work assigned.

The Plan shall include a comprehensive description of the work to be performed and a schedule for completion of each major activity and submission of each deliverable. The submittals shall include all information necessary to enable the Navy to conduct field inspections and comparison to verification criteria.  The plan shall include, but not be limited to:

- Introduction

- Site conditions

- Regulatory Framework

- Pre-clearance Activities

- Site Work

- Report Requirements

Attachments to the Plan include the following:

Accident and Prevent Plan (APP)

The Contractor shall develop and submit an Accident Prevention Plan (APP), including an Activity Hazardous Analysis (AHA) as an appendix.  The APP with AHA shall be developed in the format provided in Appendix A of the EM 385-1-1, 15 Sep 2008 or latest edition with updates, US Army Corps of Engineers’ Safety and Health Requirements Manual and shall address each of the elements and sub/elements in the outline.

The APP shall provide a safe and healthful environment for all of the Contractor’s personnel and others that may be exposed to potentially hazardous conditions at the project site. Contractor shall address the minimum requirements and other areas of the EM 385-1-1 that are pertinent to the contract.  The Contractor shall certify to the NAVFAC NTR that the final APP has been reviewed with each Contractor’s and subcontractor’s employees prior to mobilization and start of fieldwork activities.

A draft and final APP shall be submitted according to the schedule in the Deliverable Schedule Matrix.  The APP shall be accessible to all site employees at all times during the project and a copy shall be mounted on or placed adjacent next to the Contractor’s safety and health bulletin board in an area commonly accessed by workers.  If a bulletin board is not practical to have at the site, make the APP available in a vehicle or designated spot utilized for work under this Task Order.

The draft APP with AHA, including copies of the Plans, must be submitted to the contracting officer or designated representative for review and acceptance.  The Contractor will work directly with the NAVFAC project manager or government designated authority to coordinate review, respond to comments, and obtain final acceptance of the APP.

# The Contractor shall forward a copy (with all corrections) of the final APP to the project manager prior to start of the work.

Schedule

A detailed schedule for completion of the work described in the SOW.  This schedule should account for any approvals associated with the removal of MEC.

  - Work Element 3 – Mobilization and Setup Activities

All fieldwork shall be executed in accordance with approved project plans.  The following general requirements are applicable to this task order:

2.3.1  	Special Use Permit: The Contractor shall obtain a Special Use Permit and any other approvals required by USFWS prior to initiating field activities. Although this action is being undertaken under CERCLA, the Special Use Permit is still required.

2.3.2  	Work Restrictions shall be adhered to by all Contractor personnel.  Contractor personnel shall become familiar with USFWS requirements, the O&M Plan and Coast Guard regulations.  Do not enter any environmentally sensitive areas unless required to do so and unless cleared for such entry.

2.3.3  	Temporary Construction Facilities:  The Contractor shall provide all temporary facilities required to support their activities including telephone, office facilities, power and portable toilets.  Water, communications and electricity are not available on the island. No other facilities or shelter are available.

2.3.4  	Mowing:  The USFWS mowing schedule may not coincide with this field effort. Therefore, the Contractor shall assume mowing will be required as part of mobilization.

2.3.5 	The following tasks should be included within this work element.

2.3.5.1  	Mobilization of Site Equipment/Materials. Mobilization of materials and equipment to support site activities

2.3.5.2  Staging Area Setup. The Contractor shall set up a staging area, decontamination area, and appropriate entrance and exits to the site.  Any area impacted by construction activities shall be restored to original condition.  The Contractor shall identify entrance and exit locations for the staging area.

2.3.5.3  	Explosives Acquisition and Management.  The Contractor will hold the appropriate BATFE license(s) for acquiring, storing and using explosives.  Additionally, the Contractor will follow all work plan and ESS procedures related to explosives management, use and operations.  All activities conducted in the area potentially containing MEC must have the appropriate MEC avoidance and/or construction support.

  - Work Element 4 – Fieldwork – Limited MEC Surface Clearance

All fieldwork shall be executed in accordance with the approved project plans and the approved ESS.  The following general requirements are applicable to this task order and supplement the Basic Contract requirements.

Perform a MEC surface removal in accordance with the approved Plans and adhere to explosives safety procedures in the approved ESS. Include the following:

- Conduct detector-aided survey

- Identify all MPPEH by type, nomenclature, and condition

- Document the location of all MPPEH via GPS

- Digitally photograph all MPPEH

- MPPEH will be inspected and classified as MDAS or MEC per the ESS

- MDAS and MEC will be dispositioned per the ESS

  - Work Element 5 – Transportation and Disposition of Munitions Debris

Material classified as MD and other cultural debris shall be recycled and all MD management, transportation, and recycling be documented in accordance with the final QAPP and approved ESS.

  - Work Element 6 – Site Restoration

Any areas impacted by work activities will be restored to their original condition.

During the most recent land use control inspection, it was noted that several of the required warning signs appear to be missing or damaged. A copy of the appropriate sections of that report is included as an attachment to this document.  In addition to the restoration, the Contractor will also procure and reinstall any of the missing or damaged warning signs at the Site. Assume that a total of ten signs will need to be installed as part of this work. The signs should measure 4’ by 8’ and shall be manufactured of a durable material to ensure they remain visible in the marine environment.

  - Work Element 7 – Demobilization

In preparation for demobilization, equipment decontamination and site clean-up will be conducted. All support equipment, staging areas, waste materials, and heavy equipment will be removed.  The Contractor shall demobilize all equipment, supplies, material and facilities following completion of activities and shipment of waste.

  - Work Element 8 – Post Construction Deliverables

2.8.1  	Waste Manifests/BOLs and load tickets:  Provide a copy of all load tickets for construction material and waste disposal in the contract closeout report.  Any waste manifests shall be coordinated with the RPM.

2.8.2  	Records management:  The Contractor shall submit all documentation in accordance with the Deliverable Schedule Matrix in Table A, Environmental Restoration Recordkeeping Program Manual, and Appendix G, NAVFAC Contractor Work Instruction.  Documentation shall include Administrative Record files, post decision files and site files.

2.8.3  	Daily Field Activities Form: The Contractor shall provide a Daily Field Activities Form at the end of each workday during field work implementation. Any discrepancies shall be noted on the Daily Field Activities Form.

2.8.4 	Project Completion Report:  The Contractor shall prepare a Project Completion Report documenting completion of activities in accordance with all approved plans.

2.8.5 	After-Action Report:  The Contractor shall prepare and submit an After-Action Report in accordance with NOSSA Instruction 8020.15E.

- SPECIAL REQUIREMENTS/CONDITIONS

  - The property has been transferred to USFWS for use as a non-staffed wildlife refuge. The Contractor shall comply will all directives from USFWS for protection of wildlife and resources. Depending on the timing of the field effort, it may be necessary to avoid certain nesting areas or other sensitive areas. Further information

will be provided from USFWS when the schedule is finalized. https://www.fws.gov/refuge/Great_Meadows/About_the_Complex.html

  - USFWS requires a Special Use Permit prior to work on the Island. The Contractor shall obtain this permit from U.S. Fish and Wildlife Service by contacting the POC identified in Appendix A of this Statement of Work.

  - All communications with parties outside of the Navy team shall be coordinated through the RPM, unless otherwise directed.

  - Public Affairs - The Contractor shall not disclose any information or data resulting from actions in this contract to the news media or public.  The Contractor shall refer all press or public contacts to the RPM.  The Contractor may not distribute reports or data to any source, unless specifically authorized by the RPM and the Public Affairs Officer in accordance with NAVFAC Instruction 5720.10A.  (Reference: NAVFAC 5720/6 Form - Publication Security Review and Clearance).

  - Any oral directives, instructions, explanations, commitments and/or acceptances given by any government employee to the Contractor or his personnel, shall not be construed by the Contractor as a change in scope to this Contract Task Order.  Any change in scope of work must be issued to the Contractor, in writing, by the Contracting Officer to be binding on the government.

  - The Contractor shall verbally brief the RPM at least once a week as to the progress of work, status of field operations, and percentage of report completion.

  - The contract number shown in the CTO award is the number assigned to the contract for this work and shall be used on all reports and correspondence relative to this contract.

  - The Contractor is responsible for preparing all Accident Prevention Plans in accordance with the United States Army Corp of Engineers (USACEs) EM 385-1-1 Safety and Health Requirements Manual.

- GOVERNMENT FURNISHED EQUIPMENT/DATA

The following documents shall be provided to the Contractor:

- Limited Munitions and Explosives of Concern Surface Clearance, Final Explosive Safety Submission, Nomans Land Island, Chilmark, MA, Tetra Tech, 7/16/14.

- Limited Munitions and Explosives of Concern Surface Clearance Work Plan, Nomans Land Island, Chilmark, MA, Tetra Tech, 8/21/14.

- Final After-Action Report, Limited Munitions and Explosives of Concern Surface Clearance, Nomans Land Island, Chilmark, MA, Tetra Tech, 11/13/15

- PERFORMANCE MEASUREMENT AND PAYMENT SUMMARY AND OFFER SCHEDULE

5.1  	The Contractor shall be responsible for achieving the performance objectives in this PWS and   successfully performing all the tasks required for successful performance.

5.2  	Payments shall be made to the Contractor upon completion of the following:

- Verification that the corresponding performance standards and Acceptable Quality Levels defined below have been satisfactorily achieved.

- Submission of a properly prepared invoice.  Invoices that fail to meet the requirements of this paragraph and/or the invoicing or prompt payment clauses of the contract may be rejected in their entirety.

5.2.1 	If a task associated with a work element is eliminated or not necessary for accomplishing project completion, the contract/task order shall be modified and the payment amount associated with that task shall not be paid to Contractor.

5.2.2 	Failure to demonstrate that the performance objectives are achieved and the work elements are complete may result in non-payment of the final work element and/or a negative evaluation.

5.3 	Performance measurements provided in the basic contract also apply to this SOW.

| Work Element / Task | Performance Standard | Acceptable Quality Level | Assessment Method | Performance Payment and Incentive |
| --- | --- | --- | --- | --- |
| Project Management/ Support/ Administration | Accurate and timely cost and schedule management and administrative support to keep the project on scope, schedule and budget. | 100% Navy acceptance | Review project progress reports. Navy acceptance by task order COR. | Award fee will vary according to performance. Incentive includes positive evaluations. Penalties include negative evaluations. |
| Project Meetings | Completion and distribution of all meeting materials including agenda, handouts, figures, schedule, PowerPoint presentation, and meeting minutes. Accurate and timely support, attendance and technical support at the meetings. | 100% Navy acceptance | Review of meeting minutes, deliverables, and customer feedback. Navy acceptance by task order COR. | Award fee will vary according to performance. Incentive includes positive evaluations. Penalties include negative evaluations. |
| Work Plans and Attachments | Navy acceptance of deliverables. This task includes the work plan, SAP, health and safety plan, accident prevention plan, dust control plan, traffic plan and waste management plan. Work Plan and all sections and appendices shall include sufficient information to implement the mobilization of equipment and personnel to construct the durable cap. The work plan must be clearly written, factually accurate and complete and have minimal transcription, typographical, and grammatical errors. The AHAs, and APPs must be accepted by BRAC PMO, NAVFAC, NMCPHC, and comply with all applicable codes, standards, and regulations (including the NCP). | 100% Navy acceptance 100% Regulatory acceptance | Review of deliverables for content, quality and adherence to applicable laws, guidance and regulations. Navy acceptance by task order COR. | Award fee will vary according to performance. Incentive includes positive evaluations. Penalties include negative evaluations. |
| Fieldwork | Complete fieldwork as specified in the Project Plans and other requirements of this PWS with no health and safety incidents. No significant violations of the dust monitoring causing work stoppage. Survey and sample data collection in a timely manner with no or very minimal schedule delays. Concurrence on meeting remedial action completion criteria from BRAC PMO. | 100% Navy Acceptance 100% Regulatory Acceptance | Announced and unannounced field inspections, schedule, check lists, Review of field logs. Navy acceptance by task order COR. | Award fee will vary according to performance. Incentive includes positive evaluations. Penalties include negative evaluations. |
| Reports | Document is organized, well written and can be comprehended by the general community. Document reviewed by an editor and includes persuasive discussions. Document includes for each site: - background summaries - data analysis - conclusions - recommendations | 100% Navy Acceptance 100% Regulatory Acceptance | Factually accurate and complete with no more than 2 major deficiencies (e.g. missing information) and 5 minor deficiencies (e.g. spelling, format, wrong date). Navy acceptance by task order COR. | Award fee will vary according to performance. Incentive includes positive evaluations. Penalties include negative evaluations. |

- REFERENCES

- Navy Environmental Restoration Program Manual (NERP)

- EM 385-1-1. (15Sep08) US Army Corps of Engineers Safety and Health Requirements

- NAVSEA OP-5, Vol. 1, Seventh Revision, “Ammunition and Explosives Ashore Safety Regulations for Handling, Storing, Production, Renovation and Shipping”

- DOD Explosives Safety Board (DDESB) Standard 6055.9-STD

- Federal Regulation 29 CFR 1910.120, Hazardous Waste Operations and Emergency Response (HAZWOPER)

- Military Munitions Rule [Federal Register: (Volume 62, Number 29)]

- Handbook on the Management of Munitions Response Actions, USEPA

- Management and Disposition of Material Potentially Presenting an Explosive Hazard (MPPEH), DODI 4140.62

- Naval Ordnance Safety and Security Activity (NOSSA).  NOSSAINST 8020.15E, Explosives Safety Review, Oversight, and Verification of Munitions Responses.