Specification for LHD/A Machinery Control Systems

1.0 INTRODUCTION

The Naval Surface Warfare Center Philadelphia Division (NSWCPD) is a Department of Defense entity responsible for research and development, test and evaluation, engineering and fleet support organization for the Navy’s ships, submarines, military watercraft and unmanned vehicles.  This requirement is for NSWCPD Code 515, which is responsible for The Landing Helicopter Dock and Landing Helicopter Assault (LHD/A) Machinery Control Systems (MCS) Land Based Test Environment (LBTE).

  - BACKGROUND

LHD/A MCS is currently experiencing obsolescence issues with respect to both their hardware/computer software configuration items (HWCI/CSCI) that will require a technology refresh and re-engineering to sustain shipboard tactical operations. The legacy design of the Gas Turbine Workstation (GTWS) includes a Woodward Micronet. These legacy Micronet units include subcomponents that have reached EOL and/or will not be able to meet functional requirements of the modernized MCS systems. These Micronet upgrade kits allow the MCS team to reuse much of the existing legacy Micronets, while replacing only the subcomponents that are EOL or unable to meet Tech Refresh system requirements.

1.2 SCOPE OF WORK

The Contractor shall provide hardware to meet or exceed the specifications found below.

2.0 APPLICABLE DOCUMENTS

- Navy Issuances

https://www.secnav.navy.mil/doni/default.aspx

- DOD Issuances

https://www.defense.gov/Resources/Forms-Directives-Instructions/

- NAVSEAINST Library

https://www.navsea.navy.mil/Resources/Instructions/

The Contractor shall reference and utilize the latest version available when performing tasks within this PWS.

3.0.  REQUIREMENTS

Contractor shall provide the following hardware and software components that meet or exceed the specifications defined in the table below.

| Part Number | Description | Qty |
| --- | --- | --- |
| 8928-5007 | SOFTWARE LICENSE - GAP MONITOR LICENSE | 11 |
| 8923-4001 kit | KIT - LHD8 ECU (LM2500+) OBSOLESCENCE UPGRADE KIT; Includes the line items below: | 3 |
| 1219-1078 | CAP - 35/15 DIN RAIL END (PHOENIX, 1206573) | 2 per kit |
| 1604-1055 | BLOCK - DIN RAIL END E/NS 35 N (PHOENIX, 0800886) | 2 per kit |
| 1604-801 | BLOCK - SECTIONAL TERMINAL UK 5, UK 5 N | 3 per kit |
| 1604-813 | BLOCK - SECTIONAL TERMINAL, GROUND (USLKG 5, PHOENIX, 0441504) | 1 per kit |
| 1606-588 | CHANNEL - MOUNTING, DRILLED (NS 35/15, PHOENIX, 1201730) | 0.3 meters per kit |
| 1635-2535 | CONNECTOR - CAN, NETWORK TERMINATOR, M12 FEMALE 120 OHM | 1 per kit |
| 1635-2548 | CONNECTOR - CAN, NETWORK TEE, M12, M/F BUS, F DROP | 1 per kit |
| 1649-072 | RESISTOR - 121 OHM .125W/.25W 1% METAL FILM EDS | 3 per kit |
| 1751-147 | MODULE - 4 CH. MPU WITH 1:3 TRANSFORMER | 1 per kit |
| 1751-4011 | JUMPER - ANALOG FTM SHIELDS TO GROUND (5437-523/-524/-525) | 1 per kit |
| 1751-4012 | JUMPER - FTM SHIELDS TO GROUND (5437-672) | 1 per kit |
| 1751-4013 | JUMPER - DATAFORTH SHIELDS TO GROUND (5441-695/-697) | 2 per kit |
| 1784-655 | CONVERTER - 100 OHM RTD TO 0-5 VOLT | 1 per kit |
| 1784-659 | CONVERTER - NON-ISOLATED PASS THROUGH WITH 200 OHM RESISTER | 19 per kit |
| 3799-301 | PANEL - NETCON IIIB VME BLANK | 2 per kit |
| 5417-031 | CABLE - NETCON LOW DENSITY ANALOG BLACK 16FT | 1 per kit |
| 5417-043 | CABLE - NETCON 16FT DISCRETE GRA W/BACKSHELL TAPE | 1 per kit |
| 5417-1510-0.5 | CABLE - CAN, MICRONET DROP, M8 FEMALE TO M12 MALE (0.5M) | 1 per kit |
| 5417-1513-1.0 | CABLE - CAN, THICK DROP, M12 MALE TO PIGTAIL (1.0M) | 1 per kit |
| 5417-177 | CABLE - MICRONET HIGH DENSITY ANALOG/DISCRETE GRA 18FT UL | 4 per kit |
| 5432-039 | DRIVER - LAYOUT OF SOLENOID DRIVER MODULE | 1 per kit |
| 5437-523 | MODULE - ANALOG FIELD TERM (STD) | 1 per kit |
| 5437-672 | MODULE - NETCON FIELD TERMINAL | 1 per kit |
| 5441-693 | MODULE - HD DISCRETE I/O FTM (24 IN/12 OUT) | 2 per kit |
| 5441-695 | MODULE - SIMPLEX FTM (DATAFORTH) | 2 per kit |
| 5450-1710-16FT | CABLE - RTCNET & LINKNET HT CAN CABLE WITH FERRULES - 16 FT | 1 per kit |
| 5450-1710-2FT | CABLE - RTCNET & LINKNET HT CAN CABLE WITH FERRULES - 2 FT | 3 per kit |
| 5450-1733-2FT | CABLE - RTCNET & LINKNET HT PWR CABLE WITH FERRULES - 2 FT | 4 per kit |
| 5453-829 | CHASSIS - MICRONET PLUS 8 SLOT, REDUND WITH SMART FANS | 1 per kit |
| 5466-1001 | MODULE - MICRONET-PLUS, 110 VAC/125 VDC INPUT, 2 SLOT POWER SUPPLY | 2 per kit |
| 5466-1050 | MODULE - MICRONET SIMPLEX 48/24 HDDIO SMART-PLUS | 1 per kit |
| 5466-1511 | MODULE - MICRONET PLUS P1020 CPU | 1 per kit |
| 5466-5000 | MODULE - MICRONET SPEED SENSOR, 4 CHANNEL, MPU SMART-PLUS | 1 per kit |
| 5466-5006 | MODULE - SIO MODULE, W/SCREW POSTS, SMART-PLUS | 1 per kit |
| 5466-5027 | MODULE - MICRONET SIMPLEX HIGH-DENSITY ANALOG I/O - DATAFORTH 24X 0-5 V INPUTS, 8X 4-20 MA OUTPUTS | 1 per kit |
| 5501-1432 | MODULE - 2CH ACTUATOR CONTROLLER (200MA) W/ FFL & WEIGHTED AVE | 1 per kit |
| 8200-1100 | MODULE - RTCNET RTD (8 CH) | 2 per kit |
| 8200-1151 | MODULE - RTCNET T/C HA (8 CH, FAIL HIGH) | 2 per kit |
| 8237-2090 | CONTROL - TPM ASSEMBLY GENERAL ELECTRIC | 1 per kit |
| 8239-047 | CONTROL - MPU AMPLIFIER | 1 per kit |
| 8923-1975 | KIT - RTCNET DUAL WIRE OPTION (POWER) TERMINAL BLOCKS WITH LABELS | 4 per kit |
| 8928-5014 | SOFTWARE LICENSE - CONTROL ASSISTANT LICENSE | 1 per kit |

4.0 Manufacturing Phase-Out or Discontinuation of Production, Diminishing Sources, and Obsolete Materials or Components

The contractor shall notify the contracting officer immediately upon determining the unavailability of obsolete materials or components. The contractor may recommend a solution to include the impact on the contract price and delivery. The contractor shall not initiate any item redesign or incur any additional costs without the express, written authorization of the contracting officer.

5.0  DATA REQUIREMENTS

- N/A

6.0 SECURITY REQUIREMENTS

6.0.1  The Contractor shall protect critical information associated with this contract to prevent unauthorized disclosure.  The NSWC Philadelphia Division's (NSWCPD) CIIL (Critical Indicators and information list) will be provided on site, if warranted. Performance under this contract requires the contractor to adhere to OPSEC requirements, which may include the NSWCPD Contractor OPSEC Plan. When the NSWCPD Contractor OPSEC Plan is required, the contractor shall return acknowledgment of the plan within 15 days of receipt. The Contractor may not impose OPSEC requirements on its subcontractors unless NSWCPD approves the OPSEC requirements.  During the period of this contract, the Contractor may be exposed to, use, or produce, NSWCPD Critical Information (CI) and/or observables and indicators which may lead to discovery of CI.  NSWCPD’s CI will not be distributed to unauthorized third parties, including foreign governments, or companies under Foreign Ownership, Control, or Influence (FOCI) unless such FOCI has been appropriately mitigated through measures approved by the Defense Counterintelligence and Security Agency.  Questions concerning these requirements shall be directed to the PCO.

6.0.2  CUI correspondence transmitted internally on the contractor's unclassified networks or information systems, and externally, shall be protected per NIST SP-800-171, Protecting Controlled Unclassified Information (CUI) in Non-federal Systems and Organizations.

Assembled large components/systems being transported to and from testing areas, other production or government facilities (whether or not on public roadways) shall be in an enclosed van trailer or covered flatbed trailer.  Component/System outside storage, staging, and test areas shall be shielded/obscured from public view wherever physically possible.

6.0.3  NSWCPD’s CI shall not be publicized in corporate wide newsletters, trade magazines, displays, intranet pages or public facing websites.  Media requests related to this project shall be directed to the PCO, and the COR who will forward the required to the NSWCPD Public Release Authority for review.

6.0.4  Any attempt by unauthorized third parties to solicit, obtain, photograph, or record, or; incidents of loss/compromise of government Classified or CI, Business Sensitive, Company Proprietary information related to this or other program must be immediately reported to the contractor's Facility Security Officer and Cognizant Security Office and/or the Naval Criminal Investigative Service, and the NSWC PD Security Division (Code 105).

6.1  RECEIPT, STORAGE, AND GENERATION OF CONTROLLED UNCLASSIFIED INFORMATION (CUI)

All Controlled Unclassified Information (CUI) associated with this contract must follow the minimum marking requirements of DoDI 5200.48, Section 3, paragraph 3.4.a, and include the acronym "CUI" in the banner and footer of the document. In accordance with DoDI 5200.48, CUI must be safeguarded to prevent Unauthorized Disclosure (UD). CUI export controlled technical information or other scientific, technical, and engineering information must be marked with an export control warning as directed in DoDI 5230.24, DoDD 5230.25, and Part 250 of Title 32, CFR. Nonfederal information systems storing and processing CUI shall be protected per NIST SP-800-171, or subsequent revisions. All transmissions to personal email accounts (AOL, Yahoo, Hotmail, Comcast, etc.) and posting on social media websites (Facebook, Instagram, Twitter, LinkedIn, etc.) are prohibited. Destroy CUI associated with this contract by any of the following approved methods: A cross-cut shredder; a certified commercial destruction vendor; a central destruction facility; incineration; chemical decomposition; pulverizing, disintegration; or methods approved for classified destruction.

7.0   PLACE OF PERFORMANCE

The contractors primary place of performance shall be at the contractor’s facility.

8.0  TRAVEL

N/A

9.0   GOVERNMENT FURNISHED PROPERTY

N/A

10.0   GOVERNMENT FURNISHED INFORMATION

N/A

11.0 PURCHASES

N/A

12.0 Counterfeit Material Prevention

The Contractor shall implement steps as defined in DFARS 252.246-7007 to minimize the risk of receiving counterfeit parts and materials. Parts and materials shall not be purchased from unauthorized suppliers (e.g. independent distributor or broker) unless there is no other means for procurement. In isolated cases when an unauthorized supplier is the only possible choice for procurement, a Technical Report – Study/Services, Unauthorized Supplier Purchase Report

(CDRL A002) shall be provided to the contracting officer and the program office. The report shall contain the following information:

- Reason why an authorized supplier or OM could not provide the part or material

- Product certificate of conformance with traceability to the OM, if available

- Verification and authenticity data results (e.g., visual inspection, marking/surface finish permanency, Differential Power Analysis (DPA), Scanning Acoustic Microscopy, Energy Dispersive X-Ray Spectroscopy, Fourier Transform Infrared (FTIR) Spectroscopy, Rockwell Hardness Test, etc.)

SAE AS5553 contains information regarding the detection, avoidance, and mitigation of counterfeit electronic components, and may be used as a reference document for meeting the above steps.

12.0 PERSONNEL

N/A

13.0 NSWCPD Electronic Cost Reporting and Financial Tracking (eCRAFT) System

N/A

14.0 SPECIAL REQUIREMENTS

N/A

15.0 DELIVERY REQUIREMENTS AND POC

All items received by NSWCPD must be centrally received at the following address:

NSWC Philadelphia Division

NSLC Detachment

1601 Langley Ave, Bldg. 542

Philadelphia, PA 19112

POC:

Mitch Soper

NSWCPD LHD/A MCS C515

215-897-2468

Robert.M.Soper7.civ@us.navy.mil