Iwakuni Japan Hazardous/Industrial Waste Disposal Contract

v.04/28/2026 (PKG)

INTRODUCTION

1.0 REMOVAL PHASES

1.1 PRE-REMOVAL

1.1.1 CONTRACT LINE-ITEM NUMBER (CLIN) SELECTION CRITERIA

1.1.1.1 CLIN DISPUTES

1.1.2 PERMIT/SPECIAL REQUIREMENTS

1.1.2.1 REPORTING REQUIREMENTS

1.1.3 SHIPPING/DISPOSAL DOCUMENTATION

1.1.4 NOTIFICATIONS

1.1.5 CONVEYANCE VEHICLE OR TRAILER SECURITY

1.1.6 HOURS OF OPERATION (TABLE VERIFICATION)

1.1.7 CONTRACT RELATED INFORMATION DISCLOSURE

1.1.8 DEFENSE BIOMETRIC IDENTIFICATION SYSTEM (DBIDS) INSTALLATION ACCESS

1.1.9 TSDF CLOSURES AND OUTAGES

1.1.10 CONTRACTOR PERSONNEL

1.1.11 ADDING CONTAINERIZED WASTE TO ISSUED TASK ORDERS

1.1.12 PACKAGING, MARKING, LABELING, AND PLACARDING

1.2 REMOVAL

1.2.1 SPILL RESPONSIBILITY

1.2.2 SEGREGATION OF HAZARDOUS WASTE

1.2.3 STATEMENT ON CONTAINERS

1.2.4 GOVERNMENT EQUIPMENT & PERSONNEL

1.2.5 TREATMENT OF HAZARDOUS WASTE ON GOVERNMENT         FACILITY

1.2.6 LOADING

1.2.7 WEIGHING OF ITEMS (TABLE VERIFICATION)

1.2.7.1 TASK ORDER (TO) ADJUSTMENTS

1.2.8 DEFINITIONS, BULK & CONTAINERIZED WASTE

1.2.8.1 BULK WASTE

1.2.8.2 CONTAINERIZED WASTE

1.2.9 BULK LIQUID PUMPING

1.2.10 BULKING & CONSOLIDATION

1.2.11 EXPEDITED REMOVAL/PERFORMANCE OF SERVICES

1.3 POST REMOVAL

1.3.1 RETURN OF SHIPPING DOCUMENTATION

2.0 DISPOSAL/RECYCLING

  - CERTIFICATES OF DISPOSAL, DESTRUCTION, & RECYCLING

  - JEGS/GOJ REGULATED, NON-REGULATED ITEMS

    - ITEMS REGULATED IAW JEGS/GOJ

    - ITEMS NOT REGULATED BY JEGS/GOJ

    - CONTRACTORS ELECTED DETAILED ANALYSIS AND SAMPLING

  - ASBESTOS MANAGEMENT

    - DISPERSAL (FRIABLE) ASBESTOS CONTAINING MATERIALS (ACM) BUILDING MAINTENANCE MATERIALS (SHIW)

    - NON-DISPERSAL (NON-FRIABLE) ACM (IW)

    - NON-DISPERSAL BUILDING MAINTENANCE MATERIALS ACM (IW)

2.4 AQUEOUS FILM-FORMING FOAM (AFFF) AND OTHER PER- AND POLYFLUOROALKYL SUBSTANCES (PFAS) RELATED WASTES

2.5 COMPRESSED GAS CYLINDERS, CARTRIDGES, VESSELS & CANISTERS

2.6 RECYCLING OR RECOVERY

2.6.1 COMMODITY SPECIFIC RECYCLING/RECOVERY REQUIREMENTS

2.7 DEMILITARIZATION

2.7.1 OTHER MILITARY ITEMS

3.0 SPECIAL SERVICES

3.1 INDUSTRIAL CLEANING SERVICES (ICS), DAILY, CREW OF 2

3.2 ICS, WEEKLY, CREW OF 2

3.3 ICS, MONTHLY, CREW OF 2

3.4 ICS, DAILY, CREW OF 3

3.5 ICS, WEEKLY, CREW OF 3

3.6 ICS, MONTHLY, CREW OF 3

3.7 SURCHARGE FOR WEEKEND WORK, DAILY, CREW OF 2

3.8 SURCHARGE FOR WEEKEND WORK, DAILY, CREW OF 3

3.9 ADDITIONAL CHARGE FOR HYDRO/WATER BLASTING WORK, DAILY, CREW OF 3

3.10 ADDITIONAL CHARGE FOR PERMIT-REQUIRED CONFINED SPACE WORK, DAILY, CREW OF 3

3.11 ADDITONAL CHARGE FOR SUPER VAC TRUCK WORK, DAILY, CREW OF 3

3.12 SURCHARGE FOR EXPEDITED COMMENCEMENT OF ICS WORK, CREW OF 2 or 3

3.13 PERFORM SAMPLING AND ANALYSIS

3.14 PREPARE HAZARDOUS WASTE PROFILE SHEET

3.15 PERFORM WASTE CHARACTERIZATION TO IDENTIFY WASTE

3.16 BULK NON-REGULATED WASTEWATER MANAGEMENT

3.17 PERFORM OVERPACKING/LOOSE PACKING SERVICES

4.0 ACCEPTABLE PERFORMANCE LEVEL (APL)

4.1 ACCEPTABLE PERFORMANCE LEVEL (APL) TABLE

5.0 QUALIFIED FACILITIES AND TRANSPORTERS LISTS (QFL/QTL)

5.1 ADDING/UPDATING QUALIFIED FACILITIES AND TRANSPORTERS

5.2 FACILITY REMOVAL

5.3 TRANSPORTER REMOVAL

6.0 ADDITIONAL REQUIREMENTS

6.1 INDEMNIFICATION

6.2 PLACES OF GOVERNMENT INSPECTION

6.3 CONTRACTOR REPRESENTATIVE

6.4 INCIDENTAL FEES

7.0 STANDARD TIMES FOR WASTE REMOVALS AND PERFORMANCE OF SERVICES

7.1 EXPEDITED REMOVALS AND PERFORMANCE OF SERVICES

8.0 DOCUMENTATION REQUIREMENTS, DLA DISPOSITION SERVICES ACCEPTANCE, INVOICING, AND TRACKING

8.1 SUBMISSION OF DOCUMENTATION TO FACILITATE ACCEPTANCE FOR PAYMENT

8.2 ACCEPTANCE AND INVOICING

9.0 CONTRACT MINIMUM/MAXIMUM

10.0 CONTRACTOR ACCESS TO DLA DISPOSITION SERVICES SITES

11.0 PERIOD OF PERFORMANCE

INTRODUCTION

    - This contract is referred to as the Iwakuni Japan Hazardous/Industrial Waste Disposal Contract. This contract covers the non-personal services and requires the removal, transportation, and disposal/recycling of Industrial Waste (IW) from military installations throughout Iwakuni, Japan as identified in the Japan Environmental Governing Standards (JEGS) and wastes regulated by Government of Japan (GOJ) as IW, Specially Controlled Industrial Waste (SCIW), Specially Handled Industrial Waste (SHIW), and listed within the Price Schedule. Also included in this contract are special services in support of the removal and disposal/recycling of hazardous and regulated substances. Infectious, controlled, explosive and radioactive wastes, or mixtures of such wastes with Hazardous Waste (HW), will not be included under this contract. Waste turned in under this contract may possibly be contaminated with one or more substances not specifically identified.

    - The removal, transportation, storage, characterization, treatment, disposal, and recycling or recovery of wastes offered under this contract are to be performed in accordance with (IAW) JEGS. Additionally, the Contractor shall perform work under this contract IAW the respective Japanese national, prefecture and local environmental laws and regulations, as well as any applicable international agreements governing the transportation of dangerous goods, safety, and health laws and regulations; international agreements governing the generation, storage, transportation and disposal/recycling of dangerous goods; and in conformance with industry standards that minimize risks to human.

    - The Contractor shall comply with any changes or new requirements identified by the above regulating bodies that should be promulgated during the performance of the contract. The latest version of Overseas Environmental Baseline Guidance Document (OEBGD) is available on the Department of War (DoW) Directives Website at: http://www.esd.whs.mil/DD/ and the JEGS available at the time of this Performance Work Statement (PWS) issuance for the Japan where services will be required for this contract.

    - The Contractor, unless specifically stated in this contract to the contrary, shall provide at its cost, without reimbursement, all supplies, transportation, equipment, and personnel necessary to perform the requirement of this PWS. This includes ensuring all vehicles transporting materials subject to this contract are properly placarded IAW applicable Japanese transportation laws. All Contractor personnel are required to have appropriate current and valid working permits and shall be qualified via training and experience, commensurate with the duties they will perform.

NOTE: The words container and drum are used interchangeably throughout the PWS.

    - The Contractor agrees to provide all services necessary for the final recycling, treatment, and/or disposal of the materials listed in the Price Schedule of this contract IAW the JEGS and respective national and local environmental laws and regulations, as well as any applicable international agreements governing the transportation of dangerous goods and the terms and conditions of this contract.  This includes ensuring all vehicles transporting materials subject to this contract are properly placarded IAW applicable Japan transportation laws and requirements.  These services shall include all necessary Contractor personnel and subcontractor (henceforth referred to as “joint venture partner”) personnel labor, sampling and analysis, transportation, packaging, equipment, and compilation and submission of all documentation pertaining to include inspecting, packaging, marking, labeling, loading, unloading, removal, transportation, disposal, and recycling of materials subject to this contract from DoW/GOJ installations and activities throughout Japan.

    - All materials/wastes subject to this contract must be identified and transported IAW GOJ transportation and safety standards and/or those indicated in the Globally Harmonized System (GHS).  All items turned in under this contract, regardless of condition, are being discarded by the United States Government (USG) as waste for disposal or recycling.

    - The standard performance time for the removal of all waste at all pickup locations is 20 business days from the date of Task Order (TO) issuance, except when expedited removal/service CLINs are ordered.  Refer to Section 1.2.11 for expedited removals/services.  The Contractor agrees that for those portions of the services provided on a USG installation, services will be provided during the core hours for the specified facility.  Typical core working hours are 0700-1700 hours, Monday through Friday and do not apply to U.S. Federal and Japanese Public holidays.  Hours of operation for specific installations and facilities may be obtained from the DLA Disposition Services Iwakuni. A schedule of U.S. Federal holidays and Japanese Public holidays is presented in Section 1.1.6.

    - This contract is a firm-fixed price indefinite delivery/indefinite quantity (IDIQ) contract written as a performance-based contract.

    - This contract consists of one (1) 30-month Base Period, followed by one (1) 30-month Option Period.

    - The six-character disposal Contract Line-Item Number (CLIN) consists of two parts.  The term “Base CLIN” refers to the first four characters of the CLIN.  The Base CLIN represents the waste description according to the waste profile for a particular waste stream.  The term “CLIN Suffix” refers to the last two characters of the CLIN.  The CLIN Suffix designates the unit of measure for purposes of tracking and Contractor payment.

    - CLIN pricing for specified waste streams as awarded and reflected in the contract Price Schedule shall be for the complete removal, transportation, and final disposal or recycling of the waste for which the CLIN was ordered on the Task Order.  Transportation shall include the removal of the waste from any pickup location in Attachment 3, including all U.S. DoW/GOJ military bases and activities, and delivered to the final treatment, storage, and disposal facility (TSDF) or recycling facility.  New pickup locations may be unilaterally added to the contract by the USG at any time during the life of the contract at no additional charges to the USG.  Disposal shall include all pre-disposal treatment, such as physical, chemical, or thermal treatment, and ultimate disposal of all wastes and treatment residues at a qualified TSDF as defined below.  Recycling shall consist of the partial or complete conversion of waste into reusable energy or materials.

    - Some CLINs ordered under this contract require the Contractor to provide a Certificate of Destruction or Disposal (CD), e.g., demilitarization-required items, Aqueous Film-Forming Foam (AFFF)/ Per- and Polyfluoroalkyl Substances (PFAS), etc.  The Contractor is cautioned to closely read all CLIN descriptions where such requirements are indicated.  Unless explicitly written elsewhere in this PWS, the Contractor must only provide a CD when so reflected in the Price Schedule CLIN description and IAW Section 2.1.

    - Publications and attachments applicable to this contract will be made available upon request.  During the solicitation and life of this contract, the USG may make requirement changes, additions, and/or deletions to those initially identified.

    - When turning in containerized waste for removal by the Contractor, Generators shall utilize containers meeting United Nations (UN) specifications, JEGS, and applicable GOJ transportation laws and regulations for the storage and transportation of HW/IW on and off the installation.  The Contractor shall ensure containers and packaging meet GOJ transportation laws and regulations prior to removal of IW from the USG installation or activity.

    - Minimum Task Order charges under this contract will be ¥ 240,000.00 except when minimum Task Order charges do not apply, i.e., those Task Orders in which the following services have been ordered: general support services, expedited removal services, expedited performance services, industrial cleaning services, sampling and analysis services, and repackaging/loose packing services. A surcharge for “Less Than Minimum Task Order” will be ordered by the Government on every TO not meeting or exceeding this amount.  CLIN 1093SV will have a firm-fixed price as shown in the Price Schedule of this contract.

    - Required time frames for Contractor performance of services including waste removal are expressed in business days.  For timekeeping purposes in the performance of services, the day in which the Task Order (TO) is issued to the Contractor will constitute “Day 0” (zero), and the following business day will constitute “Day 1”.  All business days thereafter, whether in whole or in part, will constitute one (1) day for timekeeping purposes.  A business day will be defined as any day on which normal business is conducted.  It does not include weekends, U.S. federal holidays, GOJ public holidays, or other days in which the Contractor is denied installation access due to force protection or other conditions.

    - For purposes of this Contract, the term Treatment, Storage, and Disposal Facility (TSDF) refers to a commercial waste management facility and/or commercial recycling facility listed on the DLA Qualified Facility List.  Per JEGS Volume V: Waste, Volume V.A.: Definitions, a Hazardous Waste Storage Area (HWSA) refers to any location on a DoW installation where hazardous waste is collected before shipment for treatment or disposal. An HWSA may store more than the equivalent of a 208-liter [55-gallon] drum of hazardous waste, or a 1.0 liter [1.0-quart] container of an acute hazardous waste, from each waste stream.

    - The Contracting Officer's Representative (COR) is a USG representative designated by the Contracting Officer (KO) and is authorized in writing to perform specific technical or administrative functions.  The Contractor will receive a copy of the written COR designation.  It will specify the extent of the COR's authority to act on behalf of the KO.  The COR is not authorized to make any commitments or changes that will affect price, quality, quantity, delivery, or any other term or condition of the contract.

1.0 REMOVAL PHASES

The requirements for removal of waste under this contract are divided into three main phases: Pre-Removal (1.1), Removal (1.2), and Post-Removal (1.3).  The Contractor and their subcontractors must always perform in a prudent, conscientious, safe, and professional manner.  The Contractor shall ensure that its agents, employees, and subcontractors involved in handling and packaging the hazardous waste be trained for the level of expertise required for the proper performance of the task and in the areas of chemical incompatibility, general first aid procedures, and spills.  The Contractor shall provide all Material Handling Equipment (MHE) and Personal Protective Equipment (PPE) unless otherwise stated.  The MHE and PPE must be appropriate to safely handle hazardous waste to include meeting any additional military installation specific requirements.  The Contractor agrees that all personnel and equipment used in the performance of this contract are subject to safety inspections by Government personnel while on federal property.

  - PRE-REMOVAL

The Contractor has the option to perform a pre-removal inspection of all waste containers at least five (5) business days prior to the day of the removal.  Any discrepancies associated with the marking and/or labeling of containers as described in Section. 1.2.3, shall be corrected during the pre-removal inspection.  If not corrected, discrepancies shall be reported to the COR for resolution. See Section 1.1.1.1 for CLIN disputes.

1.1.1 CONTRACT LINE-ITEM NUMBER (CLIN) SELECTION CRITERIA

CLINs are assigned to waste streams by the Government and are designed around Resource Conservation and Recovery Act (RCRA) regulations, treatment, and disposal technology standards.  DOT regulations, including Proper Shipping Name (PSN) and Hazard Class are not a basis for CLIN selection.  The waste description, generation process, and physical state are the primary considerations for CLIN selection.  CLIN selection begins with the identification of a Base CLIN category and sequence number.  It is selected, if available, from the Price Schedule, and is completed with the assignment of a CLIN Suffix.

a. Base CLIN categories and their descriptions used for waste disposal CLINs under this contract are as follows:

(1) Batteries (BA) – Used for all battery types, some, or all of which may be turned in as Industrial Waste (IW) where no waste codes are assigned, or as Specially Controlled Industrial Waste (SCIW), where applicable waste codes are assigned.

(2) Corrosives (CS) – Used for all corrosive wastes meeting the JEGS/GOJ definition of corrosive and carrying the Waste Code D002, J002, or unless it is a Lab Pack/Loose Pack, or unless the waste is concurrently assigned a D001, J001 (ignitable) or D003 (reactive) waste code.  Some corrosive acid oxidizers may carry the D001 or J001 waste code and be assigned to a CS CLIN.

(3) Compressed Gas Cylinders/Cartridges/Vessels (CY) – Used for all types of compressed gas cylinders, vessels, canisters, and cartridges.  Base CLINs are assigned based on the specific chemical name of the gas or gases comprising the contents of the cylinder, vessel, or canister.  Cartridges are assigned Base CLINs based on whether the gas is flammable or inert.

(4) Flammable/Combustible/Ignitable (FL) – Used for all ignitable and other than reactive or corrosive oxidizers carrying the D001, J001 waste code, unless it is a Lab Pack or Loose Pack or unless the waste is concurrently assigned a D003 (reactive) Waste Code.  FL CLINs may also be assigned to waste streams not carrying the D001 and J001 waste code but containing flammable/combustible/ignitable constituents such as organic solvents or chemicals.

(5) Lab Packs/Loose Packs (LP) – Used for most overpacked small containers of chemicals and consumer commodity-type products.  “Lab Pack” refers predominantly to the overpacking of multiple containers of like laboratory chemicals into a single outer container.  The chemicals must be of the same JEGS/GOJ hazard class, and a detailed inventory must be affixed to the outer container.  Lab packing is a packaging method performed by the Contractor (or subcontractor) under a separate service CLIN.  “Loose Pack” refers predominantly to the overpacking of multiple containers of like consumer commodity products (e.g., paints, lubricants, expired products, etc.) into a single outer container and does not require the preparation of a detailed inventory.  Generators of this waste may be authorized to loose-pack containers for turn-in.  Lab-packed and loose-packed drums and boxes containing smaller containers of same type of waste provide the best alternative when consolidation is impractical.  For purposes of disposal, Lab Packs and Loose Packs share the same Base CLIN depending upon the contents of the inner containers and their assigned EPA, JEGS/GOJ Waste Code(s) when applicable.

(6) Non-Regulated (NR) – Used for wastes that are not deemed hazardous under the JEGS and GOJ regulations and carry no JEGS/GOJ Waste Code(s).  Some such waste may be regulated and may carry specific Waste Code(s).  Managed by the Contractor as nonhazardous or Industrial Waste (IW), as applicable, upon removal from the installation and where applicable GOJ waste codes are assigned.

(7) PFAS-Related (PF) – Used for various wastes containing or contaminated by a source of per- and PFAS including some AFFF wastes.  Managed on USG installations as a toxic substance under US EPA laws and regulations.  Managed by the Contractor as Industrial Waste (IW) as applicable, upon removal from the installation where applicable GOJ Waste Codes are assigned.

(8) Recycle/Recovery (RE) – Used for various wastes that require by contract and/or regulation the recycling/recovery of constituents and/or components contained in the waste stream, including lamps, high mercury devices and residues, antifreeze, used oil, oil filters, and fully drained fuel filters.  When “RE” CLINs are ordered, the Contractor is required to recycle/recover the waste under which the CLIN was ordered.

(9) Reactive (RX) – Used for various reactive wastes bearing the D003 Waste Code or potentially bearing reactive characteristics.  All wastes bearing the D003 Waste Code will be turned in for disposal using an “RX” or appropriate “LP” Base CLIN, except for waste also contaminated by a source of PCBs that is equal to or greater than 0.5 ppm.

(10) Toxic (TX) – Used for various toxic wastes that are not ignitable, corrosive, or reactive waste bearing the D001, J001, D002, J002, or D003 Waste Codes, respectively, apart from hexavalent chromium (Cr+6) related waste bearing the D001 Waste Code exclusively due to the oxidizing properties of Cr+6.

(11) GOJ Regulated (RG) – Used for Government of Japan wastes of friable asbestos, non-friable asbestos, and petroleum contaminated soils with heavy metals as described in the price schedule.

b. Base CLIN selection criteria is further based upon information found in the waste profile sheet to include the waste description, physical state of the waste, and the presence of Waste Code(s) assigned (only to regulated HW). The USG will ensure that all wastes being turned in for removal will carry applicable Waste Code(s). The USG is responsible for identifying their waste and assigning proper profile-based CLINs to every waste stream being turned-in for removal.

A Base CLIN is generally authorized for use by the Government when the subject waste meets the waste description, physical state, as specified for that CLIN in the contract. Base CLINs will be selected using the following Waste Codes and other descriptions as the order of precedence:

(1) PCB-Related, Containing or Contaminated with - Not applicable to this Contract.

(2) Reactive (D003)

(3) Ignitable/Oxidizer (D001, J001), including flammable/combustible waste, other than Cr+6 related waste bearing the D001, J001 Waste Code, in which a corrosive CLIN is applied.

(4) Corrosive (D002, J002)

(5) Toxic (D004-D043, J004-J030)

(6) Acutely Hazardous (P-Listed)

(7) Leachate (F039)

(8) Electroplating-Related (F006-F012, F019)

(9) Spent Solvent (F001-F005)

(10) Other F-Listed

(11) U-Listed

(12) K-Listed

(13) AFFF/PFAS-Related Wastes

(14) JEGS/GOJ-Regulated Waste (SCIW/SHIW)

(15) JEGS/GOJ Non-Regulated Waste (IW)

c. Base CLIN sequence number – Refer to the Master Disposal CLIN Catalog.

d. CLIN Suffix – Refer to the following CLIN Suffix Table:

| CLIN Suffix Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Containerized Wastes |  |  |  |  |
| CLIN Suffix→ | C1 | C2 | C3 | C4 |
| Container Size: Type: | Up to 5 Gal (DM/DF) | Up to 15 Gal (DM/DF) | Up to 30 Gal (DM/DF) | Up to 55 Gal (DM/DF) |
| Unit of Issue: | EA | EA | EA | EA |
| CLIN Suffix→ | C5 | C6 | C7 | C8 |
| Container Size: Type: | Up to 110 Gal (DM/DF) | Pallet, CYB, or Tri-Wall Box | Containerized Pounds | Containerized Kilograms |
| Unit of Issue: | EA | EA | LB | KG |

| Pumpable Liquid Wastes Removed in Totes / IBCs |  |  |  |
| --- | --- | --- | --- |
| CLIN Suffix→ | T1 | T2 | T3 |
| Container Size: Type: | Up to 220 GL 850 Liters (TP) | Up to 275 GL 1,050 Liters (TP) | Up to 330 GL 1,250 Liters (TP) |
| Unit of Issue: | EA | EA | EA |

| Bulk Solid & Liquid Wastes |  |  |  |  |
| --- | --- | --- | --- | --- |
| CLIN Suffix→ | B1 | B2 | B3 | B4 |
| Bulk Load Description | Bulk Heavy Solids (>600 Lbs./Yd3) Priced per Lb., Based on Bulk Loads that are… |  |  |  |
|  | <10,000 Lbs. | 10K-20K Lbs. | 20K-30K Lbs. | >30,000 Lbs. |
| Unit of Issue: | (Pounds) LB | LB | LB | LB |
| Notes: | Weights shown above are net weights of actual waste per bulk load. Not to be used for lightweight solids weighing less than 600 Lbs/yd3. |  |  |  |
| CLIN Suffix→ | B5 | B6 | RESERVED | RESERVED |
| Bulk Load Description: | Bulk Pumpable Liquids Removed via Vac Truck or Tanker | Bulk Solids Lightweight (<600 Lbs/Yd3) Remove via ROB |  |  |
| Unit of Issue: | Gallons (GL) | Cubic Yards (CD) |  |  |
| Minimum Load Ordered: | 1,500-Gallons | 75% of ROB Capacity |  |  |

| Compressed Gas Cylinders (Sizes do not include valve or handle.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| CLIN Suffix→ | ME | M1 | M2 | M3 |
| Size Category: | Lecture | Small | Medium | Large |
| Size Up To: | 3" x 12" | 4" x 24" | 12" x 36" | 16" x 56" |
| Unit of Issue: | EA | EA | EA | EA |
| CLIN Suffix ME (Miscellaneous, Each) also has applications for other than compressed gases. |  |  |  |  |

e. Recycle/Recovery Base CLINs beginning with “RE” are designated for mandatory recycling/recovery by the USG. If the Contractor asserts that a waste designated for recycling cannot be recycled because it does not meet the required recycling parameters:

(1) The Contractor must notify the KO in writing of the rationale and proof for waste rejection at least five (5) business days before pickup. This must include written input from at least two recycling facilities listed on the Qualified Facilities List (QFL), at least one of which is not owned by the Contractor.

(2) The KO will notify the Contracting Officer Representative (COR) to resolve requests to change the assigned mandatory recycling CLIN to the appropriate disposal CLIN.  The final decision to change a mandatory recycling CLIN to a disposal CLIN is solely that of the Government.  If the Government agrees with the CLIN change, the applicable disposal CLIN will be assigned.

1.1.1.1 CLIN DISPUTES

a. CLIN discrepancies noted on a TO during a pre-removal inspection and/or during waste removal by the Contractor will be addressed immediately via the COR and Generator.  If the COR, Generator, and KO concur with the CLIN change request, TO Modification will be issued.

b. The Government may unilaterally issue TO modifications for such reasons as quantity changes, noun description changes, CLIN changes, adding/deleting TO lines, extending TO end dates, etc.  The Contractor will be paid for the actual items and quantities removed per the awarded contract price schedule.  If upon receipt of a unilateral TO modification, the Contractor does not agree with the change(s), the Contractor reserves the right to seek resolution with the KO.  These modifications are only against TOs.  No changes are made to the terms and conditions of the contract.

c. If the Contractor asserts after removal that an inappropriate CLIN has been assigned, the burden of proof lies with the Contractor.  At the Contractor’s expense, electronic documentation must be submitted within 10 business days to the KO and COR following the TSDF receipt date supporting the assertion. Requests for a CLIN change without supporting documentation will not be accepted.  After review of the documentation, the Contractor will be notified by the KO of the Government’s final determination. Requests for a CLIN change without supporting documents will not be accepted.  After review of the documentation, the Contractor will be notified by the KO of the Government’s final determination.

d. The Contractor must ensure that the waste in question is not treated or disposed of until the Government has made a final CLIN determination.  If the subject waste has been treated and/or disposed of prior to the Government’s final determination, the Contractor will be deemed to have waived its objection to the Government’s CLIN assignment, and the disposal or service must be invoiced based on the CLIN designation on the TO.  Failure to timely dispute a CLIN designation on the TO IAW this provision, when the basis of the dispute was or should have been known to the Contractor, is itself a breach of the contract’s terms and cannot form the basis for a later claim under the Disputes clause of the contract.

e. If the CLIN dispute cannot be resolved to the satisfaction of the parties, the Contracting Officer will treat the dispute as a Contractor claim involving the interpretation of contract provisions applicable to the assignment of CLINs.  The Contractor shall be provided the opportunity to provide additional information or evidence, and the Contracting Officer shall issue a final decision within 60 days, or such other reasonable time as may be appropriate under the Contract Disputes Act.

1.1.2 PERMIT/SPECIAL REQUIREMENTS

      - The Contractor shall, without additional expense to the USG, be responsible for paying all fees, preparing, or obtaining any necessary licenses, permits, customs notifications (if applicable), other notifications, waste profiles, and/or reports, which result from a Contractor's transportation, recycling, or disposal decision.  This includes acquiring any required permits or registrations necessary to operate on any installation.  This includes signing (on behalf of the USG) waste profiles required by Treatment, Storage, and Disposal Facilities (TSDF), which includes recycling facilities.  All Contractor signed waste profile sheets shall be written for the actual waste stream using the Generator's profile for backup documentation.  The Contractor shall be responsible for all damages to persons or properties that occur because of Contractor negligence.

      - All waste turned in and removed under this contract shall be disposed of or recycled only at DLA approved TSDFs located in the GOJ.  Only newly approved TSDFs and those TSDFs already appearing on the DLA Qualified Facilities List (QFL) are considered approved for usage by the Contractor.  This applies to ALL wastes, whether hazardous, non-hazardous, designated, and non-designated.  The Contractor may submit proposals for additional TSDFs per Section 5.1.  DLA is under no obligation to approve additional facilities but will do so if/when proper credentials and documents are provided, and the facility is deemed qualified.  All disposal, storage, transportation, chemical analyses, and additional authorization costs required for the delivery of wastes/materials to facilities added to this contract, afterward, will be prepared and performed by the Contractor at no additional charge to USG.  The current lists of qualified facilities and transporters list may be accessed at:  http://www.dla.mil/DispositionServices/Offers/Disposal/HazardousWaste/HazWasteDisposal.aspx

      - The Contractor, their employees, agents, or any representative may be required to attend special training, seminars, instructions, classes, safety orientations, etc., provided by the USG or to provide information to perform work or gain access to the installation.  Example: pass and ID requirements, antiterrorist training, Environmental Management System (EMS) policies, range safety briefs, and USG inspections of Contractor cranes and/or forklift training.

d. Any incidental local fees, taxes, or penalties assessed against the USG arising from the Contractor's decisions in the performance of this contract will be paid by the Contractor, including those costs levied because of the Contractor's failure to prepare and provide reports and/or documents as required under the performance of this contract.

1.1.2.1 REPORTING REQUIREMENTS

a. The Contactor shall provide an annual report summarizing hazardous waste movements from point of generation to disposal facility. The summary shall be utilized as the tracking log. This report shall be in English and include the installation pickup name, waste description, and shall include the method of disposal (i.e., incineration, landfill, energy recovery, or recycling, etc.). The report shall be provided in Excel format no later than 15 January of each year, or the next business day should the 15th fall on a weekend, through the last TO issued under the contract and shall be provided to the KO and the competent authorities upon request. The Invoice, DLA Form 2505 may be utilized for format reference, however, it is not required the Contractor use this exact form. The tracking log should be kept current so that it may be supplied to the Generator upon request by COR/KO and must include:

1. Company Name and address

2. Contract Number

3. Invoice amount

4. Task Order Number

5. Task Order Line-item number

6. Document number (HMMS).

7. HIN/CLIN

8. Pickup Manifest Number

9. Disposal Facility and any other facilities utilized for storage, disposal, and recycling if different than a. above.

10. Quantity picked up

11. Applicable waste codes

12. Final treatment/and or disposal method description including recycling. Handling or treatment codes if applicable for Japan.

13. Quantity disposed must match quantity picked up

14. Date of final treatment, and/or disposal, and/ or recycling.

b. The content of this Summary Manifest Report will be IAW the regulations of the Japanese prefecture requiring the report.

1.1.3 SHIPPING/DISPOSAL DOCUMENTATION

a. All references to hazardous waste manifest in this contract refers to the Japanese Industrial Waste Manifest (IWM). Industrial Waste Manifests will be provided and prepared by the Contractor and will be signed by the Generator with a printed name IAW Japan Laws. DLA Disposition Services COR(s) will not co-sign any manifest(s). The Contractor shall provide to the DLA Disposition Services Iwakuni COR coordinating the waste disposal with completed copies of the shipping papers for review at least five (5) business days prior to removal. The Contractor shall also prepare all Hazardous Waste Profile Sheets (HWPS) required for acceptance of waste into a DLA Qualified Disposal Facility (QFL) and any other required shipping documents.

b. Each pickup manifest (i.e., with a Government activity identified as a Generator), as well as all other documentation required herein, shall be clearly and distinctly marked with the name DLA Disposition Services Iwakuni in addition to the Generators and the contract and task order number, as applicable. The Contractor is required to provide Emergency response information and 24-hour emergency phone numbers shall be listed on the manifest. If blocks for emergency information are not provided, this information shall be placed in the upper right-hand corner of each document.

c. The Contractor shall complete a DD Form 1155 Order for Supplies or Services Form for all items ordered or services performed on a task order (TO). DD Form 1155 latest version available on the DLA Official Forms website at https://www.dla.mil/Forms/. The Contractor shall record the type of waste codes, quantities removed, unit of measurement, manifest numbers, and date removed. The Contractor shall obtain the COR’s signature and date to verify the wastes removed.

d. Once the waste has been removed from the installation, the Contractor shall provide the COR with a copy of the signed and dated manifest(s) no later than 10 calendar days following the acceptance of the waste by the initial and all TSDFs as applicable. The documents must reference the applicable contract number, task order number, and the manifest number.

e. Mock EPA identification (ID) numbers will be created by DLA Disposition Services for any TSDF, or transporter located outside of United States territories (OCONUS) when added to the Qualified Facility List (QFL) or Qualified Transporter List (QTL). The use of the mock EPA ID number is required on all applicable shipping documentation/waste manifests. Mock EPA ID numbers for TSDFs and transporters will be used on DLA Form 2505 when submitted to DLA for invoicing purposes.

f. The Contractor shall complete and provide the COR, DLA Form 2505 Invoice, annotating all wastes removed after each removal. The Contractor must describe and document in detail any differences between the actual quantities picked up and the actual quantity disposed/ recycled. The Contractor may use attachments to the form if necessary. The Contract Manager or Alternate Contract Manager shall sign the form certifying its accuracy and completeness. The Contractor will use the most up-to-date official form.

1.1.4 NOTIFICATIONS

a. Unless otherwise specified in this contract, the Contractor shall notify the COR and/or other designated Government representative at least five (5) business days BEFORE attempting installation access to conduct business related to performance under this contract.

(1) In addition to the above notification requirement the Contractor shall provide:

(a) A weekly schedule shall be communicated via email to the COR no later than

Thursday of the preceding week

(b) Transporter company name

(c) Driver’s name

(d) Driver’s Commercial Driver License (CDL) number, HAZMAT endorsement                                                      and expiration date

(e) Contractor Personnel badge number and expiration date

(f) Truck badge number and expiration date, where applicable, and

(g) Other information as required by the Security office

b. The Contractor may proactively provide the COR with a list of qualified drivers designated to perform removals under this contract, along with the above information.  However, for proper Security notification, the Contractor must notify the COR of the specific driver scheduled to arrive for each removal effort.

c. If a driver other than the one scheduled arrives to perform a removal, the COR or other designated Government representative will verify the personnel change with the Contractor before the removal commences.  This will require written notice from the Contractor and may result in the driver’s late arrival to the pickup location (See 1.2.6).

d. For TOs containing expedited removal or other performance CLINs, the Contractor shall notify the COR or other Government representative for each location in the following timeframes BEFORE attempting installation access for any reason:

| CLINs | DESCRIPTION | REQUIRED NOTIFICATION TIMES |
| --- | --- | --- |
| 1095SV | 1-Business Day Expedited Removal/Performance | One Hour Prior to Removal/Performance |
| 1096SV | 3-Business Day Expedited Removal/Performance | One (1) Business Day Prior to Removal/Performance |
| 1097SV | 5-Business Day Expedited Removal/Performance | Four (4) Business Days Prior to Removal/Performance |
| 1098SV | 10-Business Day Expedited Removal/Performance | Five (5) Business Days Prior to Removal/Performance |
| 1099SV | Expedited Commencement of ICS Work, Crew of 2 or 3 | Based on day of week TO is issued. (Refer to Section 3.22) |

See Section 1.2.11 for additional information on expedited service CLINs.

e. Notification requirements for an excusable delay request.  If at any time following the issuance of a written TO or anytime during the performance period, the Contractor determines standard removal times for any waste streams cannot be met due to unforeseen issues at the TSDF(s) beyond the Contractor’s control, the Contractor shall notify the KO and COR in writing via email by close of business (COB) the following business day, that a delay in removals under the subject TO will ensue.  Written notifications shall include the subject TO number, reason(s) for the delay, substantiable documentation from TSDF(s), and the estimated date/time of performance completion.  Substantiable documentation shall include written evidence showing earnest attempts were made by the Contractor to utilize at least two alternatives qualified TSDF needed to meet the performance requirements in the contract.  Failure by the Contractor to make a proper notification to the KO, as needed to render an informed decision regarding the Contractor’s asserted inability to perform timely due to factors beyond their control, shall constitute a waiver by the Contractor to exercise its excusable delay request.

(1) Following proper notification, if the KO determines the Contractor has met the burden of proof to demonstrate the delay is beyond the Contractor’s control, the following conditions apply:

(a) The Government reserves the right to issue a new (standalone) TO for impacted TO line(s) or modify and re-issue a previously issued TO.  Minimum TO charges will not apply to these TOs.

(b) The Contractor will provide biweekly TO updates to the KO and COR beginning 30 days after the new or modified TO is issued.  Updates shall include the status of the TO, the estimated time/date for removal, and further substantiable documentation that will provide the KO information to make an informed decision whether the delay remains beyond the control of the Contractor.  The requirement for TO updates will terminate when the Contractor’s requirements under the TO have been met.

(c) The Contractor will immediately notify the KO and COR immediately upon determining that meeting standard removal times will resume.

(d) If a delay situation arises after issuance of a TO having a combination of impacted and non-impacted wastes, the KO, COR, and Contractor will mutually agree upon how best to address the situation with follow-on TO issuances and execute a contract modification memorializing said agreement.

1.1.5 CONVEYANCE VEHICLE OR TRAILER SECURITY

a. The Government will notify the Contractor when a conveyance vehicle, trailer, or roll off box (ROB) must be locked and/or sealed due to Government policy or preference.  The Contractor will provide the lock and, if needed, the Government will provide the seal.  The Contractor will in all cases comply with state and federal laws, including DOT regulations as they pertain to conveyance vehicle and trailer security.

b. The Government has the right to request identification and/or endorsement(s) from the driver, even beyond what is identified in this contract.  The Government further has the right to refuse installation access and to stop Contractor/Subcontractor work of any kind, at any time, for any reason if/when the Government believes that a security risk may exist.  Any demurrage resulting from the driver verification process will not be grounds for Government reimbursement to the Contractor.

1.1.6 HOURS OF OPERATION

a. The service to be performed under this contract shall be performed during normal Government working days, as applicable for the site(s) on the TO, with specific office hours available from the COR, except for Federal holidays.  Work that cannot be completed by close-of-business for the TO site(s) shall be discontinued and resumed the next business day, when approved by the KO.  Services will not be scheduled to be performed on regularly scheduled Federal holidays (see below), or one designated by Presidential proclamation.

| HOLIDAY | DATE |
| --- | --- |
| New Year’s Day | 1 January* |
| Martin Luther King's Birthday | 3rd Monday in January |
| Presidents Day | 3rd Monday in February |
| Memorial Day | Last Monday in May |
| Juneteenth | 19 June* |
| Independence Day | 4 July* |
| Labor Day | 1st Monday in September |
| Columbus Day | 2nd Monday in October |
| Veterans Day | 11 November* |
| Thanksgiving Day | 4th Thursday in November |
| Christmas Day | 25 December* |

*The holiday is observed on the preceding Friday or the following Monday when the holiday occurs on Saturday or Sunday, respectively.

Services will not be scheduled to be performed on a regular Japanese holiday or one designated by the Government of Japan:

| HOLIDAY | DATE |
| --- | --- |
| New Year’s Holidays | January 1-3 |
| Coming of Age Day | January 8 (second Monday of January) |
| Foundation Day | February 12* |
| The Emperor’s Birthday | February 23 |
| Vernal Equinox Day | March 21 |
| Shōwa Day | April 30* |
| Constitution Memorial Day | May 3 |
| Greenery Day | May 4 |
| Children’s Day | May 5 |
| Marine Day | July 16 (third Monday in July) |
| Mountain Day | August 11 |
| Respect for the Aged Day | September 17 (third Monday of September) |
| Autumnal Equinox Day | September 24* |
| Health and Sports Day | October 8 (second Monday of October) |
| Culture Day | November 3 |
| Labor Thanksgiving Day | November 23 |
| Year End Holidays | December 29-31 |
| *The holiday is observed on the preceding Friday or the following Monday when the holiday occurs on Saturday, respectively. |  |

b. Installation specific normal hours of operations for this contract are available upon request from the COR.  This contract is for various military and Government activities of which may have different work schedules.  Excluding weekends and observed Federal holidays, the Contractor agrees that, for those portions of the services provided on a government installation, the services will be provided during the normal hours of operation for the installation.

c. The generator will make every attempt to ensure that contractor access is arranged for scheduled removals. If the contractor finds that access to sites as requested on the TO are continually in conflict with available scheduling, the contractor may request through the COR a written valid reason from the generator for not being able to perform the service. The COR will elevate the issues to site leadership, the DSD and/or the KO as required.

1.1.7 CONTRACT RELATED INFORMATION DISCLOSURE

The Contractor may not publicly release or share any data, reports, or other information relating to the contract with other firms or individuals who are not performing as subcontractors under the contract with a need to know and must keep such information confidential.  Other than waste disposal of data or other information that is required pursuant to laws and regulations implemented by regulatory authorities and applicable to all businesses, the release of any information pertaining to this contract is conditioned on the Contracting Officer providing approval in writing prior to release.  The contractor shall immediately notify the Contracting Officer upon receipt of any request for information from any entity, including regulatory authorities or the Press, to include any order to disclose information received from any competent judicial or administrative authority (e.g., federal, state, local or host nation officials or entities in a foreign country).  Disposition Services will respond promptly to advise the contractor regarding whether it authorizes the requested disclosure of information.  The contractor is also prohibited from sharing information related to the contract in any forum or industry symposium without approval from the Contracting Officer.

1.1.8 DEFENSE BIOMETRIC IDENTIFICATION SYSTEM (DBIDS) INSTALLATION ACCESS

a. This Section applies to all Contractors and their representatives, including subcontractors.

b. DoW installations now require all Contractors and their representatives to be credentialed via enrollment in the DBIDS to gain access to these installations.  DBIDS requires a security background check for all Contractors and their representatives.  The Contractor should plan accordingly.  For more information on DBIDS and to complete pre-enrollment steps see the link below:

https://dbids-global-enroll.dmdc.mil/preenrollui/#/landing-page

c. DBIDS increases installation security and communications by receiving frequent database updates on changes to personnel and credential status, law enforcement warrants, lost/stolen cards, and force protection conditions.

d. Contractor registration in DBIDs and/or other security access systems does not guarantee the Contractor can gain access to every installation serviced under this contract.  Additional security measures may be required at some installations and activities. Consult the COR for information regarding additional credentialing requirements at specific locations.

e. During performance of the contract, the contractor shall be responsible for obtaining required identification for newly assigned personnel and for prompt return of credentials and vehicle passes for any employee who no longer requires access to the work site.

f. The Contractor shall be responsible for spearheading all annual DBIDS renewal requirements.

g. Upon completion or termination of the contract or expiration of the identification passes, the prime contractor shall ensure that all base identification passes issued to employees and subcontractor employees are returned to the issuing office.  Failure to comply with these requirements may result in withholding of final payment.

h. USAG-J Form 1529 is required to obtain visitor access authorization to the Hiroshima Area sponsor and submitted to the Provost Marshal in Kure Pier 6 at least four (4) working days prior to the scheduled visit date.

1.1.9 TSDF CLOSURES AND OUTAGES

Permanent or temporary closures or outages of the Contractor’s TSDF of choice, whether owned or subcontracted by the Contractor, does not alleviate the Contractor of its responsibility to perform on-time waste removals according to specified performance times and firm-fixed pricing in this contract.  Advanced notifications to the COR of scheduled TSDF outages for such reasons as maintenance may allow for flexibility in removal dates when agreed upon by the Government.  Unless the COR or KO authorize in writing to accept delays of service due to a TSDF closure or outage, the Contractor shall utilize an alternative qualified TSDF when necessary to meet containerized and bulk waste removal requirements at no additional cost to the Government.  The Contractor is advised to always have an alternative qualified TSDF in place to ensure timely removals. Refer to Section 1.1.4 for excusable delay requests needed after the issuance of a written TO.

1.1.10 CONTRACTOR PERSONNEL

All Contractor personnel performing work under this PWS shall be qualified via training and experience, commensurate with the duties being fulfilled at least to the minimum requirements under relevant and updates thereto.  The Contractor shall be responsible for all initial and refresher training required of Contractor personnel and it shall be completed in the appropriate timeframes.  Upon request by the Government, the Contractor shall provide proof that these trainings were completed.

1.1.11 ADDING CONTAINERIZED WASTE TO ISSUED TASK ORDERS

The Government will be allowed to add limited containerized waste to issued TOs up to five (5) business days prior to regularly scheduled removals.  The addition of such containers to an active TO will be limited to those waste streams having been fully profiled and having their associated CLINs available on the price schedule.  If the Contractor has already provided the COR and the Generator with a copy of the completed manifest per Section 1.1.3, the Contractor will edit the manifest on or before the time of removal.  Adding bulk waste to an issued TO will not be allowed unless agreed to by the Contractor.

1.1.12 PACKAGING, MARKING, LABELING, AND PLACARDING

a. Prior to removal, the Contractor shall verify all turned-in property for recycling and disposal are packaged in the proper container, and properly marked, labeled, manifested, and placarded to meet all applicable JEGS regulations.  The Contractor shall provide and affix the appropriate placards to each conveyance vehicle and trailer prior to leaving the Government premises.  Improvised or hand-crafted placards shall not be used for transportation of hazardous materials (HM) off the installation.

b. If property being turned in by the Government has not been properly packaged, marked, and labeled, the Generator shall either remove the waste item(s) from the TO, repackage the waste, or order services to have the Contractor repackage the waste using the applicable service CLIN(s).

c. The Contractor shall not package together (same container) with non-regulated waste.

1.2 REMOVAL

1.2.1 SPILL RESPONSIBILITY

a. The Contractor shall provide a comprehensive Spill Contingency Plan to the KO and COR that addresses the waste types to be removed under this contract. This one-time only requirement shall occur prior to conducting the first waste removal under this contract.  The driver shall have a copy of the plan in their possession during all removals and will be made available to the USG representative upon request.  The plan shall satisfactorily demonstrate to the USG that the Contractor possesses the fundamental skills and knowledge needed to contain a small liquid spill or leak up to five gallons in volume, and to overpack a leaking container when necessary. The contractor shall provide an updated plan for the KO and COR when revisions, other than name and address changes, are made to the plan.

b. The Contractor shall maintain serviceable, waste-compatible, and fully stocked spill kit(s) that are readily available to contain small spills and leaks during all waste removals.  The Contractor shall allow any USG representative who is present during the removal process access to the spill kit(s) for visual inspection.  Failure of the Contractor to demonstrate they are equipped with adequate spill kit(s) to respond to small spills and leaks during the removal process may result in cancellation of the removal and will be considered Contractor performance failure.

c. The Contractor is solely responsible for the proper cleanup and disposal of all spills or leaks occurring during the performance of this contract that occur because of, or are attributed to, the Contractor.  Cleanup will be to the satisfaction of, and at no additional cost to the USG.  HM/HW cleanup residues that are generated because of such spills or leaks shall be removed off-site within 72 hours of the cleanup.  Under extenuating circumstances and with the approval of the IEO, spill cleanup residues may be moved to an on-site permitted storage or accumulation area until they can be separately manifested and shipped off-site for disposal.

d. The Contractor shall report all such spills or leaks, regardless of their quantity, to the COR (if not already present) and the KO immediately upon discovery.  The Contractor shall adhere to all IEO spill reporting and clean-up requirements.  A written follow-up report shall be submitted to the KO not later than 24 hours after the initial telephonic report. The written report shall be in narrative form and as a minimum include the following:

(1) Description of material spilled or leaked, estimated quantity spilled, manifest number, etc.

(2) Whether quantity spilled is reportable, and if so, whether it was reported.

(3) Time, date, location, and description of spill; including but not limited to the area involved, weather, terrain, and any drainage concerns.

(4) Containment procedures initiated.

(5) Summary of Contractor communications with anyone other than the CO.

(6) Description of cleanup procedures utilized at the site including disposal location of spill cleanup residue.

(7) A copy of the Contractor’s spill contingency plan shall be attached to the written report.

e. MCAS Iwakuni Requirement for reporting Spills:

https://www.mcasiwakuni.marines.mil/Organizations/Station/Facilities/Environmental/After-Action-Spill-Report-Form/

1.2.2 SEGREGATION OF HAZARDOUS WASTE

All items collected on this contract must be segregated and kept physically separate from any containers not associated with this contract until the initial TSDF is reached.  The items must be so marked that they are readily linked to this contract.  The Contractor may transport previously loaded waste to designated onsite locations while conducting “milk runs” or picking up waste at multiple locations, provided the waste being removed is listed on the current TOs.  The Government reserves the right to issue single or multiple TO’s which may encompass different removal sites provided all TOs on a single milk run collectively meet or exceed the TO minimum per Introduction section unless otherwise specified in the contract or price schedule.

1.2.3 STATEMENT ON CONTAINERS

a. The Generator is responsible for ensuring their containers are UN approved, shippable containers that meet regulations at the time of turn-in.  The containers must be in good condition and compatible with their contents.  The Contractor is responsible for repackaging containers when necessary to meet specific waste acceptance criteria at the Contractor’s chosen TSDF and when the packaging requirements are neither DOT-required or considered an industry standard packaging practice.  The Generator shall ensure that wastes capable of freezing are suitably containerized prior to removal, though the Contractor shall be responsible for any repackaging efforts and costs for containers that become damaged during shipment due to expansion or other environmental considerations.

b. Special packaging required by Generators will be only for those waste stream items where such packaging is required by DOT regulations, and/or the packaging requirement is considered an industry standard practice as determined by DLA Disposition Services.  Some examples of Generator special packaging considered industry standards include:

(1) Applying clear electrical tape to battery terminals prior to packaging.

(2) Packaging of air/water reactive (D003) metal powders, dusts, fines, and debris under food grade mineral oil in combustible (poly/plastic) containers not to exceed five (5) gallons in size and 100 pounds in gross weight.

(3) Packaging of road flares and other authorized incendiary items under water.

c. Special packaging shall be used only to provide safe storage and transport.  It will not be used to perform treatment or to remove any waste code(s) since such removal would imply treatment.

d. The Contractor will, when possible, preemptively notify the COR and/or Generators directly when Generator special packaging will be required prior to Generators packaging for turn-in.  Generators are strongly encouraged to consult the COR (and hence the Contractor) regarding special packaging requirements prior to packaging potentially subject items for turn-in.  When warranted, the COR will consult the Contractor and advise the Generator.

e. If a Generator special packaging requirement asserted by the Contractor is in question as to whether it aligns with an industry standard practice, the COR will consult with the Contracting Officer for guidance.  The Contracting Officer may require the Contractor to provide proof that other similar TSDFs impose the same waste packaging acceptance criteria.  DLA may also conduct market research to determine whether a Contractor’s asserted special packaging requirement is widely applied by other qualified TSDFs in the industry.  DLA will not provide detailed written results of the market research as part of its decision.

1.2.4 GOVERNMENT EQUIPMENT & PERSONNEL

The Government shall not typically furnish equipment or personnel to assist in the performance of this contract.  Exceptions may include the use of Government-owned loading equipment (Section 1.2.6) and Government-owned scales (Section 1.2.7), and where applicable, various management services.  Any other offers of assistance are unauthorized, and the Contractor shall not accept any such offers.

1.2.5 TREATMENT OF HAZARDOUS WASTE ON GOVERNMENT FACILITIES

a. Treatment of hazardous waste (including solidification) on Government facilities is not permitted.  Treatment is defined as any process which meets the definition of treatment as identified in 40 CFR § 260.10 and/or the facility Permit.

b. The Contractor shall not drain and/or flush PCB items at Government facilities.  Draining will be allowed only to prevent leaking and to meet DOT regulations.

c. Treatment, disposal, or release of gases, other than inert gases, to the atmosphere on Government premises is not permitted by this contract.  The Contractor may perform gas extraction for other than inert gas cylinders at the pickup location using a self-contained apparatus.  This apparatus shall emit no gas into the atmosphere and purge the entire cylinder contents into a closed receiver for transport to a recycling or disposal site.

1.2.6 LOADING

a. The Contractor’s representative shall arrive at the pickup location no more than 30 minutes after the scheduled removal time and date.  In the event the Contractor anticipates a late driver arrival, before the scheduled appointment time, the Contractor will notify the Government representative with whom the scheduled appointment was made.

b. The Contractor shall perform loading and is responsible for furnishing all equipment appropriate for the safe and efficient loading of containers, totes, and pallets. Such equipment will include a lift gate on the conveyance vehicle or a forklift or pallet jack as appropriate for the job.  The Contractor should never assume Material Handling Equipment (MHE) will be provided by the Government.  The Government will not block, chock, brace, lash, band, or in any other manner, secure the cargo on such conveyance vehicles furnished by the Contractor.

c. The Government may offer to assist the Contractor or subcontractor with loading during normal hours of operation if the appropriate personnel and equipment are available for use.  If the offer is accepted by the Contractor, the Government will make the initial placement or “tailgate” items at the rear of the Contractor’s conveyance vehicle or trailer.  The initial placement will be determined by the Government.

d. Prior to scheduled removals, the Government will have containers staged at ground or loading dock level as applicable.  The containers may or may not be palletized. It is the Government's responsibility to ensure the waste being removed is readily accessible for loading by the Contractor at the time of removal.  Accessible is defined in this Section as unobstructed and not requiring the movement of other containers or objects that are not scheduled for removal on the subject Task Order to gain access to the waste being removed.

1.2.7 WEIGHING OF ITEMS

a. All items will be weighed prior to removal.  Government scales will be used when available, operable, and authorized by the Government.  The Contractor shall be responsible for determining the availability of Government scales by contacting the COR.  All scales used for purposes of this contract shall be calibrated IAW applicable regulations.  The Government shall ensure that Government scales are operable and accurate through certification or demonstration.  The Contractor shall ensure that non-Government scales are operable and accurate through certification or demonstration.

b. At the Government's option, the use of Government scales will be allowed at the following sites:

| SITE | EQUIPMENT AVAILABLE |
| --- | --- |
| Kawakami Ammo Depot, Bldg. 2766-1 | Pallet scale – 5000 lbs. |

c. Before bulk loading, the Contractor is required to weigh the waste hauling vehicle, trailer, and/or ROB/vacuum box, as applicable, to establish the empty (tare) weight of the conveyance.  This weight will be documented on a certified weight ticket and provided to the COR.  After loading, the Contractor will reweigh the conveyance to establish the loaded (gross) weight of the conveyance.  This weight will be documented on a certified weight ticket and provided to the COR.  The actual (net) weight of the waste cargo shall then be calculated by subtracting the tare weight from the gross weight.  When onsite Government scales are available, weights must be agreed upon by both the Contractor and the Government before removal.  When offsite commercial scales must be used, a certified weight ticket will be provided to the COR as stated above and shall be the basis for payment to the Contractor.  In lieu of a printed/certified weight ticket, a manual ticket that is signed or stamped by a witnessing Government representative will constitute a certified weight ticket when certified Government scales are available.

d.  If a weight discrepancy of ≥ 10% from that of the Government scales is reflected on a weight ticket from the TSDF scale, the contractor shall provide a copy of both weight tickets to the COR and the Generator to validate the discrepancy and shall be the basis for payment to the Contractor.

1.2.7.1 TASK ORDER (TO) ADJUSTMENTS

All TO weights and/or volumes are estimated.  The Contractor should expect that TO weights and/or volumes will require adjustment upward or downward as removals take place and the actual weights/volumes being removed are determined.  Therefore, the Government has the unilateral right to increase or decrease the TO weight/volume in those cases where the Government has underestimated or overestimated the weight/volume ordered for removal, allowing the Contractor to complete the performance.  In cases of adjustments resulting in a decreased weight/volume, these decreases are not subject to FAR 52.212-4, termination for convenience coverage.  In addition, it is agreed that it is acceptable for the Government to issue TO modifications unilaterally for quantity changes, noun description changes, CLIN changes, adding/deleting lines, extending TO end dates, etc.  The Contractor will be paid for the actual items and quantities removed per the awarded contract price schedule.  If upon receipt of a unilateral TO modification, the Contractor does not agree with the Government’s change(s), the Contractor reserves the right to seek resolution through the KO.  These modifications pertain only to the TOs issued, and no changes will be made to the terms and conditions of the contract.

1.2.8 DEFINITIONS, BULK & CONTAINERIZED WASTE

1.2.8.1 BULK WASTE

a. The term Bulk Waste refers to waste that is typically removed via roll off box (ROB), vacuum or sludge boxes, dump trucks, tank trucks, vacuum trucks, pump trucks, and trucks with vacuum-loaded or pump-loaded tanker trailers.  Bulk waste does not refer to containerized wastes including waste contained in drums, totes, cubic yard boxes, military tri-wall boxes, cubic yard bags or “super sacks”, lamp boxes, cylinders, or palletized waste.

b. The term Heavy Solids used in this document and the Price Schedule refers to heavy debris (as defined in 40 CFR § 268.2(g)) and bulk waste solids/sludges with a density greater than 600 lbs./yd3 and shall be ordered using the bulk CLIN Suffix B1, B2, B3, or B4.  The CLIN Suffix is assigned based on the net weight of each heavy bulk load.  If a bulk load with a net weight of less than 10,000 lbs. is ordered, the bulk CLIN Suffix “B1” will be used.  Loads weighing 10,000 to 20,000 lbs. will be assigned the bulk CLIN Suffix “B2”.  Loads weighing 20,000 to 30,000 lbs. will be assigned the bulk CLIN Suffix “B3”.  Loads weighing more than 30,000 lbs. will be assigned the bulk CLIN Suffix “B4”.  Bulk heavy solids turned in under PCB Base CLINs are designated with CLIN Suffix “B8”, are ordered in kilograms “KG” with a 5,000-KG minimum load.  Weights shall be determined using certified Government scales when available, or certified commercial scales when Government scales are not available.  Prior to removal, and when possible, the net weight of the waste shall be determined and agreed upon by both the Government representative and the Contractor.

c. The term Debris used in this document and the Price Schedule refers to (a) solid artifacts, fragments, or rubbles which are present, but not primarily present by volume, and are intermingled or suspended in an otherwise fully solid, liquid, or sludge waste stream as described above, or (b) a waste stream which meets the definition of debris in 40 CFR § 268.2(g).  Note: Definition (a) in this section meets the “Mixed” or “Various” Physical State designation indicated in the Price Schedule for this contract, as CLINs with a “Mixed” or “Various” physical state refers to solids, liquids, sludges, debris, or any combination thereof.  Definition (b) in this section meets the “Debris” Physical State designation indicated in the Price Schedule for this contract.

d. The term Lightweight Solids used in this document and the Price Schedule refers to light debris (as defined in 40 CFR § 268.2(g)), and bulk waste solids with a density of less than or equal to 600 lbs./yd3.  These waste streams are considered lightweight and shall be ordered using the Cubic Yard CLIN Suffix “B6”.  Cubic yards (yd3) are calculated by visual estimation.  For example, if a 20-yd3 ROB appears 90% full, the removal will be estimated and ordered at 18 CD.  Note, any CLIN with a “B6” suffix will be ordered using “CD” to denote cubic yards.  When ordering a B6 CLIN Suffix, the Government will order disposal of the waste based on a minimum of 75% of the volumetric capacity of the associated ROB.  For example, if a 20 cubic yard ROB contains 12 cubic yards of lightweight solids (B6), the Generator shall pay for disposal of 15 cubic yards, or 75% of the ROB’s volumetric capacity.  Prior to removal, the volume shall be determined and agreed upon by both the Government representative and the Contractor.

e. Base CLIN TX06 is uniquely designed around the treatment technology of Microencapsulation of hazardous debris. When ordering Base CLIN TX06 in conjunction with Bulk CLIN Suffixes B1, B2, B3, B4, or B6, the Generator will ensure that the waste stream does not contain pipes, hoses, tubes, equipment, or rigid objects greater than 10 feet in any dimension, where toxic metal surfaces cannot be easily coated with microencapsulation coating material.  In such cases where these types of articles are present in bulk loads, Base CLIN TX07 should be considered (maximum size is 18’L x 4.5’W x 4.5’H).  Furthermore, intact containers (Per 40 CFR § 268.2(g)) do not meet the definition of debris and shall not be turned in under Base CLINs TX06 or TX07.  Intact containers are those containers “that are not ruptured and that retain at least 75% of their original volume”.  When intact containers are present, the Generator should consider using “Lab Pack/Loose Pack” CLINs.

f. The term Solid(s) used in this document and the Price Schedule refers to solid material, which is largely free of free liquids, e.g., soil, silt, sand, gravel, filter cake, powder, granules, crystals, fines, and artifacts that do not meet the definition of debris (as defined in 40 CFR § 268.2(g)).

g. The term Liquid(s) used in this document and the Price Schedule refers to liquid material which is transferable by conventional pumping devices designed to pump low-viscosity liquids and largely free of suspended or settled (vs. dissolved) solids.  A liquid that has become frozen due to ambient freezing temperatures but would otherwise be of liquid state at 72°F, will still meet the definition of liquid for purposes of this contract.

h. The term Sludge(s) used in this document and the Price Schedule refers to material that is either (a) a multiphasic material with some combination of a liquid phase and a solid phase as defined above, (b) a slurry or mud-like material, such as liquid-saturated soil or tank bottoms, whereby solids are largely or completely suspended in liquid and typically flows under gravity, and may or may not be conveyable by high capacity pumping or vacuuming devices designed to move heavy material, or (c) a highly viscous liquid not pumpable by conventional pumping devices designed to pump low-viscosity liquids.

i. Bulk waste liquids will be pumped by conventional pumps typically used on tanker trucks and trailers, even if loaded with vacuum truck or trailer, and shall be ordered using the CLIN Suffix B5, indicating gallons (GL).  When removed via tanker or vacuum truck, a 1,500-gallon (GL) minimum charge for bulk liquids will apply and be ordered, as appropriate.

j. Gallons are measured by metering, tank strapping, or can be calculated using the density of the waste.  Volumes are rounded to the nearest whole gallon using conventional (non-scientific) rounding rules.  This rule requires that if the digit to the right of the decimal place is 0, 1, 2, 3, or 4, leave the whole number alone; if the digit to the right of the decimal place is 5, 6, 7, 8, or 9, add 1 to the whole number.  Example: A tanker load of water (density = 8.33 Lbs./Gal) weighing 36,290 Lbs. would be calculated as 36,290 lbs./8.33 lbs./gal = 4,356.54 gallons.  This will be rounded up to 4,357 gallons.  The density of the waste will be obtained from the waste profile sheet.

k. Tank strapping may be used to calculate the volume of liquid in a tank or tanker given a strapping rod depth measurement.  The graduated rod is inserted vertically through the top surface of the liquid to the bottom of the tank, and a measurement is taken to determine the depth of the liquid.  The depth, usually measured in inches, is compared to a table, or spreadsheet, which shows the conversion to gallons.  Only tables specifically calibrated to the tank or tanker being strapped may be used.  These measurements will be made by the Contractor and verified and agreed upon by the Government representative.  If this method is used, tanks and tankers must be on a level surface or measured at the center of the tank.

l. The Generator reserves the right to choose which method will be used to determine total gallons of waste offered.  The net volume (gallons) of the waste shall be determined and agreed upon by both the Government and the Contractor prior to removal, when possible.

m. Liquids in totes, also known as portable tanks and Intermediate Bulk Containers (IBC), come in various sizes.  For purposes of this contract, liquid waste that is removed in totes are considered containerized waste.  Refer to Section 1.2.8.2 for removal of totes.  When totes are pumped/vacuumed onsite for removal via tanker or vacuum truck, CLIN Suffix “B5” will be applied, and the waste is removed as a bulk pumpable liquid ordering the actual gallons being removed.  However, when a bulk load consisting of less than 1,500 gallons is being turned in for removal, a 1,500-gallon (GL) minimum charge for bulk liquids will apply and be ordered, except where otherwise specified in the contract.  Shipping manifests will reflect the accurate quantity of waste being transported.

n. The Contractor may be allowed, at the discretion of the Government, to “top off” their tanker or vacuum truck by pumping/stinging containers of the same Base CLIN at no additional cost to the USG.  See Section 1.2.9 for additional specifications.

1.2.8.2 CONTAINERIZED WASTE

a. The term containerized waste refers to waste that is typically transported by a box truck or semi-truck and trailer or van, including waste contained in drums, totes, cubic yard boxes, military tri-wall boxes, cubic yard bags/super sacks, lamp boxes, cylinders, or palletized waste.

b. Containers shall be counted, weighed, and documented against the TO by the Contractor prior to removal.  Although most containerized CLINs are priced by the container size (CLIN Suffixes C1-C6, and T1-T3), weights must still be documented for other purposes such as manifesting and Generator reporting.  The Contractor shall provide portable scales, which may be mounted to the truck or trailer, for weighing of items, and must first demonstrate reasonable weight accuracy to the Government representative.  The use of commercial scales is not authorized for containerized items.  Only materials being removed by the Contractor will be weighed, including the containers, pallets, and dunnage.

c. Drums larger than 55 gallons (CLIN Suffix “C5”) are to be used only for overpacking smaller drums when the smaller drum(s) are leaking or have the potential to leak.  Generators shall be required to pay the Contractor based on the size of the overpack/salvage drum, and not on the size of the inner, overpacked drum(s).

d. “CYB” (CLIN Suffix “C6”) refers to Cubic Yard Boxes, Cubic Yard Bags or Super Sacks, Gaylord Boxes, and Military Tri-Wall Boxes.

e. “Pallet” (CLIN Suffix “C6”) refers to a flat wooden, plastic or metal transport structure not more than 48” L x 48” W that supports goods in a stable manner while being lifted by a forklift, pallet jack, front loader, or other lifting device.  Skids may be used in lieu of pallets when well suited to support the cargo and are accessible to movement via typical material handling equipment.  Containers/items placed on a pallet are often secured with dunnage, i.e., banding straps, stretch or shrink wrap for stability in movement.  The sum weight of the dunnage, pallet, and palletized waste cargo will collectively comprise the weight of the waste and shall not exceed a weight of 1,600 pounds and/or a height of 42” including the pallet unless otherwise authorized by the Contractor.

f. The cost of the Generator for a CYB and Pallet are the same when the CLIN Suffix C6 is used and packaged IAW current DOT shipping requirements.  However, this does not imply that both packaging methods are always appropriate for waste assigned to a CLIN Suffix of C6.  Safety and DOT shipping regulations shall always take precedence.

g. Containers of the same Base CLIN and no larger than 5 gallons in size, may be palletized or placed in a CYB to assign the CLIN Suffix C6.  However, this provision only allows for up to eighteen (18) 5-gallon pails/buckets per pallet or cubic yard box.  No such container quantity limit shall apply to containers that are smaller than 5 gallons in size or to consumer commodities packaged in their original packaging.  Containers larger than 5 gallons in size will not be placed in a CYB for purposes of consolidating them into a single container.  Containers of all sizes may be placed on pallets and skids for ease of storage and movement, but containers larger than 5 gallons, other than consumer commodities in their original packaging which may be larger than 5 gallons in size, will have the quantity and CLIN assigned as appropriate to each container on the pallet.

h. Most containerized wastes are assigned to a CLIN Suffix of C1-C6 and T1-T3.  These CLIN Suffixes correspond to a range of different container sizes and are ordered as each “EA”.  CLINs for containerized aerosols, batteries, intact lamps, and some military items are designated with CLIN Suffix C7 and are ordered in pounds “LB”.  CLINs for containerized PCBs are designated with CLIN Suffix C8 and are ordered in kilograms “KG”.  CLINs for cylinders, vessels, canisters, and other miscellaneous waste streams have a CLIN Suffix of ME, M1, M2, or M3 and are also ordered as each “EA”.

i. “Tote” (CLIN Suffixes T1, T2, and T3) refers to a portable tank or Intermediate Bulk Container (IBC) designed for storage and transport of bulk liquids.  For purposes of this contract, liquid wastes that are removed in totes are considered containerized wastes.  Tote sizes are commonly designed in increments of 55 gallons, i.e., 220, 275, and 330 gallons.  CLIN Suffixes T1, T2, and T3 shall apply to these tote sizes, respectively.  The unit of measure for these CLIN Suffixes will be Each (EA), regardless of the actual volume of liquid contained in the tote, and not to exceed the design capacity.  Totes shall not be returned to the Government unless ordered under a separate service CLIN and disposal pricing will include any associated processing fees, if applicable.

1.2.9 BULK LIQUID PUMPING

a. When a bulk, pumpable liquid (CLIN Suffix “B5”) is ordered for removal, the Contractor is required to pump or vacuum waste from tanks, drums, totes, sumps, surface, storm, and groundwater sources.  The Contractor is responsible to furnish all equipment and personnel needed to complete this task, including but not limited to a collection vehicle equipped with a pump/vacuum mechanical device, hoses, filters, fittings, etc.  Pumps/vacuums and hoses shall have a capacity and chemical resistance to safely handle the types of waste to be collected, and remove all pumpable liquids from tanks, drums or totes that can be removed without agitation or introduction of other materials.  A 1,500-gallon minimum charge will apply for the pumping/ vacuuming and removal of bulk liquid waste on a single TO and negates the order of CLIN 1093SV.  This minimum charge will not apply and will not be ordered when the tank or vat from which the bulk waste was removed, was also cleaned under an ICS CLIN on the same TO or when the Contractor has elected to pump/vacuum/sting containers for convenience to the Contractor at no additional cost to the Government.

b. Where removal from more than one tank and/or container is performed using a single vacuum or tanker truck, only compatible waste of the same Base CLIN may be pumped and commingled into the same vacuum or tanker truck for transport.  This service will not be ordered or performed as a means of treatment per 40 CFR § 260.10, or when any risk of reaction due to incompatibility exists.

c. Bulk liquid pumping will be allowed only if the Contractor has a current spill contingency plan on file with the COR and performs operations in a safe manner (Section 1.2.1.).  The Government retains the right to stop operations if environmental or safety concerns arise.

d. When bulk liquid pumping is performed, all containers must be emptied IAW 40 CFR.

e. The actual weight of the material picked up must be identified on the manifest.  Bulk liquid removals may be ordered from any location in or around pickup locations. (Up to a 50-mile radius, see Introduction), in addition to the known sites identified below:

1.2.10 BULKING & CONSOLIDATION

a. Bulking shall be defined as the act of stinging, vacuuming, and/or pumping, as permitted by regulation, from an otherwise removable container(s) into a vacuum truck or tanker.

b. Consolidation shall be defined as any method that involves pouring, draining, or packaging like waste items, liquids, sludges, solids and/or debris from one or more containers into another container.

c. Bulking and consolidation, where permitted by the installation, may be performed at the discretion of the Contractor at no additional cost to the Government.  This will only be allowed when the Contractor has a current spill contingency plan on file with the COR (Section 1.2.1) and performs operations in a safe manner.  The Government retains the right to stop operations if environmental or safety concerns arise.

1.2.11 EXPEDITED REMOVAL/PERFORMANCE OF SERVICES – CLINs 1095SV-1098SV

a. The standard removal time for containerized waste under this contract is 20 business days, and for bulk waste is five (5) business days, per Section 7.0.  The clock for removal begins on the first full business day following issuance of the TO.

b. When expedited removal is ordered, waste must be removed within the specified timeframe, per Section 7.1.  Expedited removal CLINs may be ordered in conjunction with any disposal CLIN(s) at any pick-up location(s) in the contract.  Ordering an expedited removal CLIN will supersede the standard removal timeframe specified in Section 7.0.  Minimum TO charges will not apply to a TO containing an expedited removal CLIN.  When expedited removal is required, CLINs 1095SV-1098SV will be ordered based upon the timeframe for which the subject waste is to be removed.  One each (1 EA) will be ordered for containerized waste up to forty (40) 55-gallon containers or the volumetric equivalent.  For bulk removals, one each (1 EA) is limited to one (1) TO line not to exceed 40,000 pounds for solids or 5,000 gallons for liquids and shall not cause the gross conveyance weight of the loaded vehicle to exceed DOT weight limits for normal transport.

c. The standard performance time for performance of services under this contract varies with different services per Section 7.0.

d. When expedited performance of services is ordered, performance must begin within the specified timeframe, per Section 7.1.  The clock for expedited performance of services begins on the first full business day following issuance of the written TO.  Expedited performance CLINs may be ordered in conjunction with any service CLIN(s) at any pick-up location(s) on the contract other than industrial cleaning services (ICS) CLINs, which will be ordered under the appropriate surcharge CLIN for Expedited Commencement of ICS Work, per Section 3.22.  Ordering an expedited performance CLIN will supersede the standard performance timeframes specified in Section 7.0.  Minimum TO charges will not apply to a TO containing an expedited performance CLIN.  When expedited performance is required, one each (1 EA) of CLIN 1095SV-1098SV will be ordered based upon the timeframe for which the commencement of service is required and will be ordered for no more than five (5) Management Service TO lines.  Ordering of an expedited performance CLIN is further limited to one each (1 EA) per pick-up location, including all sites located on a single installation.

e. CLIN 1095SV is ordered for a one (1) business day expedited removal or performance, and is reserved for emergencies, e.g., major weather events, natural disasters, unforeseen compliance issues, etc.

f. The Contractor shall submit paperwork to the COR no later than these required timeframes:

1.3 POST REMOVAL

1.3.1 RETURN OF SHIPPING DOCUMENTATION

a. At the completion of removal efforts, the Contractor shall provide to the COR with a complete and accurate, transporter-signed copy of each manifest that is used for the waste removal.  If a Government representative is not present at the time of removal, the Contractor shall provide the COR with a copy of the transporter-signed pickup manifest(s) within three (3) business days following the removal.

b. Unless explicitly prohibited, the use of transfer facilities is authorized during transit of waste to the designated facility.  A transfer facility is any transportation related facility including loading docks, parking areas, storage areas and other similar areas where shipments of hazardous waste are held during the normal course of transportation, per 40 CFR § 260.10.  The Contractor will ensure the 10-day holding time is not exceeded at any transfer facility.

c. The Contractor will submit an Invoicing Packet to DLA Disposition Services IAW Section 8.0, which will include a copy of manifest(s) signed by the first TSDF, except for TOs that include the disposal of regulated PCBs, DEMIL-required and other wastes requiring the Contractor’s provision of a CD, which require an audit trail to the final TSDF.

d. If within 30 calendar days from the removal date, the waste has not yet been received at the designated facility, the Contractor shall notify the COR of the current physical location of the waste.  Within 35 calendar days from the removal date, the Contractor shall provide the COR with a facility-signed electronic copy of each manifest via email.  If multiple transporters were used during transit, the Contractor shall also provide the COR with a Uniform Hazardous or Non-Hazardous Waste Manifest Continuation Sheet, as applicable, with additional transporter signatures as necessary.

e. If the Contractor is unable to deliver the waste to the designated or alternate facility identified on the manifest(s) per 40 CFR § 263.21(a), the COR and Generator will be contacted for disposition and manifest revision instructions per 40 CFR § 263.21(b).  The Contractor shall make reasonable attempts to deliver the waste to a qualified alternate facility, rather than returning the waste to the Generator.  In the event of rejected shipments, or where the shipment is undeliverable, the Contractor shall provide the COR and Generator with a written narrative of the incident, including details of all attempts to provide delivery to a qualified TSDF.  If the signed manifest copy will not be provided within 30 calendar days from the date of removal, the Contractor shall preemptively prepare and provide the Generator with an Exception Report. An electronic copy of the Exception Report shall also be provided to the COR.  Failure of the Contractor to properly perform IAW any part of this Section will be documented accordingly in the Acceptable Performance Levels (APLs) per Section 4.0, and appropriate action taken as determined by the KO.

f. No TO modifications will be processed to correct count, volume, or weight discrepancies once the waste has been removed from the Generator site due to unavailability of a Government representative at the TSDF. The document(s) signed at the time of removal will constitute actual count, volume, weight for payment purposes.

2.0 DISPOSAL/RECYCLING

a. For all wastes other than PFAS, regulated PCBs, DEMIL Property, and other wastes requiring provision of a CD, delivery of manifested waste items to the first designated (or alternate) qualified interim or final recycling, treatment, storage and/or disposal facility, with signed acknowledgement of receipt (with no subsequent rejection) from said facility and return of shipping documents per Section 1.3.1, constitutes disposal/recycling services rendered for payment purposes regarding subject waste items turned in under this contract.  No further document trail of shipments is required for provision by the Contractor unless a certificate of disposal, destruction, or recycling is required per the disposal CLIN description or ordered under Service CLIN 6503, per Section 2.1.

b. While TSDFs are required under their Permit(s) to prevent the receipt of unauthorized wastes, the Contractor is responsible to ensure that properly profiled wastes are shipped only to those facilities that hold the appropriate Permit(s) to receive such wastes.

c. Regardless of the ordered disposal or recycling CLIN and its associated description, the Contractor shall comply with all State and Federal regulations.  Some waste may be state regulated, where such regulations are not cited or regarded in the CLIN descriptions.  It is the responsibility of the Contractor to notify the Government if an ordered CLIN requires a disposal or recycling technology that conflicts with any such regulation(s).

2.1 CERTIFICATES OF DISPOSAL, DESTRUCTION, & RECYCLING

a. Generators may require Certificates of Disposal/Destruction (CD) or Recycling for any items at their discretion.  The Government recognizes there are additional costs to the Contractor associated with obtaining these certificates from the final disposal, destruction, or recycling facility.  Unless otherwise stated in this contract, such certificates will only be required as specified in the CLIN description on the Price Schedule and/or PWS, or if CLIN 1300SV is ordered.  CLIN 1300SV will not be ordered in conjunction with a CLIN that is already identified as requiring a CD or Certificate of Recycling.

b. When service CLIN 1300SV is ordered it may be placed on a standalone TO with the subject of disposal CLIN, separate from other disposal and service CLINs.

c. CLIN 1300SV will be ordered as one each (1 EA) for every CD or Certificate of Recycling desired per TO line item.

d. CDs and Certificates of Recycling shall clearly indicate, or be supplemented with supporting documentation to clearly indicate, a complete “cradle-to-grave” audit trail from the Generator pickup location to the final recycling or disposal facility.  This audit trail will require, at a minimum, quantities, TO, EPA ID number(s), manifest(s) and line-item number(s) that serve to clearly link the initial removal shipping document, through all interim facilities (if applicable), to the final recycling or disposal facility that is issuing the certificate.  A data spreadsheet may be used by the Contractor (in lieu of manifest copies) to accompany a CD or Certificate of Recycling to provide the minimally required cradle-to-grave information.  Such information provided by the Contractor is subject to verification via Government audit of Contractor and TSDF manifest files.

e. Some facilities issue CDs or Certificates of Recycling that, in essence, state the waste “will be” versus “has been” recycled, disposed, or destroyed IAW applicable regulations.  Such certificates will be authorized under this Section, but only when issued and certified by the final recycling or disposal facility.

f. CDs and Certificates of Recycling shall be provided to the Generator and the COR within 30 calendar days after disposal, destruction, or recycling occurs. The Contractor will not be authorized payment until all required Certificates of Disposal/Destruction (CD) or Recycling are received by the COR and Generator that ordered the certification. Task order minimums will not apply to these TOs.

2.2 JEGS/GOJ REGULATED/NON-REGULATED ITEMS

2.2.1 ITEMS REGULATED IAW JEGS/GOJ

The Generator will identify all material/waste IAW JEGS. Treatment and disposal will be IAW JEGS, GOJ, and local laws and regulations. If the Contractor is aware of a contradiction between GOJ laws or regulations and the JEGS then the Contractor may request a waiver from the KO prior to removal. The Contractor will not commence performance under a waiver request until the KO has granted written approval. The Contractor shall dispose of hazardous and/or industrial waste under this contract by means of treatment or disposal technologies that best mitigate hazards of such wastes to human health and the environment. If the Contractor elects to use a newly developed alternate and effective treatment technology, the Contractor may request a waiver from the KO.

2.2.2 ITEMS NOT REGULATED BY JEGS/GOJ

a. Items not regulated by JEGS/GOJ must be disposed or recycled in compliance with all GOJ and local laws and regulations.

b. The Contractor is required to use only firms licensed/permitted by the GOJ and approved by Disposition Services. This includes any facility that may receive any material or waste under this contract, or a component thereof. Regulated waste and IW not subject to recycling or incineration may be disposed of in permitted landfill facilities IAW JEGS and Japan law and regulations.

c. The Contractor shall ensure that used oil, hazardous waste, designated waste, oil product contaminated with any hazardous waste is NOT used for dust suppression or road treatment.

2.2.3 CONTRACTORS ELECTED DETAILED ANALYSIS AND SAMPLING

The Contractor is required to use only firms licensed/permitted by the GOJ and approved by Disposition Services. This includes any facility that may receive any material or waste under this contract, or a component thereof. Regulated waste and IW not subject to recycling or incineration may be disposed of in permitted landfill facilities IAW JEGS and Japan law and If the Contractor elects to perform laboratory analysis for waste characterization/profile confirmation or as part of the Contractors final treatment/disposal decision. Field tests on site will not be authorized without a prior COR/KO request and approval. Analysis must be performed IAW those agreed upon by U.S. Forces Japan (USFJ) and the host nation, Japan. The U.S. Government may request copies (written in English) of the actual analytical results and quality control checks. The Contractor shall be responsible for the proper disposal of all samples and their containers.

2.3 ASBESTOS MANAGEMENT

When disposing of asbestos waste, the Contractor shall ensure that all asbestos-containing materials (ACM) and asbestos-containing waste are properly disposed of in a permitted facility. The asbestos wastes may be melted and/or disposed of in a permitted municipal solid waste landfill IAW local Japan laws and other regulatory requirements.

2.3.1 DISPERSAL (FRIABLE) ACM WASTE DISPERSAL ASBESTOS, (BUILDING MAINTENANCE MATERIALS) (SHIW)

a. Waste dispersal asbestos is any material containing asbestos such as, but not limited to spray-applied insulation, boiler jackets (insulation) or pipe lagging (insulation) and PPE (plastic cover sheets, masks, coveralls from asbestos operation).

b. Dispersal asbestos shall be disposed of in a GOJ approved facility, vault, or impervious type landfill, or by an approved burner IAW Japanese regulations.

c. The asbestos must be in a closed plastic bag, placed and sealed in a second (outer) plastic bag. The bag size shall comply with the requirements of the disposal facility. The outer bags must be clear and have bilingual labels indicating dispersal asbestos. Any items with sharp edges must be taped before bagging process. Asbestos bags will have an English and Japanese label affixed to it as stated in JEGS.

2.3.2 NON-DISPERSAL (NON-FRIABLE) ACM (IW)

Non-dispersal (non-friable) ACM – any material containing more than 0.1 percent asbestos by weight, such as but not limited to, fire-proof safes and filing cabinets, brake shoes, brake pads and gaskets) which cannot be easily crumbled, pulverized, or reduced to powder by hand pressure from on-construction sources. These items shall be properly packaged, marked and labeled IAW Japan or local government laws and regulations. Non-dispersal asbestos shall be disposed of in an approved GOJ facility, vault, impervious type landfill, or approved burner, and IAW Japan Laws. Packing will be IAW Japanese asbestos regulations.

2.3.3 NON-DISPERSAL BUILDING MAINTENANCE MATERIALS ACM (IW)

a. Non-dispersal ACM Building Maintenance Materials – any material containing more than 0.1 percent asbestos by weight from maintenance sources. These are items such as, but not limited to, roofing tiles and shingles, floor tiles and ceiling tiles.

NOTE: ACM Items such as floor tiles and roofing must be removed from wooden sub-floor planking / roof sheeting; then properly packaged, marked, and labeled IAW Japan or local government laws and regulations.

b. Non-dispersal (non-friable) asbestos shall be disposed of in an approved Government of Japan facility, vault, impervious type landfill, or approved burner, and IAW Japan Laws. Packaging will be IAW Japanese Asbestos regulations.

2.4 AQUEOUS FILM-FORMING FOAM (AFFF) AND OTHER PER- AND POLYFLUOROALKYL SUBSTANCES (PFAS) RELATED WASTES

a. Aqueous Film-Forming Foam (AFFF) and other Per- and Polyfluoroalkyl Substances (PFAS) related wastes are covered under this contract.  CLINs that are uniquely designed for the disposal of AFFF, and other PFAS-related wastes include Base CLINs PF20, PF21, and PF22 for incineration. All such CLINs require the Contractor to provide a Certificate of Destruction for incineration or a Certificate of Disposal for landfill disposal, per Section 2.1, as required in the CLIN description.  If first authorized by DLA, when an AFFF/PFAS waste stream should meet the definition of a HW, the waste shall be managed and turned in by the Government under the appropriate HW profile based CLIN for disposal.  The contractor is obligated, in the event of any bulk pumping of Aqueous Film Forming Foam (AFFF), to provide all necessary AFFF anti-foaming agent as required by the terms outlined in this agreement.

b. The term AFFF Solid(s), for purposes of this contract, refers to AFFF-contaminated solids, sludges, and/or debris that has been impacted with AFFF concentrate or rinsate.  Solids may include but are not limited to soil, sand, gravel, PPE, filters, filter media, pliable bladder tanks and other debris.  If first authorized by DLA, Base CLIN PF20 will be assigned to AFFF/PFAS solids and sludges waste streams designated for incineration.  There are various Base CLINs with the “PF” prefix available for disposal of AFFF and other PFAS impacted solids and sludges.

c. The term AFFF Concentrate(s), for purposes of this contract, refers either to undiluted, pumpable AFFF-related liquid product that is unused, or is greater than 10% product by volume.  Concentrates are typically technical grade products as sold by the manufacturer.  If first authorized by DLA, Base CLIN PF21 will be assigned to AFFF concentrates designated for RCRA incineration.

d. The term AFFF Rinsate(s), for purposes of this contract, refers to diluted, pumpable AFFF-related liquids such as rinse water generated by a rinse out process of tanks, piping, and firefighting equipment.  The term shall also apply to AFFF-impacted water, including surface, storm, and groundwater.  Rinsates contain at least 90% water and typically contain <1% technical grade product as sold by the manufacturer.  If first authorized by DLA, Base CLIN PF22 will be assigned to AFFF/PFAS rinsate waste streams designated for incineration.  There are various Base CLINs with the “PF” prefix available for disposal of AFFF rinsates and other PFAS impacted liquids.

e. The term AFFF Fire Extinguisher(s), for purposes of this contract, refers to portable firefighting equipment containing pressurized AFFF liquid with varying PFAS formulations.  The liquid-filled bottle, cylinder, or tank is pressurized by the manufacturer using an inert gas.  AFFF fire extinguishers come in various sizes and models.  Handheld models are turned in under Base CLIN PF41.  Cart-mounted models are turned in under Base CLIN PF51.  CLIN Suffixes designate the liquid capacity of the equipment.  Cart-mounted models will have a tank and cart frame with possible attachments including wheels, hoses, and other accessories.  No compressed gas cylinders will be attached. Certificates of Disposal provided under these CLINs pertain only to the liquid contents of the equipment and not to the remaining cart, parts, accessories, empty bottles, cylinders, or tanks.

f. Other PFAS-related waste(s), for purposes of this contract, refers to wastes that contain or are impacted by a PFAS source other than AFFF.  If/when the Government should have other PFAS-related wastes requiring disposal through this contract, CLIN descriptions that reference “AFFF” and/or “PFAS” may be utilized for turn-in by the Government and removal and disposal of the waste by the Contractor.  The total PFAS concentration of the waste stream, as profiled, will be the basis for CLIN selection.

g. The Contractor shall utilize only TSDFs annotated in the QFL as authorized for the disposal of AFFF/PFAS.

2.5 COMPRESSED GAS CYLINDERS, CARTRIDGES, VESSELS, AND CANISTERS

a. Base CLINs are assigned to compressed gas cylinders (CGCs), cartridges, vessels, and canisters according to the Compressed Gas Cylinder Reference Guide as contained in this contract.  A general reference copy is also available on the DLA Disposition Services webpage. See “Compressed Gas Cross-Reference” at the below link:

http://www.dla.mil/DispositionServices/Offers/Disposal/HazardousWaste/HazWasteDisposal.aspx.

b. The Contractor shall provide all services necessary for the assessment of all compressed gas cylinders, cartridges, vessels and canisters, the discharge of inert gases (cylinders, vessels, and canisters only), the final treatment/disposal or recycling of the non-inert gases, and the decommissioning, cleansing, and recycling of the cylinders/vessels/canisters, the scrap metal derived from this waste and shall assume responsibility for any additional paperwork or forms required for the transportation and disposal process.

c. The Contractor shall not remove any compressed gas cylinders, cartridges, vessels, or canisters whose contents cannot be identified/confirmed or when no known disposal method is available.  Sampling of contents for identification purposes is authorized.

d. Cylinder(s), cartridge(s), vessel(s) or canister(s) refused by the TSDF/recycler must be identified by serial number or other identifiable markings, or by an identification number assigned by the Contractor or COR.  This number must be identified in the written explanation of refusal.

e. The Contractor and Generator will handle, package, mark, label, transport, recycle, and dispose of compressed gases and compressed gas cylinders IAW 49 CFR § 173.301 General requirements for shipment of compressed gases, and DLAI 4145.25 are available at the following link:

https://www.dla.mil/Portals/104/Documents/DispositionServices/ddsr/docs/cylinderjointpub.pdf

f. The Contractor is solely responsible for all atmospheric releases of gases occurring during or resulting from performance of this contract.  The Contractor agrees to reimburse the Government for all costs incurred for accidents resulting from improper handling, gas extraction, grinding, drilling, torching, etc., of any cylinders.

g. Inert gases which are not regulated by federal, state, or local regulations as wastes or pollutants, may be discharged to the atmosphere at the Government facility or may be transported to a TSDF for discharge.  All proper safety precautions must be observed.  Cylinders containing inert gas that have inoperable valves may be vented with the use of a safety relief device IAW DLAI 4145.25, Section 7-2.

h. If CGCs are sent to a recycler, the Contractor shall perform the following prior to smelting: de-valve the cylinder and mark or stamp the cylinder “CONDEMNED” and fully render the cylinder unusable per one or more of the following methods: drilling or puncturing a hole through the cylinder of at least ½ inch in diameter in the center of the cylinder; sufficiently destroying the inlet threads to prevent installing a valve with a gas-tight seal; or, the Contractor may cut the cylinder approximately in half.  Cylinders too small for marking/stamping are required to be tagged or labeled with the words “CONDEMNED”.

i. If the Contractor decommissions/disposes cylinders that previously contained non-inert gases, the Contractor shall remove or obliterate all DOW identification markings on every cylinder prior to release.  For example, grinding or cutting out (with a cutting torch) may meet this requirement.

j. When applicable, prior to disposing of asbestos containing cylinders in an approved landfill, the Contractor shall de-valve the cylinder.  Note: This is the only method acceptable for disposal of asbestos-containing acetylene cylinders (Base CLIN CY09), which are NOT reusable.  All acetylene cylinders will be considered asbestos containing for purposes of this contract unless otherwise verified by the Contractor.

2.6 RECYCLING OR RECOVERY

Waste that is assigned a Recycle/Recovery Base CLIN beginning with “RE” is required by the Government to be recycled or recovered.  Other items, such as certain batteries, are also required to be recycled when so stated in the CLIN description.  Certificates of Recycling are only required if specified in the Price Schedule or PWS, or if CLIN 1300SV is ordered.

2.6.1 COMMODITY SPECIFIC RECYCLING/RECOVERY REQUIREMENTS

a. Fuels Blendable Materials – Fuels blending waste will contain a minimum of 2,500 BTUs per pound of waste.  For lab pack and loose pack containers having multiple small containers packed within, the BTU value is based on the overall contents, and not each individual small container.  Since BTU values are not always known, the waste profile information may be used to determine eligibility for fuels blending.  Latex paint which has not been diluted with water will be considered fuels blendable.  Oil-based paint and its paint-related materials, as well as other organic-based, flammable, combustible, and ignitable wastes, are also considered fuels blendable.  In general, for purposes of this contract, paint and paint-related materials (PPRM) will be defined as any mixture of pigment and a suitable carrier liquid that forms a closely adherent coating when spread on a surface, as well as their respective thinners and dried solids.  This may include but is not limited to paints and paint thinners, stains, varnishes, shellacs, and coatings.  It does not include such painting-related discarded items as paint brushes, paint rollers and pans, tape, PPE, sometimes referred to as paint and paint-related waste (PPRW).

(1) Base CLINs FL05–FL10 and LP14 were designed around fuels blending; a method used to blend and utilize combustible hazardous wastes in such a way as to meet energy (BTU) and chlorine specifications at Permitted cement kilns as an alternative fuel source in the manufacturing of cement.  Wastes turned in under these CLINs will be thermally destroyed and presumed by the Government to be fuels blended for energy recovery, whether ultimately processed as an energy source at a cement kiln or at an incineration facility.

b. Batteries – Battery Recycling/Disposal CLINs beginning with “BA” are only to be recycled if recycling is the only option specified in the CLIN description.  Some Base CLINs, such as BA01 (alkaline batteries), give the Contractor the option to recycle or dispose, provided the Government has not explicitly required recycling of these batteries. Recycling of batteries shall meet all the requirements of all state and federal regulations.

c. Oil Filters –The Contractor shall recycle at least 90% (by weight) of the filter and any drained oil.  Refer to description for base CLIN RE13.

d. Dental Amalgam – Base CLIN RE08

(a) When base CLIN RE08 is ordered, the Contractor shall recycle or recover contact and/or non-contact dental amalgam contaminated with fillings, capsules, and human dental tissue.  The Generator shall ensure filters, capsules, and teeth are treated with a disinfectant solution and shall provide the Contractor with a Non-Infectious Waste certification statement signed by the designated Government representative.

(b) The Contractor shall transport, handle, and recycle the dental amalgam waste per the requirements found in the JEGS.

2.7 DEMILITARIZATION

a. When waste items requiring Demilitarization (also referred to as “DEMIL”) are ordered by the Government, they may be placed on a standalone TO separate from other disposal and service CLINs, and the items will be so listed on the DD Form 1155.  The Contractor will not be authorized payment until all required Certificates of Destruction (CD) are received by the Government per Section 2.1.

b. For items requiring DEMIL, the Contractor is required to dispose of these items per the CLIN description and provide all documentation, in addition to the documentation requirements of Section 8.0 for invoicing.  The Contractor shall ensure appropriate DEMIL/destruction occurs within 90 calendar days of pickup and provide a CD within 30 calendar days following destruction.  Documentation shall be returned to the Generator and the COR unless otherwise noted.

2.7.1 OTHER MILITARY ITEMS

a. Military waste items are assigned CLINs according to the physical and chemical properties of the waste. and not all of these will be represented on the Price Schedule.

b. JEGS Hazardous Waste Military Items – When a military item meets the definition of Regulated hazardous waste, a CLIN may be assigned to the waste similarly as other hazardous wastes, e.g., flammable debris, toxic solid, etc. Military items that are deemed regulated hazardous waste do not require a CLIN that is uniquely assigned to military items for DEMIL. In some cases, for DEMIL requirements, the USG may require a CD for specific regulated hazardous military items. DEMIL-specific CLINs with a CD requirement will be used for these items.

c. JEGS Non-Hazardous Military Items – When a military waste item does not meet the definition of regulated hazardous waste and carries no Waste Codes, a CLIN may be assigned to the waste that is provided specifically to military items for DEMIL. Non-hazardous military items assigned these CLINs, e.g., NR98 require specific destruction and disposal technologies and a CD.

3.0 SPECIAL SERVICES

This section describes the requirements for special services provided to further support the DoW installation’s waste management mission. Prices for special services listed in the Price Schedule of this contract do not include costs for waste removal, transportation, disposal, or recycling, which are covered under separate Disposal/Recycling CLINs.  Standard TO minimums do not apply to special services CLINs unless specifically stated otherwise in the contract.

DoW installation representatives, including HW Generators, who order special services under this contract are solely responsible for providing direct Government oversight of the Contractor's work, e.g., initial placement of rental equipment, industrial cleaning, waste sampling, etc.

Upon completion of the special service ordered, the installation representative is strongly encouraged to complete and submit a DLA Form 2539 (Customer Verification of Services) to the COR. This form is used to document the Government's assessment of the Contractor's performance, including the quality of service and overall satisfaction level. While DLA can only encourage the submission of this form, it is an important tool for the COR and KO to maintain performance documentation provided by the end-user.

3.1 INDUSTRIAL CLEANING SERVICES (ICS), DAILY, CREW OF 2 – CLIN 1002SV

a. When CLIN 1002SV is ordered, the Contractor shall perform ICS work using a 2-person work crew and equipment identified below for one (1) standard workday in and around the Generator sites identified in this contract.

b. The term ICS for purposes of this contract refers to performing industrial-grade heated power washing (not to include hydro blasting/water blasting), scrubbing, wiping, squeegeeing, rinsing, wet vacuuming, dry vacuuming, and other means necessary to remove dirt, grease, grime and other readily and semi-readily removable materials from the internal and external hard surfaces of a broad spectrum of tanks, vats, degreasers, oil-water separators, wash racks, floors, metal floor gratings, walls, ducts, sumps, pipes, filters, filter racks, filter chambers and other equipment associated with the many operations conducted at locations covered under this contract.  This CLIN also includes collection of the resulting rinsates/waste residues into a vacuum truck and/or Government-provided containers for removal and transportation to a qualified TSDF.  Operations conducted at sites covered by this contract may include painting, paint stripping, sanding, sand blasting, grinding, milling, fabricating, solvent and vapor degreasing, parts cleaning, anodizing, metal plating, engine maintenance, wastewater treatment, oil-water separation, filtration, organic and inorganic chemical storage, hazardous and non-hazardous waste accumulation. Not all wastes generated by these operations will necessarily be turned in for removal under this contract.

c. Contractor pricing for performance of work under this CLIN will be based upon the requirement to perform industrial-grade cleaning in open and non-permit-required confined spaces to remove waste residues (not to include stains) from a broad array of hard surfaces to meet Government operational requirements using a 2-person work crew, one (1) self-contained heated power washing unit, one (1) standard vacuum truck with a 3,500-gallon minimum capacity and up to 300 feet of vacuum hose, one (1) liquid transfer pump and up to 300 feet of hosing that is capable of transferring light pumpable liquids, and all associated PPE, tools, and equipment necessary to perform the work, including setup and takedown (but not including work breaks or meal breaks), for a period of one (1) standard workday on normal business days, typically beginning at 7:00 a.m.

d. Standard time for commencement of work under this CLIN will be within five (5) business days. The date of issuance of the written TO will constitute “Day 0” (zero), and the following business day will constitute “Day 1”.  Alternative work schedules may be arranged as agreed upon by both the Contractor and Government representative.  Refer to Section 1.1.6 for a listing of Federal holidays.  An expedited commencement of ICS work may be ordered by the Government using CLIN 1099SV per Section 3.22.

e. Per OSHA 1910.146(b) and for purposes of this contract, a “Non-Permit Confined Space” means a confined space that does not contain or, with respect to atmospheric hazards, have the potential to contain any hazard capable of causing death or serious physical harm.  The Contractor will not perform work in a Permit-Required Confined Space without the proper Confined Space Entry Permit.  For work requiring a Confined Space Entry Permit, CLIN 1083SV will be ordered as deemed necessary by the Government, per Section 3.20.

f. The vacuum truck provided under this CLIN shall arrive on-site with a clean tank and hose fittings that are constructed of material that is chemically compatible with the waste to be removed and free of visible waste residue unless the truck is being used for multiple removals from the same project involving the same waste stream (in which case it must still arrive as RCRA empty per 40 CFR 261.7) and shall be subject to Government inspection prior to use.  If rejected by the Government due to unsatisfactory inspection findings, the vacuum truck shall be required to leave the installation and be replaced with a vacuum truck that meets the Government’s requirements.  If such a replacement vacuum truck is necessary, any consequential delay of the work performed under this CLIN shall be cause for KO notification via the COR and may result in cancellation/non-payment for the CLIN or may require compensatory work beyond the scheduled hours of work to fulfill the requirements set forth in this Section.

g. Additional requirements such as weekend work, permit-required confined space entry, a super vacuum (or “super sucker”) truck, hydro blasting/water blasting equipment and crew, and expedited commencement of ICS work will each be ordered separately under the appropriate Additional Charge or Surcharge CLIN (only when offered under this contract) in conjunction with CLINs 1002SV-1004SV.  Requirements for supplied containers or rental equipment such as drums, totes or vacuum boxes will also be ordered separately using the appropriate supply or rental CLIN(s).

h. Work performed under this CLIN does not include the transportation or disposal of any waste.  Contractor pricing for transportation and disposal of the resulting waste residues from the ICS worksite to the TSDF shall be incorporated in the disposal price under the appropriate disposal CLIN in the price schedule.  The Contractor shall provide pricing accordingly.

i. Disposal CLINs shall be ordered by the Government in conjunction with CLINs 1002SV-1004SV, as appropriate to the resultant waste being removed for disposal.  If a vat or tank should contain waste that must first be removed prior to commencement of ICS work, the same vacuum truck may be used for the dual purpose of removing the original waste and collecting the cleaning residue – only when doing so does not require a change in the disposal CLIN that will be needed for disposal of the resultant (combined) waste.  Under this scenario, the 1,500-gallon minimum volume for bulk liquid waste removal does not apply.  A separate vacuum truck or conveyance vehicle will be required for removal of waste that was originally generated by the Government if by using a single, dual-purpose vacuum truck will result in a disposal CLIN that is different than that of the original waste, or if by using a single vacuum truck will negatively impact or delay the work ordered under CLINs 1002SV-1004SV.  Under this scenario, the 1,500-gallon minimum for bulk liquid waste removal does apply to the removal of the original waste but does not apply to the cleaning residue/rinsate removed by the vacuum truck in the performance of ICS work. The Contractor is required for providing any spill containment equipment, should that be necessary during ICS performance.

j. The Government will predetermine and notify the Contractor of ICS work to be performed and may order work at multiple locations on the same installation under a single ICS CLIN.

k. When performing ICS work under this contract, Government personnel shall provide a clear demarcation of work areas using appropriate signage, safety boundary markings, traffic control, and removal and replacement of floor gratings (where present) in areas where work will be required by the Contractor.

l. The Contractor shall provide full access to the Government to perform mid-work and post-work inspections for all work performed under this CLIN.  The Contractor will monitor the vacuum truck and all other equipment during their operation.  This checklist will indicate the Government’s assessment of the quality, completion, and acceptance or non-acceptance of the Contractor’s services provided under the subject TO.  The COR will transmit the checklist information to the KO for documentation of performance.

m. If a particular task or “job” cannot be completed within the standard workday(s) allotted under this CLIN, where delays were not caused by the Contractor or their equipment, and when additional work is required, the Government will order an additional quantity of this CLIN as necessary to complete the job and may use any remaining standard workday time to accomplish other predetermined ICS work on the same installation.

3.2 ICS, WEEKLY, CREW OF 2 – CLIN 1003SV

a. When CLIN 1003SV is ordered, the Contractor shall provide the same ICS work as described under Section 3.11, but for five (5) standard workdays, performed over consecutive normal business days or as otherwise agreed upon by both the Government and the Contractor.

b. The Contractor will be provided space on the installation to stage equipment, including space with containment as necessary and allowed under regulation, for temporary storage to minimize setup and takedown time when work is to proceed on the following business day.

3.3 ICS, MONTHLY, CREW OF 2 – CLIN 1004SV

a. When CLIN 1004SV is ordered, the Contractor shall provide the same ICS work as described under Section 3.11, but for twenty (20) standard workdays, performed over consecutive normal business days or as otherwise agreed upon by both the Government and the Contractor.

b. The Contractor will be provided space on the installation to stage equipment, including space with containment as necessary and allowed under regulation, for temporary storage to minimize setup and takedown time when work is to proceed on the following business day.

3.4 ICS, DAILY, CREW OF 3 – CLIN 1007SV

a. When CLIN 1007SV is ordered, the Contractor shall perform ICS work using a 3-person work crew and equipment identified below for one (1) standard workday in and around the Generator sites identified in this contract.

b. The term ICS for purposes of this contract refers to performing industrial-grade heated power washing (not to include hydro blasting/water blasting), scrubbing, wiping, squeegeeing, rinsing, wet vacuuming, dry vacuuming, and other means necessary to remove dirt, grease, grime and other readily and semi-readily removable materials from the internal and external hard surfaces of a broad spectrum of tanks, vats, degreasers, oil-water separators, wash racks, floors, metal floor gratings, walls, ducts, sumps, pipes, filters, filter racks, filter chambers and other equipment associated with the many operations conducted at locations covered by this contract.  This CLIN also includes collection of the resulting rinsate/waste residues into a vacuum truck and/or Government-provided containers for removal and transportation to a qualified TSDF.  Operations conducted at sites covered by this contract may include painting, paint stripping, sanding, sand blasting, grinding, milling, fabricating, solvent and vapor degreasing, parts cleaning, anodizing, metal plating, engine maintenance, wastewater treatment, oil-water separation, filtration, organic and inorganic chemical storage, hazardous and non-hazardous waste accumulation. Not all wastes generated by these operations will necessarily be turned in for removal under this contract.

c. Contractor pricing for performance of work under this CLIN will be based upon the requirement to perform industrial-grade cleaning in open and non-permit-required confined spaces to remove waste residues (not to include stains) from a broad array of hard surfaces to meet Government operational requirements using a 3-person work crew, one (1) self-contained heated power washing unit, one (1) standard vacuum truck with a 3,500-gallon minimum capacity and up to 300 feet of vacuum hosing, one (1) liquid transfer pump and up to 300 feet of hosing that is capable of transferring light pumpable liquids, and all associated PPE, tools, and equipment necessary to perform the work, including setup and takedown (but not including work breaks or meal breaks), for a period of one (1) standard workday on normal business days, typically beginning at 7:00 a.m.

d. Standard time for commencement of work under this CLIN will be within five (5) business days. The date of issuance of the written TO will constitute “Day 0” (zero), and the following business day will constitute “Day 1”.  Alternative work schedules may be arranged as agreed upon by both the Contractor and Government representative.  Refer to Section 1.1.6 for a listing of Federal holidays.  An expedited commencement of ICS work may be ordered by the Government using CLIN 1099SV, per Section 3.22.

e. Per OSHA 1910.146(b) and for purposes of this contract, a “Non-Permit Confined Space” means a confined space that does not contain or, with respect to atmospheric hazards, have the potential to contain any hazard capable of causing death or serious physical harm.  The Contractor will not perform work in a Permit-Required Confined Space without the proper Confined Space Entry Permit.  For work requiring a Confined Space Entry Permit, CLIN 1083SV will be ordered as deemed necessary by the Government, per Section 3.20.

f. The vacuum truck provided under this CLIN shall arrive on-site with a clean tank and hose fittings that are constructed of material that is chemically compatible with the waste to be removed and free of visible waste residue unless the truck is being used for multiple removals from the same project involving the same waste stream (in which case it must still arrive as RCRA empty per 40 CFR 261.7) and shall be subject to Government inspection prior to use.  If rejected by the Government due to unsatisfactory inspection findings, the vacuum truck shall be required to leave the installation and be replaced with a vacuum truck that meets the Government’s requirements.  If such a replacement vacuum truck is necessary, any consequential delay of the work performed under this CLIN shall be cause for KO notification via the COR and may result in cancellation/non-payment for the CLIN or may require compensatory work beyond the scheduled hours of work to fulfill the requirements set forth in this Section.

g. Additional requirements such as weekend work, permit-required confined space entry, a super vacuum (or “super sucker”) truck, hydro blasting/water blasting equipment and crew, and expedited commencement of ICS work will each be ordered separately under the appropriate Additional Charge or Surcharge CLIN (when offered under this contract) in conjunction with CLINs 1007SV-1009SV.  Requirements for supplied containers or rental equipment such as drums, totes or vacuum boxes will also be ordered separately using the appropriate supply or rental CLIN(s).

h. Work performed under this CLIN does not include the transportation or disposal of any waste.  Contractor pricing for transportation and disposal of the resulting waste residues from the ICS worksite to the TSDF shall be incorporated in the disposal price under the appropriate disposal CLIN in the price schedule.  The Contractor shall provide pricing accordingly.

i. Disposal CLINs shall be ordered by the Government in conjunction with CLINs 1007SV-1009SV, as appropriate to the resultant waste being removed for disposal.  If a vat or tank should contain waste that must first be removed prior to commencement of ICS work, the same vacuum truck may be used for the dual purpose of removing the original waste and collecting the cleaning residue – only when doing so does not require a change in the disposal CLIN that will be needed for disposal of the resultant (combined) waste.  Under this scenario, the 1,500-gallon minimum volume for bulk liquid waste removal does not apply.  A separate vacuum truck or conveyance vehicle will be required for removal of waste that was originally generated by the Government if by using a single, dual-purpose vacuum truck will result in a disposal CLIN that is different than that of the original waste, or if by using a single vacuum truck will negatively impact or delay the work ordered under CLINs 1007SV-1009SV.  Under this scenario, the 1,500-gallon minimum for bulk liquid waste removal does apply to the removal of the original waste but does not apply to the cleaning residue/rinsate removed by the vacuum truck in the performance of ICS work.

j. The Government will predetermine and notify the Contractor of ICS work to be performed and may order work at multiple locations on the same installation under a single ICS CLIN.

k. When performing ICS work under this contract, Government personnel shall provide a clear demarcation of work areas using appropriate signage, safety boundary markings, traffic control, and removal and replacement of floor gratings (where present) in areas where work will be required by the Contractor.

l. The Contractor shall provide full access to the Government to perform mid-work and post-work inspections for all work performed under this CLIN.  The Contractor will monitor the vacuum truck and all other equipment during their operation.  Government entities receiving services under this CLIN will monitor all work that is conducted at their respective operational areas and will provide the COR from the local DLA Field Office with a DLA Form 2539, Customer Verification of Services. This checklist will indicate the Government’s assessment of the quality, completion, and acceptance or non-acceptance of the Contractor’s services provided under the subject TO.  The COR will transmit the checklist information to the KO for documentation of performance.

m. If a particular task or “job” cannot be completed within the standard workday(s) allotted under this CLIN, where delays were not caused by the Contractor or their equipment, and when additional work is required, the Government will order an additional quantity of this CLIN as necessary to complete the job and may use any remaining standard workday time to accomplish other predetermined ICS work on the same installation.

3.5 ICS, WEEKLY, CREW OF 3 – CLIN 1008SV

a. When CLIN 1008SV is ordered, the Contractor shall provide the same ICS work as described under Section 3.14, but for five (5) standard workdays, performed over consecutive normal business days or as otherwise agreed upon by both the Government and the Contractor.

b. The Contractor will be provided space on the installation to stage equipment, including space with containment as necessary and allowed under regulation, for temporary storage to minimize setup and takedown time when work is to proceed on the following business day.

3.6 INDUSTRIAL CLEANING SERVICES (ICS), MONTHLY, CREW OF 3 – CLIN 1009SV

a. When CLIN 1009SV is ordered, the Contractor shall provide the same ICS work as described under Section 3.1, but for twenty (20) standard workdays, performed over consecutive normal business days or as otherwise agreed upon by both the USG and the Contractor.

3.7 SURCHARGE FOR WEEKEND ICS WORK, DAILY, CREW OF 2 – CLIN 1080SV -

One (1) EA of CLIN 1080SV will be ordered only in conjunction with CLINs 1002SV-1004SV and pertains only to the added cost of overtime pay for crew of two (2), and consequential surcharge imposed by the Contractor for performing the same work as described and ordered under CLINs 1002SV-1004SV on one weekend day, i.e., either one Saturday or one Sunday.

3.8 SURCHARGE FOR WEEKEND ICS WORK, DAILY, CREW OF 3 – CLIN 1081SV

a. One (1) EA of CLIN 1081SV will be ordered only in conjunction with CLINs 1007SV-1009SV and pertains only to the added cost of overtime pay for crew of three (3), and consequential surcharge imposed by the Contractor for performing the same work as described and ordered under CLINs 1007SV-1009SV on one weekend day, i.e., either one Saturday or one Sunday.

3.9 ADDITIONAL CHARGE FOR HYDRO/WATER BLASTING WORK, DAILY, CREW OF 3 – CLIN 1082SV

One (1) EA of CLIN 1082SV will be ordered only in conjunction with CLINs 1007SV-1009SV and pertains only to the added cost and additional charge imposed by the Contractor for providing hydro blasting/water blasting equipment and crew of three (3) for up to one (1) standard workday in place of the industrial-grade heated power washing equipment and crew for work as described and ordered under CLINs 1007SV-1009SV.

3.10 ADDITIONAL CHARGE FOR PERMIT-REQUIRED CONFINED SPACE WORK, DAILY, CREW OF 3 – CLIN 1083SV

a. One (1) EA of CLIN 1083SV will be ordered only in conjunction with ICS CLINs 1007SV-1009SV and pertains only to the added cost and additional charge imposed by the Contractor for providing up to one (1) standard workday of Permit-Required Confined Space work in addition to the work described and ordered under CLINs 1007SV-1009SV.

b. The Contractor shall prepare the confined space entry permit and coordinate work with the COR and the installation Health & Safety Department.

3.11 ADDITIONAL CHARGE FOR SUPER VAC TRUCK WORK, DAILY, CREW OF 3 – CLIN 1084SV

One (1) EA of CLIN 1084SV will be ordered only in conjunction with CLINs 1007SV-1009SV and pertains only to the added cost and additional charge imposed by the Contractor for performing up to one (1) standard workday of work employing a super vacuum (or “super sucker”) truck and crew in place of a standard vacuum truck and crew for work described and ordered under CLINs 1007SV-1009SV.

3.12 SURCHARGE FOR EXPEDITED COMMENCEMENT OF ICS WORK, CREW OF 2 or 3 – CLIN 1099SV

a. One (1) EA of CLIN 1099SV will be ordered only in conjunction with CLINs 1002SV-1004SV and 1007SV-1009SV and pertains only to the surcharge price imposed by the Contractor for the expedited commencement of work as described and ordered under CLINs 1002SV-1004SV and 1007SV-1009SV.

b. Commencement of work under CLIN 1099SV shall begin no later than 7:00 a.m. or an agreed upon time by both Contractor and Government on the first normal business day after a 48-hour lapse in time has transpired following the issuance date and time of the written TO.  Written TOs will only be issued on normal business days.  Specifically,

(1) If issuance of a written TO occurs on a Monday, commencement of work under CLIN 1099SV will begin no later than 7:00 a.m. or an agreed upon time by both Contractor and Government on the following Thursday, provided Thursday is not a Federal Holiday.

(2) If issuance of a written TO occurs on a Tuesday, commencement of work under CLIN 1099SV will begin no later than 7:00 a.m. or an agreed upon time by both Contractor and Government on the following Friday, provided Friday is not a Federal Holiday.

(3) If issuance of a written TO occurs on a Wednesday, Thursday, or Friday, commencement of work under CLIN 1099SV will begin no later than 7:00 a.m. or an agreed upon time by both Contractor and Government on the following Monday, provided Monday is not a Federal Holiday.

3.13 PERFORM SAMPLING AND ANALYSIS

a. When laboratory analysis is required for purposes of waste identification, it is ordered to service Generators on this contract.  It is not intended to supplant the Contractor obligations designated elsewhere in this contract.

b. The Government will order this service using any of the analytical CLINs listed in the Price Schedule.  The unit of issue for analytical CLINs is each on the TO.  One each (1 EA) includes one waste stream to be sampled using laboratory-provided containers, shipment of samples to the subcontracted laboratory, laboratory analysis, and analytical reporting.  The Contractor shall:

(1) Collect a representative sample IAW 40 CFR § 261, Appendix I, and perform testing using methods identified in 40 CFR § 260.11 or other appropriate methods.  Note: The Government may choose to collect the representative sample using laboratory-provided sample containers (provided by the Contractor).

(2) Complete the Chain of Custody form and provide a copy to the COR.

(3) Provide a statement from the laboratory indicating that any excess sample remaining after analysis was properly disposed of.  This statement may be provided separately or included in the analytical report.

c. The Contractor will perform the analysis and provide a complete analytical report to the COR within 15 business days following issuance of a written TO per Section 7.0, unless otherwise stated in the contract.  For example, the standard performance period under CLIN 1002SA is five (5) business days.  The first full business day following issuance of a written TO will constitute Day 1.

d. In all cases where the Government is paying for laboratory analysis, a complete and comprehensive analytical report shall be provided to the Government.  The report shall include a completed Chain of Custody Form and detailed analytical results, including QA/QC results as required by current GOJ protocol or as otherwise applicable.  If for any reason the laboratory report is not provided to the Government or is deemed not scientifically defensible by the Government, a reanalysis of the waste will be required at no additional cost to the Government.  Expedited analyses will only be ordered when the Contractor can meet the requirement.

e. The Contractor will be required to collect sample(s) from various collection points, including but not limited to containers, piles, tanks, surface water, storm water, sumps, ROB’s, etc.  The Contractor will not be required to collect samples under hazardous conditions or from a collection point that is in a permit-required confined space, unless the Contractor is already performing work in these areas.  The Contractor will not be required to provide a ladder, man lift, or other such equipment needed to achieve access to the sampling point.

f. The Contractor will collect a representative sample(s) of sufficient quantity required to perform all ordered analyses, provide proper preservation, meet sample holding times, temperature, and storage requirements.  Failure to comply with these requirements may require that additional sampling and analysis be performed at no additional expense to the Government, as determined by the KO.

3.14 PREPARE HAZARDOUS WASTE PROFILE SHEET - CLIN 1301SV

a. When CLIN 1301SV is ordered, the Contractor is required to prepare a typed Hazardous Waste Profile Sheet (HWPS), DLA Form 2511 provided by the Government, even when the subject waste does not meet the JEGS definition of HW.  One each (1 EA) will be ordered per waste stream being profiled.  Using analytical data and/or information provided by the Generator, the Contractor will complete the form IAW attached instructions.   Subject to COR approval, the Contractor may use an equivalent waste profile form, provided the alternate form minimally contains the same information contained in the DLA Form 2511.

b. The purpose of this service is to enable the Generator to turn in items to the Contractor for removal.  It is not intended to supplant Contractor obligations identified in other Sections of this contract.

3.15 PERFORM WASTE CHARACTERIZATION TO IDENTIFY WASTE (D001-D043) AND PROVIDE HWPS – CLIN 1001SA

a. When CLIN 1001SA is ordered, all provisions described and ordered under Section 2.2.3 shall apply and, additionally, the Contractor will provide to the Government a Hazardous Waste Profile Sheet (HWPS) – DLA Form 2511 (or equivalent).

b. When the Government has a requirement for the purposes of proper waste management for identifying a solid waste per 40 CFR § 261.2 or JEGS, whether of certain or uncertain origin and composition, a written TO will be issued using CLIN 1001SA.  The Contractor will collect samples in laboratory-supplied sample containers according to JEGS protocol, and ship properly preserved samples to a credible commercial environmental laboratory to perform a complete waste characterization analysis.  The Government will provide pertinent information as available regarding the waste prior to sample collection. When CLIN 1001SA is ordered, the analysis shall include determination of:

(1) Ignitability, if free liquids are present, or solids if applicable (D001),

(2) Corrosivity (pH) if free liquids are present, (D002),

(3) Reactivity, if the potential for a reactive characteristic exists, (D003), and if such a test method is available,

(4) Toxicity, if Japanese Industrial Standards (JIS)-regulated toxic inorganic (metals) and/or organic constituents exist, as reportable by JIS test methods and protocols or U.S. EPA standard tests in lieu or in support of Japan test methods.

e. The Contractor will provide a complete analytical report and HWPS to the Government via email to the COR within 15 business days from the issuance of a written TO, (per Section 7.0) except where an expedited analysis is ordered.  The first full business day following issuance of a written TO will constitute Day 1.

3.16 BULK NON-REGULATED WASTEWATER MANAGEMENT - CLINs NR03B5 & NR05B5 Note to AO: CLIN NR03B5 should be required and offered in nearly every contract.  If NR03B5 is offered, NR05B5 must also be offered. No less than 1,500 gallons should ever be estimated for either of these CLINs.

a. When disposal CLIN NR03B5 (Bulk Non-RCRA Wastewater) is ordered, the subject waste stream must qualify for either (1) direct discharge to a publicly owned treatment works (POTW) at a State-Permitted commercial wastewater treatment plant (WWTP), or (2) treatment at a State-Permitted WWTP, followed by discharge to the local POTW, and associated filtercake landfilled at a minimum Subtitle D (or equivalent) landfill.  Oil/water phase separation is authorized as a pre-treatment in either case, whereby the oily phase (primarily oil) is ultimately processed and recycled as used oil.  Within 30 calendar days following award of this contract, the Contractor will provide the Government with detailed waste acceptance criteria from the WWTP(s), which the Contractor plans to utilize during the performance of this contract, for disposal of wastewater pertaining to CLIN NR03B5.

b. If the subject wastewater is deemed not directly dischargeable or treatable to meet Permitted discharge limits by two (if two exist) qualified WWTPs within a 50-mile radius from the point of generation, CLIN NR05B5 will be ordered for solidification and landfill.

c. A 1,500-gallon minimum order will apply to all bulk liquid removals unless otherwise stated elsewhere in this contract.

d. The Government will not select CLINs NR03B5 and NR05B5 for the turn-in of AFFF and other PFAS-related wastes.  Refer to Section 2.4.

3.17 PERFORM OVERPACKING/LOOSE PACKING SERVICES – CLINs 1202SV-1207SV

The Government may order Contractor services to perform overpacking or loose-packing of containers for such reasons as containing a leaking container, precautionary packaging to prevent leaking, assurance of proper packaging for TSDF acceptance, and for convenience in general.  While overpacking and loose packing serve different purposes, both require the placement of smaller container(s) into a larger container.  As such, both services are provided under common CLINs based on the outer container sizes.

a. An overpack drum may be used as a precautionary measure to preemptively contain waste, should the primary container(s) pose a potential to leak.  However, a salvage drum will be used for packaging a leaking, damaged, or defective package(s), per 49 CFR 173.3(c).  The salvage drum must be compatible with the lading in the leaking/damaged package(s).  When overpacking of container(s) is ordered by the Government under CLINs 1202SV-1206SV, the Contractor is responsible for utilizing the appropriate container size, type, materials, markings, and labeling.  Inner container(s) shall be kept in an upright position when practicable.  Overpack and salvage drums that are used for liquids shall be marked with directional arrows indicating the orientation of the inner container(s).

b. A loose pack container is an overpack of smaller containers, typically consumer commodity products of the same type, e.g., paints, lubricants, adhesives, expired products, etc. and provides the best alternative when consolidation is impractical.  The loose pack container shall be of the appropriate UN rating for the lading contained in the drum.  When loose packing of container(s) is ordered by the Government under CLINs 1202SV-1207SV, the Contractor is responsible for ensuring the appropriate container type, size, materials, markings, and labeling are utilized.

c. The Contractor shall utilize an appropriate container provided by the Government.  If the Government does not possess an appropriate container for the waste(s) being packed, the Government shall order the container(s) needed to perform the work on the Task Order.

d. Services provided under CLINs 1202SV-1207SV (below) do not include transportation or disposal of the resulting containers.  Removal and disposal of overpack and loose pack containers shall be separately ordered under the appropriate disposal CLINs on the same or separate TO.

| CLIN | Overpack/Loose Pack Service Description |
| --- | --- |
| 1202SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Generator-provided overpack container up to five (5) gallons in size. |
| 1203SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Generator-provided overpack container up to 15 gallons in size. |
| 1204SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Generator-provided overpack container up to 30 gallons in size. |
| 1205SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Generator-provided overpack container up to 55 gallons in size. |
| 1206SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Generator-provided overpack container up to 110 gallons in size. |
| 1207SV | The unit of issue for CLIN 1207SV is each (EA). One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to loose pack smaller containers into one (1) Generator-provided cubic yard box or military tri-wall box. |

4.0 ACCEPTABLE PERFORMANCE LEVEL (APL)

a. Acceptable Performance Level (APL) is the minimum performance of each requirement before the Government considers performance unsatisfactory.  Specification of an acceptable performance level does not allow the Contractor to knowingly provide defective service; it is recognition of the fact that defective performance may sometimes occur unintentionally.  If the performance percentage does not fall below the specified acceptable performance level, the Government will not deduct for poor performance.  However, the Contractor shall be required to re-perform or correct the defective service or product at no additional cost to the Government.

b. The surveillance method shall be from Collection Summary Report(s) (CSR) furnished by the COR, customer feedback including but not limited to DLA Form 2539, Customer Verification of Services and tracking of final disposal/recycling submittals.  All negative and positive actions/performance shall be the basis for monetary deductions and/or past performance annotations.

c. Performance incentives for the Contractor shall include, but are not necessarily limited to, favorable impact on option exercise decisions, future awards of the same requirement, and future awards of similar requirements by DLADS as well as other Government and non-Government entities that may review performance of this contract.

4.1 ACCEPTABLE PERFORMANCE LEVEL (APL) TABLE

| DELIVERABLE | STANDARD | ACCEPTABLE PERFORMANCE LEVEL (APL) | METHODS OF SURVEILLANCE |
| --- | --- | --- | --- |
| Timely Performance of Services | Performance of services within the timeframes specified in Section 7 – Standard Times for Removals and Performance of Special Services. | No more than 3 occurrences during a 12-month period. | 100% Review of documentation utilizing the Quality Assurance Surveillance Plan (QASP) and the Customer’s Verification of Services (DLA Form 2539) if applicable. |
| Compliant Pre-Removal | The Contractor completes all requirements, documents and notifications IAW Section 1.1 – Pre-Removal. | No more than 5 occurrences where submissions were returned for missing or incorrect data. | 100% Review of documentation utilizing the Quality Assurance Surveillance Plan (QASP) and the Customer’s Verification of Services (DLA Form 2539) if applicable. |
| Compliant Removal | The Contractor arrived on time and with the required equipment and in good working order to safely complete removal or service IAW Section 1.2 - Removal. | No more than 3 occurrences where Contractor did not meet these requirements during a 12-month period. | 100% Review of documentation utilizing the Quality Assurance Surveillance Plan (QASP) and the Customer’s Verification of Services (DLA Form 2539) if applicable. |
| Compliant Post-Removal | The Contractor provided all required disposal documentation IAW Section 1.3 – Post Removal. | No more than 5 occurrences where submissions were returned for missing or incorrect data. | 100% Review of documentation utilizing the Quality Assurance Surveillance Plan (QASP) and the Customer’s Verification of Services (DLA Form 2539) if applicable. |

The Government reserves the right to conduct additional surveillance/review of final tracking information and/or delay payments until after complete and accurate tracking information is received, as warranted, when performance problems occur in this area.

5.0 QUALIFIED FACILITIES AND TRANSPORTERS LISTS (QFL/QTL)

a. The Contractor will only utilize facilities and transporters listed on the Qualified Facilities List (QFL) and Qualified Transporters List (QTL) when performing removals for recycling or disposal services.  The QFL and QTL is located on the DLA Disposition Services Website at the below link:

https://www.dla.mil/DispositionServices/Offers/Disposal/HazardousWaste/HazWasteDisposal.aspx.

b. The Contractor is responsible for ensuring their services and subcontractor services are performed IAW:

(1) The contract terms and conditions, and

(2) All federal, state, local, and host nation laws and regulations including Overseas Baseline Guidance Document (OEBGD), Final Governing Standards (FGS), Contingency Location Environmental Standards (CLES), and Basel Convention if applicable.

c. When a firm is listed on the QFL and QTL, DLA Disposition Services is not declaring that the facility and transporter meet the requirements of this contract or that the facility and transporter provides services IAW all applicable laws, regulations, and standards.  COR approval of shipping paperwork also does not absolve the Contractor from ensuring their services or subcontractor services meet these requirements.  If firms listed on the QFL or QTL do not meet contract terms and conditions or are unable to perform IAW all applicable laws, regulations, and standards, the Contractor will propose additional firms that meet these requirements.

d. Any damages, spills, contamination, violations and/or fines resulting from Contractor or subcontractor provided services will be the responsibility of the Contractor to correct such issues to the satisfaction of the Government at no additional cost to the Government.  The Contractor will provide the Government with proposed corrective actions, and the Government will determine if the corrective actions are sufficient.  The Contractor will reimburse the Government for any fines that result from the Contractor or Subcontractor services.  The Contractor will also reimburse the Government for paid services that result in damages, spills, contamination, violations and/or fines.

e. For supply services where waste is not being shipped, those transporters are not required to be on the QTL, e.g., transporters dropping off empty roll off boxes are not required to be on the QTL.

f. Transporters relocating waste within a DoW installation are required to be on the QTL.

5.1 ADDING/UPDATING QUALIFIED FACILITIES AND TRANSPORTERS

a. TOs bearing disposal CLINs that haven’t been assigned in the Hazardous Material Management System (HMMS) to their designated TSDF EPA ID Number cannot be issued until DLA Disposition Services has completed the assignments for those CLINs going to that TSDF. Most well-established TSDFs in the U.S. have associated CLINs already assigned them through other DLA contracts.  However, when a new or previously unused TSDF is added to the QFL, CLINs that are destined for TSDF must first be assigned to the TSDF in HMMS by DLA Disposition Services. To accomplish this, the Contractor will complete a Facility-CLIN Assignment Sheet referencing potential Base CLINs destined for the TSDF Name and EPA ID# during the life of the contract. The Contractor will email the completed Facility-CLIN Assignment Sheet to TSDFandTransporterInquiries@dla.mil and cc: the KO.  This is only required when the designated TSDF has never received these CLINs in the past.  When the Contractor requests the addition or update of a qualified facility, the Contractor will indicate all disposal Base CLINs to the QFL Application or QFL Update form, respectively.  CLINs may be added to a qualified facility anytime during the life of the contract, provided the CLIN descriptions including permissible EPA Waste Codes are compliant with the facility’s RCRA Permit.

b. The Contractor will review the QFL and QTL prior to each removal to ensure the facilities and transporters are listed on the QFL and QTL and have the appropriate permits to perform the services they provide

c. The awarded prime Contractor will be the only entity to request additions and updates to the QFL and QTL.

d. Contractor inquiries and applications for additions and updates will be sent to the following email address with a cc: to your KO and COR: TSDFandTransporterInquiries@dla.mil.

e. The Contractor will complete DLA Form 2507, Application for Qualified Facility List (QFL) to add new or update existing facilities. The most current version of this form is available at https://www.dla.mil/Forms/.

f. The Contractor will complete DLA Form 2503, Application for Qualified Transporter List (QTL) to add new or update existing transporters. The most current version of this form is available at https://www.dla.mil/Forms/.

g. If the Contractor requests or updates a facility or transporter where required documents are not in English, the Contractor will provide the original supporting documents in the host nation language with English translated documents at no additional cost to the Government.  All translations must be typed and legible.  Documents must contain a translation certification but can be self-certified by the Contractor without using a professional translation service.  The person who translates the document into English for the Contractor must attach a formal letter stating that they:

(1) Are qualified to translate the document because they are competent in both English and the document’s original language.

(2) Translated the document accurately and completely to the best of their abilities.

(3) In addition, the letter must include the following information about the translator:

- Full name of translator

- Translator’s signature

- Translator’s address

- Date the translator wrote the letter

- Translator’s citizenship

The Government reserves the right to reject any translation and may require translation from a professional translation service at no additional cost to the Government.

h. The proposed facility or transporter will not be added or updated to the QFL and QTL if requested documentation is not provided or if any of the reasons in section 5.2 through 5.3 apply.

i. The request by the Contractor to have a facility or transporter added to the QFL and QTL after contract award does not relieve the Contractor of their obligation to perform IAW existing CLIN prices.

j. The rejection/removal of a facility or transporter from the QFL/QTL does not relieve the Contractor from performing all contract requirements IAW existing CLIN prices.

5.2 FACILITY REMOVAL

At any time during the period of this contract, the Government will remove a facility from the QFL if any of the following apply:

a. The facility is no longer in operation.

b. The facility is a significant non-complier (including, but not limited to, RCRA Class 1 violations, OEBGD/FGS/CLES non-compliance, or host nation significant environmental violations) and has not entered a compliance schedule or similar action.

c. The facility received an administrative order or judicial action and has not entered into a compliance schedule or similar action within 180 days from time of issued order or judicial action.

d. The facility has a history of noncompliance (including, but not limited to OEBGD/FGS/CLES non-conformance, RCRA class I and II violations, OSHA violations, state and local violations, host nation violations, etc.) or exhibits a lack of "good faith" in correcting 	violations. A "good faith" effort is promptly signing a consent agreement with the regulatory authorities and performing in compliance with the agreement for at least six months.  Repeated violations indicate a lack of "good faith".

e. The facility has groundwater contamination or does not meet groundwater anti-degradation policy.

f. The facility does not have adequate permits or is not capable of handling the proposed waste.

g. The facility received a negative recommendation resulting from a DLA Disposition Services site visit or a third-party audit conducted on DLA Disposition Services behalf without substantive evidence of corrected deficiencies.

h. The facility transports DoW waste to a facility not on the QFL.

i. The facility's financial assurance is not sufficient to protect the Government's long-term interests.

j. The facility is unable to demonstrate tracking of DoW waste from entry to exit.

k. The facility manages property in a manner that causes the generator of the manifest to file exception reports IAW 40 CFR 262.42 or state/host nation equivalent.

l. The facility has not received DLA Disposition Services wastes for a period of twelve consecutive months.

m. Requested facility documentation is not provided.

n. The facility operates in a manner that is not IAW this contract.

o. The facility does not have or does not enforce 24-hour security measures.

p. The facility does not allow or limits access for a site visit by DLA Disposition Services personnel or representatives.

q. The facility did not produce certified English translations for foreign language documents.

r. The facility caused damages, spills, contamination, and/or fines while providing services for this contract.

s. The Government reserves the right to disapprove any facility when it is in the public interest to do so.

5.3 TRANSPORTER REMOVAL

At any time during the period of this contract, the Government will remove a transporter from the QTL if any of the following apply:

a. The transporter does not have the appropriate hazardous or non-hazardous Federal/state/local/host nation permits to transport property under this contract.

b. Transporter has an "unsatisfactory" or “conditional” rating from DOT Office of Motor Carriers (OMC).  For OCONUS, the transporter has exhibited a history of noncompliance.

c. The transporter has exhibited a history of acute/critical violations in the Federal Motor Carrier Safety Administration (FMCSA) Safety Management System (SMS) and there is no get-well plan or the get-well plan milestones have been missed.  This is not applicable to 	OCONUS.

d. The transporter received an administrative order or judicial action and has not entered into a compliance schedule or similar action within 180 days from the time the order or issued judicial action.

e. The transporter does not have or does not enforce 24-hour security measures.

f. The transporter manages property in a manner that causes the generator of the manifest to file exception reports IAW 40 CFR 262.42 or state/host nation equivalent.

g. Transporter transfer locations are unable to track property from entry to exit.

h. The transporter has not shipped DLA Disposition Services wastes for a period of twelve consecutive months.

i. The transporter co-mingles DOW and non-DOW waste prior to receipt at the manifested facility.

j. The transporter operates in a manner that is not IAW this contract.

k. Requested transporter documentation is not provided.

l. The transporter did not produce certified English translations for foreign language documents.

m. The transporter caused damages, spills, contamination, and/or fines while providing services for this contract.

n. The Government reserves the right to disapprove any transporter when it is in the public interest to do so.

6.0 ADDITIONAL REQUIREMENTS

6.1 INDEMNIFICATION

a. Upon receipt/removal of items from the various Government installations, the Contractor assumes full accountability and physical custody of such items.

b. DLA, its officials, agents, Contractors, subcontractors, employees, and representatives shall be held harmless from all claims or causes of action arising from, or on account of, negligent or other wrongful acts, omissions of the Contractor, its officers, directors, employees, 	agents, or subcontractors, in performing the Work pursuant to this contract.  In addition, the Contractor agrees to pay the United States all costs incurred by the United States, including but not limited to attorney’s fees and other expenses of litigation and settlement, arising 	from or on account of claims made against the United States based on negligent or other wrongful acts or omissions of the Contractor, its officers, directors, employees, agents, subcontractors and any persons acting on its behalf or under its control, in performing the Work 	pursuant to this contract.

c. The Contractor also agrees to hold the Government harmless and indemnify the Government for any and all costs, including those that arise from violation of any law, regulation, order, right, or public or private enforcement program under which the government incurs 	environmental clean-up or response costs, judgments, action, debt, liability costs and attorney’s fees or any other requests for monies or any other type of relief arising from or incident to the work performed pursuant to this contract.

d. The United States shall not be held out as a party to any contract entered into by or on behalf of the Contractor in performing the Work pursuant to this Contract.  Neither Contractor nor any such subcontractors shall be considered an agent of the United States.

e. The United States shall give Contractor notice of any claim for which the United States plans to seek indemnification pursuant to this Section and shall consult with Contractor prior to settling such claim.

  - PLACES OF GOVERNMENT INSPECTION

a. All services will always be subject to inspection by the KO and his/her authorized representative(s) to include Government contracted audit companies.  The Government and/or its representatives shall have the right to inspect and obtain copies of all written licenses, 			permits, and approvals issued by any Governmental entity or agency to the Contractor or its subcontractors which are applicable to the performance of services under this contract; to inspect and test, at its own expense, transportation vehicles or vessels, containers, and 			disposal facilities provided by the Contractor; and to inspect the handling, loading, transportation, storage, and disposal operations conducted by the Contractor or its subcontractors in the performance of this contract.

b. The Government and/or its representatives shall be afforded free access to any facility used by the Contractor and any subcontractors in performing services under this contract, including offices and facilities where contract-related records are retained.  Government 		inspections of Contractor facilities may be scheduled or unscheduled, i.e., announced, or unannounced.  The purpose of these inspections is to assist the Government in determining the conformance of services with contract requirements.

c. The Contractor is solely and exclusively responsible for the quality of all services performed under this contract.  The Government's right to conduct inspections at Government, Contractor, or subcontractor facilities does not relieve the Contractor of this responsibility.  	Neither Government failure to make such inspection, nor failure to discover nonconforming services, shall prejudice the rights of the Government thereafter to reject services, nor relieve the Contractor of its obligation to perform work strictly IAW the contract.

d. The Contractor, in its agreements with subcontractors, shall ensure that the inspection rights described herein are afforded the Government and/or its representatives by each subcontractor performing services under this contract.

e. As part of the U.S. Government’s right to conduct inspection of services under this contract, it shall be allowed to obtain information from host nation regulators that may be relevant to assessing the Contractor’s history of, or current, compliance with host nation 	environmental law and regulations.

6.3 CONTRACTOR REPRESENTATIVE

The Contractor shall provide in the spaces below the name, title, and telephone number(s) of at least one responsible individual who will serve to respond to problems occurring during transportation, problems at TSDFs and recycling facilities having received waste items from the Contractor under this contract, and emergencies 24 hours per day, 7 days per week, and 365 days per year, or 366 days per year in leap years.

The Contractor agrees that notice to the Contractor representative(s) listed constitutes notice to the Contractor and agrees to be bound by any commitments or representations made by this representative.

NAME:

TITLE:

PHONE NUMBER:

24 HOUR EMERGENCY NUMBER:

6.4 INCIDENTAL FEES

Any incidental state or local fee, tax or penalty assessed against DOW/DLADS arising from the Contractor’s decisions in the performance of this contract will be paid by the Contractor, including, but not limited to: Any fee, tax, or penalty levied because of a contractor’s failure to prepare and provide reports and/or documents required as part of the performance of this contract.

7.0 STANDARD TIMES FOR WASTE REMOVALS AND PERFORMANCE OF SERVICES

a. The standard time for removal of containerized waste under this contract is twenty (20) business days, unless otherwise specified in this contract.

b. The standard time for removal of bulk waste under this contract is five (5) business days, unless otherwise specified in this contract.

c. All waste removals shall be completed by close of business on the last day of the required removal time.  The clock for waste removal begins on the first full business day following issuance of the written Task Order.

d. The standard time for performance of services under this contract is three (3) business days, unless otherwise specified in this contract.  The Contractor shall begin performance of services by close of business on the last day of the required performance time.  The clock for performance of services begins on the first full business day following issuance of the written Task Order.  Equipment rental and ICS related services are among those services that require a different performance of services time.  The Contractor must perform in 	accordance with specifications regarding all services.

e. Time sensitive removals and performance of services include but are not limited to the following:

| PERFORMANCE TIME | PERFORMANCE REQUIREMENT |
| --- | --- |
| 5 business days | - Standard removal of bulk waste (Section 1.2.8.1) - Commencement of ICS work (Section 3.1-3.12) |
| 20 business days | - Standard removal of containerized waste (Section 1.2.8.2) - Perform Standard analysis (Section 3.13, 3.15) |

    - For excusable delays, refer to Section 1.1.4.c.

7.1 EXPEDITED REMOVALS AND PERFORMANCE OF SERVICES

| CLINS | DESCRIPTION | PAPERWORK SUBMISSION |
| --- | --- | --- |
| 1095SV | 1-Business Day Expedited Removal/Performance | One Hour Prior to Removal/Performance |
| 1096SV | 3-Business Day Expedited Removal/Performance | One (1) Business Day Prior to Removal/Performance |
| 1097SV | 5-Business Day Expedited Removal/Performance | Four (4) Business Days Prior to Removal/Performance |
| 1098SV | 10-Business Day Expedited Removal/Performance | Five (5) Business Days Prior to Removal/Performance |
| 1099SV | Expedited Commencement of ICS Work, Crew of 2 or 3 | Based on day of week TO is issued (Section 3.22) |

8.0 DOCUMENTATION REQUIREMENTS, DLA DISPOSITION SERVICES ACCEPTANCE, INVOICING, AND TRACKING

8.1 SUBMISSION OF DOCUMENTATION TO FACILITATE ACCEPTANCE FOR PAYMENT

a. The Contractor shall submit one invoicing packet, via email, for all line items listed on the Task Order (TO) after waste has been received at the first TSDF/Recycler as designated on the pickup manifest, services have been completed, and/or supplies have been delivered, except for regulated PCBs, DEMIL-required waste, and other waste requiring a CD which require tracking to the final destruction/disposal facility.

b. PRIOR to submission of invoicing packet, the Contractor shall coordinate with the COR to ensure:

(1) Return copies of all shipping papers have been received by the COR.

(2) All modifications, changes, certifications, e.g., certification of destruction/disposal/recycle, and/or updates have been completed and/or received by the Government.

c. An invoice packet shall consist of:

(1) Invoice (DLA Form 2505)

(a) Must be submitted using the Microsoft Excel file provided as part of this contract.  Scanned copies or other formats are not acceptable and will result in rejection of the payment request.

(b) Must be in Line-Item order.

(c) Complete appropriate fields and columns 1 through 11 for the supplies or services provided per the instructions.

(d) Must reflect all changes on MOD paperwork.

(2) Pick-up Report (DD Form 1155)

(a) A short-form Business Objects report may be used in lieu of the original Pick-up Report provided it includes the 1st and 2nd pages of the original DD form 1155.

(b) Must be in Line-Item order.

(c) All data fields must be completed, and all applicable signature fields signed.

(d) Must be legible and accurate.

(e) Must provide full manifest/document number.

(f) Signed copies of all modifications (if applicable).

(3) All return shipping documents, which include but are not limited to, Hazardous Waste Manifests and Non-Hazardous Waste Manifests, must be complete, accurate and legible, and will include:

(a) Correct EPA ID number(s) for the Generator, Transporter, and the TSDF/Recycler (or a Mock EPA ID number when no EPA ID number exists).

(b) The contract and task order number.

(c) Signatures and dates from the Generator, the transporter, and the accepting facility.

d. The invoicing packet shall be submitted to DLADispSvcsManifesttracking@dla.mil using the following guidance:

(1) Subject line shall be formatted Contract Number/Task Order Number, Company Name (e.g., SP450017D0012/18F0012, Waste Pickup Co.).

(2) Attachments shall be named Contract Number/Task Order Number (e.g., SP450017D0012/18F0012).

(3) ONLY packets that exceed 10MB in size are authorized to be split into separate emails using the following guidance:

(a) Emails shall be numbered at the end of the subject line to identify a multiple email submission (e.g., SP450017D0012/18F0012, Waste Pickup Co. 1/3).

(b) Attachment names shall receive a lowercase letter as a suffix to identify a multiple attachment packet (e.g., SP450017D0012/18F0012a).

(4) Each email submitted shall contain only one file.

e. Packets received by the tracking department that do not meet these requirements will be rejected.

1. Rejected packets must be resubmitted in their entirety.

2. Reviews will not be continued after the first discrepancy is identified.

8.2 ACCEPTANCE AND INVOICING

a. Except for regulated PCBs and DEMIL required TOs, for purposes of payment only, all disposal/recycling CLINs are considered disposed/recycled on the date the items are accepted by the first, properly permitted, qualified TSDF/Recycler. For regulated PCBs and DEMIL, for purposes of payment, disposal/demilitarization for all property is considered completed when final disposal/demilitarization has occurred. For service “1000 and 6000 Series” CLINs, the services are considered complete on the date the services are completed and/or supplies are delivered.  For service CLIN 6503, the service will be considered complete once the Government receives the certification(s) ordered from the Contractor. The Government period for acceptance of the invoice package is within 20 calendar days of receipt of the Contractor’s acceptable invoicing packet that provides final Government acceptance of the services performed and/or supplies delivered.  The contractor will be paid using the Evaluated Receipt Settlement (ERS) process when their manifest tracking packet is authorized for payment in the Hazardous Material Management System (HMMS). Payment to the Contractor will be made through Electronic Funds Transfer (EFT)IAW the Prompt Payment Act.  The Prompt Payment Act is not triggered until an acceptable invoice package with supporting documentation, as described above, has been provided to the Contracting Officer.  Any missing or incorrect information or deficiencies with the invoicing package must be corrected before the Act’s payment timeframe begins.

b. Failure to submit timely and complete documentation for acceptance of payment on a Task Order will result in late payment or non-payment for services rendered.  Payment may be approved upon resubmission of the correct documentation for all line items on the Task 	Order.  The Government can, at the discretion of the KO, utilize other remedies including those indicated below.

c. Note that consideration pursuant to FAR 52.212-4(a) can be taken against any open Task Order under the contract, not necessarily the Task Order with documented deficiencies.  In addition, consideration may also be taken against another contract if there are no open task orders on the subject contract.

d. By submitting for payment IAW the above terms including any post-payment documentation requirements, the Contractor certifies that all services, including receipt at initial qualified TSDF and ultimate disposition of the items, conform in all respects with contract requirements.

e. The Contractor shall comply with the requirements of FAR Subpart 42.12 regarding a company change-of-name and agree in writing to the timeline and procedures specified by the responsible KO.  The Contractor must provide the KO with documentation to support the legally changed company name.

f. For any discrepancies in payment, contact j8bvendorpayments@dla.mil.

9.0 CONTRACT MINIMUM/MAXIMUM

This is a firm-fixed price indefinite quantity contract. The contract minimum shall be $25,000.00.  The maximum shall be 150% of the estimated value of the contract.  There is no contract minimum for any option period if included in the contract.

10.0 CONTRACTOR ACCESS TO DLA DISPOSITION SERVICES SITES

Throughout the life of the contract, the Contracting Officer may authorize the Contractor to obtain a Common Access Card (CAC) to perform work under this contract. The Contractor shall provide justification for the CAC to the Contracting Officer. If a CAC is authorized, DLA Form 1728, DD Form 1172-2, CAC Justification Form, and OF306 will be submitted to the Contracting Officer for review and approval.  If authorized issuance of a CAC, the Contractor is required to follow procedures identified in Attachment “CAC Application Process”.

11.0 PERIOD OF PERFORMANCE

This contract has one 30-month base period and one 30-month option period.  The period of performance of this contract starts from the award/effective date of contract as reflected in block 3 of the SF1449 through 30 months and any further extensions to the contract performance period.

This solicitation consists of the following documents/attachments:

Attachment 1. Performance Work Statement (PWS).

Attachment 2. Price Schedule of Services (Excel Spreadsheet).

Attachment 3. Primary Pickup/Services Locations.

Attachment 4. Japan Environmental Governing Standards (JEGS).

Attachment 5. Invoice DLA Form 2505 (Excel Spreadsheet).

Attachment 6. Customer Verification of Services, DLA Form 2539.

Attachment 7. Hazardous Waste Profile Sheet (HWPS) DLA Form 2511.

Attachment 8. USAG-J Form 1529