|  | Description | Quantity |
| --- | --- | --- |
| 1 | Similar or equal to Foster + Freeman VSC 9000 Specifications for foster + freeman VSC 9000 PRO (dual-camera) with Monitor and Spectrometer: - High-resolution, multi-spectral imaging system for forensic document examination - Spectral range: UV (254nm) to IR (1000nm) - Multiple light sources: transmitted, oblique, coaxial, and incident - 30x optical zoom camera, 20 Megapixel resolution - Software for image capture, enhancement, comparison, and archiving - Integrated OCR and measurement tools - Secure, audit-trailed data management - Touchscreen interface and remote operation capability - The form and design must adapt to a wide range of evidence types, and must contain a dual-camera imaging system with continuous zoom lens, capability for images larger than A4/US Letter size, magnification up to 480x on a 42” monitor, fully LED transmitted light base, configurable coaxial illumination set, extended access control for documents, high dynamic range (HDR) imaging, artificial intelligence assistance for MRZ. - It must contain a primary high-performance dual-camera imaging system that includes: 12 MP high-resolution, high-sensitivity camera sensor, continuous zoom lens with reduced distortion, field of view range: 1.5 x 1.15 mm to 285 x 215 mm, minimum static field of view: 1.5 x 1.15 mm, maximum static field of view: 223 x 168 mm, maximum dynamic field of view: 285 x 212 mm, maximum transmitted field of view: 166 x 215 mm, focus and iris controls. - It must contain a motorized viewing filter: high-magnification lens, visible bandpass filter (430-660 nm) to replicate the human eye response, long-pass filters: 495 nm, 515 nm, 530 nm, 550 nm, 570 nm, 590 nm, 610 nm, 630 nm, 645 nm, 665 nm, 695 nm, 715 nm, 725 nm, 780 nm, 830 nm, 850 nm, 925 nm, 1000 nm, UV pass filter (350-380 nm), linear polarizer for cross-polarized imaging. It must support UV (365 nm) excited phosphorescence decay imaging, anti-stroke imaging function. - It must include a secondary high-performance dual-camera imaging system, internal 12MP camera, motorized, field of view range: 310 x 232 mm, spectral range from 350 to 1100 nm to allow imaging of fluorescence excited by white light, IR (850 nm), and UV (365 nm). - It must have several narrow bandpass illumination sources, allowing multispectral scanning and precise color selection to illuminate a document, continuously variable in a range of 400-1000 nm in 1 nm steps, maximum emission bandwidth of 30-50 nm (depending on wavelength), 100 W bulb, adjustable bandpass spot illumination, a multifunctional spot light source that allows the user to independently select high and low wavelength limits. A useful light source for IR fluorescence, low-pass filters from 505 to 800 nm, high-pass filters: 380 to 745 nm. - 5000 K white LED flood illumination, dimmable and capable of being turned on with other light sources to enhance the background, 255 (UV-C), 308 nm (UV-B), 365 nm (UV-A), 385 nm (UV-A), 660 nm LED illumination, 740 nm, 780 nm, 850 nm, 940 nm, 980 nm, 1050 nm. - It must have transmitted illumination: 365 nm UV flood, 5000 K white, 780 nm, 850 nm, 940 nm, broadband illumination spot from 400-1000 nm, high-intensity 980 nm LED for anti-stroke imaging. - Coaxial illumination compatible with “normal” mode, small area and off-axis: 37x OVD LED illumination array for OVD/OVI/holographic security functions, side/oblique illumination (white and 850 nm IR): left and right, independent, with variable height adjustment (manual), polarized white light sources: linear, left and right circular. - Motorized XY platform that allows smooth, hands-free movement and evidence searching, must support high-resolution imaging modes such as pixel shifting and image stitching. Movement range of at least 60 x 45 mm (X and Y axes respectively), movement resolution of 10μm. - It must contain a spectrometer module that allows: Measurement of absorption, reflectance, transmission, and fluorescence spectra of document features, must include self-test function to ensure proper operation, spectral range: 400 to 1000 nm, spectral resolution of 2 nm, capability to identify and calibrate the point of interest, graphical user interface, start/stop/record/pause controls, integration time control, chromaticity diagram, integral alignment LED. Connectivity, Power, and Compliance: - Connectivity: Ethernet, USB, HDMI - Power: 100-240V AC, 50/60Hz - Compliance: CE, FCC, RoHS | 1 |
| 2 | VSC9000 Suite Software - Integrated control interface for all illumination, imaging, and examination tools - Barcode reading/scanning functionality - Macro creation and utilization, including configuration recovery and quick verification functions - ICAO Machine Readable Zone (MRZ) area compatible with Optical Character Recognition (OCR) - Database software for identification documents from different countries, with a machine-readable zone (MRZ) decoder for cross-verification of identification information. - Database software for authentic and counterfeit banknotes from various countries, featuring a quick and efficient method for searching and comparing banknotes. - Automatic MRZ recognition via AI Assist MRZ - Compatibility with electronic RFID readers - Image decoders to reveal and inspect invisible information - Operating system: Windows 10 and Windows 11 | 1 |
| 3 | USB3 Flash Drive (128GB): - 250 W light bulb – Quantity: 5 - 100 W light bulb – Quantity: 5 - Spare fuses – Quantity: 5 | 5 |
| 4 | PC: - Intel i7 12th generation 127XX processor - 32 GB RAM - 1 TB SATA hard drive - 512 GB SATA SSD - 3 USB3 ports - NVIDIA Gigabyte GTX1660 4 GB graphics card - Wireless Keyboard and mouse - 42-inch monitor | 1 |
| 5 | Reference Database subscription for five years: - Keesing Travel and Identity documents database(updates subscription) - Keesing Travel and Identity Documents Database (Archive) - Keesing banknotes documents database (updates subscription) - Keesing banknotes documents database (archive) - Assure ID Sentinel (5 year subscription) | 1 |
| 6 | Accessories: - ATR Thermochromic ink viewer - Polarisafe UV Lighting Module - Document Tilt Plate - MFSI-2 Magnetic Security Feature Viewer - Quartz Glass Plate - Large 4A Quartz Glass Plate - Card Edge Viewing Mirror - 3D Imaging Software module - Thick Artefact examination tray - Papeweights E reader accessories: - Ereader 2A - Ereader 5 - Ereader 6 Calibration accesories: - NIST Traceable calibration standards Facial Identification accesories: - Facial ID Expert Hardware kit - Facial ID Standard Hardware kit - Live Facial Identification software Integrated data decoders: Secure Trust Authentication Chain; Extended RFID Capability; Letterscreen Decoder; IPI Decoders. | 1 |
| 7 | Microscope Specifications: Similar to the Leica microscope, Model M205C - Total zoom 20.5:1 - Magnification: zoom 0.78x to 16x - Objective: Plan APO 1x - Typical magnification range 7.8× – 160× - Maximum resolution 1050 lp/mm - Working distance 61mm - Illumination LED5000RL ringlight with polarizer accessory - LAS-X dongle with EDOF and 3D surface package - Optical system: apochromatic CMO, Fusion Optics; encoded functions: zoom, iris, nosepiece - Camera adapters for microscope: 1.0x C-mount, for stereo microscopes | 1 |
| 8 | Camera similar to Leica DMC5400 20 MP USB 3.0 digital interface - LAS X 3.4 or higher software for windows 10/11 - 20 Megapixel CMOS sensor with latest Sony ExmorR sensor technology. | 1 |
| 9 | UPS UPS shall be able to provide protection and support to the equipment for a minimum period of 15 minutes. | 1 |
| 10 | Installation Contractor shall provide a suggested work area characteristics such as physical, electrical, environmental, temperature and other conditions necessary for the optimal performance of the microscope. The UPS must be connected to the institution’s transfer line that includes the power plant. Test all equipment and applicable configurations. After equipment has been imported into Guatemala, INL shall inform Contractor to proceed with the installation, integration, configuration, and to put into operation the equipment included in this SOW, no later than thirty (30) calendar days after notice to proceed. All coordination shall be arranged with INL representatives. Vendor shall be responsible for complying with INL policies and procedures. Task shall be performed by brand certified technicians. | 1 |
| 11 | Shipping | 1 |