DESCRIPTION/WORK STATEMENT/SPECIFICATION

FOR

KAHN MODEL 102-240 DYNAMOMETERS

1.0 Scope:

This specification establishes a requirement for procuring technical overhaul/remanufacturing package for the Kahn Model 102-240 Dynamometers.  These dynos are currently used in test cell one through five located in building 128 Turbine Test Cells at Anniston Army Depot (ANAD) Anniston, Alabama.

2.0 Background:

The current Dyno’s are installed and operational in Turbine Test Cells. Anniston Army Depot has been maintaining the Dyno since they have in Operation for over 20 Years. Kahn Dyno is no longer supporting the Kahn Model 102-240. To change to another style Dyno would impact the footprint of the current Turbine test cells which will impact the overall layout of the test cell.

3.0 Contractor Requirements:

The contractor shall provide the following drawings and documents necessary for the specific overhaul materials, repair or machining of new expendable/wear components, bill of materials and assembly of the Kahn Model 102-240 dynamometers:

- Bill of Material Hydraulic Dynamometer 102-240-001 (including manufacturer’s part numbers for Detail 100/101 shaft bearing, Detail 104 Carbon seals, Detail 120-126 O-rings, detail 180 vacuum breakers and specific nuts/bolts/washers replaced at overhaul)

- Assembly hydraulic Dynamometer (drawing 102-240-001)

- Overhaul Manual 19715B-1531

- Detailed manufacturing drawings for expendable/wear items including:

  - Det. 1 Shaft (drawing 102-240-103)

  - Det. 2 Rotor (drawing 102-240-104)

  - Det. 6 Mating Ring (drawing 102-240-116)

  - Det. 14 Bearing Support (drawing 102-240-111)

  - Det. 15 Stator (drawing 102-240-117)

  - Det. 16 Stator Plate (drawing 102-240-107)

  - Det. 17 Discharge Ring (drawing 102-240-110)

  - Det. 18 Stator Plate-Aft (drawing 102-240-105)

  - Det. 19 Stator Plate-Fwd. (drawing 102-240-106)

  - Det. 29 Clamping Ring (drawing 120-240-118)

- Wear limit dimensions for bearing journals

- Balancing specification (drawing 102-240-121)

3.2 All documents that are provided for this contract shall be without obscured information that pertains to the remanufacturing of the Kahn Model 102-240 Dyno.  The contractor shall provide two (2) electronic copies of all documentation unless otherwise specified.  The electronic documentation shall be provided on USB flash drives.  Electronic copies of the documentation shall be in Adobe Portable Document Format (PDF).  All documentation the contractor deems to be proprietary in nature must be clearly marked.  The Government must have the right to print hard copies of all documentation from the electronic copies as needed.

3.3 Contractor shall comply with all applicable federal, state, and local environmental and safety laws, statutes, regulations, executive orders, permits, Army regulations (with supplements), as well as Major Subordinate Command (MSC) and installation regulations, policies, Host Tenant Agreement, or Interagency Service Support Agreement. Contractor shall immediately report any conflicts between applicable federal, state, local environmental and safety laws, statutes, executive orders, and provisions of Army Regulation 200-1, and any specifications within this contract to the Contracting Officer Representative (COR).

3.4 Contractor shall immediately notify the Designated Government Representative (DGR) and COR of the arrival on site of any Federal, State, and/or DoD environmental or safety regulator or enforcement agent and/or receipt of any correspondence from a Federal or State environmental agency.

3.5 Contractor shall submit to potential Federal, State, Army and installation work site environmental or safety regulatory inspections and/or investigations into noncompliance and fully cooperate with such inspections/investigations by providing the appropriate records and documentation. Environmental regulatory agencies are authorized by law to inspect any work site for environmental compliance with regulatory requirements. If an inspection is conducted, it will not stop or disrupt ongoing contract activities. The inspection will only require the work site environmental or safety officer, or supervisor/manager to answer questions and/or escort the inspector to specific work site areas with the potential to affect environmental quality.

3.6 Contractor shall immediately report any nonconformance and/or noncompliance with applicable Federal, State or Local environmental or safety laws, Army and installation environmental regulations or policies to the COR and DGR.

3.7 Contractor shall take the necessary actions to identify, monitor, and control those contract operations and activities that pose risk of contamination, or can negatively impact the natural and/or human environment.

4.0 Project Completion Requirements:

4.1 This contract shall have a performance period of 6 to 8 weeks after contract is awarded to complete all requirements of this specification.

4.2 Acceptance of goods shall be done by the assigned Government Contracting Officer’s Representative (COR).