STATEMENT OF WORK (SOW)

Of note: Any reference to a brand name specific item (Motorola, etc.) will be “or equal”

| Contract Number: |  |
| --- | --- |
| Task Order Number: |  |
| IFCAP Tracking Number: | 691-26-3-4685-0027 |
| Follow-on to Contract and Task Order Number: | N/A |

1.  Contracting Officer’s Representative (COR

| Name: | Robert Dalley |
| --- | --- |
| Section: | Greater Los Angeles VA Healthcare System |
| Address: | 11301 Wilshire Boulevard, Los Angeles, CA 90073 |
| Phone Number: | 310-268-3031 |
| Fax Number: |  |
| E-Mail Address: | Robert.Dalley@va.gov |

2.  Contract Title

GLAVA Radio Communications System Expansion with Additional Motorola APX NEXT & Lifecycle Support Services.

3.  Background

The VA Police Service at the Greater Los Angeles Healthcare System operates one of the largest federal law enforcement agencies within the Veterans Health Administration, providing law enforcement and emergency response services across multiple VA medical centers, outpatient clinics, and veteran housing facilities throughout the Greater Los Angeles region. Current operations require the expansion of radio communications infrastructure and dispatch capabilities to support growing mission requirements, officer safety, and interoperability with local, state, and federal law enforcement partners including the Los Angeles Police Department (LAPD), Los Angeles County Sheriff’s Department (LASD), Los Angeles Fire Department (LAFD), Federal Bureau of Investigation (FBI), and Department of Homeland Security (DHS).

This procurement effort is also in support of the Executive Order on the National Center for Warrior Independence (NCWI), which necessitates enhanced communication capabilities to ensure efficient coordination and secure communications. The procurement includes adding 100 new Motorola APX NEXT portable radios to the existing fleet to meet the demands of increased personnel and expanded operational requirements.

The expansion aligns with the goals of the National Emergency Communications Plan (NECP) to ensure effective interoperability, coordination, lifecycle management, and strengthening the cybersecurity posture of public safety communications as a whole.

4.Objective

The objective of this procurement is to expand the existing radio communications system and dispatch capability with modern, secure, fully interoperable equipment that meets federal law enforcement standards and supports VA Police operational requirements for the next five years. The system expansion shall provide:

- Secure, encrypted voice communications for additional VA Police personnel

- Seamless integration with existing ASTRO 25 infrastructure

- Advanced digital features including voice control, smart programming, and connectivity management

- Lifecycle support ensuring system reliability, security patching, and operational readiness

- Compliance with federal information security standards and VA policy requirements

5. Scope

This Statement of Work / Performance Work Statement (SOW/PWS) defines the requirements for the procurement, implementation, and lifecycle support of 100 additional Motorola APX NEXT portable radio communications equipment. The Contractor shall provide all equipment, software, licensing, installation support, training, maintenance, and technical support necessary to establish and sustain a fully operational, secure, interoperable radio communications system supporting VA Police operations at the Greater Los Angeles Healthcare System (GLA).

6. Specific Tasks

6.1  Task 1 (Phase 1):  Procurement of Additional Portable Radios, Console and Accessories

6.1 Task 1: Procurement of Additional Portable Radios and Accessories

The Contractor shall procure and deliver the following Motorola equipment in quantities specified:

6.1.1 Portable Radios

The Contractor shall provide one hundred (100) Motorola APX NEXT portable radios configured to VA Police specifications and programmed for integration with the existing ASTRO 25 system. Each portable radio shall include:

- Model: Motorola APX NEXT (or equal)

- Frequency bands: VHF, UHF, 700 MHz, and 800 MHz capability

- LTE and Wi-Fi connectivity enabled

- AES-256 encryption capability

- Over-The-Air-Rekeying (OTAR) support

- P25 Phase II trunking capability

- Integrated GPS

- Bluetooth connectivity

- Voice control (ViQi) capability

- Smart Programming and Smart Connect features

- Minimum IP68 environmental rating

- Mission Critical Push-to-Talk (MCPTT) ready

- Complete codeplug development and programming for integration with existing GLA ASTRO 25 system

6.1.2 Console

- APEX 8500 (2)

- Frequency bands: VHF, UHF, 700 MHz, and 800 MHz capability

- LTE and Wi-Fi connectivity enabled

- AES-256 encryption capability

- Over-The-Air-Rekeying (OTAR) support

- P25 Phase II trunking capability

- Integrated GPS

- Bluetooth connectivity

- Voice control (ViQi) capability

- Smart Programming and Smart Connect features

- Minimum IP68 environmental rating

- Mission Critical Push-to-Talk (MCPTT) ready

- Complete codeplug development and programming for integration with existing GLA ASTRO 25 system

- Programming cables and interfaces (as needed for Government programming capability)

- Install Antenna, coax and all other cabling to R65 standards from roof to console location within the police station.

6.1.2 Portable Radio Accessories

The Contractor shall provide accessories and support equipment for all 100 portable radios including:

- High-capacity smart batteries (minimum two per portable radio)

- Multi-unit charging systems (sufficient capacity for 100 radios)

- Belt clips and carrying accessories

- Remote speaker microphones with emergency button

- Earpieces and acoustic tube kits

- Vehicular charging adapters

- Programming cables and interfaces (as needed for Government programming capability)

- Replacement antennas

- Protective cases

- All accessories shall be Motorola-branded, genuine equipment compatible with APX NEXT platform (or equal)

6.2 Task 2: Software, Licensing, and Programming

6.2.1 Radio Management Platform

The Contractor shall provide enterprise-level radio management software licensing sufficient to support the expanded VA Police GLA fleet. Software licensing shall include:

- Fleet-wide device management and monitoring

- Remote diagnostics and troubleshooting

- Software distribution and update management

- Configuration template management

- Usage reporting and analytics

- Security policy enforcement

6.2.2 Feature Licensing

The Contractor shall enable and license the following advanced features on all radios:

- ViQi voice control system

- Smart Programming (over-the-air configuration updates)

- Smart Connect (automatic network switching)

- Text messaging capability

- GPS location services

- Emergency alert and lone worker protection features

6.2.3 Programming and Provisioning

The Contractor shall provide:

- Complete codeplug development and programming for all 100 portable radios and console to VA Police operational templates

- Integration with existing ASTRO 25 system talk groups, zones, and system parameters

- System key provisioning and encryption key loading

- Talk group and zone configuration per VA Police requirements coordinated with Government technical staff

- Documentation of all programming parameters and configurations

6.3 Task 3: Installation and Integration

6.3.1 Integration with Existing ASTRO 25 Infrastructure

The Contractor shall configure all equipment to seamlessly integrate with the existing GLA ASTRO 25 trunked radio system, ensuring compatibility with existing encryption key management and network integration.

6.3.2 System Installation Services

The Contractor shall provide installation technicians to perform physical installation, cabling, and equipment setup, coordination with Government facilities staff for site access, and post-installation verification and testing.

6.4 Task 4: Professional Services and Project Management

6.4.1 Project Management

The Contractor shall provide comprehensive project management services including:

- Dedicated Project Manager

- Coordination of all Contractor resources and subcontractors

- Development and maintenance of detailed project schedule

- Regular status reporting

- Management of all deliverables to ensure on-time, on-budget completion

- Resolution of technical and administrative issues

6.4.2 System Engineering

The Contractor shall provide system engineering services for technical design validation, finalization of system design, and support during installation, integration, and acceptance testing.

6.4.3 Training Services

The Contractor shall provide training for VA Police personnel on operation and basic troubleshooting of APX NEXT portable radios, covering user operation, emergency features, and basic troubleshooting.

6.4.4 Technical Training

The Contractor shall provide technical training for VA Police communications technicians, including radio programming procedures, system troubleshooting, and basic maintenance procedures.

6. Performance Monitoring

## The government COR will provide overall project supervision; however, contractor is responsible for onsite inspections that meet manufacturer installation requirements. The COR will perform onsite inspections throughout the project.

## 7. Security Requirements

There are no security concerns. The system does not touch the VA network and resides on its own network. The C&A requirements do not apply, and a Security Accreditation Package is not required. Vendor must be authorized to work with US Secret Service for Mt. Disappointment.

8.  Government-Furnished Equipment (GFE)/Government-Furnished Information (GFI)

N/A

9.  Other Pertinent Information or Special Considerations

a.  Identification of Possible Follow-on Work.

b.  Identification of Potential Conflicts of Interest (COI).

c.  Identification of Non-Disclosure Requirements.

d.  Packaging, Packing and Shipping Instructions.

Vendor will provide full shipping and packaging to manufacturers specifications. Vendor will provide all tracking documentation and receiving reports as to what items have been shipped and accepted. All items will be shipped to 11301 Wilshire Blvd, Bldg. 297 with PO number on all documents and packaging with ATTN: Robert Dalley or Ben Spivey.

e.  Inspection and Acceptance Criteria.

COR and facility will sign off on system functionality prior to final payment.

## 10. Risk Control

11.  Places of Performance

11301 Wilshire Blvd Bldg 236, Los Angeles, Ca 90073 (VA West Los Angeles)

16111 Plummer St. North Hills, Ca 91343 (VA Sepulveda)

12. Deliverables and Acceptance Criteria

12.1 Deliverable Items

- 100 APX NEXT portable radios with all specified features and accessories

- Documentation deliverables

- Documentation of all programming parameters and configurations

- Network diagrams and system architecture documentation

- Training materials and training completion documentation

- Program Management Services including VA complaint invoicing and accountability measures

12.2 Inspection and Acceptance Testing

The Government will conduct inspection and testing of all delivered equipment for up to seven (7) days from the date of shipment to determine if the equipment conforms to contract requirements.

12.3 Non-Conforming Equipment or Services

If equipment or services fail to meet acceptance criteria, the Government may reject non-conforming items and require replacement at no additional cost or terminate the contract for default.

## 12.  Period of Performance

## 365 calendar days

13.  Delivery Schedule

Vendor will provide upon contract award.