8. Functional/Performance/Specification

Brand Name or Equal: UTV Gravel Spreader

Description & Salient Characteristics

UTV with 40 hp engine, three-person seating and compatibility with V-Blade mount and SaltDogg attachment.

- Must have a 2-cylinder, SOHC gasoline engine.

- Must have a displacement of 999 cc.

- Must have a peak power of 40 hp.

- Must have a peak torque of 33.4 lb-ft.

- Must be liquid cooled.

- Must have an electric starter.

- Must have a fuel delivery via electronic fuel injection and a fuel capacity of 11.5 gallons.

- Must have CVT transmission.

- Must have an on-demand true AWD/2WD/Versa-Trac Turf Mode drive system.

- Must have a top speed of 35mph in high gear and 15mph in low gear and reverse.

- Must be compatible with Sno-Way snow and ice control equipment, vehicle mount kit 99101212.

- Must be compatible with SHPE0750 series SaltDogg Electric Drive Poly Hopper Spreader.

- Must have towing capacity of 2,500 lb.

- Must have cargo bed volume of 18 cu ft.

- Must have overall length of 125” x 62.5” W x 75” H and wheelbase of 85”.

- Must have dash-mounted transmission shifter H-L-N-R-P.

- Must have 3-point seatbelts for all three seats.

Required Individual Line Items & Quantity of Items - Brand Name or Equal

- (1 each) Gravely JSV 3400

- (1 each) Hard doors with crank windows

- (1 each) Heater

- (1 each) Roof

- (1 each) Head Liner

- (1 each) Glass Windshield

- (1 each) Electric Wiper / Washer

- (1 each) Rear panel

- (1 each) Horn

- (1 each) Rear view mirror

- (1 each) Freight

Optional Line Items

- Delivery Requirements:

  - Please include any shipping costs that are not included in the cost of the line items.