### Salient Characteristics for VA Police Radio System

- Compliance and Standards

  - Must comply with P25 (Project 25) standards for interoperability with federal, state, and local agencies.

  - FCC-approved frequencies and encryption standards.

- Coverage and Reliability

  - Provide 100% coverage within VA facility grounds and parking areas.

  - Minimum signal strength of -95 dBm in all operational areas.

- Encryption and Security

  - AES-256 encryption for secure communications.

  - Capability for Over-the-Air Rekeying (OTAR).

- Hardware Requirements

  - Portable radios: Rugged, water-resistant, MIL-STD-810 compliant.

  - Mobile radios: Vehicle-mounted with external antenna for extended range.

  - Dispatch consoles: Integrated with existing VA security systems.

- Features

  - Emergency alert button on all radios.

  - GPS tracking for officer location.

  - Text messaging and status indicators.

- Scalability

  - Support for future expansion (additional channels, users).

  - Ability to integrate with regional interoperability networks.

- Maintenance and Support

  - 24/7 technical support.

  - Minimum 5-year warranty on all equipment.

  - On-site training for VA Police personnel.