Contract Acronyms	5

SCHEDULE OF SUPPLIES/SERVICES	6

A1.1 On-Call Aircraft	6

A1.2 Exclusive Use Aircraft	7

A1.3 Exclusive Use Aircraft Locations with Specific Requirements:	7

A2.1 PRICING SCHEDULES	8

A2 Aircraft and Price Schedule (See Attachment X)	9

SECTION B – TECHNICAL SPECIFICATIONS	12

GENERAL REQUIREMENTS	12

B1 Scope of Contract	12

B2 Certifications	12

B3 Order of Precedence (Specifications)	13

B4 Contracts and Task Orders	13

EQUIPMENT REQUIREMENTS	13

B5 Condition of Equipment	13

B6 Aircraft Equipment Requirements	13

B7 Avionics Requirements	14

B8 Reserved	20

PERSONNEL REQUIREMENTS	20

B9 Pilot Requirements	20

B10 Pilot Qualifications	21

B11 Flight Crewmembers Duty and Flight Limitations	22

B12 Personnel Duty Limitations	23

B13 Mechanic Requirement	23

B14 Mechanic Qualifications	23

B15 Mechanic Duty Limitations	24

B16 Reserved	24

B17 Reserved	24

B18 Relief Crew Requirement	24

OPERATIONS	24

B19 Pilot Authority and Responsibility	24

B20 Flight Operations	25

B21 Security of Aircraft and Equipment	27

B22 Personnel Protective Equipment (PPE) for Flight Operations	28

B23 Reserved	28

B24 Transportation of Hazardous Materials	28

B25 Fuel and Servicing Requirements	28

AIRCRAFT MAINTENANCE REQUIREMENTS	29

B26 General - Maintenance	29

B27 Airworthiness Directives (ADs) and Manufacturer’s Mandatory Service Bulletins (MMSBs)	30

B28 Manuals/Records	30

B29 Maintenance	30

B30 Maintenance Test Flight	31

B31 Time Between Overhaul (TBO) and Life-Limited Parts	31

B32 Weight and Balance	31

B33 Reserved	32

SECTION C-CONTRACT TERMS AND CONDITIONS	33

1452.201-70 Authorities and Delegations (SEP 2011)	39

1452.203-70 Restrictions on Endorsements (JUL 1996)	39

1452.228-71 Aircraft and General Public Liability Insurance (Mar 1989)	40

FAR 52.212-4(q) Other Compliances (Only Applies to Alaska)	43

GENERAL CONTRACT TERMS AND CONDITIONS	47

C2 Aircraft Use Reports	47

C3 Substitute Personnel, Aircraft, or Equipment. 	47

C4 Aircraft Insurance	48

C5 Post Award Evaluation of Contractor Performance Contractor Performance Assessment Reporting System (Dec 2015)	48

C6 Personnel Conduct	49

C7 Safety Management System (SMS)	49

C8 Mishaps	49

C9 Suspension of Pilot	51

C10  Electronic Invoicing and Payment Requirements - Invoice Processing Platform (IPP) (February 2021)	52

(End of Local Clause)	52

C11 Contractor Personnel Security Requirements	52

C12 Prework Meeting	52

C13 Economic Price Adjustment (Does not apply to Alaska)	52

C15 Exclusive Use Period	54

C16 Extension of Exclusive Use Period	54

AVAILABILITY REQUIREMENTS	54

C17 Availability Requirements	54

C18 Schedule of Operations and Reaction Time	55

C19 Maintenance During Availability Period	55

C20 Unavailability and Damages	55

MEASUREMENT AND PAYMENT	56

C21 Daily Availability	56

C22 Flight Time	56

C23 Mobilization/Demobilization	56

C24 Additional Pay Items (from Schedule of Items)	56

C25 Government Miscellaneous Charges	58

C27 Add/Remove Aircraft/Equipment After Contract Award	59

SECTION C EXHIBITS	61

Exhibit 1 Aerial Supervision Mission Checklist	62

Exhibit 2 Unacceptable lap belt and shoulder harness conditions	63

Exhibit 3 First Aid and Survivial Kits	64

Exhibit 4 Safety Management System (SMS), Flight Time, Accident History, and FAA Violation Questionnaire (Pre-AWARD ver. 6.2 – 2025)	65

Exhibit 5 Airplane Approximate “Recommended Cruise" Fuel Consumption Rates	69

Exhibit 6 Civil Aircraft Certificate of Insurance, DD Form 2400	70

Exhibit 7 Civil Aircraft Landing Permit, DD Form 2401	70

Exhibit 8 Civil Aircraft Hold Harmless Agreement, DD Form 2402	70

Exhibit 9 Transportation Worksheet	71

Exhibit 10 Unavailability Conversion Chart	74

Exhibit 11 Add/Remove – Aircraft/Equipment Request Form	75

Exhibit 12 Aircraft Questionnaire	76

Exhibit 13 Aircraft Information Form	77

Exhibit 14 Equipment for Conducting Aircraft Operational Load Monitoring (OLM)	78

Exhibit 15  Task Order Competition Procedures	81

SECTION D – INSTRUCTION TO OFFERORS	82

INTRODUCTION	82

D1 General Information	82

D2 North American Industry Classification System (NAICS) Code and Small Business Size Standard	82

D3 Instructions to Offerors – Commercial Items (52.212-1 MAR 2023)	82

D4 Offer Contents	83

Volume I, General	83

Volume II, Technical Capability	83

Volume III, Organizational Safety	84

Volume IV, Past Performance	85

EVALUATION OF PROPOSALS	85

Evaluation—Commercial Items (52.212-2 Nov 2021)	85

ADDENDUM TO FAR 52.212-2 EVALUATION—COMMERCIAL PRODUCTS AND COMMERCIAL SERVICES (Nov 2021	85

Evaluation Factors	85

Organizational Safety Adjectival Ratings	88

SECTION E – CONTRACT PROVISIONS AND SOLICITATION ATTACHMENTS	91

PROVISIONS INCORPORATED BY REFERENCE	91

Provisions Incorporated by Full Text	91

SECTION E EXHIBITS	92

Exhibit E-1 Offeror’s Miscellaneous Information	93

Exhibit E-2 Aircraft Questionnaire	94

Exhibit E-3 Aircraft Information Form	95

Exhibit E-4 Past Performance Questionnaire	96

Exhibit E-5 Pricing Sheet	97

Exhibit E-6 Wage Determinations	98

# Contract Acronyms

AC	Advisory Circular

ACS 	Airmen Certification Standards

AD	Airworthiness Directive

ADS-B	Automatic Dependent Surveillance – Broadcast

AFF	Automated Flight Following

AFM	Aircraft / Airplane Flight Manual

AGL	Above Ground Level

ALSE	Aviation Life Support Equipment

AOM	Aviation Operations and Management

A&P	Airframe and Power plant

AQD	Acquisition Services Directorate

ASM	Aviation Safety Manager

ASR	Airport Surveillance Radar

ASTM	American Society for Testing and Material

ATC	Air Traffic Control

ATGS	Air Tactical Group Supervisor

AUR	Aircraft Use Report

BLM	Bureau of Land Management

CFR	Code of Federal Regulations

CO	Contracting Officer

COR	Contracting Officer’s Representative

CPARS Contractor Performance Assessment Reporting

CRM	Crew Resource Management

CS	Contract Specialist

CTCSS	Continuous Tone Coded Squelch System

DIAR	Department of the Interior Acquisition Regulation

DME 	Distance Measuring Equipment

DOI	Department of Interior

DOT	Department of Transportation

EFB	Electronic Flight Bag

ELT-AF	Emergency Locator Transmitter –  Automatic Fixed

ELT-AP	Electronic Locator Transmitter – Automatic Portable

EPA	Environmental Protection Agency

ET	Eastern Time

EU	Exclusive Use

FAA	Federal Aviation Administration

FAR	Federal Acquisition Regulation

FBO	Fixed Base Operator

FTA	Fire Traffic Area

FTR	Federal Travel Regulations

GFE	Government Furnished Equipment

GPS	Global Positioning System

GS	Glideslope

IAT	Interagency Aviation Training

ICA	Instructions for Continued Airworthiness

ICS	Intercommunication System

ICAO	International Civil Aviation Organization

IDIQ	Indefinite Delivery Indefinite Quantity

IFR	Instrument Flight rules

IIC	Agency Investigator in Charge

IPP	Invoice Processing Platform

ISA	International Standard Atmosphere

IS-BAO 	International Standard for Business Aircraft Operations

MEL	Minimum Equipment List

MFD	Multifunction Display

MIL-DTL Military Detail

MMSB	 Manufacturer’s Mandatory Service Bulletins

MSL 	Mean Sea Level

NAC	National Access Code

NAICS	North American Industrial Classification System

NFES	National Fire Equipment System

NFPA	National Fire Protection Association

NTE	Not To Exceed

NTP	Notice to Proceed

NTSB	National Transportation Safety Board

NWCG	National Wildfire Coordinating Group

AOMOEM	Original Equipment Manufacturer

OPM	Operational Procedures Memoranda

PA	Public Address

PAR	Precision Approach Radar

PI	Project Inspector

PIC	Pilot-In-Command

POA	Privately Owned Aircraft

POC	Point of Contact

POH	Pilot Operating Handbook

PPE	Personal Protective Equipment

PTT	Push to Talk

RFP	Request for Proposals

RNAV	Area Navigation

SAM	System for Award Management

SAS	Standards for Aerial Supervision

SDR	Service Difficulty Reports

SMS	Safety Management System

STC	Supplemental Type Certificate

T&M	Time and Materials

TAS	Traffic Avoidance System

TBO	Time Between Overhaul

TCAD	Traffic Collision Avoidance System

TCAS	Traffic Collision Avoidance System

TGID 	Talk Group ID

TIS	Traffic Information Systems

TO	Task Order

UAS	Uncrewed Aircraft System

UAT	Universal Access Transceiver

USWFS	United States Wildland Fire Service

VFR	Visual Flight rules

VIN	Vehicle Identification Number

VOR	Very High Frequency Omni-Directional Range

VOX	Voice Activated

VSWR	Voltage Standing Wave Ratio

# SCHEDULE OF SUPPLIES/SERVICES

## A1.1 On-Call Aircraft

| Bureau: |  |  | United States Wildland Fire Service (USWFS) |
| --- | --- | --- | --- |
| Designated Base: |  |  | Aircraft may be requested to position to any base or incident in the Contiguous United States and Alaska. All equipment, facilities, and personnel required under this contract shall be delivered to and removed from the designated base location identified in the Pricing Schedule. |
| Aircraft Required: |  |  | Multi or single engine turbine, turbofan, or turboprop powered land airplane equipped as specified in Section B for Air Tactical Group Supervisor (ATGS) operations. |
| Crew Compliment Required: |  |  | Pilot in Command (PIC) and relief PIC |
| Minimum Aircraft Requirements: |  |  |  |
| Must have a Standard Airworthiness Certificate. Aircraft certified under 14 Code of Federal Regulations (CFR) Part 23 or Part 25 (or Civil Aviation Regulation (CAR) 3 as applicable). An aircraft make and model for which engineering and logistical support, for continued airworthiness, is provided from the current type certificate holder or supplemental type certificate holder. Certified for VFR/IFR, Day/Night; Known Ice. Aircraft must have environmental controls including heating and air conditioning. Must be single pilot certified. ATGS will occupy Co-pilot seat. Insured seating for three (3) passengers which includes the right front seat located in the cockpit. Removable seats for cargo configuration. No external obstructions within the field of regard, 9 o’clock to 3 o’clock, as viewed from the cockpit and minimal restrictions to outward visibility of the ground in level flight for the rear observers. Cruise airspeed minimum 160 knots true airspeed at 8,000 feet pressure altitude, ISA+20ºC. Endurance of four (4.0) hours of fuel plus 30-minute reserve with cruise power at 8,000 feet pressure altitude and temperature ISA +20°C. Payload of 900 pounds with fuel for 250 nautical mile flight, VFR fuel reserve, at 8,000 feet pressure altitude, temperature ISA+20°C, 200-pound pilot and 25-pound survival kit. |  |  |
| Multi Engine Airplanes Only: |  |  |  |
| Cruise airspeed minimum 220 knots true airspeed at 8,000 feet pressure altitude, ISA+20ºC. Positive single engine rate of climb when equipped for this contract with a 200-pound pilot, one 220-pound observer, 4 hours of fuel plus a 30- minute reserve, at 5,000 feet pressure altitude, ISA+33º C. Pressurized aircraft with environmental controls, heater and air conditioner. |  |

## A1.2 Exclusive Use Aircraft

Minimum Aircraft Requirements:

Must have a Standard Airworthiness Certificate. Aircraft certified under 14 Code of Federal Regulations (CFR) Part 23 or Part 25 (or Civil Aviation Regulation (CAR) 3 as applicable). An aircraft make and model for which engineering and logistical support, for continued airworthiness, is provided from the current type certificate holder or supplemental type certificate holder. Certified for VFR/IFR, Day/Night; Known Ice. Must be single pilot certified. ATGS will occupy Co-pilot seat. Insured seating for three (3) passengers which includes the right front seat located in the cockpit. Removable seats for cargo configuration. No external obstructions within the field of regard, 9 o’clock to 3 o’clock, as viewed from the cockpit and minimal restrictions to outward visibility of the ground in level flight for the rear observers. Cruise airspeed minimum 220 knots true airspeed at 8,000 feet pressure altitude, ISA+20ºC. Endurance of four (4.0) hours of fuel plus 30-minute reserve with cruise power at 8,000 feet pressure altitude and temperature ISA +20°C. Payload of 900 pounds with fuel for 250 nautical mile flight, VFR fuel reserve, at 8,000 feet pressure altitude, temperature ISA+20°C, 200-pound pilot and 25-pound survival kit. Pressurized aircraft with environmental controls, heater and air conditioner.

Multi-Engine Airplanes Only:

Positive single engine rate of climb when equipped for this contract with a 200-pound pilot, one 220-pound observer, 4 hours of fuel plus a 30- minute reserve, at 5,000 feet pressure altitude, ISA+33º C.

## A1.3 Exclusive Use Aircraft Locations with Specific Requirements:

Pocatello, ID:

Must meet the aircraft requirements of section A1.1 Multi or single engine airplanes plus:

Aircraft must be certified under 14 Code of Federal Regulations (CFR) Part 23 or Part 25.

Must be pressurized.

Single engine airplanes minimum 220 knots true airspeed 8,000 feet pressure altitude, ISA+20ºC.

Grand Junction, CO

Must meet the aircraft requirements of section A1.1 Multi or single engine airplanes plus:

Aircraft must be certified under 14 Code of Federal Regulations (CFR) Part 23 or Part 25.

Must be pressurized.

Single engine airplanes minimum 220 knots true airspeed 8,000 feet pressure altitude, ISA+20ºC.

All of Alaska:

Must meet the aircraft requirements of section A1.1 Multi or single engine airplanes plus:

Aircraft must be certified under 14 Code of Federal Regulations (CFR) Part 23 or Part 25.

Must be pressurized.

Single engine airplanes minimum 220 knots true airspeed 8,000 feet pressure altitude, ISA+20ºC.

# A2.1 PRICING SCHEDULES

Notes

The name and location correspond with the name and location entered on the SF1449 and the 14 CFR 135 holder’s name and operating location(s) as established by the certificate holder, NOT the location of any one aircraft. (Remittance address information different than that entered on the SF1449 form should be included on the “Offeror’s Miscellaneous Information” information page included in Section E.)

A Geographic Area Coordinating Center (GACC) that corresponds to the OFFEROR’S OPERATING BASE LOCATION is included on the pricing schedules. Conflicts between the offerors principle base location and the GACC entered will be resolved by the Contracting Officer (CO) through communications with the offeror. Any corrections will be included in the awarded contract. (Offerors that have more than one principal base of operations, may identify such locations for an item, however, a separate aircraft must be available at each location identified.

The applicable make and model of the aircraft being offered along with a daily availability and flight rate for all contract years is included in the pricing schedules. The individual FAA registration number(s) of each aircraft where identified is included on the pricing schedules.

The Schedule of Items pricing pages included in this document and that are awarded are hereby made a part of the subsequent individual conformed contract.

## A2 Aircraft and Price Schedule (See Attachment X)

THE ADDITIONAL PAY ITEMS SHOWN BELOW APPLY TO ALL LINE ITEMS ABOVE

| ADDITIONAL PAY ITEMS |  |  |  | PAY ITEM CODE | UNIT | UNIT PRICE |
| --- | --- | --- | --- | --- | --- | --- |
| Extended Standby – Pilot (C18.1) |  |  |  | EP | Hour | $75 |
| Fuel Charge (Fairbanks, AK Only – See B25.1.1) |  |  |  | FC | Gallons | Actual Cost |
| Additional Crew |  |  |  | AC | Day | $1000 |
| Additional Pilot | P03 | Day |  |
| Additional Pay Items (C20) |  |  |  | SC | Each | Actuals |
| Per Diem |  |  |  | PD | Each | Per FTR |

A2.2 Minimum Guarantee/Maximum Quantity -The Minimum Guarantee under the contract, as required by Federal Acquisition Regulation (FAR) 16.504(a)(1), is $500.00, which will be obligated on a Task Order (TO) at contract award. If no aircraft are ordered for an amount that exceeds the stated Minimum Guarantee by the end of the first ordering year, then the guarantee can be submitted through the Invoice Processing Platform (IPP) and paid to each vendor.  The minimum Order Quantity as specified in FAR Clause 52.216-22 is $500.00. The Maximum Indefinite Delivery Indefinite Quantity (IDIQ) quantity will be a shared ceiling that will be established at contract award.

A2.3 Exhibits that will be applicable under the IDIQ contract are included at the end of Section C.

FOR GOVERNMENT USE ONLY – DO NOT WRITE IN THIS AREA

Contracting Officer (CO) will complete at time of award and again when fuel adjustments are made.

| AVERAGE BASE PRICES |  | AVERAGE REFERENCE PRICES |  |  |
| --- | --- | --- | --- | --- |
| Jet Fuel |  | Jet Fuel |  |  |
| 100LL Aviation Gasoline |  | 100LL Aviation Gasoline |  |  |
| EFFECTIVE DATE | Award | EFFECTIVE DATE |  |  |
| SOURCE DOCUMENT | Original Contract | SOURCE DOCUMENT |  |  |
| Re-established Base Price(s) | Jet Fuel | 100LL | Effective Date |  |

Full Service Fuel prices obtained from http://www.airnav.com/fuel

|  | 100LL AVIATION GASOLINE | JET FUEL $/GAL |
| --- | --- | --- |
| Pocatello (KPIH) |  |  |
| Twin Falls (KTWF) |  |  |
| Ontario Municipal (KONO) |  |  |
| Cedar City Regional Airport, Cedar City, UT (KCDC) |  |  |
| Boise Air Terminal/Gowen Field, Boise, ID (KBOI) |  |  |
| Salt Lake City International Airport (KSLC) |  |  |
| Grand Junction Regional Airport, Grand Junction, CO (KGJT) |  |  |
| Reno/Tahoe International Airport, Reno, NV (KRNO) |  |  |
| ELKO Regional Airport (KEKO) |  |  |
| Billings Logan International Airport (KBIL) |  |  |
| San Bernardino International Airport, CA (KSBD) |  |  |
| Albuquerque International Support Airport, Albuquerque, NM (KABQ) |  |  |
| Tucson International Airport, Tucson, AZ (KTUS) |  |  |
| Yakima Air Terminal/McAllister Field, Yakima, WA (KYKM) |  |  |
| Burlington/Alamance Regional Airport, Burlington, NC (KBUY) |  |  |
| Waycross – Ware County Airport, Waycross, GA (KAYS) |  |  |
| Average |  |  |
| Date fuel prices obtained |  |  |

# SECTION B – TECHNICAL SPECIFICATIONS

# GENERAL REQUIREMENTS

## B1 Scope of Contract

B1.1 The intent of this contract is to obtain fully Contractor-operated and maintained Air Attack (OC and/or EU) flight services in support of Government missions in the contiguous United States and Alaska. Contractor services include provisions of an airplane, pilot(s), fuel (when not operating in Alaska), aircraft maintenance support, and all other associated equipment, as prescribed in this solicitation. Mission activities may include, but are not limited to, air tactical operations, aerial fire reconnaissance and detection, law enforcement limited to non-threatening surveillance and reconnaissance, and other administrative and related resource activities. The Government will direct aircraft to support its missions and objectives.

B1.1.1 Typical mission profile:

Engine start, taxi and takeoff from 4,500’ Mean Sea Level (MSL) at ISA plus 20. Climb to 22,000’ MSL pressure altitude cruise 150 nm descend to working/loiter altitude of 14,500’ MSL pressure altitude, for a minimum of 3 hours.

Mission flights consist of a high amount of flight time at speeds that require high angles of attack and/or flap extension in a right-hand orbit

Aircraft loiter is typically at altitudes between 10,000’ Above Ground Level (AGL) and 3500’ AGL.

Aircraft will leave the area of operation climb back to 22,000’ MSL pressure altitude and return to base.

B1.2 The Government and Contractor must establish an effective working relationship to successfully complete this contract. The cooperation, professionalism, and positive attitude of the Contractor’s employees toward aviation safety and accomplishment of the mission are an integral element of this relationship.

B1.3 The Government has interagency and cooperative agreements with other Federal agencies, State agencies, and private landholders and may dispatch aircraft under this contract for such cooperative use which is subject to the ordering procedures and legal limitations applicable to the underlying IDIQ Contract(s).

B1.4 During any task order period, aircraft furnished shall be subject to the exclusive use of the Government.

B1.5 This aircraft may be dispatched to Alaska. Alaska-specific equipment and/or operational requirements are referenced throughout this document, and in the First Aid/Survival Equipment. Because deployment to Alaska from the Lower 48 usually requires transit through Canada all contractor personnel are required to possess a current passport. For operations in Alaska, the contractor must provide personal tents and sleeping bags for the aircraft’s crew for use in field camp environments.

## B2 Certifications

The Contractor must obtain and keep current all of the following required certificates and must ensure that contract aircraft are operated and maintained in compliance with those certificates at all times:

B2.1 A 14 CFR Part 135 Air Carrier Certificate or Operating Certificate which authorizes the Contractor to operate in the category and class of aircraft and under flight conditions and locations required by this contract (e.g., airplane, multiengine, VFR, IFR, day/night, passengers, and cargo).

B2.2 The aircraft must be carried on the list required by 14 CFR Part 135.63 (Operations Specifications Part D, “Aircraft Listing,”).

B2.3 The contract aircraft must have a Standard Airworthiness Certificate and have engineering support for continued air worthiness from the current type certificate holder or supplemental type certificate (STC) holder. Installation of any equipment required by this contract must be Federal Aviation Administration (FAA) approved.

B2.4 The Contractor must have Operations Specifications that permit all contiguous United States and Alaska operations.

## B3 Order of Precedence (Specifications)

In the event of inconsistencies within the technical specification, the following order will be used in such resolution: (1) typed provisions of these specifications; (2) United States Wildland Fire Service (USWFS) Aviation Operations and Management (AOM)AOM supplements and/or exhibits incorporated by reference; (3) 14 CFR incorporated by reference; (4) aircraft manufacturer’s specifications; and (5) other documents incorporated by reference.

## B4 Contracts and Task Orders

B4.1 The Contractor must maintain a complete, current copy of the contract, modifications, and task orders (if applicable) in each contract aircraft throughout the performance period.

B4.2 The pilot must have task order information (i.e. task number, performance period) in his/her possession prior to any flights under this contract and make this information available to government representatives on request.

B4.3 Electronic copies of contracts and task orders may be used. However, the contractor is responsible for ensuring that the documents are uploaded on an appropriate viewing device (e.g., laptop or tablet), which must be charged and made available at the aircraft for reference by government representatives upon request. Further, the contractor must agree to hold the government harmless for any inadvertent or accidental damage to the device.

# EQUIPMENT REQUIREMENTS

## B5 Condition of Equipment

The Contractor-furnished aircraft and all other required equipment must be operable, free of damage, and in good repair. Aircraft systems and components must be free of leaks, except where specified by the manufacturer.

B5.1 Prior to inspection and acceptance, the Contractor must permanently repair or replace all windows and windshields that have been temporarily repaired. All windows and windshields must be maintained at all times and must be clean and free of scratches, cracks, crazing, distortion, repairs, or tinting which hinder visibility.

B5.2 The aircraft interior must be clean and neat with no unrepaired tears, rips, or other damage. The exterior finish, including the paint, must be clean, neat, and in good condition. Any corrosion must be within manufacturer or FAA acceptable limits.

B5.3 See the Unacceptable Lap Belt and Shoulder Harness Conditions Exhibit for lap belt and shoulder harness conditions that are not acceptable.

## B6 Aircraft Equipment Requirements

Any aircraft provided for use under this contract must be equipped with items as shown below:

B6.1 Current aeronautical charts covering area of operations (including Canada when in transit). Electronic Flight Bags (EFB) meeting OpSpec A061 are authorized.

B6.2 Reserved.

B6.3 Free air temperature gauge.

B6.4 One set of individual lap belts for each installed seat.

B6.5 Shoulder harness with automatic or manual-locking inertia reel for each front seat occupant. Shoulder straps and lap belts must fasten with one single-point, metal-to-metal, quick-release mechanism. Heavy-duty (military-style) harnesses with fabric loop connecting the shoulder harness to the male portion of the lap belt buckle are acceptable.

B6.6 Fire extinguisher(s), as required by 14 CFR Part 135, for the purposes of this contract, must be a handheld bottle, minimum 2-B:C rating, mounted and accessible to the flight crew while seated. The fire extinguisher must be maintained in accordance with National Fire Protection Association (NFPA) Manual 10, Standards for Portable Fire Extinguishers or the Contractor’s 135 operations manual.

B6.7 Dual flight controls for initial pilot performance evaluation.

B6.8 Aircraft lighting for night operation in accordance with 14 CFR Part 91.205(c), including instrument lights.

B6.9 A wing and tail strobe light system with an independent activating switch.

B6.10 High-visibility, pulsating, forward facing, conspicuity lighting.

B6.11 Air Conditioning

B6.12 Cabin heater and window defogger.

B6.13 Reserved.

B6.14 A first aid kit containing at least the items specified in the First Aid and Survival Kits (Exhibit 3) must be carried aboard the aircraft on all flights.

B6.15 A survival kit containing at least the items specified in the First Aid and Survival Kits Exhibit must be carried aboard the aircraft on all flights and must be included in weight and balance calculations.

B6.16 FAA-approved cargo restraints, cargo nets, and cargo straps for cargo tie down in cabin and baggage compartment area.

B6.17 Aerial Supervision Mission checklist. This checklist is required to be in the aircraft and utilized in-accordance-with the National Wildfire Coordinating Group (NWCG) Standards for Aerial Supervision (SAS), PMS 505. The aerial supervision mission checklist is attached as an exhibit to Section C of this contract. These checklists can be in print or electronic format. Copies of the SAS may be obtained from any interagency fire cache; item National Fire Equipment System (NFES) 2544.

## B7 Avionics Requirements

B7.1 General

B7.1.1 The Contractor must provide, install, and maintain the following systems in accordance with the manufacturer’s specifications and the installation and maintenance standards of Section B7. Detailed avionics systems performance requirements are listed in Avionics Operational Test Standards (copies available upon request from AOM Avionics, or the most recent list may be found online at: https://www.nifc.gov/resources/NIICD/niicd-documents)

B7.2 Avionics Installation and Maintenance Standards

B7.2.1 Strict adherence to the recommendations in the following FAA Advisory Circulars (AC) is required: AC 43.13 1B Chapter 11, “Aircraft Electrical Systems,” and Chapter 12, “Aircraft Avionics Systems”; AC 43.13 2B Chapter 1, “Structural Data,” Chapter 2, “Communication, Navigation, and Emergency Locator Transmitter System Installations,” and Chapter 3, “Antenna Installation.”

B7.2.2 All avionics systems requiring an antenna must be installed with a properly matched, aircraft-certified antenna, unless otherwise specified. Antennas must be polarized as required by the avionics system and must have a voltage standing wave ratio (VSWR) of 3.0 to 1 or better.

B7.2.3 The aircraft’s static pressure system, altimeter instrument system, and automatic pressure altitude reporting system must be maintained in accordance with the IFR requirements of 14 CFR Part 91.411 and inspected and tested every 24 calendar months, as specified by 14 CFR Part 43, appendices E and F.

B7.2.4 Transmitters must not open squelch on, or interfere with, other AM or FM transceivers in the aircraft which are monitoring different frequencies. So-called “Transmit Interlock” functions must not be used with communication transceivers.

B7.2.5 All installed transceivers must be placarded with designations matching the transmitter/receiver controls on the installed audio control system(s).

B7.3 Communications Systems

B7.3.1 One automatic-portable or automatic-fixed 406 MHz Electronic Locator Transmitter – Automatic Portable (ELT/AP) or Electronic Locator Transmitter – Automatic Fixed (ELT/AF), meeting FAA Technical Standard Order (TSO)-C126 and COSPAS/SARSAT specifications, must be installed in the aircraft per the manufacturer’s installation manual, in a conspicuous or marked location. The ELT must include a 121.5 MHz homing beacon and be equipped with an external fixed-type antenna, mounted in a location approved by the aircraft manufacturer. Documentation of bi-annual registration from the National Oceanic and Atmospheric Administration (NOAA) is required for all TSO C126 and newer ELTs.

B7.3.2 Three panel-mounted VHF-AM (VHF-1, VHF-2, VHF-3) aeronautical transceivers, each with a minimum of 760 channels covering 118.000 to 136.975 MHz. The transceivers must have channels selectable in no greater than 25 kHz increments and a minimum of 5 watts carrier output power. The transceivers’ operational controls must be mounted so they are readily visible and accessible to the pilot, and the aircraft’s radio call sign (normally the aircraft registration number) must be displayed on the instrument panel, in view of the flight crew.

B7.3.3 Three P25-compliant VHF-FM transceivers. Each transceiver (FM-1, FM-2, FM-3) must provide selection of narrowband analog (12.5 kHz) or narrowband digital (12.5kHz) operation on each of a minimum of 100 channels.  Each transceivers’ operational controls must be located and arranged so that the pilot and observer/copilot when seated, have full and unrestricted movement of each control without interference from clothing, the cockpit structure, or the flight controls.

B7.3.3.1 Each transceiver’s operational frequency range must include the band of 136.0000 MHz to 173.9975 MHz. The operator(s) must be able to program any usable channels within that band, along with any required Continuous Tone Coded Squelch System (CTCSS) tones, National Access Codes (NAC’s), or Talk Group ID’s (TGID’s), while in flight. Each transceiver must also incorporate a separate, programmable GUARD receiver, with accompanying GUARD transmit capability. Unless instructed by the Government for use on a specific project, all frequencies programmed for use under this Contract must be in the narrowband analog mode.

B7.3.3.2 Carrier output power for each transceiver must be 10 watts nominal value (original design specification). Each transceiver must be capable of displaying receiver and transmitter operating frequency, alpha-numeric channel labels, and must provide both receiver and transmitter activation indicators for MAIN and GUARD. Simultaneous monitoring of both MAIN and GUARD receivers is required.  Scanning of the GUARD frequency is not acceptable.

B7.3.3.3 Prior to acceptance under this contract, at least one transceiver must be programmed with the narrowband analog GUARD receive and transmit frequencies of 168.625 MHz, with a 110.9 Hz CTCSS tone on transmit only.

B7.3.3.4 The following VHF-FM aeronautical transceivers are known to meet the above requirements:

Technisonics: TDFM-136, TDFM-136A, TDFM-136/NV, TDFM-136A/NV, TDFM-136B, TDFM-136B/NV.

Cobham: (formerly NAT) NPX-136D-070, Anodyne Electronics Manufacturing MTP136D

B7.3.4 Each required VHF-FM transceiver specified in B7.3.4 above must also be equipped with a conveniently located programming port, to facilitate programming via a Government-owned laptop computer. The port(s) must be protected from accidental damage via contact, be hard-wired to the transceiver(s), not require switching of any cables for utilization, and must be conveniently located for ease of use. The Contractor must also furnish appropriate cables of adequate length, and/or any necessary adapters, to interconnect the aircraft programming ports to the serial and/or USB port(s) of the Government laptop computers as required.

Note: The “DIN” type connector receptacle on the front panel of some TDFM-136-series radios is part of an encryption feature and may not function as a programming port connector.  This connector can be utilized for channel programming on model TDFM-136A, serial numbers FDA1200 and higher, and model TDFM-136B ONLY.

B7.3.5 Automated Flight Following (AFF) Program

B7.3.5.1 One satellite-based aircraft tracking hardware for each aircraft offered, compatible with the government’s AFF Program (https://www.aff.gov). Not all available satellite-based tracking systems are compatible with the Government’s AFF Program, nor meets AFF’s requirements. The contractor must ensure that the aircraft hardware offered is compatible with AFF.gov. For questions about current compatibility requirements, select the “HELP” button at https://www.aff.gov.

B7.3.5.2 The AFF aircraft hardware must be powered by the aircraft’s electrical system, installed per the manufacturer’s installation manual, and operational in all phases of flight. AFF aircraft hardware must utilize as a minimum: Satellite communications, an externally or internally mounted antenna, provide data to the Government’s AFF viewing software, use aircraft power via a dedicated circuit breaker for power protection, and be mounted so as to not endanger any occupant from AFF aircraft hardware during periods of turbulence. Antennas should be placed where they have the best view of the overhead sky as possible. Externally mounted antennas are recommended to improve system performance. Any visual indicators for remotely installed units must be mounted so that they can be easily viewed by the pilot.

B7.3.5.3 AFF communications must always be fully operational in all areas of operation. Contractors working in or accepting dispatches to the State of Alaska, Southern Canada, or Western Canada must have an AFF system capable of being always tracked in these locations. Not all manufacturers’ AFF equipment communication links will operate effectively in all geographic areas.

B7.3.5.4 The contractor must maintain a subscription service through the AFF aircraft hardware provider allowing AFF position reporting for satellite tracking via the Government AFF viewing software. The position-reporting interval must be every two minutes while the aircraft is in flight. The contractor must register their AFF aircraft hardware with the Government through https://www.aff.gov providing complete tail number; manufacturer and serial number of the AFF transceiver; aircraft make and model; and contractor contact information. If the contractor relocates previously registered AFF aircraft hardware into another aircraft, then the contractor must contact the government’s AFF Program making the appropriate changes prior to aircraft use. In all cases, the contractor must ensure that the correct aircraft information is indicated within AFF. The contractor must notify the Government of system changes, scheduled maintenance, and planned service outages.

B7.3.5.5 Registration contact information, a web accessible feedback form, and additional information is available at: https://www.aff.gov.

B7.3.5.6 Prior to the aircraft’s annual contract inspection, the contractor must ensure compliance with all AFF systems requirements. The contractor must additionally perform an operational check of the system. As a minimum, the operational check must consist of confirming the aircraft being tested is displayed in AFF.gov (indicating it is currently transmitting data to AFF.gov) and that all information displayed in AFF.gov is current. A username and password are required to access AFF.gov. Log on to the AFF website at https://www.aff.gov to request a username and password.

B7.3.5.7 If AFF becomes inoperable/unreliable the aircraft may, at the discretion of the Government, remain available for service utilizing radio/voice system for flight following. The contractor will return the AFF system to full operational capability within 72 hours after the inoperative/unreliable unit is first discovered as defective.

B7.3.5.8 AFF Systems must comply with supplemental requirements contained in Specification Section Supplement document available at: https://www.aff.gov/support/Specification_Section_Supplement.pdf.

B7.3.6 Satellite Phone System (For Alaska location only)

B7.3.6.1 One Iridium-network based Satellite Telephone (SatPhone) system. The installation must be in accordance with 14 CFR 43.

B7.3.6.2 The SatPhone handset, or remote dialer, must be mounted in a convenient location in the cockpit. The location must provide ready and unencumbered access to its controls by both front seat occupants.

B7.3.6.3 The SatPhone system audio must be permanently interfaced to the aircraft audio system in the same manner as the aircraft radios, so that the pilot and observer/copilot can utilize the SatPhone via their headsets. It is preferred that all occupants of the aircraft be able to converse over the SatPhone through their headsets, though keypad access may be limited to the two front seat occupants.

B7.3.6.4 The SatPhone system must provide the capability to pre-program at least six (6) telephone numbers that can be quickly selected by either front seat occupant while in flight.

B7.3.6.5 A TSO C-129a L-band antenna acceptable to the manufacturer of the Sat Phone system must be installed on top of the fuselage, giving as much a view of the overhead hemisphere as possible.

B7.3.6.6 The SatPhone system may be integrated into the AFF system (see B7.3.5) using either the same Iridium communicator or a separate Iridium communicator. In the former case, it is acceptable for phone calls to interrupt the transmission of AFF position reports for brief periods of time, and flight crews must be instructed as to this conflict.

B7.4 Navigational Systems

B7.4.1 One permanently installed, panel-mounted global positioning system (GPS-1) utilizing an approved, fixed external aircraft antenna and powered by the aircraft electrical system or an aviation portable GPS unit (Garmin GPSMap 296/396/496 or equivalent) provided the portable unit is securely mounted, presents information from an overhead orientation (not a drive-along-the-road type), and is powered by the aircraft electrical system. The GPS (permanently installed or portable) must utilize the WGS-84 datum and reference latitude and longitude coordinates in the degrees/minutes/decimal minutes (DM) mode for aircraft positioning. The GPS navigation database must be updated annually covering the geographic areas where the aircraft will operate.

B7.4.1.1 A GPS data port connector must be installed for the purposes of external data retrieval by a GIS laptop computer from the above GPS. The connector must be a DB-9F type D sub-connector, must be wired for RS-232C serial format for laptop computers (pin 2=transmit data, pin 3=receive data [if applicable], and pin 5=ground) and must be mounted in a location convenient to the front seat observer. The contractor must also furnish an adapter cable to interface the DB-9F Type D sub-connector to a USB connector.

B7.4.2 Additional Navigational Systems. The aircraft must be equipped with the navigational equipment required for IFR operations as specified in 14 CFR 135.165, including, but not limited to, the following:

B7.4.2.1 Two VHF, VOR/LOC aeronautical navigation receivers with indicators (NAV-1, NAV-2), each with a minimum of 100 navigation receive channels.

B7.4.2.2 One glideslope (GS) system, with a minimum of 20 receive channels.

B7.4.2.3 One marker beacon receiver system with three-light indicator must be installed. The marker beacon receiver system is not required, if the aircraft is equipped with approved substitute equipment as described 14 CFR 91.175 (k).

Note: 14 CFR Part 91.175 (k) Approved substitute equipment is described as compass locator; precision approach radar (PAR) or airport surveillance radar (ASR); DME, VOR, or nondirectional beacon fixes authorized in the standard instrument approach procedure; or a suitable RNAV system in conjunction with a fix identified in the standard instrument approach procedure.

B7.4.2.4 One DME system, with a minimum of 100 channels and NAV-1/NAV-2 channeling. Note: If the GPS unit supplied to meet the requirements of B7.4.1 above is certified for IFR Supplemental Navigation under TSO-C129, a separate DME unit is not required.

B7.5 Audio Control System(s).

B7.5.1 Two separate audio control systems (dual systems installed in a single unit are NOT acceptable) must be provided for the pilot and ATGS/copilot, in locations convenient to each. Each system must provide pilot and ATGS/copilot with separate controls for selection of receiver audio outputs and transmitter microphone/Push to Talk (PTT) audio inputs for all installed radios and Public Address (PA) systems. Each system must also provide pilot and ATGS/copilot with separate controls for adjustment of both the intercommunications system (ICS) and the receiver audio output levels.

B7.5.1.1 Transmitter selection and operation. Separate transmitter selection controls must be provided for the microphone/PTT inputs of both pilot and ATGS/copilot. The system must be configured so that the pilot, ATGS/copilot, and (where required) Aft ATGS Instructor may each simultaneously select and utilize a different transmitter (or PA system when installed) via their respective microphone/PTT. Whenever a transmitter is selected, the companion receiver audio must automatically be selected for the corresponding earphone. Transmitter sidetone audio must be provided for the user as well as for cross-monitoring via the corresponding receiver selection switch on the other audio control system.

B7.5.1.2 Receiver selection and operation.  Separate controls must be provided for both pilot and ATGS/co-pilot selection of audio from one or any combination of available receivers. Any ICS-equipped aft seat passenger positions not furnished with its own audio system must monitor the receiver(s) as selected by the ATGS/copilot. The receiver audio output must be free of excessive distortion, hum, noise, and crosstalk, and must be amplified sufficiently to facilitate ease of use in a noisy cockpit/cabin environment.

B7.5.1.3 The controls of the audio system(s) must be located and arranged so that both the pilot and ATGS/copilot, when seated, have full and unrestricted movement of each control without interference from their clothing, the cockpit structure, or the flight controls. Labeling and marking of controls must be clear, understandable, legible, and permanent. Electronic label maker marking is acceptable.

B7.5.1.4 Aft ATGS Instructor Station.  One aft cabin position must be provided for use by an ATGS instructor. A third audio control system, providing the same functions as those described in B7.5.1 et. Seq. above, is required at this position.

B7.5.2 An ICS must be provided for the pilot, ATGS/copilot, the aft cabin ATGS instructor, and at least one other aft seat passenger positions. ICS audio must mix with, but not mute, selected receiver audio. An ICS audio level control must be provided for each position above. Adjustment of the ICS audio level at any position must not affect the level at any other position. A “hot mic" capability, controlled via voice activation (VOX), must be provided for each position above. ICS sidetone audio must be provided for the earphones corresponding with the microphone in use. The ICS audio output must be free of excessive distortion, hum, noise, and crosstalk, and must be amplified sufficiently to facilitate ease of use in a noisy cockpit/cabin environment.

B7.5.3 Earphones, microphones, PTTs, and jacks designed for operation with 600-ohm earphones and carbon-equivalent, noise-canceling boom type microphones (Gentex electret type Model 5060 2, military dynamic typeM 87/AIC with CE 100 TR preamplifier, or equivalent) must be furnished at all required positions.

B7.5.3.1 A single set of J-033 and J-034 type connector jacks, providing all specified radio and ICS functions, must be provided at each required position in the aircraft. These jacks must accept the PJ-055B and PJ-068 type connector plugs as utilized with the headset/ microphone.

B7.5.3.2 Separate PTT switches must be provided for radio transmitter microphone operation at the pilot, ATGS/copilot, and aft cabin ATGS instructor positions. The pilot's PTT switch must be mounted on the control yoke. The PTT switches for the ATGS/copilot and ATGS instructor positions must be mounted on the cord to the headset/microphone connector, or otherwise conveniently located for use, However, PTT switches for the ATGS/copilot, other than any mounted on the flight controls must also be furnished.

B7.6 Other Required Avionics

B7.6.1 One Air Traffic Control (ATC) transponder and altitude reporting system meeting the requirements of 14 CFR Part 91.215 (a) and (b) and inspected and tested every 24 months in accordance with 14 CFR Part 91.413.

B7.6.2 The aircraft must be equipped with an FAA Technical Standard Order (TSO) approved traffic awareness and avoidance system (TCAS/TAS/TCAD/etc.) featuring active interrogation of threat aircraft. The system’s antennas must be mounted on the aircraft to minimize airframe shadowing and provide 360-degree coverage. The system must also incorporate visual alerts for both pilot and co-pilot and an aural alerting feature which announces an alert of threat aircraft whenever such aircraft enter a zone of a programmable size with range selections from no more than two nautical miles to at least ten nautical miles around the aircraft. The audio alert output must be interconnected to the aircraft's audio system in such a manner that all ICS-equipped positions will receive the alert.  If the aircraft is equipped with a Multi-Function Display (MFD), GPS, or other system capable of displaying threat aircraft output data, the system must also be interfaced to such.

B7.6.2.1 The system must be installed in accordance with a STC or FAA field approval based upon an existing STC and the manufacturers installation manual. Installation of the system must be accomplished by a certified avionics repair station which has been approved for such by the system’s manufacturer. The system must be maintained for continued airworthiness but may be listed in an approved Minimum Equipment List (MEL), provided the MEL does not permit the system to be inoperable for a period exceeding 15 days.

B7.6.2.2 The following systems are known to meet the above requirements:

TCAS: Any TSO’d system which provides a range selection of 2 nautical miles or less.

TAS: Bendix-King KTA 870, Goodrich Skywatch HP

TCAD: Avidyne TAS600/605/615/620 (formerly Ryan International TCAD 9900BX) only when a separate, approved MFD is used to display threat aircraft data.

B7.6.2.3 Satellite-based Traffic Information Systems (TIS), and Automatic Dependent Surveillance - Broadcast (ADS-B) systems do not meet the above requirements.

B7.6.4 Automatic Dependent Surveillance – Broadcast (ADS-B)

B7.6.4.1 All aircraft must be equipped to meet the ADS-B OUT requirements of 14 CFR Part 91.225. ADS-B OUT systems must be approved to either TSO-C154c (978MHz Universal Access Transceiver [UAT]) or TSO-C166b (1090MHz Extended Squitter [1090ES]). Aircraft operating outside of the United States must be equipped with systems approved to TSO-C166b.

B7.6.4.2 ADS-B IN system is required, system furnished must be TSO-approved, use diversity antennas on both the top and bottom of the aircraft, receive both UAT and 1090ES, and be interfaced to a multifunction display (MFD) capable of displaying TIS-B traffic and FIS-B weather.

B7.6.4.3 All ADS-B IN systems installed in aircraft that are required to transition from the contiguous United States through Canada to Alaska are required to use TSO-approved diversity antennas on both the top and bottom of the aircraft.

B7.6.4.4 All aircraft must be equipped with Bluetooth or Wi-Fi wireless connectivity to allow ADS-B IN information to be displayed on multiple government personnel-owned tablets or mobile devices using third party application software. ADS-B IN information must include the aircraft current position, TIS-B traffic, and FIS-B weather.

B7.6.5 One thunderstorm detection (or weather radar) system, meeting the same requirements as those specified in 14 CFR Part 135.173.

B7.6.6 One Terrain Awareness and Warning System class B (TAWS-B) or Enhanced Ground Proximity Warning System (EGPWS) ensuring they are operational and maintained per manufacturer guidelines. TAWS-B or EGPWS installations will comply with applicable regulations in the Code of Federal Regulations (CFR), specifically 14 CFR Part 91, and TSO-C151c. The audio alert output must be interconnected to the aircraft's audio system in such a manner that flight crew positions will receive the alert.  The aircraft must include an approved terrain situational awareness display such as a Multi-Function Display (MFD), GPS, or other system capable of displaying TAWS/EGPWS output data.

B7.6.6.1 The Contractor shall also install a pilot-controlled TAWS inhibit switch in the aircraft. This switch must enable the flight crew to temporarily disable TAWS alerts under specific conditions where the alerts may not be applicable. The following requirements apply:

The switch must be clearly labeled and easily accessible to the pilot.

The system must provide visual or audible confirmation when the inhibit function is activated.

The switch must comply with all relevant regulatory standards and guidance.

B7.6.6.2 The Airplane Flight Manual (AFM) shall contain appropriate procedures for: The use of the terrain awareness and warning system; and proper flight crew reaction in response to the terrain awareness and warning system audio and visual warnings.

B7.6.7. Reserved.

B7.6.8 The aircraft must be equipped with two permanently installed circuit breaker-protected, dual USB charging ports. The ports must be approved under TSO-C71, providing a maximum of 2.1A power at 5VDC and mounted in a location convenient to the pilot and copilot/observer positions. (Cigarette lighter adapter is not acceptable).

## B8 Reserved

# PERSONNEL REQUIREMENTS

## B9 Pilot Requirements

B9.1 The Contractor must furnish a pilot for each day the aircraft is required to be available (see B10.2.6).

B9.2 The pilot must be familiar with this contract and all applicable task orders issued under this contract.

B9.2.1 The pilot must be able to provide contract and/or task order information to the AOM Technical Inspector, AOM Inspector, Contracting Officer’s Representative (COR), project inspector (PI), or Government Manager as requested.

## B10 Pilot Qualifications

B10.1 General.

Further verification of pilot flight hours must be provided if requested by the COR.

B10.2 Minimum Qualifications.

The Contractor must provide a pilot(s) who meets the following minimum qualifications and who possesses the required certificates and evidence of having satisfactorily passed the evaluations for the required tasks:

B10.2.1 An FAA commercial pilot certificate or higher with airplane category, appropriate class(es), type if required and instrument rating for offered aircraft.

B10.2.2 A minimum of a current second-class medical certificate issued in accordance with 14 CFR Part 67.

B10.2.3 A current FAA competency check completed in accordance with 14 CFR Part 135.293(a and b) in the same make and model as the contract aircraft; a current 14 CFR Part 135.299 line check; and, if appropriate (for airplane multiengine and single engine turbine), a current 14 CFR Part 135.297 instrument proficiency check.

B10.2.4 An initial and 60-month recurrent ATGS, fire-reconnaissance and resource-reconnaissance agency flight evaluation is required to be flown, and any flight evaluations thereafter are at the COR’s discretion in the same make and model as the contract aircraft. The Contractor must supply the aircraft for the flight evaluation at no expense to the Government.

B10.2.5 Proficient operation of all equipment identified in Section B (e.g., aircraft, FM radios and GPS). The agencies may require pilots to demonstrate this proficiency with an AOM/USFS approved inspector pilot during an evaluation flight.

B10.2.6 The Contractor must complete an OAS-64A for each contract pilot. The OAS-64A must be submitted to the AOM Inspector Pilot annually and prior to any carding or pilot inspection event. The AOM 64A can be found at: https://www.doi.gov/aviation/library/forms#inspforms

B10.2.7 Reserved.

B10.2.8 PIC Qualifications

(a) 1,500 hours…total pilot time.

(b) 1,200 hours…Pilot in command in category.

(c) 200 hours…in category/class to be flown.

(d) 100 hours…night flying to include the recent flight experience requirements of 14 CFR 61.

(e) 75 hours…actual or simulated instrument flight time (including 50 hours in flight).

(f) 100 hours…total time in class, preceding 12 months.

(g) 250 hours…large airplanes over 12,500lbs if offered aircraft is over 12,500lbs MTOW.

(h) 50 hours…PIC in make and model of aircraft to be used on this contract. (25 hours if the pilot has attended a formal training course which includes ground school for the make and model and a flight check.).

(i) 100 hours…PIC in Turboprop or jet airplanes as appropriate.

(j) 200 hours…in mountainous terrain or 100 hours after successfully graduating from a recognized mountain flying school. See Exhibit 2 for recognized schools. Mountainous terrain experience is defined as experience in operating airplanes in mountainous terrain as identified in 14 CFR 95 Subpart B, Designated Mountainous Area. Operating includes maneuvering near terrain, crossing ridgelines, and evaluating conditions such as wind, temperature, and density altitude.

Note: RE: B10.2.8(k) The Contractor may request that this pilot flight hour requirement be waived for a pilot under special circumstances; however, the waiver may or may not be granted. The Contractor should contact the CO in advance of this need for additional information on this process. No other pilot qualification exceptions will be considered by the Government.

Vendor in-house FAA approved 14 CFR Part 135 training courses do not qualify for the 50% reduction in B10.2.8(i).

## B11 Flight Crewmembers Duty and Flight Limitations

Assigned duty of any kind must not exceed 14 hours in any 24-hour period. “Duty” includes flight time, ground duty of any kind, and standby. Local travel up to a maximum of 30 minutes each way between the worksite and place of lodging will not be considered duty time. Flight crewmembers will be subject to the following duty hour limitations:

B11.1 A maximum of 14 consecutive duty hours during any assigned duty period.

B11.1.1 All crewmembers must have two 24-hour periods of rest (off duty) within any 14 consecutive calendar days. In the contiguous United States, these two 24-hour rest periods must be 2 calendar days off duty.

B11.1.2 The pilot must be given a minimum of 10 consecutive hours of rest (off duty) prior to any assigned duty period.

B11.1.3 Regardless of the above limits, pilots are expected to notify the government if they become fatigued prior to reaching the duty day limit.

B11.2 Flight Limitations.

B11.2.1 Each crewmember must report all flight time, regardless of how or where performed, except personal pleasure flying. Crewmembers and relief crewmembers reporting for duty may be required to furnish a record of all duty and/or flight time during the previous 14 days. This record will be used to administer flight and duty time limitations.

B11.2.2 Flight time to and from a duty station as a flight crewmember (commuting) must be reported and counted toward limitations if it is flown on a duty day. Flight time includes but is not limited to military flight time, charter, flight instruction, 14 CFR Part 61.56 flight review, flight examinations by FAA designees, any flight time for which a flight crewmember is compensated, or any other flight time of a commercial nature whether compensated or not.

B11.2.3 Flight crewmembers will be limited to the following flight hour limitations, which must fall within their duty hour limitations:

B11.2.3.1 A maximum of 8 hours flight time during any assigned duty period.

B11.2.3.2 A maximum of 42 hours flight time during any consecutive 6-day period. When a pilot acquires 36 or more flight hours in a consecutive 6-day period, the pilot will be given the following 1 calendar day off duty for rest, after which a new 6-day cycle will begin.

## B12 Personnel Duty Limitations

B12.1 The Contractor must monitor and remove from duty any personnel for fatigue or other causes before they reach their daily duty or flight limitations.

B12.2 Federal agencies may issue a notice reducing the length of personnel duty days and/or increasing days off either on a geographic or agency-wide basis.

## B13 Mechanic Requirement

B13.1 A mechanic must maintain the aircraft in accordance with the Contractor’s FAA-approved maintenance program.

B13.2 For the Billings/Fairbanks location only. When assigned to the designated base in Fairbanks, AK, the Contractor must furnish a mechanic who is on-site and immediately available to service and inspect the contract aircraft at the designated/alternate base.

B13.3 For operations in the contiguous United States, The mechanic does not need to remain at the designated/alternate base but must be available when aircraft maintenance is required.

## B14 Mechanic Qualifications

The Contractor may enter into an agreement with a qualified mechanic or qualified maintenance facility. The mechanic provided to support this contract must possess the required certificates and minimum qualifications shown below:

B14.1 The mechanic must have a valid FAA mechanic certificate with airframe and powerplant (A&P) ratings and must have held the certificate, or foreign equivalent, with both ratings for a period of 24 months.

B14.2 The mechanic must have been actively engaged in aircraft maintenance as a certificated mechanic for at least 18 months out of the last 24 months.

B14.3 The mechanic must have 12 months experience as an A&P mechanic, or foreign equivalent, in maintaining the same category (i.e. airplane/rotorcraft) of aircraft as the contract aircraft (3 of those 12 months must have been in the preceding 24 months).

B14.4 The mechanic must have 12 months maintenance experience on the same make and model as the contract aircraft or satisfactory completion of a manufacturer, or equivalent, maintenance course for the same make and model as the contract aircraft.

B14.5 For the Billings/Fairbanks location only. The mechanic must show evidence of maintaining an aircraft of the same make and model as the contract aircraft under “field” conditions for one full season. Three months experience maintaining the aircraft away from the operator’s base of operations, while under minimal supervision, will meet this requirement.

B14.6 For the Billings/Fairbanks location only. The contractor will provide the AOM Technical Inspector, or their designated representative, with a completed OAS-41 Aircraft Maintenance Personnel Experience Summary (https://www.doi.gov/aviation/library/forms#inspforms) for mechanics provided under paragraph B13.1.2. The individual mechanic and a contractor representative (Director of Maintenance or higher) must sign the form to indicate that the data listed has been verified from logbooks, employment records, etc.

B14.6.1 For the Billings/Fairbanks location only. When requested by the Government, each mechanic must furnish a valid Interagency Mechanic Qualification Card for review. The designated Interagency Maintenance Inspector will issue the card.

## B15 Mechanic Duty Limitations

Contractor supplied mechanics deployed to the aircraft’s operating location (on-site) for field maintenance must not exceed the following duty limitations:

B15.1 Within any 24-hour period, mechanics must have a minimum of 8 consecutive hours off duty immediately prior to the beginning of any duty day. Local travel up to a maximum of 30 minutes each way between the worksite and place of lodging will not be considered duty time.

B15.2 Mechanics must have 2 full days off duty during any 14-day period during the performance of this contract. Off duty days need not be consecutive.

B15.3 “Duty time” includes availability and work or alert status at any job site for which a mechanic is compensated, or any other time of a commercial nature whether compensated or not.

B15.4 The Government may further restrict duty hours and may remove the mechanic for fatigue or other causes before reaching their daily duty limitations.

B15.5 The mechanic is responsible for keeping the Government apprised of his or her duty limitation status.

B15.6 Relief or substitute mechanics reporting for duty may be required to furnish a record of all duty time during the previous 14 days.

## B16 Reserved

## B17 Reserved

## B18 Relief Crew Requirement

B18.1 The Contractor must provide a qualified relief crew, consisting of a pilot that is available to perform duties during the regular crewmember’s scheduled days off.

B18.2 The Contractor must provide a planned schedule of relief duty days to the COR at the start of any EU period. Relief crewmembers must arrive at the designated base before the scheduled duty period begins to ensure compliance with rest periods set forth herein.

# OPERATIONS

## B19 Pilot Authority and Responsibility

The Contractor must ensure that the pilot is responsible for (1) operating the aircraft within its operating limits, (2) the safety of the aircraft, (3) its occupants, and (4) the cargo.

The contract pilot:

B19.1 Must have the authority to represent the Contractor in all matters except changes in price and time unless the CO is notified otherwise, in writing, prior to performance. The pilot must be familiar with the contract and all applicable task orders assigned to this contract and must be able to provide contract and/or task order information to the PI or manager as requested.

B19.2 Must comply with Government directions except, when in the pilot's judgment, such compliance would violate Federal or State regulations or contract terms and conditions. The pilot has final authority to determine whether the flight can be accomplished safely and must refuse any flight or landing that is considered hazardous or unsafe.

B19.3 Must not permit any passenger to ride in the aircraft or any cargo to be loaded therein unless authorized by the COR or his/her authorized representative.

B19.4 Is responsible for computing the aircraft’s weight and balance for all flights and for ensuring that the gross weight and center of gravity do not exceed the aircraft's limitations. The pilot must also properly secure all cargo.

B19.5 Must perform takeoff performance calculations which include:

Take off distance required vs. runway available.

Climb performance to include single engine performance.

Under no circumstances will a takeoff or landing be attempted if existing environmental conditions at takeoff (e.g., temperature and density altitude) cannot be accurately addressed in the Aircraft Flight Manual (AFM) or Pilot’s Operating Handbook (POH).

B19.6 May not perform preventative maintenance in accordance with 14 CFR Part 43.3(h) unless authorized by the Contractor’s FAA-issued operations specifications.

B19.7 May function as a mechanic when the aircraft is not available due to unscheduled maintenance provided the pilot meets the requirements of B14.1, B14.4, and the contractor’s operations specifications.  Must hold an A&P certificate and meet all of the mechanic qualifications required by the contractor’s operations specifications.

B19.8 Must not perform scheduled maintenance and inspections when on duty as the primary or relief pilot.

B19.9 Will apply against the pilot's duty day limitations any time during which the pilot is engaged in mechanic duties performing unscheduled maintenance, or as an authorized pilot performing preventative maintenance. All time in excess of 2 hours (not necessarily consecutive) will be applied against the pilot's flight hour limitations. After 2 hours, every hour spent as a mechanic, or as an authorized pilot performing preventative maintenance, will be applied against pilot flight time limitation on a one-to-one basis.

## B20 Flight Operations

Regardless of any status as a public aircraft operation, the Contractor must operate in accordance with their approved FAA operations specifications and all portions of 14 CFR Part 91 (including those portions applicable to civil aircraft) and each certification required under Section B2 unless otherwise authorized by the CO. The Contractor must ensure that all personnel operate in compliance with the following requirements:

B20.1 Manifesting. The PIC must ensure that a manifest of all crewmembers and passengers on board has been completed and that a copy of this manifest remains at the appropriate dispatch office. Manifest changes must be made with that office.

B20.2 Passenger briefings. Before each takeoff, the PIC must ensure that all passengers have been briefed in accordance with 14 CFR Part 135. Briefings for short flights do not need to be repeated unless new passengers come aboard. The briefing must describe the location/use of the following:

Emergency locator transmitter

Emergency Exits

First aid/survival kits

Personal protective equipment (PPE)

B20.3 The pilot must brief the occupant of a pilot position to remain clear of the flight controls at all times.

B20.4 Day/night use. Both day and night flight may be required by this contract.

B20.4.1 Night/IFR Operations. Only multiengine and single engine turbine aircraft are approved for transporting passengers at night or in instrument meteorological conditions. Pilots flying night missions shall not land at an airport unless it meets FAA airport lighting standards.

B20.5 Flight plans. Pilots must file and operate on an FAA, International Civil Aviation Organization (ICAO), or agency approved flight plan. Contractor flight plans are not acceptable. Flight plans must be filed prior to takeoff when possible.

B20.6 Flight following. Pilots are responsible for flight following with the FAA, ICAO, and/or in accordance with the agency approved procedures.

B20.7 Smoking (including electronic cigarettes and personal vaporizers) will not be allowed in the aircraft.

B20.8 The pilot must remain at the flight controls while any engine is running.

B20.9 Prior to passenger or cargo loading/unloading, all engines must be shut down, and all propellers or engines must have ceased rotation.

B20.10 Crew briefings. The Air Attack Pilot, Air Tactical Group Supervisor and ATGS trainee (when onboard) are considered the flight crew.

B20.9.1 Crew briefing. Pilot and ATGS must complete the Aerial Supervision - Aircraft Mission Checklist for each flight. Each section of the checklist must be completed at or before the implied time indicated on the checklist. A copy of the checklist is required to be in the aircraft on all ATGS missions (preferably a laminated hard copy) and utilized in accordance with the NWCG SAS. See Aircraft Mission Checklist Exhibit 1 in Section C. The aerial supervision mission checklist is attached as an exhibit to this contract and is an appendix in the SAS. Copies of the SAS may be obtained from any interagency fire cache; item NFES 2544.

B20.9.2 Pre takeoff briefings. Prior to departure the PIC must ensure the ATGS has given the pilot a mission safety briefing conducted in accordance with the SAS. And, in addition to a passenger briefing, the PIC must brief the flight crew. The briefing must include at least:

Emergency procedures

Crew Resource Management (CRM) procedures

Fuel on board

Performance considerations including maximum forecast temperature for the day and minimum runway requirements.

B20.9.3 Enroute briefing. After departure and before entering the Fire Traffic Area (FTA) the PIC must ensure a crew briefing is completed. The briefing will be done in accordance with the SAS and must include at least:

Communications/radios

Other aircraft on scene

Level of supervision on scene

Alternate airports

Initial altitude and altimeter setting

Time on scene, based on current fuel available and planned refueling location.

B20.10 Airplanes in Alaska may be required to operate from gravel runways. Contractors should not accept orders or dispatches requiring relocation to Alaska if the contractor is not willing to operate from gravel runways.

B20.11 Operational Pause. In the event of credible safety concerns that may pose an immediate or significant risk to personnel, operations, or the public, the Government reserves the right to initiate a full or partial pause of contractor operations. The contractor will be unavailable and shall comply with all directives and take immediate action to mitigate identified risks.

B20.11.1 – Return-to-Work Conditions Following Operational Pause. Following an operational pause, the Government will provide the contractor with written notification outlining the specific conditions required for resuming work. These conditions may include, but are not limited to, corrective actions, safety verifications, documentation of hazard mitigation, and any additional compliance measures deemed necessary.

## B21 Security of Aircraft and Equipment

B21.1 The Contractor is solely responsible for the security of their contract aircraft, vehicles, and associated equipment.

B21.2 Any aircraft used under this contract must be physically secured and disabled via a dual-lock method whenever the aircraft is unattended. Any combination of two different anti-theft devices designed to lock aircraft flight control surfaces when not in use, or designed to secure an aircraft to the ground, is acceptable, provided they are appropriate for the aircraft. Operational environments and personnel safety must be considered when selecting the locking devices and methods to be used.

B21.2.1 Removal and/or disabling of locking devices and methods must be incorporated into preflight checklists to prevent accidental damage to the aircraft. The devices must be installed in a manner that precludes their inadvertent interference with in-flight operations.

B21.2.2 Using other means of securing or disabling an aircraft is acceptable, provided it achieves a level of security equal to or greater than the following example locking devices and methods:

Locking hangar door

Keyed ignition switch

Keyed starter switch

Keyed master power switch

Hidden battery cutoff switches

Throttle/power lever lock

Mixture/fuel lever lock

Locking fuel cutoff

Locking tie-down cable

Locking control surface “gust-lock”

Propeller lock

Propeller chain lock

Propeller cable lock

Locking wheel lock or chock

Locking “club-type” devices for control yoke

The following locking devices and security methods do not satisfy the physical security requirements:

Locking aircraft doors

Fenced or gated parking area

## B22 Personnel Protective Equipment (PPE) for Flight Operations

B22.1 The Contractor must provide and require personnel to wear PPE in accordance with the Interagency Aviation Life Support Equipment (ALSE) Guide/Handbook, Chapter 2 Personal Protective Equipment.

B22.2 The DOI/USFS ALSE Guide/Handbook can be located here and the Aviation Helmet Standard, along with the list of approved helmets and associated certificates of compliance, are available on the AOM website at: https://www.doi.gov/aviation/safety/helmet.

## B23 Reserved

## B24 Transportation of Hazardous Materials

B24.1 Regardless of any FAA Air Carrier Operations Manual declaration of Will or Will Not Carry, the Contractor may be required to transport hazardous materials. Such transportation must be in accordance with 49 CFR, Department of Transportation Special Permit DOT-SP 9198, and the NWCG Standards for Aviation Transport of Hazardous Materials.

B24.2 A copy of the current Department of Transportation (DOT) Special Permit, DOI Grantee Status Letter, NWCG Standards, and DOT Emergency Response Guidebook must be carried aboard each aircraft transporting hazardous materials. All documents are available online at  www.doi.gov/aviation/library/guides.

B24.3 The Contractor must ensure that each employee who may perform a function subject to this DOT Special Permit has completed the Interagency Aviation Training (IAT) Training Module A-110, Aviation Transportation of Hazardous Materials, within the previous 3 years. The training can be completed online at https://www.iat.gov/. The Contractor must document this training in the employee’s records and make it available to the Government when requested.

## B25 Fuel and Servicing Requirements

B25.1 General

B25.1.1 The Contractor must supply all fuel and lubricating oils required to operate all equipment during the contract period. For operations in Alaska, Government fuel is provided. However, the contractor must be capable of purchasing fuel and understand that the Government will reimburse the contractor for the fuel purchased as provided under additional pay items in Section C. The contractor will be responsible for assuring fuel quality when procuring fuel from commercial sources.

B25.1.1.1 Government Furnished Fuel – When Operating in Alaska

B25.1.1.2 Grades of Government-furnished fuel vary from location to location, and the Contractor shall use the grade available.

B25.1.1.3 The appropriate type of fuel (Avgas or Jet fuel), in one of the following grades, will be available at each location:

| AVGAS | JET FUEL |
| --- | --- |
| 100 100LL | Jet A Jet-A 50 Jet B JP-4, JP-5, or JP-8 |

B25.1.2 All fuel must be commercial (or military) grade aviation fuel approved for use by the airframe and engine manufacturer. Only fuels meeting American Society for Testing and Material (ASTM) or Military Detail (MIL-DTL) specifications are authorized for use. ASTM D-1655 (Jet A, A-1, or B), MIL-DTL-83133 (JP-8).

B25.1.3 Contractors must ensure that fuel obtained from distributors or fixed-base operators (FBO) meets the specifications of B25.1.2 and the aircraft is serviced with the proper type of fuel. The Contractor must retain fuel delivery records throughout the contract period.

B25.2 Fueling Operations.

B25.2.1 Aircraft must not be refueled while any engine is running.

B25.2.2 The NFPA 407: Standard for Aircraft Fuel Servicing handbook must be used as a guide. Copies of NFPA 407 can be obtained from the NFPA, www.nfpa.org.

B25.2.3 Government personnel are not allowed in the safety zone during aircraft fueling operations. The safety zone is defined as within 50 feet of the aircraft refueling receptacle.

B25.2.4  Government personnel will not be involved with refueling of the contract aircraft, unless specifically trained/approved (Alaska operations only) or the pilot has determined that it is an absolute necessity due to an emergency situation.  Such emergencies must be documented by the pilot using the SAFECOM system.

B25.2.5 Smoking (to include electronic cigarettes/personal vaporizers) is prohibited within 50 feet of the aircraft and fuel servicing facilities/vehicles.

B25.2.6 Cell phone use is prohibited within 50 feet of the aircraft and fuel servicing facility/vehicle during fueling operations.

B25.2.7 A contract pilot must be present during all fueling operations.

B25.4 See Exhibit 13, Alaska Supplement

# AIRCRAFT MAINTENANCE REQUIREMENTS

## B26 General - Maintenance

B26.1 The Contractor must ensure that the aircraft and all required equipment are operated and maintained in accordance with the Original Equipment Manufacturers (OEM) or approved STC holder’s current instructions including airframe, engine, appliances, emergency equipment, and all instructions for continued airworthiness (ICA’s).

B26.1.1 The contractor must obtain an inspection program or supplement from the OEM/TC holder for the aerial supervision missions described in Section B1. The contractor shall provide a copy of the inspection program with the contract proposal, when received from the OEM/TC holder but no later than 12 months from date of award.

B26.1.2 The government acknowledges that the Aerial Supervision mission requires a flight profile or operating envelope that may deviate from that for which the aircraft was originally designed. See B1.1.1, Aerial Supervision Flight Profile, for a description of the mission.

B26.1.3 The Contractor is required to submit evidence that the offered aircraft OEM/TC holder will continue to provide engineering support for updating Instructions for Continued Airworthiness (ICA) and any elevated Airworthiness Limitations from a Fatigue and Damage Tolerance Evaluation if the aircraft is operated in accordance with this profile. If the aircraft has previously been flown in the aerial supervision environment or in other than a standard Part 135 operation, the Contractor will also submit evidence of previously developed OEM approved ICAs and elevated Airworthiness Limitations.

B26.2 Before the start date of the contract, the Contractor must ensure that all maintenance deficiencies have been corrected or deferred in accordance with 14 CFR Part 91.213 or the operator's FAA approved maintenance program. Deferred discrepancies will be evaluated, and the aircraft approved for contract use on a case-by-case basis.

B26.3 The Contractor must correct deficiencies that occur during contract performance in accordance with the appropriate Federal Aviation Regulations or the FAA approved maintenance program.

B26.4 The Contractor must immediately notify the COR and AOM Technical Inspector of any unscheduled maintenance, change to any engine, propeller, flight control, major airframe component, or any major repair following an incident or accident and must describe the circumstances involved.

B26.5 Operational Load Monitoring (OLM) to be installed at the Governments request and Governments expense. See Exhibit 14.

## B27 Airworthiness Directives (ADs) and Manufacturer’s Mandatory Service Bulletins (MMSBs)

B27.1 The Contractor must comply with all applicable FAA ADs and MMSBs before and during contract performance at the Contractor’s expense.

B27.2 The Contractor must provide, at the time of agency inspection, a list of issued FAAs ADs and MMSBs identifying all those that are applicable and not applicable to the contract aircraft. The list must be in a format similar to the one shown in AC 43-9C, Appendix 1, complete with authorized signature, certificate type and number. This list must include all accessories and equipment installed in each aircraft offered. Signature of the person verifying accuracy of the list is required.

## B28 Manuals/Records

B28.1 The Contractor must ensure that all aircraft maintenance is recorded in accordance with 14 CFR Parts 43, 91, and 135 (reference 14 CFR Parts 43.9, 43.11, 91.417, and 135.439). Aircraft total time in service must be recorded. A copy of the current maintenance record must be kept with the aircraft. Electronic copies of manuals and records are acceptable.

B28.2 The Contractor must provide any documentation or record requested by the AOM Technical Inspector regarding the aircraft, equipment, and/or maintenance.

B28.3 If requested by the Government, the Contractor must furnish to the AOM Technical Inspector a copy of the Contractor's procedures manuals, as outlined in 14 CFR Part 135.21, along with any revisions made during the contract period.

## B29 Maintenance

Aircraft must be maintained and inspected in accordance with the operator’s FAA approved maintenance program.

B29.1 All maintenance, including inspection, rebuilding, alteration, and installation must be accomplished by a person authorized to perform maintenance in accordance with 14 CFR Part 43. Contracted maintenance must be in accordance with 14 CFR 135.426.

B29.2 The Contractor must ensure that a mechanic who meets the contract qualification requirements, identified in B14, inspects the contract aircraft in accordance with the procedures outlined in the operator’s FAA approved maintenance program. Aircraft total time in service must be recorded.

B29.3 All aircraft maintenance entries must include a reference to the approved technical data used to perform any installation, overhaul, major repair, or replacement of any engine, propeller, flight control, or major airframe component. The reference must include the title of the maintenance publication and identify the procedure performed or chapter, page, and paragraph where the procedure can be found. Adherence to this requirement will begin the date of contract award and continue through the duration of the contract.

B29.4 Routine/preventative maintenance must be performed before or after the Government’s scheduled daily use period or as approved by the COR.

B29.5 The fire extinguisher must be maintained in accordance with NFPA 10: Standards for Portable Fire Extinguishers or the Contractor’s 135 operations manual.

B29.6 After contract award and for the duration of the contract, for aircraft that have an applicable inspection program available, the aircraft must be inspected and maintained in accordance with the OEM/TC holder’s aerial supervision inspection program.

B29.6.1 The Government will not accept Twin Commander 690 model aircraft that incorporate the Modified Saunders Spar, unless an OEM/TC holder approved inspection program is developed.

B29.6.2 Discrepancies identified from structural inspections must be repaired using OEM/TC holder repair data. Structural repairs not related to the aerial supervision inspection program must have documentation of “No Technical Objection” from the OEM/TC holder when OEM/TC holder repair data is not used.

## B30 Maintenance Test Flight

The Contractor must, at their own expense, perform an operational check flight following installation, overhaul, major repair, or replacement of any engine, propeller, flight control system, or when requested by the CO or AOM Technical Inspector. This must be accomplished before the aircraft resumes service under the contract. The pilot must enter the result of this test flight in the aircraft records, as outlined in 14 CFR Part 91.407.

## B31 Time Between Overhaul (TBO) and Life-Limited Parts

B31.1 All components, including engines, must be replaced upon reaching the manufacturer’s recommended TBO or FAA approved extension. Life-limited parts must be replaced at the specified time-in-service hours or cycles.

B31.2 Aircraft operated with components or accessories on FAA approved TBO extension programs are acceptable. The Contractor must be the holder of the approved extension authorization (not the owner if the aircraft is leased). The Contractor must operate in accordance with the extension authorization. The extension authorization must be kept with the aircraft.

B31.3 The Contractor must provide, at the time of agency inspection, a list of all items installed on the aircraft that are required to be overhauled or replaced on a specified time basis. This list must include the component’s name, part number, serial number, total time, service life (or inspection/overhaul time interval), time remaining, and time and date when the component was overhauled, replaced, or inspected.

## B32 Weight and Balance

The aircraft weight and balance report must include a (1) weight and balance record, (2) equipment list, and any (3) calculations due to changes in the equipment list.  A copy of the current weight and balance report must be kept with the aircraft.

B32.1 Weight and Balance Record: The aircraft's required weight and balance record must be determined by actual weighing of the aircraft.  The weight and balance record must be current based on the chart below. Scale readings must be recorded on the weight and balance record. Signature of a rated mechanic verifying accuracy of the record is required. This record is not superseded until the aircraft is reweighed.

| Weighing Frequency on All Aircraft |  |
| --- | --- |
| Aircraft | Frequency (Calendar Months) |
| Single engine airplanes | 60 |
| Multiengine airplanes | 36 |

B32.1.1 The aircraft must be weighed following any major repair, major alteration, or change to the equipment list that significantly affects the center of gravity of the aircraft.

B32.1.2 All aircraft must be weighed on scales that have been certified as accurate within the preceding 12 calendar months. Any accredited weights and measures laboratory may serve as the certifying agency. The weight and balance record must include the make, model, serial number, and calibration date of the scales used to weigh the aircraft.

B32.2 Equipment List: The Contractor must compile a complete list of equipment installed in the aircraft at the time of weighing. Each page of the equipment list must identify the specific aircraft by its serial and registration numbers and must be dated to indicate the last date of weighing. Weight and Balance Handbook (FAA-H-8083-1B) may be used as a guide.

B32.2.1 Items that may be easily removed or installed for aircraft configuration changes (seats, doors, radios, special mission equipment, etc.) must be listed and include the name, weight, and arm of each item.

B32.2.2 All required equipment listed in section B6 and B7 must be listed on the equipment list.

B32.3 Calculations: Each time equipment is removed or installed, and the aircraft is not reweighed, the aircraft’s weight and balance must be calculated. A weight and balance revision or continuous history form must be used to show the calculated weight and balance of the aircraft resulting from the change in equipment. This is an additional document and does not supersede the weight and balance record. Weight and Balance Handbook (FAA-H-8083-1B) may be used as a guide.

## B33 Reserved

# SECTION C-CONTRACT TERMS AND CONDITIONS

## C1 CONTRACT CLAUSES

All Offerors must comply with the following FAR, Department of the Interior Acquisition Regulation (DIAR) and Acquisition Services Directorate (AQD) clauses which apply to this acquisition:

## FAR 52.252-2 Clauses Incorporated by Reference. (Feb 1998)

This contract incorporates one or more clauses by reference, with the same force and effect as if they were given in full text. Upon request, the Contracting Officer will make their full text available. Also, the full text of a clause may be accessed electronically at this/these address(es): https://acquisition.gov

(End of clause)

## CLAUSES INCORPORATED BY REFERENCE

52.203-6 	Restrictions on Subcontractor Sales to the Government		Jun 2020

52.203-6 Alt 1	Restrictions on Subcontractor Sales to the Government (Alternate 1)	Nov 2021

52.203-13	Contractor Code of Business Ethics and Conduct			Nov 2021

52.203-17	Contractor Employee Whistleblower Rights				Nov 2023

52.203-19	Prohibition on Requiring Certain Internal Confidentiality		Jan 2017

Agreements or Statements

52.204-13	System for Award Management – Maintenance			Oct 2018

52.209-6		Protecting the Government’s Interest When Subcontracting With	Jan 2025

Contractors Debarred, Suspended, Proposed for Debarment, or

Voluntarily Excluded

52.209-9		Updates of Publicly Available Information Regarding 		Oct 2018

Responsibility Matters

52.209-10	Prohibition on Contracting with Inverted Domestic Corporations	Nov 2015

52.212-4		Contract Terms and Conditions – Commercial Products and 		Nov 2023

Commercial Services

52.212-4 Alt 1	Contract Terms and Conditions – Commercial Products and 		Nov 2021

Commercial Services

52.219-32	Orders Issued Directly Under Small Business Reserve		Mar 2020

52.222-3		Convict Labor							June 2003

52.222-19	Child Labor-Cooperation with Authorities and Remedies		Mar 2026

52.222-35	Equal Opportunity for Veterans					Jun 2020

52.222-37	Employment Reports on Veterans					Jun 2020

52.222-40	Notification of Employee Rights Under the National Labor		Dec 2010

Relations Act

52.222-41	Service Contract Labor Standards					Aug 2018

WD 1995-0221 Rev 73

WD 1995-0222 Rev 71

52.222-43	Fair Labor Standards Act and Service Contract Labor Standards- 	Aug 2018

Price Adjustment (Multiple Year and Option Contracts)

52.222-50	Combat Trafficking in Persons					Oct 2025

52.222-54	Employment Eligibility Verification				Jan 2025

52.222-62	Paid Sick Leave Under Executive Order 13706			Jan 2022

52.232-29	Terms for Financing of Purchases of Commercial Products and 	Nov 2021

Commercial Services

52.232-33	Payment by Electronic Funds Transfer – System for Award 		Oct 2018

Management

52.233-3		Protest After Award						Aug 1996

52.233-4		Applicable Law for Breach of Contract Claim			Oct 2004

## CLAUSES INCORPORATED BY FULL TEXT

### FAR 52.216-5 Price Redetermination – Prospective (Jan 2022)

(a) General. The unit prices and the total price stated in this contract shall be periodically redetermined in accordance with this clause, except that-

(1) The prices for supplies delivered and services performed before the first effective date of price redetermination (see paragraph (c) of this clause) shall remain fixed; and

(2) In no event shall the total amount paid under this contract exceed any ceiling price included in the contract.

(b) Definition. "Costs," as used in this clause, means allowable costs in accordance with part  31 of the Federal Acquisition Regulation (FAR) in effect on the date of this contract.

(c) Price redetermination periods. For the purpose of price redetermination, performance of this contract is divided into successive periods. The first period shall extend from the date of the contract to 8/22/2026-8/21/2031 and the second and each succeeding period shall extend from 8/22/2031-8/21/2036 months from the end of the last preceding period, except that the parties may agree to vary the length of the final period. The first day of the second and each succeeding period shall be the effective date of price redetermination for that period.

(d) Data submission.

(1) Not more than 180 days  before the end of each redetermination period, except the last, the Contractor shall submit-

(i) Proposed prices for supplies that may be delivered or services that may be performed in the next succeeding period, and-

(A) An estimate and breakdown of the costs of these supplies or services in the format of Table  15-1, FAR 15.408, or in any other form on which the parties may agree;

(B) Sufficient data to support the accuracy and reliability of this estimate; and

(C) An explanation of the differences between this estimate and the original (or last preceding) estimate for the same supplies or services; and

(ii) A statement of all costs incurred in performing this contract through the end of the 60th month before the submission of proposed prices in the format of Table  15-1, FAR 15.408 (or in any other form on which the parties may agree), with sufficient supporting data to disclose unit costs and cost trends for-

(A) Supplies delivered and services performed; and

(B) Inventories of work in process and undelivered contract supplies on hand (estimated to the extent necessary).

(2) The Contractor shall also submit, to the extent that it becomes available before negotiations on redetermined prices are concluded-

(i) Supplemental statements of costs incurred after the date stated in subdivision (d)(1)(ii) of this section for-

(A) Supplies delivered and services performed; and

(B) Inventories of work in process and undelivered contract supplies on hand (estimated to the extent necessary); and

(ii) Any other relevant data that the Contracting Officer may reasonably require.

(3) If the Contractor fails to submit the data required by paragraphs (d)(1) and (2) of this section, within the time specified, the Contracting Officer may suspend payments under this contract until the data are furnished. If it is later determined that the Government has overpaid the Contractor, the Contractor shall repay the excess to the Government immediately. Unless repaid within 30 days after the end of the data submittal period, the amount of the excess shall bear interest, computed from the date the data were due to the date of repayment, at the rate established in accordance with the Interest clause.

(e) Price redetermination. Upon the Contracting Officer’s receipt of the data required by paragraph (d) of this section, the Contracting Officer and the Contractor shall promptly negotiate to redetermine fair and reasonable prices for supplies that may be delivered or services that may be performed in the period following the effective date of price redetermination.

(f) Contract modifications. Each negotiated redetermination of prices shall be evidenced by a modification to this contract, signed by the Contractor and the Contracting Officer, stating the redetermined prices that apply during the redetermination period.

(g) Adjusting billing prices. Pending execution of the contract modification (see paragraph (f) of this section), the Contractor shall submit invoices or vouchers in accordance with the billing prices stated in this contract. If at any time it appears that the then-current billing prices will be substantially greater than the estimated final prices, or if the Contractor submits data showing that the redetermined price will be substantially greater than the current billing prices, the parties shall negotiate an appropriate decrease or increase in billing prices. Any billing price adjustment shall be reflected in a contract modification and shall not affect the redetermination of prices under this clause. After the contract modification for price redetermination is executed, the total amount paid or to be paid on all invoices or vouchers shall be adjusted to reflect the agreed-upon prices, and any requested additional payments, refunds, or credits shall be made promptly.

(h) Quarterly limitation on payments statement. This paragraph (h) applies only during periods for which firm prices have not been established.

(1) Within 45 days after the end of the quarter of the Contractor’s fiscal year in which a delivery is first made (or services are first performed) and accepted by the Government under this contract, and for each quarter thereafter, the Contractor shall submit to the contract administration office (with a copy to the contracting office and the cognizant contract auditor) a statement, cumulative from the beginning of the contract, showing-

(i) The total contract price of all supplies delivered (or services performed) and accepted by the Government and for which final prices have been established;

(ii) The total costs (estimated to the extent necessary) reasonably incurred for, and properly allocable solely to, the supplies delivered (or services performed) and accepted by the Government and for which final prices have not been established;

(iii) The portion of the total interim profit (used in establishing the initial contract price or agreed to for the purpose of this paragraph (h)) that is in direct proportion to the supplies delivered (or services performed) and accepted by the Government and for which final prices have not been established; and

(iv) The total amount of all invoices or vouchers for supplies delivered (or services performed) and accepted by the Government (including amounts applied or to be applied to liquidate progress payments).

(2) The statement required by paragraph (h)(1) of this section need not be submitted for any quarter for which either no costs are to be reported under subdivision (h)(1)(ii) of this section, or revised billing prices have been established in accordance with paragraph (g) of this section, and do not exceed the existing contract price, the Contractor’s price-redetermination proposal, or a price based on the most recent quarterly statement, whichever is least.

(3) Notwithstanding any provision of this contract authorizing greater payments, if on any quarterly statement the amount under subdivision (h)(1)(iv) of this section exceeds the sum due the Contractor, as computed in accordance with subdivisions (h)(1)(i), (ii), and (iii) of this section, the Contractor shall immediately refund or credit to the Government the amount of this excess. The Contractor may, when appropriate, reduce this refund or credit by the amount of previous refunds or credits affected under this clause. If any portion of the excess has been applied to the liquidation of progress payments, then that portion may, instead of being refunded, be added to the unliquidated progress payment account, consistent with the Progress Payments clause. The Contractor shall provide complete details to support any claimed reductions in refunds.

(4) If the Contractor fails to submit the quarterly statement within 45 days after the end of each quarter and it is later determined that the Government has overpaid the Contractor, the Contractor shall repay the excess to the Government immediately. Unless repaid within 30 days after the end of the statement submittal period, the amount of the excess shall bear interest, computed from the date the quarterly statement was due to the date of repayment, at the rate established in accordance with the Interest clause.

(i) Subcontracts. No subcontract placed under this contract may provide for payment on a cost-plus-a-percentage-of-cost- basis.

(j) Disagreements. If the Contractor and the Contracting Officer fail to agree upon redetermined prices for any price redetermination period within 60 days (or within such other period as the parties agree) after the date on which the data required by paragraph (d) of this section are to be submitted, the Contracting Officer shall promptly issue a decision in accordance with the Disputes clause. For the purpose of paragraphs (f), (g), and (h) of this section, and pending final settlement of the disagreement on appeal, by failure to appeal, or by agreement, this decision shall be treated as an executed contract modification. Pending final settlement, price redetermination for subsequent periods, if any, shall continue to be negotiated as provided in this clause.

(k) Termination. If this contract is terminated, prices shall continue to be established in accordance with this clause for (1) completed supplies and services accepted by the Government and (2) those supplies and services not terminated under a partial termination. All other elements of the termination shall be resolved in accordance with other applicable clauses of this contract.

(End of clause)

### FAR 52.216-18 Ordering (Aug 2020)

(a) Any supplies and services to be furnished under this contract shall be ordered by issuance of delivery orders or task orders by the individuals or activities designated in the Schedule. Such orders may be issued from 8/22/2026-8/21/2036.

(b) All delivery orders or task orders are subject to the terms and conditions of this contract. In the event of conflict between a delivery order or task order and this contract, the contract shall control.

(c) A delivery order or task order is considered "issued" when—

(1) If sent by mail (includes transmittal by U.S. mail or private delivery service), the Government deposits the order in the mail;

(2) If sent by fax, the Government transmits the order to the Contractor's fax number; or

(3) If sent electronically, the Government either—

(i) Posts a copy of the delivery order or task order to a Government document access system, and notice is sent to the Contractor; or

(ii) Distributes the delivery order or task order via email to the Contractor's email address.

(d) Orders may be issued by methods other than those enumerated in this clause only if authorized in the contract.

(End of clause)

### FAR 52.216-19 Order Limitations (Oct 1995)

(a) Minimum order. When the Government requires supplies or services covered by this contract in an amount of less than $500, the Government is not obligated to purchase, nor is the Contractor obligated to furnish, those supplies or services under the contract.

(b) Maximum order. The Contractor is not obligated to honor-

(1) Any order for a single item in excess of $1,000,000;

(2) Any order for a combination of items in excess of $4,500,000; or

(3) A series of orders from the same ordering office within 7 days that together call for quantities exceeding the limitation in paragraph (b)(1) or (2) of this section.

(c) If this is a requirements contract (i.e., includes the Requirements clause at subsection 52.216-21 of the Federal Acquisition Regulation (FAR)), the Government is not required to order a part of any one requirement from the Contractor if that requirement exceeds the maximum-order limitations in paragraph (b) of this section.

(d) Notwithstanding paragraphs (b) and (c) of this section, the Contractor shall honor any order exceeding the maximum order limitations in paragraph (b), unless that order (or orders) is returned to the ordering office within 2 days after issuance, with written notice stating the Contractor’s intent not to ship the item (or items) called for and the reasons. Upon receiving this notice, the Government may acquire the supplies or services from another source.

(End of clause)

### FAR 52.216-22 Indefinite Quantity (Oct 1995)

(a) This is an indefinite-quantity contract for the supplies or services specified, and effective for the period stated, in the Schedule. The quantities of supplies and services specified in the Schedule are estimates only and are not purchased by this contract.

(b) Delivery or performance shall be made only as authorized by orders issued in accordance with the Ordering clause. The Contractor shall furnish to the Government, when and if ordered, the supplies or services specified in the Schedule up to and including the quantity designated in the Schedule as the "maximum." The Government shall order at least the quantity of supplies or services designated in the Schedule as the "minimum."

(c) Except for any limitations on quantities in the Order Limitations clause or in the Schedule, there is no limit on the number of orders that may be issued. The Government may issue orders requiring delivery to multiple destinations or performance at multiple locations.

(d) Any order issued during the effective period of this contract and not completed within that period shall be completed by the Contractor within the time specified in the order. The contract shall govern the Contractor’s and Government’s rights and obligations with respect to that order to the same extent as if the order were completed during the contract’s effective period; provided, that the Contractor shall not be required to make any deliveries under this contract after 12 months after the expiration of the IDIQ.

(End of clause)

### FAR 52.216-32 Task-Order and Delivery-Order Ombudsman (Sep 2019)

(a) In accordance with 41 U.S.C. 4106(g), the Agency has designated the following task-order and delivery-order Ombudsman for this contract. The Ombudsman must review complaints from the Contractor concerning all task-order and delivery-order actions for this contract and ensure the Contractor is afforded a fair opportunity for consideration in the award of orders, consistent with the procedures in the contract.

Appointed Competition Advocate for the Department of the Interior:

Kaprice Tucker

Email: kaprice_tucker@ios.doi.gov

Phone Number: 202-208-3466

(b) Consulting an ombudsman does not alter or postpone the timeline for any other process (e.g., protests).

(c) Before consulting with the Ombudsman, the Contractor is encouraged to first address complaints with the Contracting Officer for resolution. When requested by the Contractor, the Ombudsman may keep the identity of the concerned party or entity confidential, unless prohibited by law or agency procedure.

(End of clause)

### FAR 52.217-8 Option to Extend Services (Nov 1999)

The Government may require continued performance of any services within the limits and at the rates specified in the contract. These rates may be adjusted only as a result of revisions to prevailing labor rates provided by the Secretary of Labor. The option provision may be exercised more than once, but the total extension of performance hereunder shall not exceed 6 months. The Contracting Officer may exercise the option by written notice to the Contractor prior to the expiration date of the contract.

(End of clause)

### FAR 52.217-9 Option to Extend the Term of the Contract (Mar 2000)

(a) The Government may extend the term of this contract by written notice to the Contractor  prior to the expiration date of the contract; provided that the Government gives the Contractor a preliminary written notice of its intent to extend prior to the expiration date of the contract. The preliminary notice does not commit the Government to an extension.

(b) If the Government exercises this option, the extended contract shall be considered to include this option clause.

(c) The total duration of this contract, including the exercise of any options under this clause, shall not exceed 120 months.

(End of clause)

### FAR 52.222-42 Statement of Equivalent Rates for Federal Hires (May 2014)

In compliance with the Service Contract Labor Standards statute and the regulations of the Secretary of Labor ( 29 CFR Part 4), this clause identifies the classes of service employees expected to be employed under the contract and states the wages and fringe benefits payable to each if they were employed by the contracting agency subject to the provisions of 5 U.S.C.5341 or 5 332.

This Statement is for Information Only: It is not a Wage Determination

| Employee Class | Monetary Wage – Fringe Benefits |
| --- | --- |
| Aircraft Pilot, Hawaii GS-2181-11, Step 1 | $36.24 |
| Aircraft Mechanic, WG-8852-10, Step 2 | $31.36 |
| Fringe benefits such as, life, accident, health insurance, and sick leave, are not less than 5.1 percent of the basic hourly rate. The percentage of the basic hourly rate that is contributed by the contracting agency for retirement is currently 7 to 17.5 percent. Fringe benefits also include 11 paid holidays, paid vacation time as follows: Two (2) hours of annual leave each week for an employee with less than three (3) years of service. Three (3) hours of annual leave each week for an employee with three (3) but less than fifteen (15) years of service. Four (4) hours of annual leave each week for an employee with fifteen (15) or more years of service. |  |

## 1452.201-70 Authorities and Delegations (SEP 2011)

(a) The Contracting Officer is the only individual authorized to enter into or terminate this contract, modify any term or condition of this contract, waive any requirement of this contract, or accept nonconforming work.

(b) The Contracting Officer will designate a Contracting Officer’s Representative (COR) at time of award.  The COR will be responsible for technical monitoring of the contractor’s performance and deliveries.  The COR will be appointed in writing, and a copy of the appointment will be furnished to the Contractor.  Changes to this delegation will be made by written changes to the existing appointment or by issuance of a new appointment.  :

(c) The COR is not authorized to perform, formally or informally, any of the following actions:

(1) Promise, award, agree to award, or execute any contract, contract modification, or notice of intent that changes or may change this contract;

(2) Waive or agree to modification of the delivery schedule;

(3) Make any final decision on any contract matter subject to the Disputes Clause;

(4) Terminate, for any reason, the Contractor’s right to proceed; or

(5) Obligate in any way, the payment of money by the Government.

(d) The Contractor shall comply with the written or oral direction of the Contracting Officer or authorized representative(s) acting within the scope and authority of the appointment memorandum.  The Contractor need not proceed with direction that it considers to have been issued without proper authority.  The Contractor shall notify the Contracting Officer in writing, with as much detail as possible, when the COR has taken an action or has issued direction (written or oral) that the Contractor considers to exceed the COR’s appointment, within 3 days of the occurrence.  Unless otherwise provided in this contract, the Contractor assumes all costs, risks, liabilities, and consequences of performing any work it is directed to perform that falls within any of € categories defined in paragraph (c) prior to receipt of the Contracting Offeror’s response issued under paragraph (e) of this clause.

(e) The Contracting Officer shall respond in writing within 30 days to any notice made under paragraph (d) of this clause.  A failure of the parties to agree upon the nature of a direction, or upon the contract action to be taken with respect thereto, shall be subject to the provisions of the Disputes clause of this contract.

(f) The Contractor shall provide copies of all correspondence to the Contracting Officer and the COR.

(g) Any action(s) taken by the Contractor, in response to any direction given by any person acting on behalf of the Government or any Government official other than the Contracting Officer or the COR acting within his or her appointment, shall be at the Contractor’s risk.

(End of Clause)

## 1452.203-70 Restrictions on Endorsements (JUL 1996)

The Contractor shall not refer to contracts awarded by the Department of the Interior in commercial advertising, as defined in FAR 31.205–1, in a manner which states or implies that the product or service provided is approved or endorsed by the Government, or is considered by the Government to be superior to other products or services. This restriction is intended to avoid the appearance of preference by the Government toward any product or service. The Contractor may request the Contracting Officer to make a determination as to the propriety of promotional material.

(End of clause)

## 1452.228-71 Aircraft and General Public Liability Insurance (Mar 1989)

(a) The Contractor, at the Contractor's expense, agrees to maintain, during the continuance of this contract, aircraft liability and general public liability insurance with limits of liability for:

(1) Bodily injury to or death of aircraft passengers of not less than $75,000 for any one passenger and a limit for each occurrence in any one aircraft of at least an amount equal to the sum produced by multiplying $75,000 by 75 percent of the total number of passenger seats installed in the aircraft;

(2) Bodily injury to or death of persons (excluding passengers) of not less than $75,000 for any one person in any one occurrence and $300,000 for occurrence; and

(3) Property damage of not less than $100,000 for each occurrence; or

(4) a single limit of liability for each occurrence equal to or greater than the combined required minimums set forth in paragraphs (a)(1) through (3) of this clause.

(b) The Contractor also agrees to maintain worker's compensation and other legally required insurance with respect to the Contractor's own employees and agents.

(End of clause)

## 52.212-4 Contract Terms and Conditions – Commercial Items-Addendum

### 52.212-4(a) Inspection/Acceptance-The following is added:

AOM Technical Inspector

The AOM Technical Inspector is authorized to take any or all actions necessary to assure compliance with the technical portions of the contract. The AOM Technical Inspector will conduct all requested or required inspections.

The AOM Technical Inspector for this contract is:

DOI – Office of Aviation Services (AOM)

300 E. Mallard Dr., Ste 180

Boise, ID 83706-3991

AOMProject Inspector (PI).

If necessary, due to distance or geographic dispersion of sites, the COR may request in writing to the CO, a Project Inspector to monitor the contract in their absence. The CO will appoint the PI in writing with copies to the contractor and the COR. The PIs will not be delegated COR authority and must immediately bring any potentially controversial matter to the COR for action. The COR will remain the delegated Government representative directly responsible to the CO.

A qualified Manager will be assigned to each base of operations and may be designated as a Project Inspector. In addition to directing work of the Air attack the manager has the following contract administration duties and authority:

Conduct pre-use inspection. *

Order aircraft services as provided in the contract.

Secure compliance with all contract provisions and specifications.

Record and agree to availability and flight times.

Approve authorized breaks.

Suspend Operations.

Complete Contractor evaluation at end of assignment.

*The pre-use inspection is required each time the aircraft is hired for use. If the pre-use inspection reveals equipment problems the manager should call the appropriate AM Area office and consult with an AM technical specialist.

The AOM Aviation Safety Manager.

The AOM Aviation Safety Manager (ASM) is responsible for all matters concerning accident and incident with potential investigations. The ASM is:

AOMUSWFS Aviation Operations and Management (AOM) ASM

300 E. Mallard Dr., Ste. 200

Boise, ID 83706-3991

Phone:	208-433-5071

BLM Aviation Safety Advisor

BLM Aviation Safety Advisor may at any time conduct assurance reviews with company employees performing on the contract. Reviews will be based on Safety Management System (SMS) manual as required in contract. (See C7).

Inspection Scheduling and Process. After either contract award or renewal, the AOM Technical Inspector will schedule an initial inspection of all of the Contractor’s proposed aircraft, equipment and personnel to ensure contract compliance. This inspection is expected to be accomplished when the AOM Technical Inspector’s inspectors’ normal schedule brings them to the Contractor’s vicinity. Contractors who have not been inspected but are requested for use should immediately contact the AOM Technical Inspector to schedule an inspection. Failure to contact the AOM Technical Inspector may result in the use of a different Contractor. The inspection will be conducted at the Contractor's facility or other location acceptable to the Government at a mutually agreeable time. The inspection time and date will be scheduled for between 0730 and 1630 local time, Monday through Friday, unless otherwise agreed upon by the AOM Technical Inspector. The AOM Technical Inspector will normally confirm the inspection details in writing. Contractor written requests for inspection rescheduling that are received by the AOM Technical Inspector at least 10 calendar days prior to the originally scheduled inspection date may be accommodated by the AOM Technical Inspector, depending upon their work schedule.

The Contractor must provide information specific to the aircraft, equipment, and personnel being proposed for use during each year of the contract when requested by the AOM Technical Inspector.

The Contractor must notify the CO, the COR, and the AOM Technical Inspector when an action has been imposed by the FAA on the operator’s certificate or on any pilot or aircraft carded under this contract. The Contractor must also notify the AOM Technical Inspector of any changes in the Director of Operations, Chief Pilot, and Director of Maintenance as well as any additional positions approved under 14 CFR Part 119.69(b).

Approved aircraft, and pilots will be issued an Interagency Aircraft Data Card, an Interagency Data Card and Interagency Pilot Qualification card, as applicable. The aircraft and pilot cards detail the activities for which they are authorized. Interagency Data/Qualification cards remain the property of the government and may be suspended or revoked in response to an unsafe act, incident or incident with potential. Interagency Qualification cards are automatically suspended following any accident. Contractors must report mishaps (see C-8) involving equipment and/or personnel in possession of interagency data/qualification card(s) regardless of the contract or oversight agency in place at the time of the event.

Contractor must submit the Aircraft Data card, Interagency Pilot card for inspection upon request of a government official.

The aircraft data card will be kept in the aircraft and available for inspection at all times.

The pilot qualification card will be kept in the possession of the pilot and available for inspection at all times.

If the AOM Technical Inspector determines any aircraft, equipment, personnel, records, or documents presented for inspection are not completely ready for the inspection or are determined to be nonconforming as required by the contract, the AOM Technical Inspector may suspend the inspection(s) and schedule a re-inspection for another time/date/site. The Contractor may be charged for the cost of re-inspection, in accordance with the below.

Failure to have an originally offered aircraft presented for inspection within 60 calendar days after notice for an inspection may result in removal of the aircraft from the contract.

When an aircraft has not flown under a DOI issued TO within a 12 consecutive month timeframe, the card may be revoked and aircraft removed from the contract.

Equipment.

The aircraft will be inspected to ensure compliance with all contract requirements. The Government may require in-flight dynamic testing of aircraft systems. This testing may be conducted in conjunction with pilot evaluation flight(s) and will be performed at no cost to the Government.

Fuel cache(s) and other equipment will be inspected to ensure contract compliance.

Personnel.

Pilots. Only those individuals whose past flight time and experience can be verified from log books, employment records, etc., will be approved for contract use. The Contractor cannot substitute any pilot flight evaluation time for any of the total pilot flight hour requirements listed in this contract.

The AOM Technical Inspector’s representative may conduct a pilot flight evaluation to further verify each pilot’s ability to perform under this contract, when determined necessary. The evaluation may include but is not limited to: weight and balance performance, center of gravity limitations, aircraft performance charts, density altitude considerations, load calculation preparation and actual flying of the aircraft.  Portions of the evaluation may be evaluated orally. The flight evaluation will be conducted in accordance with the Interagency Mission Certification Standards and the FAA Airman Certification Standards (ACS). A pilot must also be capable of demonstrating proficient operation of all aircraft equipment identified in Section B during an evaluation flight.

The aircraft used for the evaluations must be the same make, model, and series awarded for this contract and equipped with dual controls. Flight evaluations will usually be performed in areas that provide access to terrain similar to that to be flown during the contract period. Flight evaluations are conducted at the Contractor’s expense.

During the flight evaluation, pilot inspectors retain discretionary authority in determining the competency of the pilot. The Government will make the final determination as to the pilot’s ability to successfully meet contract requirements.

Services provided under this contract require DOI special use flight activities as identified herein. Pilots must have satisfactorily completed an initial agency and/or periodic flight evaluations for these activities before being approved for use under the contract, unless otherwise indicated in the contract. The AOM Technical Inspector will provide detailed information concerning the types and frequency of special use pilot flight evaluations when requested.

List of Special Use Flight Activities:

See Operational Procedure Manual (OPM) OPM-29 for complete list of special use activities and requirements. https://www.doi.gov/aviation/library/opm

Special Use Activities required under this contract:

Reconnaissance

Mountainous Terrain

Air Attack Pilot (ATGS)

Re-inspection Expenses.

The Government may charge the Contractor for the cost of any substitute inspections in accordance with the following section (Re-inspection Expenses).

The bureau may require substitute pilots to obtain up to three hours each of training or orientation flight time at Contractor’s expense. (Note: This flight time is in addition to any necessary pilot evaluation flight(s).)

Reinspection expenses may include but are not limited to: inspector’s time at $100.00 per hour (including travel time); transportation and subsistence expenses at actual cost; and any applicable indirect cost recovery rates.

52.212-4(k) Taxes

Important Notice:

In accordance with 52.212-4(k), the price(s) in the schedule within Section A of the contract include all applicable Federal, State, and local taxes and duties. The Government’s electronic business systems will not calculate nor pay for any federal, state, local taxes, or duties separately under the contract. Examples of some but not all taxes and duties that are considered included in the contract prices are:

Federal Airport and Airway Excise Taxes

Fuel Taxes

Transportation Taxes (passengers and cargo)

### FAR 52.212-4(q) Other Compliances (Only Applies to Alaska)

Operations in Alaska will be scheduled by the Government and paid by the Government in accordance with the contract Availability Requirements and Measurement and Payment Requirements contained in the contract. Additionally, the following will apply.

Other compliances. The Contractor shall comply with all applicable Federal, State and local laws, executive orders and rules and regulations applicable to its performance under this contract.

It is the Contractor’s responsibility to comply with the above, even though this contract may not address every individual item that the Contractor may encounter during performance. As a minimum the Contractor should carry and maintain aircraft insurance for operations in Alaska as would be required and to obtain visas for employees as would be applicable if required to enter Canada.

### Contractor Responsibilities - Conduct of Business on a Military Installation. (Alaska Only)

Performance under this contract involves basing aircraft, support equipment and personnel on military installations. The Department of the Interior, Bureau of Land Management, and Alaska Fire Service has Support Agreements covering the use of the grounds and facilities. The Contractor agrees to cooperate in the adherence to the terms of said agreements as a condition of performing under this contract.

Rules of Conduct and Regulations

The Contractor and its employees are expected to adhere to the rules of conduct and regulations prescribed by the military installation Commander applicable to civilians entering or doing business with the Government on military installations. The contractor and its employees shall be required to maintain automobile insurance on company and personal owned vehicles that are used on the military installation.

The minimum vehicle insurance levels are those prescribed by the State of Alaska. A certificate of insurance is required for entry to Ft. Wainwright. Vehicle operators shall be prepared to show proof of insurance upon request of the military or BLM personnel.

Contractor shall submit the vehicle identification number (VIN) for all restricted Bureau of Land Management retardant ramp site vehicles to the Contracting Officer 10 days prior to award or when such vehicles are presented to the site. The Government will reserve the right to require insurance on the restricted ramp site vehicles.

The Government will issue Fort Wainwright base vehicle passes. Passes are available at the Ft. Wainwright front gate.

Department Of War Approval to Utilize Military Airfields Within the State Of Alaska.

Aircraft Landing Permits

Performance under this contract requires that the Contractor use military airfields within the State of Alaska as either reporting and/or operating base. As a condition of this use, the contractor must comply with the following requirements imposed by the Department of War. The following forms must be completed and submitted to the Contracting Officer:

Civil Aircraft Landing Permit, DD Form 2401

Civil Aircraft Certificate of Insurance, DD Form 2400

Civil Aircraft Hold Harmless Agreement, DD Form 2402

Civil Aircraft Landing Permit, DD Form 2401, and Civil Aircraft Hold Harmless Agreement, DD Form 2402. The contractor must submit these forms within ten calendar days after receipt of contract award, to the COR.

Civil Aircraft Certificate of Insurance, DD Form 2400. Contractor shall be required to submit a DD Form 2400, Civil Aircraft Certificate of Insurance within ten calendar days after receipt of contract award or the award.

Forms can be found at http://www.usaasa.tradoc.army.mil/ob_CivilAirfieldLandingPermits.asp

### Government Identification Cards - Contractor Employees (Alaska Only)

Contractor employees, who are assigned to operate in and out of Ft. Wainwright, Alaska may be issued an U.S. Government Identification Card. The Bureau of Land Management, Alaska Fire Service, will issue this card. The card will be clearly marked as "Contractor Employee" and include the name of the contractor they are employed by. This Identification Card is the property of the US Government.

Identification cards shall be returned to the COR upon request at any time. Cards shall also be returned to the COR upon the employee's release either at the end of each EU period or upon permanent dispatch to an alternate base.

The Government may withhold final payment to the contractor until such time as all cards have been turned in.

Contractor Employee Background Investigation. Contractor employees, who are assigned to operate in and out of Ft. Wainwright, Alaska may be subject to a background investigation by the Government. This background investigation shall be at the expense of the Government. At the request of the Contracting Officer, the Contractor shall submit information on each employee to facilitate this investigation. Failure to provide such information or upon receipt of an unsatisfactory background check, the employee shall be denied access to Ft. Wainwright or other Federal Installations. The contractor agrees to replace employees who refuse to provide information, or those who, in the Government's opinion result in an unsatisfactory background check.

Weapons.

All weapons in the aircraft survival kit shall be registered with the Ft Wainwright Provost Marshal.

Use of Support/staging and Storage Space at Ft. Wainwright, Alaska.

The Government will assign the Contractor a limited amount of space on or adjacent to the aircraft/fire suppressant material ramp for supporting its aircraft. The space is limited and will be apportioned (by the COR) based upon the number of aircraft furnished by the Contractor, as well as the total space available for this purpose. Only serviceable spare parts and support equipment will be permitted to be stored in this area. The Contractor will be required to keep their designated area clean and orderly. All items must be properly stored and/or disposed. The use of this space is limited to the direct support of the contract aircraft. No other use is permitted.

The Contractor shall be required to comply with all State, Federal and local Environmental Protection (EPA) laws and regulations as well as those prescribed by the military installation Commander in the handling, storage, transportation, utilization and disposal of hazardous materials and waste such as oil solvents, etc. At the time of space assignment, the Contractor shall designate an individual responsible for hazardous waste management.

Occupancy of the space shall be limited to a period NTE 5 calendar days prior to and after the EU period stated in the schedule or as established in the Notice to Proceed. Storage of a limited number of items outside this time period (i.e., winter period) shall only be permitted with the written permission of the COR. At the end of the contract term, all Contractor equipment, supplies, automobiles, and aircraft must be removed within 5 calendar days after the end of the EU period.

All usage of the assigned area is subject to the approval of the COR.

The Government assumes no responsibility/liability for loss of or damage to the Contractor's equipment stored at the site.

Contract Retainage.

The Government reserves the right to withhold interim or final payments under this contract pending compliance with the provisions contained in this section.

Additional Aircraft Insurance. The Contractor must have aircraft insurance coverage for operating in Alaska.

### Flight Time

Ferry flights through Canada

Northbound flight time thru Canada shall be paid at the wet flight rate until the first enroute stop is required for fuel. The Contractor shall close out the AOM-23 Flight Use report at this time and enter fuel on board. (Contractor will be provided a credit for fuel on board, using the price per gallon contained in the contract) The Contractor shall begin a new AOM-23 Flight Use Report upon resuming flight through Canada. This flight time shall be at the dry flight rate specified in the schedule of items. The Contractor shall be reimbursed for fuel acquired upon submission of an itemized fuel receipt that identifies the purchase location, type of fuel purchased, gallons purchased, price per gallon, and total cost. Conversion of Canadian currency to US currency shall be made at time of payment.

Southbound flight time thru Canada shall be paid at the dry flight rate until the first enroute stop is required in the contiguous United States. The Contractor shall close out the invoice at this time and enter total remaining Government fuel on board. (A deduction will be made for the remaining Government reimbursed fuel at this time) The Contractor shall begin a new invoice line upon resuming flight thru the contiguous United States. The Contractor shall be paid the wet flight rate from this point until such time as they are released from service.

The cost of fuel purchased and provided by the Contractor in lieu of Government-furnished fuel while operating in Alaska will be reimbursed to the Contractor as provided below:

The Contractor shall not charge any fuel acquired under this contract directly to the Government. All fuel not otherwise furnished by the Government must be purchased by or charged to the Contractor. The purchase must be approved by the Contracting Officer. Fuel related costs shall be recorded as a line entry (i.e., date, fuel charge, dollar amount, and use-item code fuel charge [FC]), shall be used and shall be supported by paid legible, itemized invoices from the supplier. Certified true copies may be submitted in lieu of the original invoice.

Government-furnished fuel utilized by the Contractor in support of maintenance flights, repositioning aircraft, crew transportation, or any other flight for the convenience of the Contractor, will result in a deduction from the Contractor using the Contractor’s proposed wet flight rate specified in Section A times the flight time for the Contractor’s benefit.

The Government will furnish, transport and store all aircraft fuel required at no expense to the Contractor. A fuel servicing vehicle and driver are not required.

(End Addendum)

# GENERAL CONTRACT TERMS AND CONDITIONS

## C2 Aircraft Use Reports

C2.1 The Contractor, or Contractor’s representative, and the Government must complete and sign an Aircraft Use Report (AUR), AQD-23/23E form or other form as directed by the CO. An electronic report will be initiated by the Contractor in a Department of the Interior electronic reporting system that documents the daily services recorded on the signed AQD-23/23E or other form as directed by the CO. Hard copies of the signed AQD-23/23E are to be uploaded/attached to the electronic system.

C2.2 Supporting documentation as required by the contract to support actual additional pay items (i.e. relief transportation costs, tie-downs, landing fees, etc.) shall be attached electronically to the applicable AUR. Failure to include such documentation would result in rejection of the report back to the Contractor for inclusion and resubmission.

C2.3 AURs are to be submitted no sooner than every two weeks or upon conclusion of a project, if less than two weeks duration.

C2.4 Subsequent electronic invoicing through IPP will match the same period as the AUR submission or other form as directed by the CO.

## C3 Substitute Personnel, Aircraft, or Equipment.

The Contractor may request the use of substitute personnel, aircraft, or equipment that was not initially approved for use. All proposed substitutes must meet pertinent contract specifications and be subject to inspections and approvals identified herein prior to use. The Contractor must submit a written request for inspections of pilot substitutes to the AOM Technical Inspector seven days prior to the scheduled arrival at the site. Requests for aircraft substitution must be submitted to the CO for approval prior to inspection. The CO may issue a bilateral modification prior to submitting a request to the AOM Technical Inspector for inspection scheduling. Requests received with fewer than seven days’ notice will be accomplished as permitted by the AOM Technical Inspector’s schedule.

Any request for aircraft substitutions must include:

Completed Aircraft Questionnaire

Completed Aircraft Information Form

Airworthiness Certificate

Aircraft Registration

Current Weight and Balance (based on table in Section B32),

Equipment list certified accurate at the time of weighing,

Any calculations indicating changes to the aircraft and equipment list that may have occurred after the time of weighing,

Payload calculations showing performance characteristics of the offered aircraft,

Applicable performance charts from the aircraft flight manual

Copy of the 14 CFR Part 135 Operations Specifications (D085)

The Contractor must transport substitute personnel, aircraft, or equipment to the point of use at their expense.

C3.1 Safety and Accident Prevention

C3.1.1 The Contractor shall keep and maintain programs necessary to assure safety of ground and flight operations. The development and maintenance of these programs are a material part of the performance of the contract. Examples of such programs are (1) personnel activities, (2) maintenance, (3) safety, and (4) compliance with regulations.

C3.1.2 The Contractor must submit a copy of all reports required by the Federal Aviation Regulations that relate to pilot and maintenance personnel performance, aircraft airworthiness or operations to the Aviation Safety Manager (ASM).

C3.1.2.1 Examples of these reports are shown in paragraphs 14 CFR Part 135.415 Service Difficulty Reports (SDR) and Part 135.417 Mechanical Interruption Summary Reports required of the Federal Aviation Regulations, 49 CFR Part 830.5 and 49 CFR 830.15, and FAA Form 8010-4, Malfunction or Defect Report.

## C4 Aircraft Insurance

The Contractor must maintain as a minimum, aircraft insurance coverage required by 14 CFR, Part 205, during contract performance in accordance with DIAR 1452.228-71.

## C5 Post Award Evaluation of Contractor Performance Contractor Performance Assessment Reporting System (Dec 2015)

(1) FAR 42.1502 directs all Federal agencies to collect past performance information on contracts. DOI has implemented the Contractor Performance Assessment Reporting System (CPARS) to comply with this regulation. One or more past performance evaluations will be conducted in order to record your contract performance as required by FAR 42.15.

(2) The past performance evaluation process is a totally paperless process using CPARS. CPARS is a web-based system that allows for electronic processing of the performance evaluation report. Once approved it is viewable in CPARS and will used by the Government in evaluating past performance as part of a source selection action.

(3) We request that you furnish the CO with the name, position title, phone number, and email address for each person designated to have access to your firm's past performance evaluation(s) for the contract no later than 30 days after award. Each person granted access will have the ability to provide comments in the Contractor portion of the report and state whether or not the Contractor agrees with the evaluation, before returning the report to the Assessing Official (AO). Information in the report must be protected as source selection sensitive information not releasable to the public.

(4) When your Contractor Representative(s) are registered in CPARS, they will receive an automatically generated email with detailed login instructions. Further details, systems requirements, and training information for CPARS is available at https://www.cpars.gov/.

(5) Within 60 days after the end of a performance period, the AO will complete an interim or final past performance evaluation, and the report will be accessible at https://www.cpars.gov/.

(i) Contractor Representatives may then provide comments in response to the evaluation, or return the evaluation without comment.

(ii) Your comments should focus on objective facts in the AO's narrative and should provide your views on the causes and ramifications of the assessed performance.

(iii)All information provided should be reviewed for accuracy prior to submission.

(iv) If you elect not to provide comments, please acknowledge receipt of the evaluation by indicating "No comment" in the space provided, and then selecting “Accept the Ratings and Close the Evaluation”.

(v)Your response is due within 60 calendar days after receipt of the CPAR. On day 15, the evaluation will become available in PPIRS-RC marked as “Pending” with or without comments and whether or not it has been closed.

(vi) If you do not sign and submit the CPAR within 60 days, it will automatically be returned to the Government and will be annotated: "The report was delivered/received by the contractor on (date). The contractor neither signed nor offered comment in response to this assessment."

(6) The following guidelines apply concerning your use of the past performance evaluation:

(i) Protect the evaluation as source selection information. After review, transmit the evaluation by completing and submitting the form through CPARS. If for some reason you are unable to view and/or submit the form through CPARS, contact the CO for instructions.

(ii) Strictly control access to the evaluation within your organization. Ensure the evaluation is never released to persons or entities outside of your control.

(iii) Prohibit the use of or reference to evaluation data for advertising, promotional material, pre-award surveys, responsibility determinations, production readiness reviews, or other similar purposes.

(7) If you wish to discuss a past performance evaluation, you should request a meeting in writing to the CO no later than seven days following your receipt of the evaluation. The meeting will be held in person or via telephone or other means during your 60-day review period.

(8) A copy of the completed past performance evaluation will be available in CPARS for your viewing and for Government use supporting source selection actions after it has been finalized.

## C6 Personnel Conduct

C6.1 Replacement of Contractor Personnel.

C6.1.1 Contractor employees required to work or reside on Federal property (National Parks, Refuges, Indian Reservations, etc.) are expected to follow the facility manager’s rules of conduct that apply to both Government or non-Government personnel working or residing at these facilities. The COR will make available a copy of such rules. The Contractor may be required to replace employees who do not comply with these rules of conduct.

C6.1.2 The Contractor must replace any employee who performs unsafely, ineffectively; refuses to cooperate; is unable or unwilling to adapt to field living conditions; or whose general performance is unsatisfactory, disruptive or detrimental to the purpose for which contracted.

C6.1.3 The CO will notify the Contractor of all known unsatisfactory personnel conduct or unsafe performance. The employee may be afforded an opportunity for corrective action when the conditions warrant. When directed by the CO, the Contractor must replace unacceptable personnel not later than 24 hours after such notification, or as otherwise mutually agreed. The decision as to unacceptability will be at the sole discretion of the CO.

## C7 Safety Management System (SMS)

C7.1 The Contractor must meet the FAA’s SMS requirements for Part 135 or air tour operators operating under § 91.147 no later than May 28, 2027 (FAA Final Rule, May 2024).

C7.2 The Contractor must have and maintain a Safety Management System (SMS) Manual for its organization

C7.2.1 The Contractor must submit within 30 days after contract award an electronic copy of its SMS Plan to the CO and ASM. The Contractor’s SMS Manual should include implemented practices vice a generic ready-made Manual that was purchased and never implemented.

C7.1.2 The Contractor shall submit electronic updates and/or revisions of their SMS Manual to the CO and ASM throughout the life of this contract within 30 days of the revision/update.

C7.1.3 Contractor employees working under this contract must be familiar with the Offeror’s SMS Manual and be in compliance with its requirements.

C7.1.4 The government may at any time conduct reviews with the Contractor’s employees performing on this contract to determine their familiarity with the Contractor’s SMS Manual. The Contractor’s adherence to their SMS and the standards within Exhibit 4 may be reviewed during an on-site assurance review and documented in CPARS.

C7.1.5 Additional information on SMS can be found at:

FAA AC 120-92D: https://www.faa.gov/regulations_policies/advisory_circulars/

FAA initiatives: https://www.faa.gov/about/initiatives/sms/

International Standard for Business Aircraft Operations (IS-BAO): https://nbaa.org/flight-department-administration/sms/

## C8 Mishaps

C8.1 Following a mishap, and based on an investigation by the Agency Investigator In Charge (IIC), Bureau representative, and the CO, the Government will determine whether the Contractor was in compliance with contract terms and conditions or with the Federal Aviation Regulations applicable to the Contractor's operations, Offeror policy, procedures, practices, or programs, or whether there was negligence on the part of the Offeror officers or employees that may have caused or contributed to the mishap. The Contractor must fully cooperate with the agency IIC, ASM, and CO during the evaluation.

C8.2 Mishap Definitions. As used throughout this contract, the following terms will have the meanings set forth below.

C8.2.1 The following terms are as defined in 49 CFR Part 830:

Aircraft Accident

Fatal Injury

Incident

Operator

Serious Injury

Substantial Damage

C8.2.2 Airspace Conflict. A near mid-air collision, intrusion, or violation of airspace rules.

C8.2.3 Aviation Hazard. Any condition, act, or set of circumstances that exposes an individual to unnecessary risk or harm during aviation operations.

C8.2.4 Incident with Potential. An incident that narrowly misses being an accident and in which the circumstances indicate significant potential for substantial damage or serious injury. Classification of an incident as an "Incident with Potential" is determined by the agency ASM.

C8.2.5 Maintenance Deficiency. An equipment defect or failure which affects, or could affect, the safety of operations or that causes an interruption to the services being performed.

C8.2.6 Mishap – Aviation Mishap. Mishaps include aircraft accidents, incidents with potential, aircraft incidents, aviation hazards, and aircraft maintenance deficiencies.

C8.2.7 SAFECOM (https://www.safecom.gov/). An agency Aviation Safety Communique used to report any condition, observance, act, maintenance problem, or circumstance, which has the potential to cause an aviation-related mishap. A SAFECOM’s sole purpose is for mishap prevention. (Form OAS-34 or FS 5700-14).

C8.3 Mishap Reporting.

C8.3.1 For “Aircraft Accident” or National Transportation Safety Board (NTSB) reportable “Incident”, the Contractor must immediately, and by the most expeditious means available, notify the NTSB, CO, and AOM ASM.

C8.3.2 For any mishap involving damage or injury, or overdue aircraft suspected of having an accident, the Contractor must immediately, and by the most expeditious means available, notify the AOM by calling 1-888-4MISHAP.

C8.3.3 In an effort to prevent future mishaps, the Contractor must report aviation hazards and maintenance deficiencies.

C8.3.4 The toll-free 24-hour Aircraft Accident Reporting Hot Line number is: 1-888-4MISHAP (1-888-464-7427)

C8.4 Mishap Investigations. It is the Department of the Interior’s responsibility to investigate Interior aircraft mishaps using one of the following investigation procedures.

C8.4.1 On-site investigations will be conducted whenever possible for all aircraft accidents and selected incidents with potential.

C8.4.2 Limited investigations will be conducted for selected incidents with potential. A limited investigation will not normally include a visit to the incident site.

C8.4.3 Administrative investigations will be conducted for reports of conditions, observances, acts, maintenance problems, or circumstances, which may have the potential to cause an aircraft mishap.

C8.4.4 The Contractor must maintain an accurate record of all aircraft accidents, incidents, aviation hazards, and injuries to Contractor or Government personnel arising during this contract.

C8.4.5 Following a mishap, the Contractor must ensure that pilots, mechanics or other personnel associated with the aircraft remain in the vicinity of the mishap until released by the CO or their designated representative. The Contractor must cooperate with the agency during any investigation and make available personnel and aircraft records, and any equipment, damaged or undamaged, that the agency deems necessary.

C8.5 Forms Submission.

C8.5.1 Following an "Aircraft Accident" or when requested by the NTSB following notification of a reportable "Incident," the Contractor must provide the AOM ASM with information necessary to complete a NTSB Form 6120.1 “Pilot/Operator Aircraft Accident Report”.

C8.5.2 The Contractor must submit a SAFECOM within 5 days upon the occurrence of any condition, observance, act, maintenance problem, or circumstance which has potential to cause an aviation-related mishap. The SAFECOM submission instructions are available at: https://www.safecom.gov.

The submission of an NTSB Form 6120.1 does not replace the Contractor's responsibility to submit a SAFECOM.

C8.6 Pilot Suspension. See Suspension of Pilot in C9.

C8.7 Preservation Requirements. The Contractor must not permit removal or alteration of the aircraft, aircraft equipment, or records following an Aircraft Accident, Incident, or Incident with Potential until authorized to do so by the CO or other authorized agency representative. Permitted exceptions to this requirement may be when life or property are threatened, when the aircraft is blocking an airport runway, etc. The Contractor must immediately notify the AOM ASM, NTSB and the CO before taking such actions.

C8.7.1 The NTSB's release of the wreckage does not constitute a release by the CO and IIC.

C8.8 Costs Related to Investigation. The NTSB or agency will determine their individual agency’s investigation cost responsibility. The Contractor will be fully responsible for any cost associated with the reassembly, approval for return-to-service, and return transportation of any items disassembled by the Government.

C8.9 Rescue and Salvage Responsibilities. The Contractor must be responsible for the cost of search, rescue, and salvage operations made necessary due to causes other than negligent acts of a Government employee.

## C9 Suspension of Pilot

C9.1 Upon receipt of any information that indicates a safety concern or notification of a reportable incident as defined within 49 CFR 830.5, the Government (carding authority) may suspend the pilot from their duties and from any other activity authorized under the Interagency Pilot Qualification card(s), dependent upon any findings during the course of the investigation.

C9.1.1 Upon involvement in an Aircraft Accident, a pilot will be suspended from pilot duties and from any other activity authorized under the Interagency Pilot Qualification card(s). Their return to service is dependent upon any findings during the course of the investigation.

C9.1.2 Upon involvement in an Incident with Potential as defined under Mishaps, a pilot may be suspended from pilot duties and from any other activity authorized under the Interagency Pilot Qualification card(s), dependent upon any findings during the course of the investigation.

C9.1.3 When requested, a suspended pilot must surrender all Interagency Pilot Qualification card(s) to the AOM Technical Inspector or other authorized agency representative. A pilot’s suspension will continue until the carding authority determines that no further suspension is required. The Interagency Pilot Qualification card(s) is returned to the pilot; or revoked by the issuing agency if the investigation fails to support a pilot’s return to service.

## C10  Electronic Invoicing and Payment Requirements - Invoice Processing Platform (IPP) (February 2021)

Payment requests must be submitted electronically through the U. S. Department of the Treasury's Invoice Processing Platform System (IPP).

"Payment request" means any request for contract financing payment or invoice payment by the Contractor. To constitute a proper invoice, the payment request must comply with the requirements identified in the applicable Prompt Payment clause included in the contract, or the clause 52.212-4 Contract Terms and Conditions - Commercial Items included in commercial item contracts. The IPP website address is: https://www.ipp.gov.

Under this contract, the following documents are required to be submitted as an attachment to the IPP invoice:

- Documents required are Aircraft Use Reports (AQD Form 23/23E) or other form as directed by the CO documenting daily services provided as set forth by their contract. This form must have the appropriate Government Representative signature approving the services.

- Supporting documentation as required by the contract to support actual additional pay items (i.e. relief transportation costs, tie-downs, landing fees, etc.). See C1.2.

The Contractor must use the IPP website to register access and use IPP for submitting requests for payment. The Contractor Government Business Point of Contact (as listed in SAM) will receive enrollment instructions via email from the Federal Reserve Bank of St. Louis (FRBSTL) within 3 - 5 business days of the contract award date. Contractor assistance with enrollment can be obtained by contacting the IPP Production Helpdesk via email IPPCustomerSupport@fiscal.treasury.gov or phone (866) 973-3131.

If the Contractor is unable to comply with the requirement to use IPP for submitting invoices for payment, the Contractor must submit a waiver request in writing to the Contracting Officer with its proposal or quotation.

(End of Local Clause)

## C11 Contractor Personnel Security Requirements

C11.1 It has been determined that Contractor personnel utilized in the support of this contract will not be allowed routine and regular unsupervised access to a federally controlled facility for more than 180 days, nor will they need unsupervised access to a Federally controlled Level 3 or 4 information system.

C11.2 Contractor employees utilized in support of this contract, will be treated as visitors (uncredentialed Contractor) and not be required to receive background investigations and credentialing. However, uncredentialed Contractors may be subject to the screening processes utilized at each federally controlled facility where the Contractor services are required. As a minimum, Contractor employees will be issued a temporary/visitor badge and shall display it at all times during contract performance when accessing a federally controlled facility. The COR is responsible for ensuring that all Contractor employees are issued a temporary/visitor badge.

## C12 Prework Meeting

A prework meeting between the Government and the Contractor along with their primary crew members may be held at or near the starting designated base and is usually in conjunction with the start of the EU period. The Contractor’s primary crew members must attend any prework meeting. The meeting may include, but is not limited to: (1) review of the contract in detail; (2) operational procedures (dispatch, flight following, hazard/risk assessment and reduction, airspace coordination, incident/accident reporting, etc.; and (3) review of the local base procedures.

## C13 Economic Price Adjustment (Does not apply to Alaska)

The Contractor may request in writing an hourly flight rate adjustment as set forth herein to reflect increases and decreases in the cost of commercial aviation fuel. This request may be made annually, but no more than 30 days after the start of a new ordering year. The Contractor warrants that the prices offered for this contract do not include any allowances for any contingency to cover increased costs for which adjustment is provided under this clause.

Base Price. The base price is the commercial price obtained by the Government for the specific fuel type at the specified fuel location as identified in Section A, Requirements and Prices, of the contract. The fuel source is normally at or near the designated base specified in the solicitation. The base price may also be an average price of more than one fuel source location if the solicitation is for on-call flight services for more than one designated base.

Reference Price. The reference price is the commercial fuel price at the Fuel Source Location cited in Section A of the contract in effect at the time of adjustment. No other locations will be considered when making adjustments. The CO will establish an updated base price in a modification to the contract once the new flight rates are established.

Flight Rate Adjustment. Adjustment to the hourly flight rate is the difference between the reference price and the base price multiplied by the hourly fuel consumption rate for the type of aircraft involved as shown in the Helicopter/Fixed-Wing Fuel Consumption and Weight Reduction Chart Exhibit titled fuel consumption rates. Amounts 50 cents or less will be rounded down and amounts 51 cents or more will be rounded up.

The hourly flight rate will be adjusted upward by the CO in a contract modification, whenever the CO confirms the contractor's Reference Price is more than 10 percent higher than the Base Price at the Fuel Source Location identified in the Fuel Adjustment Table in Section A.

The hourly flight rate will be adjusted downward by a contract modification, whenever the CO confirms that the contract reference price is more than 10 percent lower than the base price at the fuel source location identified in the Fuel Adjustment Table of the contract.

Fuel price adjustments are subject to review by the CO at any time during the contract period. However, the CO will review the fuel prices at least annually. The revised base price will remain in effect for the duration of the contract. The flight rate adjustment will be issued through a modification to the contract and will be effective as of the effective date of the modification.

The effective date of the flight rate adjustment will be stated via a modification and will be effective as of the date of the modification.

## C14 Fair Opportunity.

Except as provided in C14.1 below, fair opportunity will be provided for task orders in accordance with FAR 16.505(b)(1), When required by FAR 16.505(b)(1), contractors will be provided an opportunity to submit an offer through a Task Order Request for Proposal (TORP). Contractors’ pricing for task orders shall not exceed the prices in the IDIQ price schedule. Discounted pricing is permitted. Each TORP will provide applicable evaluation criteria. As required by FAR 16.505(b), all TORPs will include price as an evaluation factor. Award will be made to the offeror whose submission offers the best value to the Government, price and other factors considered. Award may be made to other than the lowest-priced offeror. The relative importance of non-price factors will be specified in the TORP. Offerors shall not submit rates in response to a TORP which exceed the ceiling rates established in A2 above.

C14.1 Due to the nature of firefighting, urgent orders are likely under this IDIQ. Pursuant to FAR 16.505(b)(2), fair opportunity need not be provided for urgent orders. In such cases, the ordering activity will select the contractor it deems to offer the best value to the Government. Depending on the circumstances, the best value may be determined based on location, price, and/or other considerations at the ordering activities discretion. Because urgent orders may be issued under the IDIQ without the opportunity to submit a task order proposal with revised pricing, offerors are encouraged to include their best pricing in their IDIQ price proposals.

C14.2 Task Orders. Task Orders will be placed by the CO for all services under this contract. The CO will issue a minimum of one task order numbers to the contractor for Urgent/On-Call Services. All costs (availability, flight hours, additional pay items, etc.) will be recorded using task orders. The minimum guarantee will be satisfied via issuance of a separately-priced task order against each contract award at contract award.

C14.3 The contractor is responsible for using the appropriate task order relative to the Resource Order received at the time the aircraft is hired until it is released off contract. If a contractor is reassigned and not released, they shall continue to use the same task order until such time they are released off contract. The task order number shall be used on all AQD-23 Reports and when submitting invoice data into the electronic payment systems (IPP, etc.). See Paragraph CXX for Ordering Offices and Resource Order information.

C14.4 The Government will not consider any contract aircraft to be under its operational control when the Contractor is not available or capable of providing Government scheduled services.

C14.5 Authorized Ordering Official.  Orders for Urgent/On-Call services may be placed only by a government employee within the authorized organizational components identified below, in addition to the CO. Once a contractor agrees to accept an order, they will receive a Resource Order from the ordering office in an email and/or fax. Orders may be received from any of the following:

- Local Dispatch Office (hire Daily rate only)

- Geographic Area Coordination Center (GACC) (hire Daily rate only)

- NICC located at Boise, Idaho. (hire Daily rate only)

- Contracting Officer (hire Daily and issue TORPs)

## C15 Exclusive Use Period

C15.1 The EU period will extend for up to 120 days (90 day minimum). The EU period per location will be specified on the awarded task order for each location and shall start on either of the following:

C15.1.1 The date stipulated in Section A above.

C15.1.2 A dated established by the Government not later than 20 days in advance of the intended start date which establishes a start date that commences not more than 14 days prior to nor 14 days after the start date stipulated in Section A.

C15.1.2.1 The Government may also mutually agree with the Contractor to establish a start date within less than 20 days of the start date.

C15.2 The Government will not consider any contract aircraft to be under its operational control when the Contractor is not available or capable of providing Government scheduled services.

C15.3 The CO must authorize by modification any use outside the EU period and any agreed upon extension.

## C16 Extension of Exclusive Use Period

The Government reserves the right to unilaterally extend the EU period at the same rates as in the base EU period . These extensions are estimated and will be on a day to day basis, meaning the vendor can be release at any time with no warning in the extension period.

# AVAILABILITY REQUIREMENTS

## C17 Availability Requirements

During the EU period and any extension, the Contractor must be in compliance with all contract requirements and available and capable of providing service up to 14 hours each day, as scheduled by the Government. Personnel must be available a minimum of nine hours each day, or as scheduled by the Government. Pre- and post-flight activities must be accomplished within the 14- hour duty day. Routine maintenance must be performed before or after the scheduled 14-hour period, or as permitted elsewhere in the contract.

C17.1 Extended standby is intended to provide the Contractor compensation for employee time when ordered services are provided in excess of the first 9 hours of service. Ordered standby must not exceed individual crew members' daily duty limitations. Extended standby is not intended to compensate the Contractor on a one-to-one basis for all hours necessary to service and maintain the aircraft.

## C18 Schedule of Operations and Reaction Time

The Government will schedule daily operations with the pilot. The Contractor's personnel must provide service, as directed by the Government, in one of the following categories:

C18.1 Standby. Personnel must be on standby each day as scheduled and must be ready for takeoff/dispatch within 15 minutes (or longer as authorized by the Government; e.g. flight planning purposes for long range dispatch) after the Government attempts to contact the Contractor's representative.

C18.2 Alert. After standby Contractor personnel may be authorized to leave the immediate vicinity of the work site, but remain in an on call status subject to call back. When authorized to leave, they must maintain communications acceptable to the Government and must be ready for takeoff/dispatch within 60 minutes (or longer, if authorized by the Government) after the Government attempts to contact the Contractor's representative. Failure to return to service as required will result in loss of availability status and extended standby, as applicable.

C18.3 Release From Duty. Contractor personnel may be released and considered to be off duty prior to lapse of their individual crew duty limitation period. Once released, they cannot be required to return to duty status that day and service will be recorded as fully available status, provided the COR/PI has approved in advance release of the Contractor's personnel.

## C19 Maintenance During Availability Period

C19.1 The COR or PI may approve Contractor requests to remove the aircraft from service to permit the Contractor to perform scheduled or unscheduled maintenance. The Government will continue to measure and pay for service availability throughout periods approved for maintenance. The COR/PI may require the Contractor to resume service within 60 minutes or any other agreed upon time period. Failure to do so would result in unavailability status.

C19.2 If the aircraft is not scheduled for service or service is unavailable, the aircraft may be removed from the operating base for maintenance, provided the Contractor: (1) Obtains the schedule of operations from the COR/PI, (2) returns the aircraft to service before the beginning of the next availability period, AND (3) uses the aircraft for maintenance test flights, or flight to and from maintenance facilities only.

## C20 Unavailability and Damages

C20.1 The Contractor will be considered to be unavailable when they are not in compliance with all contract requirements or are not capable of providing service as scheduled by the Government. Unavailability status will continue until the Contractor has notified the COR or PI, that they are available and the COR is satisfied that all the conditions below have been met.

C20.1.2 The contractor may be required to demonstrate their availability by providing documented evidence to the COR and AOM Technical Inspector that the deficiency has been corrected. Evidence may be in the form of pictures and/or aircraft record/logbook entries documenting the corrective action, including the date, signature and certificate number of the person clearing the deficiency. Depending on the magnitude of the deficiency, the COR and/or AOM Technical Inspector may also require a physical inspection by an AOM inspector.

C20.1.3 Once the documented evidence is approved by the AOM Technical Inspector, the COR will consider the contractor available from the time the contractor notified the COR of their availability. If the AOM Technical Inspector requires additional actions from the contractor, the COR will consider the contractor available from the actual date that all deficiencies were corrected and approved by the AOM Technical Inspector.

C20.2 During periods of Contractor unavailability, the CO may obtain replacement services elsewhere and charge the Contractor for any resulting excess costs. The Contractor may be liable for any additional actual damages to the Government resulting from such failure to perform.

C20.3 If the contractor is unable to be in compliance due to conditions beyond their control (i.e. AFF subscription service inoperable, etc.) contractor may not be considered unavailable. The contractor needs to notify the COR or PI of the situation immediately.

# MEASUREMENT AND PAYMENT

## C21 Daily Availability

C21.1 Availability is measured in full days for the daily period of time (maximum of 14 hours) scheduled by the Government and provided by the Contractor. Payment for availability will be made as actual services are provided and paid at the rate and for the number of days set forth in Section A. The Government will measure unavailability in full hours and will round up periods of unavailability to the next whole hour. For each instance of unavailability, payment will be reduced by one hour for each hour, or portion thereof, in accordance with the Unavailability Conversion Chart Exhibit XX, when services are unavailable or when the aircraft has been released for the Contractor’s benefit.

C21.1.1 Contract Pricing. Unit prices for daily availability and flight hours must be in whole dollars. If these unit prices are adjusted during the life of the contract, they will be adjusted to a whole dollar as follows: amounts of 50 cents or less will be rounded down and amounts of 51 cents or more will be rounded up.

C21.2 Availability will be measured for payment on the crew’s mandatory days off.

C21.3 Extended Standby. The Government will measure extended standby in full hours and will round up to the next whole hour, not to exceed each crew member's duty limitations specified under Section B. The Government will pay for extended standby at the prices set forth in Section A, and as measured above. If unavailability occurs, extended standby will be measured and paid only for full hours of service provided.

## C22 Flight Time

C22.1 Measurement of Flight Time. Flight will be measured from the time the aircraft commences its take-off roll until it returns to the blocks. Elapsed time will be measured in hours and tenths/hundredths of hours.

C22.2 Payment for Flight Time. The Government will pay for all flights ordered by the CO and flown by the Contractor at the rates set forth in Section A. The Government does not guarantee any minimum or maximum number of flight hours under this contract.

C22.3 Flights Associated with Inspections. Flight time associated with the DOI, AOM (agency) inspection will be at the expense of the Contractor and will not be measured for payment.

C22.4 Flights for Contractor's Benefit. The Government will not pay for flights benefiting the Contractor, such as flights for maintenance testing for ferrying to and from maintenance facilities, flights required following an engine change, commercial charters, and flights solely for transporting Contractor's personnel.

## C23 Mobilization/Demobilization

C23.1 The Contractor is responsible for all mobilization and demobilization costs to and from the Designated/Reporting Base(s) stipulated in Section A. The Government may, at its discretion, release the Contractor from other than the Designated/Reporting Base stipulated in Section A. The contract price will be adjusted as described below whenever the distance from the point of release to the Contractor's home base is greater than the distance between the Contractor's home base and the Designated/Reporting Base stipulated in Section A. No adjustments will be made when the point of release is less than the distance between the Contractor's home base and the Designated/Reporting Base.

C23.2 Flight distance will be measured using the most direct route taken from low level in route aeronautical charts. The net distance will be converted into hours of flight using the most economical cruise speed of the aircraft. The adjustment will be determined by multiplying the difference in distance (hours of flight) by the flight rate stipulated in Section A.

## C24 Additional Pay Items (from Schedule of Items)

C24.1 Claims for additional pay items addressed herein must be documented on the invoice for payment and supported by invoice(s) and/or document(s), as required below. The Government will not pay claims submitted with incomplete or missing supporting documentation.

C24.2 Travel will be in accordance with the Federal Travel Regulations (FTR) currently in effect on the date of travel and FAR 31.205-46. In accordance with FAR 31.205-46, Cost for lodging, meals, and incidental expenses may be based on per diem, actual expenses, or on a combination thereof, provided the method used results in a reasonable cost. Therefore, the Government will utilize a combination of methods to determine reasonable cost. The per diem rates can be found at https://www.gsa.gov/portal/content/104877.

C24.2.1 Meals and incidentals will be reimbursed at the applicable maximum per diem rates per location. Lodging will be reimbursed up to the applicable maximum per diem rates per location. In locations, where costs may be escalated because of special or unusual events, and lodging within prescribed allowances cannot be obtained, the contractor is responsible for ensuring invoices submitted are accurate and complete, and all travel costs are in accordance with federal guidelines, the FTR and other Government mandates and directives. When known in advance, the Contractor should notify the Contracting Officer that lodging costs for their travel will exceed per diem. In the written notification, the travel dates, lodging location(s), and estimated lodging costs should be provided. When not known in advance, the Contractor should notify the Contracting Officer as soon as practicable. A signed written justification for the use of the higher rate shall be provided by the Contractor with the submission of the invoice. Receipts* ARE required for all lodging expenses.

*NOTE: Lodging receipts are NOT required when lodging is provided by the Government.

C24.2.2 If the Contractor does not use Government-provided meals and/or lodging, the Government will NOT reimburse the Contractor for costs incurred utilizing an alternate method of procuring meals or lodging accommodations. Mileage will NOT be paid for transportation to and from lodging accommodations or meals.

C24.2.3 In accordance with FAR 52.212-4, Contractors must make any records associated with travel in support of the services required under this contract available to the Government upon request.

C24.2.4 Per Diem (lodging, meals, and incidentals) is authorized when the crew is away from their designated base for overnight stays. Per Diem is authorized for the flight crews’ mandatory days off.

C24.3 Transportation Costs Associated with Operating Away From the Designated Base. When operating from an alternate base, the Contractor is required to provide for transporting relief personnel, unless otherwise directed by the Government. Prior to the exchange, the Contractor must advise the COR of the anticipated costs. The Contractor will be paid actual necessary and reasonable costs for transporting personnel and required equipment listed below:

Relief Crew members. The complement must be the same as required in Section A.

Maintenance personnel and equipment required to accomplish scheduled maintenance, i.e. 50 and 100 hour inspections.

C24.3.1 The Contractor must complete and submit the Transportation Worksheet Exhibit, attach supporting transportation invoices to the Transportation Worksheet, and enter the total dollar amount as a line entry on the invoice for payment (SC pay item code). Claims that do not include these items or other documents necessary to verify incurred costs will be returned to the Contractor for proper completion.

C24.3.2 Unless approved in advance by the CO, payment for crew member exchanges is limited to one round trip for one crew members once every 12 days. Additional payment may be appropriate for circumstances such as personnel reaching flight or duty time limits including agency imposed temporary flight or duty restrictions as specified in Section C17.

C25.5.5 The following are lists of generally authorized and unauthorized expenses. These lists are not all inclusive. Any questions on the acceptability or unacceptability of expenses can be directed to the CO prior to incurring the expense.

Authorized Expenses:

Flights (Economy Class ONLY)

Checked Bags (2)

Privately Owned Vehicle (POV) Mileage (see http://www.gsa.gov/portal/content/100715 for current rates)

Privately Owned Aircraft (POA) Mileage (see http://www.gsa.gov/portal/content/100715 for current rates)

Rental Cars (Mid-Size or smaller) where authorized

Rental Car Fuel

Rental Car Basic Insurance

Ride Share Services (Uber, Lyft, etc.)/Taxi Fare

Airport Parking when Ride Share Services/Taxi are not available.

Tie Down Fees

Hanger Fees (case by case basis)

Unauthorized Expenses:

Rental Car Upgrades with no explanation

Airline Seat Upgrades

Primary Pilot/Support Driver Rental Car

Prepaid Rental Car Fuel

Rental Car GPS

Rental Car Total Loss Insurance

Rental Car Satellite Radio

In-and-Around Miles at the duty location

If wanting to utilize a chartered aircraft (aircraft rented for hire), authorization may be given only by the CO in writing, prior to the Contractor incurring the expense. The expense or charter resources must not exceed the constructive cost of commercial travel to the fire base. The Contractor shall be required to provide documentation of the constructive cost of commercial transportation to the fire base with the approval request.

C24.3.4 Miscellaneous Contractor Costs. Miscellaneous unforeseeable costs that cannot be recovered through the contract payment rates and that are the direct result of ordered services away from the designated base may be paid at actual costs, when authorized in advance by the COR. Examples of such items are airport use costs (tie-downs) and truck permits at ports-of-entry, etc. The Contractor must support any cost exceeding $75.00 with an itemized, paid invoice.

C24.3.6 Landing Fees. The Government will pay the Contractor for all landing fees the Contractor is required to pay. The Contractor must support any cost exceeding $75.00 with an itemized, paid invoice.

## C25 Government Miscellaneous Charges

The Government will deduct payment for miscellaneous charges for goods or services furnished to the Contractor.

## C26 On-Ramping and Off-Ramping

C26.1 On-Ramping Procedures: The Government reserves the right to announce a new competition (on-ramping) for the purpose of adding additional vendors, multiple award, IDIQ contract holders. On-ramping procedures may be implemented at any time over the life of the contract by reopening the competition and utilizing the same basis of award established in the original solicitation (140D042XRXXXX). Bureau customers will initiate the need for additional contract holders by contacting the Contracting Officer. The Contracting Officer will assess the need for additional support. Should additional support be required, the Contracting Officer will publicize a notice in SAM.gov, issue a solicitation amendment, and complete evaluation in the same manner as the initial solicitation (140D042XRXXXX). Contracts awarded utilizing the on-ramping procedures will include the same terms and conditions as those in the initially awarded contracts. The start date of the period of performance will be adjusted to the award date of any subsequent contract awarded, the end date will not be adjusted. The overall ceiling of the basic contract will not be revised as a result of on-ramping.

C26.2 Off-Ramping Procedures: The Government reserves the right to off-ramp contractors from this IDIQ contract vehicle at any time during the period of performance. Off-ramping may occur for reasons including, but not limited to, consistent failure to meet task order requirements, poor performance evaluations, non-compliance with contract terms, or failure to actively participate in task order competitions. Contractors may also be off-ramped due to changes in agency priorities, funding constraints, or strategic realignments that reduce the need for specific services or capabilities.

Additionally, contractors may be removed from the IDIQ if they no longer meet eligibility requirements (e.g., small business status), are subject to debarment or suspension, or if the Government exercises its right to terminate for convenience or default. Off-ramping decisions will be made in accordance with applicable federal acquisition regulations and will be communicated in writing. The Government may also conduct periodic reviews to assess contractor performance and engagement, which may inform off-ramping determinations.

## C27 Add/Remove Aircraft/Equipment After Contract Award

After contract award and initial inspection, the Contractor may request in writing to the CO to add aircraft(s)/equipment during the month of December each year or as otherwise deemed necessary by the Government. When out of cycle additions are made, a notice will be sent to all vendors inviting them to add additional aircraft at that time. All additions are the Government’s discretion and based on current Government needs. The aircraft(s) requested to be added must be of equal or greater performance capability as the aircraft originally awarded. The optional accessory or miscellaneous equipment must be the same equipment listed in Section A, Requirements and Pricing. It is at the Government’s discretion as to whether additional aircraft(s)/equipment will be added to the contract. Each request will be evaluated based on DOI needs. The CO will make the final determination to add aircraft(s)/equipment to a contract through a bilateral modification. The request to remove aircraft can be done anytime during the contract period.

C27.1 Add Aircraft

C27.1.1 Same make, model and series may be offered at the same price as originally awarded and identified in the contract. Additions offered that are above the capabilities currently proposed by vendors may be allowed to provide revised pricing.

C27.1.2 The written request to add an aircraft(s) must include a signed copy of the Add/Remove Aircraft/Equipment Request Form (See Exhibit 13, Section C) and a copy of the Aircraft Questionnaire (Exhibit 14-a or 14-b, Section C). The required documents shall be submitted to the Contracting Officer.

C27.1.3 Any request to add an aircraft must include:

•	Airworthiness Certificate

•	Aircraft Registration

•	Current Weight and Balance (based on Section B32),

•	Equipment list certified accurate at the time of weighing,

•	Any calculations indicating changes to the aircraft and equipment list that may have occurred after the time of weighing,

•	Payload calculations showing performance characteristics of the offered aircraft,

•	Applicable performance charts from the aircraft flight manual

The contractor shall be responsible for contacting the AOM Technical Inspector for scheduling an inspection (C1 52.212-4(a) Inspection/Acceptance) after you have received confirmation from the CO that the aircraft will be added.

C27.3 Add Equipment

C27.3.1 The written request to add Optional accessory and miscellaneous equipment must include a signed copy of the Add/Remove Aircraft/Equipment Request Form (See Exhibit 13, Section C).

C27.4 Remove Aircraft(s)/Equipment

C27.4.1 The removal of either aircraft or equipment can be done at any time during the contract period. The written request shall be done by signing the Add/Remove Aircraft/Equipment Request Form (See Exhibit 13, Section C) and submitted to the Contracting Officer.

C27.4.2 If the contractor returns a leased aircraft or sells an aircraft on contract, the contractor is required to notify the Contracting Officer and the AOM Technical Inspector within 30 days of the action. To remove an aircraft after award, the Contractor must request in writing to the CO by submitting a signed copy of the Add/Remove Aircraft/Equipment Request Form. (See Exhibit 13, Section C)

C27.5 Substitute Personnel, Aircraft, or Equipment.

C27.5.1. The contractor (while on contract) may request the use of substitute personnel, aircraft, or equipment that was not initially approved for use. All proposed substitutes are at the sole discretion of the government and must meet contract specifications and be subject to inspections and approvals identified herein prior to use. All aircraft substitutions shall be of equal or greater capability than the aircraft being substituted for. The contractor must submit a written request for inspections of pilot substitutes to the AOM Technical Inspector seven days prior to the scheduled arrival at the site. Requests for aircraft substitution must be submitted to the CO for approval prior to inspection. The CO may issue a bilateral modification prior to submitting a request to the AOM Technical Inspector for inspection scheduling. Requests received with fewer than seven days’ notice will be accomplished as permitted by the AOM Technical Inspector schedule. Substitutions offered that are above the capabilities currently proposed by vendors may be allowed to provide revised pricing.

C27.5.2 Any request for aircraft substitution must include:

•	Completed Aircraft Questionnaire (Exhibit 14a)

•	Completed Aircraft Information Form (Exhibit 14b)

•	Airworthiness Certificate

•	Aircraft Registration

•	Current Weight and Balance (based on Section B32),

•	Equipment list certified accurate at the time of weighing,

•	Any calculations indicating changes to the aircraft and equipment list that may have occurred after the time of weighing,

•	Payload calculations showing performance characteristics of the offered aircraft,

•	Applicable performance charts from the aircraft flight manual

C27.5.3 The Contractor must transport substitute personnel, aircraft, or equipment to the point of use at their expense.

C27.5.4 The Government may charge the Contractor for the cost of any substitute inspections in accordance with this section.

# SECTION C EXHIBITS

The following exhibits are enclosed and made part of the contract:

- Aerial Supervision Mission Checklist

Unacceptable Lap Belt and Shoulder Harness Conditions

First Aid Kit and Survival Kit

Safety Management Systems (SMS), Flight Time, Accident History, and FAA Violation Questionnaire

Airplane Approximate Recommended Cruise” Fuel Consumption Rates

Civil Aircraft Certificate of Insurance, DD Form

Civil Aircraft Landing Permit, DD Form 2401

Civil Aircraft Hold Harmless Agreement, DD Form 2402

Transportation Worksheet

Unavailability Conversion Chart

Add/Remove – Aircraft Equipment Request Form

Aircraft Questionnaire

Aircraft Information Form

Equipment for Conducting Aircraft Operational Load Monitoring (OLM)

Task Order Competition Procedures

# Exhibit 1
Aerial Supervision Mission Checklist

Note: These checklist items are meant to be performed using challenge and response between pilot and aerial supervisor.

Preflight

Mission fuel 	Confirmed.

Weather en route/destination 		Checked.

Resource order/mission brief 		Accomplished.

Standard aircraft brief			Accomplished.

GPS 					Set.

After Takeoff/Enroute

Communication/radios 			Confirmed/set.

Other aircraft on scene/en route 		Confirmed.

Level of supervision on scene 		Confirmed.

Alternate airport(s)			Confirmed.

Time on station 				Determined/Re-evaluate*.

CRM (re-evaluate above tasks)		Accomplished.

Prior to FTA Entry

Altimeter 				Set.

Pulse/landing lights 			On/considered.

Transponder 				ALT, Squawk 1255 unless assigned by ATC.

*In the event of divert to a new incident, checklist items will be re-done.

# Exhibit 2
Unacceptable lap belt and shoulder harness conditions

| Item | Unacceptable Conditions |
| --- | --- |
| Webbing | - Frayed: 5 percent or more Torn Crushed Swelling: twice the thickness of original web or if difficult to operate through hardware Creased: no structural damage allowed Sun deterioration: severe fading, brittleness, discoloration, and stiffness |
| Hardware | - Inoperable buckle or other hardware Nylon bushing at shoulder-harness-to-lap-belt connection missing or damaged. Fabricated bushings or tie wraps used as bushings. Rust/corrosion: Only minor surface rust/corrosion allowed. Wear: wear beyond normal use |
| Stitches | - Broken or missing Severe fading or discoloring Inconsistent pattern |
| TSO Tags (see 14 CFR 21.607) | - Missing Illegible |
| Age | Belts/fabric over 10 years from date of manufacture will be closely inspected for possible damage from exposure to the elements, but do not have to be replaced if they can be determined to be in serviceable condition. |

# Exhibit 3
First Aid and Survivial Kits

Minimum First Aid Kit Items (includes Alaska) Each kit must be in a dust-proof and moisture-proof container. The kit must be readily accessible to the pilot and passengers. Passenger Seats Passenger Seats Item 0-9 10-50 Adhesive bandage strips, (3 inches long) 8 16 Antiseptic or alcohol wipes (packets) 10 20 Bandage compresses, 4 inches 2 4 Triangular bandage, 40 inches (sling) 2 4 Roller bandage, 4 inches x 5 yards (gauze) 2 4 Adhesive tape, 1 inch x 5 yards (standard roll) 1 2 Bandage scissors 1 1 Body fluids barrier kit: 1 1 2 pair latex gloves 1 face shield 1 mouth-to-mouth barrier 1 protective gown 2 antiseptic towelettes 1 biohazard disposable bag NOTE: Splints are recommended if space permits.

Minimum Aircraft Survival Kit Items • Fire starter (can be two boxes of matches in waterproof containers, “metal match” etc.) • Magnesium fire starter • Signal mirror • Whistle • One knife (includes “multi-tools” with knives) • Wire saw, axe, hatchet, or machete • Nylon rope or parachute cord (50 feet, minimum 1/8-inch [3mm] thick) • Collapsible water container (Sealing clear plastic bag(s)) • Water purification tablets • Water (one quart per occupant required except when operating over areas without adequate drinking water) • Food (2 days emergency rations per occupant, with a caloric value of 1,000 calories per day • At least one of the following will be in the aircraft: • Automated flight following system • Satellite phone • 406 MHz personal locator beacon (PLB) with GPS or aircraft-mounted 406 MHz ELT • Handheld UHF or VHF radio

Alaska Specific Rations for each occupant to sustain life for one week One Axe or hatchet An assortment of fishing tackle such as hooks, flies, lines, sinkers, etc. One mosquito head net for each occupant Mosquito repellant containing minimum 40% DEET Laser rescue light Signal flares (4 Each) Personal Locator Beacon (PLB) Note: Required only if aircraft ELT requires tools to be removed. October 15 to April 1 Pair of snowshoes (1) One sleeping bag (1) Wool blanket or equivalent for each occupant over 4 years of age (1) See the following ALSE link for other recommended survival kit items. https://www.doi.gov/sites/default/files/interagency-aviation-life-support-equiment-handbook-guide-v3.0.pdf

# Exhibit 4
Safety Management System (SMS), Flight Time, Accident History, and
FAA Violation Questionnaire (Pre-AWARD ver. 6.2 – 2025)

The Department of the Interior aviation program views Safety Management Systems (SMS) (safety policy, safety risk management, safety assurance, and safety promotion) as a critical element for aviation operations and contract evaluation. This Exhibit, abbreviated to ‘SMS Questionnaire’, seeks to identify safe and effective aviation practices of an Offeror that include implemented policies and practices that support and demonstrate the use of SMS. Refer to Section C7, Safety Management System (SMS), for more information on Contractor SMS review and site visits.

The following resources may assist with the completion of the SMS Questionnaire:

- SMS Questionnaire Supplement: Explanations of Questions 1-33 in the SMS Questionnaire, examples of suggested documents to provide, and common response errors. Located at: https://www.doi.gov/aviation/library

- SMS Questionnaire Form: An optional form that can be used to complete the SMS Questionnaire. Located at: https://www.doi.gov/aviation/library

- FAA Advisory Circular AC 120-92D: Safety Management Systems for Aviation Service Providers. Located at: https://www.faa.gov/regulations_policies/advisory_circulars/

- International Standard for Business Aircraft Operations (IS-BAO): Established as a recommended code of (SMS) best practices. Located at: https://nbaa.org/flight-department-administration/sms/

Overview

The Offeror must provide sufficient evidence of implementation for each Question (1-33). Include both evidence of implemented policies and records that indicate the SMS is functioning. Proposals are evaluated based on the evidence provided and the results that were achieved from the activities that were performed. Blank forms do not suffice as evidence of policy or records of practice and responses shall include the Exhibit Question number (e.g., Question 1, Question 2).

Be sure to review the Questions thoroughly since the Exhibit has recently been revised. Failure to provide documentation listed above is considered unacceptable and may remove the offer from further consideration.

Refer to FAA Advisory Circular AC 120-92D and IS-BAO for additional SMS information. For example, in the SMS Questionnaire, FAA Component Reference number refers to the section in AC 120-92D and the IS-BAO Element number refers to the IS-BAO standard/protocol. These documents are intended to provide additional guidance to address the information requested in the Exhibit.

Scalability

The basic SMS components (safety policy, safety risk management, safety assurance, and safety promotion) remain the same regardless of an Offeror’s size or complexity. The way in which organizations implement such components may be accomplished in a variety of ways but should be appropriate the Offeror’s operations. Per AC 120-92D, the “…FAA expects each air carrier to develop an SMS that works for its unique operation.” For example, it will take fewer details to describe safety communication practices for a single-pilot Offeror than an Offeror with multiple employees working in different locations.

| Question Number | FAA AC 120-92D Reference | IS-BAO Element (21st Ed.) | Requested Information |
| --- | --- | --- | --- |
| Safety Policy (Questions #1-12) |  |  |  |
| 1 | 3.3 | 3.1.3.1 | Provide reference to policy (company document(s)) which shows there is one individual responsible for the effective administration/implementation of the SMS (e.g., safety manager, director of safety). Provide evidence to show the position is filled. |
| 2 | 3.3 | 7.2.1 | Provide reference to and evidence from company document(s) that show SMS duties, authorities, and accountabilities for all employees. |
| 3 | 3.3 | 3.1.1.1 | Provide reference to and evidence of a strong organizational commitment and clear statement (e.g., safety/SMS policy) about the provision of necessary resources for the SMS. |
| 4 | 3.3 | 6.1.1 | Provide reference to and evidence of the company manual(s) that contain flight operations policy and aircraft maintenance policy. |
| 5 | 3.3 | 3.2.1 | Provide reference to the Operations Manual distribution policy. Provide evidence the current version of the Operations Manual is available to employees in accordance with company policy. |
| 6 | 3.3 | 6.1.1 | Provide reference to the Operations Manual internal approval policy. Provide evidence the Operations Manual is approved in accordance with company policy. |
| 7 | 3.3 | 6.1.1 | Provide reference to the Operations Manual revision policy. Provide evidence the Operations Manual has been revised in accordance with company policy. |
| 8 | 3.3 | 3.1.4.1 | Provide reference to the Emergency Response Plan (ERP) and ERP revision policy. Provide evidence the ERP is up-to-date in accordance with company policy. |
| 9 | 3.3 | 8.5 | Provide reference to policy showing how aircrew member records (e.g., FAA certificates, medical, proficiency records) are maintained. Provide evidence the aircrew records are maintained in accordance with company policy. |
| 10 | 3.3 | 15.1 | Provide reference to policy showing how aircraft maintenance/servicing personnel records (i.e., FAA certificates) are maintained. Provide evidence the personnel records are maintained in accordance with the policy. |
| 11 | 3.3 | 15.2.3 | Provide reference to company policy that ensures maintenance personnel are trained and approved to conduct specific maintenance (e.g., make/model). Provide evidence records are maintained in accordance with company policy. |
| 12 | 3.3 | 15.1 | Provide reference to Operations Manual policy for obtaining aircraft maintenance services while away from home base. Provide evidence of aircraft maintenance received or denied in accordance with the policy. |
| Safety Risk Management (Questions #13-16) |  |  |  |
| 13 | 3.4 | 3.2.1.1 | Provide reference to a risk management policy that includes: system analysis, hazard identification, risk analysis (exposure), risk assessment (severity and likelihood), decision making (mitigations and control), and validation of control (controls effective). Provide evidence the policy is in use. |
| 14 | 3.4 | 3.2.1.1 | Provide reference to policy for hazard/threat reporting and tracking. Provide evidence that the program is in use. |
| 15 | 3.4 | 3.2.1.1 | Provide reference to company policy addressing the use of operational risk assessments and/or use of a flight risk assessment tool. Provide evidence of a completed risk assessment that shows the implementation this policy. |
| 16 | 3.4 | 3.1.2.1 | Provide reference to policy that describes management approval requirements for operational risk assessments that exceed predetermined levels. Provide evidence of a risk assessment completed in accordance with the approval requirements. |
| Safety Assurance (Questions #17-23) |  |  |  |
| 17 | 3.5 | 3.3.1 | Provide reference to a policy (or company document(s)) showing safety objectives and associated safety performance targets (SPTs). Provide evidence showing SPTs are current, measurable, and being monitored. |
| 18 | 3.5 | 3.3.2 | Provide reference to a management of change (MOC) or change management policy. Provide evidence of the use of the change management policy (e.g., acceptance or rejection of a change). |
| 19 | 3.5 | 3.3.3 | Provide reference to the SMS continuous improvement policy. Provide evidence the SMS is being monitored and assessed in accordance with the policy. |
| 20 | 3.5 | 3.5 | Provide reference to policy showing that SMS audits must be completed at determined intervals (e.g., monthly, quarterly, annually). Provide evidence of a current audit schedule. |
| 21 | 3.5 | 3.5 | Provide reference to policy that shows SMS audits must be documented. Provide evidence of the most recently completed audit (i.e., audit report). |
| 22 | 3.5 | 3.5 | Provide reference to policy which requires SMS audit findings (e.g., deficiencies, nonconformities) to be addressed. Provide evidence of a finding being addressed, or corrective action being implemented. |
| 23 | 3.3 | 15.1 | Provide reference to the maintenance control system (method of documentation, tracking, scheduling, and recording). Provide evidence of the maintenance control system in use. |
| Safety Promotion (Questions #24-30) |  |  |  |
| 24 | 3.3 | 4.1 | Provide reference to policy showing Emergency Response Plan roles (i.e., key personnel), responsibilities, and duties. Provide evidence that training has been provided for these personnel to accomplish the duties. |
| 25 | 3.3 | 4.3 | Provide reference to policy showing the Emergency Response Plan must be exercised (drill/test/exercise) at least annually and documented. Provide evidence of the most recent ERP exercise. |
| 26 | 3.6 | 3.4.2 | Provide reference to the SMS safety communication policy. Provide evidence the policy describes the types of information to be communicated and how it will be communicated (e.g., method, frequency, responsibility). |
| 27 | 3.6 | 3.4 | Provide reference to approved company safety information sharing methods (e.g., intranet, bulletin board). Provide evidence of safety lessons learned shared with personnel. |
| 28 | N/A | N/A | Provide reference to a safety award/incentive program. Provide evidence of the most recent award. |
| 29 | 3.6 | 8.1 | Provide reference to the company training program (FAA and internal) that ensures ground crew and air crew are trained and competent to perform their assigned duties. Provide evidence that personnel have been trained and are competent to perform their duties. |
| 30 | 3.6 | 3.4.1 | Provide reference to policy showing initial (new employee) and recurrent SMS training requirements. Provide evidence of the most recent SMS training record (initial or recurrent). |
| Flight Time, Accident History, and FAA Violations (five years: 10/1/2019 - 9/30/2024) (Questions #31-33) |  |  |  |
| 31 | N/A | N/A | Total number of flight hours during the five years defined in the Exhibit. Flight hours must be separated by year and category: fixed-wing, rotary-wing, and uncrewed aircraft system (UAS). Example: 10/1/2019-9/30/2020: total _____ (separate by category as applicable: fixed, rotor, UAS) 10/1/2020-9/30/2021: total _____ (separate by category as applicable: fixed, rotor, UAS) 10/1/2021-9/30/2022: total _____ (separate by category as applicable: fixed, rotor, UAS) 10/1/2022-9/30/2023: total _____ (separate by category as applicable: fixed, rotor, UAS) 10/1/2023-9/30/2024: total _____ (separate by category as applicable: fixed, rotor, UAS) Five year total: ________________ |
| 32 | N/A | N/A | Total number of accidents (aircraft accident as defined within 49 CFR 830.2) during the five years defined in the Exhibit. Accident details must be provided for each accident listed (e.g., date, NTSB accident number, registration number). If the Offeror had an accident in the five years identified, provide an accident prevention plan.* The prevention plan must include the following elements: a) summary of accident (i.e., what happened?), b) description of mitigation(s) to prevent future occurrence (i.e., how was it corrected?), and c) description of assurance methods used to validate the effectiveness of mitigation measures (i.e., how do you make sure it doesn't happen again?). *The submission of an accident prevention plan is not required, but if provided, may positively impact the offer. |
| 33 | N/A | N/A | Total number of FAA violations (Civil Penalty) during the five years defined in the Exhibit. If violations were reported, Case information must be provided. |

# Exhibit 5
Airplane Approximate “Recommended Cruise" Fuel Consumption Rates

AIRCRAFT	GAL/HOUR

AIR TRACTOR

AT-802-65	82T

AT-802-67	86T

AT-802-67F	90T

AT-802-331	80T

BEECH

Baron 55/58	29

Baron 58P	35

18/45	35

King Air 90 (Series)	70T

King Air 100	80T

King Air 200	93T

King Air 300	89T

Mentor T-34	14

Mini Liner 99	85T

Queenair	43

T-Bone B-50	45

Duke	38

Queenair 80	34

Bonanza 33/35/36	14

BELLANCA

Citabria/Scout	9

BOEING

727	1000T

DC-3T	150T

DC-3	.93

DC-9	….700T

BOMBARDIER

CL-215	150

CL-215T	237T

CL-415	237T

BRITTEN-NORMAN

Islander	30

CESSNA

L-19	11

172	9

180	14

182	12

182RG	14

185	16

188	12

206/207	16

T206/207	18

210	16

T210	18

320	28

310	32

T310	34

337	16

T337	18

401	28

402	36

404	46

411	34

414	36

421	46

340	36

425	56T

441	52T

Citation I	140T

Citation II, III	160T

208 Caravan I	53T
AIRCRAFT	GAL/HOUR

CASA

Aviocar 212	105T

CONSOLIDATED

Convair 580/600	338T

DE HAVILLAND

Beaver	30

DH-6 Twin Otter	82T

Beaver T	40T

DH-7	234T

DH-8	183T

DORNIER

DO-228	90T

EMBRAER

PW 115 (1600)	.159T

FALCON

10	.175T

20	240T

50	300T

FAIRCHILD

Porter	44T

F-27	288T

SF 340	161T

GRUMMAN

S2F	75

Goose	50

Goose T	80T

Mohawk	180T

Albatross	160

G-I	288T

G-II	529T

G-III	370T

HELIO

Courier	 20

Stallion	.  44T

ISRAEL

1121/1123/1124.	 190T

LOCKHEED

P-3/C-130/Electra 188	661T

Jetstar	 374T

LEAR JET

25	220T

35/36	185T

55	200T

MAULE

M4-5	9

M7	13

MITSUBISHI

MU-2	77T

MOONEY

Exec	10

AIRCRAFT	GAL/HOUR

NOMAD

22B/24A	53T

NORTH AMERICAN

AT-6	18

T-28	80

QUEST

Kodiak K100	47T

PILATUS

PC-12	....77T

PIPER

Super Cub PA-18	9

Pawnee PA-25	13

Arrow PA-28R	10

Cherokee PA-28-140/161	8

Cherokee 6 PA-32	14

Aztec PA-23	27

Comanche PA-24	15

Navajo PA-31	32

Cheyenne PA-31T	77T

Cheyenne II PA-42T	74T

Aerostar 600/601/602P/700P	33

Cheyenne III PA-42T	110T

Chieftain PA-31-350	34

Lance PA-32-300	16

Seneca II/III PA-34	20

Mojave PA-31P	39

Malibu PA-46	15

PA-42-1000 400LS	75T

QUEST

Kodiak....................................................	47T

ROCKWELL (Gulfstream)

680	40

500	34

690A/B	80T

Sabreliner	200T

560	…33

840	65T

900	55T

1000	57T

SWEARINGEN

Merlin II, III SA 226/7	78T

SHORT

Skyvan	85T

VOLPAR

TurboLiner	74T

VULCANAIR

P68C	20

P68TC	24

“T” after the gallons indicated turbine fuel. Everything else is AVGAS.

# Exhibit 6
Civil Aircraft Certificate of Insurance, DD Form 2400

https://www.esd.whs.mil/Portals/54/Documents/DD/forms/dd/dd2400.pdf

# Exhibit 7
Civil Aircraft Landing Permit, DD Form 2401

https://www.esd.whs.mil/Portals/54/Documents/DD/forms/dd/dd2401.pdf

# Exhibit 8
Civil Aircraft Hold Harmless Agreement, DD Form 2402

https://www.esd.whs.mil/Portals/54/Documents/DD/forms/dd/dd2402.pdf

# Exhibit 9
Transportation Worksheet

| When assigned to an alternate base, the Contractor will be paid for actual necessary and reasonable costs associated with transporting authorized personnel. The Contractor is responsible for advising the on-site Government representative(s) of the anticipated cost associated with transporting relief (and/or maintenance) personnel to the alternate base prior to the relief exchange. Claims must be supported by itemized invoices. See contract clause “Transportation Costs Associated with Operating Away from the Designated Base” for detailed information. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: |  | Alternate Base Location: |  |  |  |  |  |  |
| Relief Exchange – Involved Crew Member(s) |  |  |  |  |  |  |  |  |
| Pilot |  |  |  | Name: |  |  |  |  |
| Fuel Servicing Vehicle Driver |  |  |  | Name: |  |  |  |  |
| Mechanic (if required by contract) |  |  |  | Name: |  |  |  |  |
| Scheduled Maintenance |  |  |  |  |  |  |  |  |
| Mechanic |  |  |  | Name: |  |  |  |  |
| Other (specify): |  |  |  | Name: |  |  |  |  |
| Maintenance Accomplished: |  |  |  | Reason for providing additional personnel: |  |  |  |  |
| Itemization of Costs – Invoices and/or receipts are attached (copies are acceptable) |  |  |  |  |  |  |  |  |
| Airline Transportation |  |  | Name: |  |  |  | $ |  |
| Airline Transportation |  |  | Name: |  |  |  | $ |  |
| Charter Aircraft |  |  | Invoice to include aircraft make/model, flight time, hourly rate, passengers, and departure/destination location, date and time (Authorized by CO only) |  |  |  | $ |  |
| Rental Car |  |  |  |  |  |  | $ |  |
| Rental Car Fuel |  |  |  |  |  |  | $ |  |
| POV Auto: At the FTR rate (http://www.gsa.gov/portal/content/100715) |  |  |  |  |  |  |  |  |
| From: |  |  | To: |  | Total Miles | Rate | $ |  |
| POV Aircraft: Measured in statute miles at the FTR rate (http://www.gsa.gov/portal/content/100715) |  |  |  |  |  |  |  |  |
| From: |  |  | To: |  | Total Miles | Rate | $ |  |
| Other (explain): | 1. |  |  |  |  |  | $ |  |
|  | 2. |  |  |  |  |  | $ |  |
|  | 3. |  |  |  |  |  | $ |  |
| Total ACTUAL Cost |  |  |  |  |  |  | $ |  |
| Yes, the COR or On-site Government Representative was notified of the anticipated cost for this alternate base transportation expense prior to mobilization of the relief personnel. |  |  |  |  |  |  | Date: |  |

Version 1 (03/18/26)

# Exhibit 10
Unavailability Conversion Chart

| HOURS UNAVAILABLE | UNITS OF AVAILABILITY RECORDED AS: | UNITS OF UNAVAILABILITY RECORDED AS: |
| --- | --- | --- |
| 0 | 1.00 | 0.00 |
| 1 | .93 | .07 |
| 2 | .86 | .14 |
| 3 | .79 | .21 |
| 4 | .71 | .29 |
| 5 | .64 | .36 |
| 6 | .57 | .43 |
| 7 | .50 | .50 |
| 8 | .43 | .57 |
| 9 | .36 | .64 |
| 10 | .29 | .71 |
| 11 | .21 | .79 |
| 12 | .14 | .86 |
| 13 | .07 | .93 |
| 14 | 0.00 | 1.00 |

# Exhibit 11
Add/Remove – Aircraft/Equipment Request Form

| Contractor Name |  |  |
| --- | --- | --- |
| Contractor Telephone Number |  |  |
| Name of Contractor’s Representative Making this Request |  |  |
| Date and Signature of Above |  |  |
| Contract Number |  |  |
| REMOVE – Aircraft Make, Model, and Series |  |  |
| REMOVE – Equipment |  |  |
| ADD – Check the appropriate request below: Same Aircraft Make, Model, and Series Equipment (To Existing Contract Aircraft) Different Aircraft Make, Model, and Series (Must include pricing information on following pages) |  |  |
| FAA N Number |  |  |
| Reason for addition/deletion |  |  |
| If adding aircraft, attach a copy of the Aircraft Questionnaire (Exhibit 17) and Aircraft Information Form (Exhibit 18) as appropriate that shows the aircraft to be added to the contract and documents per B5 through B7. |  |  |
| If adding aircraft, confirm by checking the box to the right and provide data to support that the aircraft is of equal or greater performance capability (must demonstrate aircraft is of the same make, model and series) as the aircraft originally awarded. |  |  |
| Any other comments or pertinent information: |  |  |
| Submit form and applicable documents to your Contracting Officer |  |  |

# Exhibit 12
Aircraft Questionnaire

You may recreate this form, but it must include all the information listed.

This questionnaire accurately completed will be used to determine the proposed aircraft payload capability.

COMPLETE SHADED BLOCKS

| Minimum Aircraft Requirements: Must have a Standard Airworthiness Certificate. Aircraft certified under 14 CFR Part 23 or 25, or CAR 3. An aircraft make and model for which engineering and logistical support, for continued airworthiness, is provided from the current type certificate holder or supplemental type certificate holder. Certified for VFR/IFR, Day/Night. Certified for Flight Into Known Icing. Aircraft must have environmental controls including heating and air conditioning. Insured seating for three (3) passengers which includes the right front seat located in the cockpit. Removable seats for cargo configuration. Cruise airspeed 160 knots true airspeed at 8,000 feet pressure altitude and temperature ISA+20ºC. Cruise airspeed 220 knots true airspeed at 8,000 feet pressure altitude and temperature ISA+20ºC for Exclusive Use. Endurance of 4.0 hours of fuel and 30-minute fuel reserve at 8,000 feet pressure altitude and temperature ISA +20°C. Aircraft must be certified for single pilot operations. Visibility: No external obstructions within the field of regard, 9 o’clock to 3 o’clock, as viewed from the cockpit and minimal restrictions to outward visibility of the ground in level flight for the rear observers Payload of 900 pounds with fuel for 250 nautical mile flight, VFR fuel reserve, at 8,000 feet pressure altitude, temperature ISA+20°C, 200-pound pilot and 25-pound survival kit. Multi Engine additional requirements: Cruise airspeed minimum 220 knots true airspeed at 8,000 feet pressure altitude, ISA+20ºC. Pressurized aircraft with environmental controls, heater and air conditioner. Positive single engine rate of climb when equipped for this contract with a 200-pound pilot, one 220-pound observer, 4 hours of fuel plus a 30- minute reserve, at 5,000 feet pressure altitude, ISA+33º C. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Aircraft Make and Model | FAA Registration # |  | Serial # | Cruise Airspeed |  |  |
| Aircraft Equipped Weight (Your aircraft weight & balance report & equipment list will be used to substantiate this weight – see note 1. below) |  |  |  |  |  |  |
| Currently Installed Aircraft Equipment to be Removed to Achieve Offered Payload Below (If none, indicate NONE) |  |  | Equipment to be Added to Meet the Aircraft Specifications Requirements (If none, indicate NONE) |  |  |  |
| EQUIPMENT ITEM |  | WEIGHT | EQUIPMENT ITEM |  | WEIGHT |  |
| ENTER YOUR PROPOSED AIRCRAFT PAYLOAD (The aircraft make, model and payload that we confirm with you will be made a part of your offer and will be binding if your offer is accepted for award |  |  |  |  |  |  |

Note: By signing below, I acknowledge that I have included the following documentation to support the minimum aircraft requirements in Sections A and B:

(1) Current aircraft weight and balance report, as specified in section B32 Weight and Balance

(2) Aircraft equipment list certified accurate at the time of weighing and any revisions to the list,

(3) Applicable Performance Charts

(4) Calculations showing performance characteristics of the aircraft. Calculations must include weight of the aircraft in contract configuration, crew, and fuel as identified in Section A1. (use 6.7 lbs. per gal. for fuel weight)

(5) 14 CFR Part 135 Certificate

Failure to include the above information, will affect the evaluation you receive for your aircraft.

By signing below, I certify to the best of my knowledge that the above information is accurate.

| Name and Signature of Representative Completing this Form | Date |
| --- | --- |
| OFFEROR’S NAME |  |

# Exhibit 13
Aircraft Information Form

The proposal may recreate this form, but it must include all the information listed

| Offeror’s Name: |  |  |  |
| --- | --- | --- | --- |
| Aircraft Make and Model: |  | Aircraft Tail Number: |  |

| Cruise airspeed |  | knots |  |
| --- | --- | --- | --- |
| (8,000’ PA and ISA +20°C or (12,000’ PA and ISA +20°C) |  |  |  |
| Payload |  | Lbs. |  |
| (with 200lb per required pilot, 35lb Survival Kit, and 3 hours of fuel at 8,000’ PA and 20°C) |  |  |  |
| Endurance |  | Hrs. |  |
| (8,000’ PA and ISA +20°C) |  |  |  |
| Number of Insured Passenger Seats |  |  |  |

Minimum Aircraft Requirements:

| Positive single engine rate of climb (5,000’ PA and ISA +33°C) (Multi only) (with 200 lb. pilot, 25 lb. survival kit, 220 lb. observer, & 4.5 hrs. of fuel) | Yes | No |
| --- | --- | --- |
| Aircraft certified under 14 CFR 23, or 25 | Yes | No |
| Aircraft Certified under CAR 3 | Yes | No |
| Approved for gravel runways (Alaska dispatch only) | Yes | No |
| VFR/IFR, Day/Night, Known Ice. | Yes | No |
| Pressurized aircraft with environmental controls. Heater and air conditioner | Yes | No |
| Rear ATGS training station (full audio panel) | Yes | No |
| Rear transmit capability | Yes | No |
| EFB charging capability | Yes | No |
| Pressurized cockpit and cabin (Multi only) | Yes | No |
| Removable seats for cargo configuration | Yes | No |
| Aircraft Certified for single pilot operations (TCAS/TCAD/TAS) TIS & ADS-B do not satisfy this requirement | Yes | No |
| Environmental controls including heating and air conditioning. | Yes | No |
| Single Pilot Certified | Yes | No |
| Insured seating for three (3) passengers which includes the right front seat located in the cockpit. | Yes | No |
| No external obstructions within the field of regard, 9 o’clock to 3 o’clock, as viewed from the cockpit and minimal restrictions to outward visibility of the ground in level flight for the rear observers. | Yes | No |

Describe any visibility enhancements for the Government observers that your aircraft may have.

# Exhibit 14
Equipment for Conducting Aircraft Operational Load Monitoring (OLM)

- Contractor shall instrument aircraft with a government reviewed and approved OLM system. This system must provide to the government required and specified parameters in Table 1 below. Aircraft being offered that do not have a government approved OLM system installed shall install an OLM system that outputs data in the ".cdf “ file format.

The recorded data shall be submitted to AOM Technical Services (Jeffrey_nuttall@ios.doi.gov) every 15 days while on contract. The recorded OLM data shall be in a ".sie or ".cdf file format. All values in ".sie" and ".cdf “ files shall be in engineering units as listed in Table 1. The SIE and CDF files shall include column header descriptions as listed in Table 1, Channel Description (including engineering units for the values in each column). Acceleration data shall be described as either incremental or total. There shall be only one flight per recorded file.

The file naming convention for each flight file shall be "Aircraft N Number"_"date (YYYYMMDD)"_timestamp in GMT (E.g. N34VJ_20140228_1456)

If the OLMS system is inoperable or malfunctioning the aircraft operator shall have the problem corrected within 10 business days of finding the error. If the problem is not corrected, the aircraft will be made unavailable until the OLMS is operating and properly recording data.

The OLM system shall be installed and approved by one of the three following methods:

  - STC

  - Approved for installation by an FAA From 8110-3 with all DER supporting engineering substantiation documentation attached.

  - Field Approved on an FAA form 337 for installation with supporting FAA Form 8110- 3 and all DER supporting engineering substantiation documentation.

The OLM system shall have an annual calibration prior to annual pre-use inspection.

  - Aircraft shall be flown by the contractor in a typical mission profile.

  - Data shall be collected for this flight.

  - The contractor shall review the data to ensure all channels are recording data and that the data is correct in accordance with this contract and the contractors OLM program.

  - The contractor shall notify AOM Technical Services (Jeffrey_nuttall@ios.doi.gov) of the specific flight date and time for the calibration flight. The government will review this data to ensure it meets requirements.

OLM System and Program:

  - Criteria: To properly monitor the ASM/ATGS usage of a specific model aircraft, a complete instrumentation package and recording device are required. The following section details the minimum required parameters and instrumentation to be recorded at a minimum sample rate of 8Hz.

  - Accelerations shall be recorded as close to the aircraft Center of Gravity as practical, or correction algorithms may need to be validated and applied. Systems shall have functional and calibration flights recorded annually.

  - Continuous Monitoring OLM Requirements: These are minimum system requirements for all other aircraft of a particular model in ASM/ATGS operation for continuous monitoring while in ASM/ATGS service. The instrumentation and equipment utilized must include all mechanical components required to measure the flight parameters listed. The system shall have detailed installation instructions, drawings and instructions for continued airworthiness (ICAs). The ICAs will also include an installation validation plan for system and scheduled calibration check due annually.

Minimum requirements for a Continuous Monitoring OLM system:

  - Data Acquisition and Transmittal Requirements: The flight data recorder utilized for the data acquisition must be capable of recording all flight parameters. Recorders shall be capable of recording flight data for up to 100 flight hours without replacing the data capture media. Recorded data shall be compatible with DOI/Forest Service Data Library software solution.

  - The Contractor's OLM program shall:

    - Identify the OLM system installation, calibration process, and frequency of recalibration.

    - OLM system shall be professionally installed using OEM recommended installation procedures.

    - Identify the location of the recording device of the OLM system. The system does not need to be crash survivable; however, the Contractor shall consider the most crash survivable location within the aircraft with regard to fire and damage from a crash for the recording unit.

    - Identify the parts or measured parameters that are required to be operational for each flight.

    - Contain procedures to assure the OLM system is fully functional for each flight, including all measured parameters.

    - Identify the specific parameters selected for recording with rationale for their selection.

    - Identify the location, purpose and use of the parameters selected. Parameters identified as being required for developing revised Instructions for Continued Airworthiness (ICA's) shall be so identified and be given greater description as to their use.

    - Provided an explanation of the analysis of the data obtained from the aircraft OLM system.

    - Contain procedures for the integration of the analyzed aircraft operational load data into the Contractor's inspection program.

    - Define and provide a detailed explanation of the exceedance for each of the recorded parameters.

    - Thoroughly explain the Contractor's definition of a structural exceedance. Structural exceedances may be single or multiple parameter exceedances.

    - Contain procedures to take (i.e., inspect, repair, or other maintenance action) when a structural exceedance occurs.

    - Contain procedures for notification (timeliness and method) to the Government for all defined exceedances and the planned actions and timeline to complete them.

    - Contain procedures for retrieval of the aircraft OLM data, analysis of the data, process for defining/deciding on a maintenance action, and implementation of the maintenance action.

    - Contain procedures for an annual calibration of the system to ensure all channels are recording correctly. The procedure shall contain a representative flight profile to be flown for the calibration flight.

Table 1 Continuous Monitoring OLM Minimum Channel List

The following are minimum required parameters to be recorded at 8 Hz.

| Channel Description | Number of Channels | Units | Sample Rate (Hz) | Record Rate (Hz) |
| --- | --- | --- | --- | --- |
| 1. Altitude (GPS) | 1 | Feet | 4Hz | 8Hz |
| 2. Groundspeed (GPS) | 1 | Knots | 4Hz | 8 Hz |
| 3. Vertical Speed (GPS) | 1 | Feet per Minute | 4Hz | 8Hz |
| 4. Heading (GPS) | 1 | Degrees | 4Hz | 8Hz |
| 5. Date and Time in GMT (GPS) | 1 | yyyy/mm/dd GMT | 4Hz | 8Hz |
| 6. Latitude (GPS) | 1 | Degrees | 4Hz | 8 Hz |
| 7. Longitude (GPS) | 1 | Degrees | 4Hz | 8 Hz |
| 8. Engine Start (one engine oil pressure) | 1 | Discrete | 8Hz | 8 Hz |
| 9. Pitot Pressure | 1 | Inches Hg | 8Hz | 8 Hz |
| 10. Static Pressure | 1 | Inches Hg | 8Hz | 8 Hz |
| 11. Outside Air Temperature | 1 | Degrees C | 8Hz | 8 Hz |
| 12. Altitude (Static Pressure) | 1 | Feet | 8Hz | 8 Hz |
| 13. Indicated Airspeed (must be derived from Pilot/Static differential) | 1 | Knots | 8Hz | 8 Hz |
| 14. Equivalent Airspeed | 1 | Knots | 8Hz | 8 Hz |
| 15. True Airspeed | 1 | Knots | 8Hz | 8 Hz |
| 16. Weight on Wheels | 1 | Discrete | 8Hz | 8 Hz |
| 17. Flap Position | 1 | Degrees | 8Hz | 8 Hz |
| 18. Speed Brake/Spoiler Position (if equipped) | 1 | Degrees | 8Hz | 8 Hz |
| 19. Normal Acceleration (Nz) | 1 | G Force | 8Hz | 8 Hz |
| 20. Longitudinal Acceleration (Nx) | 1 | G Force | 8Hz | 8 Hz |
| 21. Lateral Acceleration (Nv) | 1 | G Force | 8 Hz | 8Hz |
| 22. Pitch | 1 | Degrees | 8 Hz | 8 Hz |
| 23. Pitch Rate | 1 | Degrees per Sec. | 8 Hz | 8 Hz |
| 24. Roll | 1 | Degrees | 8 Hz | 8Hz |
| 25. Roll Rate | 1 | Degrees per Sec. | 8 Hz | 8Hz |
| 26. Yaw Rate | 1 | Degrees per Sec. | 8 Hz | 8Hz |
| 27. Vertical Accuracy (VDOP) | 1 | --- | 8 Hz | 8 Hz |
| 28. Horizontal Accuracy (HOOP) | 1 | --- | 8 Hz | 8 Hz |
| 29. Fuel Quantity (May be manually collected on Supplemental Data Sheet) | 1 | Lbs. | 8 Hz | 8 Hz |
| 30. Aircraft Gross Weight (May be manually collected on Supplemental Data Sheet) | 1 | Lbs. | 8Hz | 8Hz |

# Exhibit 15 
Task Order Competition Procedures

- The purpose of this exhibit is to describe the process for competing Task Orders (TOs). After award of the IDIQ contracts, the Government may issue a Task Order Request for Proposals (TORP). This competition will be limited to the IDIQ contract holders. When submitting pricing for TORP requests, vendors will be permitted to provide discounts from their IDIQ pricing.

- Task Orders may include options.

- The TORP may require parent contract holders to provide additional pricing and/or technical information. However, the non-price factor ratings from proposal evaluation will be used for the TORP Selection.

- Unless a TO requirement meets one of the exemptions listed in FAR 16.505 (b)(2), it will be considered a competitive requirement. Furthermore, each requirement will be considered full and open competition, regardless of dollar value in accordance with the IDIQ contracts.

- Unless noted otherwise in the TORP letter from the CO, the time frame for receipt of proposals is 10 calendar days.

- The Government reserves the right to not award any order(s) after requesting a TORP proposal. Regardless of whether an order is awarded or not, the Government shall not be responsible for the any costs incurred in preparation of the TORP response.

- The ordering office reserves the right to negotiate with TORP offerors.

- All TOs will be issued as FFP or FFP per unit of service IAW IDIQ contracts and based on the pricing offered in response to the TORP.

- Each TORP request will specify the evaluation criteria and relative order of importance for that particular request.

# SECTION D – INSTRUCTION TO OFFERORS

# INTRODUCTION

## D1 General Information

The services of this Request for Proposals (RFP) are being acquired under the authority of FAR Part 12, Acquisition of Commercial Items and FAR Part 15 Contracting by Negotiation.

Offerors who wish to compete for the contract described in Sections A through C of this RFP, must submit a proposal that includes a signed and dated offer and other requested information by the time and date shown in block 8 on the SF1449 unless changed by an amendment to the solicitation.

Offerors are reminded that the Government intends to award without discussions. Consequently, each Offeror must ensure that its proposal is sufficiently clear and complete to be evaluated without discussions. However, the Government reserves the right to engage in discussions under the provisions of FAR 15.306 if the Government deems it to be in the Government’s best interest to do so.

Assent to Solicitation. If the Offeror takes exception to any of the terms and conditions of the RFP, then the Government may consider its offer to be unacceptable. Unacceptable offers will be eliminated without further consideration of the proposal.

## D2 North American Industry Classification System (NAICS) Code and Small Business Size Standard

The NAICS code and small business size standard for this acquisition is 115310 – Support Activities for Forestry – Forest Fire Suppression (Exception 2) and $34.0 Million dollars respectively.

## D3 Instructions to Offerors – Commercial Items (52.212-1 MAR 2023)

This addendum makes the following changes to FAR 52.212-1, Instructions to Offerors - Commercial Products and Commercial Services, as follows:

Replace paragraph (b) entitled “Submission of offers” with the following instructions:

To be considered as a prospective contractor for the requirement identified in this solicitation, offerors are required to meet all solicitation requirements, including terms and conditions, representations and certifications, and technical requirements, in addition to those identified as evaluation factors or subfactors. Failure to meet any of the requirements as outlined in this RFP may result in an offer being ineligible for award.

Submission of Offer. Submit a signed and dated proposal to the email address specified in this solicitation at or before the exact time specified in this solicitation. Failure to provide all required documents or information may exclude the vendor from further consideration. Only a signed proposal submitted on the SF 1449 will be accepted.

Questions Concerning Solicitation: All questions are to be submitted electronically by the following due date of XXXXXX XX. 202X by 2:00 PM Eastern Time (ET). The final proposal is as stated in Block 8 of the SF 1449. The main Point of Contact (POCs) for questions is Angelina Clements (CO) at Clements_proposal_submissions@ibc.doi.gov.

Questions shall identify the specific Solicitation Paragraph or Page number referenced.

One (1) electronic copy of the proposal shall be submitted to the CO via email to (Clements_proposal_submissions@ibc.doi.gov) by the due date specified in Block 8 of the SF1449. The email subject line should include the Contractor/business name. The CS/CO will acknowledge receipt of each email submitted.

The email message size limit is 40MB. Depending on the size of the files, multiple emails may be required to ensure proper receipt of all required documents. If multiple emails are required, each email should be numbered “1 of x”, “2 of x”, etc. in the subject line to show how many emails there are in total.

The vendor shall remain responsible for on-time delivery to, and receipt by, the CO.

This is an electronic solicitation release. All proposals are to be submitted electronically. Proposals that are mailed, hand delivered, or submitted by facsimile will not be accepted.

### Proposal Instructions

The proposal shall be submitted in four volumes as follows:

| Volume Number | Description | Page Limit* |
| --- | --- | --- |
| Volume I | General/SF 1449/SF 30/Completed Pricing | No Limit |
| Volume II | Technical Capability | No Limit |
| Volume III | Organizational Safety | No Limit |
| Volume IV | Past Performance | 20 pages |

*Table of Contents and Cover Sheet do not count towards the page count.

Each Volume shall be submitted as a separate file.

## D4 Offer Contents

### Volume I, General

Volume I shall include:

A completed SF 1449, Solicitation/Contract/Order for Commercial Items, dated, and signed by an authorized agent of the contractor.

Offeror’s Representations and Certifications are incorporated by reference in accordance with FAR 52.212-4 paragraph (v). Offerors shall ensure their annual representations and certifications are completed online at www.sam.gov in the System for Award Management (SAM) and include a copy of the first page of their SAM registration showing current SAM registration status. The Contracting Officer will verify the representations and certifications have been completed upon receipt of proposals.

Acknowledgment of Solicitation Amendments (if any).

A completed copy of Offeror’s Miscellaneous Information included in Section E (Exhibit E1).

Pricing pages (attached to this solicitation) shall be completed for On-Call rates by the vendor where indicated with the aircraft make and model and the unit rates offered in US whole dollars for daily availability for Ordering Years 1-5 (Base Period). If a vendor intends to propose multiple aircraft, then a pricing page should be completed for each one. Vendors may offer varying prices for subsequent ordering years. All rates proposed will be ceiling rates established and incorporated into any forthcoming contract. Ordering Years 6-10 (Option Period) shall not be completed at proposal submission, these years will be proposed under prospective price redetermination in Year 5.

### Volume II, Technical Capability

Volume II shall include the following documentation:

The Offeror should identify which aircraft are being submitted for Exclusive Use.  All proposed aircraft will be listed for On-Call.

A completed copy of the Aircraft Questionnaire and Aircraft Information Form (Section E-2 and E-3 (as appropriate) for each offered aircraft.

Provide all appropriate charts and calculations to support the performance characteristics of the proposed aircraft. Use current and correct performance charts from the Aircraft Flight Manual or Flight Manual Supplement as appropriate.

Ensure fuel requirements are established using values prescribed in Section A1, Minimum Aircraft Requirements, and the fuel consumption rate for the make and model offered based on Exhibit 4, Airplane: Approximate "Recommended Cruise" Fuel Consumption Rates. Offerors must use a standard fuel weight of 6.7 lbs. per gallon for performance calculations.

Offerors must ensure the pressure altitude and temperature values specified in Section A1 Minimum Aircraft Requirements are used when preparing performance calculations for the submission.

Provide a current (Reference chart in B32.1) aircraft weight and balance record, equipment list certified accurate at the time of weighing, and any calculations indicating changes to the aircraft and equipment list that may have occurred after the time of weighing. Weight and Balance Handbook (FAA-H-8083-1B) can be used as a guide.

For all performance calculations, the aircraft empty weight must include all installed equipment required by this contract. Calculations showing equipment removed or added to meet contract specifications must be included.

For aircraft that are new and /or the empty weight is yet to be determined. The proposal must include an estimated empty weight based on the most recent weight and balance with any modifications required to bring the aircraft into contract compliance. The offeror must provide the most recent weight and balance and calculations on how they arrived at the estimated empty weight.

A detailed plan and timeline for completion must be provided for aircraft that require modifications to meet the requirements of this solicitation.

Airworthiness Certificates. A valid copy of the airworthiness certificate for each offered aircraft.

Aircraft Registration: A valid copy of the registration for each offered aircraft

Offerors shall provide a copy of the first page of any FAA certificates and a complete electronic copy of the entire FAA operation specifications Part 135.

Part 135 Operation Specifications A003 (Aircraft Authorization), demonstrating authorization to operate in the category and class of aircraft and under flight conditions required by this contract per B2.1 (e.g., airplane, multiengine, VFR, IFR, day/night, passengers, and cargo) in the contiguous United States and Alaska.

Part 135 Operation Specification B050 (Authorized Areas of En Route Operations, Limitations, and Provisions) showing authorization to operate in the contiguous United States and Alaska.

A copy of the ‘list’ required by 14 CFR Part 135.63(a)(3) (Operations Specifications part D, “D085 Aircraft Listing”) as appropriate which identifies aircraft that the offeror is authorized to operate; however, if offered aircraft is not listed on the D085, a detailed plan and timeline to meet this requirement must be provided at time of proposal submission. Offeror must provide proof of ownership or purchase/lease agreement that includes a delivery date for the aircraft.

A copy of the inspection program or supplement from the OEM/TC holder for the aerial supervision missions described in Section B1. Reference Section B26.1.1 and B29.6 of this solicitation.

Failure to provide documentation listed above to support the performance of the aircraft is considered unacceptable and may remove your offer from further consideration.

The aircraft make, and model proposed will be made a part of the binding contract if the proposal is accepted for award.

### Volume III, Organizational Safety

The Offeror must provide evidence of meeting FAA SMS requirements for Part 135 or air tour operators operating under § 91.147 no later than May 28, 2027 (FAA Final Rule, May 2024). Evidence may consist of a letter, email, or other documentation from FAA, showing the SMS requirement has been met.

A copy of the current SMS Manual must be submitted.

A response to the SMS Questionnaire (Exhibit 4) must be submitted. The SMS Questionnaire Form or comparable document may be used, as long as responses to each Question 1-33 are provided.

Other supporting documents (e.g., Operations Manual, Standard Operating Procedure (SOP), records, evidence samples) may be required to support Questionnaire responses. Documents must be clear, legible, and include identifiable information (e.g., document name, revision history, company name).

### Volume IV, Past Performance

The vendor shall provide information about its past performance by completing the Reference Questionnaire in Section E, Exhibit E3 for referenced projects or contracts. Provide additional past performance information for work related to or of a similar nature as required by this solicitation. The Government reserves the right to reject any proposals that fail to submit information on all relevant contracts and/or subcontracts for the past 3 years.

Period for Acceptance of Offers

The vendor agrees to hold the prices in its offer firm for 180 calendar days from the date specified for receipt of offers unless another time period is specified in an amendment to the solicitation.

Late Submissions, Modifications, Revisions, and Withdrawals of Offers

Late Submissions, Modifications, Revisions and Withdrawals of Offers are subject to the terms of FAR Clause 52.212-1(f), which is incorporated by reference, except that offers may be withdrawn in writing at any time before award is made.

# EVALUATION OF PROPOSALS

## Evaluation—Commercial Items (52.212-2 Nov 2021)

ADDENDUM TO FAR 52.212-2 EVALUATION—COMMERCIAL PRODUCTS AND COMMERCIAL SERVICES (Nov 2021)

This is a best value source selection conducted in accordance with FAR 15.3, Source Selection in conjunction with the procedures of FAR 12, Acquisition of Commercial Products and Commercial Services and FAR Part 15, Contracting By Negotiation. The Government intends to evaluate proposals and award a contract to the responsible offeror(s) whose proposal(s) conforming to the solicitation will be most advantageous to the Government, price and other factors considered. Therefore, the vendor’s initial proposal should contain the vendor’s best terms from a price and technical standpoint. The Government may reject any or all offers if such action is in the public interest; accept other than the lowest priced proposal; and waive informalities and minor irregularities in proposals received. As part of making the assessment, a best value analysis will be performed determining whether or not exceeding the minimum requirements at an associated price provides the best value to the Government.

Aircraft submitted for On-Call will be evaluated and award will be made on a lowest priced, technically acceptable basis in accordance with the evaluation criteria below.  Aircraft submitted for Exclusive Use will be evaluated at proposal submission for the non-price factors, however award on Exclusive Use will be made at a later date utilizing trade-off procedures described in a subsequent TORP Letter.

A written notice of award or acceptance of a proposal, mailed or otherwise furnished to the successful offeror within the time for acceptance specified in the proposal, shall result in a binding contract without further action by either party. Before the offeror’s specified expiration time, the Government may accept a proposal (or part of a proposal), whether or not there are negotiations after its receipt, unless a written notice of withdrawal is received before award.

## Evaluation Factors

The Government will evaluate offers and select the contractor based on the following evaluation factors:

Technical Capability

Organizational Safety

Past Performance

Price

### On Call Aircraft

Assigning Ratings. The Technical Capability Factor and the Organizational Safety Factor will be assigned one of the ratings defined below.

Technical Capability Ratings

| Rating | Description |
| --- | --- |
| Acceptable | Technical Capability meets the minimum requirements |
| Unacceptable | Technical Capability does not meet the minimum requirements |

Organizational Safety Ratings

| Rating | Description |
| --- | --- |
| Acceptable | Proposal meets all requirements for Safety Management System (SMS) Components identified in the Questionnaire in Exhibit 7. |
| Unacceptable | Proposal does not meet all requirements Safety Management System (SMS) Components identified in the Questionnaire in Exhibit 7. |

#### Technical Capability

The Government technical evaluation team shall evaluate each offeror’s proposed aircraft against the specifications listed in the solicitation. The technical evaluation team shall determine whether the aircraft meets the minimum standards set forth in the solicitation. Aircraft that meet or exceed the standards shall be deemed acceptable, while those that do not shall be deemed unacceptable and removed from further consideration.

The aircraft make and model proposed that the Government confirms with the offeror will be made a part of the offer and will be binding if an offer is accepted for award.

#### Organizational Safety

Safety Management System (SMS), Flight Time, Accidents, and FAA Violations. The Government will evaluate the Offeror’s response to the SMS Questionnaire (Exhibit 7), which includes SMS requirements, flight time, aircraft accidents, and FAA violations. Accidents include under all Offeror operating certificates, ownership, public aircraft operations, and CFR Part 91. The total number of flight hours provided by the Offeror will be used, as well as any data resulting from the Government’s own experiences with the Offeror. Failure to provide documentation listed above is considered unacceptable and may remove the offer from further consideration.

#### Past Performance

The purpose of the past performance evaluation is to allow the Government to assess the offeror’s ability to safely and efficiently perform the effort described in this RFQ, based on the offeror’s demonstrated present and past performance. The offeror may provide information on problems encountered on identified contracts and the offeror’s corrective actions. The Government shall consider this information, as well as information obtained from any other sources, including any information on the offeror contained in the CPARS, when evaluating the offeror’s past performance.

The Contracting Officer shall seek recent and relevant performance information. Information older than the last three (3) year period is not desired unless it is particularly pertinent to offeror current capability. If the information provided is determined to be recent by the evaluation team the offeror’s past performance will be evaluated for relevance. Relevant contracts include performance of efforts involving aviation support efforts in environments that are similar in scope, magnitude and complexity than the effort described in this solicitation.

The Government may contact some or all of the Offeror's customers to ask whether or not they believe:

(1) that the Offeror was capable, efficient, and effective;

(2) that the Offeror’s performance conformed to the terms and conditions of its contract;

(3) that the Offeror was reasonable and cooperative during performance; and

(4) that the Offeror was committed to customer satisfaction.

Information older than the last three (3)-year period will not be evaluated.

If the past performance information provided by the Offeror is determined to be recent and relevant a

Past Performance Rating will be assigned as follows:

| Rating | Definition |
| --- | --- |
| Acceptable | Based on the offeror’s performance record, the Government has a reasonable expectation that the offeror will successfully perform the required effort, or the offeror’s performance record is unknown. |
| Unacceptable | Based on the offeror’s performance record, the Government has no reasonable expectation that the offeror will be able to successfully perform the required effort. |

In the case of an offeror without a record of relevant past performance or for whom information on past performance is not available or so sparse that no meaningful past performance rating can be reasonably assigned, the offeror may not be evaluated favorably or unfavorably on past performance (see FAR 15.305 (a)(2)(iv)). Therefore, the offeror shall be determined to have unknown past performance. In the context of acceptability/unacceptability, “unknown” shall be considered “acceptable.”

If the contracting officer determines that a small business’ past performance is not acceptable, the matter shall be referred to the Small Business Administration for a Certificate of Competency determination, in accordance with the procedures contained in subpart 19.6 and 13 C.F.R. § 125.5.

#### Price

The Price Submission will not be scored or rated. The techniques and procedures described under FAR 15.404 will be the primary means of conducting price analysis to determine price reasonableness. To be viable for award, an Offeror’s price must be determined fair and reasonable by application of the techniques described above.

The Government will compare rates proposed for Daily Availability and Flight Time included in the Offeror’s Price Submission. Award will be made to Offerors’ whose prices are determined fair and reasonable by comparison with other offerors’ proposed rates across all ordering periods.

As part of price evaluation, the government will evaluate its option to extend services (see FAR Clause 52.217-8) by evaluating half of the Offerors’ final ordering year Daily Availability and Flight Time rates and determining them fair and reasonable. Offerors are not required to enter a rates for the six-month period.

Unbalanced Pricing may be evaluated in accordance with FAR 15.404-1(g), as applicable, to assess potential performance risk for unreasonably high prices and provide for the elimination of a proposal from further consideration for award.

The Government reserves the right to conduct a price realism analysis in accordance with FAR 15.404-1.

#### Basis for Award

The Contractor will be selected on the basis of the Government’s evaluation of the proposal against the evaluation factors set forth in the solicitation: Technical Capability, Organizational Safety, Past Performance, and Price.

The Government intends to make multiple IDIQ Contract awards in response to this solicitation to offeror(s) that are determined to be acceptable under the non-price evaluation factors (Technical Capability, Organizational Safety and Past Performance), with pricing determined to be fair and reasonable.

### Exclusive Use Aircraft

Only non-price factors will be evaluated for Exclusive Use Aircraft.  Prices will be requested via TORP and such TORP Letters will identify the relative importance of the factors.

The Technical Capability Factor and the Organizational Safety Factor will be assigned one of the following adjectival ratings:

#### Technical Capability Factor Adjectival Ratings

| Rating | Definition |
| --- | --- |
| Exceptional | Proposal meets all requirements and presents substantial benefits identified by a combination of strengths or a significant strength(s) that outweigh any weaknesses, if any identified. No deficiencies or significant weakness found. |
| Very Good | Proposal meets all requirements and presents benefits identified by a combination of strengths or significant strength(s) that outweigh any weaknesses, if any identified. No deficiencies are found. If weaknesses or significant weaknesses are present, they are outweighed by the evaluated benefits. |
| Satisfactory | Proposal meets all requirements and any strengths or weaknesses identified are offsetting. No deficiencies found. |
| Unsatisfactory | Proposal does not meet all requirements and/or has a combination of weaknesses, significant weaknesses and/or deficiencies that outweigh any identified strengths. |

#### Organizational Safety Adjectival Ratings

| Rating | Definition |
| --- | --- |
| Exceptional | Proposal meets all requirements and presents substantial benefits identified by a combination of strengths or a significant strength(s) that outweigh any weaknesses, if any identified. No deficiencies or significant weakness found. Expected demonstration of SMS concepts during the period of performance is high. |
| Very Good | Proposal meets all requirements and presents benefits identified by a combination of strengths or significant strength(s) that outweigh any weaknesses, if any identified. No deficiencies are found. If weaknesses or significant weaknesses are present, they are outweighed by the evaluated benefits. Expected demonstration of SMS concepts during the period of performance is considerable. |
| Satisfactory | Proposal meets all requirements and any strengths or weaknesses identified are offsetting. No deficiencies found. Expected demonstration of SMS concepts during the period of performance is moderate. |
| Unsatisfactory | Proposal does not meet all requirements and/or has a combination of weaknesses, significant weaknesses and/or deficiencies that outweigh any identified strengths. Expected demonstration of SMS concepts during the period of performance is low. |

Technical and Organizational Safety Factors will be assigned a rating of “exceptional, very good, satisfactory and unsatisfactory.”  In order to be considered for award, a minimum rating of “satisfactory” or above must be achieved on all non-price evaluation criteria.

#### Technical Capability

Minimum aircraft requirements. The vendor must propose an aircraft that meets or exceeds minimum aircraft requirements as specified in Part A of this solicitation. The proposal will be unacceptable if the proposed aircraft fails to meet any of the minimum aircraft requirements specified in Section A of this solicitation.

The Government will evaluate the technical capability of each proposed aircraft using the requirements of the solicitation and completed exhibits with the proposal. The aircraft make and model with the proposal will be made a part of the contract and will be binding if it is accepted for award. Any missing, vague or information of poor quality in the proposal will be considered high risk and may be rated as unsatisfactory.

#### Organizational Safety

Safety Management System (SMS).

The Government will evaluate the Offeror’s written submittal in response to the Safety Management System (SMS) Questionnaire in Exhibit 4. A more favorable rating will result within this area to Offerors that demonstrate implemented FAA and IS-BAO SMS standards listed in the exhibit.

Flight Time, Accidents, and FAA Violations

The Government will evaluate any known aircraft accidents and FAA violations. Accidents include under all Offeror operating certificates, ownership, public aircraft operations, and CFR Part 91. The total number of flight hours provided by the Offeror will be used, as well as any data resulting from the Government’s own experiences with the Offeror. A more favorable safety component rating will result within this area to Offerors that possess an accident rate less than the agency five-year average, have fewer FAA violations, and satisfy the components in Exhibit 4. If accident(s) occurred, the submission of an accident prevention plan may positively impact the offer. Failure to provide documentation listed above is considered unacceptable and may remove the offer from further consideration.

#### Past Performance

The purpose of the past performance evaluation is to allow the Government to assess the offeror’s ability to effectively and efficiently perform the effort described in this RFP, based on the offeror’s demonstrated present and past performance. The offeror may provide information on problems encountered on identified contracts and the offeror’s corrective actions. The Government shall consider this information, information submitted with the offer, as well as information obtained from any other sources, including information contained in the Contractor Performance Assessment Reporting System (CPARS) database located on-line at https://www.cpars.gov/ when evaluating the offeror’s past performance.

The Contracting Officer shall seek recent and relevant performance information. Information older than the last three (3) year period will not be considered recent for the purposes of this evaluation. If the information provided is determined to be recent by the evaluation team the offeror’s past performance will be evaluated for relevance. Relevant contracts include performance of efforts involving aviation support efforts that are similar in scope, magnitude and complexity to the effort described in this solicitation.

If the past performance is both recent and relevant, the Government will review the past performance information collected and determine the quality of the offer’s performance, general trends, and usefulness of the information and incorporate these into the performance confidence assessment.

At the end of the performance confidence assessment, the offeror shall be assigned one of the ratings in the table below:

| Confidence Rating | Description |
| --- | --- |
| Substantial Confidence | Based on the offeror’s recent/relevant performance record, the Government has a high expectation that the offeror will successfully perform the required effort. |
| Satisfactory Confidence | Based on the offeror’s recent/relevant performance record, the Government has a reasonable expectation that the offeror will successfully perform the required effort. |
| Neutral Confidence | No recent/relevant performance record is available or the offeror’s performance record is so sparse that no meaningful confidence assessment rating can be reasonably assigned. |
| Limited Confidence | Based on the offeror’s recent/relevant performance record, the Government has a low expectation that the offeror will successfully perform the required effort. |
| No Confidence | Based on the offeror’s recent/relevant performance record, the Government has no expectation that the offeror will be able to successfully perform the required effort. |

The Government will give greater weight to proposals with where past performance is similar in scope, magnitude and complexity to what is described in this solicitation. The Government will also give greater weight to its own experiences with the offeror, if any, than it will give to reports obtained from others. Offerors with no relevant past or present performance history shall receive a neutral rating.

# SECTION E – CONTRACT PROVISIONS AND SOLICITATION ATTACHMENTS

52.252-1 Contract Provisions Incorporated by Reference (FEB 1998)

This solicitation incorporates one or more solicitation provisions by reference, with the same force and effect as if they were given in full text. Upon request, the Contracting Officer will make their full text available. The offeror is cautioned that the listed provisions may include blocks that must be completed by the offeror and submitted with its quotation or offer. In lieu of submitting the full text of those provisions, the offeror may identify the provision by paragraph identifier and provide the appropriate information with its quotation or offer. Also, the full text of a solicitation provision may be accessed electronically at this/these address(es): https://www.acquisition.gov/

(End of Provision)

## PROVISIONS INCORPORATED BY REFERENCE

52.203-11	Certification and Disclosure Regarding Payments to Influence Certain		Sep 2024

Federal Transactions

52.203-18	Prohibition on Contracting with Entities that Require Certain Internal		Jan 2017

Confidentiality Agreements or Statements – Representation

52.204-7 	System for Award Management			 			Nov 2024

52.207-6		Solicitation of Offers from Small Business Concerns and Small Business	Aug 2024

Teaming Arrangements or Joint Ventures (Multiple Award Contracts)

52.209-12	Certification Regarding Tax Matters					Oct 2025

52.212-1 	Instructions to Offerors – Commercial Products and Commercial Services 	Sep 2023

52.212-2		Evaluation – Commercial Products and Commercial Services			Nov 2021

52.216-27	Single or Multiple Awards							Oct 1995

52.219-31	Notice of Small Business Reserve						Mar 2020

## Provisions Incorporated by Full Text

52.216-1 Type of Contract (APR 1984)

The Government contemplates award of multiple indefinite-delivery indefinite-quantity (IDIQ) contracts resulting from this solicitation from which Firm-Fixed-Price and Time-and-Material type task orders will be issued.

(End of Provision)

AQD Evaluation of Options Provision (MAR 2022)

The Government will evaluate offers for award purposes by adding the total price for all options to the total price for the basic award. To account for the possible use of the additional six-month option period(s) permitted under 52.217-8, the Government will take the price for the final (or highest priced) option period, prorated to a six-month value, and add it to the sum of the base plus all option periods. The agency's price evaluation will mathematically document the calculation of adding the six months of services for the option to extend services to the total contract price in the award summary.

This amount will be used for comparative evaluation for award purposes only. The Government may determine that an offer is unacceptable if the option prices are significantly unbalanced. Evaluation of options shall not obligate the Government to exercise the options.

FAR 52.217-8 can be exercised at any point during the period of performance of the contract and applies to the period of performance immediately preceding exercising the option.

(End of Provision)

# SECTION E EXHIBITS

E-1 Offeror’s Miscellaneous Information

E-2 Aircraft Questionnaire

E-3 Aircraft Information Form

E-4 Past Performance Questionnaire

E-5 Pricing Sheet

E-6 Wage Determinations

# Exhibit E-1
Offeror’s Miscellaneous Information

| Offeror’s Company Name |  |  |  |
| --- | --- | --- | --- |
| Offeror’s System for Award Management Unique Entity ID (UEI) |  |  |  |
| Offeror’s Name, Title and E-mail Address |  |  |  |
| Offeror’s Company Web Address |  |  |  |
| Offeror’s Office Telephone Number |  |  |  |
| Offeror’s Facsimile (FAX) Number |  |  |  |
| Government Contract Representative | Name | Email Address | Telephone (cell) |
| *Director of Operations | Name | Email Address | Telephone (cell) |
| *Chief Pilot | Name | Email Address | Telephone (cell) |
| *Director of Maintenance | Name | Email Address | Telephone (cell) |
| Invoicing/AIRS | Name | Email Address | Telephone (cell) |

*Note. All of the people named above will be verified against the Excluded Parties List for Government Contracting. The contractor must also notify the Contracting Officer of any changes in the Director of Operations, Chief Pilot, and Director of Maintenance positions, plus any additional positions approved under 14 CFR 119.69(b).

# Exhibit E-2
Aircraft Questionnaire

You may recreate this form, but it must include all the information listed.

This questionnaire accurately completed will be used to determine the proposed aircraft payload capability.

COMPLETE SHADED BLOCKS

| Minimum Aircraft Requirements: Must have a Standard Airworthiness Certificate. Aircraft certified under 14 CFR Part 23 or 25, or CAR 3. An aircraft make and model for which engineering and logistical support, for continued airworthiness, is provided from the current type certificate holder or supplemental type certificate holder. Certified for VFR/IFR, Day/Night. Certified for Flight Into Known Icing. Aircraft must have environmental controls including heating and air conditioning. Insured seating for three (3) passengers which includes the right front seat located in the cockpit. Removable seats for cargo configuration. Cruise airspeed 160 knots true airspeed at 8,000 feet pressure altitude and temperature ISA+20ºC. Cruise airspeed 220 knots true airspeed at 8,000 feet pressure altitude and temperature ISA+20ºC for Exclusive Use. Endurance of 4.0 hours of fuel and 30-minute fuel reserve at 8,000 feet pressure altitude and temperature ISA +20°C. Aircraft must be certified for single pilot operations. Visibility: No external obstructions within the field of regard, 9 o’clock to 3 o’clock, as viewed from the cockpit and minimal restrictions to outward visibility of the ground in level flight for the rear observers Payload of 900 pounds with fuel for 250 nautical mile flight, VFR fuel reserve, at 8,000 feet pressure altitude, temperature ISA+20°C, 200-pound pilot and 25-pound survival kit. Multi Engine additional requirements: Cruise airspeed minimum 220 knots true airspeed at 8,000 feet pressure altitude, ISA+20ºC. Pressurized aircraft with environmental controls, heater and air conditioner. Positive single engine rate of climb when equipped for this contract with a 200-pound pilot, one 220-pound observer, 4 hours of fuel plus a 30- minute reserve, at 5,000 feet pressure altitude, ISA+33º C. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Aircraft Make and Model | FAA Registration # |  | Serial # | Cruise Airspeed |  |  |
| Aircraft Equipped Weight (Your aircraft weight & balance report & equipment list will be used to substantiate this weight – see note 1. below) |  |  |  |  |  |  |
| Currently Installed Aircraft Equipment to be Removed to Achieve Offered Payload Below (If none, indicate NONE) |  |  | Equipment to be Added to Meet the Aircraft Specifications Requirements (If none, indicate NONE) |  |  |  |
| EQUIPMENT ITEM |  | WEIGHT | EQUIPMENT ITEM |  | WEIGHT |  |
| ENTER YOUR PROPOSED AIRCRAFT PAYLOAD (The aircraft make, model and payload that we confirm with you will be made a part of your offer and will be binding if your offer is accepted for award |  |  |  |  |  |  |

Note: By signing below, I acknowledge that I have included the following documentation to support the minimum aircraft requirements in Sections A and B:

(1) Current aircraft weight and balance report, as specified in section B32 Weight and Balance

(2) Aircraft equipment list certified accurate at the time of weighing and any revisions to the list,

(3) Applicable Performance Charts

(4) Calculations showing performance characteristics of the aircraft. Calculations must include weight of the aircraft in contract configuration, crew, and fuel as identified in Section A1. (use 6.7 lbs. per gal. for fuel weight)

(5) 14 CFR Part 135 Certificate

Failure to include the above information, will affect the evaluation you receive for your aircraft.

By signing below, I certify to the best of my knowledge that the above information is accurate.

| Name and Signature of Representative Completing this Form | Date |
| --- | --- |
| OFFEROR’S NAME |  |

# Exhibit E-3
Aircraft Information Form

The proposal may recreate this form, but it must include all the information listed

| Offeror’s Name: |  |  |  |
| --- | --- | --- | --- |
| Aircraft Make and Model: |  | Aircraft Tail Number: |  |

| Cruise airspeed |  | knots |  |
| --- | --- | --- | --- |
| (8,000’ PA and ISA +20°C or (12,000’ PA and ISA +20°C) |  |  |  |
| Payload |  | Lbs. |  |
| (with 200lb per required pilot, 35lb Survival Kit, and 3 hours of fuel at 8,000’ PA and 20°C) |  |  |  |
| Endurance |  | Hrs. |  |
| (8,000’ PA and ISA +20°C) |  |  |  |
| Number of Insured Passenger Seats |  |  |  |

Minimum Aircraft Requirements:

| Positive single engine rate of climb (5,000’ PA and ISA +33°C) (Multi only) (with 200 lb. pilot, 25 lb. survival kit, 220 lb. observer, & 4.5 hrs. of fuel) | Yes | No |
| --- | --- | --- |
| Aircraft certified under 14 CFR 23, or 25 | Yes | No |
| Aircraft certified under CAR 3 | Yes | No |
| Approved for gravel runways (Alaska dispatch only) | Yes | No |
| VFR/IFR, Day/Night, Known Ice. | Yes | No |
| Pressurized aircraft with environmental controls. Heater and air conditioner | Yes | No |
| Rear ATGS training station (full audio panel) | Yes | No |
| Rear transmit capability | Yes | No |
| EFB charging capability | Yes | No |
| Pressurized cockpit and cabin (Multi only) | Yes | No |
| Removable seats for cargo configuration | Yes | No |
| Aircraft Certified for single pilot operations (TCAS/TCAD/TAS) TIS & ADS-B do not satisfy this requirement | Yes | No |
| Environmental controls including heating and air conditioning. | Yes | No |
| Single Pilot Certified | Yes | No |
| Insured seating for three (3) passengers which includes the right front seat located in the cockpit. | Yes | No |
| No external obstructions within the field of regard, 9 o’clock to 3 o’clock, as viewed from the cockpit and minimal restrictions to outward visibility of the ground in level flight for the rear observers. | Yes | No |

Describe any visibility enhancements for the Government observers that your aircraft may have.

The following equipment requirements from Sections B6 and B7 must be identified on the aircraft equipment list, weight and balance revision, or aircraft questionnaire and be included in the aircraft weight for performance calculations.

# Exhibit E-4
Past Performance Questionnaire

You may recreate this form, but it must include all the information listed. If you fail to include all requested information or we are unable to contact your references based on the information you provide, you may not receive consideration for the work done. Include recent (not more than 3 years old) experience of the kind described in paragraph B1.1 of the specifications.

| OFFEROR NAME: |  |  |  |  | DATE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Primary Type of Work Done: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location of Work Done: |  |  |  | A/C Make/Model Used: |  |  |  |  |  |  |  |  |  |
| Contract #: ☐ None |  | ☐ Exclusive Use ☐ Call When Needed |  |  |  |  | Use Dates: TO |  |  |  |  |  | CHECK ALL THAT APPLY ☐ 2022 ☐2023 ☐2024 |
| Est. Annual Value of Work: |  | Estimated Annual Flight Hours: |  |  |  |  |  |  | Client Name/ Point of Contact: |  |  |  |  |
| Client Telephone #: |  |  |  |  |  |  |  |  | Client Facsimile #: |  |  |  |  |
| Primary Type of Work Done: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location of Work Done: |  |  |  |  |  |  |  |  |  |  | A/C Make/Model Used: |  |  |
| Contract #: ☐ None |  | ☐ Exclusive Use ☐ Call When Needed |  |  |  |  | Use Dates: TO |  |  |  |  |  | CHECK ALL THAT APPLY ☐ 2022 ☐2023 ☐2024 |
| Est. Annual Value of Work: |  | Estimated Annual Flight Hours: |  |  |  |  |  |  | Client Name/ Point of Contact: |  |  |  |  |
| Client Telephone #: |  |  |  |  |  |  |  |  | Client Facsimile #: |  |  |  |  |
| Primary Type of Work Done: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location of Work Done: |  |  |  |  |  |  |  |  |  |  | A/C Make/Model Used: |  |  |
| Contract #: ☐ None |  | ☐ Exclusive Use ☐ Call When Needed |  |  |  |  | Use Dates: TO |  |  |  |  |  | CHECK ALL THAT APPLY ☐ 2022 ☐2023 ☐2024 |
| Est. Annual Value of Work: |  | Estimated Annual Flight Hours: |  |  |  |  |  |  | Client Name/ Point of Contact: |  |  |  |  |
| Client Telephone #: |  |  |  |  |  |  |  |  | Client Facsimile #: |  |  |  |  |
| Primary Type of Work Done: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location of Work Done: |  |  |  |  |  |  |  |  |  | A/C Make/Model Used: |  |  |  |
| Contract #: ☐ None |  |  | ☐ Exclusive Use ☐ Call When Needed |  |  |  |  | Use Dates: TO |  |  |  | CHECK ALL THAT APPLY ☐ 2022 ☐2023 ☐2024 |  |
| Est. Annual Value of Work: | Estimated Annual Flight Hours: |  |  |  |  | Client Name/ Point of Contact: |  |  |  |  |  |  |  |
| Client Telephone #: |  |  |  |  |  | Client Facsimile #: |  |  |  |  |  |  |  |

# Exhibit E-5
Pricing Sheet

Attached in SAM

# Exhibit E-6
Wage Determinations

REGISTER OF WAGE DETERMINATIONS UNDER THE SERVICE CONTRACT ACT

By direction of the Secretary of Labor

Daniel W. Simms			Division of

Director				Wage Determination

U.S. DEPARTMENT OF LABOR EMPLOYMENT STANDARDS ADMINISTRATION

WAGE AND HOUR DIVISION

WASHINGTON, D.C. 20210

Wage Determination No.: 1995-0021

Revision No.: 73

Date of Last Revision: 1/15/2026

NATIONWIDE: Applicable in the continental U.S., Hawaii, Alaska, and American Samoa. Alaska: Entire state.

American Samoa: Entire state Hawaii: Entire state.

Midwestern Region: Illinois, Indiana, Iowa, Kansas, Michigan, Minnesota, Missouri, Nebraska, North Dakota, Ohio, South Dakota, Wisconsin

Northeast Region: Connecticut, Maine, Massachusetts, New Hampshire, New Jersey, New York, Pennsylvania, Rhode Island, Vermont

Southern Region: Alabama, Arkansas, Delaware, District of Columbia, Florida, Georgia, Kentucky, Louisiana, Maryland, Mississippi, North Carolina, Oklahoma, South Carolina, Tennessee, Texas, Virginia, West Virginia

Western Region: Arizona, California, Colorado, Idaho, Montana, Nevada, New Mexico, Oregon, Utah, Washington, Wyoming

** Fringe Benefits Required Follow the Occupational Listing **

Employed on contracts for Fire Safety services only.

| CODE | OCCUPATION TITLE | FOOTNOTE | RATE |
| --- | --- | --- | --- |
| 01000 | Administrative Support And Clerical Occupations |  |  |
| 01070 | Document Preparation Clerk: Alaska | (see 1) | 24.14 |
|  | Hawaii and American Samoa |  | 20.59 |
|  | Midwestern Region |  | 18.75 |
|  | Northeast Region |  | 19.23 |
|  | Southern Region |  | 18.51 |
|  | Western Region |  | 18.86 |
| 01090 | Duplicating Machine Operator: Alaska | (see 1) | 24.14 |
|  | Hawaii and American Samoa |  | 20.59 |
|  | Midwestern Region |  | 18.75 |
|  | Northeast Region |  | 19.23 |
|  | Southern Region |  | 18.51 |
|  | Western Region |  | 18.86 |

| 01613 | Word Processor III: | (see 2) |  |
| --- | --- | --- | --- |
|  | Alaska |  | 26.51 |
|  | Continental U.S. |  | 26.51 |
|  | Hawaii and American Samoa |  | 26.21 |

05000	Automotive Service Occupations

| 05190 | Motor Vehicle Mechanic: Alaska | (see 2) | 37.44 |
| --- | --- | --- | --- |
|  | Hawaii and American Samoa |  | 24.85 |
|  | Midwestern Region |  | 29.54 |
|  | Northeast Region |  | 27.73 |
|  | Southern Region |  | 25.83 |
|  | Western Region |  | 29.89 |
| 05220 | Motor Vehicle Mechanic Helper: Alaska | (see 2) | 27.08 |
|  | Hawaii and American Samoa |  | 19.13 |
|  | Midwestern Region |  | 19.14 |
|  | Northeast Region |  | 21.60 |
|  | Southern Region |  | 16.65 |
|  | Western Region |  | 20.24 |
| 07000 | Food Preparation And Service Occupations |  |  |
| 07010 | Baker: Alaska | (see 2) | 22.49 |
|  | Hawaii and American Samoa |  | 22.45 |
|  | Midwestern Region |  | 18.85 |
|  | Northeast Region |  | 21.38 |
|  | Southern Region |  | 15.38 |
|  | Western Region |  | 23.46 |
| 07041 | Cook I: Alaska | (see 2) | 19.38 |
|  | Hawaii and American Samoa |  | 18.90 |
|  | Midwestern Region |  | 13.85 |
|  | Northeast Region |  | 17.31 |
|  | Southern Region |  | 13.19 |
|  | Western Region |  | 15.78 |
| 07042 | Cook II: Alaska | (see 2) | 22.31 |
|  | Hawaii and American Samoa |  | 21.11 |
|  | Midwestern Region |  | 15.62 |
|  | Northeast Region |  | 19.48 |
|  | Southern Region |  | 14.86 |
|  | Western Region |  | 17.82 |
| 07070 | Dishwasher: Alaska | (see 2) | 16.59 |
|  | Hawaii and American Samoa |  | 18.15 |
|  | Midwestern Region |  | 11.21 |
|  | Northest Region |  | 12.03 |
|  | Southern Region |  | 11.65 |

|  | Western Region |  | 12.09 |
| --- | --- | --- | --- |
| 07130 | Food Service Worker: Alaska | (see 2) | 16.92 |
|  | Hawaii and American Samoa |  | 16.83 |
|  | Midwestern Region |  | 13.23 |
|  | Northeast Region |  | 15.85 |
|  | Southern Region |  | 12.52 |
|  | Western Region |  | 13.76 |
| 07210 | Meat Cutter: Alaska | (see 2) | 27.59 |
|  | Hawaii and American Samoa |  | 26.79 |
|  | Midwestern Region |  | 23.25 |
|  | Northeast Region |  | 27.08 |
|  | Southern Region |  | 19.41 |
|  | Western Region |  | 25.40 |

11000	General Services And Support Occupations

| 11150 | Janitor: | (see 2) |  |
| --- | --- | --- | --- |
|  | Alaska |  | 19.43 |
|  | Hawaii and American Samoa |  | 18.47 |
|  | Midwestern Region |  | 17.65 |
|  | Northeast Region |  | 19.12 |
|  | Southern Region |  | 15.75 |
|  | Western Region |  | 18.15 |

12000	Health Occupations

| 12040 | Emergency Medical Technician: | (see 2) |  |
| --- | --- | --- | --- |
|  | Alaska |  | 32.35 |
|  | Continental U.S. |  | 23.56 |
|  | Hawaii and American Samoa |  | 26.51 |

21000	Materials Handling And Packing Occupations

| 21020 | Forklift Operator: Alaska | (see 2) | 31.08 |
| --- | --- | --- | --- |
|  | Hawaii and American Samoa |  | 24.21 |
|  | Midwestern Region |  | 22.35 |
|  | Northeast Region |  | 21.83 |
|  | Southern Region |  | 18.47 |
|  | Western Region |  | 23.90 |
| 21150 | Stock Clerk: Alaska | (see 2) | 20.07 |
|  | Hawaii and American Samoa |  | 15.83 |
|  | Midwestern Region |  | 17.74 |
|  | Northeast Region |  | 17.50 |
|  | Southern Region |  | 17.01 |
|  | Western Region |  | 17.97 |

23000 	Mechanics And Maintenance And Repair Occupations

| 23021 | Aircraft Mechanic I: |  | (see 2) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Alaska |  |  |  |  |  |  | 39.41 |  |
|  | Continental U.S. |  |  |  |  |  |  | 40.54 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 40.75 |  |
| 23022 | Aircraft Mechanic II: Alaska |  | (see 2) |  |  |  |  | 40.90 |  |
|  | Continental U.S. |  |  |  |  |  |  | 41.59 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 42.40 |  |
| 23023 | Aircraft Mechanic III: Alaska |  | (see 2) |  |  |  |  | 43.17 |  |
|  | Continental U.S. |  |  |  |  |  |  | 43.46 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 44.48 |  |
| 23040 | Aircraft Mechanic Helper: Alaska |  | (see 2) |  |  |  |  | 30.91 |  |
|  | Continental U.S. |  |  |  |  |  |  | 30.48 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 29.41 |  |
| 23060 | Aircraft Servicer: Alaska |  | (see 2) |  |  |  |  | 34.53 |  |
|  | Continental U.S. |  |  |  |  |  |  | 34.77 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 34.14 |  |
| 23160 | Electrician, Maintenance: Alaska |  | (see 2) |  |  |  |  | 44.47 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 38.34 |  |
|  | Midwestern Region |  |  |  |  |  |  | 33.40 |  |
|  | Northeast Region |  |  |  |  |  |  | 35.81 |  |
|  | Southern Region |  |  |  |  |  |  | 28.57 |  |
|  | Western Region |  |  |  |  |  |  | 34.12 |  |
| 23440 | Heavy Equipment Operator: Alaska |  | (see 2) |  |  |  |  | 36.40 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 25.89 |  |
|  | Midwestern Region |  |  |  |  |  |  | 29.54 |  |
|  | Northeast Region |  |  |  |  |  |  | 27.73 |  |
|  | Southern Region |  |  |  |  |  |  | 25.83 |  |
|  | Western Region |  |  |  |  |  |  | 29.88 |  |
| 23470 | Laborer: Alaska |  | (see 2) |  |  |  |  | 22.09 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 21.43 |  |
|  | Midwestern Region |  |  |  |  |  |  | 17.97 |  |
|  | Northeast Region |  |  |  |  |  |  | 18.11 |  |
|  | Southern Region |  |  |  |  |  |  | 14.62 |  |
|  | Western Region |  |  |  |  |  |  | 16.98 |  |
| 23530 | Machinery Maintenance Mechanic: Alaska |  | (see 2) |  |  |  |  | 41.53 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 40.99 |  |
|  | Midwestern Region |  |  |  |  |  |  | 25.61 |  |
|  | Northeast Region |  |  |  |  |  |  | 26.79 |  |
|  | Southern Region |  |  |  |  |  |  | 20.30 |  |
|  | Western Region |  |  |  |  |  |  | 25.41 |  |
| 23580 | Maintenance Trades Helper: Alaska |  | (see 2) |  |  |  |  | 30.36 |  |
|  |  | Hawaii and American Samoa |  |  |  |  |  | 23.41 |  |
|  |  | Midwestern Region |  |  |  |  |  | 24.12 |  |
|  |  | Northeast Region |  |  |  |  |  | 22.51 |  |
|  |  | Southern Region |  |  |  |  |  | 20.28 |  |
|  |  | Western Region |  |  |  |  |  | 20.92 |  |
| 27000 |  | Protective Service Occupations |  |  |  |  |  |  |  |
| 27070 |  | Firefighter: Alaska |  |  |  |  | (see 2) | 32.56 |  |
|  |  | Hawaii and American Samoa |  |  |  |  |  | 33.00 |  |
|  |  | Midwestern Region |  |  |  |  |  | 25.91 |  |
|  |  | Northeast Region |  |  |  |  |  | 33.75 |  |
|  |  | Southern Region |  |  |  |  |  | 22.09 |  |
|  |  | Western Region |  |  |  |  |  | 29.74 |  |
| 27101 |  | Guard I: Alaska |  |  |  |  | (see 2) | 21.39 |  |
|  |  | Hawaii and American Samoa |  |  |  |  |  | 19.75 |  |
|  |  | Midwestern Region |  |  |  |  |  | 16.80 |  |
|  |  | Northeast Region |  |  |  |  |  | 28.10 |  |
|  |  | Southern Region |  |  |  |  |  | 16.27 |  |
|  |  | Western Region |  |  |  |  |  | 17.48 |  |
| 30000 |  | Technical Occupations |  |  |  |  |  |  |  |
| 30210 |  | Laboratory Technician: Alaska |  |  |  |  | (see 2) | 32.56 |  |
|  |  | Hawaii and American Samoa |  |  |  |  |  | 30.96 |  |
|  |  | Mid Western Region |  |  |  |  |  | 28.93 |  |
|  |  | Northeast Region |  |  |  |  |  | 26.78 |  |
|  |  | Southern Region |  |  |  |  |  | 29.44 |  |
|  |  | Western Region |  |  |  |  |  | 27.51 |  |
| 31000 |  | Transportation/Mobile Equipment Operation Occupations |  |  |  |  |  |  |  |
| 31030 |  | Bus Driver: Alaska |  |  |  |  | (see 2) | 30.53 |  |
|  |  | Hawaii and American Samoa |  |  |  |  |  | 19.83 |  |
|  |  | Midwestern Region: 1 1/2 to 4 tons |  |  |  |  |  | 25.17 |  |
|  |  | Midwestern Region: over 4 tons |  |  |  |  |  | 26.31 |  |
|  |  | Midwestern Region: under 1 1/2 tons |  |  |  |  |  | 18.84 |  |
|  |  | Northeast Region: 1 1/2 to 4 tons |  |  |  |  |  | 25.79 |  |
|  |  | Northeast Region: over 4 tons |  |  |  |  |  | 26.91 |  |
|  |  | Northeast Region: under 1 1/2 tons |  |  |  |  |  | 20.01 |  |
|  |  | Southern Region: 1 1/2 to 4 tons |  |  |  |  |  | 22.97 |  |
|  |  | Southern Region: over 4 tons |  |  |  |  |  | 23.87 |  |
|  |  | Southern Region: under 1 1/2 tons |  |  |  |  |  | 12.80 |  |
|  |  | Western Region: 1 1/2 to 4 tons |  |  |  |  |  | 23.69 |  |
|  |  | Western Region: over 4 tons |  |  |  |  |  | 24.35 |  |
|  |  | Western Region: under 1 1/2 tons |  |  |  |  |  | 14.89 |  |
| 31361 |  | Truckdriver, Light: |  |  |  |  | (see 2) |  |  |
|  |  | Alaska |  |  |  |  |  | 28.59 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 15.66 |  |
|  | Midwestern Region |  |  |  |  |  |  | 18.84 |  |
|  | Northeast Region |  |  |  |  |  |  | 20.01 |  |
|  | Southern Region |  |  |  |  |  |  | 12.80 |  |
|  | Western Region |  |  |  |  |  |  | 14.89 |  |
| 31362 | Truckdriver, Medium: |  |  | (see 2) |  |  |  |  |  |
|  | Alaska |  |  |  |  |  |  | 30.94 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 19.81 |  |
|  | Midwestern Region |  |  |  |  |  |  | 25.16 |  |
|  | Northeast Region |  |  |  |  |  |  | 25.83 |  |
|  | Southern Region |  |  |  |  |  |  | 22.91 |  |
|  | Western Region |  |  |  |  |  |  | 23.69 |  |
| 31363 | Truckdriver, Heavy: |  |  | (see 2) |  |  |  |  |  |
|  | Alaska |  |  |  |  |  |  | 32.71 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 21.62 |  |
|  | Midwestern Region |  |  |  |  |  |  | 26.31 |  |
|  | Northeast Region |  |  |  |  |  |  | 26.91 |  |
|  | Southern Region |  |  |  |  |  |  | 23.84 |  |
|  | Western Region |  |  |  |  |  |  | 25.25 |  |
| 31364 | Truckdriver, Tractor-Trailer: |  |  | (see 2) |  |  |  |  |  |
|  | Alaska |  |  |  |  |  |  | 34.46 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 21.89 |  |
|  | Midwestern Region |  |  |  |  |  |  | 31.28 |  |
|  | Northeast Region |  |  |  |  |  |  | 27.10 |  |
|  | Southern Regioon |  |  |  |  |  |  | 25.04 |  |
|  | Western Region |  |  |  |  |  |  | 25.76 |  |
| 47000 | Water Transportation Occupations |  |  |  |  |  |  |  |  |
| 47021 | Cook-Baker/Second Cook/Second Cook-Baker/Assistant Cook: |  |  | (see 2) |  |  |  |  |  |
|  | Alaska |  |  |  |  |  |  | 22.24 |  |
|  | Hawaii and American Samoa |  |  |  |  |  |  | 21.11 |  |
|  | Midwestern Region |  |  |  |  |  |  | 15.62 |  |
|  | Northeast Region |  |  |  |  |  |  | 19.48 |  |
|  | Southern Region |  |  |  |  |  |  | 14.85 |  |
|  | Western Region |  |  |  |  |  |  | 17.82 |  |
| 92000 | Non Standard Occupations |  |  |  |  |  |  |  |  |
| Quality Assurance Representative I: |  |  |  | (see 2) |  |  |  |  |  |
| Alaska |  |  |  |  |  |  |  | 27.62 |  |
| Hawaii and American Samoa |  |  |  |  |  |  |  | 28.38 |  |
| Midwestern Region |  |  |  |  |  |  |  | 24.86 |  |
| Northeast Region |  |  |  |  |  |  |  | 26.28 |  |
| Southern Region |  |  |  |  |  |  |  | 27.24 |  |
| Western Region |  |  |  |  |  |  |  | 25.06 |  |
| Quality Assurance Representative II: |  |  |  | (see 2) |  |  |  |  |  |
| Alaska |  |  |  |  |  |  |  | 36.15 |  |
| Hawaii and American Samoa |  |  |  |  |  |  |  | 33.73 |  |
| Midwestern Region |  |  |  |  |  |  |  | 30.67 |  |
| Northeast Region |  |  |  |  |  |  |  |  | 32.62 |
| Southern Region |  |  |  |  |  |  |  |  | 28.80 |
| Western Region |  |  |  |  |  |  |  |  | 30.38 |
| Quality Assurance Representative III: Alaska |  |  |  |  |  | (see 2) |  |  | 38.47 |
| Hawaii and American Samoa |  |  |  |  |  |  |  |  | 36.73 |
| Midwestern Region |  |  |  |  |  |  |  |  | 36.13 |
| Northeast Region |  |  |  |  |  |  |  |  | 38.38 |
| Southern Region |  |  |  |  |  |  |  |  | 34.08 |
| Western Region |  |  |  |  |  |  |  |  | 36.43 |
| Chief Cook: Alaska |  |  |  |  |  | (see 2) |  |  | 29.54 |
| Hawaii and American Samoa |  |  |  |  |  |  |  |  | 35.41 |
| Midwestern Region |  |  |  |  |  |  |  |  | 26.03 |
| Northeast Region |  |  |  |  |  |  |  |  | 31.53 |
| Southern Region |  |  |  |  |  |  |  |  | 23.88 |
| Western Region |  |  |  |  |  |  |  |  | 29.01 |
| Environmental Protection Specialist: Alaska |  |  |  |  |  | (see 2) |  |  | 46.60 |
| Hawaii and American Samoa |  |  |  |  |  |  |  |  | 43.09 |
| Midwestern Region |  |  |  |  |  |  |  |  | 39.20 |
| Northeast Region |  |  |  |  |  |  |  |  | 47.02 |
| Southern Region |  |  |  |  |  |  |  |  | 39.91 |
| Western Region |  |  |  |  |  |  |  |  | 41.28 |
| Fire Safety Professional: Alaska |  |  |  |  |  | (see 2) |  |  | 46.57 |
| Hawaii and American Samoa |  |  |  |  |  |  |  |  | 43.13 |
| Midwestern Region |  |  |  |  |  |  |  |  | 39.20 |
| Northeast Region |  |  |  |  |  |  |  |  | 47.02 |
| Southern Region |  |  |  |  |  |  |  |  | 39.91 |
| Western Region |  |  |  |  |  |  |  |  | 41.28 |
| Aircraft Quality Control Inspector: Alaska |  |  |  |  |  | (see 2) |  |  | 41.23 |
| Continental U.S. |  |  |  |  |  |  |  |  | 42.39 |
| Hawaii and American Samoa |  |  |  |  |  |  |  |  | 42.63 |
| 99000 Miscellaneous Occupations |  |  |  |  |  |  |  |  |  |

| 99730 | Refuse Collector: Alaska | (see 2) | 16.32 |
| --- | --- | --- | --- |
|  | Hawaii and American Samoa |  | 15.10 |
|  | Midwestern Region |  | 13.95 |
|  | Northeast Region |  | 15.91 |
|  | Southern Region |  | 10.97 |
|  | Western Region |  | 13.58 |

Note: Executive Order (EO) 13706, Establishing Paid Sick Leave for Federal Contractors, applies to all contracts subject to the Service Contract Act for which the contract is awarded (and any solicitation was issued) on or after January 1, 2017. If this contract is covered by the EO, the contractor must provide employees with 1 hour of paid sick leave for every 30 hours they work, up to 56 hours of paid sick leave each year. Employees must be permitted to use paid sick leave for their own illness, injury or other health-related

needs, including preventive care; to assist a family member (or person who is like family to the employee) who is ill, injured, or has other health-related needs, including preventive care; or for reasons resulting from, or to assist a family member (or person who is like family to the employee) who is the victim of, domestic violence, sexual assault, or stalking. Additional information on contractor requirements and worker protections under the EO is available at www.dol.gov/whd/govcontracts.

Note: Executive Order 13658 generally applies to contracts subject to the Service Contract Act that were awarded on or between January 1, 2015 and January 29, 2022, and that have not been renewed or extended on or after January 30, 2022. If a contract is subject to Executive Order 13658, the contractor must pay all covered workers at least $13.30 per hour (or the applicable wage rate listed on this wage determination, if it is higher) for all hours spent performing on the contract in 2025. The applicable Executive Order minimum wage rate will be adjusted annually. Additional information on contractor requirements and worker protections under Executive Order 13658 is available at www.dol.gov/whd/govcontracts.

ALL OCCUPATIONS LISTED ABOVE RECEIVE THE FOLLOWING BENEFITS:

HEALTH & WELFARE: $5.55 per hour, up to 40 hours per week, or $222.00 per week or $962.00 per month

HEALTH & WELFARE EO 13706: $5.09 per hour, up to 40 hours per week, or $203.60 per week, or $882.27 per month*

*This rate is to be used only when compensating employees for performance on an SCA-covered contract also covered by EO 13706, Establishing Paid Sick Leave for Federal Contractors. A contractor may not receive credit toward its SCA obligations for any paid sick leave provided pursuant to EO 13706.

HOLIDAYS: A minimum of eleven paid holidays per year: New Year's Day, Martin Luther King Jr.'s Birthday, Washington's Birthday, Memorial Day, Juneteenth National Independence Day, Independence Day, Labor Day, Columbus Day, Veterans' Day, Thanksgiving Day, and Christmas Day. (A contractor may substitute for any of the named holidays another day off with pay in accordance with a plan communicated to the employees involved.) (See 29 CFR 4.174)

VACATION (Hawaii): 2 weeks paid vacation after 1 year of service with a contractor or successor; 3 weeks after 10 years, and 4 weeks after 15 years. Length of service includes the whole span of continuous service with the present contractor or successor, wherever employed, and with the predecessor contractors in the performance of similar work at the same Federal facility. (Reg. 29 CFR 4.173)

HEALTH & WELFARE (Hawaii): $2.42 per hour, up to 40 hours per week, or $96.80 per week, or $419.47 per month for all employees on whose behalf the contractor provides health care benefits pursuant to the Hawaii prepaid Health Care Act. For those employees who are not receiving health care benefits mandated by the Hawaii prepaid Health Care Act, the new health and welfare benefit rate will be $5.55 per hour, up to 40 hours per week.

HEALTH & WELFARE (Hawaii EO 13706): $1.96 per hour, up to 40 hours per week, or $78.40 per week, or

$339.73 per month for all employees on whose behalf the contractor provides health care benefits pursuant to the Hawaii prepaid Health Care Act. For those employees who are not receiving health care benefits mandated by the Hawaii prepaid Health Care Act, the new health and welfare benefit rate will be $5.09 per hour, up to 40 hours per week. *

*This rate is to be used only when compensating employees for performance on an SCA-covered contract also covered by EO 13706, Establishing Paid Sick Leave for Federal Contractors. A contractor may not receive credit toward its SCA obligations for any paid sick leave provided pursuant to EO 13706.

THE OCCUPATIONS WHICH HAVE NUMBERED FOOTNOTES IN PARENTHESES RECEIVE THE FOLLOWING:

- VACATION: 2 weeks paid vacation after 1 year of service with a contractor or successor, 3 weeks after 5 years, and 4 weeks after 15 years. Length of service includes the whole span of continuous service with the present contractor or successor, wherever employed, and with the predecessor contractors in the performance of similar work at the same Federal facility. (Reg. 29 CFR 4.173)

VACATION: 2 weeks paid vacation after 1 year of service with a contractor or successor, 3 weeks after 10 years, and 4 weeks after 15 years. Length of service includes the whole span of continuous service with the present contractor or successor, wherever employed, and with the predecessor contractors in the performance of similar work at the same Federal facility. (See 29 CFR 4.173)

** HAZARDOUS PAY DIFFERENTIAL **

An 8 percent differential is applicable to employees employed in a position that represents a high degree of hazard when working with or in close proximity to ordnance, explosives, and incendiary materials. This includes work such as screening, blending, dying, mixing, and pressing of sensitive ordnance, explosives, and pyrotechnic compositions such as lead azide, black powder and photoflash powder. All dry-house activities involving propellants or explosives. Demilitarization, modification, renovation, demolition, and maintenance operations on sensitive ordnance, explosives and incendiary materials. All operations involving re-grading and cleaning of artillery ranges.

A 4 percent differential is applicable to employees employed in a position that represents a low degree of hazard when working with, or in close proximity to ordnance, (or employees possibly adjacent to) explosives and incendiary materials which involves potential injury such as laceration of hands, face, or arms of the employee engaged in the operation, irritation of the skin, minor burns and the like; minimal damage to immediate or adjacent work area or equipment being used. All operations involving, unloading, storage, and hauling of ordnance, explosive, and incendiary ordnance material other than small arms ammunition. These differentials are only applicable to work that has been specifically designated by the agency for ordnance, explosives, and incendiary material differential pay.

** UNIFORM ALLOWANCE **

If employees are required to wear uniforms in the performance of this contract (either by the terms of the Government contract, by the employer, by the state or local law, etc.), the cost of furnishing such uniforms and maintaining (by laundering or dry cleaning) such uniforms is an expense that may not be borne by an employee where such cost reduces the hourly rate below that required by the wage determination. The Department of Labor will accept payment in accordance with the following standards as compliance:

The contractor or subcontractor is required to furnish all employees with an adequate number of uniforms without cost or to reimburse employees for the actual cost of the uniforms. In addition, where uniform cleaning and maintenance is made the responsibility of the employee, all contractors and subcontractors subject to this wage determination shall (in the absence of a bona fide collective bargaining agreement providing for a different amount, or the furnishing of contrary affirmative proof as to the actual cost), reimburse all employees for such cleaning and maintenance at a rate of $3.35 per week (or $.67 cents per day).

However, in those instances where the uniforms furnished are made of "wash and wear" materials, may be routinely washed and dried with other personal garments, and do not require any special treatment such as dry cleaning, daily washing, or commercial laundering in order to meet the cleanliness or appearance standards set by the terms of the Government contract, by the contractor, by law, or by the nature of the work, there is no requirement that employees be reimbursed for uniform maintenance costs.

** SERVICE CONTRACT ACT DIRECTORY OF OCCUPATIONS **

The duties of employees under job titles listed are those described in the "Service Contract Act Directory of Occupations", Fifth Edition (Revision 1), dated September 2015, unless otherwise indicated.

REQUEST FOR AUTHORIZATION OF ADDITIONAL CLASSIFICATION AND WAGE RATE

Standard Form 1444 (SF-1444) Conformance Process:

The contracting officer shall require that any class of service employee which is not listed herein and which is to be employed under the contract (i.e., the work to be performed is not performed by any classification listed in the wage determination), be classified by the contractor so as to provide a reasonable relationship (i.e., appropriate level of skill comparison) between such unlisted classifications and the classifications listed in the wage determination. Such conformed classes of employees shall be paid the monetary wages and furnished the fringe benefits as are determined (See 29 CFR 4.6(b)(2)(i)). Such conforming procedures shall be initiated by the contractor prior to the performance of contract work by such unlisted class(es) of employees (See 29 CFR 4.6(b)(2)(ii)). The Wage and Hour Division shall make a final determination of conformed classification, wage rate, and/or fringe benefits which shall be retroactive to the commencement date of the contract (See 29 CFR 4.6(b)(2)(iv)(C)(vi)). When multiple wage determinations are included in a contract, a separate SF-1444 should be prepared for each wage determination to which a class(es) is to be conformed.

The process for preparing a conformance request is as follows:

When preparing the bid, the contractor identifies the need for a conformed occupation(s) and computes a proposed rate(s).

After contract award, the contractor prepares a written report listing in order the proposed classification title(s), a Federal grade equivalency (FGE) for each proposed classification(s), job description(s), and rationale for proposed wage rate(s), including information regarding the agreement or disagreement of the authorized representative of the employees involved, or where there is no authorized representative, the employees themselves. This report should be submitted to the contracting officer no later than 30 days after such unlisted class(es) of employees performs any contract work.

The contracting officer reviews the proposed action and promptly submits a report of the action, together with the agency's recommendations and pertinent information including the position of the contractor and the employees, to the Wage and Hour Division, U.S. Department of Labor, for review (See 29 CFR 4.6(b)(2)(ii)).

Within 30 days of receipt, the Wage and Hour Division approves, modifies, or disapproves the action via transmittal to the agency contracting officer, or notifies the contracting officer that additional time will be required to process the request.

The contracting officer transmits the Wage and Hour decision to the contractor.

The contractor informs the affected employees.

Information required by the Regulations must be submitted on SF-1444 or bond paper.

When preparing a conformance request, the "Service Contract Act Directory of Occupations" (the Directory) should be used to compare job definitions to ensure that duties requested are not performed by a

classification already listed in the wage determination. Remember, it is not the job title, but the required tasks that determine whether a class is included in an established wage determination. Conformances may not be used to artificially split, combine, or subdivide classifications listed in the wage determination.

** OCCUPATIONS NOT INCLUDED IN THE SCA DIRECTORY OF OCCUPATIONS **

Aircraft Quality Control Inspector

Develops and implements quality control and ground safety programs to ensure compliance with contract specifications. Inspects and verifies proper completion and documentation of safety and flight discrepancies. Briefs and debriefs pilots and crew members assigned to functional check flights. Evaluates personnel, including verification of skills, training and experience. Performs audits and inspections of work centers and ongoing maintenance actions, procedures, equipment and facilities. Monitors timeliness and applicability of aircraft maintenance technical data and technical library. Reviews maintenance source documents, aircraft inspection records, notes recurring discrepancies or trends and initiates appropriate action. Manages the material deficiency and technical order improvement program. Reviews engineering investigation requests.

Initiates and reviews quality deficiency reports, technical deficiency reports and hazardous material reports, ensuring that they are accurate, clear, concise and comprehensive. Receives aircraft and explosive mishap reports and studies them for applicability. Oversees aircraft weight and balance program. Conducts safety inspections, training and drills.

Chief Cook

Directs and participates in the preparation and serving of meals; determines timing and sequence of operations required to meet serving times; inspects galley/kitchen unit and equipment for cleanliness and proper storage and preparation of food. Many plan or assist in planning meals and taking inventory of stores and equipment.

Environmental Protection Specialist

Environmental protection specialist positions require specialized knowledge of the principles, practices, and methods of program or administrative work relating to environmental protection programs. This entails (1) an understanding of the philosophy underlying environmental regulation; (2) knowledge of environmental laws and regulations; (3) knowledge of the planning, funding, organization, administration, and evaluation of environmental programs; (4) practical knowledge of environmental sciences and related disciplines, the effects of actions and technology on the environment, the means of preventing or reducing pollution, and the relationship between environmental factors and human health and well-being; and (5) practical knowledge of important historic, cultural, and natural resources (including land, vegetation, fish, wildlife, endangered species, forests) and the relationship between the preservation and management of these resources and environmental protection. Environmental protection specialists apply specialized knowledge of one or more program or functional areas of environmental protection work, but do not require full professional competence in environmental engineering or science.

Fire Safety Professional

The Fire Safety Professional works to control and extinguish fires, rescue persons endangered by fire, and reduce or eliminate potential fire hazards. It also controls hazardous materials incidents, provides emergency medical services, trains personnel in fire protection and prevention, operates fire communications equipment, develops and implements fire protection and prevention plans, procedures, and standards and, advises on improvements to structures for better fire prevention.

Quality Assurance Representative I

A Quality Assurance Representative I independently inspects a few standardized procedures, items or operations of limited difficulty. A Quality Assurance Representative I's assignments involve independent record keeping and preparation of reports, inspection and testing, interpretation of plans and specifications and observation of construction activities to check adherence to safety practices and requirements. Quality Assurance Representative I's maintain work relationships with contractor supervisory personnel. Contacts involve obtaining information on sequence of operations and work methods, explaining standard requirements of plans and specifications, and informing the contractor of inspection results.

Quality Assurance Representative II

A Quality Assurance Representative II independently inspects a wide variety of standardized items or operations requiring a substantial knowledge of the method and techniques of construction inspection and of construction methods, equipment, materials, practices and the ability to interpret varied requirements in drawings and specifications. Quality Assurance Representative II's obtain information on schedules and work methods and explain requirements of plans and specifications. They make suggestions to the contractor concerning well-established acceptable methods and practices to assist the contractor in meeting standard requirements. Quality Assurance Representative II's are typically not authorized to approve deviations in construction plans, methods and practices even of a minor nature.

Quality Assurance Representative III

A Quality Assurance Representative III is expected to interpret plans and specifications relating to construction problems of normal difficulty, that is, those for which there are precedents and those without unusual complications. Quality Assurance Representative III's resolve differences between plans and specifications when such differences do not involve questions of cost or engineering design. Engineering and supervisory assistance is readily available and is provided as needed to assist in interpreting plans and specifications and in resolving differences involving complex problems. Technical assistance is also available on unusual specialized trade, crafts or materials problems. Inspection reports are reviewed for accuracy, completeness and adequacy. Unusually difficult and novel problems are discussed with the supervisor.

Quality Assurance Representative III's are typically authorized to approve minor deviations in construction methods and practices which conform to established precedents, do not involve added costs, and are consistent with contract plans and specifications. Decisions by Quality Assurance Representative III's on the acceptability of construction methods and practices, workmanship, materials, and the finished product are considered to be final.

"REGISTER OF WAGE DETERMINATIONS UNDER  |        U.S. DEPARTMENT OF LABOR

THE SERVICE CONTRACT ACT        |  EMPLOYMENT STANDARDS ADMINISTRATION

By direction of the Secretary of Labor |         WAGE AND HOUR DIVISION

|         WASHINGTON D.C.  20210

|

|

|

| Wage Determination No.: 1995-0222

Daniel W. Simms          Division of   |           Revision No.: 71

Director            Wage Determinations|  Date Of Last Revision: 12/03/2025

_______________________________________|____________________________________________

Nationwide: Applicable in the continental U.S. Alaska, Puerto Rico, Hawaii and

Virgin Islands.

____________________________________________________________________________________

**Fringe Benefits Required Follow the Occupational Listing**

Employed on U.S. Government contracts for aerial photographer, aerial seeding,

aerial spraying, transportation of personnel and cargo, fire reconnaissance,

administrative flying, fire detection, air taxi mail service, and other flying

services.

OCCUPATION CODE - TITLE                                  FOOTNOTE               RATE

31010 - Airplane Pilot                                                         39.08

(not set) - First Officer (Co-Pilot)                                           35.58

(not set) - Aerial Photographer                                                19.52

EXCEPT SCHEDULED AIRLINE TRANSPORTATION AND LARGE MULTI-ENGINE AIRCRAFT SUCH AS THE

B-727, DC-8, AND THE DC-9.

____________________________________________________________________________________

Note: Executive Order (EO) 13706, Establishing Paid Sick Leave for Federal

Contractors, applies to all contracts subject to the Service Contract Act for which

the contract is awarded (and any solicitation was issued) on or after January 1,

2017.  If this contract is covered by the EO, the contractor must provide employees

with 1 hour of paid sick leave for every 30 hours they work, up to 56 hours of paid

sick leave each year. Employees must be permitted to use paid sick leave for their

own illness, injury or other health-related needs, including preventive care; to

assist a family member (or person who is like family to the employee) who is ill,

injured, or has other health-related needs, including preventive care; or for

reasons resulting from, or to assist a family member (or person who is like family

to the employee) who is the victim of, domestic violence, sexual assault, or

stalking.  Additional information on contractor requirements and worker protections

under the EO is available at www.dol.gov/whd/govcontracts.

Note: Executive Order 13658 generally applies to contracts subject to the Service

Contract Act that were awarded on or between January 1, 2015 and January 29, 2022,

and that have not been renewed or extended on or after January 30, 2022. If a

contract is subject to Executive Order 13658, the contractor must pay all covered

workers at least $13.30 per hour (or the applicable wage rate listed on this wage

determination, if it is higher) for all hours spent performing on the contract in

2025. The applicable Executive Order minimum wage rate will be adjusted annually.

Additional information on contractor requirements and worker protections under

Executive Order 13658 is available at www.dol.gov/whd/govcontracts.

ALL OCCUPATIONS LISTED ABOVE RECEIVE THE FOLLOWING BENEFITS:

HEALTH & WELFARE: $5.55 per hour, up to 40 hours per week, or $222.00 per week or

$962.00 per month

HEALTH & WELFARE EO 13706: $5.09 per hour, up to 40 hours per week, or $203.60 per

week, or $882.27 per month*

*This rate is to be used only when compensating employees for performance on an SCA-

covered contract also covered by EO 13706, Establishing Paid Sick Leave for Federal

Contractors.  A contractor may not receive credit toward its SCA obligations for any

paid sick leave provided pursuant to EO 13706.

VACATION: 2 weeks paid vacation after 1 year of service with a contractor or

successor, 3 weeks after 5 years, and 4 weeks after 15 years.  Length of service

includes the whole span of continuous service with the present contractor or

successor, wherever employed, and with the predecessor contractors in the

performance of similar work at the same Federal facility.  (Reg. 29 CFR 4.173)

HOLIDAYS: A minimum of eleven paid holidays per year: New Year's Day, Martin

Luther King Jr.'s Birthday, Washington's Birthday, Memorial Day, Juneteenth

National Independence Day, Independence Day, Labor Day, Columbus Day, Veterans'

Day, Thanksgiving Day, and Christmas Day. (A contractor may substitute for any of

the named holidays another day off with pay in accordance with a plan communicated

to the employees involved.)  (See 29 CFR 4.174)

VACATION (Hawaii): 2 weeks paid vacation after 1 year of service with a contractor

or successor; 3 weeks after 10 years, and 4 weeks after 15 years.  Length of service

includes the whole span of continuous service  with the present contractor or

successor, wherever employed, and with the predecessor contractors in the

performance of similar work at the same Federal facility. (Reg. 29 CFR 4.173)

HEALTH & WELFARE (Hawaii):  $2.42 per hour, up to 40 hours per week, or $96.80 per

week, or $419.47 per month for all employees on whose behalf the contractor provides

health care benefits pursuant to the Hawaii prepaid Health Care Act.  For those

employees who are not receiving health care benefits mandated by the Hawaii prepaid

Health Care Act, the new health and welfare benefit rate will be $5.55 per hour, up

to 40 hours per week.

HEALTH & WELFARE (Hawaii EO 13706):  $1.96 per hour, up to 40 hours per week, or $78.

40 per week, or $339.73 per month for all employees on whose behalf the contractor

provides health care benefits pursuant to the Hawaii prepaid Health Care Act.  For

those employees who are not receiving health care benefits mandated by the Hawaii

prepaid Health Care Act, the new health and welfare benefit rate will be $5.09 per

hour, up to 40 hours per week. *

*This rate is to be used only when compensating employees for performance on an SCA-

covered contract also covered by EO 13706, Establishing Paid Sick Leave for Federal

Contractors.  A contractor may not receive credit toward its SCA obligations for any

paid sick leave provided pursuant to EO 13706.

** HAZARDOUS PAY DIFFERENTIAL **

An 8 percent differential is applicable to employees employed in a position that

represents a high degree of hazard when working with or in close proximity to

ordnance, explosives, and incendiary materials.  This includes work such as

screening, blending, dying, mixing, and pressing of sensitive ordnance, explosives,

and pyrotechnic compositions such as lead azide, black powder and photoflash powder.

All dry-house activities involving propellants or explosives.  Demilitarization,

modification, renovation, demolition, and maintenance operations on sensitive

ordnance, explosives and incendiary materials.  All operations involving re-grading

and cleaning of artillery ranges.

A 4 percent differential is applicable to employees employed in a position that

represents a low degree of hazard when working with, or in close proximity to

ordnance, (or employees possibly adjacent to) explosives and incendiary materials

which involves potential injury such as laceration of hands, face, or arms of the

employee engaged in the operation, irritation of the skin, minor burns and the like;

minimal damage to immediate or adjacent work area or equipment being used.  All

operations involving, unloading, storage, and hauling of ordnance, explosive, and

incendiary ordnance material other than small arms ammunition.  These differentials

are only applicable to work that has been specifically designated by the agency for

ordnance, explosives, and incendiary material differential pay.

** UNIFORM ALLOWANCE **

If employees are required to wear uniforms in the performance of this contract

(either by the terms of the Government contract, by the employer, by the state or

local law, etc.), the cost of furnishing such uniforms and maintaining (by

laundering or dry cleaning) such uniforms is an expense that may not be borne by an

employee where such cost reduces the hourly rate below that required by the wage

determination. The Department of Labor will accept payment in accordance with the

following standards as compliance:

The contractor or subcontractor is required to furnish all employees with an

adequate number of uniforms without cost or to reimburse employees for the actual

cost of the uniforms.  In addition, where uniform cleaning and maintenance is made

the responsibility of the employee, all contractors and subcontractors subject to

this wage determination shall (in the absence of a bona fide collective bargaining

agreement providing for a different amount, or the furnishing of contrary

affirmative proof as to the actual cost), reimburse all employees for such cleaning

and maintenance at a rate of $3.35 per week (or $.67 cents per day).  However, in

those instances where the uniforms furnished are made of ""wash and wear""

materials, may be routinely washed and dried with other personal garments, and do

not require any special treatment such as dry cleaning, daily washing, or commercial

laundering in order to meet the cleanliness or appearance standards set by the terms

of the Government contract, by the contractor, by law, or by the nature of the work,

there is no requirement that employees be reimbursed for uniform maintenance costs.

** SERVICE CONTRACT ACT DIRECTORY OF OCCUPATIONS **

The duties of employees under job titles listed are those described in the

""Service Contract Act Directory of Occupations"", Fifth Edition (Revision 1),

dated September 2015, unless otherwise indicated.

REQUEST FOR AUTHORIZATION OF ADDITIONAL CLASSIFICATION AND WAGE RATE

Standard Form 1444 (SF-1444)

Conformance Process:

The contracting officer shall require that any class of service employee which is

not listed herein and which is to be employed under the contract (i.e., the work to

be performed is not performed by any classification listed in the wage

determination), be classified by the contractor so as to provide a reasonable

relationship (i.e., appropriate level of skill comparison) between such unlisted

classifications and the classifications listed in the wage determination. Such

conformed classes of employees shall be paid the monetary wages and furnished the

fringe benefits as are determined (See 29 CFR 4.6(b)(2)(i)). Such conforming

procedures shall be initiated by the contractor prior to the performance of contract

work by such unlisted class(es) of employees (See 29 CFR 4.6(b)(2)(ii)). The Wage

and Hour Division shall make a final determination of conformed classification, wage

rate, and/or fringe benefits which shall be retroactive to the commencement date of

the contract (See 29 CFR 4.6(b)(2)(iv)(C)(vi)). When multiple wage determinations

are included in a contract, a separate SF-1444 should be prepared for each wage

determination to which a class(es) is to be conformed.

The process for preparing a conformance request is as follows:

1) When preparing the bid, the contractor identifies the need for a conformed

occupation(s) and computes a proposed rate(s).

2) After contract award, the contractor prepares a written report listing in order

the proposed classification title(s), a Federal grade equivalency (FGE) for each

proposed classification(s), job description(s), and rationale for proposed wage

rate(s), including information regarding the agreement or disagreement of the

authorized representative of the employees involved, or where there is no authorized

representative, the employees themselves. This report should be submitted to the

contracting officer no later than 30 days after such unlisted class(es) of employees

performs any contract work.

3) The contracting officer reviews the proposed action and promptly submits a report

of the action, together with the agency's recommendations and pertinent

information including the position of the contractor and the employees, to the Wage

and Hour Division, U.S. Department of Labor, for review (See 29 CFR 4.6(b)(2)(ii)).

4) Within 30 days of receipt, the Wage and Hour Division approves, modifies, or

disapproves the action via transmittal to the agency contracting officer, or

notifies the contracting officer that additional time will be required to process

the request.

5) The contracting officer transmits the Wage and Hour decision to the contractor.

6) The contractor informs the affected employees.

Information required by the Regulations must be submitted on SF-1444 or bond paper.

When preparing a conformance request, the ""Service Contract Act Directory of

Occupations"" (the Directory) should be used to compare job definitions to ensure

that duties requested are not performed by a classification already listed in the

wage determination. Remember, it is not the job title, but the required tasks that

determine whether a class is included in an established wage determination.

Conformances may not be used to artificially split, combine, or subdivide

classifications listed in the wage determination.

** OCCUPATIONS NOT INCLUDED IN THE SCA DIRECTORY OF OCCUPATIONS **

Aerial Photographer

The aerial photographer must be skilled in reading flight maps, capable of assisting

the pilot to adhere to flight lines, be able to level and operate a cartographic

camera and its auxiliary equipment mounted in the aircraft so that the photographs

that are taken will have the required forward lap and side lap for use in

photogrammetric mapping equipment, and possess a working knowledge of aerial films

and  camera filters to insure proper exposure of the films.

First Officer (Co-Pilot)

Is second in command of commercial airplane and its crew while transporting

passengers, mail, or other cargo on scheduled or nonscheduled flights.  Assists or

relieves an airline captain in operating the controls of an airplane; monitoring

flight and engine instruments; and maintaining air-to-ground communications."