Project:  DSW Lolamai Point Generator, ATS, and MTS Startup & Testing Service

Requirement: Purchase and Installation service in accordance with attachment 2 PWS

SPECIFICATIONS

Caterpillar Propane Generator

- 30 kW standby rated

- UL2200 package, Electronic Control Panel

- 120/240-volt, 1 phase, 60 hertz. 1800rpm

- Main Circuit Breakers LSI – 100A

- No Enclosure

- Battery charger

- Lead Acid Batteries

- Jacket Water Heater

- Site testing and startup per spec, client training

ASCO Automatic Transfer Switch

- 200A ASCO ATS

- Open transition, Nema 1, 3-pole

ASCO Manual Transfer Switch with camlocks

- 200A ASCO MTS (Nema 1) with camlocks (Nema 3R)

- Open transition, 3- pole