| SUBJECT* | 621-26-S-006 Campus Wide Stripping & Painting |
| --- | --- |

GENERAL INFORMATION

| CONTRACTING OFFICE’S ZIP CODE* | 37129 |
| --- | --- |
| SOLICITATION NUMBER* | 36C24926B0004 |
| RESPONSE DATE/TIME/ZONE |  |
| ARCHIVE | 99 DAYS AFTER THE RESPONSE DATE |
| SET-ASIDE | SDVOSBC |  |
| PRODUCT SERVICE CODE* | Z1LZ |  |
| NAICS CODE* | 237310 |  |
| PLACE OF PERFORMANCE | James H. Quillen VA Medical Center (JHQVAMC) |  |
|  | Lamont & Veterans Way |  |
|  | Mountain Home TN |  |
| POSTAL CODE | 37684 |  |
| COUNTRY | USA |  |

CONTACT INFORMATION

| CONTRACTING OFFICE ADDRESS | Department of Veterans Affairs Network Contracting Office 9 (90C) NCO 9 1639 Medical Center Parkway, Suite 204 Murfreesboro TN 37129 |
| --- | --- |
| POINT OF CONTACT* | Michael R. Brown Michael.brown1305@va.gov |

ADDITIONAL INFORMATION

| AGENCY’S URL |  |
| --- | --- |
| URL DESCRIPTION |  |
| AGENCY CONTACT’S EMAIL ADDRESS | Michael.brown1305@va.gov |
| EMAIL DESCRIPTION |  |

DESCRIPTION

621-26-S-006 - Campus Wide Stripping & Painting

STATEMENT OF WORK

Campus Wide Striping and Painting Services

at James H. Quillen Veterans Affairs Medical Center (JHQVAMC)

- Objective

This Scope of Work outlines the tasks, materials, and responsibilities for re-striping services at Different parking lots located at the Mt Home VAMC. The objective is to refresh and enhance the existing pavement markings to improve visibility, safety, and traffic flow for vehicles and pedestrians. The project includes re-stripping parking spaces, directional arrows, crosswalks, handicap spaces, ADA Access Early warning tiles (50), and other necessary markings across all parking areas; Prepare area and Install 65’X5’ concrete walkway from existing healing garden walkway to S. First St sidewalk. Saw cut joints every 5’ and use control joints at existing concrete. Remove excavated dirt from the site and sow in grass to match.

Performance Period: 365 calendar days after issuance of the Notice to Proceed (NTP).

- Scope of Work

- Site Preparation

  - Clean all surfaces to be re-striped by sweeping, power washing, or other methods to remove dirt, debris, oil stains, and loose paint, ensuring proper adhesion of new markings.

  - Coordinate traffic shutdowns with COR.

- Materials and Specifications

  - Use high-quality, durable traffic paint suitable for concrete surfaces and compliant with local regulations.

  - Colors: White for standard parking lines, yellow for directional arrows and no-parking zones, blue for handicap spaces, and red for fire lanes (or as specified by client/local codes).

  - Ensure all markings meet American Disability Act standards for handicapped spaces, including size, spacing, and signage.

  - Provide stencils for arrows, numbers, letters, and symbols as needed.

- Re-Striping Execution

  - Parking Spaces: Re-stripe all parking lines on each of the 4 floors, maintaining consistent dimensions (e.g., 9 ft wide x 18 ft long for standard spaces, or as per existing layout).

  - Directional Markings: Refresh arrows, lane dividers, and stop bars to guide traffic flow.

  - Pedestrian Areas: Stripe crosswalks, walkways, and loading zones with appropriate hatching or solid fill.

  - Specialty Markings: Update handicap spaces, fire lanes, no-parking zones, and reserved parking areas with clear, legible markings.

  - Apply reflective beads or additives to paint (if specified) to enhance visibility in low-light conditions.

- Concrete walkway

  - Prepare area and Install 65’X5’ concrete walkway from existing healing garden walkway to S. First St sidewalk. Saw cut joints every 5’ and use control joints at existing concrete. Remove excavated dirt from the site and sow in grass to match.

- ADA Access Early warning tiles (50)

  - Prepare the existing access ramps to receive ADA Access Early Warning Tiles (quantity: 50), with the final locations to be determined.

  - Cut the existing concrete for the installation of Access Early Warning tiles, ensuring that the tiles do not create a tripping hazard and paint the ramps.

- Quality Control

  - Ensure straight, clean lines with no overspray or smudging.

  - Verify compliance with the original layout or approved updated design plan.

  - Conduct a final walkthrough with the property manager or designated representative to confirm satisfaction with the work.

- Safety and Traffic Management

  - Install temporary signage, cones, and barriers to cordon off work areas and ensure the safety of workers, vehicles, and pedestrians.

  - Work in sections to be done after hours and on weekends.

  - Adhere to Occupational Safety and Health Administration safety standards and local traffic control requirements.

- Cleanup and Completion

  - Remove all equipment, debris, and temporary signage upon completion of each floor.

  - Dispose of any waste materials (e.g., paint cans, cleaning supplies) in accordance with environmental regulations.

  - Provide a final report or documentation of completed work, if requested.

- Assumptions and Exclusions

Major pavement repairs (e.g., crack filling, resurfacing) are not included in this scope and will require a separate agreement.

- Warranty: Not applicable