1 EA Skid Steer Loader, Compact Radial Lift, including attachments.

Drive system - Engine

Engine type: Diesel, 4-Cylinder

Exhaust emission: Stage V / Tier IV

Min. Power: 34.3 kW (46 hp) @ 2800 rpm

Min. Torque: 146 Nm @ 1820 rpm

Min. Displacement: 2.19 L

Drive System: Hydrostatic

Performance

Max. Travel Speed: 12.9 km/h

Rated Operating Capacity: min. 612 kg

Rated Operating Capacity with optional counterweight: min. 680 kg

Tipping Load: min. 1225 kg

Chassis and tires

Tires: 10 x 16.5-10, Heavy Duty

Wheelbase: min. 950 mm

Ground Clearance: min. 200 mm

Overall Width (less bucket): approx. 1364 mm

Overall Height (to top of ROPS): approx. 1930 mm

Overall Length (less bucket): approx. 2342 mm

Approx. Operating Weight: 2384 kg

Operator Station (Cabin)

Structure: ROPS/FOPS Level II Overhead Guard, Half-cab with sliding safety glass side window.

Seat: Air-suspended luxury driver's seat with high backrest and a min. 5 cm wide safety belt.

Controls: Manual dual T-lever (hand) control system.

Safety: Parking brake with manual release.

Lighting: Halogen rotating beacon, orange.

Hydraulics & Lift Arm

Lift Arm Type: Radial Lift

Auxiliary Hydraulic Flow: min. 63.6 l/min

System Pressure: min. 207 bar

Height to Hinge Pin (fully raised): min. 2794 mm

Dump Height (fully raised): min. 2123 mm

Dump Reach (fully raised): min. 584 mm

Attachment System: Manual quick-change system (universal type).

Features: Switchable, self-leveling system for lift action.

Included Equipment & Additional Requirements

Rear cast counterweight bumper.

Earth bucket with or without teeth.

Bucket Width: approx. 1,525 mm

Bucket Capacity: approx. 350 liters

Warranty

Min. 12-month manufacturer's warranty.

Documentation

1x Operator's manual and parts list in German language.

Instruction and Training in German language after delivery and scheduled with customer.

Delivery

Delivery to: USAG Ansbach Storck Barracks Illesheim